Optics Group

The Optics Group contains all settings that relate to the physical optics of the projector, including focus, zoom, and keystoning adjustments, if these mechanical adjustments can be electromechanically controlled by the management agent.

The properties in this group must be available for writing at all times, and the value must persist across power cycles.  If the projector is running on flea power, then changing the setting value will not cause a mechanical action.  Mechanical adjustment will be made when the projector is powered on based on the current value of the setting, which might have changed while the projector was powered down.