A Project of the PWG IPPFAX Working Group

Universal Image Format (UIF)

IEEE-ISTO Printer Working Group

Draft Standard D0.5

June 20, 2001

ftp://ftp.pwg.org/pub/pwg/QUALDOCS/uif-spec-05.pdf, .doc, .rtf

Abstract

This standard specifies the Universal Image Format (UIF).  The UIF requirements [7] are derived from the requirements for IPPFAX [8] and Internet Fax [9].

In summary UIF is a raster image data format intended for use by, but not limited to, the IPPFAX protocol, which is used to provide a synchronous, reliable exchange of image Documents between Senders and Receivers. UIF is based on the TIFF-FX specification [4], which describes the TIFF (Tag Image File Format) representation of image data specified by the ITU-T Recommendations for black-and-white and color facsimile.

This document (1) formally defines a series of “UIF profiles” distinguished primarily by the method of compression employed and color space used; (2) describes the use of CONNEG in capabilities communication between two UIF-enabled Devices; and (3) defines a set of baseline capabilities that permit a CONNEG implementation to be optional.

This document is a draft of an IEEE-ISTO PWG Proposed Standard and is in full conformance with all provisions of the PWG Process (see: ftp//ftp.pwg.org/pub/pwg/general/pwg-process.pdf).  PWG Proposed Standards are working documents of the IEEE-ISTO PWG and its working groups.  The list of current PWG projects and drafts can be obtained at http://www.pwg.org.

When approved as a PWG standard, this document will be available from:

ftp://ftp.pwg.org/pub/pwg/standards/pwg510x.y.pdf, .doc, .rtf

Copyright (C) 2001, IEEE Industry Standards and Technology Organization. All rights reserved.

This document may be copied and furnished to others, and derivative works that comment on, or otherwise explain it or assist in its implementation may be prepared, copied, published and distributed, in whole or in part, without restriction of any kind, provided that the above copyright notice, this paragraph and the title of the Document as referenced below are included on all such copies and derivative works.  However, this document itself may not be modified in any way, such as by removing the copyright notice or references to the IEEE-ISTO and the Printer Working Group, a program of the IEEE-ISTO.

Title:  Universal Image Format

The IEEE-ISTO and the Printer Working Group DISCLAIM ANY AND ALL WARRANTIES, WHETHER EXPRESS OR IMPLIED INCLUDING (WITHOUT LIMITATION) ANY IMPLIED WARRANTIES OF MERCHANTABILITY OR FITNESS FOR A PARTICULAR PURPOSE.

The Printer Working Group, a program of the IEEE-ISTO, reserves the right to make changes to the document without further notice.  The document may be updated, replaced or made obsolete by other documents at any time.

The IEEE-ISTO takes no position regarding the validity or scope of any intellectual property or other rights that might be claimed to pertain to the implementation or use of the technology described in this document or the extent to which any license under such rights might or might not be available; neither does it represent that it has made any effort to identify any such rights.

The IEEE-ISTO invites any interested party to bring to its attention any copyrights, patents, or patent applications, or other proprietary rights which may cover technology that may be required to implement the contents of this document. The IEEE-ISTO and its programs shall not be responsible for identifying patents for which a license may be required by a document and/or IEEE-ISTO Industry Group Standard or for conducting inquiries into the legal validity or scope of those patents that are brought to its attention. Inquiries may be submitted to the IEEE-ISTO by e-mail at:

ieee-isto@ieee.org.

The Printer Working Group acknowledges that the IEEE-ISTO (acting itself or through its designees) is, and shall at all times, be the sole entity that may authorize the use of certification marks, trademarks, or other special designations to indicate compliance with these materials.

Use of this document is wholly voluntary.  The existence of this document does not imply that there are no other ways to produce, test, measure, purchase, market, or provide other goods and services related to its scope.

Table of Contents

1	Introduction	5

2	Terminology	5

2.1	Conformance Terminology	5

2.2	Model	5

3	Indicating Support using MIME	6

3.1	MIME content type type	6

3.2	MIME application parameter	6

4	TIFF-FX support	6

4.1	UIF Profile Relationships	7

4.2	UIF Profile S 	7

4.3	UIF Profile F 	8

4.4	UIF Profile J	10

4.5	UIF Profile C	12

4.6	UIF Profile L	13

4.7	UIF Profile M	15

5	Capabilities communication	18

5.1	Receiver capabilities string	18

5.1.1	New CONNEG Tags	18

5.1.2	Minimum Receiver capabilities	18

5.1.2.1	Minimum capabilities for UIF Profile S	18

5.1.2.2	Minimum capabilities for UIF Profile F	19

5.1.2.3	Minimum capabilities for UIF Profile J	19

5.1.2.4	Minimum capabilities for UIF Profile C	19

5.1.2.5	Minimum capabilities for UIF Profile L	20

5.1.2.6	Minimum capabilities for UIF Profile M	20

5.2	UIF profiles supported	20

5.3	Media supported	21

5.4	Media ready	21

5.5	Image reduction supported	21

6	Sender requirements	21

6.1	Image-Reduction	21

6.2	Intra-job media selection	22

7	Conformance Requirements	22

8	References	22

9	Issues	23

9.1	Outstanding Issues	23

9.2	Resolved Issues	24

10	Actions	25

11	Revision History (to be removed when standard is approved)	25

Table of Tables

Table 1.  UIF Profile S Baseline Fields	7

Table 2.  UIF Profile S Extension Fields	8

Table 3.  UIF Profile F  Baseline Fields	9

Table 4.  UIF Profile F  Extension Fields	9

Table 5.  UIF Profile F  New Fields	10

Table 6.  UIF Profile J  Baseline Fields	11

Table 7.  UIF Profile J  Extension Fields	11

Table 8.  UIF Profile J  New Fields	11

Table 9.  UIF Profile C  Baseline Fields	12

Table 10.  UIF Profile C  Extension Fields	13

Table 11.  UIF Profile C  New Fields	13

Table 12.  UIF Profile L  Baseline Fields	14

Table 13.  UIF Profile L  Extension Fields	15

Table 14.  UIF Profile L  New Fields	15

Table 15.  UIF Profile M  Baseline Fields	16

Table 16.  UIF Profile M Extension Fields	17

Table 17.  UIF Profile M New Fields	17

Table 18.   Underlying Protocol Conformance.	22

# Introduction

- This document specifies  an image data format based on TIFF-FX [4] especially suited for use with synchronous protocols (e.g., IPPFAX[10]). The increased conformance requirements found in this UIF specification reflect the need for a data format where quality document transmission is the primary concern. The complete support for TIFF-FX in this way is called Universal Image Format (UIF). There are several pieces to this support: A specification of precisely what parts of the TIFF-FX specification are to be supported

- How the UIF-capable Sender uses CONNEG to discover the UIF characteristics (resolution, drawing surface, etc.) of a potential UIF Receiver.

- How the Sender specifies options for the transmission (reducing for example).

# Terminology

This section defines the following additional terms that are used throughout this standard.

## Conformance Terminology

The key words MUST, MUST NOT, REQUIRED, SHOULD, SHOULD NOT, RECOMMENDED, MAY, and OPTIONAL in this document are to be interpreted as described in RFC 2119 [18].

## Model

The following terms are introduced and capitalized in order to indicate their specific meaning:

Baseline Field – One of the core set of TIFF fields introduced by the TIFF specification [19]

Device  – A Sender or Receiver

Document – The electronic representation of a set of one or more pages that the Sender sends to the Receiver.

Extension Field – One of the TIFF extension fields introduced by the current TIFF specification [19], specification, the set of Pagemaker TIFF Technical Notes [20], or TIFF Technical Note 2 [21].

New Field  – One of the new TIFF fields introduced by the TIFF-FX specification [4]. Note that the UIF specification does not introduce any new TIFF tags.

Receiver – This is the agent (software, hardware or some combination) that receives the Document sent by the Sender.

Sender – This is the agent (software, hardware or some combination) that is used to transmit a Document to a Receiver.

# Indicating Support using MIME

[22] describes the registration of the MIME content-type image/tiff to refer to TIFF encoded image data. In addition, an optional "application" parameter is defined for image/tiff to identify a particular application's subset of TIFF and TIFF extensions for the encoded image data, if it is known. Typically, this would be used to assist the recipient in dispatching a suitable rendering package to handle the display or processing of the image file.

## MIME content type type

When transported by MIME, the TIFF content defined by this document MUST be encoded within an ‘image/tiff’ content type.

## MIME application parameter

The two values of the image/tiff application parameter as defined for UIF are ‘uifbw’ and ‘uifcolor’.

The "uifbw" application parameter is suitable for use by Senders that can process one or more UIF  profiles used for the encoding of black and white facsimile data.

The "uifcolor" application parameter is suitable for use by Senders that can process one or more UIF profiles or subsets that can be used for the encoding of black and white, AND color facsimile data.

Since this document defines several UIF profiles, the following rules MUST be followed when setting the application parameter value. Senders that use UIF Profiles S, F, or J, MUST set the value of the application parameter to “uifbw”. Senders  which use UIF Profiles C, L, or M, MUST set the value of the application parameter to "uifcolor".

An example of the use of the image/tiff MIME Content-type with the application parameter set with the value 'uifbw' follows:

Content-type: image/tiff; application=uifbw

In this example, use of this parameter value will enable a Receiver to identify the content as being within a profile or subset of UIF that is suitable for encoding black and white image data, before attempting to process the image data.

In a similar respect, an example of the image/tiff MIME Content-type with the application parameter setting suitable for handling a color subset or profile of TIFF for facsimile is shown below:

Content-type: image/tiff; application=uifcolor

# TIFF-FX support

A profile is based on a collection of ITU-T facsimile coding methods. The UIF profiles listed below have been derived from TIFF-FX [4]. The reader is referred to this document for a complete description of each profile, as the subsections below briefly summarize each profile and list only the differences between the UIF version of the profile and TIFF-FX profile on which it is based.

A printer that supports UIF must support at least UIF Profile S.

## UIF Profile Relationships

The following tree diagram, which is adapted from TIFF-FX[4] shows the relationship among UIF profiles and between UIF profiles and coding methods.

S (MH)

/ \

B&W    /   \   Color

------------     ----------

/      \                    \

/        F (MH, MR, MMR)      C (JPEG)

/                             / \

J (JBIG)                   ----   \

/        \

L (JBIG)   \

\

M (MRC)

All implementations of UIF MUST implement UIF Profile S, which is the root node of the tree. All color implementations of UIF MUST implement UIF Profile C. The implementation of a particular profile MUST also implement those profiles on the path that connect it to the root node, and MAY optionally implement profiles not on the path connecting it to the root node. For example, an implementation of UIF Profile M must also implement UIF Profiles C and S, and may optionally implement UIF Profile F, J or L. For another example, an implementation of UIF Profile C must also implement UIF Profile S, and may optionally implement UIF Profile F or J.

## UIF Profile S

UIF Profile S is modeled after Profile S of TIFF-FX[4], which describes the minimal black-and-white subset of TIFF for facsimile. UIF Profile S uses 1-dimensional Modified Huffman (MH) compression as defined in ITU-T T.4 [11] and MUST adopt the same requirements and restrictions for Baseline Fields, Extension Fields, byte order, bit order, and image file directory (IFD) placement as stated in Section 3 of TIFF-FX[4] with the exception of the following:

- ImageWidth is not constrained

- XResolution is not constrained, but 200, 300, and 600dpi MUST be supported

- YResolution is not constrained, but 200, 300, and 600dpi MUST be supported

Note that ‘XResolution’ and ‘YResolution’ values refer to the resolutions that the Receiver is capable of processing, not necessarily the resolutions that the Receiver is physically capable of producing (e.g., printer engine delivery).

The following Baseline and Extension Fields and field values MUST be supported by all UIF implementations. For a complete description of the Baseline and Extension Fields shown below, see the TIFF-FX specification [4].

Table 1.  UIF Profile S Baseline Fields

| Baseline Fields | Values |
| --- | --- |
| BitsPerSample | 1 |
| Compression | 3: 1D Modified Huffman coding set T4Options = 0 or 4 |
| FillOrder | 2: least significant bit first |
| ImageWidth | m: width of image in pixels |
| ImageLength | n: length of image in pixels (total number of scanlines) |
| NewSubFileType | 2: Bit 1 identifies single page of a multi-page Document |
| PhotometricInterpretation | 0: pixel value 1 means black |
| ResolutionUnit | 2: inch |
| RowsPerStrip | number of scanlines per strip = ImageLength, with one strip |
| SamplesPerPixel | 1 |
| StripByteCounts | number of bytes in TIFF strip |
| StripOffsets | offset from beginning of file to single TIFF strip |
| XResolution | 200, 300, 600, other resolutions are optional (written in pixels per inch) |
| YResolution | 200, 300, 600, other resolutions are optional (written in pixels per inch) |

Table 2.  UIF Profile S Extension Fields

| Extension Fields | Values |
| --- | --- |
| PageNumber | n,m: page number n followed by total page count m |
| T4Options | 0: MH coding, EOLs not byte aligned 4: MH coding, EOLs byte aligned |

## UIF Profile F

This section defines UIF Profile F, which uses Modified Read and Modified Modified Read (MMR) compression (described in ITU-T T.4 [11] and ITU-T T.6 [12]) in addition to the Modified Huffman compression used for UIF Profile S. UIF Profile F is based on TIFF-FX Profile F. The table that follows summarizes fields and field values that are REQUIRED / RECOMMENDED for UIF Profile F. For a complete description of the Baseline, Extension, and New Fields shown below, see the TIFF-FX specification [4]. A Device implementing this profile is REQUIRED to also implement UIF Profile S.

Here are the differences between TIFF-FX Profile F and UIF Profile F. For UIF Profile F,

- ImageWidth is not constrained

- XResolution is not constrained, but 200, 300, and 600dpi MUST be supported

- YResolution is not constrained, but 200, 300, and 600dpi MUST be supported

- MMR coding (Compression=4) MUST be supported, while MH (Compression=3) is optional

- The following TIFF-FX RECOMMENDED fields have been ommitted: ‘BadFaxLines’, ‘CleanFaxData’, ‘ConsecutiveBadFaxLines’, ‘ProfileType’, and ‘FaxProfile’

Recommended fields are shown with an asterisk *.

REQUIRED fields or values are shown with a double asterisk **. If the  double asterisk is on the field name, then all the listed values are REQUIRED of implementations; if the double asterisks are in the Values column, then only the values suffixed with a double asterisk  are REQUIRED of implementations.

Optional fields have no asterisks in either the field name or the Values column, however, the Values field may contain a condition which REQUIRES the field.

Table 3.  UIF Profile F  Baseline Fields

| Baseline Fields | Values |
| --- | --- |
| BitsPerSample | 1** |
| Compression | 3: 1D Modified Huffman and 2D Modified Read coding 4**: 2D Modified Modified Read coding |
| DateTime* | {ASCII}: date/time in 24-hour format "YYYY:MM:DD HH:MM:SS" |
| FillOrder** | 1: most significant bit first 2: least significant bit first |
| ImageDescription* | {ASCII}: A string describing the contents of the image |
| ImageWidth** | n: width of image in pixels |
| ImageLength** | n: length of image in pixels (total number of scanlines) |
| NewSubFileType | 2**: Bit 1 identifies single page of a multi-page Document |
| Orientation | 1**-8, Default is 1 |
| PhotometricInterpretation** | 0: pixel value 1 means black 1: pixel value 1 means white |
| ResolutionUnit** | 2: inch 3: centimeter |
| RowsPerStrip** | n: number of scanlines per TIFF strip |
| SamplesPerPixel | 1** |
| Software* | {ASCII}: name & release number of creator software |
| StripByteCounts** | n: number of bytes in TIFF strip |
| StripOffsets** | n: offset from beginning of file to each TIFF strip |
| XResolution | 200**, 300**, 600**, other resolutions are optional (written in pixels per inch) |
| YResolution | 200**, 300**, 600**, other resolutions are optional (written in pixels per inch) |

Table 4.  UIF Profile F  Extension Fields

| Extension Fields | Values |
| --- | --- |
| T4Options | 0**: REQUIRED if Compression is Modified Huffman, EOLs are not byte aligned 1: REQUIRED if Compression is 2D Modified Read, EOLs are not byte aligned 4**: REQUIRED if Compression is Modified Huffman, EOLs are byte aligned 5: REQUIRED if Compression is 2D Modified Read, EOLs are byte aligned |
| T6Options | 0: REQUIRED if Compression is 2D Modified Modified Read |
| DocumentName* | {ASCII}: name of UIF Document |
| PageNumber** | n,m: page number followed by total page count |

Table 5.  UIF Profile F  New Fields

| New Fields | Values |
| --- | --- |
| GlobalParametersIFD* | IFD: global parameters IFD |
| CodingMethods* | n: compression algorithms used in file |

## UIF Profile J

This section defines Profile J for UIF, which uses lossless JBIG compression as it is defined in ITU-T T.82 [16] subject to the application rules given in ITU-T T.85 [17]. UIF Profile J is based on TIFF-FX Profile J. The following table summarizes fields and field values that are REQUIRED / RECOMMENDED. For a complete description of the Baseline, Extension, and New Fields shown below, see the TIFF-FX specification [4]. A Device implementing this profile is REQUIRED to also implement UIF Profile S.

Here are the differences between TIFF-FX Profile J as defined in [4] and UIF Profile J. For UIF Profile J,

- ImageWidth is not constrained

- XResolution is not constrained, but 200, 300, and 600dpi MUST be supported

- YResolution is not constrained, but 200, 300, and 600dpi MUST be supported

- The following TIFF-FX RECOMMENDED fields have been ommitted: ‘ProfileType’ and ‘FaxProfile’

Recommended fields are shown with an asterisk *.

REQUIRED fields or values are shown with a double asterisk **. If the  double asterisk is on the field name, then all the listed values are REQUIRED of implementations; if the double asterisks are in the Values column, then the attribute and only the values suffixed with a double asterisk are REQUIRED of implementations.

Optional fields have no asterisks in either the field name or the Values column, however, the Values field may contain a condition which REQUIRES the field.

Table 6.  UIF Profile J  Baseline Fields

| Baseline Fields | Values |
| --- | --- |
| BitsPerSample | 1** |
| Compression | 9**: JBIG coding |
| DateTime* | {ASCII}: date/time in 24-hour format "YYYY:MM:DD HH:MM:SS" |
| FillOrder** | 1: most significant bit first 2: least significant bit first |
| ImageDescription* | {ASCII}: A string describing the contents of the image |
| ImageWidth** | n: width of image in pixels |
| ImageLength** | n: length of image in pixels (total number of scanlines) |
| NewSubFileType** | 2: Bit 1 identifies single page of a multi-page Document |
| Orientation | 1**-8, Default is 1 |
| PhotometricInterpretation** | 0: pixel value 1 means black 1: pixel value 1 means white |
| ResolutionUnit** | 2: inch 3: centimeter |
| RowsPerStrip** | n: number of scanlines per TIFF strip |
| SamplesPerPixel** | 1 |
| Software* | {ASCII}: name & release number of creator software |
| StripByteCounts** | n: number of bytes in TIFF strip |
| StripOffsets** | n: offset from beginning of file to each TIFF strip |
| XResolution | 200**, 300**, 600**, other resolutions are optional (written in pixels per inch) |
| YResolution | 200**, 300**, 600**, other resolutions are optional (written in pixels per inch) |

Table 7.  UIF Profile J  Extension Fields

| Extension Fields | Values |
| --- | --- |
| DocumentName* | {ASCII}: name of UIF Document |
| PageNumber** | n,m: page number followed by total page count |

Table 8.  UIF Profile J  New Fields

| New Fields | Values |
| --- | --- |
| GlobalParametersIFD* | IFD: global parameters IFD |
| T82Options** | 0: T.85 profile of T.82 |
| CodingMethods* | n: compression algorithms used in file |

## UIF Profile C

This section defines Profile C for UIF, which uses lossy JPEG compression as it is defined in ITU-T T.81 [15]. UIF Profile C is based on TIFF-FX Profile C. The following table summarizes fields and field values that are REQUIRED / RECOMMENDED. For a complete description of the Baseline, Extension, and New Fields shown below, see the TIFF-FX specification [4]. A Device that implements this profile is REQUIRED to also implement UIF Profile S.

Here are the differences between TIFF-FX Profile C as defined in [4] and UIF Profile C. For UIF Profile C,

- ImageWidth is not constrained

- XResolution is not constrained, but 200 and 300dpi MUST be supported

- YResolution is not constrained, but 200 and 300dpi MUST be supported

- The following TIFF-FX RECOMMENDED fields have been ommitted: ‘ProfileType’ and ‘FaxProfile’

Recommended fields are shown with an asterisk *.

REQUIRED fields or values are shown with a double asterisk **. If the  double asterisk is on the field name, then all the listed values are REQUIRED of implementations; if the double asterisks are in the Values column, then only the values suffixed with a double asterisk  are REQUIRED of implementations.

Optional fields have no asterisks in either the field name or the Values column, however, the Values field may contain a condition which REQUIRES the field.

Table 9.  UIF Profile C  Baseline Fields

| Baseline Fields | Values |
| --- | --- |
| BitsPerSample | 8**: 8 bits per color sample 12: optional 12 bits/sample |
| Compression** | 7: JPEG |
| DateTime* | {ASCII}: date/time in 24-hour format "YYYY:MM:DD HH:MM:SS" |
| FillOrder** | 1: most significant bit first 2: least significant bit first |
| ImageDescription* | {ASCII}: A string describing the contents of the image |
| ImageWidth** | n: width of image in pixels |
| ImageLength** | n: length of image in pixels (total number of scanlines) |
| NewSubFileType** | 2: Bit 1 identifies single page of a multi-page Document |
| Orientation | 1**-8, Default is 1 |
| PhotometricInterpretation | 10**: ITULAB |
| ResolutionUnit** | 2: inch 3: centimeter |
| RowsPerStrip** | n: number of scanlines per TIFF strip |
| SamplesPerPixel** | 1**: L* (lightness) 3: LAB |
| Software* | {ASCII}: name & release number of creator software |
| StripByteCounts** | n: number of bytes in TIFF strip |
| StripOffsets** | n: offset from beginning of file to each TIFF strip |
| XResolution | 200**, 300** other resolutions are optional (written in pixels per inch) |
| YResolution | 200**, 300** other resolutions are optional (written in pixels per inch) |

Table 10.  UIF Profile C  Extension Fields

| Extension Fields | Values |
| --- | --- |
| DocumentName* | {ASCII}: name of UIF Document |
| PageNumber** | n,m: page number followed by total page count |
| ChromaSubSampling | (1,1), (2, 2)** (1, 1): equal numbers of lightness and chroma samples horizontally and vertically (2, 2): twice as many lightness samples as chroma samples horizontally and vertically |
| ChromaPositioning | 1**: centered |

Table 11.  UIF Profile C  New Fields

| New Fields | Values |
| --- | --- |
| Decode** | minL, maxL, mina, maxa, minb, maxb: minimum and maximum values for L*a*b* |
| GlobalParametersIFD* | IFD: global parameters IFD |
| CodingMethods* | n: compression algorithms used in file |
| VersionYear* | byte sequence: year of ITU std |

## UIF Profile L

This profile is modeled after TIFF-FX Profile L. It uses JBIG compression (see [16]), subject to the application rules specified in ITU-T Recommendation T.43 [13] to losslessly code three types of color and grayscale images: one bit per color CMY, CMYK and RGB images; a palettized (i.e. mapped) color image; and continuous tone color and grayscale images.

Here are the differences between TIFF-FX Profile L as defined in [4] and UIF Profile L. For UIF Profile L,

- ImageWidth is not constrained

- XResolution is not constrained, but 300dpi MUST be supported

- YResolution must match XResolution, but it is not otherwise constrained; 300dpi MUST be supported

- The following TIFF-FX RECOMMENDED fields have been ommitted: ‘ProfileType’ and ‘FaxProfile’

The table that follows summarizes fields and field values that are REQUIRED / RECOMMENDED for implementation of UIF Profile L. For a complete description of the Baseline, Extension, and New Fields shown below, see the TIFF-FX specification [4]. A Device that chooses to implement this profile is REQUIRED to also implement UIF Profile S, and UIF Profile C.

Recommended fields are shown with an asterisk *.

REQUIRED fields or values are shown with a double asterisk **. If the  double asterisk is on the field name, then all the listed values are REQUIRED of implementations; if the double asterisks are in the Values column, then only the values suffixed with a double asterisk  are REQUIRED of implementations.

Optional fields have no asterisks in either the field name or the Values column, however, the Values field may contain a condition which REQUIRES the field.

Table 12.  UIF Profile L  Baseline Fields

| Baseline Fields | Values |
| --- | --- |
| BitsPerSample | 1: Binary RGB, CMY(K) 8**: 8 bits per color sample 9-16: optional |
| Compression** | 10**: JBIG, per T.43 |
| DateTime* | {ASCII}: date/time in 24-hour format "YYYY:MM:DD HH:MM:SS" |
| FillOrder** | 1: most significant bit first 2: least significant bit first |
| ImageDescription* | {ASCII}: A string describing the contents of the image |
| ImageWidth** | n: width of image in pixels |
| ImageLength** | n: length of image in pixels (total number of scanlines) |
| NewSubFileType** | 2: Bit 1 identifies single page of a multi-page Document |
| Orientation | 1**-8, Default is 1 |
| PhotometricInterpretation | 2: RGB 5: CMYK 10**: ITULAB |
| ResolutionUnit** | 2: inch |
| RowsPerStrip** | n: number of scanlines per TIFF strip |
| SamplesPerPixel** | 1**: L* (lightness) 3: LAB, RGB, CMY 4: CMYK |
| Software* | {ASCII}: name & release number of creator software |
| StripByteCounts** | n: number of bytes in TIFF strip |
| StripOffsets** | n: offset from beginning of file to each TIFF strip |
| XResolution | 200**, 300** other resolutions are optional (written in pixels per inch) |
| YResolution | equal to XResolution (pixels MUST be square) |

Table 13.  UIF Profile L  Extension Fields

| Extension Fields | Values |
| --- | --- |
| DocumentName* | {ASCII}: name of UIF Document |
| PageNumber** | n,m: page number followed by total page count |
| Indexed | 0: not a palette-color image 1: palette-color image |

Table 14.  UIF Profile L  New Fields

| New Fields | Values |
| --- | --- |
| Decode** | minL, maxL, mina, maxa, minb, maxb: minimum and maximum values for L*a*b* |
| GlobalParametersIFD* | IFD: global parameters IFD |
| CodingMethods* | n: compression algorithms used in file |
| VersionYear* | byte sequence: year of ITU std |

## UIF Profile M

This profile is modeled after TIFF-FX Profile M, which uses Mixed Raster Content (MRC), defined in ITU-T Recommendation T.44 [14]. MRC enables different coding methods and resolutions within a single page. For a more detailed description of MRC and the Baseline, Extension, and New Fields shown below, see [4] and [14].

Here are the differences between TIFF-FX Profile M as defined in [4] and UIF Profile M. For UIF Profile M,

- ImageWidth is not constrained

- XResolution is not constrained, but 200 and 300dpi MUST be supported for the bi-level mask, foreground, and background layers.

- YResolution must match XResolution, but it is not otherwise constrained; 200 and 300 dpi MUST be supported for the bi-level mask, foreground, and background layers.

- The following TIFF-FX RECOMMENDED fields have been ommitted: ‘ProfileType’ and ‘FaxProfile’

The table that follows summarizes fields and field values that are REQUIRED / RECOMMENDED for implementation of UIF Profile M.. A device that chooses to implement this profile is REQUIRED to also implement UIF Profile S, and UIF Profile C.

Recommended fields are shown with an asterisk *.

REQUIRED fields or values are shown with a double asterisk **. If the  double asterisk is on the field name, then all the listed values are REQUIRED of implementations; if the double asterisks are in the Values column, then only the values suffixed with a double asterisk  are REQUIRED of implementations.

Optional fields have no asterisks in either the field name or the Values column, however, the Values field may contain a condition which REQUIRES the field.

Table 15.  UIF Profile M  Baseline Fields

| Baseline Fields | Values |
| --- | --- |
| BitsPerSample | 1**: binary mask, RGB, CMY(K) 2-8**: bits per color sample 9-16: optional 12 bits/sample |
| Compression** | 1: None (ImageBaseColor IFD only) 3**: Modified Huffman and Modified Read 4: Modified Modified Read 7**: JPEG 9: JBIG, per [16] 10: JBIG, per [13] |
| DateTime* | {ASCII}: date/time in 24-hour format "YYYY:MM:DD HH:MM:SS" |
| FillOrder** | 1: most significant bit first 2: least significant bit first |
| ImageDescription* | {ASCII}: A string describing the contents of the image |
| ImageWidth** | n: width of image in pixels |
| ImageLength** | n: length of image in pixels (total number of scanlines) |
| NewSubFileType** | 16, 18: ¦ Bit 1 indicates single page of a multi-page Document on Primary IFD Bit 4 indicates MRC model |
| Orientation | 1**-8, Default is 1 |
| PhotometricInterpretation | 0**: WhiteIsZero (Mask Layer) 2: RGB 5: CMYK 10**: ITULAB |
| ResolutionUnit** | 2: inch |
| RowsPerStrip | n: number of scanlines per TIFF strip |
| SamplesPerPixel** | 1**: L* (lightness) 3: LAB, RGB, CMY 4: CMYK |
| Software* | {ASCII}: name & release number of creator software |
| StripByteCounts** | n: number of bytes in TIFF strip |
| StripOffsets** | n: offset from beginning of file to each TIFF strip |
| XResolution | 300**: background & foreground layers; 600**: binary mask layer; other resolutions are optional |
| YResolution | 300**: background & foreground layers; 600**: binary mask layer; other resolutions are optional; must be equal to XResolution (pixels MUST be square) |

Table 16.  UIF Profile M Extension Fields

| Extension Fields | Values |
| --- | --- |
| T4Options | 0**: REQUIRED if Compression is Modified Huffman, EOLs not byte aligned 1: REQUIRED if Compression 2D Modified Read, EOLs are not byte aligned 4**: REQUIRED if Compression Modified Huffman, EOLs byte aligned 5: REQUIRED if Compression 2D Modified Read, EOLs are byte aligned |
| T6Options | 0: REQUIRED if Compression is 2D Modified Modified Read |
| DocumentName* | {ASCII}: name of scanned Document |
| PageNumber** | n,m: page number followed by total page count |
| ChromaSubSampling | (1,1), (2, 2)** (1, 1): equal numbers of lightness and chroma samples horizontally & vertically¦ (2, 2): twice as many lightness samples as chroma horizontally and vertically |
| ChromaPositioning | 1: centered |
| Indexed | 0: not a palette-color image 1: palette-color image |
| SubIFDs | <IFD>: byte offset to FG/BG IFDs |
| XPosition | horizontal offset in primary IFD resolution units |
| YPosition | vertical offset in primary IFD resolution units |

Table 17.  UIF Profile M New Fields

| New Fields | Values |
| --- | --- |
| Decode** | minL, maxL, mina, maxa, minb, maxb: minimum and maximum values for L*a*b* |
| ImageBaseColor | a,b,c: background color in ITULAB |
| StripRowCounts | n: number of scanlines in each strip |
| ImageLayer | n, m: layer number, imaging sequence (e.g., strip number) |
| T82Options | 0: T.85 profile of T.82 coding |
| GlobalParametersIFD* | IFD: global parameters IFD |
| CodingMethods* | n: compression algorithms used in file |
| ModeNumber* | n: version of T.44 standard |
| VersionYear* | byte sequence: year of ITU std |

# Capabilities communication

A Sender needs to discover what a potential UIF-compatible Receiver supports in terms of resolution, encoding, drawing surface etc. To do this, a UIF Sender MUST use a protocol-specific means of communication to determine a Receiver capabilities string using CONNEG [5], the UIF profiles supported, the media that is supported, and the media that is not only supported but ready. The latter three parameters are excluded from the Receiver capabilities string so that a full Sender-side implementation of CONNEG is unnecessary if a UIF Sender decides to support only the minimum capabilities for a given profile (see section Section 5.1.2).

## Receiver capabilities string

A valid Receiver capabilities string MUST be any well-formed CONNEG string obeying the syntax of RFC2879 [5]. A UIF Sender MAY request the Receiver capabilities string. A UIF Receiver MUST return a Receiver capabilities string if a Sender requests it.

This string is not expected to be more than 32Kb in length. The capabilities announced by the Receiver SHOULD indicate those things that it can do without operator intervention. For example if the Receiver is a Device that has a manually interchangeable print cartridge with only the black cartridge loaded, it SHOULD only indicate support for “color=binary”.  The method of transport is protocol-dependent and beyond the scope of this document.

### New CONNEG Tags

Section 3.7 of CONNEG[5] describes the feature tag names that have to do with image coding.  The “image-file-structure” CONNEG tag describes how the coded image data is wrapped and formatted. In addition to the legal values for the “image-file-structure” tag presented in CONNEG[5], UIF formatted data MAY also use “tiff-limited-uif”. The “tiff-limited-uif” tag MUST be interpreted as “tiff-limited”, except the requirement for one TIFF strip per page is relaxed.

### Minimum Receiver capabilities

Requiring a minimum set of Receiver capabilities on a profile-specific basis is useful because it guarantees a baseline level of compatability between a Sender and a Receiver.

The CONNEG expressions listed in the following subsections summarize the minimum set of capabilities that a Receiver MUST support before advertising support for a given profile. See the CONNEG specification [5] for a complete description of the feature tags tokens.

#### Minimum capabilities for UIF Profile S

(& (image-file-structure=TIFF-minimal)

(image-coding=MH)

(color=Binary)

(dpi=[200,300,600])

(dpi-xyratio=1)

(MRC-mode=0) )

#### Minimum capabilities for UIF Profile F

(| (& (image-file-structure=TIFF-minimal)

(image-coding=MH)

(color=Binary)

(dpi=[200,300,600])

(dpi-xyratio=1)

(MRC-mode=0) )

(& (image-file-structure=TIFF-limited-uif)

(image-coding=MMR)

(color=Binary)

(dpi=[200,300,600])

(dpi-xyratio=1)

(MRC-mode=0) ) )

#### Minimum capabilities for UIF Profile J

(| (& (image-file-structure=TIFF-minimal)

(image-coding=MH)

(color=Binary)

(dpi=[200,300,600])

(dpi-xyratio=1)

(MRC-mode=0) )

(& (image-file-structure=TIFF-limited-uif)

(image-coding=JBIG)

(image-coding-constraint=JBIG-T85)

(color=Binary)

(JBIG-stripe-size=128)

(dpi=[200,300,600])

(dpi-xyratio=1)

(MRC-mode=0) ) )

#### Minimum capabilities for UIF Profile C

(| (& (image-file-structure=TIFF-minimal)

(image-coding=MH)

(color=Binary)

(dpi=[200,300,600])

(dpi-xyratio=1)

(MRC-mode=0) )

(& (image-file-structure=TIFF-limited-uif)

(color=full)

(image-coding=JPEG)

(image-coding-constraint=JPEG-T4E)

(color-subsampling=“4:1:1")

(color-levels<=16777216)

(color-space=CIELAB)

(color-illuminant=D50)

(CIELAB-L-min>=0)

(CIELAB-L-max<=100)

(CIELAB-a-min>=-85)

(CIELAB-a-max<=85)

(CIELAB-b-min>=-75)

(CIELAB-b-max<=125)

(dpi=[200,300])

(dpi-xyratio=1)

(MRC-mode=0) )

#### Minimum capabilities for UIF Profile L

(| (& (image-file-structure=TIFF-minimal)

(color=Binary)

(image-coding=MH)

(dpi=[200,300,600])

(dpi-xyratio=1)

(MRC-mode=0) )

(& (image-file-structure=TIFF-limited-uif)

(& (color=grey)

(| (& (image-coding=JPEG)

(image-coding-constraint=JPEG-T4E) )

(& (image-coding=JBIG)

(image-coding-constraint=JBIG-T43)

(JBIG-stripe-size=128)

(image-interleave=stripe) ) )

(color-space=CIELAB)

(color-levels<=256)

(color-illuminant=D50)

(CIELAB-L-min>=0)

(CIELAB-L-max<=100)

(dpi=[200,300]) (dpi-xyratio=1) )

(MRC-mode=0) ) )

#### Minimum capabilities for UIF Profile M

(| (& (image-file-structure=TIFF-minimal)

(color=Binary)

(image-coding=MH)

(MRC-mode=0)

(dpi=[200,300,600])

(dpi-xyratio=1) )

(& (image-file-structure=TIFF-limited-uif)

(color=full)

(image-coding=JPEG)

(image-coding-constraint=JPEG-T4E)

(color-subsampling="4:1:1")

(color-levels<=16777216)

(color-space=CIELAB)

(color-illuminant=D50)

(CIELAB-L-min>=0)

(CIELAB-L-max<=100)

(CIELAB-a-min>=-85)

(CIELAB-a-max<=85)

(CIELAB-b-min>=-75)

(CIELAB-b-max<=125)

(dpi=[200,300])(dpi-xyratio=1)

(MRC-mode=0)

(& (image-file-structure=TIFF-MRC-limited)

(MRC-mode=1)

(MRC-max-stripe-size<=256) ) )

## UIF profiles supported

A UIF Sender MUST query the potential UIF Receiver for the UIF profiles supported by the Receiver. A UIF Receiver MUST respond with the UIF profiles that it supports. Possible values MUST be among the UIF profiles described in this document, namely, UIF Profile S, F, J, C, L, and M.  The Sender MUST interpret a missing or otherwise invalid response as an indication that the Receiver does not support UIF. The method of transport and the actual data values used to indicate supported UIF profiles are protocol-specific and beyond the scope of this document.

## Media supported

A UIF Sender MUST query the potential UIF Receiver for media supported. A UIF Receiver MUST respond with the media supported by the Receiver (e.g., letter, legal, A4, etc.). The method of transport, the valid range of media, and the actual data values used to indicate supported media are protocol-specific and beyond the scope of this document; however, the Sender MUST be able to infer actual dimensions from the media values used.

## Media ready

A UIF Sender MUST query the potential UIF Receiver for media ready. A UIF Receiver MUST respond with the subset of media supported that is ready to print with no user intervention. The method of transport, the valid range of media, and the actual data values used to indicate ready media are protocol-specific and beyond the scope of this document; however, the Sender MUST be able to infer actual dimensions from the media values used.

## Image reduction supported

A UIF Sender MAY query the potential UIF Receiver to determine whether or not image reduction is supported. A Receiver MUST be capable of indicating whether or not it supports image reduction. The method by which this query occurs is protocol-specific and beyond the scope of this document.

# Sender requirements

## Image-Reduction

It is possible that a Sender might send an image that does not match the announced drawing surface of the Receiver (for example a Sender  may have an image that it cannot change). In this case the Sender MAY indicate to the Receiver in a protocol-specific manner whether or not the RECEIVER is to reduce the image.

If the Receiver does not support image reduction (see section 5.5) and the received image dimensions are larger than what is allowed by the supported media, then the Receiver MUST flow extra data to the next page.   If the Receiver does support image reduction, then the Sender MAY request in a protocol-specific manner that the Receiver use image-reduction if necessary. If the Receiver receives such a request, and the received image dimensions are larger than what is allowed by the supported media, then the Receiver MUST reduce the image so as to fit it to the page while maintaining the aspect ratio. If the Receiver uses image reduction,  the  Receiver must determine if reduction is necessary for each page and if so, apply reduction.  The scaling is calculated separately for each page.The scaling applies to all pages of the job unless the protocol used by the Sender and Receiver  supports a means of specifying image reduction on a page-by-page basis (e.g., IPPFAX’s potential use of page level overrides[6]).

## Intra-job media selection

When the image dimensions are different on a page-by-page basis such that use of a single type of media is not possible without scaling, the Sender / Receiver protocol MUST arbitrate media selection. The ImageWidth and ImageLength TIFF tags MUST NOT select the media.

# Conformance Requirements

For the listed operations, Table 18 below shows conformance requirements that apply to the protocol used to transport UIF data.

Table 18.   Underlying Protocol Conformance.

| Operation | UIF-capable Sender | UIF-capable Receiver | Section |
| --- | --- | --- | --- |
| Receiver capabilities string | MAY | MUST | 5.1.1 |
| UIF profiles supported | MUST | MUST | 5.1.2 |
| Media supported | MUST | MUST | 5.1.3 |
| Media ready | MUST | MUST | 5.1.4 |
| Image reduction supported | MAY | MUST | 5.1.5 |

# References

[1] 	deBry, Hastings, Herriot, Isaacson, Powell, "Internet Printing Protocol/1.1: Model and Semantics", RFC 2911

[2] 	Herriot, Butler , Moore, Turner, Wenn. "Internet Printing Protocol/1.1: Encoding and Transport", RFC 2910

[3] 	Hastings, Manros,  Kugler, Holst,  "Internet Printing Protocol/1.1: Implementer's Guide", draft-ietf-ipp-implementers-guide-v11-00.txt

[4] 	McIntyre, Zilles, Buckley, Venable, Parsons, Rafferty "File Format for Internet Fax", RFC2301, March 1998.

[5] 	Klyne, McIntyre. "Content Feature Schema for Internet Fax (V2)", RFC2879.

[6] 	PWG Standard 5100.4-2001  “Internet Printing Protocol (IPP): Override Attributes for Documents and Pages”. ftp://ftp.pwg.org/pub/pwg/standards/pwg5100.4.pdf

[7]	Moore, P., “Universal Image Format requirements”, October 16, 2000, ftp://ftp.pwg.org//pub/pwg/QUALDOCS/requirements/ifx-transport-requirements-01.pdf

[8]	Moore, P., “IPP Fax transport requirements”, October 16, 2000, ftp://ftp.pwg.org//pub/pwg/QUALDOCS/requirements/ifx-transport-requirements-01.pdf

[9] 	Masinter , "Terminology and Goals for Internet Fax", RFC2542

[10] 	Moore, Songer, Hastings, “IPP Fax Protocol”, May 24, 2001

[11]	ITU-T Recommendation T.4, Standardization of group 3 facsimile apparatus for document transmission, October 1997

[12]	ITU-T Recommendation T.6, Facsimile coding schemes and coding control functions for group 4 facsimile apparatus, November 1988

[13]	ITU-T Recommendation T.43, Colour and gray-scale image representations using lossless coding scheme for facsimile, February 1997

[14]	ITU-T Recommendation T.44, Mixed Raster Content (MRC), April 1999.

[15] 	ITU-T Recommendation T.81, Information technology - Digital compression and coding of continuous-tone still images - Requirements and guidelines, September 1992

[16]	ITU-T Recommendation T.82, Information technology - Coded representation of picture and audio information - Progressive bi-level image compression, March 1995

[17] 	ITU-T Recommendation T.85, Application profile for Recommendation T.82 - Progressive bi-level image compression (JBIG coding scheme) for facsimile apparatus, August 1995

[18]	Bradner, S., "Key words for use in RFCs to Indicate Requirement Levels", BCP 14, RFC 2119, March 1997.

[19]	Tag Image File Format, Revision 6.0, Adobe Developers Association, June 3, 1992, ftp://ftp.adobe.com/pub/adobe/devrelations/ devtechnotes/pdffiles/tiff6.pdf

The TIFF 6.0 specification dated June 3, 1992 specification (c) 1986-1988, 1992 Adobe Systems Incorporated. All Rights Reserved.

[20]	Adobe PageMaker 6.0 TIFF Technical Notes, Sept. 14, 1995, http://www.adobe.com/supportservice/devrelations/PDFS/TN/TIFFPM6.pdf

[21]	Draft TIFF Technical Note 2, Replacement TIFF/JPEG specification, March 17, 1995, ftp://ftp.uu.net/graphics/jpeg/

[22]	Parsons, G., Rafferty J. and S. Zilles, "Tag Image File Format (TIFF) - image/tiff MIME Sub-type Registration", work in progress, draft-ietf-fax-tiff-regbis-01.txt, May 2000.

Note: [22] is being progressed as BCP and is expected to be issued prior to the issuing of TIFF-FX as a Draft Standard.Issues

# Issues

## Outstanding Issues

- Should the MIME types “image/tiff; application=uif” and “image/tiff; application=uifcolor” be changed to the following?

image/tiff; application=uif-s

image/tiff; application=uif-f

image/tiff; application=uif-j

image/tiff; application=uif-c

image/tiff; application=uif-l

image/tiff; application=uif-m

## Resolved Issues

- Add description of new CONNEG tag used to indicate capabilities that are available *with* user intervention? We’re going to use media ready.

- What should be done concerning media selection when the TIFF image sizes are different on a page by page basis? Either determine media size by media size attribute or let the Receiver determine for itself the media to be used on each page

At the May 30 telecon, We agreed that for now, the TIFF “ImageWidth” and “ImageLength” tags do NOT select the media, but that the IPPFAX “media” Job Template attribute does.  This decision works fine for documents where the image size is the same for all pages in the document.  For documents that have differing image sizes within the same document, we’ll wait for a future requirement/extension to see whether to add another Job Template attribute so that the Sender can request that the TIFF image tags be used to select media (or not).  We also agreed NOT to bring in the IPP “page-overrides” attribute to allow the protocol to select media on a page by page basis (though an IPP Printer implementation might support such a thing). Incorporate this information into the IPPFAX spec.

- Should the IPP attribute descriptions be moved to the IFX spec so that UIF can be made independent of the IPPFAX protocol in case other protocols would like to use it?   Yes. Definitions of IPP attributes have been removed from the UIF spec, and requirements have been restated in a non protocol-specific manner.

Now the IPPFAX document will include two levels of conformance: ‘uif-only’ and ‘authenticated’.  The level being used needs to be  reflected in a Printer Description attribute. Make the appropriate changes to the IFX document.

- Change “uif-scale” attribute name to “uif-reduce”?   Yes. The IFX spec should be changed to reflect this. The UIF spec has been changed using more generic terminology to reflect this.

- Rename “uif-conneg” IPP attribute to “uif-receiver-capabilities”?  Yes. The IFX spec should be changed to reflect this. The UIF spec has been changed using more generic terminology to reflect this.

- Should additional resolutions be made mandatory?  Yes. X & Y Resolution values of 200 & 300 (in addition to 600dpi) are now also REQUIRED for UIF Profiles S, F, and J. X & Y Resolution values of 200dpi (in addition to 300 dpi) are now REQUIRED for UIF Profiles C and L.  For UIF Profile M,  REQUIRED binary, foreground, and background X &Y resolutions have been changed to include only 200 and 300 dpi.

- Should we change the minimum required compression for Profile F from MH to MMR? Yes, this has been done to reflect industry practice. The minimum CONNEG expressions have been changed to reflect this.

- Should we change the minimum required color space for Profile C from grayscale to color? Yes, this has been done to reflect industry practice. The minimum CONNEG expressions have been changed to reflect this.

- The term “default conneg” is a different meaning for “default”, than used in IPP.  In IPP, “default” means what the Printer does if the Sender doesn’t supply some attribute.  The “default conneg” is what the implementation MUST support for a given profile if the implementer doesn’t choose do to more.

Resolution: the spec has been changed so “Minimum” is used instead of “default”.

# Actions

- Teleconference scheduled on June 27, 2001 for 10:00am – 12:00pm (Pacific Time) to review updated spec.

Next meeting:  Toronto. Wednesday, August 1, 2001.

# Revision History (to be removed when standard is approved)

| Revision | Date | Author | Notes |
| --- | --- | --- | --- |
| 1 | 1/16/01 | Paul Moore, Netreon | Initial version |
| 2 | 1/28/01 | Gail Songer, Netreon | Added formal definition of new attributes |
| 3 | 4/11/01 | John Pulera, Minolta | Added UIF-specific Profile U and described UIF support for other TIFF-FX profiles |
| 4 | 5/07/01 | John Pulera, Minolta | Modifications made at Portland meeting. |
| 5 | 6/14/01 | John Pulera, Minolta | Added description of UIF profiles and minimal capabilities strings; generalized document so there is no dependence on IPP. |