A Project of the PWG IPPFAX Working Group

Universal Image Format (UIF)

IEEE-ISTO Printer Working Group

Draft Standard 5102.2-D0.7

October 16, 2001

ftp://ftp.pwg.org/pub/pwg/QUALDOCS/uif-spec-07.pdf, .doc

Abstract

This standard specifies an extension to TIFF-FX known as Universal Image Format (UIF) by formally defining a series of TIFF-FX “profiles” distinguished primarily by the method of compression employed and color space used.  The UIF requirements [7] are derived from the requirements for IPPFAX [8] and Internet Fax [9].

In summary UIF is a raster image data format intended for use by, but not limited to, the IPPFAX protocol, which is used to provide a synchronous, reliable exchange of image Documents between Senders and Receivers. UIF is based on the TIFF-FX specification [4], which describes the TIFF (Tag Image File Format) representation of image data specified by the ITU-T Recommendations for black-and-white and color facsimile.

This document is a draft of an IEEE-ISTO PWG Proposed Standard and is in full conformance with all provisions of the PWG Process (see: ftp//ftp.pwg.org/pub/pwg/general/pwg-process.pdf).  PWG Proposed Standards are working documents of the IEEE-ISTO PWG and its working groups.  The list of current PWG projects and drafts can be obtained at http://www.pwg.org.

When approved as a PWG standard, this document will be available from:

ftp://ftp.pwg.org/pub/pwg/standards/pwg510x.y.pdf, .doc, .rtf

Copyright (C) 2001, IEEE Industry Standards and Technology Organization. All rights reserved.

This document may be copied and furnished to others, and derivative works that comment on, or otherwise explain it or assist in its implementation may be prepared, copied, published and distributed, in whole or in part, without restriction of any kind, provided that the above copyright notice, this paragraph and the title of the Document as referenced below are included on all such copies and derivative works.  However, this document itself may not be modified in any way, such as by removing the copyright notice or references to the IEEE-ISTO and the Printer Working Group, a program of the IEEE-ISTO.

Title:  Universal Image Format

The IEEE-ISTO and the Printer Working Group DISCLAIM ANY AND ALL WARRANTIES, WHETHER EXPRESS OR IMPLIED INCLUDING (WITHOUT LIMITATION) ANY IMPLIED WARRANTIES OF MERCHANTABILITY OR FITNESS FOR A PARTICULAR PURPOSE.

The Printer Working Group, a program of the IEEE-ISTO, reserves the right to make changes to the document without further notice.  The document may be updated, replaced or made obsolete by other documents at any time.

The IEEE-ISTO takes no position regarding the validity or scope of any intellectual property or other rights that might be claimed to pertain to the implementation or use of the technology described in this document or the extent to which any license under such rights might or might not be available; neither does it represent that it has made any effort to identify any such rights.

The IEEE-ISTO invites any interested party to bring to its attention any copyrights, patents, or patent applications, or other proprietary rights which may cover technology that may be required to implement the contents of this document. The IEEE-ISTO and its programs shall not be responsible for identifying patents for which a license may be required by a document and/or IEEE-ISTO Industry Group Standard or for conducting inquiries into the legal validity or scope of those patents that are brought to its attention. Inquiries may be submitted to the IEEE-ISTO by e-mail at:

ieee-isto@ieee.org.

The Printer Working Group acknowledges that the IEEE-ISTO (acting itself or through its designees) is, and shall at all times, be the sole entity that may authorize the use of certification marks, trademarks, or other special designations to indicate compliance with these materials.

Use of this document is wholly voluntary.  The existence of this document does not imply that there are no other ways to produce, test, measure, purchase, market, or provide other goods and services related to its scope.

Table of Contents

1	Introduction	6

2	Terminology	6

2.1	Conformance Terminology	6

2.2	Model	6

3	TIFF-FX support	7

3.1	New TIFF-FX Extensions	7

3.1.1	TIFF-FX Extension 20: Relaxed Image Widths and Resolutions	7

3.1.2	TIFF-FX Extensions 21 – Required Resolution	7

3.1.3	TIFF-FX Extensions 22 – Required Resolution	7

3.1.4	TIFF-FX Extensions 23 – Required Resolution	7

3.1.5	TIFF-FX Extensions 24 – Required Resolution	7

3.2	Relationships among UIF Profiles	8

3.3	Summary of UIF Profiles	8

3.3.1	UIF Profile S	9

3.3.2	UIF Profile F	10

3.3.3	UIF Profile J	12

3.3.4	UIF Profile C	14

3.3.5	UIF Profile L	16

3.3.6	UIF Profile M	18

3.4	Potential UIF Profiles	21

4	Sender requirements	22

4.1	Indicating Document format using MIME	22

4.2	Image-Reduction	22

4.3	Intra-Document media selection	22

5	References	23

6	Outstanding Issues	24

7	Revision History (to be removed when standard is approved)	24

Appendix A.   Capabilities communication (Informative)	26

A.1	Receiver capabilities string	26

A.1.1 Minimum Receiver capabilities	26

A.1.1.1  Minimum capabilities for UIF Profile S	26

A.1.1.2  Minimum capabilities for UIF Profile F	27

A.1.1.3  Minimum capabilities for UIF Profile J	27

A.1.1.4  Minimum capabilities for UIF Profile C	27

A.1.1.4.1 Minimum grayscale capabilities for UIF Profile C	27

A1.1.4.2  Minimum full color capabilities for UIF Profile C	28

A.1.1.5  Minimum capabilities for UIF Profile L	28

A.1.1.5.1 Minimum grayscale capabilities for UIF Profile L	29

A.1.1.5.2 Minimum full color capabilities for UIF Profile L	29

A.1.1.6  Minimum capabilities for UIF Profile M	30

A.1.2   New CONNEG tags and values	31

A.1.2.1   Definition of ‘profile’ tag and tag values	31

A.1.2.2   Application of ‘profile’ tag and tag values	33

A.2   UIF Profiles supported	34

A.3   Media supported	34

A.4   Media ready	34

A.5   Image reduction supported	34

A.6   Conformance Requirements Summary	34

Table of Tables

Table 1.  UIF Profile S Baseline Fields	9

Table 2.  UIF Profile S Extension Fields	9

Table 3.  UIF Profile S  New Fields	10

Table 4.  UIF Profile F  Baseline Fields	10

Table 5.  UIF Profile F  Extension Fields	11

Table 6.  UIF Profile F  New Fields	11

Table 7.  UIF Profile J  Baseline Fields	12

Table 8.  UIF Profile J  Extension Fields	13

Table 9.  UIF Profile J  New Fields	13

Table 10.  UIF Profile C  Baseline Fields	14

Table 11.  UIF Profile C  Extension Fields	15

Table 12.  UIF Profile C  New Fields	15

Table 13.  UIF Profile L  Baseline Fields	17

Table 14.  UIF Profile L  Extension Fields	17

Table 15.  UIF Profile L  New Fields	18

Table 16.  UIF Profile M  Baseline Fields	19

Table 17.  UIF Profile M Extension Fields	20

Table 18.  UIF Profile M New Fields	21

Table 19.   Underlying Protocol Conformance.	34

# Introduction

This document specifies a set of extensions to the TIFF-FX profiles defined in TIFF-FX [4] that are especially suited for use with synchronous protocols (e.g., IPPFAX[10]). The increased conformance requirements found in this UIF specification reflect the need for a data format where quality document transmission is the primary concern. When the profiles described in TIFF-FX [4] are used with the extensions described in this document, the data format is known as Universal Image Format (UIF).

# Terminology

This section defines the following additional terms that are used throughout this standard.

## Conformance Terminology

The key words MUST, MUST NOT, REQUIRED, SHOULD, SHOULD NOT, RECOMMENDED, MAY, and OPTIONAL in this document are to be interpreted as described in RFC 2119 [18].

## Model

The following terms are introduced and capitalized in order to indicate their specific meaning:

Baseline Field – One of the core set of TIFF fields introduced by the TIFF specification [19]

Implementation  – A Sender or Receiver

Document – The UIF-formatted electronic representation of a set of one or more pages that the Sender sends to the Receiver.

Extension Field – One of the TIFF extension fields introduced by the current TIFF specification [19], specification, the set of PageMaker TIFF Technical Notes [20], or TIFF Technical Note 2 [21].

New Field  – One of the new TIFF fields introduced by the TIFF-FX specification [4]. Note that the UIF specification does not introduce any new TIFF tags.

Receiver – This is the agent (software, hardware or some combination) that receives the Document sent by the Sender.

Sender – This is the agent (software, hardware or some combination) that is used to create and transmit a Document to a Receiver.

UIF Profile – A TIFF-FX profile used with a specific combination of the TIFF-FX extensions that are described in section 3.1.

# TIFF-FX support

A UIF Document is a TIFF file that adheres to the requirements of (1) Baseline TIFF (see [19]) and (2) one or more UIF Profiles. A UIF Profile is based on a collection of ITU-T facsimile coding methods. The UIF Profiles listed below have been derived from TIFF-FX [4]. The reader is referred to this document and the TIFF-FX Extensions Set 1 document [24] for a complete description of each profile, as the subsections below briefly summarize each UIF Profile and list only the additional TIFF-FX extensions that MUST be used.

Pages within a UIF Document MAY be encoded using different UIF Profiles.

An Implementation that supports UIF MUST support at least UIF Profile S. Note that for the TIFF fields “ImageDescription”, “DocumentName”, “Software”, and “DateTime”, Adobe Baseline TIFF specifies only ASCII and does not provide a language tag or alternate character set facility.

## New TIFF-FX Extensions

Five new TIFF-FX extensions are introduced as described in the following subsections.

### TIFF-FX Extension 20: Relaxed Image Widths and Resolutions

The allowances shown below supersede the TIFF-FX requirements specified in [4] concerning the ImageWidth, XResolution, and YResolution TIFF fields:

- The ImageWidth, XResolution, and YResolution TIFF fields are not constrained.

### TIFF-FX Extensions 21 – Required Resolution

The requirement shown below supersedes the TIFF-FX requirements in [4] concerning the XResolution, YResolution, and ResolutionUnit TIFF fields:

- Receivers MUST support XResolution=YResolution=200 and ResolutionUnit=2 (inches)

### TIFF-FX Extensions 22 – Required Resolution

The requirement shown below supersedes the TIFF-FX requirements in [4] concerning the XResolution, YResolution, and ResolutionUnit TIFF fields:

- Receivers MUST support XResolution=YResolution=300 and ResolutionUnit=2 (inches)

### TIFF-FX Extensions 23 – Required Resolution

The requirement shown below supersedes the TIFF-FX requirements in [4] concerning the XResolution, YResolution, and ResolutionUnit TIFF fields:

- Receivers MUST support XResolution=YResolution=400 and ResolutionUnit=2 (inches)

### TIFF-FX Extensions 24 – Required Resolution

The requirement shown below supersedes the TIFF-FX requirements in [4] concerning the XResolution, YResolution, and ResolutionUnit TIFF fields:

- Receivers MUST support XResolution=YResolution=600 and ResolutionUnit=2 (inches)

## Relationships among UIF Profiles

The following tree diagram, which is adapted from TIFF-FX[4], shows the relationship among UIF Profiles and between UIF Profiles and coding methods.

S (MH)

/ \

B&W    /   \   Color/Grayscale

------------     ----------

/      \                    \

/        F (MH, MR, MMR)      C (JPEG)

/                             / \

J (JBIG)                   ----   \

/        \

L (JBIG)   \

\

M (MRC)

All UIF Senders and/or Receivers MUST implement UIF Profile S, which is the root node of the tree. All color Senders and/or Receivers of UIF MUST implement UIF Profile C. Senders and/or Receivers that implement a particular profile MUST also implement those profiles on the path that connect it to the root node, and MAY optionally implement profiles not on the path connecting it to the root node. For example, a Sender and/or Receiver that implements UIF Profile M MUST also implement UIF Profiles C and S, and MAY optionally implement UIF Profile F, J or L. For another example, a Sender/Receiver that implements UIF Profile C MUST also implement UIF Profile S, and MAY optionally implement UIF Profile F or J.

## Summary of UIF Profiles

The following subsections summarize Implementation requirements and list the TIFF-FX extensions that MUST be supported for each of the UIF Profiles. Each subsection contains one or more tables that show the TIFF fields and field values that are REQUIRED, RECOMMENDED, or OPTIONAL for UIF Implementations. For profiles other than UIF Profile S, single asterisks (*) and double asterisks (**) indicate the level of Receiver conformance (see the legend below each table). For profiles other than UIF Profile S, the rightmost column is used to indicate Sender conformance, i.e., those fields that a user MUST, SHOULD, or MAY include in the Image File Directory (IFD) of a UIF Document.  For fields that a Receiver MUST support, note that a Sender MUST support at least one of the REQUIRED field values that the Receiver MUST support.

If there is a default value associated with a TIFF field, and the default value is a legal value for the given UIF Profile, then the Sender MAY choose to physically omit this field from the UIF file, as the presence of the TIFF field and its value are implied. The tables in the following subsections show default values for TIFF fields only when the default values are permitted.

### UIF Profile S

- When TIFF-FX Extensions 20, 21, 22, and 24 are applied to Profile S in TIFF-FX[4], the result is UIF Profile S. UIF Profile S is modeled after Profile S of TIFF-FX[4], which describes the minimal black-and-white subset of TIFF for facsimile. Tables 1, 2, and 3 summarize the fields and field values that are REQUIRED for all Implementations of UIF Profile S. A UIF Profile S Implementation MUST use 1-dimensional Modified Huffman (MH) compression as defined in ITU-T T.4 [11] and MUST adopt the same requirements and restrictions for Baseline Fields, Extension Fields, byte order, bit order, and image file directory (IFD) placement as stated in Section 3 of TIFF-FX[4] except where overridden by TIFF-FX Extensions 20,21,22, and 24.

Note that ‘XResolution’ and ‘YResolution’ values refer to the resolutions that the Receiver is capable of processing, not necessarily the resolutions that the Receiver is physically capable of producing (e.g., printer engine delivery).

All UIF Receivers MUST support the following Baseline, Extension, and New Fields and accompanying field values. All UIF Senders MUST be capable of creating a UIF Document that contains the following Baseline, Extension, and New Fields or MUST be otherwise capable of verifying that these fields are present before sending a Document. For a complete description of the Baseline and Extension Fields shown below, see the TIFF-FX specification [4] and TIFF-FX Extension Set 1[24].

Table 1.  UIF Profile S Baseline Fields

| Baseline Fields | Values |
| --- | --- |
| BitsPerSample | 1 |
| Compression | 3: 1D Modified Huffman coding set T4Options = 0 or 4 |
| FillOrder | 2: least significant bit first |
| ImageWidth | m: width of image in pixels |
| ImageLength | n: length of image in pixels (total number of scanlines) |
| NewSubFileType | 2: Bit 1 identifies single page of a multi-page Document |
| PhotometricInterpretation | 0: pixel value 1 means black |
| ResolutionUnit | 2: inch (Default = 2) |
| RowsPerStrip | number of scanlines per strip = ImageLength, with one strip |
| SamplesPerPixel | 1 |
| StripByteCounts | number of bytes in TIFF strip |
| StripOffsets | offset from beginning of file to single TIFF strip |
| XResolution | 200, 300, 600, other resolutions are OPTIONAL (written in pixels per inch) |
| YResolution | 200, 300, 600, other resolutions are OPTIONAL (written in pixels per inch) |

Table 2.  UIF Profile S Extension Fields

| Extension Fields | Values |
| --- | --- |
| PageNumber | n,m: page number n followed by total page count m |
| T4Options | 0: MH coding, EOLs not byte aligned (Default = 0) 4: MH coding, EOLs byte aligned |

Table 3.  UIF Profile S  New Fields

| New Fields | Values |
| --- | --- |
| GlobalParametersIFD | IFD: global parameters IFD |
| TIFF-FXExtensions | 0x1700000 (Bits indicating use of TIFF-FX Extensions 20,21,22 and 24) |

### UIF Profile F

This section defines UIF Profile F, which uses Modified Read and Modified Modified Read (MMR) compression (described in ITU-T T.4 [11] and ITU-T T.6 [12]) in addition to the Modified Huffman compression used for UIF Profile S. When TIFF-FX Extensions 20, 21, 22, and 24 are applied to Profile F in TIFF-FX[4], the result is UIF Profile F. UIF Profile F is based on TIFF-FX Profile F. Tables 4, 5, and 6 summarize the fields and field values that are REQUIRED / RECOMMENDED / OPTIONAL for UIF Profile F. Asterisks are used to denote levels of Receiver conformance, while the rightmost column indicates Sender conformance, i.e., those fields that a Sender MUST, SHOULD, or MAY include in an image file directory (IFD) of a UIF Document. For a complete description of the Baseline, Extension, and New Fields shown below, see the TIFF-FX specification [4] and TIFF-FX Extension Set 1[24]. A Sender/Receiver implementing this profile is REQUIRED to also implement UIF Profile S.

Table 4.  UIF Profile F  Baseline Fields

| Baseline Fields | Values | Sender Conformance |
| --- | --- | --- |
| BitsPerSample | 1** | MUST |
| Compression | 3: 1D Modified Huffman and 2D Modified Read coding 4**: 2D Modified Modified Read coding | MUST |
| DateTime* | {ASCII}: date/time in 24-hour format "YYYY:MM:DD HH:MM:SS" | SHOULD |
| FillOrder** | 1: most significant bit first 2: least significant bit first (Default = 2) | MUST |
| ImageDescription* | {ASCII}: A string describing the contents of the image | SHOULD |
| ImageWidth** | n: width of image in pixels | MUST |
| ImageLength** | n: length of image in pixels (total number of scanlines) | MUST |
| NewSubFileType | 2**: Bit 1 identifies single page of a multi-page Document | MUST |
| Orientation | 1**-8, (Default = 1) | MUST |
| PhotometricInterpretation** | 0: pixel value 1 means black 1: pixel value 1 means white | MUST |
| ResolutionUnit** | 2: inch (Default = 2) 3: centimeter | MUST |
| RowsPerStrip** | n: number of scanlines per TIFF strip | MUST |
| SamplesPerPixel | 1** | MUST |
| Software* | {ASCII}: name & release number of creator software | SHOULD |
| StripByteCounts** | n: number of bytes in TIFF strip | MUST |
| StripOffsets** | n: offset from beginning of file to each TIFF strip | MUST |
| XResolution | 200**, 300**, 600**, other resolutions are OPTIONAL (written in pixels per inch) | MUST |
| YResolution | 200**, 300**, 600** in pixels per inch with x-y aspect ratio (XResolution / YResolution) equal to 1; other resolutions and aspect ratios are OPTIONAL (written in pixels per inch) | MUST |

*  Receiver SHOULD support this field.

** (If double asterisk is in ‘Baseline Fields’ column) Receiver MUST support the given field and all values shown in ‘Values’ column.

(If double asterisk is in ‘Values’ column) Receiver MUST support the given field and the value immediately preceding the double asterisk.

Table 5.  UIF Profile F  Extension Fields

| Extension Fields | Values | Sender Conformance |
| --- | --- | --- |
| T4Options | 0: REQUIRED if Compression is Modified Huffman (MH), EOLs are not byte aligned (Default = 0) 1: REQUIRED if Compression is 2D Modified Read (MR), EOLs are not byte aligned 4: REQUIRED if Compression is Modified Huffman, EOLs are byte aligned 5: REQUIRED if Compression is 2D Modified Read, EOLs are byte aligned | MUST if Compression=3 |
| T6Options | 0**: REQUIRED if Compression is 2D Modified Modified Read (MMR) (Default = 0) | MUST if Compression=4 |
| DocumentName* | {ASCII}: name of UIF Document | SHOULD |
| PageNumber** | n,m: page number followed by total page count | MUST |

*  Receiver SHOULD support this field.

** (If double asterisk is in ‘Extension Fields’ column) Receiver MUST support the given field and all values shown in ‘Values’ column.

(If double asterisk is in ‘Values’ column) Receiver MUST support the given field and the value immediately preceding the double asterisk.

Table 6.  UIF Profile F  New Fields

| New Fields | Values | Sender Conformance |
| --- | --- | --- |
| GlobalParametersIFD** | IFD: global parameters IFD | MUST |
| TIFF-FXExtensions | 0x1700000** (Bits indicating use of TIFF-FX Extensions 20,21,22 and 24) | MUST |
| FaxProfile* | n: ITU-compatible FAX profile | SHOULD |
| MultiProfiles* | n: profiles or profile(s) plus extension(s) applied within this file | SHOULD |
| CodingMethods* | n: compression algorithms used in file | SHOULD |

*  Receiver SHOULD support this field.

** (If double asterisk is in ‘New Fields’ column) Receiver MUST support the given field and all values shown in ‘Values’ column.

(If double asterisk is in ‘Values’ column) Receiver MUST support the given field and the value immediately preceding the double asterisk.

### UIF Profile J

This section defines Profile J for UIF, which uses lossless JBIG compression as it is defined in ITU-T T.82 [16] subject to the application rules given in ITU-T T.85 [17]. When TIFF-FX Extensions 20, 21, 22, and 24 are applied to Profile J in TIFF-FX[4], the result is UIF Profile J. Tables 7, 8, and 9 summarize fields and field values that are REQUIRED / RECOMMENDED / OPTIONAL. Asterisks are used to denote levels of Receiver conformance, while the rightmost column indicates levels of Sender Conformance, i.e., those fields that a Sender MUST, SHOULD, or MAY include in an IFD of a UIF document. For a complete description of the Baseline, Extension, and New Fields shown below, see the TIFF-FX specification [4] and TIFF-FX Extension Set 1[24]. A Sender/Receiver implementing this profile is REQUIRED to also implement UIF Profile S.

Here are the differences between TIFF-FX Profile J as defined in [4] and UIF Profile J. For UIF Profile J,

.

Table 7.  UIF Profile J  Baseline Fields

| Baseline Fields | Values | Sender Conformance |
| --- | --- | --- |
| BitsPerSample | 1** | MUST |
| Compression | 9**: JBIG coding | MUST |
| DateTime* | {ASCII}: date/time in 24-hour format "YYYY:MM:DD HH:MM:SS" | SHOULD |
| FillOrder** | 1: most significant bit first 2: least significant bit first | MUST |
| ImageDescription* | {ASCII}: A string describing the contents of the image | SHOULD |
| ImageWidth** | n: width of image in pixels | MUST |
| ImageLength** | n: length of image in pixels (total number of scanlines) | MUST |
| NewSubFileType** | 2: Bit 1 identifies single page of a multi-page Document | MUST |
| Orientation | 1**-8, (Default = 1) | MUST |
| PhotometricInterpretation** | 0: pixel value 1 means black 1: pixel value 1 means white | MUST |
| ResolutionUnit** | 2: inch (Default = 2) 3: centimeter | MUST |
| RowsPerStrip** | n: number of scanlines per TIFF strip | MUST |
| SamplesPerPixel** | 1 | MUST |
| Software* | {ASCII}: name & release number of creator software | SHOULD |
| StripByteCounts** | n: number of bytes in TIFF strip | MUST |
| StripOffsets** | n: offset from beginning of file to each TIFF strip | MUST |
| XResolution | 200**, 300**, 600**, other resolutions are OPTIONAL (written in pixels per inch) | MUST |
| YResolution | 200**, 300**, 600** in pixels per inch with x-y aspect ratio (XResolution / YResolution) equal to 1; other resolutions and aspect ratios are OPTIONAL | MUST |

*   Receiver SHOULD support this field.

** (If double asterisk is in ‘Baseline Fields’ column) Receiver MUST support the given field and all values shown in ‘Values’ column.

(If double asterisk is in ‘Values’ column) Receiver MUST support the given field and the value immediately preceding the double asterisk.

Table 8.  UIF Profile J  Extension Fields

| Extension Fields | Values | Sender Conformance |
| --- | --- | --- |
| DocumentName* | {ASCII}: name of UIF Document | SHOULD |
| PageNumber** | n,m: page number followed by total page count | MUST |

*   Receiver SHOULD support this field.

** Receiver MUST support the given field and all values shown in ‘Values’ column.

Table 9.  UIF Profile J  New Fields

| New Fields | Values | Sender Conformance |
| --- | --- | --- |
| GlobalParametersIFD** | IFD: global parameters IFD | MUST |
| TIFF-FXExtensions | 0x1700000** (Bits indicating use of TIFF-FX Extensions 20,21,22 and 24) | MUST |
| FaxProfile* | n: ITU-compatible FAX profile | SHOULD |
| MultiProfiles* | n: profiles or profile(s) plus extension(s) applied within this file | SHOULD |
| T82Options** | 0: T.85 profile of T.82 | MUST |
| CodingMethods* | n: compression algorithms used in file | SHOULD |

*   Receiver SHOULD support this field.

** Receiver MUST support the given field and all values shown in ‘Values’ column.

### UIF Profile C

This section defines Profile C for UIF, which uses lossy JPEG compression as it is defined in ITU-T T.81 [15]. When TIFF-FX Extensions 20, 21, and 22 are applied to Profile C in TIFF-FX[4], the result is UIF Profile C. UIF Profile C is based on TIFF-FX Profile C. Tables 10, 11, and 12 summarize fields and field values that are REQUIRED / RECOMMENDED / OPTIONAL. Asterisks are used to denote levels of Receiver conformance, while the rightmost column indicates levels of Sender Conformance, i.e., those fields that a Sender MUST, SHOULD, or MAY include in an IFD of a UIF document. For a complete description of the Baseline, Extension, and New Fields shown below, see the TIFF-FX specification [4] and TIFF-FX Extension Set 1[24]. A Sender/Receiver that implements this profile is REQUIRED to also implement UIF Profile S.

Table 10.  UIF Profile C  Baseline Fields

| Baseline Fields | Values | Sender Conformance |
| --- | --- | --- |
| BitsPerSample | 8**: 8 bits per color sample 12: OPTIONAL 12 bits/sample | MUST |
| Compression** | 7: JPEG | MUST |
| DateTime* | {ASCII}: date/time in 24-hour format "YYYY:MM:DD HH:MM:SS" | SHOULD |
| FillOrder** | 1: most significant bit first 2: least significant bit first | MUST |
| ImageDescription* | {ASCII}: A string describing the contents of the image | SHOULD |
| ImageWidth** | n: width of image in pixels | MUST |
| ImageLength** | n: length of image in pixels (total number of scanlines) | MUST |
| NewSubFileType** | 2: Bit 1 identifies single page of a multi-page Document | MUST |
| Orientation | 1**-8, (Default = 1) | MUST |
| PhotometricInterpretation | 10**: ITULAB | MUST |
| ResolutionUnit** | 2: inch (Default = 2) 3: centimeter | MUST |
| RowsPerStrip** | n: number of scanlines per TIFF strip | MUST |
| SamplesPerPixel** | 1**: L* (lightness) 3: LAB | MUST |
| Software* | {ASCII}: name & release number of creator software | SHOULD |
| StripByteCounts** | n: number of bytes in TIFF strip | MUST |
| StripOffsets** | n: offset from beginning of file to each TIFF strip | MUST |
| XResolution | 200**, 300** other resolutions are OPTIONAL (written in pixels per inch). XResolution and YResolution fields MUST be equal. | MUST |
| YResolution | equal to XResolution (pixels MUST be square) | MUST |

*   Receiver SHOULD support this field.

** (If double asterisk is in ‘Baseline Fields’ column) Receiver MUST support the given field and all values shown in ‘Values’ column.

(If double asterisk is in ‘Values’ column) Receiver MUST support the given field and the value immediately preceding the double asterisk.

Table 11.  UIF Profile C  Extension Fields

| Extension Fields | Values | Sender Conformance |
| --- | --- | --- |
| DocumentName* | {ASCII}: name of UIF Document | SHOULD |
| PageNumber** | n,m: page number followed by total page count | MUST |
| ChromaSubSampling | (1,1), (2, 2)** (1, 1): equal numbers of lightness and chroma samples horizontally and vertically (2, 2): twice as many lightness samples as chroma samples horizontally and vertically | MUST |
| ChromaPositioning | 1**: centered | MUST |
| JPEGTables* | n: file pointer to JPEG quantization and/or Huffman tables | MAY |

*   Receiver SHOULD support this field.

** (If double asterisk is in ‘Extension Fields’ column) Receiver MUST support the given field and all values shown in ‘Values’ column.

(If double asterisk is in ‘Values’ column) Receiver MUST support the given field and the value immediately preceding the double asterisk.

Table 12.  UIF Profile C  New Fields

| New Fields | Values | Sender Conformance |
| --- | --- | --- |
| Decode** | minL, maxL, mina, maxa, minb, maxb: minimum and maximum values for L*a*b* | MUST |
| GlobalParametersIFD** | IFD: global parameters IFD | MUST |
| TIFF-FXExtensions | 0x1700000** (Bits indicating use of TIFF-FX Extensions 20,21,22 and 24) | MUST |
| FaxProfile* | n: ITU-compatible FAX profile | SHOULD |
| MultiProfiles* | n: profiles or profile(s) plus extension(s) applied within this file | SHOULD |
| CodingMethods* | n: compression algorithms used in file | SHOULD |
| VersionYear* | byte sequence: year of ITU std | SHOULD |

*   Receiver SHOULD support this field.

** (If double asterisk is in ‘New Fields’ column) Receiver MUST support the given field and all values shown in ‘Values’ column.

(If double asterisk is in ‘Values’ column) Receiver MUST support the given field and the value immediately preceding the double asterisk.

### UIF Profile L

When TIFF-FX Extensions 20, 21, and 22 are applied to Profile L in TIFF-FX[4], the result is UIF Profile L. This profile uses JBIG compression (see [16]), subject to the application rules specified in ITU-T Recommendation T.43 [13] to losslessly code three types of color and grayscale images: one bit per color CMY, CMYK and RGB images; a palletized (i.e. mapped) color image; and continuous tone color and grayscale images.

Here are the differences between TIFF-FX Profile L as defined in [4] and UIF Profile L. For UIF Profile L,

- ImageWidth is not constrained.

- XResolution is not constrained, but a Receiver MUST support 200 and 300dpi.

- YResolution MUST match XResolution, but it is not otherwise constrained; a Receiver MUST support 200 and 300dpi.

- The following TIFF-FX RECOMMENDED field has been omitted: ‘ProfileType’.

- UIF Implementations MUST support the GlobalParametersIFD field.

- The ‘FaxProfile’ TIFF tag introduced in [4] is re-interpreted as the ‘UIFProfile’ TIFF tag for UIF Documents. The TIFF tag ‘UIFProfile’ uses the same TIFF field identifier (402) and the same data type (Byte) as the TIFF tag ‘FaxProfile’. The values for this field are redefined as follows:

0: does not conform to a profile defined for UIF

1: minimal black & white lossless, UIF Profile S

2: extended black & white lossless, UIF Profile F

3: lossless JBIG black & white, UIF Profile J

4: lossy color and grayscale, UIF Profile C

5: lossless color and grayscale, UIF Profile L

6: Mixed Raster Content, UIF Profile M

Tables 12, 13, and 14 summarize fields and field values that are REQUIRED / RECOMMENDED / OPTIONAL for Implementations of UIF Profile L. Asterisks are used to denote levels of Receiver conformance, while the rightmost column indicates levels of Sender Conformance, i.e., those fields that a Sender MUST, SHOULD, or MAY include in an IFD of a UIF document. For a complete description of the Baseline, Extension, and New Fields shown below, see the TIFF-FX specification [4] and TIFF-FX Extension Set 1[24]. A Sender / Receiver that chooses to implement this profile is REQUIRED to also implement UIF Profile S, and UIF Profile C.

Optional fields have no asterisks in either the field name or the Values column, however, the Values field may contain a condition which REQUIRES the field.

Table 13.  UIF Profile L  Baseline Fields

| Baseline Fields | Values | Sender Conformance |
| --- | --- | --- |
| BitsPerSample | 1: Binary RGB, CMY(K) 8**: 8 bits per color sample 9-16: OPTIONAL | MUST |
| Compression | 10**: JBIG, per T.43 | MUST |
| DateTime* | {ASCII}: date/time in 24-hour format "YYYY:MM:DD HH:MM:SS" | SHOULD |
| FillOrder** | 1: most significant bit first 2: least significant bit first | MUST |
| ImageDescription* | {ASCII}: A string describing the contents of the image | SHOULD |
| ImageWidth** | n: width of image in pixels | MUST |
| ImageLength** | n: length of image in pixels (total number of scanlines) | MUST |
| NewSubFileType | 2**: Bit 1 identifies single page of a multi-page Document | MUST |
| Orientation | 1**-8, (Default = 1) | MUST |
| PhotometricInterpretation | 2: RGB 5: CMYK 10**: ITULAB | MUST |
| ResolutionUnit** | 2: inch (Default = 2) | MUST |
| RowsPerStrip** | n: number of scanlines per TIFF strip | MUST |
| SamplesPerPixel | 1**: L* (lightness) 3: LAB, RGB, CMY 4: CMYK | MUST |
| Software* | {ASCII}: name & release number of creator software | SHOULD |
| StripByteCounts** | n: number of bytes in TIFF strip | MUST |
| StripOffsets** | n: offset from beginning of file to each TIFF strip | MUST |
| XResolution | 200**, 300** other resolutions are OPTIONAL (written in pixels per inch) | MUST |
| YResolution | equal to XResolution (pixels MUST be square) | MUST |

*   Receiver SHOULD support this field.

** (If double asterisk is in ‘Baseline Fields’ column) Receiver MUST support the given field and all values shown in ‘Values’ column.

(If double asterisk is in ‘Values’ column) Receiver MUST support the given field and the value immediately preceding the double asterisk.

Table 14.  UIF Profile L  Extension Fields

| Extension Fields | Values | Sender Conformance |
| --- | --- | --- |
| DocumentName* | {ASCII}: name of UIF Document | SHOULD |
| PageNumber** | n,m: page number followed by total page count | MUST |
| Indexed | 0: not a palette-color image (Default = 0) 1: palette-color image | MUST if image uses palette color; otherwise, MAY |

*   Receiver SHOULD support this field.

** Receiver MUST support the given field and all values shown in ‘Values’ column.

Note:  Fields that the Receiver MAY support have no asterisks in either the field name or the values column

Table 15.  UIF Profile L  New Fields

| New Fields | Values | Sender Conformance |
| --- | --- | --- |
| Decode** | minL, maxL, mina, maxa, minb, maxb: minimum and maximum values for L*a*b* | MUST if PhotoMetric-Interpretation is set to ITULAB |
| GlobalParametersIFD** | IFD: global parameters IFD | MUST |
| TIFF-FXExtensions | 0x1700000** (Bits indicating use of TIFF-FX Extensions 20,21,22 and 24) | MUST |
| FaxProfile* | n: ITU-compatible FAX profile | SHOULD |
| MultiProfiles* | n: profiles or profile(s) plus extension(s) applied within this file | SHOULD |
| CodingMethods* | n: compression algorithms used in file | SHOULD |
| VersionYear* | byte sequence: year of ITU std | SHOULD |

*   Receiver SHOULD support this field.

** Receiver MUST support the given field and all values shown in ‘Values’ column.

### UIF Profile M

When TIFF-FX Extensions 20, 21, 22, and 23 are applied to Profile M in TIFF-FX[4], the result is UIF Profile M. This profile is modeled after TIFF-FX Profile M, which uses Mixed Raster Content (MRC), defined in ITU-T Recommendation T.44 [14]. MRC enables different coding methods and resolutions within a single page. For a more detailed description of MRC and the Baseline, Extension, and New Fields shown below, see the TIFF-FX specification [4], ITU-T T.44 Mixed Raster Content [14], and TIFF-FX Extension Set 1 [24].

Tables 16, 17, and 18 summarize fields and field values that are REQUIRED / RECOMMENDED / OPTIONAL for Implementations of UIF Profile M.. Asterisks are used to denote levels of Receiver conformance, while the rightmost column indicates levels of Sender Conformance, i.e., those fields that a Sender MUST, SHOULD, or MAY include in an IFD of a UIF document. A Sender/Receiver that chooses to implement this profile is REQUIRED to also implement UIF Profile S, and UIF Profile C.

Optional fields have no asterisks in either the field name or the Values column, however, the Values field may contain a condition which REQUIRES the field.

Table 16.  UIF Profile M  Baseline Fields

| Baseline Fields | Values | Sender Conformance |
| --- | --- | --- |
| BitsPerSample | 1**: binary mask, RGB, CMY(K) 2-8**: bits per color sample 9-16: OPTIONAL 12 bits/sample | MUST |
| Compression | 1: None (ImageBaseColor IFD only) 3: Modified Huffman and Modified Read 4**: Modified Modified Read 7**: JPEG 9: JBIG, per [16] 10: JBIG, per [13] | MUST |
| DateTime* | {ASCII}: date/time in 24-hour format "YYYY:MM:DD HH:MM:SS" | SHOULD |
| FillOrder** | 1: most significant bit first 2: least significant bit first | MUST |
| ImageDescription* | {ASCII}: A string describing the contents of the image | SHOULD |
| ImageWidth** | n: width of image in pixels | MUST |
| ImageLength** | n: length of image in pixels (total number of scanlines) | MUST |
| NewSubFileType** | 16, 18: Bit 1 indicates single page of a multi-page Document on Primary IFD Bit 4 indicates MRC model | MUST |
| Orientation | 1**-8, (Default = 1) | MUST |
| PhotometricInterpretation | 0**: WhiteIsZero (Mask Layer) 2: RGB 5: CMYK 10**: ITULAB | MUST |
| ResolutionUnit** | 2: inch (Default = 2) | MUST |
| RowsPerStrip** | n: number of scanlines per TIFF strip | MUST |
| SamplesPerPixel | 1**: L* (lightness) 3: LAB, RGB, CMY 4: CMYK | MUST |
| Software* | {ASCII}: name & release number of creator software | SHOULD |
| StripByteCounts** | n: number of bytes in TIFF strip | MUST |
| StripOffsets** | n: offset from beginning of file to each TIFF strip | MUST |
| XResolution | 200**, 300**, 400**: binary mask, background & foreground layers; other resolutions are OPTIONAL | MUST |
| YResolution | 200**, 300**, 400**: binary mask, background & foreground layers; other resolutions are OPTIONAL; MUST be equal to XResolution (pixels MUST be square) | MUST |

*   Receiver SHOULD support this field.

** (If double asterisk is in ‘Baseline Fields’ column) Receiver MUST support the given field and all values shown in ‘Values’ column.

(If double asterisk is in ‘Values’ column) Receiver MUST support the given field and the value immediately preceding the double asterisk.

Table 17.  UIF Profile M Extension Fields

| Extension Fields | Values | Sender Conformance |
| --- | --- | --- |
| T4Options | 0: REQUIRED if Compression is Modified Huffman, EOLs not byte aligned (Default = 0) 1: REQUIRED if Compression 2D Modified Read, EOLs are not byte aligned 4: REQUIRED if Compression Modified Huffman, EOLs byte aligned 5: REQUIRED if Compression 2D Modified Read, EOLs are byte aligned | MUST if Compression=3 |
| T6Options | 0**: REQUIRED if Compression is 2D Modified Modified Read (Default = 0) | MUST if Compression=4 |
| DocumentName* | {ASCII}: name of scanned Document | SHOULD |
| PageNumber** | n,m: page number followed by total page count | MUST |
| ChromaSubSampling | (1,1), (2, 2)** (1, 1): equal numbers of lightness and chroma samples horizontally & vertically¦ (2, 2): twice as many lightness samples as chroma horizontally and vertically | MUST if Compression=7 and Photometric-Interpretation=10 |
| ChromaPositioning** | 1: centered (default = 1) | MAY if Compression=7 and Photometric-Interpretation=10 |
| Indexed | 0: not a palette-color image (Default = 0) 1: palette-color image | MUST if image uses palette color; otherwise, MAY |
| SubIFDs** | <IFD>: byte offset to FG/BG IFDs | MAY |
| XPosition** | horizontal offset in primary IFD resolution units | MAY |
| YPosition** | vertical offset in primary IFD resolution units | MAY |
| JPEGTables* | n: file pointer to JPEG quantization and/or Huffman tables | MAY |

*   Receiver SHOULD support this field.

** (If double asterisk is in ‘Extension Fields’ column) Receiver MUST support the given field and all values shown in ‘Values’ column.

(If double asterisk is in ‘Values’ column) Receiver MUST support the given field and the value immediately preceding the double asterisk.

Note:  Fields that the Receiver MAY support have no asterisks in either the field name or the values column

Table 18.  UIF Profile M New Fields

| New Fields | Values | Sender Conformance |
| --- | --- | --- |
| Decode** | minL, maxL, mina, maxa, minb, maxb: minimum and maximum values for L*a*b* | MUST if Photometric-Interpretation=10 |
| ImageBaseColor** | a,b,c: background color in ITULAB | MAY |
| StripRowCounts** | n: number of scanlines in each strip | MAY |
| ImageLayer** | n, m: layer number, imaging sequence (e.g., strip number) | MAY |
| T82Options | 0: T.85 profile of T.82 coding | MUST if Compression=9 |
| GlobalParametersIFD** | IFD: global parameters IFD | MUST |
| TIFF-FXExtensions | 0x1700000** (Bits indicating use of TIFF-FX Extensions 20,21,22 and 24) | MUST |
| FaxProfile* | n: ITU-compatible FAX profile | SHOULD |
| MultiProfiles* | n: profiles or profile(s) plus extension(s) applied within this file | SHOULD |
| CodingMethods* | n: compression algorithms used in file | SHOULD |
| ModeNumber* | n: version of T.44 standard | SHOULD |
| VersionYear* | byte sequence: year of ITU std | SHOULD |

*   Receiver SHOULD support this field.

** Receiver MUST support the given field and all values shown in ‘Values’ column.

## Potential UIF Profiles

While this specification was being written, a new profile, designated ‘T’, was being introduced as an extension to TIFF-FX. This new TIFF-FX profile would allow JBIG2 to be used for the lossless and lossy coding of black-and-white image data. JBIG2 coding can be used for UIF Documents as soon as the RFC for TIFF-FX Profile T  is published, and the IPPFAX Working Group publishes the additional requirements that are needed for UIF Profile T.

# Sender requirements

## Indicating Document format using MIME

If the underlying transport protocol uses MIME as defined by RFC2046 [23], then a Sender MUST describe the TIFF-FX data using one of two possible MIME content types, depending on which UIF Profiles are included in the Document. If the Document contains only UIF Profile S and/or UIF Profile F, then the UIF data content MUST be described by the ‘image/tiff’ content type/subtype. Registration of the MIME type/sub-type ‘image/tiff’ is described in the TIFF MIME Sub-type Registration document [25]*. If the Document contains any UIF Profiles besides UIF Profile S and/or UIF Profile F, then the Sender MUST describe the UIF data using the ‘image/tiffx” content type/subtype*. Registration of the ‘image/tiffx’ content type is described

* Note:  TIFF-FX [4] will be registering a new MIME media type to accommodate profiles/codings that are not compatible with TIFF 6. TIFF-FX profiles that are not compatible with TIFF 6, namely profiles J, C, L, and M, will use the new MIME type. For the purposes of this draft, the ‘image/tiffx’ MIME type is shown as a working name, since it has been suggested through email by the Internet FAX Working Group.  When the proper MIME type is agreed by the Internet FAX WG, this document will be updated.

## Image-Reduction

It is possible that a Sender might send an image that does not match the announced drawing surface of the Receiver (for example a Sender  may have an image that it cannot change). In this case the Sender MAY indicate to the Receiver in a protocol-specific manner whether or not the Receiver is to reduce the image.

If the Receiver does not support image reduction and the received image dimensions are larger than what is allowed by the supported media, then the Receiver MUST flow extra data to the next page.   If the Receiver does support image reduction, then the Sender MAY request in a protocol-specific manner that the Receiver use image-reduction if necessary. If the Receiver receives such a request, and the received image dimensions are larger than what is allowed by the supported media, then the Receiver MUST reduce the image so as to fit it to the page while maintaining the aspect ratio. If the Receiver uses image reduction,  the  Receiver MUST determine if reduction is necessary for each page and if so, apply reduction.  The scaling is calculated separately for each page. The scaling applies to all pages of the Document unless the protocol used by the Sender and Receiver  supports a means of specifying image reduction on a page-by-page basis (e.g., IPPFAX’s potential use of page level overrides[6]).

## Intra-Document media selection

When the image dimensions are different on a page-by-page basis such that use of a single type of media is not possible without scaling, the Sender / Receiver protocol MUST arbitrate media selection. The ImageWidth and ImageLength TIFF tags MUST NOT select the media.

# References

[1] 	deBry, Hastings, Herriot, Isaacson, Powell, "Internet Printing Protocol/1.1: Model and Semantics", RFC 2911, September 2000.

[2] 	Herriot, Butler , Moore, Turner, Wenn. "Internet Printing Protocol/1.1: Encoding and Transport", RFC 2910 , September 2000.

[3] 	Hastings, Manros,  Kugler, Holst,  "Internet Printing Protocol/1.1: Implementer's Guide", work in progress, draft-ietf-ipp-implementers-guide-v11-??.txt.

[4] 	McIntyre, Zilles, Buckley, Venable, Parsons, Rafferty "File Format for Internet Fax", RFC2301, March 1998.

[5] 	Klyne, McIntyre. "Content Feature Schema for Internet Fax (V2)", RFC2879, August 2000.

[6] 	PWG Standard 5100.4-2001  “Internet Printing Protocol (IPP): Override Attributes for Documents and Pages”. ftp://ftp.pwg.org/pub/pwg/standards/pwg5100.4.pdf, February 7, 2001.

[7]	Moore, P., “Universal Image Format requirements”, October 16, 2000, ftp://ftp.pwg.org//pub/pwg/QUALDOCS/requirements/ifx-transport-requirements-01.pdf

[8]	Moore, P., “IPP Fax transport requirements”, October 16, 2000, ftp://ftp.pwg.org//pub/pwg/QUALDOCS/requirements/ifx-transport-requirements-01.pdf

[9] 	Masinter , "Terminology and Goals for Internet Fax", RFC2542, March 1999.

[10] 	Moore, Songer, Hastings, “IPP Fax Protocol” PWG Draft Standard D0.5, June 21, 2001

[11]	ITU-T Recommendation T.4, Standardization of group 3 facsimile apparatus for document transmission, October 1997

[12]	ITU-T Recommendation T.6, Facsimile coding schemes and coding control functions for group 4 facsimile apparatus, November 1988

[13]	ITU-T Recommendation T.43, Colour and gray-scale image representations using lossless coding scheme for facsimile, February 1997

[14]	ITU-T Recommendation T.44, Mixed Raster Content (MRC), April 1999.

[15] 	ITU-T Recommendation T.81, Information technology - Digital compression and coding of continuous-tone still images - Requirements and guidelines, September 1992

[16]	ITU-T Recommendation T.82, Information technology - Coded representation of picture and audio information - Progressive bi-level image compression, March 1995

[17] 	ITU-T Recommendation T.85, Application profile for Recommendation T.82 - Progressive bi-level image compression (JBIG coding scheme) for facsimile apparatus, August 1995

[18]	Bradner, S., "Key words for use in RFCs to Indicate Requirement Levels", BCP 14, RFC 2119, March 1997.

[19]	Tag Image File Format, Revision 6.0, Adobe Developers Association, June 3, 1992, http://partners.adobe.com/asn/developer/pdfs/tn/TIFF6.pdf

The TIFF 6.0 specification dated June 3, 1992 specification (c) 1986-1988, 1992 Adobe Systems Incorporated. All Rights Reserved.

[20]	Adobe PageMaker 6.0 TIFF Technical Notes, Sept. 14, 1995,  http://partners.adobe.com/asn/developer/pdfs/tn/TIFFPM6.pdf

[21]	Draft TIFF Technical Note 2, Replacement TIFF/JPEG specification, March 17, 1995,  ftp://ftp.sgi.com/graphics/tiff/TTN2.draft.txt

[22]	Parsons, G., Rafferty J. and S. Zilles, "Tag Image File Format (TIFF) - image/tiff MIME Sub-type Registration", work in progress, draft-ietf-fax-tiff-regbis-??.txt.

Note: [22] is being progressed as BCP and is expected to be issued prior to the issuing of TIFF-FX as a Draft Standard.

[23]     Freed, N. and N. Borenstein, "Multipurpose Internet Mail Extensions (MIME) Part Two: Media Types", RFC 2046, November 1996.

[24] 	McIntyre, Abercrobie, Rucklidge, Buckley, “TIFF-FX Extension Set 1”, July 20, 2001.

[25]	Klyne, G., “A Syntax for Describing Media Feature Sets”, RFC 2533, March 1999.

# Outstanding Issues

- Is it still OK for a Sender to describe UIF Profile S or F TIFF data using the "image/tiff" MIME subtype since UIF Profile S relies on several TIFF-FX extensions which require the use of two TIFF fields not recognized by TIFF 6 (namely, the GlobalParametersIFD and TIFF-FXExtensions fields)

# Revision History (to be removed when standard is approved)

| Revision | Date | Author | Notes |
| --- | --- | --- | --- |
| 1 | 1/16/01 | Paul Moore, Netreon | Initial version |
| 2 | 1/28/01 | Gail Songer, Netreon | Added formal definition of new attributes |
| 3 | 4/11/01 | John Pulera, Minolta | Added UIF-specific Profile U and described UIF support for other TIFF-FX profiles |
| 4 | 5/07/01 | John Pulera, Minolta | Modifications made at Portland meeting. |
| 5 | 6/14/01 | John Pulera, Minolta | Added description of UIF profiles and minimal capabilities strings; generalized document so there is no dependence on IPP. |
| D0.6 | 7/25/01 | John Pulera, Minolta | Expanded Sender conformance requirements for UIF profiles and MIME; other modifications per June teleconference. |
| D0.7 | 10/16/01 | John Pulera, Minolta | Redefined UIF Profiles to be TIFF-FX profiles using TIFF-FX extensions; moved capabilities communication to an informative appendix. |

# Appendix A.   Capabilities communication (Informative)

This informative appendix is intended to suggest a means of capabilities communication that would allow a protocol using the UIF data format to discover what a potential UIF-compatible Receiver supports in terms of resolution, encoding, drawing surface, etc. As such, the conformance terminology used in this Appendix applies only to protocols that choose to implement capabilities communication as it is described in this Appendix. Section A.6 lists the Conformance requirements for protocols that implement capabilities communication as it is described in this appendix.

To discover a potential Receiver’s capabilities, a UIF Sender MUST query in a protocol-specific manner either the UIF Profiles supported (see section A.2) or the Receiver capabilities string (see section A.1). If the Sender wants to send a UIF file using any OPTIONAL features outside the profile-specific baseline level (see baseline levels shown in section A.1.1), then the Sender MUST query the Receiver for the capabilities string.  The Sender MUST also query the Receiver to determine the media that is supported, and the media that is not only supported but ready. The UIF Profiles supported, media supported, and media ready are excluded from the Receiver capabilities string so that a full Sender-side implementation of CONNEG is unnecessary if a UIF Sender decides to support only the minimum capabilities for a given profile (see Section 4.1.2).

## A.1	Receiver capabilities string

A valid Receiver capabilities string MUST be any well-formed CONNEG string obeying the syntax specified in RFC2533 [25] and using the feature tag and tag values described in  RFC2879 [5]. A UIF Sender MAY request the Receiver capabilities string. A UIF Receiver MUST return a Receiver capabilities string if a Sender requests it.The Receiver capabilities string is not expected to be more than 32Kb in length. The capabilities announced by the Receiver SHOULD indicate those things that it can do without operator intervention. For example if the Receiver has a manually interchangeable print cartridge with only the black cartridge loaded, it SHOULD only indicate support for “color=binary”.  The method of transport is protocol-dependent and beyond the scope of this document.

### A.1.1 Minimum Receiver capabilities

Requiring a minimum set of Receiver capabilities on a profile-specific basis is useful because it guarantees a baseline level of compatibility between a Sender and a Receiver.

The CONNEG expressions listed in the following subsections summarize the minimum set of capabilities that a Receiver MUST support before advertising support for a given profile. See RFC2879 [5] for a complete description of the feature tags tokens. The color profiles (UIF Profiles C and L) have been broken down further into minimum capabilities specification for both grayscale-only and full-color implementations.

#### A.1.1.1  Minimum capabilities for UIF Profile S

(& (image-file-structure=TIFF-minimal)

(MRC-mode=0)

(image-coding=MH)

(color=Binary)

(dpi=[200,300,600])

(dpi-xyratio=1) )

#### A.1.1.2  Minimum capabilities for UIF Profile F

(| (& (image-file-structure=TIFF-minimal)

(MRC-mode=0)

(image-coding=MH)

(color=Binary)

(dpi=[200,300,600])

(dpi-xyratio=1) )

(& (image-file-structure=TIFF-limited)

(MRC-mode=0)

(image-coding=MMR)

(color=Binary)

(dpi=[200,300,600])

(dpi-xyratio=1) ) )

#### A.1.1.3  Minimum capabilities for UIF Profile J

(| (& (image-file-structure=TIFF-minimal)

(MRC-mode=0)

(image-coding=MH)

(color=Binary)

(dpi=[200,300,600])

(dpi-xyratio=1) )

(& (image-file-structure=TIFF-limited)

(MRC-mode=0)

(image-coding=JBIG)

(image-coding-constraint=JBIG-T85)

(color=Binary)

(JBIG-stripe-size=128)

(dpi=[200,300,600])

(dpi-xyratio=1) ) )

#### A.1.1.4  Minimum capabilities for UIF Profile C

Minimum capabilities for UIF Profile C can be subdivided into a listing of minimum capabilities for a baseline grayscale implementation and a listing of minimum capabilities for a full color implementation. Subdividing the minimum capabilities in such a way gives the Sender the flexibility to encode grayscale and/or full color data without the need for a full CONNEG implementation.

##### A.1.1.4.1 Minimum grayscale capabilities for UIF Profile C

(| (& (image-file-structure=TIFF-minimal)

(MRC-mode=0)

(image-coding=MH)

(color=Binary)

(dpi=[200,300,600])

(dpi-xyratio=1) )

(& (image-file-structure=TIFF-limited)

(MRC-mode=0)

(color=grey)

(image-coding=JPEG)

(image-coding-constraint=JPEG-T4E)

(color-levels<=256)

(color-space=CIELAB)

(color-illuminant=D50)

(CIELAB-L-min>=0)

(CIELAB-L-max<=100)

(dpi=[200,300])

(dpi-xyratio=1) ) )

##### A1.1.4.2  Minimum full color capabilities for UIF Profile C

(| (& (image-file-structure=TIFF-minimal)

(MRC-mode=0)

(image-coding=MH)

(color=Binary)

(dpi=[200,300,600])

(dpi-xyratio=1) )

(& (image-file-structure=TIFF-limited)

(MRC-mode=0)

(color=grey)

(image-coding=JPEG)

(image-coding-constraint=JPEG-T4E)

(color-levels<=256)

(color-space=CIELAB)

(color-illuminant=D50)

(CIELAB-L-min>=0)

(CIELAB-L-max<=100)

(dpi=[200,300])

(dpi-xyratio=1) )

(& (image-file-structure=TIFF-limited)

(MRC-mode=0)

(color=full)

(image-coding=JPEG)

(image-coding-constraint=JPEG-T4E)

(color-subsampling=“4:1:1")

(color-levels<=16777216)

(color-space=CIELAB)

(color-illuminant=D50)

(CIELAB-L-min>=0)

(CIELAB-L-max<=100)

(CIELAB-a-min>=-85)

(CIELAB-a-max<=85)

(CIELAB-b-min>=-75)

(CIELAB-b-max<=125)

(dpi=[200,300])

(dpi-xyratio=1) ) )

#### A.1.1.5  Minimum capabilities for UIF Profile L

As with UIF Profile C, minimum capabilities for UIF Profile L can be subdivided into a listing of minimum capabilities for a baseline grayscale implementation and a listing of minimum capabilities for a full color implementation. Subdividing the minimum capabilities in such a way gives the Sender the flexibility to encode grayscale and/or full color data without the need for a full CONNEG implementation.

##### A.1.1.5.1 Minimum grayscale capabilities for UIF Profile L

(| (& (image-file-structure=TIFF-minimal)

(MRC-mode=0)

(color=Binary)

(image-coding=MH)

(dpi=[200,300,600])

(dpi-xyratio=1) )

(& (image-file-structure=TIFF-limited)

(MRC-mode=0)

(color=grey)

(| (& (image-coding=JPEG)

(image-coding-constraint=JPEG-T4E) )

(& (image-coding=JBIG)

(image-coding-constraint=JBIG-T43)

(JBIG-stripe-size=128)

(image-interleave=stripe) ) )

(color-space=CIELAB)

(color-levels<=256)

(color-illuminant=D50)

(CIELAB-L-min>=0)

(CIELAB-L-max<=100)

(dpi=[200,300])

(dpi-xyratio=1) ) )

##### A.1.1.5.2 Minimum full color capabilities for UIF Profile L

(| (& (image-file-structure=TIFF-minimal)

(MRC-mode=0)

(color=Binary)

(image-coding=MH)

(dpi=[200,300,600])

(dpi-xyratio=1) )

(& (image-file-structure=TIFF-limited)

(MRC-mode=0)

(color=grey)

(| (& (image-coding=JPEG)

(image-coding-constraint=JPEG-T4E) )

(& (image-coding=JBIG)

(image-coding-constraint=JBIG-T43)

(JBIG-stripe-size=128)

(image-interleave=stripe) ) )

(color-space=CIELAB)

(color-levels<=256)

(color-illuminant=D50)

(CIELAB-L-min>=0)

(CIELAB-L-max<=100)

(dpi=[200,300])

(dpi-xyratio=1) )

(& (image-file-structure=TIFF-limited)

(MRC-mode=0)

(color=full)

(| (& (image-coding=JPEG)

(image-coding-constraint=JPEG-T4E)

(color-subsampling=["1:1:1","4:1:1"]) )

(& (image-coding=JBIG)

(image-coding-constraint=JBIG-T43)

(JBIG-stripe-size=128)

(image-interleave=stripe) ) )

(color-levels<=16777216)

(color-space=CIELAB)

(color-illuminant=D50)

(CIELAB-L-min>=0)

(CIELAB-L-max<=100)

(CIELAB-a-min>=-85)

(CIELAB-a-max<=85)

(CIELAB-b-min>=-75)

(CIELAB-b-max<=125)

(dpi=[100,200,300])

(dpi-xyratio=1) ) )

#### A.1.1.6  Minimum capabilities for UIF Profile M

(| (& (image-file-structure=TIFF-minimal)

(MRC-mode=0)

(color=Binary)

(image-coding=MH)

(dpi=[200,300,600])

(dpi-xyratio=1) )

(& (image-file-structure=TIFF-limited)

(MRC-mode=0)

(color=full)

(image-coding=JPEG)

(image-coding-constraint=JPEG-T4E)

(color-subsampling="4:1:1")

(color-levels<=16777216)

(color-space=CIELAB)

(color-illuminant=D50)

(CIELAB-L-min>=0)

(CIELAB-L-max<=100)

(CIELAB-a-min>=-85)

(CIELAB-a-max<=85)

(CIELAB-b-min>=-75)

(CIELAB-b-max<=125)

(dpi=[200,300])(dpi-xyratio=1)

(& (image-file-structure=TIFF-MRC-limited)

(MRC-mode=1)

(MRC-max-stripe-size<=256)

(| (& (image-file-structure=TIFF-minimal)

(color=Binary)

(image-coding=MH)

(dpi=[200,300,400])

(dpi-xyratio=1) )

(& (image-file-structure=TIFF-limited)

(color=full)

(image-coding=JPEG)

(image-coding-constraint=JPEG-T4E)

(color-subsampling="4:1:1")

(color-levels<=16777216)

(color-space=CIELAB)

(color-illuminant=D50)

(CIELAB-L-min>=0)

(CIELAB-L-max<=100)

(CIELAB-a-min>=-85)

(CIELAB-a-max<=85)

(CIELAB-b-min>=-75)

(CIELAB-b-max<=125)

(dpi=[200,300,400])

(dpi-xyratio=1) ) ) ) )

### A.1.2   New CONNEG tags and values

In addition to the CONNEG tags and tag values defined in RFC2879[5], the capabilities string MAY include tag and tag values defined in the following subsections.

#### A.1.2.1   Definition of ‘profile’ tag and tag values

The new CONNEG tag ‘profile’ and accompanying tag values ‘uif-s’, ‘uif-f’, ‘uif-j’, ‘uif-cg’, ‘uif-c’, ‘uif-lg’, ‘uif-l’, and ‘uif-m’ shall be registered with the relevant authoritative body. This new tag and its tag values have been introduced to represent the incremental differences between minimum capabilities strings listed in sections A.1.1.1 through A1.1.5. This cuts down on the length of the CONNEG strings and makes it immediately apparent from a human’s perspective any OPTIONAL features that are advertised.

The CONNEG string “profile=uif-s” is defined to expand as

(& (image-file-structure=TIFF-minimal)
      (MRC-mode=0)
      (image-coding=MH)
      (color=Binary)
      (dpi=[200,300,600])
      (dpi-xyratio=1) )

The CONNEG string “profile=uif-f” is defined to expand as

(& (image-file-structure=TIFF-limited)
      (MRC-mode=0)
      (image-coding=MMR)
      (color=Binary)
      (dpi=[200,300,600])
      (dpi-xyratio=1) )

The CONNEG string “profile=uif-j” is defined to expand as

(& (image-file-structure=TIFF-limited)
      (MRC-mode=0)
      (image-coding=JBIG)
      (image-coding-constraint=JBIG-T85)
      (color=Binary)
      (JBIG-stripe-size=128)
      (dpi=[200,300,600])
      (dpi-xyratio=1) )

The CONNEG string “profile=uif-cg” is defined to expand as

(& (image-file-structure=TIFF-limited)
      (MRC-mode=0)
      (color=grey)
      (image-coding=JPEG)
      (image-coding-constraint=JPEG-T4E)
      (color-levels<=256)
      (color-space=CIELAB)
      (color-illuminant=D50)
      (CIELAB-L-min>=0)
      (CIELAB-L-max<=100)
      (dpi=[200,300])
      (dpi-xyratio=1) )

The CONNEG string “profile=uif-c” is defined to expand as

(& (image-file-structure=TIFF-limited)
      (MRC-mode=0)
      (color=full)
      (image-coding=JPEG)
      (image-coding-constraint=JPEG-T4E)
      (color-subsampling=“4:1:1")
      (color-levels<=16777216)
      (color-space=CIELAB)
      (color-illuminant=D50)
      (CIELAB-L-min>=0)
      (CIELAB-L-max<=100)
      (CIELAB-a-min>=-85)
      (CIELAB-a-max<=85)
      (CIELAB-b-min>=-75)
      (CIELAB-b-max<=125)
      (dpi=[200,300])
      (dpi-xyratio=1) )

The CONNEG string “profile=uif-lg” is defined to expand as

(& (image-file-structure=TIFF-limited)
      (MRC-mode=0)
      (color=grey)
      (image-coding=JBIG)
      (image-coding-constraint=JBIG-T43)
      (JBIG-stripe-size=128)
      (image-interleave=stripe)
      (color-space=CIELAB)
      (color-levels<=256)
      (color-illuminant=D50)
      (CIELAB-L-min>=0)
      (CIELAB-L-max<=100)
      (dpi=[200,300]) 
      (dpi-xyratio=1) )

The CONNEG string “profile=uif-l” is defined to expand as

(& (image-file-structure=TIFF-limited)
      (MRC-mode=0)
      (color=full)
      (image-coding=JBIG)
      (image-coding-constraint=JBIG-T43)
      (JBIG-stripe-size=128)
      (image-interleave=stripe)
      (color-levels<=16777216)
      (color-space=CIELAB)
      (color-illuminant=D50)
      (CIELAB-L-min>=0)
      (CIELAB-L-max<=100)
      (CIELAB-a-min>=-85)
      (CIELAB-a-max<=85)
      (CIELAB-b-min>=-75)
      (CIELAB-b-max<=125)
      (dpi=[100,200,300])
      (dpi-xyratio=1) )

#### A.1.2.2   Application of ‘profile’ tag and tag values

The ‘profile’ tag definition and its associated tag values allow the composite UIF Profile M to take the form shown below

(| (profile=[uif-s,uif-c])
   (& (image-file-structure=TIFF-MRC-limited)
      (MRC-mode=1)
      (MRC-max-stripe-size<=256)
      (profile=[uif-s,uif-c])
      (dpi=[200,300,400]) ) )

As another example, if a Receiver would like to advertise that it can support  UIF Profiles S and F with the optional resolution of 1200 dpi and can support UIF Profile C with the optional resolution of 600dpi, then the Receiver can return the following if a Sender queries its capabilities string:

(| (& (profile=[uif-s,uif-f])
      (dpi=[200,300,600,1200]) )
   (& (profile=uif-c)
      (dpi=[200,300,600]) ) )

## A.2   UIF Profiles supported

A UIF Sender MUST query the potential UIF Receiver for the UIF Profiles supported by the Receiver. A UIF Receiver MUST respond with the UIF Profiles that it supports. When a Receiver indicates the document formats / profiles that are supported, the list MUST include all the UIF Profiles described in this document that are supported and, if UIF Profile M is supported, all of the combinations with UIF-Profile M that are supported. The Sender MUST interpret a missing or otherwise invalid response as an indication that the Receiver does not support UIF. The method of transport and the actual data values used to indicate supported UIF Profiles are protocol-specific and beyond the scope of this document.

## A.3   Media supported

A UIF Sender MUST query the potential UIF Receiver for media supported. A UIF Receiver MUST respond with the media supported by the Receiver (e.g., letter, legal, A4, etc.). The method of transport, the valid range of media, and the actual data values used to indicate supported media are protocol-specific and beyond the scope of this document; however, the Sender MUST be able to infer actual dimensions from the media values used.

## A.4   Media ready

A UIF Sender MUST query the potential UIF Receiver for media ready. A UIF Receiver MUST respond with the subset of media supported that is ready to print with no user intervention. The method of transport, the valid range of media, and the actual data values used to indicate ready media are protocol-specific and beyond the scope of this document; however, the Sender MUST be able to infer actual dimensions from the media values used.

## A.5   Image reduction supported

A UIF Sender MAY query the potential UIF Receiver to determine whether or not image reduction is supported. A Receiver MUST be capable of indicating whether or not it supports image reduction. The method by which this query occurs is protocol-specific and beyond the scope of this document.

## A.6   Conformance Requirements Summary

For the listed operations, Table 18 below shows conformance requirements that apply to the protocol used to transport UIF data.

Table 19.   Underlying Protocol Conformance.

| Operation | UIF-capable Sender | UIF-capable Receiver | Section |
| --- | --- | --- | --- |
| Receiver capabilities string | MAY | MUST | A.1 |
| UIF Profiles supported | MUST | MUST | A.2 |
| Media supported | MUST | MUST | A.3 |
| Media ready | MUST | MUST | A.4 |
| Image reduction supported | MAY | MUST | A.5 |