# Default Conneg & More Detailed UIF Profile Summary

From:  John Pulera

File: default_conneg_etc.doc

Comments by T. Hastings

File: default_conneg_etc-th-comments.doc

ISSUES are highlighted like this

# Purpose

- To propose a simplified means of content negotiation whereby the transfer of Conneg would be optional.

- Provide detailed summary of UIF Profile requirements and how they differ from TIFF-FX Profiles

# Default Conneg

## Proposed usage

1. The sender issues a single Get-Printer-Attributes operation

The sender requests the values of the following IPP attributes, all of which are required for an IPP FAX Receiver to support, except “uif-conneg”:

ippfax-receiver (integer(0:MAX))

media-supported (1setOf (type3 keyword | name(MAX)))

media-ready  (1setOf (type3 keyword | name(MAX)))

uif-conneg (octetString32k)

uif-profiles-supported (1setOf type3 keyword)

2. The sender examines the “ippfax-receiver” attribute

The sender determines whether or not the receiver is operating as an IPP FAX device.  Greater than 0 means it is operating at the indicated IPP FAX level.

3. The sender  examines the “media-supported” and “media-ready” attributes

The “media-supported” and “media-ready” attributes are described in the IPP specification [1]. The “media-ready” attribute differs from “media-supported” in that legal values only include the subset of “media-supported” values that are physically loaded and ready for printing with no operator intervention required. I propose that the sender and receiver MUST support these two attributes so that the related Conneg feature tags (i.e., ‘paper-size’, ‘size-x’, and ‘size-y’) NEED NOT be included in the value of the “uif-conneg” Printer attribute returned by the IPP FAX Receiver.

4. The sender examines the “uif-profiles-supported” attribute

The response indicates the UIF profiles that the IPP FAX Receiver supports, for example, ‘uif-profile-s’, ‘uif-profile-c’.  The UIF profiles are the TIFF/FX profiles with additional IPP FAX requirements (see section 3).  See section 2.3 for the default CONNEG string representation for each of the UIF profiles.

From the “uif-profiles-supported” response, the sender then chooses a supported profile and applies the default capabilities for the selected profile.

5. The sender examines the “uif-conneg” attribute, if returned.

If the sender understands conneg and wants to determine which options the IPP FAX Receiver has implemented above the required features for the supported UIF profiles, the sender queries the “uif-conneg” Printer attribute.  If the Receiver does not return the “uif-conneg” attribute, that indicates that the Receiver doesn’t support CONNEG.  If the “uif-conneg” attribute is returned, it MUST be a valid CONNEG string.  Then the sender has the option of interpreting the returned Conneg string or the default string, as the default string MUST be a subset of what is allowed by a valid Conneg response.

A sender that does not implement Conneg MUST request the “uif-profiles-supported” attribute; a sender that does implement Conneg MAY request the “uif-profiles-supported” attribute, but MUST also request the “uif-profiles-supported” attribute, in case the IPP FAX Receiver doesn’t support the “uif-conneg” Printer attribute.

ISSUE 01:  Or is it so easy for a Receiver to support the “uif-conneg” Printer attribute (its just a canned constant string) that the UIF spec should REQUIRE an IPP FAX Receiver to support the “uif-conneg” Printer attribute?

Based on the IPP attribute values returned in steps 1 to 5, the sender now has enough information to send UIF-formatted data that is compatible with the receiver’s features.

ISSUE 02 : Should the UIF spec be made independent of IPP FAX by moving the discussion about an IPP attributes to the IFX spec?  Then UIF could be used with any protocol.

## IPP Attribute Description

The following IPP attributes would need to be added to the UIF specification:

uif-profiles-supported (1setOf  type3 keyword)

Type:		Printer Description Attribute

Description: 	List of profiles for which at least the base configuration (see Appendix B) is implemented.  Standard keyword values are:

‘uif-profile-s’

‘uif-profile-f’

‘uif-profile-j’

‘uif-profile-c’

‘uif-profile-l’

‘uif-profile-m’

Conformance: A receiver MUST support this attribute.

## UIF Default Conneg Strings

Default Conneg for UIF Profile S:

(& (image-file-structure=TIFF-minimal)

(color=Binary)

(image-coding=MH)

(dpi=600)

(dpi-xyratio=1)

(MRC-mode=0) )

Default Conneg for UIF Profile F

(& (image-file-structure=[TIFF-minimal,TIFF-limited-uif])

(color=Binary)

(image-coding=MH)

(dpi=600)

(dpi-xyratio=1)

(MRC-mode=0) )

Default Conneg for UIF Profile J

(| (& (image-file-structure=TIFF-minimal)

(color=Binary)

(image-coding=MH)

(dpi=600)

(dpi-xyratio=1)

(MRC-mode=0) )

(& (image-file-structure=TIFF-limited-uif)

(color=Binary)

(image-coding=JBIG)

(image-coding-constraint=JBIG-T85)

(JBIG-stripe-size=128)

(dpi=600)

(dpi-xyratio=1)

(MRC-mode=0) ) )

Default Conneg for UIF Profile C

(| (& (image-file-structure=TIFF-minimal)

(color=Binary)

(image-coding=MH)

(dpi=600) (dpi-xyratio=1)

(MRC-mode=0) )

(& (image-file-structure=TIFF-limited-uif)

(color=grey)

(color-levels<=256)

(image-coding=JPEG)

(image-coding-constraint=JPEG-T4E)

(color-space=CIELAB)

(CIELAB-L-min>=0)

(CIELAB-L-max<=100)

(color-illuminant=D50)

(dpi=300) (dpi-xyratio=1)

(MRC-mode=0) )

Default Conneg for UIF Profile L

(| (& (image-file-structure=TIFF-minimal)

(color=Binary)

(image-coding=MH)

(dpi=600)

(dpi-xyratio=1)

(MRC-mode=0) )

(& (image-file-structure=TIFF-limited-uif)

(& (color=grey)

(| (& (image-coding=JPEG)

(image-coding-constraint=JPEG-T4E) )

(& (image-coding=JBIG)

(image-coding-constraint=JBIG-T43)

(JBIG-stripe-size=128)

(image-interleave=stripe) ) )

(color-space=CIELAB)

(color-levels<=256)

(color-illuminant=D50)

(CIELAB-L-min>=0)

(CIELAB-L-max<=100)

(dpi=300) (dpi-xyratio=1) )

(MRC-mode=0) ) )

Default Conneg for UIF Profile M

(| (& (image-file-structure=TIFF-minimal)

(color=Binary)

(image-coding=MH)

(MRC-mode=0)

(dpi=600)

(dpi-xyratio=1) )

(& (image-file-structure=TIFF-limited-uif)

(color=grey)

(color-levels<=256)

(MRC-mode=0)

(image-coding=JPEG)

(image-coding-constraint=JPEG-T4E)

(color-space=CIELAB)

(CIELAB-L-min>=0)

(CIELAB-L-max<=100)

(color-illuminant=D50)

(dpi=300) (dpi-xyratio=1) )

(& (image-file-structure=TIFF-MRC-limited)

(MRC-mode=1)

(MRC-max-stripe-size<=256) ) )

Note that the maximum dimensions of the image are implied once the sender chooses the media and resolution it will use. For example, if the sender decides to use a resolution of 600x600 dpi, and letter paper size, then the horizontal pixel width is necessarily less than or equal to (8.5in)(600dpi) = 5100 pixels, and the pixel length is necessarily less than or equal to (11in)(600dpi) = 6600 pixels. The default values for the “size-x” and “size-y” Conneg tags SHALL be less than or equal to the value implied by the choice of media size and resolution. If there are no implicit dimensions associated with the chosen media, then a receiver must specify valid imaging dimensions using the ‘size-x’ and ‘size-y’ feature tags.

ISSUE 03:  The last sentence is a requirement for an IPP FAX Receiver to support CONNEG, if it has a paper size that is not one of the standard  (presumably the Internet FAX small set of standard sizes) paper sizes.  However, if IPP FAX were to use the Media Size Self Describing Name values for the “media”, “media-supported”, and “media-ready” attributes from the PWG Media Standardized Names, then the dimensions would be explicitly included for every media size, even custom sizes.  Should IPP FAX:

(1) REQUIRE exclusive support of,

(2) REQUIRE support plus current IPP/1.1 media size keywords,

(3) RECOMMEND, or

(4) list as an OPTION,

for an IPP FAX Sender and an IPP FAX Receiver to use the Media Size Self Describing Name keyword values for the “media”, “media-supported”, and “media-ready” attributes from the PWG Media Standardized Names?  Then this would be one more opportunity for a Receiver and a Sender to avoid having to use CONNEG, because of an unrecognized media size.

ISSUE 04:  Even if we don’t use the PWG Media Standardized Names, should IPP FAX:

(1) REQUIRE IPP media name keywords, e.g., ‘iso-a4-white’, ‘na-letter-white’

(2) REQUIRE IPP media size keywords, e.g., ‘iso-a4’, ‘na-letter’

(3) REQUIRE support of both

(4) REQUIRE support of (1), and RECOMMEND support of (2)

(5) REQUIRE support of (2), and RECOMMEND support of (1)

# UIF Profile Description

## UIF Profile S

This section defines UIF Profile S, which is the minimal black-and-white subset of TIFF that all implementations of UIF MUST support. UIF Profile S, which uses 1-dimensional Modified Huffman compression as defined in ITU-T T.4 [3], is based on TIFF-FX Profile S.

Differences between TIFF-FX Profile S and UIF Profile S:

- ImageWidth is not constrained

- XResolution is not constrained, but 600dpi MUST be supported

- YResolution is not constrained, but 600dpi MUST be supported

ISSUE 05:  Should IPP FAX REQUIRE support of 300dpi as well for Profile S and indicate that the Sender MUST send at 600dpi or higher, unless the Sending User has explicitly indicated that a degraded mode is satisfactory?

ISSUE 06:  Should IPP FAX REQUIRE support of 200dpi as well for Profile S and indicate that the Sender MUST send at 600dpi or higher, unless the Sending User has explicitly indicated that a degraded mode is satisfactory?

The following Baseline and Extension fields and field values MUST be supported by all UIF implementations. For a complete description of the Baseline and Extension TIFF fields shown below, see the TIFF-FX specification [2].

| Baseline Fields | Values |
| --- | --- |
| BitsPerSample | 1 |
| Compression | 3: 1D Modified Huffman coding set T4Options = 0 or 4 |
| FillOrder | 2: least significant bit first |
| ImageWidth | m: width of image in pixels |
| ImageLength | n: length of image in pixels (total number of scanlines) |
| NewSubFileType | 2: Bit 1 identifies single page of a multi-page document |
| PhotometricInterpretation | 0: pixel value 1 means black |
| ResolutionUnit | 2: inch |
| RowsPerStrip | number of scanlines per strip = ImageLength, with one strip |
| SamplesPerPixel | 1 |
| StripByteCounts | number of bytes in TIFF strip |
| StripOffsets | offset from beginning of file to single TIFF strip |
| XResolution | 600, other resolutions are optional (written in pixels per inch) |
| YResolution | 600, other resolutions are optional (written in pixels per inch) |

| Extension Fields | Values |
| --- | --- |
| PageNumber | n,m: page number n followed by total page count m |
| T4Options | 0: MH coding, EOLs not byte aligned 4: MH coding, EOLs byte aligned |

## UIF Profile F

This section defines UIF Profile F, which uses Modified Read and Modified Modified Read (MMR) compression (described in ITU-T T.4 [3] and ITU-T T.6 [4]) in addition to the Modified Huffman compression used for UIF Profile S. UIF Profile F is based on TIFF-FX Profile F. The table that follows summarizes fields and field values that are required / recommended for UIF Profile F. For a complete description of the Baseline, Extension, and New TIFF fields shown below, see the TIFF-FX specification [2]. Implementations of this profile are required to also implement UIF Profile S.

Differences between TIFF-FX Profile F and UIF Profile F:

- ImageWidth is not constrained

- XResolution is not constrained, but 600dpi MUST be supported

- YResolution is not constrained, but 600dpi MUST be supported

- The following TIFF-FX recommended fields have been ommitted: ‘BadFaxLines’, ‘CleanFaxData’, ‘ConsecutiveBadFaxLines’, ‘ProfileType’, and ‘FaxProfile’

ISSUE 07:  Should IPP FAX REQUIRE support of 300dpi as well for Profile F and indicate that the Sender MUST send at 600dpi or higher, unless the Sending User has explicitly indicated that a degraded mode is satisfactory?

ISSUE 08:  Should IPP FAX REQUIRE support of 200dpi as well for Profile F and indicate that the Sender MUST send at 600dpi or higher, unless the Sending User has explicitly indicated that a degraded mode is satisfactory?

Recommended fields are shown with an asterisk *.

Required fields or values are shown with a double asterisk **. If the  double asterisk is on the field name, then all the listed values are required of implementations; if the double asterisks are in the Values column, then only the values suffixed with a double asterisk  are required of implementations.

Optional fields have no asterisks in either the field name or the Values column, however, the Values field may contain a condition which REQUIRES the field.

| Baseline Fields | Values |
| --- | --- |
| BitsPerSample | 1** |
| Compression | 3**: 1D Modified Huffman and 2D Modified Read coding 4: 2D Modified Modified Read coding |
| DateTime* | {ASCII}: date/time in 24-hour format "YYYY:MM:DD HH:MM:SS" |
| FillOrder** | 1: most significant bit first 2: least significant bit first |
| ImageDescription* | {ASCII}: A string describing the contents of the image |
| ImageWidth** | n: width of image in pixels |
| ImageLength** | n: length of image in pixels (total number of scanlines) |
| NewSubFileType | 2**: Bit 1 identifies single page of a multi-page document |
| Orientation | 1**-8, Default is 1 |
| PhotometricInterpretation** | 0: pixel value 1 means black 1: pixel value 1 means white |
| ResolutionUnit** | 2: inch 3: centimeter |
| RowsPerStrip** | n: number of scanlines per TIFF strip |
| SamplesPerPixel | 1** |
| Software* | {ASCII}: name & release number of creator software |
| StripByteCounts** | n: number of bytes in TIFF strip |
| StripOffsets** | n: offset from beginning of file to each TIFF strip |
| XResolution | 600**, other resolutions are optional (written in pixels per inch) |
| YResolution | 600**, other resolutions are optional (written in pixels per inch) |

| Extension Fields | Values |
| --- | --- |
| T4Options | 0**: required if Compression is Modified Huffman, EOLs are not byte aligned 1: required if Compression is 2D Modified Read, EOLs are not byte aligned 4**: required if Compression is Modified Huffman, EOLs are byte aligned 5: required if Compression is 2D Modified Read, EOLs are byte aligned |
| T6Options | 0: required if Compression is 2D Modified Modified Read |
| DocumentName* | {ASCII}: name of scanned document |
| PageNumber** | n,m: page number followed by total page count |

| New Fields | Values |
| --- | --- |
| GlobalParametersIFD* | IFD: global parameters IFD |
| CodingMethods* | n: compression algorithms used in file |

## UIF Profile J

This section defines Profile J for UIF, which uses lossless JBIG compression as it is defined in ITU-T T.82 [8] subject to the application rules given in ITU-T T.85 [9]. UIF Profile J is based on TIFF-FX Profile J. The following table summarizes fields and field values that are required / recommended. For a complete description of the Baseline, Extension, and New TIFF fields shown below, see the TIFF-FX specification [2]. Implementations of this profile are required to also implement UIF Profile S.

Differences between TIFF-FX Profile J as defined in [2] and UIF Profile J:

- ImageWidth is not constrained

- XResolution is not constrained, but 600dpi MUST be supported

- YResolution is not constrained, but 600dpi MUST be supported

- The following TIFF-FX recommended fields have been ommitted: ‘ProfileType’ and ‘FaxProfile’

ISSUE 09:  Should IPP FAX REQUIRE support of 300dpi as well for Profile J and indicate that the Sender MUST send at 600dpi or higher, unless the Sending User has explicitly indicated that a degraded mode is satisfactory?

ISSUE 10:  Should IPP FAX REQUIRE support of 200dpi as well for Profile J and indicate that the Sender MUST send at 600dpi or higher, unless the Sending User has explicitly indicated that a degraded mode is satisfactory?

Recommended fields are shown with an asterisk *.

Required fields or values are shown with a double asterisk **. If the  double asterisk is on the field name, then all the listed values are required of implementations; if the double asterisks are in the Values column, then the attribute and only the values suffixed with a double asterisk  are required of implementations.

Optional fields have no asterisks in either the field name or the Values column, however, the Values field may contain a condition which REQUIRES the field.

| Baseline Fields | ### Values |
| --- | --- |
| BitsPerSample | 1** |
| Compression | 9**: JBIG coding |
| DateTime* | {ASCII}: date/time in 24-hour format "YYYY:MM:DD HH:MM:SS" |
| FillOrder** | 1: most significant bit first 2: least significant bit first |
| ImageDescription* | {ASCII}: A string describing the contents of the image |
| ImageWidth** | n: width of image in pixels |
| ImageLength** | n: length of image in pixels (total number of scanlines) |
| NewSubFileType** | 2: Bit 1 identifies single page of a multi-page document |
| Orientation | 1**-8, Default is 1 |
| PhotometricInterpretation** | 0: pixel value 1 means black 1: pixel value 1 means white |
| ResolutionUnit** | 2: inch 3: centimeter |
| RowsPerStrip** | n: number of scanlines per TIFF strip |
| SamplesPerPixel** | 1 |
| Software* | {ASCII}: name & release number of creator software |
| StripByteCounts** | n: number of bytes in TIFF strip |
| StripOffsets** | n: offset from beginning of file to each TIFF strip |
| XResolution | 600**, other resolutions are optional (written in pixels per inch) |
| YResolution | 600**, other resolutions are optional (written in pixels per inch) |

| Extension Fields | ### Values |
| --- | --- |
| DocumentName* | {ASCII}: name of scanned document |
| PageNumber** | n,m: page number followed by total page count |

| New Fields | ### Values |
| --- | --- |
| GlobalParametersIFD* | IFD: global parameters IFD |
| T82Options** | 0: T.85 profile of T.82 |
| CodingMethods* | n: compression algorithms used in file |

## UIF Profile C

This section defines Profile C for UIF, which uses lossy JPEG compression as it is defined in ITU-T T.81 [7]. UIF Profile C is based on TIFF-FX Profile C. The following table summarizes fields and field values that are required / recommended. For a complete description of the Baseline, Extension, and New TIFF fields shown below, see the TIFF-FX specification [2]. Implementations of this profile are required to also implement UIF Profile S.

Differences between TIFF-FX Profile C as defined in [2] and UIF Profile C:

- ImageWidth is not constrained

- XResolution is not constrained, but 300dpi MUST be supported

- YResolution is not constrained, but 300dpi MUST be supported

- The following TIFF-FX recommended fields have been ommitted: ‘ProfileType’ and ‘FaxProfile’

ISSUE 11:  Should IPP FAX REQUIRE support of 200dpi as well for Profile C and indicate that the Sender MUST send at 300dpi or higher, unless the Sending User has explicitly indicated that a degraded mode is satisfactory?

Recommended fields are shown with an asterisk *.

Required fields or values are shown with a double asterisk **. If the  double asterisk is on the field name, then all the listed values are required of implementations; if the double asterisks are in the Values column, then only the values suffixed with a double asterisk  are required of implementations.

Optional fields have no asterisks in either the field name or the Values column, however, the Values field may contain a condition which REQUIRES the field.

| Baseline Fields | Values |
| --- | --- |
| BitsPerSample | 8**: 8 bits per color sample 12: optional 12 bits/sample |
| Compression** | 7: JPEG |
| DateTime* | {ASCII}: date/time in 24-hour format "YYYY:MM:DD HH:MM:SS" |
| FillOrder** | 1: most significant bit first 2: least significant bit first |
| ImageDescription* | {ASCII}: A string describing the contents of the image |
| ImageWidth** | n: width of image in pixels |
| ImageLength** | n: length of image in pixels (total number of scanlines) |
| NewSubFileType** | 2: Bit 1 identifies single page of a multi-page document |
| Orientation | 1**-8, Default is 1 |
| PhotometricInterpretation** | 10**: ITULAB |
| ResolutionUnit** | 2: inch 3: centimeter |
| RowsPerStrip** | n: number of scanlines per TIFF strip |
| SamplesPerPixel** | 1**: L* (lightness) 3: LAB |
| Software* | {ASCII}: name & release number of creator software |
| StripByteCounts** | n: number of bytes in TIFF strip |
| StripOffsets** | n: offset from beginning of file to each TIFF strip |
| XResolution | 300** other resolutions are optional (written in pixels per inch) |
| YResolution | 300** other resolutions are optional (written in pixels per inch) |

| Extension Fields | Values |
| --- | --- |
| DocumentName* | {ASCII}: name of scanned document |
| PageNumber** | n,m: page number followed by total page count |
| ChromaSubSampling | (1,1), (2, 2)** (1, 1): equal numbers of lightness and chroma samples horizontally and vertically (2, 2): twice as many lightness samples as chroma samples horizontally and vertically |
| ChromaPositioning | 1**: centered |

| New Fields | Values |
| --- | --- |
| Decode** | minL, maxL, mina, maxa, minb, maxb: minimum and maximum values for L*a*b* |
| GlobalParametersIFD* | IFD: global parameters IFD |
| CodingMethods* | n: compression algorithms used in file |
| VersionYear* | byte sequence: year of ITU std |

## UIF Profile L

This profile is modeled after TIFF-FX Profile L. It uses JBIG compression (see [8]), subject to the application rules specified in ITU-T Recommendation T.43 [5] to losslessly code three types of color and grayscale images: one bit per color CMY, CMYK and RGB images; a palettized (i.e. mapped) color image; and continuous tone color and grayscale images.

Differences between TIFF-FX Profile L as defined in [2] and UIF Profile L:

- ImageWidth is not constrained

- XResolution is not constrained, but 300dpi MUST be supported

- YResolution must match XResolution, but it is not otherwise constrained; 300dpi MUST be supported

- The following TIFF-FX recommended fields have been ommitted: ‘ProfileType’ and ‘FaxProfile’

ISSUE 12:  Should IPP FAX REQUIRE support of 200dpi as well for Profile L and indicate that the Sender MUST send at 300dpi or higher, unless the Sending User has explicitly indicated that a degraded mode is satisfactory?

The table that follows summarizes fields and field values that are required / recommended for implementation of UIF Profile L. For a complete description of the Baseline, Extension, and New TIFF fields shown below, see the TIFF-FX specification [2]. Implementations of this profile are required to also implement UIF Profile S, and UIF Profile C.

Recommended fields are shown with an asterisk *.

Required fields or values are shown with a double asterisk **. If the  double asterisk is on the field name, then all the listed values are required of implementations; if the double asterisks are in the Values column, then only the values suffixed with a double asterisk  are required of implementations.

Optional fields have no asterisks in either the field name or the Values column, however, the Values field may contain a condition which REQUIRES the field.

| Baseline Fields | Values |
| --- | --- |
| BitsPerSample | 1: Binary RGB, CMY(K) 8**: 8 bits per color sample 9-16: optional |
| Compression** | 10**: JBIG, per T.43 |
| DateTime* | {ASCII}: date/time in 24-hour format "YYYY:MM:DD HH:MM:SS" |
| FillOrder** | 1: most significant bit first 2: least significant bit first |
| ImageDescription* | {ASCII}: A string describing the contents of the image |
| ImageWidth** | n: width of image in pixels |
| ImageLength** | n: length of image in pixels (total number of scanlines) |
| NewSubFileType** | 2: Bit 1 identifies single page of a multi-page document |
| Orientation | 1**-8, Default is 1 |
| PhotometricInterpretation | 2: RGB 5: CMYK 10**: ITULAB |
| ResolutionUnit** | 2: inch |
| RowsPerStrip** | n: number of scanlines per TIFF strip |
| SamplesPerPixel** | 1**: L* (lightness) 3: LAB, RGB, CMY 4: CMYK |
| Software* | {ASCII}: name & release number of creator software |
| StripByteCounts** | n: number of bytes in TIFF strip |
| StripOffsets** | n: offset from beginning of file to each TIFF strip |
| XResolution | 300** other resolutions are optional (written in pixels per inch) |
| YResolution | equal to XResolution (pixels MUST be square) |

| Extension Fields | Values |
| --- | --- |
| DocumentName* | {ASCII}: name of scanned document |
| PageNumber** | n,m: page number followed by total page count |
| Indexed | 0: not a palette-color image 1: palette-color image |

| New Fields | Values |
| --- | --- |
| Decode** | minL, maxL, mina, maxa, minb, maxb: minimum and maximum values for L*a*b* |
| GlobalParametersIFD* | IFD: global parameters IFD |
| CodingMethods* | n: compression algorithms used in file |
| VersionYear* | byte sequence: year of ITU std |

## UIF Profile M

This profile is modeled after TIFF-FX Profile M, which uses Mixed Raster Content (MRC), defined in ITU-T Recommendation T.44 [6]. MRC enables different coding methods and resolutions within a single page. For a more detailed description of MRC and the Baseline, Extension, and New TIFF fields shown below, see [2] and [6].

Differences between TIFF-FX Profile M as defined in [2] and UIF Profile M:

- ImageWidth is not constrained

- XResolution is not constrained, but 600dpi MUST be supported with the bi-level mask layer, and 300dpi MUST be supported with the foreground and background layers.

- YResolution must match XResolution, but it is not otherwise constrained; 600dpi MUST be supported with the bi-level mask layer; 300dpi MUST be supported with the foreground and background layers

- The following TIFF-FX recommended fields have been ommitted: ‘ProfileType’ and ‘FaxProfile’

ISSUE 13:  Should IPP FAX REQUIRE support of 300dpi as well for Profile M with the bi-level mask layer and indicate that the Sender MUST send at 600dpi or higher, unless the Sending User has explicitly indicated that a degraded mode is satisfactory?

ISSUE 14:  Should IPP FAX REQUIRE support of 200dpi as well for Profile M with the bi-level mask layer and indicate that the Sender MUST send at 600dpi or higher, unless the Sending User has explicitly indicated that a degraded mode is satisfactory?

ISSUE 15:  Should IPP FAX REQUIRE support of 200dpi as well for Profile M with the foreground and background layers and indicate that the Sender MUST send at 300dpi or higher, unless the Sending User has explicitly indicated that a degraded mode is satisfactory?

The table that follows summarizes fields and field values that are required / recommended for implementation of UIF Profile M.. Implementations of this profile are required to also implement UIF Profile S, and UIF Profile C.

Recommended fields are shown with an asterisk *.

Required fields or values are shown with a double asterisk **. If the  double asterisk is on the field name, then all the listed values are required of implementations; if the double asterisks are in the Values column, then only the values suffixed with a double asterisk  are required of implementations.

Optional fields have no asterisks in either the field name or the Values column, however, the Values field may contain a condition which REQUIRES the field.

| Baseline Fields | Values |
| --- | --- |
| BitsPerSample | 1**: binary mask, RGB, CMY(K) 2-8**: bits per color sample 9-16: optional 12 bits/sample |
| Compression** | 1: None (ImageBaseColor IFD only) 3**: Modified Huffman and Modified Read 4: Modified Modified Read 7**: JPEG 9: JBIG, per [9] 10: JBIG, per [5] |
| DateTime* | {ASCII}: date/time in 24-hour format "YYYY:MM:DD HH:MM:SS" |
| FillOrder** | 1: most significant bit first 2: least significant bit first |
| ImageDescription* | {ASCII}: A string describing the contents of the image |
| ImageWidth** | n: width of image in pixels |
| ImageLength** | n: length of image in pixels (total number of scanlines) |
| NewSubFileType** | 16, 18: ¦ Bit 1 indicates single page of a multi-page document on Primary IFD Bit 4 indicates MRC model |
| Orientation | 1**-8, Default is 1 |
| PhotometricInterpretation | 0**: WhiteIsZero (Mask Layer) 2: RGB 5: CMYK 10**: ITULAB |
| ResolutionUnit** | 2: inch |
| RowsPerStrip | n: number of scanlines per TIFF strip |
| SamplesPerPixel** | 1**: L* (lightness) 3: LAB, RGB, CMY 4: CMYK |
| Software* | {ASCII}: name & release number of creator software |
| StripByteCounts** | n: number of bytes in TIFF strip |
| StripOffsets** | n: offset from beginning of file to each TIFF strip |
| XResolution | 300**: background & foreground layers; 600**: binary mask layer; other resolutions are optional |
| YResolution | 300**: background & foreground layers; 600**: binary mask layer; other resolutions are optional; must be equal to XResolution (pixels MUST be square) |

| Extension Fields | Values |
| --- | --- |
| T4Options | 0**: required if Compression is Modified Huffman, EOLs not byte aligned 1: required if Compression 2D Modified Read, EOLs are not byte aligned 4**: required if Compression Modified Huffman, EOLs byte aligned 5: required if Compression 2D Modified Read, EOLs are byte aligned |
| T6Options | 0: required if Compression is 2D Modified Modified Read |
| DocumentName* | {ASCII}: name of scanned document |
| PageNumber** | n,m: page number followed by total page count |
| ChromaSubSampling | (1,1), (2, 2)** (1, 1): equal numbers of lightness and chroma samples horizontally & vertically¦ (2, 2): twice as many lightness samples as chroma horizontally and vertically |
| ChromaPositioning | 1: centered |
| Indexed | 0: not a palette-color image 1: palette-color image |
| SubIFDs | <IFD>: byte offset to FG/BG IFDs |
| XPosition | horizontal offset in primary IFD resolution units |
| YPosition | vertical offset in primary IFD resolution units |

| New Fields | Values |
| --- | --- |
| Decode** | minL, maxL, mina, maxa, minb, maxb: minimum and maximum values for L*a*b* |
| ImageBaseColor | a,b,c: background color in ITULAB |
| StripRowCounts | n: number of scanlines in each strip |
| ImageLayer | n, m: layer number, imaging sequence (e.g., strip number) |
| T82Options | 0: T.85 profile of T.82 coding |
| GlobalParametersIFD* | IFD: global parameters IFD |
| CodingMethods* | n: compression algorithms used in file |
| ModeNumber* | n: version of T.44 standard |
| VersionYear* | byte sequence: year of ITU std |

# References

[1] 	Herriot, Butler , Moore, Turner, Wenn. "Internet Printing Protocol/1.1: Encoding and Transport", RFC 2910

[2]	McIntyre, Zilles, Buckley, Venable, Parsons, Rafferty "File Format for Internet Fax", RFC2301

[3]	ITU-T Recommendation T.4, Standardization of group 3 facsimile apparatus for document transmission, October 1997

[4]	ITU-T Recommendation T.6, Facsimile coding schemes and coding control functions for group 4 facsimile apparatus, November 1988

[5]	ITU-T Recommendation T.43, Colour and gray-scale image representations using lossless coding scheme for facsimile, February 1997

[6]	ITU-T Recommendation T.44, Mixed Raster Content (MRC), April 1999.

[7] 	ITU-T Recommendation T.81, Information technology - Digital compression and coding of continuous-tone still images - Requirements and guidelines, September 1992

[8]	ITU-T Recommendation T.82, Information technology - Coded representation of picture and audio information - Progressive bi-level image compression, March 1995

[9] 	ITU-T Recommendation T.85, Application profile for Recommendation T.82 - Progressive bi-level image compression (JBIG coding scheme) for facsimile apparatus, August 1995