A Project of the PWG-IPP Working Group

Printer Working Group (PWG):

Semantic Model

IEEE-ISTO Printer Working Group

Standard XXXX.X-200X

October 29, 2002

Version 0.15

Abstract

This document is a high level overview of the Semantic Model defined by the PWG.  This document briefly describes the semantic elements defined in various PWG documents and PWG documents submitted to the IETF.  The Semantic Model also incorporates additions made by other groups addressing print systems.  With every semantic element included a reference is provided to the document and section that details the semantic definition.

The Semantic Model contains a high level description of the Actions that operate on the objects and attributes in the model.  This document does not describe the mapping of the semantics onto a specific protocol or network environment.

Copyright (C) 2002, IEEE Industry Standards and Technology Organization. All rights reserved.

This document may be copied and furnished to others, and derivative works that comment on, or otherwise explain it or assist in its implementation may be prepared, copied, published and distributed, in whole or in part, without restriction of any kind, provided that the above copyright notice, this paragraph and the title of the Document as referenced below are included on all such copies and derivative works.  However, this document itself may not be modified in any way, such as by removing the copyright notice or references to the IEEE-ISTO and the Printer Working Group, a program of the IEEE-ISTO.

Title:  Printer Working Group (PWG): Semantic Model

The IEEE-ISTO and the Printer Working Group DISCLAIM ANY AND ALL WARRANTIES, WHETHER EXPRESS OR IMPLIED INCLUDING (WITHOUT LIMITATION) ANY IMPLIED WARRANTIES OF MERCHANTABILITY OR FITNESS FOR A PARTICULAR PURPOSE.

The Printer Working Group, a program of the IEEE-ISTO, reserves the right to make changes to the document without further notice.  The document may be updated, replaced or made obsolete by other documents at any time.

The IEEE-ISTO takes no position regarding the validity or scope of any intellectual property or other rights that might be claimed to pertain to the implementation or use of the technology described in this document or the extent to which any license under such rights might or might not be available; neither does it represent that it has made any effort to identify any such rights.

The IEEE-ISTO invites any interested party to bring to its attention any copyrights, patents, or patent applications, or other proprietary rights which may cover technology that may be required to implement the contents of this document. The IEEE-ISTO and its programs shall not be responsible for identifying patents for which a license may be required by a document and/or IEEE-ISTO Industry Group Standard or for conducting inquiries into the legal validity or scope of those patents that are brought to its attention. Inquiries may be submitted to the IEEE-ISTO by e-mail at:

ieee-isto@ieee.org.

The Printer Working Group acknowledges that the IEEE-ISTO (acting itself or through its designees) is, and shall at all times, be the sole entity that may authorize the use of certification marks, trademarks, or other special designations to indicate compliance with these materials.

Use of this document is wholly voluntary.  The existence of this document does not imply that there are no other ways to produce, test, measure, purchase, market, or provide other goods and services related to its scope.

Table of Contents

1	Introduction	7

2	Terminology	7

3	Model Overview	8

4	Data Classes	9

4.1	Printer Object Class	9

4.1.1	Printer Status Elements	10

4.1.2	Printer Description Elements	11

4.1.3	Printer Defaults, Supported and Ready Processing Elements	11

4.2	Job Object Class	12

4.2.1	Job Status Elements	12

4.2.2	Job Description Elements	14

4.3	Document Object Class	14

4.3.1	Document Status Elements	14

4.3.2	Document Description Elements	16

4.4	Processing Elements	16

4.4.1	Job Processing Elements	17

4.4.2	Document Processing Elements	18

5	Actions	19

5.1	Job Creation and document submission Actions	20

5.1.1	CreateJob	22

5.1.2	PrintJob	22

5.1.3	PrintUri	22

5.1.4	SendDocument	22

5.1.5	SendUri	22

5.1.6	ValidateDocument	23

5.1.7	ValidateJob	23

5.2	Job and Document Control Actions	23

5.2.1	CancelCurrentJob	23

5.2.2	CancelDocument	23

5.2.3	CancelJob	23

5.2.4	DeleteDocument	23

5.2.5	HoldJob	23

5.2.6	PromoteJob	23

5.2.7	ReleaseJob	23

5.2.8	ReprocessJob	24

5.2.9	RestartJob	24

5.2.10	ResumeJob	24

5.2.11	ScheduleJobAfter	24

5.2.12	SetDocumentElements	24

5.2.13	SetJobElements	24

5.2.14	SuspendCurrentJob	24

5.3	Status and information Actions	24

5.3.1	GetDocumentElements	24

5.3.2	GetDocuments	24

5.3.3	GetJobElements	25

5.3.4	GetJobs	25

5.3.5	GetPrinterElements	25

5.3.6	GetPrinterSettableElementValues	25

5.4	Printer Control Actions	25

5.4.1	ActivatePrinter	25

5.4.2	DeactivatePrinter	25

5.4.3	DisablePrinter	25

5.4.4	EnablePrinter	26

5.4.5	HoldNewJobs	26

5.4.6	PausePrinter	26

5.4.7	PausePrinterAfterCurrentJob	26

5.4.8	PurgeJobs	26

5.4.9	ReleaseHeldNewJobs	26

5.4.10	RestartPrinter	26

5.4.11	ResumePrinter	26

5.4.12	SetPrinterElements	26

5.4.13	ShutdownPrinter	26

5.4.14	StartupPrinter	26

6	Globalization	27

7	Summary of elements	27

7.1	Processing Elements (Job and Document)	27

7.2	Job Elements (Status and Description)	38

7.3	Document Elements (Status and Description)	42

7.4	Printer Elements (Status and Description)	45

8	Status Strings	50

9	Change Log	54

10	References	55

Author's Addresses	56

11	Appendix A – UPnP Definitions	57

11.1	DeviceID	57

12	Appendix B – IPP Mapping	58

12.1	Changes to remove some IPP specific aspects	58

12.2	Attribute Group Mapping	59

Table of Figures

Figure 1 Model Overview	8

Figure 2 Data Classes	9

Figure 3 Printer Status Elements	10

Figure 4 - The "PrinterState" element and the Printer Life Cycle	10

Figure 5 Printer Description Elements	11

Figure 6 Job Status Elements	13

Figure 7 The "JobState" Job Element and the Job object life cycle	13

Figure 8 Job Description Elements	14

Figure 9 Document Status Elements	15

Figure 10  "DocumentState" Element and Document object life Cycle	15

Figure 11 Document Description Elements	16

Figure 12 - Processing Elements	17

Figure 13 Job Processing Elements	17

Figure 14 Finishing Elements	18

Figure 15 Imposition Elements	19

Figure 16 Rendering Elements	19

Figure 17 Processing Instruction Processing	21

Table of Tables

Table 1-Integer syntaxes whose ProcessingElementSupported syntax isn’t RangeOfInteger	12

Table 2 - Summary of Actions	20

Table 3 - Processing Elements (Job and Document)	28

Table 4- Job Elements (Status and Description)	38

Table 5 – Document Elements (Status and Description)	42

Table 6 - Printer Elements (Status and Description)	45

Table 7  Status strings indicating some degree of success	51

Introduction

This document is a high level overview of the Semantic Model defined by the PWG.  This document briefly describes the semantic elements defined in various PWG documents and PWG documents submitted to the IETF.  The Semantic Model also incorporates additions made by other groups addressing print systems.  With every semantic element included a reference is provided to the document and section that details the semantic definition.

The Semantic Model contains a high level description of the Actions that operate on the objects and Elements in the model.  This document does not describe the mapping of the semantics onto a specific protocol or network environment.

# Terminology

| Action | A request that a Print Client makes to an object to perform some activity. The object returns a response to the Print Client that contains some information about the effect of the action on the object. |
| --- | --- |
| Data Class | A template for data describing an object and representing its state. Each Element in the data class represents a semantic element of the associated object. |
| Document | An object containing descriptive and state information for a logical unit of information to be printed. The object may contain processing information. The document content is represented by a single data (e.g. PDL, image) file and contains Pages. |
| Document Processing Elements | Document Elements supplied by the Print Client to direct the printing of a Document that the Printer copies to the Document. Examples: Copies, Finishings, Media, NumberUp. |
| End User | A print client that has no special rights on the printer. The End User typically submits jobs. The End User is allowed to query the printer, jobs and documents and control jobs based on policy. |
| Impression | Everything printed on a single side of a media |
| Job | An object that represents the submission of work for the printer. It contains descriptive and state information as well as default Document Processing Elements. Jobs contain one or more Documents |
| Job Description Elements | Job Elements supplied by the Print Client to describe the Job. Examples: JobName, RequestingUserName, JobRecipient |
| Job Processing Elements | Job Elements supplied by the Print Client to direct the printing of the Job as a whole that the Printer copies to the Job. Examples: JobHoldUntil, JobPriority, JobCopies, JobFinishings. |
| Object | A entity that instantiates a data class and implements the appropriate actions. |
| Operator | A print client that has special rights on the printer. The Operator typically oversees the printer. The Operator is allowed to query and control the printer, jobs and documents based on site policy. |
| MediaSheet | A sheet of paper, or other material, used for printing |
| Page | A logical entity that represents the information contained on a single side of a sheet of media. Note that this is the electronic form and that multiple pages can be rendered into a single impression through N-Up printing |
| PDL | (Page Description Language) A language that describes the content to be printed and how it will be laid out on a page (e.g. Adobe PostScript®, Hewlett Packard PCL®). |
| Print Client | An application or network entity that performs actions |
| Printer | An object that represents a printing device, set of printing devices, or a printing service and contains zero or more Jobs |
| Type 1 keyword | All the values are defined in the specification. Additional values require a new specification. |
| Type 2 keyword | An initial set of values is defined in the specification. This working group registers additional values after review. The initial versions of the specification will contain the values registered so far. After the specification is approved, this working group will register additional values after approval. |
| Type 3 keyword | An initial set of values is defined in the specification. Additional values are registered without working group review. The initial versions of the specification contain the values registered so far. After the specification is approved, this working group will register additional values without approval. |

# Model Overview

The Printer Working Group (PWG) has defined a simplified printing model.  It represents printing in either a client/server print paradigm or a peer-to-peer print paradigm.  The PWG model describes the device as a Printer object.  A Printer object may represent one or more physical Printers.  Another object is the Job.  A Printer can contain zero or more Jobs and a Job is contained in only one Printer.  Each Job can contain zero or more documents.  A Job can contain zero or more Documents and a Document is contained in only one Printer.  The PWG model contains methods that act upon these objects.

Figure 1 Model Overview

The objects are represented in the semantic model as data classes. The methods are represented as a set of actions that act upon those data classes.  The actions permit the creation and control of Jobs and documents as well as the submission of Document data.  The content of a Document is included in the submission or can be accessed via a URL reference.  There are also actions to query a Printer, Job or Document to access their Elements or to list their contained objects.

The model uses a number of terms with specific meaning for a printer.

# Data Classes

This section describes the data classes in the PWG semantic model.  Some of the classes are taken from the model and semantics of IPP [rfc2911].    Shows the data classes, theireelements and the containment relationship between the classes

Figure 2 Data Classes

## Printer Object Class

The Printer class is represented by a collection of elements as shown in .  The PrintereElements are presented in detail in Table 6.  The printer object also contains elements that describe the valid processing element values. (See section 4.3.2 for processing elements)  The Printer class is the container for Jobs.

### Printer Status Elements

Figure 3 below shows the PrintereStatuseElements.  These elements represent the state of the printer such as the number of jobs or existing error conditions.  Automata change the values of the elements in this group.  End Users cannot directly modify their values.  The End User can affect the values of these elements through actions (e.g. PausePrinter can change the value of PrinterIsAcceptingJobs”).  The semantics of the elements are summarized in Table 6.

Figure 3 Printer Status Elements

The “PrinterState” element is one of the most important Printer Status elements.  Figure 4 shows the values of the “PrinterState” element and the Printer life cycle as affected by actions on the Printer and job processing.

Figure 4 - The "PrinterState" element and the Printer Life Cycle

### Printer Description Elements

below shows the Printer DescriptioneElements.  These elements contain information that describes the printer such as its make, where it’s located and its speed.  An automaton controls some of the elements in this group (e.g. “PagesPerMinute”). Others elements in this group can be modified by Operators or Administrators (e.g. “PrinterName”).  The semantics of the elements are summarized in Table 6.

Figure 5 Printer Description Elements

### Printer Defaults, Supported and Ready Processing Elements

See section 4.3.2 below for the elements that may comprise these groups.  Processing Elements are the union of Job Processing Elements and Document Processing Elements. If a Processing element (e.g. Media) is supported, the Printer must have an associated Processing Supported Element (e.g. MediaSupported) and Processing Default Element (e.g. MediaDefault) Printer element.  There may be an associated Processing Ready Element (e.g. MediaReady) Printer element.  By retrieving the Printer Processing elements, a Client can determine all the Job and Document Processing elements and values that may be used in creating Jobs and Documents.

#### Processing Supported Elements

These elements list all the currently configured valid values for each Job Processing Element and Document Processing Element.  Though the Printer is configured to support the feature, human intervention may be required to process the job (e.g. selected paper may have to be loaded into a tray).  The syntax for Processing Elements Supported is multi-valued when the associated processing element is a string.  When syntax of the processing element is an integer, the syntax of the corresponding Processing Supported Element is usually RangeOfInteger which indicates the minimum and maximum values supported by the Printer.  However, there are some exceptions as indicated in Table 1.

Table 1-Integer syntaxes whose ProcessingElementSupported syntax isn’t RangeOfInteger

| “xxx” element name | “xxx” syntax | “xxxSupported” syntax |
| --- | --- | --- |
| JobPriority | Integer | Integer (Max value) |
| Copies | Integer | Integer (Max value) |
| PageRanges | RangeOfInteger (multivalued) | Boolean (are PageRanges supported) |

#### Processing Default Elements

These elements give the default value for the associated processing instruction if the Processing Element of the job and document are not supplied and the instructions is not embedded in the PDL.  The syntax for the Processing Default Elements is the same as the corresponding Processing Element.  The only exception is that the PageRanges element does not have a PageRangesDefault element.

#### Processing Ready Elements

These elements give the features available without human intervention.  The syntax for a Processing Ready Element is the same as the corresponding Processing Element.

## Job Object Class

The Job object class is represented by a collection of elements divided into four groups as shown in .  The Job class also contains the document class

Job Status Elements – See Section 4.2.1

Job Description Elements – See section 4.2.2.

Job Processing Elements – See section 4.4.1

Document Processing Elements – See section 4.4.2

### Job Status Elements

Figure 6 below shows the Joe StatuseElements. Automata primarily control the elements in this group.  End Users cannot directly modify their values.  The End User can affect the values of these elements through actions (e.g. CancelJob can change the value of JobStateReasons”).  The semantics of the elements are summarized in Table 4.

Figure 6 Job Status Elements

#### The Job Life Cycle

The “JobState” element is one of the most important Job Status elements. Figure 7 shows the values of the “JobState” element and the Job life cycle as affected by actions on the Job, Printer, and job processing.

Figure 7 The "JobState" Job Element and the Job object life cycle

### Job Description Elements

Figure 8 below shows the JobeElements. These elements contain information from the End User at Job creation that describes the Job such as its name.  Automaton may modify the value of some of the elements in this group (e.g. “KOctets”) if more reliable data is obtained. The semantics of the elements are summarized in Table 4.

Figure 8 Job Description Elements

## Document Object Class

The Document object class is represented by a collection of elements divided into Three groups as shown in .  The Document class contains the document class

Document Status Elements – See Section 4.3.1.

Document Description Elements – See section 4.3.2.

Document Processing Elements – See section 4.4.2

### Document Status Elements

shows the Documene StatuseElements.  Automata primarily control the elements in this group.  End Users cannot directly modify their values.  The End User can affect the values of these elements through actions (e.g. CancelDocument can change the value of DocumentsState”). The semantics of the elements are summarized Table 5

Figure 9 Document Status Elements

#### The Document Life Cycle

The “DocumentState” element is one of the most important Document Status Elements.  Figure 10 shows the values of the “DocumentState” element and the Document life cycle as affected by Actions and job processing.  Documents are not active objects and their life cycle is closely tied to the lifecycle of a Job.  Documents basically have three states.   The first is waiting to be processed by a Job (i.e., pending).  The second state is from the time the Job first starts processing the Document (i.e., processing) and until it reaches its terminating state.  The last state for a Document is its terminal state (i.e., completed, canceled, aborted)

Figure 10  "DocumentState" Element and Document object life Cycle

### Document Description Elements

shows the Document DescriptioneElements.  These elements contain information from the End User at Document creation that describes the document such as its size.  Automaton may modify the value of some of the elements in this group (e.g. “KOctets”) if more reliable data is obtained. The semantics of the elements are summarized in Table 5

Figure 11 Document Description Elements

## Processing Elements

Processing elements are instructions to be applied to jobs and documents.  They indicate such things as the priority for scheduling a job or the number of copies for a document.  A Printer should support each Processing Element that represents a feature of the Printer.  The Processing elements are split into two groups.  One groups applies to Jobs and the other to Documents.  The Document Processing group contains three sub-groups. (See Figure 12)

- Job Processing Elements are processing instructions applied the Job level.  See section 4.4.1.

- Document Processing Elements are specific to documents.  See section 4.4.2.

Figure 12 - Processing Elements

### Job Processing Elements

Figure 13 shows the Job ProcessingeElements.  These elements apply to the job as a whole as opposed to each document in the job.  The semantics of the elements are summarized in Table 3 along with a brief description of each element.

Figure 13 Job Processing Elements

### Document Processing Elements

Document Processing Elements are elements that are applied to documents (e.g. “copies”).  The Document Processing Elements can be applied at the Job or Document level.  If the elements are applied at the Job level they are the default values for all the Documents in the Job.  If the elements are applied at the Document level they apply only to that Document.  The semantics of the Processing elements are summarized in Table 3.  The Document Processing elements are split into three groups as shown in Figure 12:

- Finishing Elements define how multiple physical sheets are manipulated to create final output products. See section 4.4.2.1.

- Imposition Elements identify how the logical pages look on the output media.  See section 4.4.2.2.

- Rendering Elements determine the quality and resolution of how marks are made on the page.  See section 4.4.2.3.

#### Finishing Elements

Figure 14 shows the FinishingeElements.  FinishingeElements define how multiple physical sheets are manipulated to create final output products.  See Table 3 for summary of element semantics.

Figure 14 Finishing Elements

#### Imposition Elements

Figure 15 shows the ImpositioneElements. ImpositioneElements identify how the logical pages look on the output media.  See Table 3 for summary of element semantics.

Figure 15 Imposition Elements

#### Rendering Elements

Figure 16 shows the RenderingeElements. RenderingeElements determine the quality and resolution of how marks are made on the page.  See Table 3 for summary of element semantics.

Figure 16 Rendering Elements

# Actions

The PWG has defined a number of operations that affect Printers, Jobs and their document.  Below is a description of the semantics of these Actions.  Naturally different protocol bindings will use differing subsets of the Actions or define new ones.  Another difference will be the precise parameters to the Actions.  Below is an abstract definition of the Actions. Action Summary

This table summarizes the actions defined for the Job and Printer.  See section 4.4.2 for more details.

| Job Creation and Document submission | Job and Document Control | Status and Information access | Printer Control |
| --- | --- | --- | --- |
| CreateJob | CancelCurrentJob | GetDocumentElements | ActivatePrinter |
| PrintJob | CancelDocument | GetDocuments | DeactivatePrinter |
| PrintUri | CancelJob | GetJobElements | DisablePrinter |
| SendDocument | DeleteDocument | GetJobs | EnablePrinter |
| SendURI | HoldJob | GetPrinterElements | HoldNewJobs |
| ValidateDocument | PromoteJob | GetPrinterSettableElementValues | PausePrinter |
| ValidateJob | ReleaseJob |  | PausePrinterAfter CurrentJob |
|  | ReprocessJob |  | PurgeJobs |
|  | RestartJob |  | ReleaseHeldNew Jobs |
|  | ResumeJob |  | RestartPrinter |
|  | ScheduleJobAfter |  | ResumePrinter |
|  | SetDocumentElements |  | SetPrinterElements |
|  | SetJobElements |  | ShutdownPrinter |
|  | SuspendCurrentJob |  | StartupPrinter |

Table 2 - Summary of Actions

## Job Creation and document submission Actions

This section describes the Job Creation actions that create a Job and the ones that create add Document to a Job.  The Job Creation actions are: PrintJob, PrintUri, and CreateJob.  The PrintJob action also submits the Document.  The PrintUri action submits a URI reference to the Document which the Printer then retrieves when needed at a later time.  The CreateJob action only creates the job and the Client must issue subsequent SendDocument and SendUri actions in order to submit document content or a URI reference, respectively, for a job.

Processing instructions and descriptive information contained in the arguments of the Job Creation action are combined with Printer supplied information to create a Job instance.

The last action in this section is ValidateJob.  This operation allows a Client to send a request with all the information to create a Job, except the document content.  The Printer does not create a Job but informs the client whether a CreateJob, PrintJob or PrintUri with the same information would have succeeded.  This is useful for allowing a Client to verify the processing instructions before sending a large PrintJob request.

A concept that is important in the PWG model is a set of instructions that can be applied to a print job.  Examples of these instructions include the number of copies and the media to use.  These instructions are referred to as Processing Elements.  The Processing Elements are made up of the Job Processing Elements (see section 4.4.1) and the Document Processing Elements (see section 4.4.2) sent in a Job or Document Creation Action.

Figure 17 Processing Instruction Processing

In the real world, processing instructions are also contained in the document content for a job.  Page Description Languages (PDL) such as PostScript® and PCL® often contain processing instructions.   Some environments use a printer specific driver to generate the PDL stream based on feature selections made through a user interface.  Given that processing instructions can occur in both the PDL and in an associated Job, the PWG model allows a Printer to declare its capability to resolve this conflict.  The Printer’s element “PdlOverride” declares if an attempt will be made to override the instructions in the PDL with the instructions in the Job.

There is a wide variety of capabilities in Printers.  An instance of a Printer is to subject to changes in its configured capabilities.  An example would be an administrative change in the media the Printer supports or disabling two-sided printing.  Clients need not check the capabilities of a Printer before creating their Job Processing Elements and submitting a job.  Since this is a client/server paradigm, it is always possible that the capabilities could change after checking a Printer’s capabilities and before a Job is submitted.  On the other hand, a client may use the Printer’s configured capabilities to create their Job Processing Elements and submit a job.

The PWG model allows a client to control the Printer’s acceptance of a job submission based on the job request and the Printer’s current configured capabilities as follows.  When the client supplies a ‘true’ value for the “ElementFidelity” Job Processing element, the Printer must reject the job unless the Printer supports all of the supplied Job Processing elements and values.  When the client supplies a ‘false’ value or omits the element, the Printer must accept the job submission and ignore or substitute elements and values, respectively, that it does not support.  Note that the “ElementFidelity” Job Processing element covers only the creation of the Job.  It is implementation specific how a Printer handles processing a job when the Printer encounters unsupported processing instructions in the document content.

### CreateJob

([rfc2911] §3.2.4)  Similar to the PrintJob operation (see section 5.1.1), except that in the CreateJob request the Client does not supply Document Data.  The client supplies a single set of Job Processing elements that the Printer applies to the Output Document(s) of the job.  The “MultipleDocumentHandling” Job Processing element controls whether the Printer produces separate Output Documents or combines the Input Documents into a single Output Document (see section 22).

### PrintJob

([rfc2911] §3.2.1)  Submit a print job with only one document and supply the document content data.  If the Printer accepts the job, it creates the Job object and returns a unique “JobId” element for the Printer and a globally unique “JobUri” element.  The Printer also sets the corresponding Job elements with these values.

### PrintUri

([rfc2911] §3.2.2)  Identical to the PrintJob operation (see section 5.1.1) except that a client supplies a URI reference to the document data.

### The “MultipleDocumentHandling” Job Processing element

When a client submits a job with more than one Input Document, the “MultipleDocumentHandling” Job element allows the client to specify whether the Printer is to (1) produce corresponding separate Output Documents or (2) combine the Input Documents into a single Output Document.  For example, the ‘single-document’ and ‘single-document-new-sheet’ values allow the client to staple all of the Input Documents into a single Output Document, with the latter value forcing each Input Document to start on a new sheet (useful when doing two-sided printing).  When requesting multiple Copies, the ‘separate-document-uncollated-Copies’ value results in the Copies of each Input Document being together in an Output set, while the ‘separate-document-collated-Copies’ value keeps a copy of each Input Document together in an Output set.  For example, a job with Input Documents A, B, C and “Copies” = 2 will result in A, A, B, B, C, C or A, B, C, A, B, C, respectively.  If the Printer supports multiple documents per job, the Printer must support this Job Processing element with at least one value.

### SendDocument

([rfc2911] §3.3.1, [doc-obj] §3)  Submits the entire Document Content for the next Input Document of a job created by a previous CreateJob action (see section 5.1.3.1).

### SendUri

([rfc2911] §3.3.2, [doc-obj] §3)  Identical to the SendDocument operation (see section 5.1.4) except that a client supplies a URI reference to the Document Content data, instead of supplying the document content.

### ValidateDocument

([doc-obj] §3)  This operation is used only to verify capabilities of a Printer object against whatever elements are supplied by the client in the ValidateDocument request. By using the ValidateDocument action a client can validate that an identical SendDocument or SendUri would be accepted.

### ValidateJob

([rfc2911] §3.2.3)  This operation is used only to verify capabilities of a Printer object against whatever elements are supplied by the client in the ValidateJob request. By using the ValidateJob action a client can validate that an identical PrintJob, PrintUri or CreateJob would be accepted.

### Job and Document Control Actions

This section describes the actions that allow a client to control a Job after it has been submitted:  CancelJob, HoldJob, ReleaseJob, and RestartJob.

### CancelCurrentJob

([admin-ops] §4.2)  Allows a client to cancel the current Job in the “processing” or “processing-stoped” state.

### CancelDocument

([doc-obj] §3)  Prevents the processing of the specified Document if the Document has not yet been processed.  Stops the processing of any active Document in an implementation specific manner.

### CancelJob

([rfc2911] §3.3.3)  Allows a client to cancel a Print Job from the time the Job is created up to the time it is completed, canceled, or aborted.

### DeleteDocument

([doc-obj] §3)  Removes the Document and its content from the Job.

### HoldJob

([rfc2911] §3.3.5)  Allows a client to hold a pending Job in the Printer so that it is not eligible for scheduling.

### PromoteJob

([admin-ops] §4.4.1)  Allows a client to make the pending target job be processed after the current job completes.

### ReleaseJob

([rfc2911] §3.3.6)  Release a previously held Job so that it is again eligible for scheduling.

### ReprocessJob

([admin-ops] §4.1)  Allows a client to re-process a copy of a job retained after processing was completed. This operation is the similar to RestartJob except that a new job that is a copy of the target job  is created and processed.

### RestartJob

([rfc2911] §3.3.7)  Restart a job that is retained in the Printer after processing has completed.

### ResumeJob

([admin-ops] §4.3.2)  Resume the job at the point where it was suspended.

### ScheduleJobAfter

([admin-ops] §4.4.2)  Request the target job be processed immediately after the specified job

### SetDocumentElements

([doc-obj] §3)  Set the values of the supplied Document Processing and Document Description elements of the indicated Document. (Was SetDocumentAttributes)

### SetJobElements

([rfc3380] §4.2)  Set the values of the supplied Job Processing, Document Processing and Job Description elements of the indicated Job. (Was SetJobAttributes)

### SuspendCurrentJob

([admin-ops] §4.4.2)  Stop the current job and allow other jobs to be processed instead.

### Status and information Actions

This section describes the actions that allow a client to obtain status and elements of Jobs and Printers:  GetJobs, GetPrinterElements, GetJobElements and GetPrinterSupportedValues.

### GetDocumentElements

([doc-obj] §3)  Returns the requested Document elements or element groups in the indicated Document in the indicated Job. (Was GetDocumentAttributes)

### GetDocuments

([doc-obj] §3)  Returns the requested Document elements or element groups in all Documents in the indicated Job.

### GetJobElements

([rfc2911] §3.3.4)  Returns the values of the requested job elements and/or element groups of a Job (i.e., Job Description, Job Status, Job Processing and Document Processing).  (Was GetJobAttributes)

### GetJobs

([rfc2911] §3.3.4)  Retrieve the list of Jobs belonging to the Printer. The Client may supply some simple filters (e.g. “MyJobs, “Limit) to control which jobs will be returned. The Client may supply a list of Job element and/or element group names to be returned in the response (See ). A group of Job elements will be returned for each returned Job.

### GetPrinterElements

([rfc2911] §3.2.5)  Returns the values of the requested printer elements and/or element groups of a Printer (i.e. Printer Status, Printer Description, Processing Supported, Processing Default, Processing Ready).  (Was GetPrinterAttributes)

### GetPrinterSettableElementValues

([rfc3380] §4.3)  Returns the possible values of each of the requested Printer Processing and Printer Description elements that may be set with the SetPrinterElements action. (Was GetPrinterSupportedValues) (Was GetPrinterSettableAttributeValues)

### Printer Control Actions

This section describes actions which allow a client to control a Printer and may require operator credentials:  PausePrinter, ResumePrinter, PurgeJobs , DisablePrinter, EnablePrinter, and SetPrinterElements.

### ActivatePrinter

([admin-ops] §3.4.2)  The Printer will now start sending jobs to its Output Devices or Subordinate Printers and begin accepting all requests.

### DeactivatePrinter

([admin-ops] §3.4.1)  The Printer will now stop sending any more jobs to its Output Devices or Subordinate Printers and begin refusing all requests except ActivatePrinter, SendDocument, and SendUri requests and query requests.

### DisablePrinter

([adm-ops] §3.1.1)  Prevents the Printer from accepting any more Job Creation operations.  The Printer sets the PrinterIsAcceptingJobs Printer Status element to ‘false’.

### EnablePrinter

([adm-ops] §3.1.2)  Allows the Printer to start accepting Job Creation operations.  The Printer sets the PrinterIsAcceptingJobs Printer Status element to ‘true’.

### HoldNewJobs

([admin-ops] §3.3.1)  Complete the current 'pending' and  'processing' Jobs but do not start processing any subsequently created Jobs.

### PausePrinter

([rfc2911] §3.2.7)  Stops the Printer object from scheduling jobs.  Job processing should also cease.

### PausePrinterAfterCurrentJob

([admin-ops] §3.2.1)  Stops the Printer from starting to send jobs to any of its Output Devices or Subordinate Printers.

### PurgeJobs

([rfc2911] §3.2.9)  Removes all jobs from the Printer, regardless of their state.

### ReleaseHeldNewJobs

([admin-ops] §3.3.2)  Undo the effect of HoldNewJobs and release all Jobs held as a consequence of HoldNewJobs.

### RestartPrinter

([admin-ops] §3.5.1)  This action has the effect of a software re-boot.

### ResumePrinter

([rfc2911] §3.2.8)  Resume the processing and scheduling of Jobs in the Printer.

### SetPrinterElements

([rfc3380] §4.1)  Set the values of the supplied Printer Processing and Printer Description elements. (Was SetPrinterAttributes)

### ShutdownPrinter

([admin-ops] §3.5.2)  Stop processing jobs without losing any jobs and make the Printer no longer available for any Actions.

### StartupPrinter

([admin-ops] §3.5.3)  Allows a hosted implementation of the Printer to be started after the host is available.

# Globalization

The two aspects of globalization being addressed are the character sets and natural language of the human readable strings.  Determining what character set is being used is left up to the protocol mapping of this semantic model.  The natural language being used is represented in the Printer and the Job.  The Printer declares the natural language it uses for all its semantic elements of type string.  Administrators are free to change the localization and the values in the string elements.  Each job creator declares the natural language for the Job and all its contained Documents.  Not all string elements are treated the same.

Any semantic element that is labeled type1, type2 or type3 keyword in the constraint column is the following tables do not have any globalization issues from the Printer’s point of view.  They are simply a sequence of octets that have a semantic meaning attached to them.  The fact that the sequence of octets can be interpreted as ASCII strings is unimportant.  The keywords are intended for consumption by automata.  We leave it to Client implementations to determine how the keywords will be presented to end-users.

There are also strings with specific formats.  These formats are URI, URI Scheme, MIME, IEEE 1284 and DateTime.  Any semantic element whose string value must adhere to one of the previous formats is excluded from this discussion.

There are a few elements whose value is set by automata.  Those values are “JobStateMessage”, “DocumentStateMessage” and “PrinterStateMessage”.  If the semantic model is mapped to a protocol that allows the Client to request a language, the Printer will return these strings in the requested language if possible.

All the remaining Printer element strings are assumed to be in the Printer’s language.  All the remaining Job element strings are assumed to be in the language of the Job.

# Summary of elements

This section summarizes the elements for the Document, Job and Printer objects.  Included in the definition are the processing elements that can be applied at either the Job or Document level.    For each element, the tables contain the element name, whether the element is multi-valued, its syntax, constraints, a short description and a reference to the Document where the semantics of the element is completely specified.  The basic syntax types are “Boolean”, “String” and “Integer”.  “Complex” types are a container for elements of any type.  Members are listed in the description field.    “RangeOfInteger” is a complex type that contains “Upperbound” and “Lowerbound” integer value members.  “Resolution” is a complex type that contains “CrossFeedDir” and “FeedDir” integer value members and a “Units” string value member.

## Processing Elements (Job and Document)

* Group key: J=Job Processing Elements, D=Document Processing Elements

Table 3 - Processing Elements (Job and Document)

| Element Name |  | Multivalued |  |  |  |  |  |  |  | Syntax |  |  |  |  | Constraint |  |  | Group* |  |  |  |  |  | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Description (values) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ElementFidelity |  |  |  |  |  |  |  |  |  |  | Boolean |  |  |  |  |  |  |  | J |  |  |  |  | [rfc2911] §15.1 |
|  | Allows a user to control whether or not the Printer MUST honor all supplied Processing elements in the Job Creation operation. For a ‘true’ value the Printer rejects the job submission if any of the supplied Processing element values are unsupported. For a ‘false’ value the Printer MUST accept the job submission and do best effort. Default = ‘false’ NOTE: Use “JobMandatoryElements” to explicitly specify a subset of the supplied elements that the Printer MUST honor. (Was IPPAttributeFidelity) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Copies |  |  |  |  |  |  |  |  |  | Integer |  |  |  |  | 1:MAX |  |  | D |  |  |  |  |  | [rfc2911] §4.2.5 |
|  | The number of copies of the Output Document(s) to be printed. (See also JobCopies Job element) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CoverBack |  |  |  |  |  |  |  |  |  | complex |  |  |  |  |  |  |  | D |  |  |  |  |  | [PWG5100.3] §3.1 |
|  | The back cover to apply this Document. (Includes Media/MediaCol, CoverType) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CoverFront |  |  |  |  |  |  |  |  |  | complex |  |  |  |  |  |  |  | D |  |  |  |  |  | [PWG5100.3] §3.1 |
|  | The front cover to apply to this Document. (Includes Media/MediaCol, CoverType) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CoverType |  |  |  |  |  |  |  |  | String |  |  |  | Type2 keyword |  |  |  |  | D |  |  |  |  | [PWG5100.3] §3.1.2 |  |
|  | Indicates if covers are requested and which sides will contain print stream pages. (Keywords: no-cover, print-none, print-front, print-back, print-both) (See CoverBack & CoverFront for use) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DocumentCopies |  | Yes |  |  |  |  |  | RangeOfInteger |  |  |  |  |  |  |  |  |  | J |  |  |  |  | [PWG5100.4] §5.1.3 |  |
|  | Specifies which copies of an Output Document to apply these document override elements. (See DocumentOverrides for use) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DocumentOverrides |  |  |  |  | Yes |  |  |  |  | complex |  |  |  |  |  |  |  | J |  |  |  |  |  | [PWG5100.4] §5.1 |
|  | Provides for the overriding of processing instructions on a document basis. Applied to job, see PageOverrides for overrides supplied at the document level. (Includes InputDocuments/OutputDocuments, DocumentCopies, DocumentFormat, DocumentName, Compression, DocumentNaturalLanguage, PageRanges, and any other processing element that affects documents) NOTE: Deprecated in favor of supporting and using the Document Object |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| FeedOrientation |  |  |  |  |  |  |  |  |  | String |  |  |  |  | Type3 keyword |  |  |  |  | D |  |  |  | [prod-print2] §5.1 |
|  | Specifies the media edge which is fed into the print engine from the paper tray. (Keywords: long-edge-first, short-edge-first). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Finishings |  |  |  |  | Yes |  |  |  |  | String |  |  |  |  | Type2 keyword |  |  |  |  | D |  |  |  | [rfc2911] §4.2.6 [PWG5100.1] §2 |
|  | Identifies the finishings that the Printer uses for each copy of the Output Document. (See also JobFinishings Job element) (Keywords: bale, bind, bind-bottom, bind-left, bind-right, bind-top, booklet-maker, cover, edge-stitch, edge-stitch-bottom, edge-stitch-left, edge-stitch-right, edge-stitch-top, fold, jog-offset, none, punch, saddle-stitch, staple, staple-bottom-left, staple-bottom-right, staple-dual-bottom, staple-dual-left, staple-dual-right, staple-dual-top, staple-top-left, staple-top-right, trim) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| FinishingsCol |  |  |  |  |  |  |  |  |  | complex |  |  |  |  |  |  |  | D |  |  |  |  |  | [PWG5100.3] §3.2 |
|  | Enables an end user to specify detailed finishing options not possible with the “Finishings” element for the Output Document. (See also JobFinishingsCol Job element) (Includes FinishingTemplate, Stitching) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| FinishingTemplate |  |  |  |  |  |  |  |  |  | String |  |  | Maxlength=1023 |  |  |  |  |  |  | D |  |  |  | [PWG5100.3] §3.1 |
|  | A string specifying some particular finishing operation. (See FinishingsCol/JobFinishingsCol for use) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| FontNameRequested |  |  |  |  |  |  |  |  |  | String |  |  | Maxlength=255 |  |  |  |  | D |  |  |  | [prod-print2] §5.2 |  |  |
|  | Specifies the font name if the document data is in a format that does not have inherent font information (e.g., ‘text/plain’), otherwise, this element is ignored. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| FontSizeRequested |  |  |  |  |  |  |  |  |  | Integer |  |  |  | 1:MAX |  |  |  | D |  |  |  | [prod-print2] §5.3 |  |  |
|  | Specifies the font size in points (1/72 of an inch) if the document data is in a format that does not have inherent font information (e.g., ‘text/plain’), otherwise, this element is ignored. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ForceFrontSide |  |  |  |  | yes |  |  |  |  | Integer |  |  |  |  |  |  |  | D |  |  |  | [PWG5100.3] §3.3 |  |  |
|  | Forces the specified pages to be printed on the front side of a sheet of media. The pages of the output document start at 1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ImpositionTemplate |  |  |  |  |  |  |  |  |  | String |  |  | Type2 keyword |  |  |  |  |  | D |  |  |  | [PWG5100.3] §3.4 |  |
|  | Specifies imposition method for laying out finished page images onto the surface of output media. (Keywords: none, signature) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| InputDocuments |  | Yes |  |  |  |  |  | RangeOfInteger |  |  |  |  |  |  |  |  |  | D |  |  |  |  | [PWG5100.4] §5.1.1 |  |
|  | Specifies the input documents for override processing. (See DocumentOverrides for use) NOTE: Deprecated since DocumentOverrides are deprecated |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| InsertAfterPageNumber |  |  |  |  |  |  |  |  |  | Integer |  |  |  |  |  |  |  | D |  |  |  |  | [PWG5100.3] §3.5.1 |  |
|  | Specifies the input page after which the Insert Sheet will be placed. (See InsertSheet for use) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| InsertCount |  |  |  |  |  |  |  |  |  | Integer |  |  |  |  |  |  |  | D |  |  |  |  | [PWG5100.3] §3.5.2 |  |
|  | Specifies the number of Insert Sheet to insert. (See InsertSheet for use) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| InsertSheet |  |  |  |  | Yes |  |  |  |  | complex |  |  |  |  |  |  |  | D |  |  |  |  | [PWG5100.3] §3.5 |  |
|  | Specifies how Insert Sheets are to be inserted into the sequence of media sheets that are produced for each copy of the documents. (Includes InsertAfterPageNumber, InsertCount, Media/MediaCol) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| JobAccountingOutputBin |  |  |  |  |  |  |  |  |  | String |  |  | Type3 keyword |  |  |  |  | J |  |  |  |  | [PWG5100.3] §3.8.3 |  |
|  | Specifies the output bin where the accounting sheet is to be placed. (See JobAccountingSheet for use) (Keywords: top, middle, bottom, side, left, right, center, rear, face-up, face-down large-capacity, my-mailbox, stacker-N, mailbox-N, tray-N *Note: N is replaced by a cardinal number, *Note: See [PWG5100.2 §2.1 for description of keywords) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| JobAccountingSheet |  |  |  |  |  |  |  |  |  | complex |  |  |  |  |  |  |  | J |  |  |  |  |  | [PWG5100.3] §3.8 |
|  | Specifies the accounting sheet for a job. (Includes JobAccountingSheetType, Media/ MediaCol, JobAccountingOutputBin). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| JobAccountingSheetType |  |  |  |  |  |  |  |  |  | String |  |  | Type3 keyword |  |  |  |  | J |  |  |  |  | [PWG5100.3] §3.8.1 |  |
|  | Specifies the accounting sheet format for a job. (See JobAccountingSheet for use) (Keywords: none, standard) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| JobCopies |  |  |  |  |  |  |  |  |  | Integer |  |  |  |  | 1:MAX |  |  | J |  |  |  |  |  | [rfc2911] §4.2.5 [doc-obj] |
|  | The number of copies of the Job to be printed. (See also Copies Document Processing element) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| JobCoverBack |  |  |  |  |  |  |  |  |  | complex |  |  |  |  |  |  |  | D |  |  |  |  |  | [PWG5100.3] §3.1 [doc-obj] |
|  | The back cover to apply this Job. (Includes Media/MediaCol, CoverType) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| JobCoverFront |  |  |  |  |  |  |  |  |  | complex |  |  |  |  |  |  |  | D |  |  |  |  |  | [PWG5100.3] §3.1 [doc-obj] |
|  | The front cover to apply to this Job. (Includes Media/MediaCol, CoverType) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| JobErrorSheet |  |  |  |  |  |  |  |  |  | complex |  |  |  |  |  |  |  | J |  |  |  |  |  | [PWG5100.3] §3.9 |
|  | Specifies the error sheet for a job. (Includes JobErrorSheetType, JobErrorSheetWhen, Media/MediaCol). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| JobErrorSheetType |  |  |  |  |  |  |  |  |  | String |  |  | Type3 keyword |  |  |  |  | J |  |  |  |  | [PWG5100.3] §3.9.1 |  |
|  | Specifies the error sheet format for a job. (See JobErrorSheet for use) (Keywords: none, standard) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| JobErrorSheetWhen |  |  |  |  |  |  |  |  |  | String |  |  | Type2 keyword |  |  |  |  | J |  |  |  |  | [PWG5100.3] §3.9.2 |  |
|  | Specifies the accounting sheet format for a job. (See JobErrorSheet for use) (Keywords: on-error, always) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| JobFinishings |  |  |  |  | Yes |  |  |  |  | String |  |  |  |  | Type2 keyword |  |  |  |  | J |  |  |  | [rfc2911] §4.2.6 [doc-obj] |
|  | Identifies the finishing that the Printer uses for each job copy of the Job. (See also Finishings Document element) (Keywords: none, staple, punch, cover, bind, saddle-stitch, edge-stitch, staple-top-left, staple-bottom-left, staple-top-right, staple-bottom-right, edge-stitch-left, edge-stitch-top, edge-stitch-right, edge-stitch-bottom, staple-dual-left, staple-dual-top, staple-dual-right, staple-dual-bottom) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| JobFinishingCol |  |  |  |  |  |  |  |  |  | complex |  |  |  |  |  |  |  | J |  |  |  |  |  | [PWG5100.3] §3.2 [doc-obj] |
|  | Enables an end user to specify detailed finishing options not possible with the “JobFinishings” element. . (See also FinishingsCol Document element) (Includes FinishingTemplate, Stitching) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| JobHoldUntil |  |  |  |  |  |  |  |  |  | String |  |  | Type3 keyword |  |  |  |  | J |  |  |  |  |  | [rfc2911] §4.2.2 |
|  | Specifies the named time period during which the Job must become a candidate for printing. (keywords: no-hold, indefinite, day-time, evening, night, weekend, second-shift, third-shift) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| JobHoldUntilTime |  |  |  |  |  |  |  |  |  | String |  |  | DateTime [rfc1123] |  |  |  |  |  |  | J |  |  |  | [prod-print2] §5.4 |
|  | Specifies the date and time after which the Job must become a candidate for printing. (example: Fri, 03 May 2002 08:49:37 GMT) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| JobMandatoryElements |  | Yes |  |  |  |  |  |  |  | String |  |  | Type3 keyword |  |  |  |  | J |  |  |  |  |  | Need reference |
|  | Allows a user to list which Processing elements the Printer must honor. The Printer rejects the job submission if any of the listed elements are unsupported or contain values that the Printer does not support. All of the remaining supplied elements are best effort. This element is ignored if ElementFidelity is supplied with a ‘true’ value. (See [rfc2911] §15.1) (Keywords: none and any Processing element names. Member elements of collection elements are named as Attr.Member. For example, JobSheetsCol.Media) NOTE: New element to align fidelity with FSG work was JobMandatoryAttributes). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| JobPriority |  |  |  |  |  |  |  |  |  | Integer |  |  |  |  | 1:100 |  |  | J |  |  |  |  |  | [rfc2911] §4.2.1 |
|  | Priority for scheduling the Job. A higher value specifies a higher priority. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| JobSaveDisposition |  |  |  |  |  |  |  |  |  | Complex |  |  |  |  |  |  |  | J |  |  |  |  |  | [prod-print2] §5.7 |
|  | Specifies that the Printer is to save the job as a file that can be re-printed on demand anytime in the future using the Print-URI operation (see section 5.1.3). ) (Includes SaveDisposition, SaveInfo) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| JobSheets |  |  |  |  |  |  |  |  |  | String |  |  | type3 keyword |  |  |  |  | J |  |  |  |  |  | [rfc2911] §4.2.3 [PWG5100.3] §6.2 |
|  | Specifies which job start/end sheet(s), will be printed with a job. (Keywords: none, standard, job-start-sheet, job-end-sheet, job-both-sheets, first-print-stream-page) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| JobSheetsCol |  |  |  |  |  |  |  |  |  | complex |  |  |  |  |  |  |  | J |  |  |  |  | [PWG5100.3] §3.11 |  |
|  | Allows the client to specify the media for the JobSheet. (Includes JobSheets, Media/MediaCol) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| JobSheetMessage |  |  |  |  |  |  |  |  |  | String |  |  | Maxlength=1023 |  |  |  |  |  | J |  |  |  | [PWG5100.3] §3.12 |  |
|  | Conveys a message that is delivered with the job. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Media |  |  |  |  |  |  |  |  |  | String |  |  | type3 keyword |  |  |  |  | D |  |  |  |  |  | [rfc2911] §4.2.11 |
|  | The name of the medium that the Printer uses for all impressions of the Job. (Keyword examples: na_letter_8.5x11in, iso_a4_210x297mm, na_monarch_3.875x7.5in. See [pwg5101.1]) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MediaCol |  |  |  |  |  |  |  |  |  | complex |  |  |  |  |  |  |  | D |  |  |  |  | [PWG5100.3] §3.13 |  |
|  | Enables a client end user to submit a list of media characteristics to the Printer as a way to more completely specify the media to be used than the Media element. (Includes MediaBackCoating, MediaColor, MediaFrontCoating, MediaGrain , MediaHoleCount, MediaInfo, MediaKey, MediaMaterial, MediaOrderCount, MediaPrePrinted, MediaRecycled, MediaSize, MediaThickness, MediaTooth, MediaType, MediaWeightMetric) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MediaBackCoating |  |  |  |  |  |  |  | String |  |  |  | Type3 keyword |  |  |  |  | D |  |  |  | [PWG5100.3] §3.13.10 |  |  |  |
|  | Indicates the pre-process coating applied to the back of the media. (See MediaCol for use) (Keywords: none, glossy, high-gloss, semi-gloss, satin, matte) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MediaColor |  |  |  |  |  |  |  | String |  |  |  | Type3 keyword |  |  |  |  |  | D |  |  |  | [PWG5100.3] §3.13.4 |  |  |
|  | Indicates the desired color of the media being specified. (See MediaCol for use) (Keywords: no-color, white, pink, yellow, blue, green, buff, goldenrod, red, gray, ivory, orange) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MediaFrontCoating |  |  |  |  |  |  |  | String |  |  |  | Type3 keyword |  |  |  |  |  | D |  |  | [PWG5100.3] §3.13.10 |  |  |  |
|  | Indicates the pre-process coating applied to the front of the media. (See MediaCol for use) (Keywords: none, glossy, high-gloss, semi-gloss, satin, matte) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MediaGrain |  |  |  |  |  |  | String |  |  |  |  | Type3 keyword |  |  |  |  |  |  | D |  |  | [prod-print2] §8.4.2 |  |  |
|  | Indicates the grain of the media. (See MediaCol for use) (Keywords: x-direction, y-direction) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MediaHoleCount |  |  |  |  |  |  |  | Integer |  |  |  |  |  |  |  |  |  | D |  |  |  | [PWG5100.3] §3.13.6 |  |  |
|  | Indicates the number of pre-drilled holes in the desired media. (See MediaCol for use) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MediaInfo |  |  |  |  |  |  |  | String |  |  |  |  | Maxlength=255 |  |  |  |  | D |  |  |  | [PWG5100.3] §3.13.3 |  |  |
|  | Specifies information that helps describe the media instance. Intended for human consumption. (See MediaCol for use) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MediaInputTrayCheck |  |  |  |  |  |  |  | String |  |  |  |  | Type3 keyword |  |  |  |  | D |  |  | [PWG5100.3] §3.13.14 |  |  |  |
|  | Indicates that the characteristics of the media in the identified input tray must match the characteristics of the media identified by the "media" or "media-col" element. (Keywords: top, middle, bottom, side, large-capacity, envelope, main, manual. See [RFC2911] Appendix C) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MediaKey |  |  |  |  |  |  |  |  |  | String |  |  | Type3 keyword |  |  |  |  | D |  |  |  | [PWG5100.3] §3.13.1 |  |  |
|  | The name of the media represented as a keyword or name. Values are the same as the keyword and name values for the Media Document Processing element and represent the same media, except for media size and input tray keywords. (See MediaCol for use) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MediaMaterial |  |  |  |  |  |  |  |  |  | String |  |  | Type3 keyword |  |  |  |  | D |  |  |  | [prod-print] §8.4.3 |  |  |
|  | The material of the media. (See MediaCol for use) (Keywords: aluminum, dry-film, paper, polyester, wet-film) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MediaOrderCount |  |  |  |  |  |  |  |  |  | Integer |  |  |  |  | 1:MAX |  |  | D |  |  |  | [PWG5100.3] §3.13.7 |  |  |
|  | Indicates the number of sheets, within an ordered sequence of sheets; after which the sequence begins to repeat. (See MediaCol for use) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MediaPrePrinted |  |  |  |  |  |  |  |  |  | String |  |  | Type3 keyword |  |  |  |  | D |  |  | [PWG5100.3] §3.13.11 |  |  |  |
|  | Indicates the pre-printed characteristics of the desired media. (See MediaCol for use) (Keywords: blank, pre-printed, letter-head) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MediaRecycled |  |  |  |  |  |  |  |  |  | String |  |  | Type3 keyword |  |  |  |  | D |  |  | [PWG5100.3] §3.13.10 |  |  |  |
|  | Indicates the recycled characteristics of the media. (See MediaCol for use) (Keywords: none, standard) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MediaSize |  |  |  |  |  |  |  |  |  | Complex |  |  |  |  |  |  |  | D |  |  |  | [PWG5100.3] §3.13.8 |  |  |
|  | Explicitly specifies the numerical media width and height dimensions. (See MediaCol for use) (Includes XDimension, YDimension) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MediaSizeName |  |  |  |  |  |  |  |  |  | String |  |  | Type3 keyword |  |  |  |  | D |  |  |  |  |  | Need UPnP ref |
|  | The medium size that the Printer uses for all impressions of the Job. (Keywords: na_letter_8.5x11in. See [pwg5101.1] §5) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MediaThickness |  |  |  |  |  |  |  |  |  | Integer |  |  | 1:MAX |  |  |  |  | D |  |  |  |  | [prod-print2] §8.4.4 |  |
|  | The thickness of the media in units of one hundredth of a millimeter. This unit is equivalent to 1/2540 th of an inch. (See MediaCol for use) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MediaTooth |  |  |  |  |  |  |  |  |  | String |  |  | Type3 keyword |  |  |  |  | D |  |  |  |  | [prod-print2] §8.4.1 |  |
|  | The tooth (or roughness) of the media. (See MediaCol for use) (Keywords: fine, medium, coarse) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MediaType |  |  |  |  |  |  |  |  |  | String |  |  | Type3 keyword |  |  |  |  | D |  |  |  | [PWG5100.3] §3.13.2 |  |  |
|  | The medium type that the Printer uses for all impressions of the Job. (See MediaCol for use) (Keywords: stationery, transparency envelope, envelope-plain, envelope-window, continuous, continuous-long, continuous-short, tab-stock, pre-cut-tabs, full-cut-tabs, multi-part-forms, labels, multi-layer, screen, screen-paged, photographic, cardstock, other See also [pwg5101.1] §3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MediaWeightMetric |  |  |  |  |  |  |  |  |  | Integer |  |  |  |  |  |  |  | D |  |  |  | [PWG5100.3] §3.13.9 |  |  |
|  | Indicates the weight of the desired media rounded to the nearest whole number of grams per square meter. (See MediaCol for use) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MultipleDocumentHandling |  |  |  |  |  |  |  |  |  | String |  |  | type2 keyword |  |  |  |  | J |  |  |  |  |  | [rfc2911] §4.2.4 |
|  | Controls whether Input Document in multi-Document jobs are combined into a single Output Document or are kept as separate Output Document Useful for application of Finishings and the placement of one or more print-stream pages into impressions and onto media sheets for multi-Document Jobs. (Keywords: single-Document, separate-Document-uncollated-Copies, separate-Document-collated-Copies, single-Document-new-sheet) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NumberUp |  |  |  |  |  |  |  |  |  | Integer |  |  |  |  | 1:MAX |  |  | D |  |  |  |  |  | [rfc2911] §4.2.9 |
|  | Indicates the number of Input pages that the Printer is to image on one impression. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OrientationRequested |  |  |  |  |  |  |  |  |  | String |  |  | type2 keyword |  |  |  |  | D |  |  |  |  |  | [rfc2911] §4.2.10 |
|  | The desired orientation for printed pages for document formats that don’t have a built-in orientation. (Keywords: portrait, landscape, reverse-landscape, reverse-portrait) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OutputBin |  |  |  |  |  |  |  |  |  | String |  |  | Type2 keyword |  |  |  |  | J |  |  |  |  | [PWG5100.2] §2.1 |  |
|  | Specifies the output bin where the job is to be delivered. (Keywords: bottom, center, face-down, face-up, large-capacity, left, mailbox-N*, middle, my-mailbox, rear, right, side, stacker-N*, top, tray-N*. *Note: N is replaced by a cardinal number) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OutputDocuments |  | Yes |  |  |  |  |  |  | RangeOfInteger |  |  |  |  |  |  |  |  | D |  |  |  |  | [PWG5100.4] §5.1.2 |  |
|  | Specifies the output documents for override processing. (See DocumentOverrides for use) NOTE: Deprecated DocumentOverrides are deprecated. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PageDelivery |  |  |  |  |  |  |  |  |  | String |  |  | Type2 keyword |  |  |  |  | D |  |  |  |  | [PWG5100.3] §3.15 |  |
|  | Indicates whether the pages of the job are to be delivered to the output bin or finisher in the same page order as the original document and face up or face down. . See the PageOrderReceived Document Description element and the CurrentPageOrder Document Status element. (Keywords: reverse-order-face-down, reverse-order-face-up, same-order-face-down, same-order-face-up, system-specified) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PageOverrides |  |  |  |  | Yes |  |  |  |  | complex |  |  |  |  |  |  |  | D |  |  |  |  |  | [PWG5100.4] §5.2 |
|  | Provides for the overriding of processing instructions on a page basis. (Includes InputDocuments/OutputDocuments, DocumentCopies, Pages, Sides, media and any other processing element that affects pages) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Pages |  | yes |  |  |  |  |  | RangeOfInteger |  |  |  |  |  |  |  |  |  | D |  |  |  |  | [PWG5100.4] §5.2.4 |  |
|  | Specifies a range of pages in the document data. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PagesPerSubset |  | yes |  |  |  |  |  | Integer |  |  |  |  |  |  |  |  |  | D |  |  |  |  | [PWG5100.4] §5.3 |  |
|  | Combines all of the Input Pages of all of the Input Documents into a single stream of Input-Pages. Then the Printer partitions that single stream into contiguous subsets of Input-Pages according to the list of integers. Each subset is defined to be an Output-Document. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PageRanges |  | yes |  |  |  |  |  | RangeOfInteger |  |  |  |  |  |  |  |  |  | D |  |  |  |  | [RFC2911] §4.2.7 |  |
|  | Specifies a range of pages in the document data to be output. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PdlInitFile |  | Yes |  |  |  |  |  |  |  | Complex |  |  |  |  |  |  |  |  |  | D |  |  |  | [prod-print2] §5.8 |
|  | Controls initialization of the Printer’s Page Description Language (PDL) interpreter. (Includes PdlInitFileEntry, PdlInitFileLocation. PdlInitFileName) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PdlInitFileEntry |  |  |  |  |  |  |  |  |  | String |  |  |  | Maxlength=255 |  |  |  |  |  | D |  | [prod-print2] §5.8.1.3 |  |  |
|  | Specifies an entry point within the init file at which the PDL interpreter starts. (See PdlInitFile for use) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PdlInitFileLocation |  |  |  |  |  |  |  |  |  | String |  |  | Maxlength=1023 |  |  |  |  |  | D |  |  | [prod-print2] §5.8.1.1 |  |  |
|  | Contains a URL that specifies the path to the directory where the initialization file for the Printer’s PDL interpreter will be found. (See PdlInitFile for use) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PdlInitFileName |  |  |  |  |  |  |  |  |  | String |  |  |  | Maxlength=255 |  |  |  |  | D |  |  | [prod-print2] §5.8.1.2 |  |  |
|  | Specifies the name of the PDL interpreter’s initialization file within the directory specified by the PdlInitFileLocation element. (See PdlInitFile for use) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PresentationDirectionNumberUp |  |  |  |  |  |  |  |  |  | String |  |  | Type2 keyword |  |  |  |  | D |  |  |  |  | [PWG5100.3] §3.17 |  |
|  | Specifies the placement order of the page images on a Finished-Page Image with the "number-up" element. (Keywords: toright-tobottom, tobottom-toright, toleft-tobottom, tobottom-toleft, toright-totop, totop-toright, toleft-totop) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PrintQuality |  |  |  |  |  |  |  |  |  | String |  |  | type2 keyword |  |  |  |  | D |  |  |  |  |  |  |
|  | The print quality that the Printer uses for the Job. (Keywords: draft, normal, high) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PrinterResolution |  |  |  |  |  |  |  |  |  | resolution |  |  |  |  |  |  |  | D |  |  |  |  |  | RFC2911] §4.2.12 |
|  | The resolution that Printer uses for the Job in cross-feed and feed direction in units of dpi or dpcm. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ProofPrint |  |  |  |  |  |  |  |  |  | Complex |  |  |  |  |  |  |  |  |  | J |  |  | [prod-print2] §5.9 |  |
|  | Specifies the elements for zero or more proof prints of the job that are to be printed prior to the printing the full run of the job. (Includes ProofPrintCopies , Media/MediaCol and any other Processing elements). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ProofPrintCopies |  |  |  |  |  |  |  |  |  | Integer |  |  |  | 0:MAX |  |  |  |  |  | J |  |  | [prod-print2] §5.9.1 |  |
|  | Specifies the number of proof prints to be printed prior to the printing the full run of the job. (See ProofPrint for use) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SaveDisposition |  |  |  |  |  |  |  |  |  | String |  |  |  |  | type3 keyword |  |  | J |  |  |  |  | [prod-print2] §5.7.1.1 |  |
|  | Specifies whether the Printer must print and/or save the job. (See JobSaveDisposition for use) (Keywords: none, save-only, print-save) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SaveDocumentFormat |  |  |  |  |  |  |  | String |  |  |  | MimeMediaType [rfc2046], [rfc2048] |  |  |  |  | J |  |  |  | [prod-print2] §5.7.1.2.3.3 |  |  |  |
|  | Indicates the document format in which the Printer saves the Document Data. (See DocumentFormat Document Description element) (See SaveInfo for use) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SaveInfo |  | Yes |  |  |  |  |  |  |  | complex |  |  |  |  |  |  |  | J |  |  |  |  | [prod-print2] §5.7.1.2 |  |
|  | Contains sets of elements that each tell the Printer how to create each copy of the saved job. (See JobSaveDisposition for use) (Includes SaveLocation, SaveName, SaveDocumentFormat) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SaveLocation |  |  |  |  |  |  |  |  |  | String |  |  | Maxlength=1023 |  |  |  |  | J |  |  |  |  |  | [prod-print2] §5.7.1.2.3.1 |
|  | Specifies the path to the directory as a URI where the Printer saves the Document Data and other Job information. (See SaveInfo for use) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SaveName |  |  |  |  |  |  |  |  |  | String |  |  |  |  | Maxlength=255 |  |  | J |  |  |  |  | [prod-print2] §5.7.1.2.3.2 |  |
|  | Specifies the name of the saved job in the directory specified by the “save-location” member element. The value may be a relative path. (See SaveInfo for use) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SeparatorSheets |  |  |  |  |  |  |  |  |  | complex |  |  |  |  |  |  |  | D |  |  |  |  | [PWG5100.3] §3.18 |  |
|  | Specifies the separator sheets to be printed with the Document. (Includes SeparatorSheestType, Media/MediaCol) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SeparatorSheetsType |  |  |  |  |  |  |  |  |  | String |  |  | Type3 keyword |  |  |  |  | D |  |  |  | [PWG5100.3] §3.18.1 |  |  |
|  | Specifies the separator sheets type. (See SeparatorSheets for use) (Keywords: none, slip-sheets, start-sheet, end-sheet, both-sheets) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SheetCollate |  |  |  |  |  |  |  |  |  | String |  |  | Type2 keyword |  |  |  |  | D |  |  |  | [rfc3381] §3.1 |  |  |
|  | Specifies if the media sheets of each copy of each printed document in a job are to be in sequence. (Keywords: uncollated, collated) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sides |  |  |  |  |  |  |  |  |  | String |  |  | type2 keyword |  |  |  |  | D |  |  |  |  |  | [rfc2911] §4.2.8 |
|  | Indicates how an impression is to be placed upon the side(s) of the media. (Keywords: one-sided, two-sided-long-edge, two-sided-short-edge, two-sided-long-edge, tumble) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Stitching |  |  |  |  |  |  |  |  |  | complex |  |  |  |  |  |  |  | D |  |  |  |  | [PWG5100.3] §3.2.2 |  |
|  | Provides detailed stitching parameters. (See FinishingsCol/JobFinishingsCol for use) (Includes StitchingReferenceEdge, StitchingOffset, StitchingLocations) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| StitchingLocations |  |  |  |  | yes |  |  |  |  | Integer |  |  |  |  |  |  |  | D |  |  |  | [PWG5100.3] §3.2.2.3 |  |  |
|  | The distance along the stitching axis where a stitch will be placed in hundredths of a millimeter. (See Stitching for use) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| StitchingOffset |  |  |  |  |  |  |  |  |  | Integer |  |  |  |  |  |  |  | D |  |  |  | [PWG5100.3] §3.2.2.2 |  |  |
|  | The perpendicular distance from the reference edge to the stitching axis in hundredths of a millimeter. (See Stitching for use) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| StitchingReferenceEdge |  |  |  |  |  |  |  |  |  | String |  |  | type2 keyword |  |  |  |  | D |  |  |  | [PWG5100.3] §3.2.2.1 |  |  |
|  | Specifies the stitching reference edge of the output media. (See Stitching for use) (Keyword: bottom, top, left, right) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| XDimension |  |  |  |  |  |  |  |  |  | Integer |  |  |  |  | 0:MAX |  |  | D |  |  | [PWG5100.3] §3.13.8.1 |  |  |  |
|  | Size of the media in hundredths of a millimeter along the bottom edge. (See MediaSize for use) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| XImagePosition |  |  |  |  |  |  |  |  |  | String |  |  | type2 keyword |  |  |  |  | D |  |  |  | [PWG5100.3] §3.19.2 |  |  |
|  | Causes the specified point of the Finished-Page Image to be positioned at a specified location. (Keywords: none, center, left, right) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| XImageShift |  |  |  |  |  |  |  |  |  | Integer |  |  |  |  |  |  |  | D |  |  |  | [PWG5100.3] §3.19.3 |  |  |
|  | Causes the Finished-Page Image to be shifted in position with respect to the x-axis of the media. The unit of measure for this element is hundredths of a millimeter. The sign of the value indicates the direction of the shift. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Xside1ImageShift |  |  |  |  |  |  |  |  |  | Integer |  |  |  |  |  |  | D |  |  |  |  | [PWG5100.3] §3.19.4 |  |  |
|  | Causes each Finished-Page Image that would be placed on the front side of a sheet to be shifted in position with respect to the x-axis of the media. The unit of measure for this element is hundredths of a millimeter. The sign of the value indicates the direction of the shift. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Xside2ImageShift |  |  |  |  |  |  |  |  |  | Integer |  |  |  |  |  |  | D |  |  |  |  | [PWG5100.3] §3.19.5 |  |  |
|  | Causes each Finished-Page Image that would be placed on the back side of a sheet to be shifted in position with respect to the x-axis of the media. The unit of measure for this element is hundredths of a millimeter. The sign of the value indicates the direction of the shift. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| YDimension |  |  |  |  |  |  |  |  |  | Integer |  |  |  |  | 0:MAX |  |  | D |  |  | [PWG5100.3] §3.13.8.2 |  |  |  |
|  | Size of the media in hundredths of a millimeter along the left edge. (See MediaSize for use) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| YImagePosition |  |  |  |  |  |  |  |  |  | String |  |  | type2 keyword |  |  |  |  | D |  |  |  | [PWG5100.3] §3.19.6 |  |  |
|  | Causes the specified point of the Finished-Page Image to be positioned at a specified location. (Keywords: none, center, top, bottom) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| YImageShift |  |  |  |  |  |  |  |  |  | Integer |  |  |  |  |  |  |  | D |  |  |  | [PWG5100.3] §3.19.7 |  |  |
|  | Causes the Finished-Page Image to be shifted in position with respect to the y-axis of the media. The unit of measure for this element is hundredths of a millimeter. The sign of the value indicates the direction of the shift. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Yside1ImageShift |  |  |  |  |  |  |  |  |  | Integer |  |  |  |  |  |  |  | D |  |  |  | [PWG5100.3] §3.19.8 |  |  |
|  | Causes each Finished-Page Image that would be placed on the front side of a sheet to be shifted in position with respect to the y-axis of the media. The unit of measure for this element is hundredths of a millimeter. The sign of the value indicates the direction of the shift. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Yside2ImageShift |  |  |  |  |  |  |  |  |  | Integer |  |  |  |  |  |  |  | D |  |  |  | [PWG5100.3] §3.19.9 |  |  |
|  | Causes each Finished-Page Image that would be placed on the back side of a sheet to be shifted in position with respect to the y-axis of the media. The unit of measure for this element is hundredths of a millimeter. The sign of the value indicates the direction of the shift. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

## Job Elements (Status and Description)

* Group Key: S=Status, D=Description

Table 4- Job Elements (Status and Description)

| Element Name |  |  |  | Multivalued |  |  |  |  |  |  | Syntax |  |  | Constraint | Group* |  |  |  |  | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Description (values) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DateTimeAtCreation |  |  |  |  |  |  |  |  |  |  | String | DateTime [rfc1123] |  |  |  | S |  |  | [rfc2911] §4.3.14.5 |  |
|  | Indicates the date and time at which the Job was created . (example: Fri, 03 May 2002 08:49:37 GMT) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DateTimeAtProcessing |  |  |  |  |  |  |  |  |  | String |  |  | DateTime [rfc1123] |  |  |  | S |  | [rfc2911] §4.3.14.6 |  |
|  | Indicates the date and time at which the Job first began processing. (example: Fri, 03 May 2002 08:49:37 GMT) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DateTimeAtCompleted |  |  |  |  |  |  |  |  |  | String |  |  | DateTime [rfc1123] |  |  |  | S |  | [rfc2911] §4.3.14.7 |  |
|  | Indicates the date and time at which the Job completed. (example: Fri, 03 May 2002 08:49:37 GMT) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DetailedStatusMessage |  |  |  |  | Yes |  |  |  |  |  | String |  | Maxlength=1023 |  |  | S |  |  | [rfc2911] §4.3.10 |  |
|  | Specifies additional detailed and technical information about the job. Intended for use by the system administrator or other experienced technical persons and so is not localized by the Printer. (example: “PostScript error: stack overflow”) (Was JobDetailedStatusMessage) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DocumentAccessErrors |  |  |  |  |  | Yes |  |  |  |  | String |  | Maxlength=1023 |  |  | S |  |  | [rfc2911] §4.3.11 |  |
|  | Information about each Document access error for this job encountered by the Printer. (example: “(404) http://www.company.com/pub/fileToPrint.pdf ”) (Was JobDocumentAccessErrors) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ElementsNaturalLanguage |  |  |  |  |  |  |  |  |  | String |  |  | Natural language |  |  |  | D |  | [rfc2911] §4.3.20 |  |
|  | Indicates the natural language of the elements with string syntax that were set by the End User. (Was AttributesNaturalLanguage) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Impressions |  |  |  |  |  |  |  |  |  |  | Integer |  |  | 0:MAX |  | D |  |  | [rfc2911] §4.3.17.2 |  |
|  | The total size in number of impressions in all the Job’s Document(s). (Was JobImpressions) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ImpressionsCompleted |  |  |  |  |  |  |  |  | Integer |  |  |  |  | 0:MAX |  | S |  |  | [rfc2911] §4.3.18.2 |  |
|  | The number of impressions completed for the Job so far. (Was JobImpressionsCompleted) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ImpressionsCompletedCurrentCopy |  |  |  |  |  |  |  |  | Integer |  |  |  |  | 0:MAX |  | S |  |  | [rfc3381] §4.4 |  |
|  | The number of impressions completed for the current iteration of this Job so far. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| JobAccountId |  |  |  |  |  |  |  |  |  |  | String | Maxlength=255 |  |  | D |  |  | [PWG5100.3] §3.6 |  |  |
|  | Account associated with this Job. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| JobAccountingUserID |  |  |  |  |  |  |  |  |  | String |  | Maxlength=255 |  |  | D |  |  | [PWG5100.3] §3.7 |  |  |
|  | Specifies the User ID associated with the “JobAccountId”. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| JobCollationType |  |  |  |  |  |  |  |  |  | String |  | Type2 keyword |  |  | S |  |  | [rfc3381] §4.1 |  |  |
|  | Identifies the collation type of the Job. (Keywords: other, unknown, uncollated-sheets, uncollated-documents, collated-documents) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| JobId |  |  |  |  |  |  |  |  |  |  | Integer |  |  | 1:MAX |  | S |  |  | [rfc2911] §4.3.2 |  |
|  | The Printer sets this to the ID of this Job , which is unique for the Printer. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| JobMessageFromOperator |  |  |  |  |  |  |  |  |  |  | String | Maxlength=127 |  |  |  | D |  |  | [rfc2911] §4.3.16 |  |
|  | Message to the end user indicating the reasons for any management action taken on this Job. (example: “Job canceled due to length”, “Pick job up in mailbox”) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| JobMessageToOperator |  |  |  |  |  |  |  |  |  |  | String | Maxlength=1023 |  |  |  | D |  | [PWG5100.3] §3.10 |  |  |
|  | Message from the end user to indicate something about the processing of this Job. (example: “Call 555-1234 before running this job”) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| JobName |  |  |  |  |  |  |  |  |  |  | String | Maxlength=255 |  |  |  | D |  |  | [rfc2911] §4.3.5 |  |
|  | The Printer sets this to the client-supplied end-user friendly name for the Job, else the Printer must generate a name from other information. (example: “license agreement memo”) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| JobOriginatingUserName |  |  |  |  |  |  |  |  |  |  | String |  | Maxlength=255 |  |  | D |  |  | [rfc2911] §4.3.6 |  |
|  | The Printer sets this element to the most authenticated printable name that it can obtain (example: “John Doe”, \authDomain\John Doe”) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| JobPassword |  |  |  |  |  |  |  |  |  |  | String |  | Maxlength=255 |  |  | D |  |  | [prod-print2] §4.1 |  |
|  | Contains a password supplied by the client encrypted according to method specified by the client in the JobPasswordEncryption element. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| JobPasswordEncryption |  |  |  |  |  |  |  |  |  |  | String |  | Type3 keyword |  |  | D |  |  | [prod-print2] §4.2 |  |
|  | Specifies the type of encryption that the client is used for the supplied value of the JobPassword element. (Keywords: none, md2, md4, md5, sha) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| JobPhoneNumber |  |  |  |  |  |  |  |  |  | String |  |  | Maxlength=127 |  |  | D |  |  | [prod-print2] §5.5 |  |
|  | Contains the contact telephone number for this Job. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| JobPrinterMakeAndModel |  |  |  |  |  |  |  |  |  |  | String |  | Maxlength=127 |  |  | S |  |  | [prod-print] §6.1 |  |
|  | Identifies the make and model of the output device which saved this Job according to the JobSaveDisposition Job Processing element. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| JobPrinterUri |  |  |  |  |  |  |  |  |  |  | String |  |  | uri |  | S |  |  | [rfc2911] §4.3.3 |  |
|  | The Printer set this to the URI of Printer that created this Job. (example: ipp://www.company.com/printer) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| JobRecipientName |  |  |  |  |  |  |  |  |  | String |  |  | Maxlength=255 |  |  | D |  |  | [prod-print2] §5.6 |  |
|  | Contains the name of the person that is to receive the output of this Job and is commonly printed on the job sheet. It may also be used to reference a data base containing delivery instructions for the recipient. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| JobState |  |  |  |  |  |  |  |  |  |  | String |  | Type1 keyword |  |  | S |  |  | [rfc2911] §4.3.7 |  |
|  | The current state of this Job (see section 4.2.1.1). See also JobStateReasons element below. (Keywords: pending, pending-held, processing, processing-stopped, canceled, aborted, completed) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| JobStateMessage |  |  |  |  |  |  |  |  |  |  | String |  | Maxlength=1023 |  |  | S |  |  | [rfc2911] §4.3.6 |  |
|  | Specifies information about the "JobState" and "JobStateReasons" elements in human readable text localized by the Printer according to the natural language supplied in the client’s query request. (example: “Job completed successfully with warnings" for an English request) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| JobStateReasons |  |  |  | Yes |  |  |  |  |  |  | String |  | type2 keyword |  |  | S |  |  | [rfc2911] §4.3.8 |  |
|  | Provides additional information about this Job's current state. (Keywords: aborted-by-system, canceled-at-device, canceled-by-operator, canceled-by-user, completed-successfully, completed-with-errors, completed-with-warnings, compression-error, document-access-error, document-format-error, incoming, interpreting, job-data-insufficient, job-hold-until-specified, job-password-wait, job-restartable, job-resuming, job-saved-successfully, job-save-error, job-saving, job-scheduling, job-suspended, job-suspended-by-operator, job-suspended-by-system, job-suspended-by-user, job-suspending, none, outgoing, printer-stopped, printer-stopped-partly, printing, processing-to-stop-point, proof-print-wait, queued, queued-for-marker, queued-in-device, resources-are-not-ready, resources-are-not-supported, service-off-line, spooling, streaming, submission-interrupted, transforming, unsupported-compression, unsupported-document-format, warnings-detected) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| JobUri |  |  |  |  |  |  |  |  |  |  | String |  |  | uri |  | S |  |  | [rfc2911] §4.3.1 |  |
|  | The Printer sets this to the URI for this Job. (example: ipp://www.company.com/printer/jobs/22) The URI is globally unique. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| KOctets |  |  |  |  |  |  |  |  |  |  | Integer |  |  | 0:MAX |  | D |  |  | [rfc2911] §4.3.17.1 |  |
|  | The total size of this Job’s Document(s) in integral units of 1024 octets. (Was JobKOctets) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| KOctetsProcessed |  |  |  |  |  |  |  |  | Integer |  |  |  |  | 0:MAX |  | S |  |  | [rfc2911] §4.3.18.1 |  |
|  | the total number of octets processed in integral units of 1024 octets so far. (Was JobKOctetsProcessed) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MediaSheets |  |  |  |  |  |  |  |  | Integer |  |  |  |  | 0:MAX |  | D |  |  | [rfc2911] §4.3.17.3 |  |
|  | The total number of media sheets to be produced for this Job’s Document(s). . (Was JobMediaSheets) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MediaSheetsCompleted |  |  |  |  |  |  |  |  |  |  | Integer |  |  | 0:MAX |  | S |  |  | [rfc2911] §4.3.18.3 |  |
|  | The media-sheets completed marking and stacking so far. (Was JobMediaSheetsCompleted) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MoreInfo |  |  |  |  |  |  |  |  |  |  | String |  |  | uri | S |  |  |  | [rfc2911] §4.3.4 |  |
|  | URI used to obtain information intended for end user consumption about this specific Job/Document. (example: “http://www.company.com/printer/embededjobpage”) . (Was JobMoreInfo) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NumberOfDocuments |  |  |  |  |  |  |  |  |  |  | Integer |  |  | 0:MAX | S |  |  |  | [rfc2911] §4.3.12 |  |
|  | The number of Documents in this Job. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NumberOfInterveningJobs |  |  |  |  |  |  |  |  |  |  | Integer |  |  | 0:MAX | S |  |  |  | [rfc2911] §4.3.15 |  |
|  | The number of jobs that are "ahead" of this Job assuming the current scheduled order. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OutputDeviceAssigned |  |  |  |  |  |  |  |  |  |  | String |  | Maxlength=127 |  | S |  |  |  | [rfc2911] §4.3.13 |  |
|  | Identifies the output device to which the Printer has assigned this Job (example: “Pete’s Printer”) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PrinterUpTime |  |  |  |  |  |  |  |  |  |  | Integer |  |  | 1:MAX | S |  |  |  | [rfc2911] §4.3.14.4 |  |
|  | The amount of time (in seconds) that the Printer has been up and running. See Printer element “PrinterUpTime” (Was JobPrinterUpTime) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SheetsCompletedCopyNumber |  |  |  |  |  |  |  |  | Integer |  |  |  |  | 0:MAX | S |  |  |  | [rfc3381] §4.2 |  |
|  | Number of the copy being stacked for the current Document. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SheetsCompletedDocumentNumber |  |  |  |  |  |  |  |  | Integer |  |  |  |  | 0:MAX | S |  |  |  | [rfc3381] §4.3 |  |
|  | Number of the document in this Job currently being stacked. . The Documents in a Job are numbered 1, 2, 3. A 0 value means no Document is currently being stacked. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TimeAtCreation |  |  |  |  |  |  |  |  |  |  | Integer |  |  | MIN:MAX | S |  |  |  | [rfc2911] §4.3.14.1 |  |
|  | The time at which the Job was created in “PrinterUpTime” seconds. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TimeAtProcessing |  |  |  |  |  |  |  |  |  |  | Integer |  |  | MIN:MAX | S |  |  |  | [rfc2911] §4.3.14.2 |  |
|  | The time at which the Job first began processing in “PrinterUpTime” seconds. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TimeAtCompleted |  |  |  |  |  |  |  |  |  |  | Integer |  |  | MIN:MAX | S |  |  |  | [rfc2911] §4.3.14.3 |  |
|  | The time at which the Job completed in “PrinterUpTime” seconds. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| WarningsCount |  |  |  |  |  |  |  |  |  |  | Integer |  |  | MIN:MAX | S |  |  |  | [PWG5100.4 §6.1 |  |
|  | The total number of warnings that a Printer has generated while processing and printing a Job’s Document(s). (Was JobWarningsCount) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

## Document Elements (Status and Description)

* Group Key: S=Status, D=Description

Table 5 – Document Elements (Status and Description)

| Element Name |  |  |  | Multivalued |  |  |  |  |  |  | Syntax |  |  | Constraint |  | Group* |  |  |  | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Description (values) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Compression |  |  |  |  |  |  |  |  |  |  | String |  |  | Type2 keyword |  |  |  | D |  | [rfc2911] §4.4.32 |
|  | Compression algorithm used on the Document Data, if any. (Keywords: none, deflate, gzip, compress) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CurrentPageOrder |  |  |  |  |  |  |  |  |  |  | String | Type2 keyword |  |  |  |  | S |  | [PWG5100.3] §4.1 |  |
|  | Indicates the page order of the pages in the document data. Initially set to PageOrderReceived and updated if data is transformed. (Keywords: 1-to-n-order, n-to-1-order) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DateTimeAtCompleted |  |  |  |  |  |  |  | String |  |  |  | DateTime [rfc1123] |  |  |  |  |  | S |  | [rfc2911] §4.3.14.7 |
|  | Indicates the date and time at which this Document completed. (example: Fri, 03 May 2002 08:49:37 GMT) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DateTimeAtCreation |  |  |  |  |  |  |  |  |  |  | String | DateTime [rfc1123] |  |  |  |  |  | S |  | [rfc2911] §4.3.14.5 |
|  | Indicates the date and time at which this Document was created . (example: Fri, 03 May 2002 08:49:37 GMT) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DateTimeAtProcessing |  |  |  |  |  |  |  | String |  |  |  | DateTime [rfc1123] |  |  |  |  |  | S |  | [rfc2911] §4.3.14.6 |
|  | Indicates the date and time at which this Document first began processing. (example: Fri, 03 May 2002 08:49:37 GMT) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DetailedStatusMessage |  |  |  |  | Yes |  |  |  | String |  |  | Maxlength=1023 |  |  |  |  |  | S |  | [rfc2911] §4.3.10 |
|  | Specifies additional detailed and technical information about this Document. Intended for use by the system administrator or other experienced technical persons. (example: “PostScript error: stack overflow”) (Was JobDetailedStatusMessage) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DocumentAccessErrors |  |  |  |  |  | Yes |  |  |  |  | String | Maxlength=1023 |  |  |  |  |  | S |  | [rfc2911] §4.3.11 |
|  | Information about each Document access error for this Document encountered by the Printer. (example: “(404) http://www.company.com/pub/fileToPrint.pdf ”) (Was JobDocumentAccessErrors) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DocumentFormat |  |  |  |  |  |  |  |  | String |  |  | MimeMediaType [rfc2046], [rfc2048] |  |  |  |  |  | D |  | [rfc2911] §3.2.1.1 |
|  | The Document format (i.e., PDL) for this Document. The value “application/octet-stream” has a special meaning. This value is used to indicate that a Printer is capable of auto-sensing the format of the Document. (Examples: application/octet-stream, application/postscript, application/vnd.hp-PCL, “text/plain; charset=utf-8”) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DocumentName |  |  |  |  |  |  |  |  | String |  |  | Maxlength=127 |  |  |  |  |  | D |  | [rfc2911] §3.2.1.1 |
|  | Name for this Document to be used in an implementation specific manner. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DocumentNaturalLanguage |  |  |  |  |  |  |  |  |  |  | String |  |  | Maxlength=127 |  |  |  | D |  | [rfc2911] §3.2.1.1 |
|  | Identifies the Natural Language of this Document |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DocumentNumber |  |  |  |  |  |  |  |  |  |  | integer |  |  |  |  |  | S |  | [PWG5100.4] §9.2, [doc-obj] §6.1 |  |
|  | The order of this document within a job starting at a base of 1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DocumentState |  |  |  |  |  |  |  |  |  |  | String |  |  | Type1 keyword |  |  |  | S |  | [doc-obj] §6.3.2 |
|  | The current state of this Document. See also DocumentStateReasons element below. (Keywords: pending, processing, canceled, aborted, completed) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DocumentStateMessage |  |  |  |  |  |  |  |  |  |  | String |  | Maxlength=127 |  |  |  |  | S |  | [doc-obj] §6.7 |
|  | Specifies information about the "DocumentState" and "DocumentStateReasons" elements of this Document in human readable text localized by the Printer according to the language supplied in the client’s query request. (Example: “Document completed successfully with warnings" for an English request) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DocumentStateReasons |  |  |  | Yes |  |  |  |  |  |  | String |  |  | type2 keyword |  |  |  | S |  | [doc-obj] §6.5 |
|  | Provides additional information about this Document’s current state. (Keywords: none, aborted-by-system, canceled-at-device, canceled-by-operator, canceled-by-user, completed-successfully, completed-with-errors, completed-with-warnings, compression-error, document-access-error, document-format-error, incoming, interpreting, outgoing, printing, queued, queued-for-marker, queued-in-device, resources-are-not-ready, resources-are-not-supported, spooling, streaming, submission-interrupted, transforming, unsupported-compression, unsupported-document-format, warnings-detected) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DocumentUri |  |  |  |  |  |  |  |  |  |  | String |  | Maxlength=1023 |  |  |  |  | D |  | [rfc2911] §3.2.2 |
|  | Reference to the Document to be printed (Print by reference) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Impressions |  |  |  |  |  |  |  |  |  |  | Integer |  |  | 0:MAX |  |  |  | D |  | [rfc2911] §4.3.17.2 |
|  | The total size in number of impressions in this Document. (Was JobImpressions) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ImpressionsCompleted |  |  |  |  |  |  |  |  | Integer |  |  |  |  | 0:MAX |  |  |  | S |  | [rfc2911] §4.3.18.2 |
|  | The number of impressions completed for this Document so far. (Was JobImpressionsCompleted) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ImpressionsCompletedCurrentCopy |  |  |  |  |  |  |  |  | Integer |  |  |  |  | 0:MAX |  |  |  | S |  | [rfc3381] §4.4 |
|  | The number of impressions completed for the current iteration of this Document so far. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| JobId |  |  |  |  |  |  |  |  |  |  | Integer |  |  |  | 1:MAX |  |  | S |  | [rfc2911] §4.3.2 |
|  | The Printer sets this to the ID of the job containing this Document. The ID is unique for the Printer. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| JobUri |  |  |  |  |  |  |  |  |  |  | String |  |  |  | uri |  |  | S |  | [rfc2911] §4.3.1 |
|  | The Printer sets this to the URI for the job. (example: ipp://www.company.com/printer/jobs/22) The URI is globally unique. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| KOctets |  |  |  |  |  |  |  |  |  |  | Integer |  |  | 0:MAX |  |  |  | D |  | [rfc2911] §4.3.17.1 |
|  | The total size of this Document in integral units of 1024 octets. (Was JobKOctets) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| KOctetsProcessed |  |  |  |  |  |  |  |  | Integer |  |  |  |  | 0:MAX |  |  |  | S |  | [rfc2911] §4.3.18.1 |
|  | the total number of octets processed in integral units of 1024 octets so far. (Was JobKOctetsProcessed) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| LastDocument |  |  |  |  |  |  |  |  |  |  | Boolean |  |  |  |  |  |  | D |  | [rfc2911] §3.3.1 |
|  | Has a ‘true’ value if this Document is the last Input Document for the Job. Default = ‘false’. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MediaSheets |  |  |  |  |  |  |  |  | Integer |  |  |  |  | 0:MAX |  |  |  | D |  | [rfc2911] §4.3.17.3 |
|  | The total number of media sheets to be produced for this Document. (was JobMediaSheets) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MediaSheetsCompleted |  |  |  |  |  |  |  |  |  |  | Integer |  |  | 0:MAX |  |  |  | S |  | [rfc2911] §4.3.18.3 |
|  | The media-sheets completed marking and stacking for this Document so far. (Was JobMediaSheetsCompleted) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MoreInfo |  |  |  |  |  |  |  |  |  |  | String |  |  | uri |  | S |  |  |  | [rfc2911] §4.3.4 |
|  | URI used to obtain information intended for end user consumption about this specific Document. (example: “http://www.company.com/printer/embededjobpage”) . (Was JobMoreInfo) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PageOrderReceived |  |  |  |  |  |  |  |  |  | String |  | Type2 keyword |  |  |  | D |  |  | [PWG5100.3] §3.16 |  |
|  | Indicates the order of pages in this Document data as supplied with the job. (Keywords: 1-to-n-order, n-to-1-order) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PrinterUpTime |  |  |  |  |  |  |  |  |  |  | Integer |  |  | 1:MAX |  |  |  | S |  | [rfc2911] §4.3.14.4 |
|  | The amount of time (in seconds) that the Printer has been up and running. (See Printer element “PrinterUpTime”) (Was JobPrinterUpTime) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SheetsCompletedCopyNumber |  |  |  |  |  |  |  |  | Integer |  |  |  |  | 0:MAX |  | S |  |  |  | [rfc3381] §4.2 |
|  | Number of the copy being stacked for this Document. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TimeAtCreation |  |  |  |  |  |  |  |  |  |  | Integer |  |  | MIN:MAX |  |  | S |  |  | [rfc2911] §4.3.14.1 |
|  | The time at which this Document was created in “PrinterUpTime” seconds. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TimeAtProcessing |  |  |  |  |  |  |  |  |  |  | Integer |  |  | MIN:MAX |  |  | S |  |  | [rfc2911] §4.3.14.2 |
|  | The time at which this Document first began processing. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TimeAtCompleted |  |  |  |  |  |  |  |  |  |  | Integer |  |  | MIN:MAX |  |  | S |  |  | [rfc2911] §4.3.14.3 |
|  | The time at which this Document completed. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| WarningCount |  |  |  |  |  |  |  |  |  |  | Integer |  |  | MIN:MAX |  |  | S |  |  | [PWG5100.4 §6.1 |
|  | The total number of warnings that a Printer has generated while processing and printing the Document. (Was Job WarningCount) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

## Printer Elements (Status and Description)

* Group Key: S=Status, D=Description

Table 6 - Printer Elements (Status and Description)

| Element Name |  | Multivalued |  |  |  |  |  |  |  |  |  |  |  |  | Syntax |  |  |  |  |  |  |  |  | Constraint | Group* |  |  |  |  |  | reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Description (values) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ColorSupported |  |  |  |  |  |  |  |  |  |  |  |  |  |  | boolean |  |  |  |  |  |  |  |  |  | D |  |  |  |  |  | [rfc2911] §4.4.26 |
|  | Indicates if this Printer is capable of any type of color printing at all, including highlight color. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CompressionSupported |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  | String |  |  |  |  |  |  | Type3 keyword |  |  |  | D |  |  |  |  | [rfc2911] §4.4.32 |
|  | Identifies the set of Compression algorithms for Document content that this Printer supports. (Keywords: none, deflate, gzip, compress) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DeviceId |  |  |  |  |  |  |  |  |  |  |  |  |  |  | String |  |  |  |  |  |  |  |  | IEEE 1284 | D |  |  |  |  |  | See Appendix 11.1 |
|  | An identifier based on IEEE 1284 to identify the device that the Printer represents. Often used to load an appropriate driver on the client device. (example: “MANUFACTURER:ACME;COMMAND SET:PCL,PJL,PS,XHTML-Print+xml;MODEL:LaserBeam 9;COMMENT:example;ACTIVE COMMAND SET:PCL”) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DocumentFormatDefault |  |  |  |  |  |  |  |  |  |  |  | String |  |  |  |  |  |  | MimeMediaType [rfc2046], [rfc2048] |  |  |  |  |  |  | D |  |  |  |  | [rfc2911] §4.4.21 |
|  | The document format (i.e. PDL) that this Printer has been configured to assume if the client does not specify a document format in any of the actions that supply document content for a Job. The value “application/octet-stream” has a special meaning. This value is used to indicate that a Printer is capable of auto-sensing the format of the document. (examples: application/octet-stream, application/postscript, application/vnd.hp-PCL, “text/plain; charset=utf-8”) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DocumentFormatSupported |  |  |  |  |  | YES |  |  |  |  |  |  | String |  |  |  |  |  | MimeMediaType |  |  |  |  |  |  | D |  |  |  |  | [rfc2911] §4.4.22 |
| Identifies both the Document and Image formats supported by this Printer. Specifies the set of Document formats that the Printer supports. (examples: application/octet-stream, application/postscript, application/vnd.hp-PCL, “text/plain; charset=utf-8”). Also specifies the set of Image formats that the Printer supports. (examples: ‘image/jpeg’ which is a registered MIME Media Type with IANA. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| GeneratedNaturalLanguageSupported |  |  |  |  |  | YES |  |  |  |  |  |  | String |  |  |  |  |  | Natural Language |  |  |  |  |  |  | D |  |  |  |  | [rfc2911] §4.4.20 |
| Identifies the natural language(s) that the Printer supports in returned values of messages generated by the Printer, that is, the JobStateMessage, DocumentStateMessage, and PrinterStateMessage elements. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ImpressionsSupported |  |  |  |  |  |  |  |  |  |  |  | RangeOfInteger |  |  |  |  |  |  |  |  |  |  |  | 0:MAX |  | D |  |  |  |  | [rfc2911] §4.4.34 |
|  | Specifies the upper and lower bounds for the number of impressions allowed per job. (Was JobImpressionsSupported) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| JobCreationElementsSupported |  |  |  |  |  |  |  | YES |  |  |  |  | String |  |  |  |  |  | Type2 keyword |  |  |  |  |  |  | D |  |  |  |  | [prod-print1] §7.1 |
|  | Identifies the set of Job Processing and Job Description elements (but not member elements) that this Printer will accept in a JobCreation action (Was JobCreationAttributesSupported) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| JobPasswordEncryptionSupported |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  | String |  |  |  |  |  | type3 keyword |  |  |  |  | D |  |  | [prod-print1] §7.3 |
|  | Identifies which encryption methods this Printer supports as values of the JobPasswordEncryption Job Description element for Secure Print. (Keywords: none, md2, md4, md5, sha) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| JobPasswordSupported |  |  |  |  |  |  |  |  |  |  |  |  | Integer |  |  |  |  |  | 0:MAX |  |  |  |  |  |  | D |  |  |  |  | [prod-print1] §7.2 |
|  | Indicates the maximum length that this Printer will accept for the unencrypted password which the client will encrypt as the value of the JobPassword Description Element. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| JobSpoolingSupported |  |  |  |  |  |  |  |  |  |  |  |  | String |  |  |  |  |  | type2 keyword |  |  |  |  |  |  | D |  |  |  |  | [prod-print1] §7.4 |
|  | Indicates whether or not the Printer spools Jobs before interpreting the document data (RIPing). (Keywords: spool, stream, automatic) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| KOctetsSupported |  |  |  |  |  |  |  |  |  |  |  | RangeOfInteger |  |  |  |  |  |  |  |  |  |  |  | 0:MAX |  | D |  |  |  |  | [rfc2911] §4.4.33 |
|  | Specifies the allowable upper and lower bounds of the total size per Job in integral units of 1024 octets that this Printer will accept. (Was JobKOctetsSupported) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MaxSaveInfoSupported |  |  |  |  |  |  |  |  |  |  |  |  | Integer |  |  |  |  |  |  |  |  |  |  | 1:MAX |  | D |  |  |  |  | [prod-print1] §7.5 |
|  | Identifies the maximum number of SaveInfo member element collections that this Printer can accept in a job request. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MediaColDatabase |  |  |  |  |  |  |  | Yes |  |  |  |  | Complex |  |  |  |  |  |  |  |  |  |  |  |  | D |  |  |  |  | [prod-print1] §7.6 |
|  | Identifies all of the Media supported by this Printer using a collection value for each which identifies the media characteristics. This element is not returned when ‘all’ is requested. (Includes any of the MediaCol member elements) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MediaSheetsSupported |  |  |  |  |  |  |  |  |  |  |  | RangeOfInteger |  |  |  |  |  |  |  |  |  |  |  | 0:MAX |  | D |  |  |  |  | [rfc2911] §4.4.35 |
|  | Specifies the upper and lower bounds for the number of media sheets allowed per job by this Printer. (Was JobMediaSheetsSupported) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MultipleDocumentJobsSupported |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | boolean |  |  |  |  |  |  |  | D |  |  |  |  | [rfc2911] §4.4.16 |
|  | Indicates whether this Printer supports more than one Document per job, i.e., more than one SendDocument and/or SendUri request per job. A multi-Document per job Printer must implement this element and have a value of ‘true’. A single Document per job Printer may either not support this element or support it with a value of ‘false’. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MultipleOperationTimeOut |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Integer |  |  |  |  |  |  |  |  | 1:MAX |  | D |  |  |  |  | [rfc2911] §4.4.31 |
|  | Identifies the minimum time (in seconds) that this multi-Document per job Printer will wait between actions on an open job before timing out. The actions can add Document to the open Job or close the Job. Timeouts are handled in an implementation specific manner. Multi-Document per job Printers must implement this element. The recommended value is greater than 60 and less than 240. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NaturalLanguageConfigured |  |  |  |  |  |  |  |  |  |  |  |  |  | String |  |  |  |  |  |  |  |  | Natural language |  |  |  |  |  | D |  | [rfc2911] §4.4.19 |
|  | Indicates the natural language of the elements with string syntax that were set by the Administrator or Manufacturer. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OperationsSupported |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  | String |  |  |  |  | type2 keyword |  |  |  |  |  | D |  |  |  |  | [rfc2911] §4.4.15 |
|  | The set of supported actions for the Printer and Job. (Keywords: PrintJob, PrintUri, CreateJob, SendDocument, SendURI, ValidateJob, ValidateDocument, CancelJob, HoldJob, ReleaseJob, RestartJob, SetJobElements, SetDocumentElements, CancelDocument, DeleteDocument, GetJobs, GetPrinterElements, GetJobElements, GetDocuments, GetDocumentElements, GetPrinterSupportedValues, PausePrinter, ResumePrinter, PurgeJobs, DisablePrinter, EnablePrinter, SetPrinterElements). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PagesPerMinute |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Integer |  |  |  |  |  |  |  |  | 0:MAX |  | D |  |  |  |  | [rfc2911] §4.4.36 |
|  | Specifies the nominal number of pages per minute which may be generated by this Printer. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PagesPerMinuteColor |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Integer |  |  |  |  |  |  |  |  | 0:MAX |  | D |  |  |  |  | [rfc2911] §4.4.37 |
|  | Specifies the nominal number of pages per minute which may be generated by this Printer when printing color. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ParentPrintersSupported |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  | String |  |  |  |  |  |  |  |  | Uri |  | D |  |  |  |  | [admin-ops] §7.2 |
|  | Contains the URI of the non-leaf Printer for which this Printer is the immediate subordinate. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PdlOverrideSupported |  |  |  |  |  |  |  |  |  |  |  |  |  |  | String |  |  |  |  | type2 keyword |  |  |  |  |  | D |  |  |  |  | [rfc2911] §4.4.28 |
|  | Expresses the ability of this Printer to (1) guaranteed, (2) attempt to, or (3) not attempt to override a Document’s processing instructions with Job Processing Elements. (Keywords: attempted, guaranteed, not-attempted) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PrinterCurrentTime |  |  |  |  |  |  |  |  |  |  |  |  |  |  | String |  |  |  |  | DateTime [rfc1123] |  |  |  |  |  |  | S |  |  |  | [rfc2911] §4.4.30 |
|  | Indicates the current date and time. (example: Fri, 03 May 2002 08:49:37 GMT) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PrinterDetailedStatusMessages |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  | String |  |  |  |  | Maxlength=1023 |  |  |  |  |  | S |  |  |  |  | [prod-print2] §7.7 |
|  | Specifies additional detailed and technical information about this Printer for the technical staff. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PrinterDriverInstaller |  |  |  |  |  |  |  |  |  |  |  |  |  |  | String |  |  |  |  |  |  |  |  | Uri |  | D |  |  |  |  | [rfc2911] §4.4.8 |
|  | Intended for consumption by automata to locate the driver installer for this Printer object. (example: “http://www.company.com/printer/installerProgram”) Note: This element has not been used by any known implementation and is therefore deprecated. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PrinterInfo |  |  |  |  |  |  |  |  |  |  |  |  |  |  | String |  |  |  |  | Maxlength=127 |  |  |  |  |  |  | D |  |  |  | [rfc2911] §4.4.6 |
|  | Descriptive information about this Printer object.(example: “Out of courtesy for others, please print only small (1-5 page) jobs at this printer") |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PrinterIsAcceptingJobs |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Boolean |  |  |  |  |  |  |  |  |  |  | S |  |  |  |  | [rfc2911] §4.4.23 |
|  | Indicates whether this Printer is currently able to accept jobs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PrinterLocation |  |  |  |  |  |  |  |  |  |  |  |  |  |  | String |  |  |  |  | Maxlength=127 |  |  |  |  |  | D |  |  |  |  | [rfc2911] §4.4.5 |
|  | Identifies the location of the device that this Printer represents. (Example: Pete’s Office) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PrinterMakeAndModel |  |  |  |  |  |  |  |  |  |  |  |  |  |  | String |  |  |  |  | Maxlength=127 |  |  |  |  |  | D |  |  |  |  | [rfc2911] §4.4.9 |
|  | Identifies the make and model of the device that this Printer object represents. (Example: “Xerox Phaser 7700”, “HP LaserJet 1000”, “Lexmark Optra Color 45”) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PrinterMessageFromOperator |  |  |  |  |  |  |  |  |  |  |  |  |  |  | String |  |  |  |  | Maxlength=127 |  |  |  |  |  | D |  |  |  |  | [rfc2911] §4.4.25 |
|  | End user information for this Printer. (Example: “printer unavailable until 1pm due to preventive maintenance”) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PrinterMoreInfo |  |  |  |  |  |  |  |  |  |  |  |  |  |  | String |  |  |  |  |  |  |  |  | uri | D |  |  |  |  |  | [rfc2911] §4.4.7 |
|  | URI used to obtain information intended for end user consumption about this specific Printer. (Example: “http://www.company.com/printer/embeddedwebpage”) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PrinterMoreInfoManufacturer |  |  |  |  |  |  |  |  |  |  |  |  |  |  | String |  |  |  |  |  |  |  |  | uri | D |  |  |  |  |  | [rfc2911] §4.4.10 |
|  | URI used to obtain more information for end user consumption about this type of device that this Printer represents. (Example: “http://www.xerox.com/go/xrx/template/012.jsp?Xcntry=USA&Xlang=en_US&prodID=7700”, “http://www.lexmark.com/US/products/overview/0,1224,MjQ5fDE=,00.html”) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PrinterName |  |  |  |  |  |  |  |  |  |  |  |  |  |  | String |  |  |  |  | Maxlength=127 |  |  |  |  | D |  |  |  |  |  | [rfc2911] §4.4.4 |
|  | The end-user friendly name of this Printer object. (example: “Pete’s Printer”) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PrinterState |  |  |  |  |  |  |  |  |  |  |  |  |  |  | String |  |  |  |  | type1 keyword |  |  |  |  | S |  |  |  |  |  | [rfc2911] §4.4.11 |
|  | Identifies the current state of the device(s) that this Printer represents (see Figure 4). (See “PrinterStateReasons” below) (Keywords: idle, processing, stopped) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PrinterStateMessage |  |  |  |  |  |  |  |  |  |  |  |  |  |  | String |  |  |  |  | Maxlength=1023 |  |  |  |  |  | S |  |  |  |  | [rfc2911] §4.4.13 |
|  | Information about the "printer- state" and "printer-state-reasons" elements in human readable text localized by the Printer according to the natural language supplied in the client’s query request. (Example: “Printer stopped due to paper jam” for an English request) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PrinterStateReasons |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  | String |  |  |  |  | type2 keyword |  |  |  |  | S |  |  |  |  |  | [rfc2911] §4.4.12 |
|  | Augments the "printer-state" element to give more detailed information about this Printer’s state. Each keyword value may have a suffix to indicate its level of severity. The three suffixes (levels) are: “Report” (least severe), “Warning”, and “Error” (most severe). Keywords without suffixes are assumed to be “Error” (most severe). See reference for semantics of defined keywords. (Keywords: other, none, connecting-to-device, cover-open, deactivated, developer-empty, developer-low, door-open, fuser-over-temp, fuser-under-temp, hold-new-jobs, input-tray-missing, interlock-open, interpreter-resource-unavailable, marker-supply-empty, marker-supply-low, marker-waste-almost-full, marker-waste-full, media-empty, media-jam, media-low, media-needed, moving-to-paused, opc-life-over, opc-near-eol, output-area-almost-full, output-area-full, output-tray-missing, paused, shutdown, spool-area-full, stopped-partly, stopping, timed-out, toner-empty, toner-low) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PrinterUpTime |  |  |  |  |  |  |  |  |  |  |  |  |  |  | integer |  |  |  |  | 1:MAX |  |  |  |  |  | S |  |  |  |  | [rfc2911] §4.4.29 |
|  | The amount of time (in seconds) that this Printer has been up and running |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PrinterUriSupported |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  | String |  |  |  |  |  |  |  |  | uri |  | D |  |  |  |  | [rfc2911] §4.4.1 |
|  | Contains at least one URI for this Printer object. The PrinterUriSupported, UriAuthenticationSupported and the UriSecuritySupported are parallel elements. Each of these elements must have the same cardinality. The “i”th value of each of these elements describes the URI for the printer, the authentication mechanism used and the security method used. (Example: ipp://www.company.com/printer) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| QueuedJobCount |  |  |  |  |  |  |  |  |  |  |  |  |  |  | integer |  |  |  |  |  |  |  |  | 0:MAX |  | S |  |  |  |  | [rfc2911] §4.4.24 |
|  | The number of jobs that this Printer has accepted but has not yet completed. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ReferenceUriSchemesSupported |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | String |  |  |  |  |  |  |  |  | UriScheme |  | D |  |  |  |  | [rfc2911] §4.4.27 |
|  | Which URI schemes are supported by this Printer to retrieve Document This element must be supported if the Printer is capable of print by reference. (Example: ftp, http) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SubordinatePrintersSupported |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  | String |  |  |  |  |  |  |  |  | Uri |  | D |  |  |  |  | [admin-ops] §7.1 |
|  | Contains the URI of the immediate subordinate Printers associated with this Printer. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UriAuthenticationSupported |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  | String |  |  |  |  | type2 keyword |  |  |  |  |  | D |  |  |  |  | [rfc2911] §4.4.2 |
|  | The Client authentication mechanism that this Printer object uses to identify the user. (See PrinterUriSupported for additional information) (Keywords: none, requesting-user-name, basic, digest and certificate) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UriSecuritySupported |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  | String |  |  |  |  | type2 keyword |  |  |  |  |  | D |  |  |  |  | [rfc2911] §4.4.3 |
|  | Identifies the security mechanisms used for accessing this Printer object. (See PrinterUriSupported for additional information) (Keywords: none, ssl3, tls) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| VersionsSupported |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  | String |  |  |  |  | type2 keyword |  |  |  |  | D |  |  |  |  | [rfc2911] §4.4.14 |
|  | The versions of the semantics that this Printer supports. (Keywords: 1.0, 1.1, etc. ). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| WhichJobsSupported |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  | String |  |  |  |  | type2 keyword |  |  |  |  | D |  |  |  | [prod-print2] §7.8 |  |
|  | Contains the set of values that this Printer supports for the WhichJobs operation element that the client may supply in the Get-Jobs operation as a job filter. (Keywords: aborted, all, canceled, completed, not-completed, pending, pending-held, processing, processing-stopped ) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

# Status Strings

This Appendix lists the status strings that the Printer returns in each action response.

Table 7  Status strings indicating some degree of success

| Status String |  |  | Actions where status may occur |
| --- | --- | --- | --- |
| Reference |  | Description of status |  |
| successful-ok |  |  | Any |
| Rfc2911 | Action succeeded and no requested element were substituted or ignored. |  |  |
| successful-ok-conflicting-attributes |  |  | PrintJob, PrintUri, CreateJob, SendDocument, SendUri |
|  | Action succeeded but some elements were conflicting and have been substituted or ignored. |  |  |
| successful-ok-ignored-or-substituted-attributes |  |  | PrintJob, PrintUri, CreateJob, SendDocument, SendUri |
|  | Action succeeded but some unsupported elements were ignored or substituted. |  |  |

| Status String |  |  | Actions where status may occur |
| --- | --- | --- | --- |
|  |  | Description of status |  |
| The following status strings are returned when the Printer rejects the action indicating some error on the part of the Client: |  |  |  |
| client-error-bad-request |  |  | Any |
|  | malformed syntax or constraint exceeded. |  |  |
| client-error-forbidden |  |  | Any |
|  | The Printer understood the request, but is refusing to fulfill it for authentication and/or authorization reasons. The client should not try again even with credentials. |  |  |
| client-error-not-authenticated |  |  | Any |
|  | The request requires user authentication. The client may try again with suitable authentication. |  |  |
| client-error-not-authorized |  |  | Any |
|  | The requester is not authorized to perform the request. The Client should not try again. |  |  |
| client-error-not-possible |  |  |  |
|  | The action cannot be performed, because of the state of the target object. |  |  |
| client-error-timeout |  |  |  |
|  | The client did not produce a subsequent request within the time that the Printer was prepared to wait. |  |  |
| client-error-not-found |  |  |  |
|  | The target object was not found. |  |  |
| client-error-gone |  |  |  |
|  | The target object is no longer available. |  |  |
| client-error-request-entity-too-large |  |  |  |
|  | The request and/or the Document Content is too large. |  |  |
| client-error-request-value-too-long |  |  |  |
|  | An element value in the request is longer than the Printer supports. |  |  |
| client-error-document-format-not-supported |  |  |  |
|  | The document format is not supported. |  |  |
| client-error-attributes-or-values-not-supported |  |  |  |
|  | An element and/or value is not supported and must be in order to carry out the request. The Printer must return the unsupported elements or values in the Unsupported Elements group. |  |  |
| client-error-uri-scheme-not-supported |  |  |  |
|  | The URI scheme is not supported. |  |  |
| client-error-charset-not-supported |  |  |  |
|  | The charset is not supported. |  |  |
| client-error-conflicting-attributes |  |  |  |
|  | Some supplied elements are conflicting. The Printer must return them in the Unsupported Elements group. |  |  |
| client-error-compression-not-supported |  |  |  |
|  | The compression of the Document Content is not supported. |  |  |
| client-error-compression-error |  |  |  |
|  | An error occurred when uncompressing the Document Content. |  |  |
| client-error-document-format-error |  |  |  |
|  | An error occurred when interpreting the Document Content. |  |  |
| client-error-document-access-error |  |  |  |
|  | An error occurred when the Printer attempted to access the Document Content through the URI supplied. |  |  |
| client-error-attributes-not-settable |  |  |  |
|  | The supplied element(s) are not settable |  |  |

| Status String |  | Actions where status may occur |
| --- | --- | --- |
| Reference | Description of status |  |
| The following status strings are returned when the Printer rejects the action indicating some error on the part of the Printer: |  |  |
| server-error-internal-error |  |  |
|  | An unexpected internal error occurred. |  |
| server-error-operation-not-supported |  |  |
|  | The Printer does not support the requested action. |  |
| server-error-service-unavailable |  |  |
|  | The Printer is unable to service the request at this time due to overloading or maintenance. The client should try again later as per the “message” Operation element. |  |
| server-error-version-not-supported |  |  |
|  | The Printer doesn’t support the requested major version of the protocol and returns the closest version that it does support. |  |
| server-error-device-error |  |  |
|  | The Printer encountered a device error that causes it to be unable to accept a new request. For example, a paper jam for a Printer that doesn’t spool and so cannot accept a new job submission until the jam is fixed. |  |
| server-error-temporary-error |  |  |
|  | A temporary error such as a buffer full write error, a memory overflow, or a disk full condition. |  |
| server-error-not-accepting-jobs |  |  |
|  | The Printer is not currently accepting jobs. Its “PrinterIsAcceptingJobs” Printer Description element is ‘false’. |  |
| server-error-busy |  |  |
|  | A temporary error indicating that the Printer is too busy processing jobs and/or other requests. A Client should try again later. |  |
| server-error-job-canceled |  |  |
|  | The job has been canceled by an operator or aborted by the system. For example, while the Client is transmitting the Document Content to the Printer. |  |
| server-error-multiple-document-jobs-not-supported |  |  |
|  | The Printer doesn’t support multiple document jobs and the client attempted to supply a second SendDocument or SendUri request. The Printer’s “MultipleDocumentJobsSupported” Printer Description element is ‘false’. |  |
| server-error-printer-is-deactivated |  |  |
|  | The Printer has been deactivated using the Deactivate-Printer operation and is only accepting the Activate-Printer |  |

# Change Log

5/16/02	PJZ	original draft

5/23/02	TH	re-organize draft with comments from Melinda Grant

5/26/02	TH	detailed review of the draft

5/29/02	PJZ	Incorporated comments prior to initial release

6/4/02   	SAA	Modified to split the Job Attributes into 3 categories:

- Processing Attributes

- Content Attributes

- Job Attributes

The Processing Attributes were further split into 3 subcategories:

- Rendering attributes

- Imposition Attributes

- Finishing Attributes

Added attributes from UPnP Print Basic service template:   MediaSize, MediaType, DeviceId attributes.

Removed references to Mandatory vs. Optional since a semantic model should not dictate what is used or not used by the future solutions targeted at specific markets.  For example, UPnP picked specific attributes for the SOHO market and did not need all of the Mandatory IPP attributes.

Modified Printer Description Attributes with the following:

- Added in DeviceId.

- Changed Document* to Content*.

- Removed VersionsSupported and OperationsSupported since these are dependent on the interface used in specific solutions.

6/17/02	PJZ	Added high level description of PWG Action semantics and Printer state transitions.  Returned VersionsSupported and OperationsSupported.

8/16/02	PJZ	Changed Content back to document, Added PWG5100.1, PWG5100.2, PWG5100.3, PWG5100.4, job-progress to model.  Filled out document object, added “Job Level” subcategory to Processing attributes

9/1/02		PJZ	Changes from email input and PWG meeting.  Printer/Job/Document Attribute groups broken out into State and Description groups

9/9/02		PJZ	Final edits to ready document for review.  Updated all figures and added highlighting of sections to review.

9/16/02	PJZ	Added more definitions and document actions.  Incorporated the comments from teleconference and TH mail note.  Updated references.

9/27/02	TNH	Version 0.11: Spell checked, corrected some misspelled attribute names,.  Finished moving Compression and DocumentFormat from the Processing to the Document Description tables.  Improved the attributes descriptions, especially those that are related to other attributes.  Added the attributes and values from [prod-print2].  Added several attributes from IPP documents that were missing for some reason.  Corrected a number of Maxlength values.  Sorted the values of JobStateReasons, DocumentStateReasons, and PrinterStateReasons, so easier to keep track of.  Add References:  [adm-ops], [prod-print2].

9/30/02	PJZ	Began conversion of status string section to table. Corrected and updated figures.  Removed detailed IPP encoding section.  Added globalization section

10/07/02	PJZ	Updated references. Added JobCoverFront, JobCoverBack, and natural language elements.  Reworked section 5.3.5 GetPrinterSettableAttributeValues.  Corrected Action table and section.

10/14/01	TNH	Fixed some Figure caption problems.  Instead of deprecating AttributeFidelity, made it work with JobMandatoryAttributes.  Added way to specify the member attribute in a collection attribute (Attr.Member).  Clarified PagesPerSubset as combining all Input Documents into a single contiguous Input-Pages stream and then subsetting it into Output Documents.  Added GeneratedNaturalLanguageSupported from RFC 2911.

10/28/02	PJZ	“XML”ified attributes and object & added IPP mapping information describing change.  Completed adding [admin-ops], [PWG5100.1]. Rationalized “Pages” and “PageRanges”. Changed “State” groups to “Status” to avoid name collision with “State” elements (e.g. “JobState”)

# References

[adm-ops] Kugler, C, Hastings, T., Lewis, H., "Internet Printing Protocol (IPP): Job and Printer Administrative Operations", July 17, 2001, < draft-ietf-ipp-ops-set2-03.txt >.

[doc-obj] Hastings, T., and P. Zehler, "Internet Printing Protocol (IPP): Document Object", September 27, 2002, ftp://ftp.pwg.org/pub/pwg/ipp/new_DOC/IPP-Document-Object.pdf, work in progress to become IEEE-ISTO 5100.5-2001.

[ntfy] "Internet Printing Protocol/1.1: Event Notifications and Subscriptions", November 19, 2001, Herriot, R., Hastings, T., Shepherd, M., deBry, R., Isaacson, S., Martin, J., and R. Bergman,<draft-ietf-ipp-not-spec-08.txt>.

[prod-print2] Hastings, T., and D. Fullman, “Internet Printing Protocol (IPP): Production Printing Attributes - Set 2”, to become a PWG IEEE-ISTO standard, work in progress, August 21, 2002, ftp://ftp.pwg.org/pub/pwg/ipp/new_PPE/pwg-ipp-prod-print-set2-draft-v0_1-020821.pdf

[PWG5100.1] IEEE-ISTO 5100.1-2001, "Internet Printing Protocol (IPP): “finishings” attribute values extension”, Hastings, T., and D. Fullman,  February 5, 2001, ftp://ftp.pwg.org/pub/pwg/standards/pwg5100.1.pdf

[PWG5100.2] IEEE-ISTO 5100.2-2001, “Internet Printing Protocol (IPP): output-bin attribute extension”, February 7, 2001, Hastings, T., and R. Bergman, ftp://ftp.pwg.org/pub/pwg/standards/pwg5100.2.pdf

[PWG5100.3] IEEE-ISTO 5100.3-2001, "Internet Printing Protocol (IPP): Production Printing Attributes - Set1", February 12, 2001, Ocke, K., Hastings, T., ftp://ftp.pwg.org/pub/pwg/standards/pwg5100.3.pdf

[PWG5100.4] IEEE-ISTO 5100.4-2001, "Internet Printing Protocol (IPP): Override Attributes for Documents and Pages", February 7, 2001, Herriot, R., Ocke, K., ftp://ftp.pwg.org/pub/pwg/standards/pwg5100.4.pdf

[PWG5101.1] IEEE-ISTO 5101.1-2001 Media Standardized Names <work in progress>, ftp://ftp.pwg.org/pub/pwg/standards/pwg5101.1.pdf , .doc, .rtf for standardized names

[rfc1123] RFC 1123 " Requirements for Internet Hosts -- Application and Support ", October 1989, Branden, R. , ftp://ftp.rfc-editor.org/in-notes/rfc1123.txt

[rfc2046] RFC 2046 "Multipurpose Internet Mail Extensions (MIME) Part Two: Media Types", November 1996, Freed, N. and N. Borenstein, ftp://ftp.rfc-editor.org/in-notes/rfc2046.txt

[rfc2048] RFC 2048 "Multipurpose Internet Mail Extension (MIME) Part Four: Registration Procedures", November 1996, Freed, N., Klensin, J. and J. Postel, ftp://ftp.rfc-editor.org/in-notes/rfc2048.txt

[rfc2911] RFC 2566 “Internet Printing Protocol/1.0 Model and Semantics”, March 1999 and RFC 2911 “Internet Printing Protocol/1.1 Model and Semantics”, September 2000, T. Hastings, R. Herriot, R. Debry, S. Isaacson, P. Powell, ftp://ftp.rfc-editor.org/in-notes/rfc2911.txt

[rfc3380] "Internet Printing Protocol (IPP): Job and Printer Set Operations", September 2002, Hastings, T., Herriot, R., Kugler, C., and H. Lewis, ftp://ftp.rfc-editor.org/in-notes/rfc3380.txt

[rfc3381]"Internet Printing Protocol (IPP): Job Progress Attributes", September 2002, Hastings, T., Lewis, H., and R. Bergman, ftp://ftp.rfc-editor.org/in-notes/rfc3381.txt

# Author's Addresses

Peter Zehler

Xerox Corporation

800 Phillips Road

Webster, NY 14580

Phone: 585 265-8755

Fax: 585-265-8871

e-mail:  pzehler@crt.xerox.com

PWG Semantic Model Web Page:  http://www.pwg.org/sm/

PWG Semantic Model Mailing List:  sm@pwg.org

To subscribe to the sm mailing list, send the following email:

1) send it to majordomo@pwg.org

2) leave the subject line blank

3) put the following two lines in the message body:

subscribe sm

end

Implementers of this specification document are encouraged to join IPP Mailing List in order to participate in any discussions of clarification issues and review of registration proposals for additional attributes and values.

Other Participants:

| Alan Berkema – HP | Lee Farrell - Canon Information Systems |
| --- | --- |
| –Don Fullman - Xerox | Melinda Grant - HP |
| David Hall - HP | Tom Hastings - Xerox |
| Harry Lewis - IBM | –Ira Mcdonald – High North |
| Gail Songer - Netreon | Bob Taylor - HP |
| William Wagner - NetSilicon/DPI |  |

# Appendix A – UPnP Definitions

## DeviceID

The value of this variable MUST exactly match the IEEE 1284-2000 Device ID string, except the length field MUST not be specified.. The value is assigned by the Printer vendor and MUST NOT be localized by the Print Service.

The IEEE 1284-2000 Device ID is a length field followed by a case-sensitive string of ASCII characters defining peripheral characteristics and/or capabilities. For the purposes of this specification, the length bytes MUST NOT be included. The Device ID sequence is composed of a series of keys and values of the form:

key: value {,value} repeated for each key

As indicated, each key will have one value, and MAY have more than one value. The minimum necessary keys (case-sensitive) are  MANUFACTURER, COMMAND SET, and MODEL. (These keys MAY be abbreviated as MFG, CMD, and MDL respectively.) Each implementation MUST supply these three keys and possibly additional ones as well. Each key (and each value) is a string of characters. Any characters except colon (:), comma (,), and semi-colon (;) MAY be included as part of the key (or value) string. Any leading or trailing white space (SPACE[x'20'], TAB[x'09'], VTAB[x'0B'], CR[x'0D'], NL[x'0A'], or  FF[x'0C']) in the string is ignored by the parsing program (but is still counted as part of the overall length of the sequence).

An example ID String, showing optional comment and active command set keys and their associated values (the text is actually all on one line):

MANUFACTURER:ACME Manufacturing;

COMMAND SET:PCL,PJL,PS,XHTML-Print+xml;

MODEL:LaserBeam 9;

COMMENT:Anything you like;

ACTIVE COMMAND SET:PCL;

(See IEEE 1284-2000 clause 7.6)

Note: One of the purposes of the DeviceId variable is to select a printer driver for those clients that need a printer driver.  The values of the COMMAND SET key are interpreted by the printer driver provided by the vendor and so are vendor-defined, rather than being standardized.

# Appendix B – IPP Mapping

## Changes to remove some IPP specific aspects

This section lists some changes to remove some IPP specific aspects from the PWG Semantic Model.

- IPP enumerations use their well-known string name instead of the integer enumeration.  This applies not only to IPP attributes but also to IPP Operations.

- Any attribute name containing “ipp” has had the “ipp” removed.

- All attribute and operation keywords have the substring “attribute” replaced with “element”.

- All operation and attribute keyword names have had the first letter capitalized and the ‘-‘ character removed and the character following the ‘-‘ has been capitalized.  (All mixed case PWG Semantic Model keywords can be interpreted without regard to case.)

- The attribute value keywords defined remain unchanged and are all lower case, except for the ones that specify other attributes names (which are changed to be the mixed case without hyphens).

- The types of the attributes have been simplified.  All keyword, text, name, DateTime, uri, UriScheme, enum and mimeMediaType  types are represented by the simple string type.

- The “1setOf X” types are represented as the base type and the “Multivalued” field in the tables set to “Yes”.  Integers and Boolean types remain the same.  Any applicable constraints placed on the attribute values has been noted in the tables.

The term “keyword” continues to be used for string values enumerated as part of the PWG Model.  The term “object” is sometimes changed to “data class”.  The term “operation” has been changed to “action” to use the term more frequently used with XML.

The following IPP attributes are not included: operation-id, attributes-charset, , page-overrides, request-id, version-number

## Attribute Group Mapping

IPP Actions may contain a number of parameters.  The first parameter is always the Operation Attributes for the Action.  The IPP Operation Attributes have been mapped to the Printer and Job Description Element Groups.

The IPP Printer Description Attributes map to the PWG Printer Status Elements and Printer Description Elements.  The IPP Job Description Attributes map to the PWG Job Status Elements and Job Description Elements.

The IPP Job Template Attributes map to the PWG Job Processing Elements and Document Processing Elements.  IPP does not differentiate between the PWG Processing Elements subgroups of Rendering, Imposition and Finishing Elements.