February 27, 2013

Implementation Guide: Working Draft

Printer Work Group

Implementation Guide

Printer Working Group: Print Job Ticket Support Levels

(PWG:PJT – Support Levels)

Status: Initial

Abstract: This White Paper provides an implementation reference for three (3) levels of supports of the Print Working Group (PWG) Print Job Ticket (PJT).  The first support level in intended for printing simply Print Jobs; the second support level is intended for modest to high-end office printing; and, finally, the third support level, supporting all the features of the PWG:PJT is intended for high-end office and production printing.  Specifying a Support Level by a Print Client provides a Print Service with knowledge of the Print Client capabilities.    End-User will benefit from (and/or with the capability to set0) the Support Level by experiencing a consistent Print Client interface at Support Level.   And, while a Printer's capabilities may not fully implement a specific Support Level; a Print Service can, through the use of Print Transforms, augment a Printer's capabilities to meet the requirements of a specific support level

--- This document does not use the PWG Approved Document Template ---

Copyright © 2013-2013         Printer Working Group                          All rights reserved.

This document may be copied and furnished to others, and derivative works that comment on, or otherwise explain it or assist in its implementation may be prepared, copied, published and distributed, in whole or in part, without restriction of any kind, provided that the above copyright notice, this paragraph and the title of the Document as referenced below are included on all such copies and derivative works.  However, this document itself may not be modified in any way, such as by removing the copyright notice or references to the IEEE-ISTO and the Printer Working Group, a program of the IEEE-ISTO.

Title:  Printer Working Group: Print Job Ticket Support Levels

The IEEE-ISTO and the Printer Working Group DISCLAIM ANY AND ALL WARRANTIES, WHETHER EXPRESS OR IMPLIED INCLUDING (WITHOUT LIMITATION) ANY IMPLIED WARRANTIES OF MERCHANTABILITY OR FITNESS FOR A PARTICULAR PURPOSE.

The Printer Working Group, a program of the IEEE-ISTO, reserves the right to make changes to the document without further notice.  The document may be updated, replaced or made obsolete by other documents at any time.

The IEEE-ISTO takes no position regarding the validity or scope of any intellectual property or other rights that might be claimed to pertain to the implementation or use of the technology described in this document or the extent to which any license under such rights might or might not be available; neither does it represent that it has made any effort to identify any such rights.

The IEEE-ISTO invites any interested party to bring to its attention any copyrights, patents, or patent applications, or other proprietary rights which may cover technology that may be required to implement the contents of this document. The IEEE-ISTO and its programs shall not be responsible for identifying patents for which a license may be required by a document and/or IEEE-ISTO Industry Group Standard or for conducting inquiries into the legal validity or scope of those patents that are brought to its attention. Inquiries may be submitted to the IEEE-ISTO by e-mail at: ieee-isto@ieee.org.

The Printer Working Group acknowledges that the IEEE-ISTO (acting itself or through its designees) is, and shall at all times, be the sole entity that may authorize the use of certification marks, trademarks, or other special designations to indicate compliance with these materials.

Use of this document is wholly voluntary. The existence of this document does not imply that there are no other ways to produce, test, measure, purchase, market, or provide other goods and services related to its scope.

About the IEEE-ISTO

The IEEE-ISTO is a not-for-profit corporation offering industry groups an innovative and flexible operational forum and support services.  The IEEE-ISTO provides a forum not only to develop standards, but also to facilitate activities that support the implementation and acceptance of standards in the marketplace.  The organization is affiliated with the IEEE (http://www.ieee.org/) and the IEEE Standards Association (http://standards.ieee.org/).

For additional information regarding the IEEE-ISTO and its industry programs visit:

http://www.ieee-isto.org

About the IEEE-ISTO PWG

The Printer Working Group (or PWG) is a Program of the IEEE Industry Standards and Technology Organization (ISTO) with member organizations including printer manufacturers, print server developers, operating system providers, network operating systems providers, network connectivity vendors, and print management application developers.  The group is chartered to make printers and the applications and operating systems supporting them work together better.  All references to the PWG in this document implicitly mean “The Printer Working Group, a Program of the IEEE ISTO.” In order to meet this objective, the PWG will document the results of their work as open standards that define print related protocols, interfaces, procedures and conventions. Printer manufacturers and vendors of printer related software will benefit from the interoperability provided by voluntary conformance to these standards.

In general, a PWG standard is a specification that is stable, well understood, and is technically competent, has multiple, independent and interoperable implementations with substantial operational experience, and enjoys significant public support.

For additional information regarding the Printer Working Group visit:

http://www.pwg.org

Contact information:

Printer Working Group

c/o The IEEE Industry Standards and Technology Organization

445 Hoes Lane

Piscataway, NJ 08854

USA

About the Cloud Work Group and Semantic Model Work Group

For information regarding Cloud Work Group visit:

http://www.pwg.org/cloud/

For information regarding Multi-Semantic Model Work Group visit:

http://www.pwg.org/mfd/

Implementers of this specification are encouraged to join the Cloud Work Group and the Semantic Model Work Group mailing list in order to participate in any discussions of the specification. Suggested additions, changes, or clarification to this specification, should be sent to the Cloud Work Group and/or Semantic Model Work Group Mailing list for consideration.

.      Table of Contents     .

1. Introduction	6

2. Terminology	6

2.1 Conformance Terminology	6

2.2 Other Terminology	6

2.3 Acronyms	6

2.4 Organizations	6

3. Requirements	7

3.1 Rationale for PWG:PJT Support Levels Implementation Guide	7

3.2 Use Cases	7

3.3 Out of Scope	7

3.4 Design Requirements	7

4. Specification	7

5. Conformance	8

5.1 Conformance: PWG:PJT Support Level – 1	9

5.2 Conformance: PWG:PJT Support Level – 2	13

5.3 Conformance: PWG:PJT Support Level – 3	21

5.4 Charset-Supplied Support Level	36

5.4.1 Conformance: PWG:PJT:Charset-Supplied Support Level – 1	36

5.4.2 Conformance: PWG:PJT:Charset-Supplied Support Level – 2	36

5.4.3 Conformance: PWG:PJT:Charset-Supplied Support Level – 3	36

6. Internationalization	42

7. Security Considerations	42

8. IANA Registration Information	42

9. References	43

9.1 Normative References	43

9.2 Informative References	43

10. Authors' Contact Information	43

.      List of Figures     .

.      List of Tables     .

# Introduction

To be supplied

# Terminology

## Conformance Terminology

Capitalized terms, such as MUST, MUST NOT, RECOMMENDED, REQUIRED, SHOULD, SHOULD NOT, MAY, and OPTIONAL, have special meaning relating to conformance as defined in Key words for use in RFCs to Indicate Requirement Levels [RFC2119]. The term CONDITIONALLY REQUIRED is additionally defined for a conformance requirement that applies to a particular capability or feature.

## Other Terminology

## Acronyms

## Organizations

| IANA | Internet Assigned Numbers Authority | www.iana.org |
| --- | --- | --- |
| IETF | Internet Engineering Task Force | www.ietf.org |
| ISO | International Organization for Standardization | www.iso.org |
| PWG | Printer Working Group | www.pwg.org |

# Requirements

## Rationale for PWG:PJT Support Levels Implementation Guide

The Printer Working Group (PWG) Print Job Ticket (PJT) consist of a rich set of printing objects, attributes, elements and values that support the simplest level of printing to most complex.  The PWG:PJT supports printing from/in embedded, mobile, home-office, business office and production environments.  However, supporting the capabilities of the PWG:PJT by Applications and/or Print Clients in the various environments is burdensome.  While a Print Service only needs to provide capability information that it supports; currently, there is an implicit requirement that a Print Client support potentially all PWG:PJT objects, elements, attributes and values that a Print Service could instantiate as a capability; this is just not practical in all situations and environments.

## Use Cases

Not acceptable for this implementation guide.

## Out of Scope

This document identify Support Levels for the PWG: PJT but does identify

- Vendor Extensions for Elements

- Vendor Extensions for element Values

## Design Requirements

< should put some words in here on why the three level and some brief design rationale for only 3 level and what each level represents.   Maybe (!) use-cases are needed to identify the 3 levels???>

# Specification

All content in discussed in the document and, specifically, the Elements and Values outlined in Section 5. Conformance is identified and defined in the PWG-5108.07-2012 candidate Standard titled  "PWG Print Job Ticket and Associated Capabilities Version 1.0 (PJT)" [PWG5108.07].

# Conformance

At the top level, the PWG:PJT consist of three required major elements; namely;

- Print Job Ticket

  - Print Document Processing

  - Print Job Description

  - Print Job Processing.

The details for each Support Level are elaborated in the following subsection.

## Conformance: PWG:PJT Support Level – 1

- Print Job Ticket

  - Print Document Processing

    - Number Up

    - Copies

    - Font Name Requested

        - text/plain

        - Vendor Extension

    - Font Size Requested

    - Media Type

    - Media (= Media Size Name)

    - Orientation Requested

        - Landscape

        - Portrait

        - Reverse Landscape

        - Reverse Portrait

        - None

    - Output bin

        - Top

        - Bottom

        - Rear

        - Large Capacity

    - Page Ranges

        - Lower Bound

        - Upper Bound

        - Vendor Extension

    - Print Content Optimize

        - Text

        - Graphics

        - Text and Graphics

        - Photo

        - Vendor Extension

    - Print Color Mode

        - Auto

        - Bi-Level (Only Required for B/W Printers)

        - Color

        - Monochrome

        - Process Bi-Level

        - Process Monochrome

        - Vendor Extension

    - Print Rendering Intent

        - Auto

        - Perceptual

      - Vendor Extension

    - Quality

        - Draft

        - Normal

        - High

        - Vendor Extension

    - Resolution

        - Cross Feed Dir

        - Feed Dir

      - Resolution Unit

          - DPI

          - DPCM

          - Vendor Extension

        - Vendor Extension

    - Sheet Collate

        - Uncollated

        - Collated

        - Vendor Extension

    - Sides

        - One Sided

        - Two Sided Long Edge

        - Two Sided Short Edge

        - Vendor Extension

    - X Image Position

        - None

        - Center

        - Left

        - Right

        - Vendor Extension

    - X Image Shift

    - X Side 1 Image Shift

    - X Side 2 Image Shift

    - Y Image Position

        - None

        - Center

        - Top

        - Bottom

        - Vendor Extension

    - Y Image Shift

    - Y Side 1 Image Shift

    - Y Side 2 Image Shift

    - Vendor Extension

  - Print Job Description

    - Job Accounting ID

    - Job Accounting User ID

    - Job Collation Type

        - Uncollated Sheets

        - Collated Documents

        - Uncollated Documents

        - Vendor Extension

    - Job More Info

    - Job Name

    - Job Originating User Name

    - Document Format Details Supplied

      - Document Format

          - Application PDF

          - Application VND PWG XHTML Print XML

          - Image JPEG

          - Text HTML

          - Text Plain

          - Text Plain Charset ISO 8859 1

          - Text Plain Charset US ASCII

          - Text Plain Charset UTF 8

      - Document Format Version

      - Document Natural Language

          - VERY, VERY Long list – How to Handle

    - Document Format Supplied

        - Application PDF

        - Application VND PWG XHTML Print XML

        - Image JPEG

        - Text HTML

        - Text Plain

        - Text Plain Charset US ASCII

        - Text Plain Charset UTF 8

    - Document Format Version Supplied

    - Document Name Supplied

    - Document URI

    - Page Order Received

        - 1 to N Order

        - N to 2 Order

        - Vendor Extension

    - Vendor Extension

  - Print Job Processing

    - Job Copies

    - Job Error Action

        - Abort Job

        - Cancel Job

    - Job Collation Type

        - Uncollated Sheets

        - Collated Documents

        - Uncollated Documents

        - Vendor Extension

    - Job Copies

    - Output Device

    - Vendor Extension

## Conformance: PWG:PJT Support Level – 2

Note that items in blue are the elements and values that are added from Support Level 1 to define Support Level 2.

- Print Job Ticket

  - Print Document Processing

    - Number Up

    - Copies

    - Cover Front

      - Cover Type

          - No Cover

          - Print None

          - Print Front

          - Print Back

          - Print Both

          - Vendor Extension

      - Media Type

          - To – Be – Determined

      - Media

          - To – Be – Determined

      - Media Col

          - To – Be – Determined

      - Vendor Extension

    - Feed Orientation

        - Long Edge First

        - Short Edge First

        - Vendor Extension

    - Finishings

        - None

        - Jog Offset

        - Staple Top Left

        - Vendor Extension

    - Font Name Requested

    - Font Size Requested

    - Media Type

    - Media (= Media Size Name)

    - Media Col

        - To – Be – Determined

    - Orientation Requested

        - Landscape

        - Portrait

        - Reverse Landscape

        - Reverse Portrait

        - None

    - Output bin

        - Top

        - Middle

        - Bottom

        - Side

        - Rear

        - Large Capacity

        - Mailbox 1

        - Mailbox 2

        - Mailbox 3

        - Mailbox 4

        - Mailbox 5

        - My Mailbox

    - Page Delivery

        - Same Order Face Up

        - Same Order Face Down

        - Reverse Order Face Up

        - Reverse Order Face Down

        - System Specified

        - Vendor Extension

    - Page Ranges

        - Lower Bound

        - Upper Bound

        - Vendor Extension

    - Print Content Optimize

        - Text

        - Graphics

        - Text and Graphics

        - Photo

        - Vendor Extension

    - Print Color Mode

        - Auto

        - Bi-Level (Only Required for B/W Printers)

        - Color

        - Monochrome

        - Process Bi-Leel

        - Process Monochrome

        - Vendor Extension

    - Print Rendering Intent

        - Auto

        - Absolute

        - Perceptual

        - Relative

        - Relative BPC

        - Saturation

      - Vendor Extension

    - Quality

        - Draft

        - Normal

        - High

        - Vendor Extension

    - Resolution

        - Cross Feed Dir

        - Feed Dir

      - Resolution Unit

          - DPI

          - DPCM

          - Vendor Extension

        - Vendor Extension

    - Separator Sheets

        - None

        - Slip Sheet

        - Start Sheet

        - End Sheet

        - Both Sheets

        - Vendor Extension

    - Sheet Collate

        - Uncollated

        - Collated

        - Vendor Extension

    - Sides

        - One Sided

        - Two Sided Long Edge

        - Two Sided Short Edge

        - Vendor Extension

    - X Image Position

        - None

        - Center

        - Left

        - Right

        - Vendor Extension

    - X Image Shift

    - X Side 1 Image Shift

    - X Side 2 Image Shift

    - Y Image Position

        - None

        - Center

        - Top

        - Bottom

        - Vendor Extension

    - Y Image Shift

    - Y Side 1 Image Shift

    - Y Side 2 Image Shift

    - Vendor Extension

  - Print Job Description

    - Job Accounting ID

    - Job Accounting User ID

    - Job More Info

    - Job Name

    - Job Originating User Name

    - Job Originating User URI

    - Job Pages

    - K Octets

    - Document Format Details Supplied

      - Document Format

          - Application PDF

          - Application VND PWG XHTML Print XML

          - Application VND HP PCL

          - Image JPEG

          - Image TIFF

          - Image TIFF FX

          - Text HTML

          - Text Plain

          - Text Plain Charset US ASCII

          - Text Plain Charset UTF 8

          - Vendor Extension

      - Document Format Version

      - Document Format Device ID

      - Document Natural Language

          - VERY, VERY Long list – How to Handle

      - Document Source Application Name

      - Document Source Application Version

      - Document Source OS Name

      - Long list – How to Handle

      - Document Source OS Version

    - Document Format Supplied

        - Application PDF

        - Application VND PWG XHTML Print XML

        - Application VND HP PCL

        - Image JPEG

        - Image TIFF

        - Image TIFF FX

        - Text HTML

        - Text Plain

        - Text Plain Charset ISO 8859 1

        - Text Plain Charset US ASCII

        - Text Plain Charset UTF 8

        - Vendor Extension

    - Document Format Version Supplied

    - Document Name Supplied

    - Document URI

    - Impressions

    - Job Collation Type

        - Uncollated

        - Collated

        - Vendor Extension

    - Media Sheets

    - Page Order Received

        - 1 to N Order

        - N to 2 Order

        - Vendor Extension

    - Vendor Extension

  - Print Job Processing

    - Job Copies

    - Job Delay Output Until

      - No Hold

      - Indefinite

      - Vendor Extension

    - Job Error Action

        - Abort Job

        - Cancel Job

    - Job Hold Until

      - No Hold

      - Indefinite

      - Vendor Extension

    - Job Priority

    - Job Recipient Name

    - Job Accounting Sheets

      - Job Accounting Sheets Type

          - None

          - Standard

          - Vendor Extension

      - Job Accounting Output Bin

          - Top

          - Middle

          - Bottom

          - Side

          - Rear

          - Large Capacity

          - Mailbox 1

          - Mailbox 2

          - Mailbox 3

          - Mailbox 4

          - Mailbox 5

          - My Mailbox

      - Media

          - To – Be – Determined

      - Media Type

          - To – Be – Determined

      - Media Col

          - To – Be – Determined

    - Job Collation Type

        - Uncollated Sheets

        - Collated Documents

        - Uncollated Documents

        - Vendor Extension

    - Job Copies

    - Job Cover Front

      - Cover Type

          - No Cover

          - Print None

          - Print Front

          - Print Back

          - Print Both

          - Vendor Extension

      - Media Type

          - To – Be – Determined

      - Media

          - To – Be – Determined

      - Media Col

      - To – Be – Determined

    - Job Error Sheet

      - Job Error Sheet Type

          - None

          - Standard

          - Vendor Extension

      - Job Error Sheet When

          - On Error

          - Always

          - Vendor Extension

      - Media Type

          - To – Be – Determined

      - Media

          - To – Be – Determined

      - Media Col

          - To – Be – Determined

    - Job Finishings

        - None

        - Jog Offset

        - Staple Top Left

    - Job Sheets

        - None

        - Standard

        - Vendor Extension

    - Job Sheets Col

      - Job Sheets

          - None

          - Standard

          - Vendor Extension

      - Media Type

          - To – Be – Determined

      - Media

          - To – Be – Determined

      - Media Col

          - To – Be – Determined

      - Vendor Extension

    - Multiple Documents Handle

        - Separate Documents Collated Copies

        - Separate Documents Uncollated Copies

        - Single Document

        - Single Document New Sheet

        - Vendor Extension

    - Output Device

    - Vendor Extension

## Conformance: PWG:PJT Support Level – 3

Note that items in blue are the elements and values that are added from Support Level 1 and Level 2 to define Support Level 3.

- Print Job Ticket

  - Print Document Processing

    - Number Up

    - Presentation Direction Up

        - To Right To Bottom

        - To Bottom To Right

        - To left To Bottom

        - To Bottom To Left

        - To Right To Top

        - To Top To Right

        - To Left To Top

        - To Tope To Left

        - Vendor Extension

    - Copies

    - Cover Back

      - Cover Type

          - No Cover

          - Print None

          - Print Front

          - Print Back

          - Print Both

          - Vendor Extension

      - Media Type

          - To – Be – Determined

      - Media

          - To – Be – Determined

      - Media Col

          - To – Be – Determined

      - Vendor Extension

    - Cover Front

      - Cover Type

          - No Cover

          - Print None

          - Print Front

          - Print Back

          - Print Both

          - Vendor Extension

      - Media Type

          - To – Be – Determined

      - Media

          - To – Be – Determined

      - Media Col

          - To – Be – Determined

    - Feed Orientation

        - Long Edge First

        - Short Edge First

        - Vendor Extension

    - Finishings

        - None

        - Staple

        - Punch

        - Cover

        - Bind

        - Saddle Stitch

        - Edge Stitch

        - Fold

        - Trim

        - Bale

        - Booklet Maker

        - Jog Offset

        - Staple Top Left

        - Staple Bottom Left

        - Staple Top Right

        - Staple Bottom Right

        - Edge Stitch Left

        - Edge Stitch Top

        - Edge Stitch Right

        - Edge Stitch Bottom

        - Staple Dual Left

        - Staple Dual Top

        - Staple Dual Right

        - Staple Dual Bottom

        - Bind Left

        - Bind Top

        - Bind Right

        - Bind Bottom

        - Vendor Extension

    - Finishings Col

        - Finishing Template

      - Stitching

          - Bottom

          - Top

          - Left

          - Right

          - Vendor Extension

    - Font Name Requested

    - Font Size Requested

    - Force Front Side

    - Imposition Template

        - None

        - Signature

        - Z Fold

        - Same Up

        - Vendor Extension

    - Insert Sheet

        - Insert After Page Number

        - Insert Count

      - Media Type

          - To – Be – Determined

      - Media

          - To – Be – Determined

      - Media Col

          - To – Be – Determined

    - Media Type

    - Media (= Media Size Name)

    - Media Col

        - To – Be – Determined

    - Media Input Tray Check

    - Orientation Requested

        - Landscape

        - Portrait

        - Reverse Landscape

        - Reverse Portrait

        - None

        - Vendor Extension

    - Output bin

        - Top

        - Middle

        - Bottom

        - Side

        - Left

        - Right

        - Center

        - Rear

        - Face Up

        - face down

        - Large Capacity

        - Stacker 1

        - Stacker 2

        - Stacker 3

        - Stacker 4

        - Stacker 5

        - Stacker 6

        - Stacker 7

        - Stacker 8

        - Stacker 9

        - Stacker 10

        - Mailbox 1

        - Mailbox 2

        - Mailbox 3

        - Mailbox 4

        - Mailbox 5

        - Mailbox 6

        - Mailbox 7

        - Mailbox 8

        - Mailbox 9

        - Mailbox 10

        - Mailbox 11

        - Mailbox 12

        - Mailbox 13

        - Mailbox 14

        - Mailbox 15

        - Mailbox 16

        - Mailbox 17

        - Mailbox 18

        - Mailbox 19

        - Mailbox 20

        - Mailbox 21

        - Mailbox 22

        - Mailbox 23

        - Mailbox 24

        - Mailbox 25

        - My Mailbox

        - Tray 1

        - Tray 2

        - Tray 3

        - Tray 4

        - Tray 5

        - Tray 6

        - Tray 7

        - Tray 8

        - Tray 9

        - Tray 10

        - Tray 11

        - Tray 12

        - Tray 13

        - Tray 14

        - Tray 15

        - Tray 16

        - Tray 17

        - Tray 18

        - Tray 19

        - Tray 20

    - Page Delivery

        - Same Order Face Up

        - Same Order Face Down

        - Reverse Order Face Up

        - Reverse Order Face Down

        - System Specified

        - Vendor Extension

    - Page Ranges

        - Lower Bound

        - Upper Bound

        - Vendor Extension

    - Pdl Init File

    - Print Content Optimize

        - Text

        - Graphics

        - Text and Graphics

        - Photo

        - Vendor Extension

    - Print Color Mode

        - Auto

        - Bi-Level (Only Required for B/W Printers)

        - Color

        - Monochrome

        - Process Bi-Leel

        - Process Monochrome

        - Vendor Extension

    - Print Rendering Intent

        - Auto

        - Absolute

        - Perceptual

        - Relative

        - Relative BPC

        - Saturation

      - Vendor Extension

    - Quality

        - Draft

        - Normal

        - High

        - Vendor Extension

    - Resolution

        - Cross Feed Dir

        - Feed Dir

      - Resolution Unit

          - DPI

          - DPCM

          - Vendor Extension

        - Vendor Extension

    - Separator Sheets

        - None

        - Slip Sheet

        - Start Sheet

        - End Sheet

        - Both Sheets

        - Vendor Extension

    - Sheet Collate

        - Uncollated

        - Collated

        - Vendor Extension

    - Sides

        - One Sided

        - Two Sided Long Edge

        - Two Sided Short Edge

        - Vendor Extension

    - X Image Position

        - None

        - Center

        - Left

        - Right

        - Vendor Extension

    - X Image Shift

    - X Side 1 Image Shift

    - X Side 2 Image Shift

    - Y Image Position

        - None

        - Center

        - Top

        - Bottom

        - Vendor Extension

    - Y Image Shift

    - Y Side 1 Image Shift

    - Y Side 2 Image Shift

    - Vendor Extension

  - Print Job Description

    - Job Accounting ID

    - Job Accounting User ID

    - Job Message From Operator

    - Job Message To Operator

    - Job Password

    - Job Password Encryption

        - MD2

        - MD4

        - MD5

        - SHA

        - None

    - Job More Info

    - Job Name

    - Job Originating User Name

    - Job Originating User URI

    - Job Pages

    - K Octets

    - Template Creator User Name

    - Template ID

    - Template Info

    - Template Name

    - Template Type

        - None

        - Vendor Extension

    - Compression Supplied

        - Deflate

        - GZIP

        - Compress

        - None

        - Vendor Extension

    - Document Charset Supplied

        - To – Be – Determined

    - Document Digital Signature Supplied

        - DSS

        - None

        - PGP

        - SMIME

        - XDSIG

        - Vendor Extension

    - Document Format Details Supplied

      - Document Format

          - Application Octet Stream

          - Application PDF

          - Application POSTSCRIPT

          - Application VND PWG XHTML Print XML

          - Application VND HP PCL

          - Image G3 FAX

          - Image JPEG

          - Image TIFF

          - Image TIFF FX

          - Text HTML

          - Text Plain

          - Text Plain Charset ISO 8859 1

          - Text Plain Charset US ASCII

          - Text Plain Charset UTF 8

          - Vendor Extension

      - Document Format Version

      - Document Format Device ID

      - Document Natural Language

      - Document Source Application Name

      - Document Source Application Version

      - Document Source OS Name

      - Document Source OS Version

      - Vendor Extension

    - Document Format Supplied

        - Application Octet Stream

        - Application PDF

        - Application POSTSCRIPT

        - Application VND PWG XHTML Print XML

        - Application VND HP PCL

        - Image G3 FAX

        - Image JPEG

        - Image TIFF

        - Image TIFF FX

        - Text HTML

        - Text Plain

        - Text Plain Charset ISO 8859 1

        - Text Plain Charset US ASCII

        - Text Plain Charset UTF 8

        - Vendor Extension

    - Document Format Version Supplied

    - Document Message Supplied

    - Document Meta Data

    - Document Name Supplied

    - Document URI

    - Impressions

    - Job Collation Type

        - Uncollated

        - Collated

        - Vendor Extension

    - Media Sheets

    - Page Order Received

        - 1 to N Order

        - N to 2 Order

        - Vendor Extension

    - Vendor Extension

  - Print Job Processing

    - Job Copies

    - Job Delay Output Until

      - No Hold

      - Indefinite

      - Day Time

      - Evening

      - Night

      - Weekend

      - Second Shift

      - Third Shift

      - Vendor Extension

    - Job Delay Output Until Time

    - Job Error Action

        - Abort Job

        - Cancel Job

        - Continue Job

        - Suspend Job

        - Vendor Extension

    - Job Hold Until

      - No Hold

      - Indefinite

      - Day Time

      - Evening

      - Night

      - Weekend

      - Second Shift

      - Third Shift

      - Vendor Extension

    - Job Hold Until Time

    - Job Phone Number

    - Job Priority

    - Job Recipient Name

    - Job Accounting Sheets

      - Job Accounting Sheets Type

          - None

          - Standard

          - Vendor Extension

      - Job Accounting Output Bin

          - Top

          - Middle

          - Bottom

          - Side

          - Left

          - Right

          - Center

          - Rear

          - Face Up

          - face down

          - Large Capacity

          - Stacker 1

          - Stacker 2

          - Stacker 3

          - Stacker 4

          - Stacker 5

          - Stacker 6

          - Stacker 7

          - Stacker 8

          - Stacker 9

          - Stacker 10

          - Mailbox 1

          - Mailbox 2

          - Mailbox 3

          - Mailbox 4

          - Mailbox 5

          - Mailbox 6

          - Mailbox 7

          - Mailbox 8

          - Mailbox 9

          - Mailbox 10

          - Mailbox 11

          - Mailbox 12

          - Mailbox 13

          - Mailbox 14

          - Mailbox 15

          - Mailbox 16

          - Mailbox 17

          - Mailbox 18

          - Mailbox 19

          - Mailbox 20

          - Mailbox 21

          - Mailbox 22

          - Mailbox 23

          - Mailbox 24

          - Mailbox 25

          - My Mailbox

          - Tray 1

          - Tray 2

          - Tray 3

          - Tray 4

          - Tray 5

          - Tray 6

          - Tray 7

          - Tray 8

          - Tray 9

          - Tray 10

          - Tray 11

          - Tray 12

          - Tray 13

          - Tray 14

          - Tray 15

          - Tray 16

          - Tray 17

          - Tray 18

          - Tray 19

          - Tray 20

      - Media

          - To – Be – Determined

      - Media Type

          - To – Be – Determined

      - Media Col

          - To – Be – Determined

    - Job Collation Type

        - Uncollated Sheets

        - Collated Documents

        - Uncollated Documents

        - Vendor Extension

    - Job Copies

    - Job Cover Back

      - Cover Type

          - No Cover

          - Print None

          - Print Front

          - Print Back

          - Print Both

          - Vendor Extension

      - Media Type

          - To – Be – Determined

      - Media

          - To – Be – Determined

      - Media Col

          - To – Be – Determined

      - Vendor Extension

    - Job Cover Front

      - Cover Type

          - No Cover

          - Print None

          - Print Front

          - Print Back

          - Print Both

          - Vendor Extension

      - Media Type

          - To – Be – Determined

      - Media

          - To – Be – Determined

      - Media Col

      - To – Be – Determined

    - Job Error Sheet

      - Job Error Sheet Type

          - None

          - Standard

          - Vendor Extension

      - Job Error Sheet When

          - On Error

          - Always

          - Vendor Extension

      - Media Type

          - To – Be – Determined

      - Media

          - To – Be – Determined

      - Media Col

          - To – Be – Determined

    - Job Finishings

        - None

        - Staple

        - Punch

        - Cover

        - Bind

        - Saddle Stitch

        - Edge Stitch

        - Fold

        - Trim

        - Bale

        - Booklet Maker

        - Jog Offset

        - Staple Top Left

        - Staple Bottom Left

        - Staple Top Right

        - Staple Bottom Right

        - Edge Stitch Left

        - Edge Stitch Top

        - Edge Stitch Right

        - Edge Stitch Bottom

        - Staple Dual Left

        - Staple Dual Top

        - Staple Dual Right

        - Staple Dual Bottom

        - Bind Left

        - Bind Top

        - Bind Right

        - Bind Bottom

        - Vendor Extension

    - Finishings Col

        - Finishing Template

      - Stitching

          - Bottom

          - Top

          - Left

          - Right

          - Vendor Extension

        - Vendor Extension

    - Job Save Disposition

      - Save Disposition

          - None

          - Save Only

          - Print Save

          - Vendor Extension

      - Save Info

          - Save Document Format

          - Save Location

          - Save Name

          - Vendor Extension

      - Vendor Extension

    - Job Sheet message

    - Job Sheets

        - None

        - Standard

        - Vendor Extension

    - Job Sheets Col

      - Job Sheets

          - None

          - Standard

          - Vendor Extension

      - Media Type

          - To – Be – Determined

      - Media

          - To – Be – Determined

      - Media Col

          - To – Be – Determined

      - Vendor Extension

    - Multiple Documents Handle

        - Separate Documents Collated Copies

        - Separate Documents Uncollated Copies

        - Single Document

        - Single Document New Sheet

        - Vendor Extension

    - Output Device

    - Overrides

        - To – Be – Determined

    - Pages Per Subset

    - Proof Print

      - Proof Print Copies

      - Media Type

          - To – Be – Determined

      - Media

          - To – Be – Determined

      - Media Col

          - To – Be – Determined

    - Overrides

        - Document Copies

        - Document Numbers

        - To – Be – Determined

    - Vendor Extension

## Charset-Supplied Support Level

### Conformance: PWG:PJT:Charset-Supplied Support Level – 1

        - EBCDIC US

        - ISO 8859 1

        - US ASCII

        - UTF 8

### Conformance: PWG:PJT:Charset-Supplied Support Level – 2

        - EBCDIC US

        - ISO 8859 1

        - US ASCII

        - UTF 8

### Conformance: PWG:PJT:Charset-Supplied Support Level – 3

        - ADOBE STANDARD ENCODING

        - ADOBE SYMBOL ENCODING

        - ANSI X3 110 1983

        - ASMO 449

        - BIG5

        - BIG5 HKSCS

        - BOCU 1

        - BS 4730

        - BS VIEWDATA

        - CESU 8

        - CSA Z243 4 1985 1

        - CSA Z243 4 1985 2

        - CSA Z243 4 1985 GR

        - CSN 369103

        - DEC MCS

        - DIN 66003

        - DK US

        - DS 2089

        - EBCDIC AT DE

        - EBCDIC AT DE A

        - EBCDIC CA FR

        - EBCDIC DK NO

        - EBCDIC DK NO A

        - EBCDIC ES

        - EBCDIC ES A

        - EBCDIC ES S

        - EBCDIC FI SE

        - EBCDIC FI SE A

        - EBCDIC FR

        - EBCDIC IT

        - EBCDIC PT

        - EBCDIC UK

        - EBCDIC US

        - ECMA CYRILLIC

        - ES

        - ES2

        - EUC JP

        - EUC KR

        - EXTENDED UNIX CODE FIXED WIDTH FOR JAPANESE

        - GB18030

        - GB 1988 80

        - GB2312

        - GB 2312 80

        - GBK

        - GOST 19768 74

        - GREEK7

        - GREEK7 OLD

        - GREEK CCITT

        - HP DESKTOP

        - HP LEGAL

        - HP MATH8

        - HP PI FONT

        - HP ROMAN8

        - HZ GB 2312

        - IBM00858

        - IBM00924

        - IBM01140

        - IBM01141

        - IBM01142

        - IBM01143

        - IBM01144

        - IBM01145

        - IBM01146

        - IBM01147

        - IBM01148

        - IBM01149

        - IBM037

        - IBM038

        - IBM1026

        - IBM1047

        - IBM273

        - IBM274

        - IBM275

        - IBM277

        - IBM278

        - IBM280

        - IBM281

        - IBM284

        - IBM285

        - IBM290

        - IBM297

        - IBM420

        - IBM423

        - IBM424

        - IBM437

        - IBM500

        - IBM775

        - IBM850

        - IBM851

        - IBM852

        - IBM855

        - IBM857

        - IBM860

        - IBM861

        - IBM862

        - IBM863

        - IBM864

        - IBM865

        - IBM866

        - IBM868

        - IBM869

        - IBM870

        - IBM871

        - IBM880

        - IBM891

        - IBM903

        - IBM904

        - IBM905

        - IBM918

        - IBM SYMBOLS

        - IBM THAI

        - IEC P27 1

        - INIS

        - INIS 8

        - INIS CYRILLIC

        - INVARIANT

        - ISO 10367 BOX

        - ISO 10646 UCS 2

        - ISO 10646 UCS 4

        - ISO 10646 UCS BASIC

        - ISO 10646 UNICODE LATIN1

        - ISO 10646 UTF 1

        - ISO 2022 CN

        - ISO 2022 CN EXT

        - ISO 2022 JP

        - ISO 2022 JP 2

        - ISO 2022 KR

        - ISO 2033 1983

        - ISO 5427

        - ISO 5427 1981

        - ISO 5428 1980

        - ISO 646 BASIC 1983

        - ISO 646 IRV 1983

        - ISO 6937 2 25

        - ISO 6937 2 ADD

        - ISO 8859 1

        - ISO 8859 10

        - ISO 8859 13

        - ISO 8859 14

        - ISO 8859 15

        - ISO 8859 16

        - ISO 8859 1 WINDOWS 3 0 LATIN 1

        - ISO 8859 1 WINDOWS 3 1 LATIN 1

        - ISO 8859 2

        - ISO 8859 2 WINDOWS LATIN 2

        - ISO 8859 3

        - ISO 8859 4

        - ISO 8859 5

        - ISO 8859 6

        - ISO 8859 6 E

        - ISO 8859 6 I

        - ISO 8859 7

        - ISO 8859 8

        - ISO 8859 8 E

        - ISO 8859 8 I

        - ISO 8859 9

        - ISO 8859 9 WINDOWS LATIN 5

        - ISO 8859 SUPP

        - ISO IR 90

        - ISO UNICODE IBM 1261

        - ISO UNICODE IBM 1264

        - ISO UNICODE IBM 1265

        - ISO UNICODE IBM 1268

        - ISO UNICODE IBM 1276

        - IT

        - JIS C6220 1969 JP

        - JIS C6220 1969 RO

        - JIS C6226 1978

        - JIS C6226 1983

        - JIS C6229 1984 A

        - JIS C6229 1984 B

        - JIS C6229 1984 B ADD

        - JIS C6229 1984 HAND

        - JIS C6229 1984 HAND ADD

        - JIS C6229 1984 KANA

        - JIS ENCODING

        - JIS X0201

        - JIS X0212 1990

        - JUS I B1 002

        - JUS I B1 003 MAC

        - JUS I B1 003 SERB

        - KOI8 R

        - KOI8 U

        - KS C 5601 1987

        - KSC5636

        - LATIN GREEK

        - LATIN GREEK 1

        - LATIN LAP

        - MACINTOSH

        - MICROSOFT PUBLISHING

        - MNEM

        - MNEMONIC

        - MSZ 7795 3

        - NATS DANO

        - NATS DANO ADD

        - NATS SEFI

        - NATS SEFI ADD

        - NC NC00 10 81

        - NF Z 62 010

        - NF Z 62 010  1973

        - NS 4551 1

        - NS 4551 2

        - PC8 DANISH NORWEGIAN

        - PC8 TURKISH

        - PT

        - PT2

        - PTCP154

        - SCSU

        - SEN 850200 B

        - SEN 850200 C

        - SHIFT JIS

        - T 101 G2

        - T 61 7BIT

        - T 61 8BIT

        - TIS 620

        - UNICODE 1 1

        - UNICODE 1 1 UTF 7

        - UNKNOWN 8BIT

        - US ASCII

        - US DK

        - UTF 16

        - UTF 16BE

        - UTF 16LE

        - UTF 32

        - UTF 32BE

        - UTF 32LE

        - UTF 7

        - UTF 8

        - VENTURA INTERNATIONAL

        - VENTURA MATH

        - VENTURA US

        - VIDEOTEX SUPPL

        - VIQR

        - VISCII

        - WINDOWS 1250

        - WINDOWS 1251

        - WINDOWS 1252

        - WINDOWS 1253

        - WINDOWS 1254

        - WINDOWS 1255

        - WINDOWS 1256

        - WINDOWS 1257

        - WINDOWS 1258

        - WINDOWS 31J

# Internationalization

For interoperability and basic support for multiple languages, conforming implementations MUST support the Universal Character Set (UCS) Transformation Format -- 8 bit (UTF-8) [RFC3629] encoding of Unicode [UNICODE] [ISO10646] and the Unicode Format for Network Interchange [RFC5198].

# Security Considerations

Not acceptable for this implementation guide.

# IANA Registration Information

Not acceptable for this implementation guide.

# References

## Normative References

| [PWG5108.07] | P. Zehler, "PWG Print Job Ticket and Associated Capabilities Version 1.0 (PJT)", PWG5108.07-2012, August 1, 2012, ftp://ftp.pwg.org/pub/pwg/candidates/cs-sm20-pjt10-20120801-5108.07.pdf |
| --- | --- |

## Informative References

# Authors' Contact Information

Glen Petrie

Epson Imaging Technology Center

Email: glen.petrie@eitc.epson.com

Voice: 408.576.4131

FAX: 408.436.9508

US Mail:

Glen Petrie

Epson Imaging Technology Center

214 Devcon Drive

San Jose, CA, 95112

Additional Contributors:

Xxx

------------------------------------  End – of – Document ------------------------------------