List of Printer MIB, UPnP, and Wireless Print Document Formats

ISSUES are highlighted like this.

File: Print-MIME-tab.doc, .pdf

Date: 3/2/2001

| Enum (decimal) | Enum (hex) | Name or Acronym | MIME Media Type (already registered) | Description, including full title | Reference | Company or Organization |
| --- | --- | --- | --- | --- | --- | --- |
|  | 01 | Other |  |  |  |  |
|  | 02 | Unknown |  |  |  |  |
|  | 03 | PCL | application/vnd.hp-PCL | Starting with PCL version 5, HP-GL/2 is included as part of the PCL language. PCL and HP-GL/2 are registered trademarks of Hewlett-Packard Company. | PCL-PJL Technical Reference Manual Documentation Package HP Part No. 5012-0330 | Hewlett-Packard Company |
|  | 04 | HPGL | application/vnd.hp-HPGL | Hewlett-Packard Graphics Language. HP-GL is a registered trademark of Hewlett-Packard Company. | The HP-GL/2 and HP RTL Reference Guide Addison-Wesley Publishing Co. ISBN: 0-201-63325-6 | Hewlett-Packard Company |
|  | 05 | PJL |  | Peripheral Job Language. Appears in the data stream between data intended for a page description language |  | Hewlett-Packard Co. |
|  | 06 | PostScript (tm) Language | application/postscript | Postscript – a trademark of Adobe Systems Incorporated, which may be registered in certain jurisdictions. | RFC2046 | Adobe Systems |
|  | 07 | IPDS |  | Intelligent Printer Data Stream. Bi-directional print data stream for documents consisting of data objects (text, image, graphics, bar codes), resources (fonts, overlays) and page, form and finishing instructions. Facilitates system level device control, document tracking and error recovery throughout the print process. | Title: IPDS Reference Authors: David E. Stone Document Number: S544-3417 | IBM Corporation |
|  | 08 | PPDS |  | IBM Personal Printer Data Stream. Originally called IBM ASCII, the name was changed to PPDS when the Laser Printer was introduced in 1989 |  | Lexmark International, Inc. www.lexmark.com |
|  | 09 | ESC/P |  | ESC/P means Epson Standard Code for Printers. Epson originally developed the printer language ESC/P for use with their early dot-matrix printers. Today it is supported also by Epson inkjet and laser printers as well as by many other dot-matrix printers in the market. | ftp://ftp.epson.com/impact/ESCODE.TXT The Epson "ESC/P2 Reference Manual" includes ESC/P and ESC/P2, as well as a complete description of the differences in commands supported by each printer. The newest version is August 1992, but there is also an addendum, which includes the new Stylus Color commands, identified as: "Epson Stylus COLOR Programming Note" dated July 8, 1994. | Seiko-Epson Corporation. |
|  | 0A | Epson |  | What is this? ISSUE 01: Should we deprecate this value or clarify its definition to mean ESC/P2 (see number 68, below)? No one from Epson at meeting to answer this question. |  | Sieko-Epson Corporation. |
|  | 0B | DDIF |  | Digital Document Interchange Format |  | Digital Equipment Corp. (now Compaq) |
|  | 0C | Interpress |  | Xerox predecessor to Postscript |  | Xerox Corp. www.xerox.com |
|  | 0D | ISO 6429 |  | Control functions for Coded Character Sets (has ASCII control characters, plus additional controls for character imaging devices.) | ISO 6429 Control functions for Coded Character Sets | ISO |
|  | 0E | LineData |  | Line-data - Lines of data as separate ASCII or EBCDIC records and containing no control functions (no CR, LF, HT, FF, etc.) For use with traditional line printers. May use CR and/or LF to delimit lines, instead of records. ISSUE 02: Should we define MIME parameters to handle the variability defined in ISO DPA (and used in practice with Line Data)? | ISO 10175 Document Printing Application (DPA) ISSUE 03: Or should we reference the IBM documents directly? | ISO |
|  | 0F | MODCA | application/vnd.ibm.modcap ISSUE: 04 This value was found in the IANA MIME registry, is this the correct value? | Mixed Object Document Content Architecture. Definitions that allow the composition, interchange, and presentation of final form documents as a collection of data objects (text, image, graphics, bar codes), resources (fonts, overlays) and page, form and finishing instructions | MIME Registry talks about MO:DCA-P. ISSUE 05: Is this the same as the Printer MIB enum? Yes. “P” stands for the Presentation form of MO:DCA (as opposed to the stored form) Title: MO:DCA Reference Document Number: SC31-6802 | IBM Corporation www.ibm.com |
|  | 10 | ReGIS |  | Remote Graphics Instruction Set |  | Digital Equipment Corp. (now Compaq) |
|  | 11 | SCS |  | SNA Character String Bi-directional print data stream for SNA LU-1 mode of communication |  | IBM Corporation www.ibm.com |
|  | 12 | SPDL |  | ISO 10180 Standard Page Description Language. | ISO 10180 Standard Page Description Language | ISO |
|  | 13 | TEK4014 |  |  |  | Tektronix Corp., now part of Xerox Corp. |
|  | 14 | PDS |  |  |  | ISSUE 06: Who? |
|  | 15 | IGP |  |  |  | Printronix Corp. |
|  | 16 | CodeV |  | Magnum Code-V, Image and printer control language used to control impact/dot-matrix printers | Mabry Dozier will investigate. | QMS, Inc., Mobile AL |
|  | 17 | DSC-DSE |  | Data Stream Compatible and Emulation Bi-directional print data stream for non-SNA (DSC) and SNA LU-3 3270 controller (DSE) communications |  | IBM Corporation www.ibm.com |
|  | 18 | WPS |  | Windows Printing System, Resource based command/data stream used by Microsoft At Work Peripherals |  | Developed by the Microsoft Corporation |
|  | 19 | LN03 |  | Early DEC-PPL3 |  | Digital Equipment Corp., now Compaq |
|  | 1A | CCITT |  | Group 3 or 4 FAX. |  | ITU |
|  | 1B | QUIC |  | QUIC (Quality Information Code), Page Description Language for laser printers. Included graphics, printer control capability and emulation of other well-known printer. | Mabry Dozier will investigate. | QMS, Inc. |
|  | 1C | CPAP |  | Common Printer Access Protocol |  | Digital Equipment Corp. (now COMPAQ) |
|  | 1D | DEC-PPL |  | Digital ANSI-Compliant Printing Protocol |  | Digital Equipment Corp. (now COMPAQ) |
|  | 1E | SimpleText | No | Simple-text: Character coded data, including NUL, CR , LF, HT, and FF control characters. SimpleText is more complex than the text/plain in RFC 2046, section 4.1.2 | See ISO 10175 Document Printing Application (DPA), Appendix F.4.13 simple-text | ISO |
|  | 1F | NPAP |  | Network Printer Alliance Protocol. This protocol has been superseded by the IEEE 1284.1. |  | Network Printer Alliance (NPA) |
|  | 20 | DOC |  | Document Option Commands, Appears in the data stream between data intended for a page description | Mabry Dozier will investigate. | QMS, Inc. |
|  | 21 | ImPRESS |  | Page description language originally developed for the ImageServer product line. A binary language providing representations of text, simple graphics, and some large forms (simple bit-map and CCITT group 3/4 encoded). The language was intended to be sent over an 8-bit channel and supported early document preparation languages (e.g., TeX and TROFF) | Mabry Dozier will investigate. | QMS, Inc. |
|  | 22 | Pinwriter |  | 24 wire dot matrix printer for USA, Europe, and Asia except Japan. More widely used in Germany, and some Asian countries than in US |  | NEC Corp. |
|  | 23 | NPDL |  | Page printer for Japanese market |  | NEC Corp. |
|  | 24 | NEC201PL |  | Serial printer language used in the Japanese market |  | NEC Corp. |
|  | 25 | Automatic | application/octet-stream | Automatic PDL sensing. Automatic sensing of the interpreter language family by the printer examining the document content. Which actual interpreter language families are sensed depends on the printer implementation. | RFC2045,RFC2046 (extended by RFC2911, Internet Printing Protocol/1.1 Model and Semantics, for use in printing) | IETF |
|  | 26 | Pages |  | Page printer Advanced Graphic Escape Set |  | IBM Japan |
|  | 27 | LIPS |  | LaserBeam Image Processing System Analogous to CaPSL LIPS supports Diablo 630 (the factory setting for the command mode), ISO mode (for printing text data and raster graphics) and VDM mode (for vector graphics and character printing) | Already registered by Canon. | Canon |
|  | 28 | TIFF | image/tiff image/tiff;faxbw image/tiff;faxcolor | Tagged Image File Format | RFC2302 | Aldus (now Adobe Systems Incorporated |
|  | 29 | Diagnostic |  | A hex dump of the input to the interpreter | RFC 1759 | IETF |
|  | 2A | PSPrinter |  | The PostScript Language used for control (with any PDLs) |  | Adobe Systems Incorporated |
|  | 2B | CaPSL |  | Canon Print Systems Language Another name for this emulation is LIPS (Laser-beam Image Processing System). | Already registered by Canon. | Canon |
|  | 2C | EXCL |  | Extended Command Language |  | Talaris Systems Inc. |
|  | 2D | LCDS |  | Line Conditioned Data Stream |  | Xerox Corporation |
|  | 2E | XES |  | Xerox Escape Sequences |  | Xerox Corporation |
|  | 2F | PCLXL | application/vnd.hp-PCLXL | Printer Control Language eXtended Language features for printing, and printer control | MIME Registry has: Specification by example: Within a Windows application select: Print and select as the printer HP 'LaserJet 5/5M – Enhanced Driver' (this driver is available from www.hp.com). Select: 'Print to File' and then 'Okay' | Hewlett-Packard Co. |
|  | 30 | ART |  | Advanced Rendering Tools (ART). Page Description language originally developed for the Laser Press printers. | Technical reference manual: "ART IV Reference Manual", No F33M | Fuji Xerox Co., Ltd. |
|  | 31 | TIPSI |  | Transport Independent Printer System Interface (ref. IEEE Std. 1284.1) | IEEE Std. 1284.1 | IEEE |
|  | 32 | Prescribe |  | Page description and printer control language. It can be described with ordinary ASCII | Technical reference manual: "PRESCRIBE II Programming Manual" | Kyocera |
|  | 33 | LinePrinter |  | A simple-text character stream which supports the control codes LF, VT, FF, and plus Centronics or Dataproducts Vertical Format Unit (VFU) language is commonly used on many older model line and matrix printers. |  |  |
|  | 34 | IDP |  | Imaging Device Protocol |  | Apple Computer. |
|  | 35 | XJCL |  | Xerox Job Control Language (JCL). A Job Control language originally developed for the LaserPress printers and is capable of switching PDLs. | Technical reference manual: "ART IV Reference Manual", No F33M | Fuji Xerox Co., Ltd. |
|  | 36 | PDF | application/pdf | Adobe Portable Document Format | Version 1.0: Portable Document Format Reference Manual, Adobe Systems, Inc, Addison-Wesley Publishing Co. ISBN 0-201-62628-4, 1993. Version 1.1: An Adobe Systems, Inc Technical Note | Adobe Systems, Inc. |
|  | 37 | RPDL |  | Ricoh Page Description Language for printers. | Technical manual "RPDL command reference" No.307029 | RICOH, Co. LTD |
|  | 38 | IntermecIPL |  | Intermec Printer Language for label printers. | Technical Manual: "IPL Programmers Reference Manual" | Intermec Corporation |
|  | 39 | UBIFingerprint |  | An intelligent basic-like programming language for label printers. | Reference Manual: "UBI Fingerprint 7.1", No. 1-960434-00 | United Barcode Industries |
|  | 3A | UBIDirectProtocol |  | An intelligent control language for label printers. | Programmers Guide: "UBI Direct Protocol", No. 1-960419-00 | United Barcode Industries |
|  | 3B | Fujitsu |  | Fujitsu Printer Language | Reference Manual: "FM Printer Sequence" No. 80HP-0770 | FUJITSU LIMITED |

The following Print Document Formats have been proposed for registration with IANA and should be added to the Printer MIB as well.

| Enum (decimal) | Enum (hex) | Name or Acronym | MIME Media Type (already registered) | Description, including full title | Reference | Company or Organization |
| --- | --- | --- | --- | --- | --- | --- |
|  | 3D | vCard | text/directory |  | RFC2426 | International Mail Consortium www.imc.org |
|  | 3E | vCal | text/calendar |  | RFC2445 | International Mail Consortium www.imc.org |
|  | 3F | vNote |  |  | Infrared Data Association, IrMC (Ir Mobile Communications) Specification with Published Errata, Version 1.1, February 1999 | International Mail Consortium www.imc.org |
|  | 40 | vMessage |  |  | Infrared Data Association, IrMC (Ir Mobile Communications) Specification with Published Errata, Version 1.1, February 1999 | International Mail Consortium www.imc.org |
|  | 41 | HTML | text/html | The 'text/html' Media Type | RFC 2854, The 'text/html' Media Type. D. Connolly, L. Masinter. June 2000. | IETF www.ietf.org |
|  | 42 | XHTML | application/xhtml+xml (Internet-Draft only) | The Extensible HyperText Markup Language |  | W3C w3c.org |
|  | 43 | XHTML-Basic | no separate MIME registry planned See draft-baker-xhtml-media-reg-00.txt ISSUE 07: Or do we need a separate MIME Registry entry? NO – do nothing on MIME registering but add to MIB? | XHTML Basic |  | W3C w3c.org |
|  | 44 | XHTML-Print | application/vnd.pwg-xhtml-print+xml | XHTML- Print |  | PWG (Don Wright to register) |
|  | 45 | ESC/P2 |  | ESC/P2 means Epson Standard Code for Printers, Level 2. ESC/P2 is an enhancement of ESC/P with new functions for font scaling, raster graphic printing and more. ISSUE 08: or should ESC/P2 use the 10 value above? | ftp://ftp.epson.com/impact/ESCODE.TXT The Epson "ESC/P2 Reference Manual" includes ESC/P and ESC/P2, as well as a complete description of the differences in commands supported by each printer. The newest version is August 1992, but there is also an addendum, which includes the new Stylus Color commands, identified as: "Epson Stylus COLOR Programming Note" dated July 8, 1994. | Sieko-Epson Corporation |
|  | 46 | ditroff |  | Device-independent troff (ditroff). An ASCII description of a typeset document. The description specifies the typesetting device, the fonts, and the point sizes of characters to be used, as well as the position of each character on the page. | AIX Version 4 Files Reference | IBM |
|  | 47 | d630 |  | Originally used with daisywheel printers and typewriters. Supports tabulation, line and character spacing, attribute selection (bold, double-strike, underline), horizontal movement in both directions, proportional spacing and automatic centering and justification among others. This emulation is sometimes used as a basis for other, vendor specific, emulations. |  | Diablo? |
|  | 48 | gif | image/gif |  | [RFC2045, RFC2046] |  |
|  | 49 | jpeg | image/jpeg |  | [RFC2045, RFC2046] |  |
|  | 4A | kgl |  | KGL is a superset of postscript supported by Kodak/Heidleberg's printers |  | Kodak? Heidleberg? |
|  | 4B | sap |  | OTF data stream. (composed data) | R/3 System Generic Output Format SAPGOF | SAP AG |
|  | 4C | sap-abap |  | SAP Advanced Business Application Programming (line data) | R/3 System Generic Output Format SAPGOF | SAP AG |

  XHTML is a trademark of the World Wide Web Consortium.

  XHTML is a trademark of the World Wide Web Consortium.