Service Location Working Group                           Pete St. Pierre

INTERNET DRAFT                                          Sun Microsystems

Expires 8 September 2000                                  Scott Isaccson

Novell

Ira McDonald

High North

8 March 2000

Definition of the Printer Abstract Service Type v2.0

<draft-ietf-svrloc-printer-scheme-06.txt>

Copyright (C) The Internet Society (2000). All Rights Reserved.

Status of This Memo

This document is an Internet-Draft and is in full conformance with

all provisions of Section 10 of RFC2026.  Internet-Drafts are working

documents of the Internet Engineering Task Force (IETF), its areas,

and its working groups.  Note that other groups may also distribute

working documents as Internet-Drafts.

Internet-Drafts are draft documents valid for a maximum of six months

and may be updated, replaced, or obsoleted by other documents at any

time.  It is inappropriate to use Internet-Drafts as reference

material or to cite them other than as "work in progress."

To view the list of Internet-Draft Shadow Directories, see

http://www.ietf.org/shadow.html.

Abstract

This document is a product of the Service Location and Internet

Printing Protocol Working Groups of the Internet Engineering Task

Force (IETF).  Comments should be sent to the srvloc@srvloc.org and

ipp@pwg.org mailing lists.

This document defines the 'service:printer:' abstract type and the

attributes associated with it.  This 'service:printer:' template is

designed to be used in conjunction with the Service Location

Protocol, Version 2 (SLPv2) [13], but may be used with any directory

service which supports attribute/value pair registration, such as the

Lightweight Directory Access Protocol, Version 3 (LDAPv3 [16]).

The 'service:printer:' type is specified as an abstract service type.

Printers advertised with the 'service:printer:' abstract type may be

accessible by one or more print protocols, such as IPP [3] and LPR

[2].

St. Pierre, Isaacson, McDonald     Expires 8 September 2000     [Page 1]

Internet Draft      Printer Abstract Service Type v2.0      3 March 2000

Table of Contents

1.  Introduction ...............................................       3

2.  Terminology ................................................       3

3.  Template url-path Definition ...............................       4

3.1.  IPP ....................................................       4

3.2.  LPR ....................................................       4

4.  Printer Service Template ...................................       5

5.  Summary of Required Attributes .............................      16

5.1.  Rationale for Required Attributes ......................      16

6.  Summary of Recommended Attributes ..........................      17

6.1.  Rationale for Recommended Attributes ...................      17

7.  Summary of Optional Attributes .............................      18

7.1.  Rationale for Optional Attributes ......................      19

8.  Summary of Attribute Origins ...............................      20

9.  IANA Considerations ........................................      21

10.  Internationalization Considerations .......................      21

11.  Security Considerations ...................................      21

12.  References ................................................      22

13.  Acknowledgments ...........................................      24

14.  Authors' Addresses ........................................      24

15.  Full Copyright Statement ..................................      25

St. Pierre, Isaacson, McDonald     Expires 8 September 2000     [Page 2]

Internet Draft      Printer Abstract Service Type v2.0      3 March 2000

1.  Introduction

Service Type templates are used to describe in a standard way those

services which use the 'service:' URL.  The template described in

this document is an abstract service type that includes concrete

types such as the printing protocol defined in RFC 1179 [2], "Line

Printer Daemon Protocol".  This template defines the

'service:printer:' abstract type.

The 'service:printer:' abstract type supports consistent registration

of printer services.  The attributes described within this document

are derived from previous specifications developed by the IETF IPP WG

and IETF Printer MIB WG.

The attributes specified are intended to facilitate both automatic

and manual selection of services, not to provide service state,

statistics, or notification.  The target audience of service

attributes consists of users, either software agents or end users.

2.  Terminology

The key words "MUST", "MUST NOT", "REQUIRED", "SHALL", "SHALL NOT",

"SHOULD", "SHOULD NOT", "RECOMMENDED", "MAY", and "OPTIONAL" in this

document are to be interpreted as described in RFC 2119 [14].

St. Pierre, Isaacson, McDonald     Expires 8 September 2000     [Page 3]

Internet Draft      Printer Abstract Service Type v2.0      3 March 2000

3.  Template url-path Definition

The 'service:printer:' type is an abstract type.  Because an abstract

service type may contain one of many concrete types, the 'url-path'

associated with a 'service:printer:' URL may vary.  IPP and LPR

printers are presently supported by this 'service:printer:' template.

Other printer protocols MAY be supported by extension templates or by

future updates of this abstract 'service:printer:' template.

3.1.  IPP

An IPP Printer object is defined in "IPP/1.1 Model and Semantics"

[3].  An IPP printer concrete URL is defined in "IPP/1.1 Encoding and

Transport" [9].

Examples of valid 'service:' URLs for IPP printers include:

service:printer:ipp://server.sun.com/cgi-bin/public-printer

service:printer:ipp://foo.sun.com/printers?public-printer

3.2.  LPR

An LPR accessible printer MAY include a queue name as the last part

of the URL.  If the queue name is absent, the print server is assumed

to use a default print queue.  The definition of an LPR printer

concrete URL is specified in this document, in the 'service:printer:'

template.

Examples of valid 'service:' URLs for LPR printers include:

service:printer:lpr://server.sun.com

service:printer:lpr://server.sun.com/public-printer

St. Pierre, Isaacson, McDonald     Expires 8 September 2000     [Page 4]

Internet Draft      Printer Abstract Service Type v2.0      3 March 2000

4.  Printer Service Template

The 'service:printer:' template defined below conforms to the grammar

described in "Service Templates and service: Schemes".  Please refer

to [4] for detailed explanation of the syntax.

Name of submitters: Pete St. Pierre <Pete.StPierre@eng.sun.com>

Scott Isaacson <sisaacson@novell.com>

Ira McDonald <imcdonald@sharplabs.com>

Language of service template: en (English)

Security Considerations:

By advertising the security methods for each supported printer URL

the printer may expose information useful to attackers.  SLPv2

security or another security method SHOULD be used to authenticate

any service advertisements.

Template Text:

-------------------- template begins here --------------------

template-type = printer

template-version = 2.0

template-description =

The 'service:printer:' template describes the attributes

supported by network printing devices.  Devices may be

either directly connected to a network or managed by a

print server.  The device or server understands one or

more network print protocols such as IPP or LPR.

template-url-syntax =

url-path    = ippurl / lprurl

ippurl      = IPP URL as defined in [9]

lprurl      = "lpr://" hostport [ "/" qname ]

hostport    = host [ ":" port ]

host        = hostname / hostnumber

hostname    = *( domainlabel "." ) toplabel

domainlabel = alphanum /

alphanum * [alphanum / "-"] alphanum

toplabel    = alpha / alpha * [alphanum / "-"] alphanum

hostnumber  = ipv4-number / ipv6-number

ipv4-number = 1*3digit 3*3("." 1*3digit)

ipv6-number = 32*hex

3digit      = digit digit digit

port        = 1*digit

alphanum    = alpha / digit

alpha       = "a" / "b" / "c" / "d" / "e" / "f" / "g" /

"h" / "i" / "j" / "k" / "l" / "m" / "n" /

St. Pierre, Isaacson, McDonald     Expires 8 September 2000     [Page 5]

Internet Draft      Printer Abstract Service Type v2.0      3 March 2000

"o" / "p" / "q" / "r" / "s" / "t" / "u" /

"v" / "w" / "x" / "y" / "z" /

"A" / "B" / "C" / "D" / "E" / "F" / "G" /

"H" / "I" / "J" / "K" / "L" / "M" / "N" /

"O" / "P" / "Q" / "R" / "S" / "T" / "U" /

"V" / "W" / "X" / "Y" / "Z"

digit       = "0" / "1" / "2" / "3" / "4" / "5" / "6" /

"7" / "8" / "9"

printer-xri-supported = STRING L

# REQUIRED for all registrations by conforming SLP SAs

# IPP 'printer-uri-supported'

# IPP 'uri-authentication-supported'

# IPP 'uri-authentication-supported'

# The unordered list of XRI (extended resource identifiers)

# supported by this printer.  Each member of the list

# consists of a URI (uniform resource identifier) followed

# by optional authentication and security metaparameters.

# The keywords for URI and their metaparameters are:

#   'uri'  == IPP 'printer-uri-supported' value

#   'auth' == IPP 'uri-authentication-supported' value

#   'sec'  == IPP 'uri-security-supported' value

# Legal values of the 'auth' metaparameter include

#   'none' (no authentication for this URI)

#   'requesting-user-name' (from operation request)

#   'basic' (HTTP/1.1 Basic, RFC 2617 [18])

#   'digest' (HTTP/1.1 Basic, RFC 2617 [18])

#   'certificate' (from certificate)

# per IPP Model [3] (extensions MAY also be used).

# A missing 'auth' metaparameter SHALL mean 'none'.

# Legal values of the 'sec' metaparameter include

#   'none' (no security for this URI)

#   'ssl3' (Netscape SSL3)

#   'tls' (IETF TLS/1.0, RFC 2246 [15])

# per IPP Model [3] (extensions MAY also be used).

# A missing 'sec' metaparameter SHALL mean 'none'.

# Each metaparameter of a list member is delimited by '<'.

# Each list member (including the last) is delimited by '>'.

# For example:

#   'uri=ipp://foo.com< auth=digest< sec=tls< >

#   'uri=lpr://bar.com< auth=none< sec=none< >

# Registrations MAY consolidate values for metaparameters,

# as in the following example:

#   'uri=ipp://foo.com< auth=basic,digest< sec=tls,ssl3< >

printer-name = STRING

# REQUIRED for all registrations by conforming SLP SAs

# IPP 'printer-name'

# The site-specific administrative name of this printer.

# This value of this attribute SHOULD be in the language

# specified in 'printer-natural-language-configured'

St. Pierre, Isaacson, McDonald     Expires 8 September 2000     [Page 6]

Internet Draft      Printer Abstract Service Type v2.0      3 March 2000

# (although the printer's name may be in any language).

# This name MAY be the last part of the printer's URI or it

# MAY be completely unrelated.

# This name MAY contain characters that are not allowed in a

# conventional URI (which conforms to RFC 2396 [17]).

# If this printer information is not known to the SLP SA,

# then this attribute SHALL be set to 'unknown'.

printer-natural-language-configured = STRING L O

unknown

# RECOMMENDED for all registrations by conforming SLP SAs

# IPP 'natural-language-configured'

# The configured language in which error and status messages

# will be generated (by default) by this printer.

# Also, the (declared) language of the attributes

# 'printer-name', 'printer-location', 'printer-info',

# 'printer-make-and-model', 'printer-media-local-supported',

# 'printer-current-operator', 'printer-service-person'.

# For example:

#   'en-us' (US English) or 'fr-fr' (French in France)

# Legal values of language tags conform to RFC 1766

# 'Tags for the Identification of Languages' [5].

# For coherence with IPP Model [3], language tags in this

# attribute SHALL be lowercase normalized.

# If this printer information is not known to the SLP SA,

# then this attribute SHALL be set to 'unknown'.

# However, it is STRONGLY RECOMMENDED that this attribute

# be supported and that it be set to a meaningful value.

printer-location = STRING O

unknown

# RECOMMENDED for all registrations by conforming SLP SAs

# IPP 'printer-location'

# The physical location of this printer.

# This value of this attribute SHOULD be in the language

# specified in 'printer-natural-language-configured'.

# A free form description of this printer's physical

# location, for example:

#   'Orlando office, 2nd floor, near the fire escape'

# If this printer information is not known to the SLP SA,

# then this attribute SHALL be set to 'unknown'.

# However, it is STRONGLY RECOMMENDED that this attribute

# be supported and that it be set to a meaningful value so

# that users can determine this printer's physical location

# for printer selection (before print job submission) and

# for output collection (after print job completion).

printer-info = STRING O

unknown

# IPP 'printer-info'

# Additional descriptive information on this printer.

St. Pierre, Isaacson, McDonald     Expires 8 September 2000     [Page 7]

Internet Draft      Printer Abstract Service Type v2.0      3 March 2000

# This value of this attribute SHOULD be in the language

# specified in 'printer-natural-language-configured'.

# A free form string that may contain any site-specific

# descriptive information about this printer.

# If this printer information is not known to the SLP SA,

# then this attribute SHALL be set to 'unknown'.

printer-more-info = STRING L O

unknown

# IPP 'printer-more-info'

# A URI used to obtain more information about this specific

# printer.  For example, this could be an 'http:' URI

# referencing an HTML page accessible to a Web Browser.

# The information obtained from this URI is intended for

# end user consumption.

# If this printer information is not known to the SLP SA,

# then this attribute SHALL be set to 'unknown'.

printer-make-and-model = STRING O

unknown

# IPP 'printer-make-and-model'

# The manufacturer, make, and model of this printer,

# a simple text string defined by the manufacturer.

# This value of this attribute SHOULD be in the language

# specified in 'printer-natural-language-configured'.

# If this printer information is not known to the SLP SA,

# then this attribute SHALL be set to 'unknown'.

printer-ipp-versions-supported = STRING L M O

none

# IPP 'ipp-versions-supported'

# The unordered list of 'major.minor' IPP versions

# supported by this printer or 'none'.

# For example:

#   '1.0, 1.1'

# Legal values include 'none', '1.0', and '1.1',

# per IPP Model [3] (extensions MAY also be used).

# If this printer information is not known to the SLP SA,

# then this attribute SHALL be set to 'none'.

printer-multiple-document-jobs-supported = STRING L O

unknown

# IPP 'multiple-document-jobs-supported'

# Indicates whether this printer supports more than one

# document per job.

# This template attribute is an extended boolean.

# If this printer information is not known to the SLP SA,

# then this attribute SHALL be set to 'unknown'.

unknown, true, false

printer-charset-configured = STRING L O

St. Pierre, Isaacson, McDonald     Expires 8 September 2000     [Page 8]

Internet Draft      Printer Abstract Service Type v2.0      3 March 2000

utf-8

# IPP 'charset-configured'

# The configured charset in which error and status messages

# will be generated (by default) by this printer.

# For example:

#   'utf-8' (ISO 10646/Unicode) or 'iso-8859-1' (Latin1)

# Legal values of charset tags (names/aliases) come from

# the IANA Registry of Coded Character Sets [6] and

# the '(preferred MIME name)' SHALL be used as the tag.

# For coherence with IPP Model [3], charset tags in this

# attribute SHALL be lowercase normalized.

printer-charset-supported = STRING L M O

utf-8

# IPP 'charset-supported'

# The possible charsets in which error and status messages

# may be generated (if requested) by this printer.

# Also, the possible charsets for printer string attributes

# set by operator, system administrator, or manufacturer.

# For example:

#   'utf-8' (ISO 10646/Unicode) or 'iso-8859-1' (Latin1)

# Legal values of charset tags (names/aliases) come from

# the IANA Registry of Coded Character Sets [6] and

# the '(preferred MIME name)' SHALL be used as the tag.

# This attribute SHALL include the value 'utf-8', per

# RFC 2277, 'IETF Policy on Charsets and Languages' [12].

# For coherence with IPP Model [3], charset tags in this

# attribute SHALL be lowercase normalized.

printer-generated-natural-language-supported = STRING L M O

unknown

# RECOMMENDED for all registrations by conforming SLP SAs

# IPP 'generated-natural-language-supported'

# The possible languages in which error and status messages

# may be generated (if requested) by this printer.

# Also, the possible languages for printer string attributes

# set by operator, system administrator, or manufacturer.

# For example:

#   'en-us' (US English) or 'fr-fr' (French in France)

# Legal values of language tags conform to RFC 1766

# 'Tags for the Identification of Languages' [5].

# For coherence with IPP Model [3], language tags in this

# attribute SHALL be lowercase normalized.

# If this printer information is not known to the SLP SA,

# then this attribute SHALL be set to 'unknown'.

# However, it is STRONGLY RECOMMENDED that this attribute

# be supported and that it be set to a meaningful value.

printer-document-format-supported = STRING L M O

unknown

# RECOMMENDED for all registrations by conforming SLP SAs

St. Pierre, Isaacson, McDonald     Expires 8 September 2000     [Page 9]

Internet Draft      Printer Abstract Service Type v2.0      3 March 2000

# IPP 'document-format-supported'

# The possible document formats in which data may be

# interpreted and printed by this printer.

# Legal values of document formats (MIME types) come from

# the IANA Registry of Internet Media Types [11].

# If this printer information is not known to the SLP SA,

# then this attribute SHALL be set to 'unknown'.

# However, it is STRONGLY RECOMMENDED that this attribute

# be supported and that it be set to a meaningful value.

printer-color-supported = STRING L O

unknown

# IPP 'color-supported'

# Indicates whether this printer is capable of any type of

# color printing at all, including highlight color.

# This template attribute is an extended boolean.

# If this printer information is not known to the SLP SA,

# then this attribute SHALL be set to 'unknown'.

unknown, true, false

printer-compression-supported = STRING L M O

none

# RECOMMENDED for all registrations by conforming SLP SAs

# IPP 'compression-supported'

# The unordered list of compression algorithms supported

# by this printer.

# For example:

#   'deflate, gzip'

# Legal values include

#   'none'

#   'deflate' (public domain ZIP)

#   'gzip' (GNU ZIP)

#   'compress' (UNIX)

# per IPP Model [3] (extensions MAY also be used).

# If this printer information is not known to the SLP SA,

# then this attribute SHALL be set to 'none'.

# However, it is STRONGLY RECOMMENDED that this attribute

# be supported and that it be set to a meaningful value.

printer-pages-per-minute = INTEGER O

-1

# IPP 'pages-per-minute'

# The nominal number of pages per minute which may be

# output by this printer (e.g., simplex, black-and-white).

# This attribute is informative, NOT a service guarantee.

# Typically, it is the value used in marketing literature

# to describe this printer.

# A value of '-1' indicates unknown.

# If this printer information is not known to the SLP SA,

# then this attribute SHALL be set to '-1'.

St. Pierre, Isaacson, McDonald    Expires 8 September 2000     [Page 10]

Internet Draft      Printer Abstract Service Type v2.0      3 March 2000

printer-pages-per-minute-color = INTEGER O

-1

# IPP 'pages-per-minute-color'

# The nominal number of color pages per minute which may be

# output by this printer (e.g., simplex, color).

# This attribute is informative, NOT a service guarantee.

# Typically, it is the value used in marketing literature

# to describe this printer.

# A value of '-1' indicates unknown.

# If this printer information is not known to the SLP SA,

# then this attribute SHALL be set to '-1'.

printer-finishings-supported = STRING L M O

none

# IPP 'finishings-supported'

# The possible finishing operations supported by this

# printer.

# Legal values of finishings are defined in IPP Model [3].

# If this printer information is not known to the SLP SA,

# then this attribute SHALL be set to 'none'.

none, staple, punch, cover, bind,

saddle-stitch, edge-stitch,

staple-top-left, staple-bottom-left,

staple-top-right, staple-bottom-right,

edge-stitch-left, edge-stitch-top,

edge-stitch-right, edge-stitch-bottom,

staple-dual-left, staple-dual-top,

staple-dual-right, staple-dual-bottom

printer-number-up-supported = INTEGER M O

1

# IPP 'number-up-supported'

# The possible numbers of print-stream pages to impose

# upon a single side of an instance of a selected

# medium.

# Legal values of this attribute include 1, 2, and 4 [3].

# Implementations may support other values.

# If this printer information is not known to the SLP SA,

# then this attribute SHALL be set to '1'.

printer-sides-supported = STRING L M O

one-sided

# IPP 'sides-supported'

# (Compare with Salutation 'duplex-mode')

# The number of impression sides (one or two) and the

# two-sided impression rotations supported by this printer.

# Salutation 'simplex' == IPP 'one-sided'

# Salutation 'duplex' == IPP 'two-sided-long-edge'

# Salutation 'tumble' == IPP 'two-sided-short-edge'

# If this printer information is not known to the SLP SA,

St. Pierre, Isaacson, McDonald    Expires 8 September 2000     [Page 11]

Internet Draft      Printer Abstract Service Type v2.0      3 March 2000

# then this attribute SHALL be set to 'one-sided'.

one-sided, two-sided-long-edge, two-sided-short-edge

printer-media-supported = STRING L M O

unknown

# IPP 'media-supported' - only the standard keyword values

# The standard names/types/sizes (and optional color

# suffixes) of the media supported by this printer.

# The values specified are NOT localized according to

# 'printer-natural-language-configured', but are in a

# fixed language of 'en-us' (US English).  For example:

#   'iso-a4' or 'envelope' or 'na-letter-white'

# Legal values of this attribute conform to ISO 10175

# 'Document Printing Application (DPA)' [10] and

# any IANA registered extensions, per IPP Model [3].

# If this printer information is not known to the SLP SA,

# then this attribute SHALL be set to 'unknown'.

printer-media-local-supported = STRING M O

unknown

# IPP 'media-supported' - only site-specific name values

# Site-specific names of media supported by this printer.

# This value of this attribute SHOULD be in the language

# specified in 'printer-natural-language-configured'

# (although the media names may be in any language).

# For example:

#   'purchasing-form' (site-specific name)

# as opposed to (in 'media-supported' above):

#   'na-letter' (standard keyword from ISO 10175 [10]).

# If this printer information is not known to the SLP SA,

# then this attribute SHALL be set to 'unknown'.

printer-resolution-supported = STRING L M O

unknown

# IPP 'printer-resolution-supported'

# (Compare with 'printer-xri-supported' in this template)

# The unordered list of resolutions supported for printing

# documents by this printer.

# Each resolution supported is a string with 3 fields:

# 1) Crossfeed direction resolution (positive integer)

# 2) Feed direction resolution (positive integer)

# 3) Resolution unit (enumerated value - see below)

# Each resolution field is delimited by '>', for example:

#   '300> 300> dpi>, 600> 600> dpi>'

# Legal values of resolution unit are

#   'dpi' (dots per inch)

#   'dpcm' (dots per centimeter)

# per IPP Model [3] (extensions SHOULD NOT be used).

# This attribute is derived from the Printer MIB [7].

# If this printer information is not known to the SLP SA,

# then this attribute SHALL be set to 'unknown'.

St. Pierre, Isaacson, McDonald    Expires 8 September 2000     [Page 12]

Internet Draft      Printer Abstract Service Type v2.0      3 March 2000

printer-print-quality-supported = STRING L M O

unknown

# IPP 'print-quality-supported'

# The unordered list of print qualities supported for

# printing documents on this printer.

# For example:

#   'draft, normal'

# Legal values are defined in IPP Model [3].

# If this printer information is not known to the SLP SA,

# then this attribute SHALL be set to 'unknown'.

unknown, draft, normal, high

printer-job-priority-supported = INTEGER O

1

# IPP 'job-priority-supported'

# (Compare with Salutation 'priority-levels-supported')

# Indicates the number of job priority levels supported.

# An IPP conformant printer which supports job priority

# must always support a full range of priorities from

# '1' to '100' (to ensure consistent behavior), therefore

# this template attribute describes the 'granularity'.

# Legal values of this attribute are from '1' to '100'.

# If this printer information is not known to the SLP SA,

# then this attribute SHALL be set to '1'.

printer-copies-supported = INTEGER O

-1

# IPP 'copies-supported'

# The maximum number of copies of a document

# that may be printed as a single job.

# A value of '0' indicates no maximum limit.

# A value of '-1' indicates unknown.

# If this printer information is not known to the SLP SA,

# then this attribute SHALL be set to '-1'.

printer-job-k-octets-supported = INTEGER O

-1

# IPP 'job-k-octets-supported'

# The maximum size in kilobytes (1,024 octets actually)

# incoming print job that this printer will accept.

# A value of '0' indicates no maximum limit.

# A value of '-1' indicates unknown.

# If this printer information is not known to the SLP SA,

# then this attribute SHALL be set to '-1'.

printer-current-operator = STRING M O

unknown

# Printer MIB 'prtGeneralCurrentOperator'

# The name of the current human operator responsible for

# operating this printer.  It is suggested that this string

St. Pierre, Isaacson, McDonald    Expires 8 September 2000     [Page 13]

Internet Draft      Printer Abstract Service Type v2.0      3 March 2000

# include information that would enable other humans to

# reach the operator, such as a telephone number.

# This value of this attribute SHOULD be in the language

# specified in 'printer-natural-language-configured'

# (although the person's name may be in any language).

# If this printer information is not known to the SLP SA,

# then this attribute SHALL be set to 'unknown'.

printer-service-person = STRING M O

unknown

# Printer MIB 'prtGeneralServicePerson'

# The name of the current service person responsible for

# servicing this printer.  It is suggested that this string

# include information that would enable other humans to

# reach the service person, such as a telephone number.

# This value of this attribute SHOULD be in the language

# specified in 'printer-natural-language-configured'

# (although the person's name may be in any language).

# If this printer information is not known to the SLP SA,

# then this attribute SHALL be set to 'unknown'.

printer-delivery-orientation-supported = STRING L M O

unknown

# Printer MIB 'prtOutputPageDeliveryOrientation'

# The possible delivery orientations of pages as they are

# printed and ejected from this printer.

# If this printer information is not known to the SLP SA,

# then this attribute SHALL be set to 'unknown'.

unknown, face-up, face-down

printer-stacking-order-supported = STRING L M O

unknown

# Printer MIB 'prtOutputStackingOrder'

# (Compare with Salutation 'paper-output')

# The possible stacking order of pages as they are printed

# and ejected from this printer.

# If this printer information is not known to the SLP SA,

# then this attribute SHALL be set to 'unknown'.

unknown, first-to-last, last-to-first

printer-output-features-supported = STRING L M O

unknown

# Printer MIB 'prtOutputFeaturesGroup'

# (Compare with Salutation 'paper-output')

# The possible output features supported by this printer.

# 'bursting' == Printer MIB 'prtOutputBursting'

# 'decollating' == Printer MIB 'prtOutputDecollating'

# 'page-collating' == Printer MIB 'prtOutputPageCollated'

# 'offset-stacking' == Printer MIB 'prtOutputOffestStacking'

# If this printer information is not known to the SLP SA,

# then this attribute SHALL be set to 'unknown'.

St. Pierre, Isaacson, McDonald    Expires 8 September 2000     [Page 14]

Internet Draft      Printer Abstract Service Type v2.0      3 March 2000

unknown, bursting, decollating, page-collating,

offset-stacking

-------------------- template ends here ----------------------

St. Pierre, Isaacson, McDonald    Expires 8 September 2000     [Page 15]

Internet Draft      Printer Abstract Service Type v2.0      3 March 2000

5.  Summary of Required Attributes

Attributes are associated with 'service:' URLs in order to provide a

dynamic search capability to service location clients.  SLP SAs

SHOULD include OPTIONAL attributes in their service registrations.

Two (2) of the attributes defined in 'service:printer:' are REQUIRED

in all compliant service registrations by SLP SAs:

SLP Required Attribute                              IPP Generic Schem

----------------------                              -----------------

printer-xri-supported                               r

printer-name                                        r

r   RECOMMENDED - Appendix E 'Generic Directory Schema' IPP Model [3]

REQUIRED - SLP 'service:printer:' template

m   Missing - Appendix E 'Generic Directory Schema' IPP Model [3]

REQUIRED - SLP 'service:printer:' template

5.1.  Rationale for Required Attributes

'printer-xri-supported'

- REQUIRED in SLP 'service:printer:' to consolidate values from the

following three IPP attributes (without parallel ordered attributes):

'printer-uri-supported', 'uri-authentication-supported', and

'uri-security-supported'.

All other REQUIRED SLP 'service:printer:' template attributes are

RECOMMENDED in Appendix E 'Generic Directory Schema' IPP Model [3].

St. Pierre, Isaacson, McDonald    Expires 8 September 2000     [Page 16]

Internet Draft      Printer Abstract Service Type v2.0      3 March 2000

6.  Summary of Recommended Attributes

Attributes are associated with 'service:' URLs in order to provide a

dynamic search capability to service location clients.  SLP SAs

SHOULD include RECOMMENDED attributes in their service registrations.

Some of the attributes defined in 'service:printer:' are RECOMMENDED

in all compliant service registrations by SLP SAs:

SLP Recommended Attribute                           IPP Generic Schem

-------------------------                           -----------------

printer-location                                    o

printer-natural-language-configured                 m

printer-generated-natural-language-supported        o

printer-document-format-supported                   r

printer-compression-supported                       r

r   RECOMMENDED - Appendix E 'Generic Directory Schema' IPP Model [3]

OPTIONAL - SLP 'service:printer:'' template (with specified defau

o   OPTIONAL - Appendix E 'Generic Directory Schema' IPP Model [3]

OPTIONAL - SLP 'service:printer:'' template (with specified defau

m   Missing - Appendix E 'Generic Directory Schema' IPP Model [3]

OPTIONAL - SLP 'service:printer:'' template (with specified defau

6.1.  Rationale for Recommended Attributes

'printer-location'

- RECOMMENDED in SLP 'service:printer:' template to aid users in

selecting appropriate printers (by physical proximity);

- OPTIONAL in Appendix E 'Generic Directory Schema' of IPP Model [3].

'printer-natural-language-configured'

- RECOMMENDED in SLP 'service:printer:' template to specify language

tags for 'printer-name', 'printer-location', 'printer-info',

'printer-make-and-model', 'printer-media-local-supported',

'printer-current-operator', and 'printer-service-person' attributes;

- missing in Appendix E 'Generic Directory Schema' of IPP Model [3],

because the corresponding IPP attributes are language tagged in-band

via the IPP 'textWithLanguage' syntax.

'printer-generated-natural-language-supported'

- RECOMMENDED in SLP 'service:printer:' template to aid users in

selecting appropriate printers;

- OPTIONAL in Appendix E 'Generic Directory Schema' of IPP Model [3].

All other RECOMMENDED SLP 'service:printer:' template attributes are

RECOMMENDED in Appendix E 'Generic Directory Schema' IPP Model [3].

St. Pierre, Isaacson, McDonald    Expires 8 September 2000     [Page 17]

Internet Draft      Printer Abstract Service Type v2.0      3 March 2000

7.  Summary of Optional Attributes

Attributes are associated with 'service:' URLs in order to provide a

dynamic search capability to service location clients.  SLP SAs MAY

include OPTIONAL attributes in their service registrations.

Most of the attributes defined in 'service:printer:' are OPTIONAL in

all compliant service registrations by SLP SAs:

SLP Optional Attribute                              IPP Generic Schem

----------------------                              -----------------

printer-info                                        o

printer-more-info                                   o

printer-make-and-model                              r

printer-ipp-versions-supported                      r

printer-multiple-document-jobs-supported            o

printer-charset-configured                          m (SLP enhancemen

printer-charset-supported                           o

printer-color-supported                             r

printer-finishings-supported                        r

printer-number-up-supported                         o

printer-sides-supported                             r

printer-media-supported                             r

printer-media-local-supported                       r (site-specific)

printer-pages-per-minute                            o

printer-pages-per-minute-color                      o

printer-resolution-supported                        o

printer-print-quality-supported                     o

printer-job-priority-supported                      m (SLP enhancemen

printer-copies-supported                            m (SLP enhancemen

printer-job-k-octets-supported                      m (SLP enhancemen

printer-current-operator                            m (from Printer M

printer-service-person                              m (from Printer M

printer-delivery-orientation-supported              m (from Printer M

printer-stacking-order-supported                    m (from Printer M

printer-output-features-supported                   m (from Printer M

r   RECOMMENDED - Appendix E 'Generic Directory Schema' IPP Model [3]

OPTIONAL - SLP 'service:printer:'' template (with specified defau

o   OPTIONAL - Appendix E 'Generic Directory Schema' IPP Model [3]

OPTIONAL - SLP 'service:printer:'' template (with specified defau

m   Missing - Appendix E 'Generic Directory Schema' IPP Model [3]

OPTIONAL - SLP 'service:printer:'' template (with specified defau

St. Pierre, Isaacson, McDonald    Expires 8 September 2000     [Page 18]

Internet Draft      Printer Abstract Service Type v2.0      3 March 2000

7.1.  Rationale for Optional Attributes

'printer-location', 'printer-make-and-model',

'printer-color-supported', 'printer-finishings-supported',

'printer-sides-supported', 'printer-media-supported', and

'printer-media-local-supported'

- OPTIONAL in SLP 'service:printer:' template because network print

servers may not be able to determine this information;

- RECOMMENDED in Appendix E 'Generic Directory Schema' of IPP Model

[3].

'printer-ipp-versions-supported'

- OPTIONAL in SLP 'service:printer:' template because network

printers may not support IPP;

- RECOMMENDED in Appendix E 'Generic Directory Schema' of IPP Model

[3].

'printer-charset-configured'

- OPTIONAL in SLP 'service:printer:'' template to aid users in

selecting appropriate printers;

- missing in Appendix E 'Generic Directory Schema' of IPP Model [3].

'printer-job-priority-supported', 'printer-copies-supported', and

'printer-job-k-octets-supported'

- OPTIONAL in SLP 'service:printer:'' template to aid users in

selecting appropriate printers;

- missing in Appendix E 'Generic Directory Schema' of IPP Model [3].

'printer-current-operator', 'printer-service-person',

'printer-delivery-orientation-supported',

'printer-stacking-order-supported', and

'printer-output-features-supported'

- OPTIONAL in SLP 'service:printer:'' template to aid users in

selecting appropriate printers;

- missing in Appendix E 'Generic Directory Schema' of IPP Model [3],

because they are only specified in the Printer MIB [7], not in IPP.

All other OPTIONAL SLP 'service:printer:' template attributes are

also OPTIONAL in Appendix E 'Generic Directory Schema' IPP Model [3].

St. Pierre, Isaacson, McDonald    Expires 8 September 2000     [Page 19]

Internet Draft      Printer Abstract Service Type v2.0      3 March 2000

8.  Summary of Attribute Origins

The following table summarizes the origins of all of the attributes

in the 'service:printer:' template (except 'template-xxx' generic

attributes defined in "Service Templates and service: Schemes" [4]

and required for all 'service:' templates).  The two source documents

are IPP Model [3] and Printer MIB [7].

Attribute Name                                  IPP     Printer MIB

--------------                                  ---     -----------

printer-xri-supported                           X

printer-name                                    X

printer-natural-language-configured             X

printer-location                                X

printer-info                                    X

printer-more-info                               X

printer-make-and-model                          X

printer-ipp-versions-supported                  X

printer-multiple-document-jobs-supported        X

printer-charset-configured                      X

printer-charset-supported                       X

printer-generated-natural-language-supported    X

printer-document-format-supported               X

printer-color-supported                         X

printer-compression-supported                   X

printer-pages-per-minute                        X

printer-pages-per-minute-color                  X

printer-finishings-supported                    X

printer-number-up-supported                     X

printer-sides-supported                         X

printer-media-supported                         X

printer-media-local-supported                   X

printer-resolution-supported                    X

printer-print-quality-supported                 X

printer-job-priority-supported                  X

printer-copies-supported                        X

printer-job-k-octets-supported                  X

printer-current-operator                                X

printer-service-person                                  X

delivery-orientation-supported                          X

stacking-order-supported                                X

output-features-supported                               X

'printer-xri-supported'

- REQUIRED in SLP 'service:printer:' to consolidate values from the

following three IPP attributes (without parallel ordered attributes):

'printer-uri-supported', 'uri-authentication-supported', and

'uri-security-supported'.

St. Pierre, Isaacson, McDonald    Expires 8 September 2000     [Page 20]

Internet Draft      Printer Abstract Service Type v2.0      3 March 2000

9.  IANA Considerations

Registration of this 'service:printer:' template will follow the

procedures defined in section 4 'A Procedure for Standardizing New

Service Types' and section 5 'IANA Considerations' of "Service

Templates and service: Schemes" [4].

10.  Internationalization Considerations

All attribute values in registrations using the 'service:printer:'

template MUST be encoded in UTF-8 characters, as required by "Service

Location Protocol, Version 2" [13].  Also, a language tag for all of

the textual attributes in the 'service:printer:' template SHOULD be

supplied in the attribute 'printer-natural-language-configured'.

Therefore, all registrations using the 'service:printer:' template

conform to "IETF Policy on Character Sets and Languages" [12].

11.  Security Considerations

When used with SLPv2, this 'service:printer:' template inherits all

of the security issues described in section 17 'Security

Considerations' of "Service Location Protocol, Version 2" [13].

By advertising the security methods for each supported printer URL

the printer may expose information useful to attackers.  SLPv2

security or another security method SHOULD be used to authenticate

any service advertisements.

St. Pierre, Isaacson, McDonald    Expires 8 September 2000     [Page 21]

Internet Draft      Printer Abstract Service Type v2.0      3 March 2000

12.  References

[1] J. Veizades, E. Guttman, C. Perkins, S. Kaplan.  Service Location

Protocol, RFC 2165, June 1997.

[2] L. McLaughlin.  Line Printer Daemon Protocol, RFC 1179, August

1990.

[3] R. deBry, T. Hastings, R. Herriot, S. Isaacson, P. Powell.

IPP/1.1:  Model and Semantics, <draft-ietf-ipp-model-v11-06.txt>

(work in progress), March 2000.

[4] E. Guttman, C. Perkins, J. Kempf.  Service Templates and service:

Schemes, RFC 2609, June 1999.  (Updates RFC 2165)

[5] H. Alvestrand.  Tags for the Identification of Languages, RFC

1766.  March 1995.

[6] IANA Registry of Coded Character Sets:

ftp://ftp.isi.edu/in-notes/iana/assignments/character-sets.

[7] R. Smith, F. Wright, T. Hastings, S. Zilles, J. Gyllenskog.

Printer MIB, RFC 1759, March 1995.

[8] Salutation Consortium, Inc.  Salutation Architecture

Specification V2.0, December 1996.

[9] R. Herriot, S. Butler, P. Moore, R. Turner.  IPP/1.1:  Encoding

and Transport, <draft-ietf-ipp-protocol-v11-05.txt> (work in

progress), March 2000.

[10] Information Technology - Document Printing Application (DPA),

ISO/IEC 10175, June 1996.

[11] IANA Registry of Internet Media Types (also called MIME types):

ftp://ftp.isi.edu/in-notes/iana/assignments/media-types

[12] H. Alvestrand.  IETF Policy on Character Sets and Languages, RFC

2277, January 1998.

[13] E. Guttman, C. Perkins, J. Veizades, M. Day.  Service Location

Protocol, Version 2, RFC 2608, June 1999.  (Updates RFC 2165)

[14] S. Bradner.  Key words for use in RFCs to Indicate Requirement

Levels, RFC 2119, March 1997.

[15] T. Dierks, C. Allen.  TLS Protocol Version 1.0, RFC 2246,

January 1999.

St. Pierre, Isaacson, McDonald    Expires 8 September 2000     [Page 22]

Internet Draft      Printer Abstract Service Type v2.0      3 March 2000

[16] M. Wahl, T. Howes, S. Kille.  Lightweight Directory Access

Protocol Version 3, RFC 2251, December 1997.

[17] T. Berners-Lee, R. Fielding, L. Masinter.  URI Generic Syntax,

RFC 2396, August 1998.

[18] J. Franks, P. Hallam-Baker, J. Hostetler, S. Lawrence, P. Leach,

A. Luotonen, L. Stewart.  HTTP Authentication: Basic and Digest

Access Authentication, RFC 2617, June 1999.

St. Pierre, Isaacson, McDonald    Expires 8 September 2000     [Page 23]

Internet Draft      Printer Abstract Service Type v2.0      3 March 2000

13.  Acknowledgments

This document is a product of the Service Location and Internet

Printing Protocol Working Groups of the Internet Engineering Task

Force (IETF).  Comments should be sent to the srvloc@srvloc.org and

ipp@pwg.org mailing lists.

Thanks to Jean Beha (Xerox), Jim Erwin (Novell), Pat Fleming (IBM),

Tom Hastings (Xerox), Bob Herriot (Xerox), Ken Jones (Sun), James

Kempf (Sun), Harry Lewis (IBM), Hugo Parra (Novell), Todd Rupper

(Novell), and Randy Turner (formerly Sharp).

Appendix E 'Generic Directory Schema' in IPP/1.1 Model [3]

(recommended and optional attributes for registering IPP printers)

was the primary input to this 'service:printer:' template.

14.  Authors' Addresses

Questions about this memo can be directed to:

Pete St. Pierre

Sun Microsystems

901 San Antonio Avenue

Palo Alto, CA 94043

USA

Phone: 650-786-5790

EMail: Pete.StPierre@eng.sun.com

Scott Isaccson

Novell

122 E 1700 S

Provo, UT 84606

USA

Phone: 801-861-7366

EMail: sisaacson@novell.com

Ira McDonald

High North

221 Ridge Ave

Grand Marais, MI 49839

USA

Phone: 906-494-2434 (or 2697)

Email: imcdonald@sharplabs.com

St. Pierre, Isaacson, McDonald    Expires 8 September 2000     [Page 24]

Internet Draft      Printer Abstract Service Type v2.0      3 March 2000

15.  Full Copyright Statement

Copyright (C) The Internet Society (2000).  All Rights Reserved.

This document and translations of it may be copied and furnished to

others, and derivative works that comment on or otherwise explain it

or assist in its implementation may be prepared, copied, published

and distributed, in whole or in part, without restriction of any

kind, provided that the above copyright notice and this paragraph are

included on all such copies and derivative works.  However, this

document itself may not be modified in any way, such as by removing

the copyright notice or references to the Internet Society or other

Internet organizations, except as needed for the purpose of

developing Internet standards in which case the procedures for

copyrights defined in the Internet Standards process must be

followed, or as required to translate it into languages other than

English.

The limited permissions granted above are perpetual and will not be

revoked by the Internet Society or its successors or assigns.

This document and the information contained herein is provided on an

"AS IS" basis and THE INTERNET SOCIETY AND THE INTERNET ENGINEERING

TASK FORCE DISCLAIMS ALL WARRANTIES, EXPRESS OR IMPLIED, INCLUDING

BUT NOT LIMITED TO ANY WARRANTY THAT THE USE OF THE INFORMATION

HEREIN WILL NOT INFRINGE ANY RIGHTS OR ANY IMPLIED WARRANTIES OF

MERCHANTABILITY OR FITNESS FOR A PARTICULAR PURPOSE."

St. Pierre, Isaacson, McDonald    Expires 8 September 2000     [Page 25]