The Printer Working Group

Internet Printing Protocol (IPP):

Document object

15 ISSUES are highlighted like this.

Draft 0.3 for 5100.5-2002

September 9, 2002

The Printer Working Group Standard for

Internet Printing Protocol (IPP):

Document Object

Abstract: This IPP specification extends the IPP Model and Semantics [RFC2911] by definiing a Document object.  The [RFC2911] Job object is extended to contain one or more Document objects which are passive objects operated on by the Job.  The Document object is created by the [RFC2911] Send-Document and Send-URI operations.  The extension also adds a Document Attributes group (and tag) to these requests which contains any Document Template attributes to be applied to this Document object being created, overriding any corresponding Job Template attribute supplied at the job level (including the “document-overrides” operation attributes at the Job or Document level).  Also there are seven new operations defined for Documents once they have been created: Cancel-Current-Document, Cancel-Document, Get-Document-Attributes, Get-Documents, Delete-Document, Set-Document-Attributes, and Validate-Document.  This specification also lists all of the attributes defined in other IPP specifications to show their relationship to corresponding attributes defined in this IPP specification for use with the Document object.  The full semantics of the “document-state” and “document-state-reasons” Document Description attributes is given along side of the corresponding [RFC2911] “job-state” and “job-state-reasons” Job Description attributes.

The purpose for specifying the Document object is so that we can have a common semantic specification for use in IPP, the PWG Semantic Model, the PWG PSI project, and the Free Standards Group Job Ticket API which all have a Document object.

This document is available electronically at:

ftp://ftp.pwg.org/pub/pwg/ipp/new_DOC/IPP-document-object.pdf, .doc, .rtf

Copyright (C) 2001, IEEE ISTO. All rights reserved.

This document may be copied and furnished to others, and derivative works that comment on, or otherwise explain it or assist in its implementation may be prepared, copied, published and distributed, in whole or in part, without restriction of any kind, provided that the above copyright notice, this paragraph and the title of the Document as referenced below are included on all such copies and derivative works.  However, this document itself may not be modified in any way, such as by removing the copyright notice or references to the IEEE-ISTO and the Printer Working Group, a program of the IEEE-ISTO.

Title:  The Printer Working Group Standard for the Internet Printing Protocol (IPP): Document Object

The IEEE-ISTO and the Printer Working Group DISCLAIM ANY AND ALL WARRANTIES, WHETHER EXPRESS OR IMPLIED INCLUDING (WITHOUT LIMITATION) ANY IMPLIED WARRANTIES OF MERCHANTABILITY OR FITNESS FOR A PARTICULAR PURPOSE.

The Printer Working Group, a program of the IEEE-ISTO, reserves the right to make changes to the document without further notice.  The document may be updated, replaced or made obsolete by other documents at any time.

The IEEE-ISTO takes no position regarding the validity or scope of any intellectual property or other rights that might be claimed to pertain to the implementation or use of the technology described in this document or the extent to which any license under such rights might or might not be available; neither does it represent that it has made any effort to identify any such rights.

The IEEE-ISTO invites any interested party to bring to its attention any copyrights, patents, or patent applications, or other proprietary rights which may cover technology that may be required to implement the contents of this document. The IEEE-ISTO and its programs shall not be responsible for identifying patents for which a license may be required by a document and/or IEEE-ISTO Industry Group Standard or for conducting inquiries into the legal validity or scope of those patents that are brought to its attention. Inquiries may be submitted to the IEEE-ISTO by e-mail at:

ieee-isto@ieee.org.

The Printer Working Group acknowledges that the IEEE-ISTO (acting itself or through its designees) is, and shall at all times, be the sole entity that may authorize the use of certification marks, trademarks, or other special designations to indicate compliance with these materials.

Use of this document is wholly voluntary.  The existence of this document does not imply that there are no other ways to produce, test, measure, purchase, market, or provide other goods and services related to its scope.

About the IEEE-ISTO

The IEEE-ISTO is a not-for-profit corporation offering industry groups an innovative and flexible operational forum and support services.  The IEEE-ISTO provides a forum not only to develop standards, but also to facilitate activities that support the implementation and acceptance of standards in the marketplace.  The organization is affiliated with the IEEE (http://www.ieee.org/) and the IEEE Standards Association (http://standards.ieee.org/).

For additional information regarding the IEEE-ISTO and its industry programs visit http://www.ieee-isto.org.

About the IEEE-ISTO PWG

The Printer Working Group (or PWG) is a Program of the IEEE Industry Standards and Technology Organization (ISTO) with member organizations including printer manufacturers, print server developers, operating system providers, network operating systems providers, network connectivity vendors, and print management application developers.  The group is chartered to make printers and the applications and operating systems supporting them work together better.  All references to the PWG in this document implicitly mean “The Printer Working Group, a Program of the IEEE ISTO.” In order to meet this objective, the PWG will document the results of their work as open standards that define print related protocols, interfaces, procedures and conventions. Printer manufacturers and vendors of printer related software will benefit from the interoperability provided by voluntary conformance to these standards.

In general, a PWG standard is a specification that is stable, well understood, and is technically competent, has multiple, independent and interoperable implementations with substantial operational experience, and enjoys significant public support.

For additional information regarding the Printer Working Group visit:  http://www.pwg.org

Contact information:

IPP Web Page:  http://www.pwg.org/ipp/

IPP Mailing List:  ipp@pwg.org

To subscribe to the ipp mailing list, send the following email:

1) send it to majordomo@pwg.org

2) leave the subject line blank

3) put the following two lines in the message body:

subscribe ipp

end

Implementers of this specification are encouraged to join the IPP Mailing List in order to participate in any discussions of clarifications or review of registration proposals for additional names.  Requests for additional media names, for inclusion in this specification, should be sent to the IPP Mailing list for consideration.

Contents

1	Introduction and Summary	1

1.1	Scope	1

2	Terminology	2

2.1	Conformance Terminology	2

2.2	Other Terminology	2

3	Document Operations	3

4	Operation Attributes for the Job and Document Creation operations	7

4.1	Operation Attributes supplied in Job Creation and Document Creation requests	7

4.2	Operation Attributes returned in Job and Document Creation responses	8

5	Job Template and Document Template attributes	9

6	Job Description and Document Description attributes	15

6.1	document-number (integer(1:MAX)) <Document>	17

6.2	job-state (type1 enum) <Job>	17

6.3	document-state (type1 enum) <Document>	17

6.3.1	The “job-state” attribute and the Job Life Cycle	19

6.3.2	The “document-state” Document Description attribute and the Document object life Cycle	20

6.3.3	Forwarding Servers	21

6.3.4	Partitioning of Job/Document States	21

6.4	job-state-reasons  (1setOf type2 keyword)  <Job>	22

6.5	document-state-reasons  (1setOf type2 keyword)  <Document>	22

7	Printer attributes	27

8	New Values for Existing Printer Description Attributes	32

8.1	operations-supported (1setOf type2 enum)	32

9	Conformance Requirements	32

9.1	Printer conformance requirements	32

9.2	Client conformance requirements	33

10	Normative References	33

11	Informative References	34

12	IANA Considerations	35

12.1	Attribute Registrations	35

12.2	Additional Enum Attribute Value Registrations for the “operations-supported” Printer Attribute	38

12.3	Operation Registrations	38

13	Internationalization Considerations	39

14	Security Considerations	39

15	Contributors	39

16	Acknowledgments	39

17	Author’s Address	39

18	Annex B: Change Log (informative)	39

Tables

Table 1 - List of Document operations	1

Table 2 - Document operations compared to Job operations	3

Table 3 - Operation Attributes supplied in Job and Document Creation operation requests	7

Table 4 - Operation attributes returned in Job and Document Creation operation responses	9

Table 5 - Job Template and Document Template attributes	11

Table 6 - Job and Document Description attributes	16

Table 7 - "job-state" and "document-state" attribute values	18

Figure 1 - The “job-state” Job Description attribute and the Job object life cycle	20

Figure 2  The “document-state” Document Description attribute and the Document object life Cycle	21

Table 8 - "job-state-reasons" and "document-state-reasons" attribute values	23

Table 9 - Printer attributes (Job Template and Printer Description)	28

Table 10 – Operation-id assignments	32

# Introduction and Summary

This specification is a proposal to extend the IPP semantics to include a Document object.  The Job object is said to contain one or more Document objects.  The Document object is created by the existing Send-Document and Send-URI operations.  The extension is to add a Document Attributes group (and tag) to the request which contains any Document Template attributes to be applied to this Document object being created, overriding any corresponding Job Template attribute supplied at the job level (including the “document-overrides” operation attributes at the Job or Document level).

The intent is to propose this specification as an IPP extension and which will be referenced as part of the PWG Semantic Model.   The purpose for specifying the Document object, is so that we can have a common specification for use in IPP, the PWG Semantic Model, the PWG PSI project, and the Free Standards Group Job Ticket API which all have a Document object.

## Scope

Part of the extension is to allow the client to supply printing instructions, called Document Template attributes, when creating each Document object.  An IPP Printer object implementation MAY support most of the existing Job Template attributes as Document Template attributes as well.  (The PWG Semantic Model currently calls the Job Template attributes and Document Template attributes simply “Processing” attributes).

The Document object is a passive object and is operated upon by the Job.  However, there are 7 new operations that affect the Document object after it is created as summarized in Table 1.

Table 1 - List of Document operations

| Operation Name | Brief Description |
| --- | --- |
| Cancel-Current-Document | Cancel the current document that is printing. |
| Cancel-Document | Cancel the specified Document in the specified Job, but don’t remove it. |
| Get-Document-Attributes | Get requested attributes of specified Document in the specified Job. |
| Get-Documents | Get requested attributes of all of the Documents in the specified Job. |
| Delete-Document | Delete the specified Document object and its data. |
| Set-Document-Attributes | Set the supplied Document attributes on the specified Document |
| Validate-Document | Validate that the supplied attributes are supported |

This document specifies the semantics of these seven new operations by comparing them to the corresponding Job operations.

ISSUE 1a:  Do we need formal definitions of each Operation or is the comparison with Job operations in Table 2 sufficient?

This specification also specifies the life cycle of the Document object by defining the “document-state” and “document-state-reasons” attributes.  No “document-state-message” is defined since the Job’s “job-state-message” text message can identify individual documents when that is needed.  Finally, the analogous Document Descriptions attributes that correspond to the Job Description attributes are proposed.

# Terminology

This section defines terminology used throughout this document.

## Conformance Terminology

Capitalized terms, such as MUST, MUST NOT, REQUIRED, SHOULD, SHOULD NOT, MAY, NEED NOT, and OPTIONAL, have special meaning relating to conformance as defined in RFC 2119 [RFC2119] and [RFC2911] section 12.1.  If an implementation supports the extension defined in this document, then these terms apply; otherwise, they do not.  These terms define conformance to this document (and [RFC2911]) only; they do not affect conformance to other documents, unless explicitly stated otherwise.  To be more specific:

REQUIRED - an adjective used to indicate that a conforming IPP Printer implementation MUST support the indicated operation, object, attribute, attribute value, status code, or out-of-band value in requests and responses.  See [RFC2911] “Appendix A - Terminology for a definition of “support”.  Since support of this entire Document Object specification is OPTIONAL for conformance to IPP/1.1, the use of the term REQUIRED in this document means “REQUIRED if this OPTIONAL Document Object  specification is implemented”.

RECOMMENDED - an adjective used to indicate that a conforming IPP Printer implementation is recommended to support the indicated operation, object, attribute, attribute value, status code, or out-of-band value in requests and responses.  Since support of this entire Document Object specification is OPTIONAL for conformance to IPP/1.1, the use of the term RECOMMENDED in this document means “RECOMMENDED if this OPTIONAL Document Object  specification is implemented”.

OPTIONAL - an adjective used to indicate that a conforming IPP Printer implementation MAY, but is NOT REQUIRED to, support the indicated operation, object, attribute, attribute value, status code, or out-of-band value in requests and responses.

## Other Terminology

This document uses the same terminology as [RFC2911], such as “client”, “Printer”, “attribute”, “attribute value”, “keyword”, “operation”, “request”, “response”, and “support” with the same meaning.  In addition, the following terms are defined for use in this document:

Job Creation operation - operations that create Job objects, specifically: Print-Job, Print-URI, and Create-Job as defined in [RFC2911].

Document Creation operation - operations that create Document objects.  Specifically: Send-Document and Send-URI as defined in [RFC2911].

Document Template attribute - an attribute that the client supplies in a Document Creation operation request

ISSUE 1b:  Should Print-Job and Print-URI also be called Document Creation operations since they create one Document object?  The answer probably depends on the answer to the following issue:

ISSUE 1c:  We’re adding a Document Attribute group (tag) to Send-Document and Send-URI so that a client can supply Document Template attributes.  OK that we don’t add this group to Print-Job and Print-URI? The problem with adding the Document Template attribute group to Print-Job and Print-URI requests is that there becomes two ways for a client to supply many of the Job Template attributes that are Document Processing attributes, such as “media”, “number-up”, etc.,  Those two ways in a Print-Job or Print-URI are (1) in the Job Template attribute group or (2) in the Document Template attribute group.  The PWG Semantic Model is defining each Processing attribute to be either a Job Processing attribute or a Document Processing attribute, but not both, so that there will only be one way to submit each Processing attribute in a PrintJob operation according to whether it is defined to be a Job Processing Attribute or a Document Processing Attribute.  But for backward compatibility, IPP’s Job Template attributes are a mixture of Job and Document Processing attributes, i.e., “job-priority” and “job-hold-until” versus “media” and “number-up” are passed mixed together in the Job Template attribute group.

Document Overrides - a way for a client to specify attributes to apply to specified documents in a multi-document job.  Specifically, as member attributes of the “document-overrides” Job Template attribute in a Job Creation operation or the “document-overrides” operation attribute in a Document Creation operation as defined in [override].

# Document Operations

The Document object semantics are as parallel to the Job object semantics as possible.  Therefore, most of the Job operations have analogous Document operations as shown in Table 2.  Operations that can apply only to an entire job, have no analogous document operation as indicated.  This extension adds 7 new Document operations that operate on the Document object once it is created by either of the existing two: Send-Document and Send-URI operations.

Note:  By Document object we mean the object once it become a contained part of a Job object, not before or after.  So a Document object does not represent data in a repository.

Table 2 compares the Job and Document operations.  The two Conf. columns indicate the conformance requirements (REQUIRED = REQ, OPTIONAL = OPT) for the Job and Document operations respectively.  A “REQ” in the Document Conformance column means if the Document object is supported.  Whether a Document operation is a method on the Job object or the Document object depends on the modeling regime and is not an essential part of the semantics described here, since these semantics are intended to apply to both to object-oriented and object-based protocols, APIs, and implementations.

ISSUE 1d:  Agree that the Cancel-Current-Document, Delete-Document, and Set-Document-Attributes operations are OPTIONAL and that the Cancel-Document, Get-Document-Attributes, Get-Documents, and Validate-Document operations are REQUIRED as indicated in Table 2 when supporting the Document object?

ISSUE 1a repeated:  Do we need formal definitions of each Operation or is the comparison with Job operations in Table 2 sufficient?

Table 2 - Document operations compared to Job operations

| Job operations | Job Conf. | Document operations | Doc Conf |
| --- | --- | --- | --- |
| Cancel-Current-Job Sets “job-state” to ‘canceled’ and “job-state-reasons” to ‘canceled-by-xxx’. Sets “document-state” to ‘canceled’ and “document-state-reasons” to ‘canceled-by-xxx’ for any documents that have not yet reached a terminal state (‘completed’, ‘canceled’, or ‘aborted’). | OPT | Cancel-Current-Document Cancels processing of the current Document no matter for which Job, set the “document-state” attributes of the current documents in the job to ‘canceled’, and proceed with processing the next Document of the current Job or with the next Job if the current Job has no more Document objects. Its an operator emergency method. In some modeling regimes, it may be a Device method, rather than a Job or Document method. Inputs: same as Cancel-Current-Job, plus an OPTIONAL “document-number” operation attribute which the Printer checks to see if the current document being processed is the specified document, else the Printer rejects the operation in a manner analogous to the handling of the OPTIONAL “job-id” input parameter to both Cancel-Current-Job and Cancel-Current-Document, if supplied. Outputs: same as Cancel-Current-Job | OPT |
| Cancel-Job Sets “job-state” to ‘canceled’ and “job-state-reasons” to ‘canceled-by-xxx’. Sets “document-state” to ‘canceled’ and “document-state-reasons” to ‘canceled-by-xxx’ for any documents that have not yet reached a terminal state (‘completed’, ‘canceled’, or ‘aborted’). | REQ | Cancel-Document Cancels the processing of the document content for the specified Document object in the job, sets the specified document’s “document-state” attribute to ‘canceled’, but does not remove the document from the Job, so that the Job can be resubmitted later. Thus Cancel-Document has the same semantics as Cancel-Job, which only cancels the processing of the job, and doesn’t delete the job/document itself. See Delete-Document to delete the Document object itself immediately. This operation does not affect the states of any of the other documents in the job. If the job was processing and there are no further documents to process, the job moves to the ‘completed’ state as usual whenever there is no more processing to be done. Note: Unlike Reprocess-Job, there is no Reprocess-Document. However, the Reprocess-Job and Restart-Job operations will process all documents in the job, including ones that have been canceled by Cancel-Document. Inputs: same as Cancel-Job, plus a REQUIRED “document-number” operation attribute. Outputs: same as Cancel-Job. | OPT |
| Create-Job The operation and Job Template attributes remain at the job level on the Job object. Document objects inherit the job level attributes unless the Document Creation operation supplies the attribute. However, the Printer MUST NOT copy down job level attributes to the Document object, including “document-overrides” (except operation attributes that initialize Document Description attributes marked with an “*” in Table 6) | OPT | N/A. See Send-Document and Send-URI |  |
| Get-Job-Attributes | REQ | Get-Document-Attributes Returns all or selected attributes and values of the specified Document object. Two new group names are defined: ‘document-template’ and ‘document-description’. Only attributes that were supplied by any client explicitly for the Document object, i.e., in a Send-Document, Send-URI, or Set-Document-Attributes operations are returned. Document attributes supplied as members of the “document-overrides” operation attribute on the Job Creation operations are not returned. In other words, a client sees what clients submitted at the Document level; the Printer MUST NOT merge the attributes that were supplied at the Job level in this response (except Document Description attributes marked with an “*” in Table 6). Inputs: same as Get-Job-Attributes, plus a REQUIRED “document-number” operation attribute. Outputs: same as Get-Job-Attributes. | REQ |
| Get-Jobs | REQ | Get-Documents Returns all or selected attributes and values of all Document objects in the specified Job. Two new group names are defined: ‘document-template’ and ‘document-description’. Only attributes that were supplied by any client explicitly for the Document object, i.e., in a Send-Document, Send-URI, or Set-Document-Attributes operations are returned. Document attributes supplied as members of the “document-overrides” operation attribute on the Job Creation operations are not returned. In other words, a client sees what clients submitted at the Document level; the Printer MUST NOT merge the attributes that were supplied at the Job level in this response (except Document Description attributes marked with an “*” in Table 6). Inputs: same as Get-Jobs, except a “which-documents” operation attribute (instead of “which-jobs” with values: ‘completed’, ‘not-completed’, and ‘all’ (default) and no equivalent to “my-jobs”. Outputs: same as Get-Jobs. | REQ |
| Hold-Job Sets “job-state” to ‘pending-held’ and “job-state-reasons” to ‘job-hold-until-specified’. The Documents’ “document-state” and “document-state-reasons” are unaffected. |  | N/A, can only hold an entire job. |  |
| Print-Job | REQ | Send-Document (existing operation - see [RFC2911]) Adds a Document object to the Job that include actual document content. The extension is to add the Document Attributes group (and tag) to the request which contains any Document Template attributes to be applied to this document, overriding any corresponding Job Template attribute supplied at the job level (including “document-overrides” operation attributes at the Job or Document level). It is a method on the Job object. | REQ |
| Print-URI | OPT | Send-URI (existing operation - see [RFC2911]) Adds a Document object to the Job that contains a URL for the specified document content. The extension is to add the Document Attributes group (and tag) to the request which contains any Job Template attributes to be applied to this document, overriding any corresponding Job Template attribute supplied at the job level (including “document-overrides” operation attributes at the Job or Document level). The device then pulls the document content data as needed. It is a method on a Job object. | OPT |
| Promote-Job | OPT | N/A, can only promote an entire job |  |
| Purge-Jobs Requires operator/administrator privileges. | OPT | Delete-Document Deletes the specified Document object and its data immediately if the Document is still in the ‘pending’ state (there is no ‘pending-held’ state for Documents), else reject the request. The “document-number” values are not re-assigned, so that the deleted document leaves a hole in the document numbering space for the job. A subsequent Get-Document-Attributes request for this document will return a ‘client-error-not-found’ status code. This operation is similar to the Purge-Jobs operation which immediately deletes all jobs from the Printer. However, Delete-Document only deletes a single Document from the Job. Requires operator/administrator privileges, otherwise a rogue client could submit, then delete, at will using up valuable spooling space. Unprivileged users use Cancel-Document which go through the normal accounting, if any. Inputs: Same as Cancel-Document. Outputs: Same as Cancel-Document. | OPT |
| Release-Held-New-Jobs | OPT | N/A, can only release entire jobs |  |
| Release-Job | OPT | N/A, can only release an entire job |  |
| Reprocess-Job | OPT | N/A, can only reprocess an entire job |  |
| Restart-Job | OPT | N/A, can only restart an entire job |  |
| Resume-Job If there are no other reasons to hold the job, changes “job-state” to ‘pending’ and removes ‘job-hold-until-specified’ value from “job-state-reasons”. The Documents’ “document-state” and “document-state-reasons” are unaffected. | OPT | N/A, can only resume an entire job |  |
| Schedule-Job-After | OPT | N/A, can only schedule an entire job |  |
| Set-Job-Attributes | OPT | Set-Document-Attributes Changes the values of the specified Document object attributes. If the specified Document object didn’t have the attribute, the attribute is added to the Document object. Such set attributes override any corresponding attributes supplied at the job level, including any “document-overrides” operation attributes supplied at the job or document level. Inputs: same as Set-Job-Attributes, plus a REQUIRED “document-number” operation attribute. Outputs: same as Set-Job-Attributes. | OPT |
| Suspend-Current-Job Sets “job-state” to ‘processing-stopped’ and “job-state-reasons” to ‘job-suspended-by-xxx’. The Documents’ “document-state” and “document-state-reasons” are unaffected. | OPT | N/A, can only suspend an entire job in order to run another one |  |
| Validate-Job | REQ | Validate-Document Checks that the print instructions and the document data are available. Note: The PWG PSI has added this operation for documents. The document data is not sent. However, if the client supplies the “document-uri” operation attribute, the Printer MUST validate that the Document exists and is accessible. Inputs: Same as Validate-Job, with the addition of an OPTIONAL “document-uri” operation attribute to be checked. Outputs: same as Validate-Job. | REQ |

# Operation Attributes for the Job and Document Creation operations

This section lists all the Operation attributes that have been defined in other IPP specifications that are used in Job Creation operations and extends their use to the Document Creation operations defined in the IPP specification.  Operation attributes relate to the operation and do not depend on the printing capabilities of the Printer.

## Operation Attributes supplied in Job Creation and Document Creation requests

Table 3 lists the Operation attributes that clients supply in Job Creation operation requests: Print-Job, Print-URI, and Create-Job and in Document Creation operation requests: Send-Document and Send-URI.  The value of the “ipp-attribute-fidelity” operation attribute does not affect the Printer’s handling of the other operation attributes, only its handling of the Job Template attributes (see section 5).

The “Supplied” column indicates whether the Operation attribute can be supplied in a Job Creation operation (Job = y) and/or a Document Creation operation (Doc = y).  The Conformance column indicates the conformance requirements for what the client supplies in a  request and the Printer supports in a request.  The Reference and Section columns list the IETF or IEEE-ISTO specification which defines the semantics of the attribute

Table 3 - Operation Attributes supplied in Job and Document Creation operation requests

| Operation attribute name for Job and Document Creation requests |  | Attribute Syntax | Supplied |  | Conformance |  |  | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Job | Doc | Client supply | Printer support |  |  |
| attributes-charset * | charset |  | y | y | MUST | MUST | [RFC2911] §3.1.4 |  |
| attributes-natural-language * | naturalLanguage |  | y | y | MUST | MUST | [RFC2911] §3.1.4 |  |
| compression | type3 keyword |  | y | y | MAY | MUST | [RFC2911] §3.2.1.1 |  |
| document-format | mimeMediaType |  | y3 | y | MAY | MUST | [RFC2911] §3.2.1.1 |  |
| document-name | name(MAX) |  | y3 | y | MAY | MUST | [RFC2911] §3.2.1.1 |  |
| document-natural-language | naturalLanguage |  | y3 | y | MAY | MAY | [RFC2911] §3.2.1.1 |  |
| document-overrides | 1setOf collection |  |  | y | MAY | MAY | [override] §5.1 |  |
| document-uri | uri |  | - | y | MAY | MAY | [RFC2911] §3.2.2 |  |
| input-document-number | integer(1:MAX) |  | - | y | MAY | MAY | [override] §9.2 |  |
| ipp-attribute-fidelity | boolean |  | y | y | MAY | MUST | [RFC2911] §3.2.1.1 |  |
| job-impressions | integer(0:MAX) |  | y | y | MAY | MAY | [RFC2911] §3.2.1.1 |  |
| job-k-octets | integer(0:MAX) |  | y | y | MAY | MAY | [RFC2911] §3.2.1.1 |  |
| job-media-sheets | integer(0:MAX) |  | y | y | MAY | MAY | [RFC2911] §3.2.1.1 |  |
| job-name | name(MAX) |  | y | - | MAY | MUST | [RFC2911] §4.3.5 |  |
| last-document | boolean |  | - | y | MAY | MUST | [RFC2911] §3.3.1.1 |  |
| operation-id * | integer(1:0x8FFF) |  | y | y | MUST | MUST | [RFC2911] §3.1.2 |  |
| original-requesting-user-name | name(MAX) |  | y | y | MAY | MAY | [adm-ops] §11.7.2.2 |  |
| page-overrides | 1setOf collection |  | - | y | MAY | MAY | [override] § |  |
| printer-uri * | uri |  | y | y | MUST | MUST | [RFC2911] §3.1.5 |  |
| request-id * | integer(1:MAX) |  | y | y | MUST | MUST | [RFC2911] §3.1.2 |  |
| requesting-user-name * | name(MAX) |  | y | y | SHOULD | MUST | [RFC2911] §3.2.1.1 |  |
| version-number * | X.Y |  | y | y | MUST | MUST | [RFC2911] §3.1.8 |  |

* - indicates attributes that are defined for all operations, not just Job Creation and Document Creation operations

## Operation Attributes returned in Job and Document Creation responses

Table 4 lists the Operation attributes that Printers return in Job Creation operation responses: Print-Job, Print-URI, and Create-Job and in Document Creation operation responses: Send-Document and Send-URI.  The column labeled “Conformance” indicates the conformance for what the Printer returns and what the Printer supports.  The Reference and Section columns list the IETF or IEEE-ISTO specification which defines the semantics of the attribute.

ISSUE 1e:  Currently the returns operation attributes are the same for Job Creation and Document Creation responses.  Should the “document-number” be added as an additional OPTIONAL response attribute to the Document Creation responses?

Table 4 - Operation attributes returned in Job and Document Creation operation responses

| Operation attribute name |  | Attribute Syntax | Conformance |  |  |  |  | References |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Values | Printer return |  | Printer support |  |  |  |
| attributes-charset * | charset |  |  | MUST |  | MUST | [RFC2911] §3.1.4 |  |
|  | ‘utf-8’ |  |  | MAY |  | MUST | [RFC2911] §4.1.7 |  |
|  | ‘us-ascii’ |  |  | MAY |  | MAY | [RFC2911] §4.1.7 |  |
|  | ‘iso-8859-1’ |  |  | MAY |  | MAY | [RFC2911] §4.1.7 |  |
|  | ‘iso-10646-ucs-2’ |  |  | MAY |  | MAY | [RFC2911] §4.1.7 |  |
| attributes-natural-language * | naturalLanguage |  |  | MUST |  | MUST | [RFC2911] §3.1.4 |  |
|  | ‘en-US’ |  |  | MAY |  | MAY | [RFC2911] §4.1.8 |  |
| detailed-status-message * | text |  |  | MAY |  | MAY | [RFC2911] §3.1.6.3 |  |
| document-access-error | text |  |  | MAY |  | MAY | [RFC2911] §3.1.6.4 |  |
| job-id | integer(1:MAX) |  |  | MUST |  | MUST | [RFC2911] §4.3.2 |  |
| job-uri | uri |  |  | MUST |  | MUST | [RFC2911] §4.3.1 |  |
| redirect-other-site | uri |  |  | MAY |  | MAY | [not-srv] §??? |  |
| request-id * | integer(1:32767) |  |  | MUST |  | MUST | [RFC2911] §3.1.2 |  |
| status-code * | integer(0:32767) |  |  | MUST |  | MUST | [RFC2911] §3.1.6.1 |  |
| status-message * | text(255) |  |  | MAY |  | MAY | [RFC2911] §3.1.6.2 |  |
| version-number * | integer(1:255), integer(0:255) |  |  | MUST |  | MUST | [RFC2911] §3.1.8 |  |

* - indicates attributes that are defined for all operations, not just Job Creation operations

# Job Template and Document Template attributes

Table 5 lists all of the Job Template attributes defined in other IPP specifications and lists their corresponding Document Template attributes defined in this IPP specification.  The Job Template attributes can be supplied in the Job Creation operations: Print-Job, Print-URI, and Create-Job operations.

A subset of these Job Template attributes  (e.g., “document-format” and “media”, but not “job-priority”) have corresponding Document Template attributes defined to apply to Documents and are called Document Template attributes.  Document Template attribute can be supplied in Document Creation operations: Send-Document and Send-URI.

These Document Template attributes can also be supplied as Document Overrides using the “document-overrides” Job Template attribute.

A subset of the Document Template attributes that apply to pages (e.g., “media”, but not “document-format” or “job-priority”) can also be supplied as Page Overrides using the “page-overrides” Job Template collection attribute (see [override]) or as a Document Template attribute.

According to [override], the “document-overrides” (collection) attribute MAY be supplied by the client in a Send-Document or Send-URI request as an Operation attribute to apply document overrides to this and/or subsequent documents in the job.  See the “document-overrides” Job Template attribute in Table 5 for the listing of the member attributes.  However, with the introduction of the Document object, the “document-overrides” (collection) attribute SHOULD NOT be used (either as a Job Template attribute or an Operation attribute).  Instead, the client simply supplies the Document Template attributes (see Table 5) for each Document Creation request (in a new Document Template attribute group) without needing a collection.

ISSUE 02:  What do we want to do with the “document-overrides” collection attributes when supporting the Document object, which is no longer needed as either a Job Template, Document Template, or operation attribute in Document Creation requests?  Not completely true, since the “document-copies” member attribute of the “document-overrides” attribute allows the client to supply different attributes to different copies of a document.  Has anyone implemented Document Overrides?

Similarly, according to [override], the “page-overrides” (collection) attribute MAY be supplied by the client in a Send-Document or Send-URI request as an Operation attribute to apply page overrides to this and/or subsequent documents in the job.  See the “page-overrides” Job Template attribute in Table 5 for the listing of the member attributes.  However, with the introduction of the Document object, the “page-overrides” (collection) attribute SHOULD be more simply supplied as one of the Document Template attributes for the document being created only.

ISSUE 03:  What do we want to do with the “page-overrides” collection attributes when supporting the Document object, which is still needed as a Job Template and Document Template attribute for overriding attributes in specified page ranges, but is not needed as an operation attribute on Document Creation requests?

The “Defined for use” column indicates whether the client MAY supply and the Printer support the attribute at the Job, Document, or Page Level with the following semantics:

| Supplied | Description |
| --- | --- |
| J=y: | Job Level: The attributes MAY be supplied as Job Template attributes at the Job Level in a Job Creation request (Print-Job, Print-URI, Create-Job) in which case each Document inherits the Job Template attribute as a default which is used unless an attribute at the document level is also supplied. Note: The “multiple-document-handling” attribute controls whether extensive attributes, such as “finishing”, “copies”, affect the entire job as a whole (‘single-xxx’ values) or affect each document separately (‘separate-xxx’ values). |
| D=y: | Document Level: The attribute MAY be supplied at the Document Level as Document Template attributes in a Document Creation operation (Send-Document, Send-URI), in which case the supplied Document Templates override any Job Template attributes supplied at the Job Level and any “document-overrides” supplied at the Job or Document level. |
| P=y: | Page Level: The attribute MAY be supplied as a member of the (1) “page-ranges” Job Template attribute at the Job Level in a Job Creation operation or (2) “page-ranges” Document Template attribute at the Document Level to affect the specified range of pages. |

Job Template and Document Template attributes are OPTIONAL for a Printer to support and for a client to supply in a Job Creation or Document Creation request or a Document Overrides or Page Overrides.  If a Printer supports a Job Template or Document Template attribute, then it MUST copy the supplied attribute to the Job or Document object, respectively so that a client MAY query the attributes in subsequent Get-Job-Attributes/Get-Jobs and Get-Document-Attributes/Get-Documents operations, respectively.  The effect of Job Template Attributes supplied in Job Creation requests are inherited by the Document objects, unless the Document Creation operation supplies the attribute with a different value.  However, the Printer MUST NOT propagate Job Template attributes supplied in Job Creation operations to the Document object. Similarly, the Printer MUST NOT promote Document Template attributes up to the Job object when no corresponding Job Template attribute was supplied in the Job Creation operation.  Thus the Printer returns in queries only Job Template and Document Template attributes that were actually supplied by a client.

ISSUE 04:  Is this query behavior of not appearing to copying down supplied Job attributes to the Document object and not bubbling up supplied Document attributes to the Job object OK?

However, if a Job Template collection attribute is supported (see [coll]), then there are conformance requirements for which member attributes MUST be supported and which member attributes a client MUST supply as indicated in the Conformance column for each collection member attribute in Table 5.

Table 5 - Job Template and Document Template attributes

| Job Template / Document Template attribute name | Attribute Syntax | Defined for use |  |  | Conformance |  | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | J | D | P | Client supply | Printer support |  |
| copies | integer(1:MAX) | y | y | n | MAY | MAY | [RFC2911] §4.2.5 |
| cover-back | collection | y | y | n | MAY | MAY | [prod-print] §3.1 |
| media | type3 keyword ¦ name(MAX) |  |  |  | MAY be neither or one of, but NOT both | MUST | [prod-print] §3.1.1 |
| media-col | collection |  |  |  |  | MAY | [prod-print] §3.1.1 |
| cover-type | type2 keyword |  |  |  | MUST | MUST | [prod-print] §3.1.2 |
| cover-front | collection | y | y | n | MAY | MAY | [prod-print] §3.1 |
| media | type3 keyword ¦ name(MAX) |  |  |  | MAY be neither or one of, but NOT both | MUST | [prod-print] §3.1.1 |
| media-col | collection |  |  |  |  | MAY | [prod-print] §3.1.1 |
| cover-type | type2 keyword |  |  |  | MUST | MUST | [prod-print] §3.1.2 |
| document-overrides | collection | y | n | n | MAY | MAY | [override] §5.1 |
| input-documents | 1setOf rangeOfInteger(MAX) |  |  |  | MUST be one or the other, but NOT both | MUST | [override] §5.1.1 |
| output-documents | 1setOf rangeOfInteger(MAX) |  |  |  |  | MUST | [override] §5.1.2 |
| document-copies | 1setOf rangeOfInteger(MAX) |  |  |  | MAY | MAY | [override] §5.1.3 |
| document-format | mimeMediaType |  |  |  | MAY | MUST | [override] §5.1.4 |
| document-name | name (MAX) |  |  |  | MAY | MUST | [override] §5.1.5 |
| compression | type3 keyword |  |  |  | MAY | MUST | [override] §5.1.6 |
| document-natural-language | naturalLanguage |  |  |  | MAY | MAY | [override] §5.1.7 |
| page-ranges | 1setOf rangeOfInteger (1:MAX) |  |  |  | MAY | MAY | [override] §5.1.8 |
| any other Job Template attributes that affect documents, such as “media” or “media-col” |  |  |  |  | MAY | MAY |  |
| finishings | 1setOf type2 enum | y | y | y | MAY be neither or one of, but NOT both | MAY, MUST if “finishing-col” is supported | [RFC2911] §4.2.6 [finish] §2 |
| finishings-col | collection | y9 | y | y |  | MAY | [prod-print] §3.2 |
| finishing-template | name(MAX) |  |  |  | MAY | MAY | [prod-print] §3.2.1 |
| stitching | collection |  |  |  | MAY | MAY | [prod-print] §3.2.2 |
| stitching-reference-edge | type2 keyword |  |  |  | MUST | MUST | [prod-print] §3.2.2.1 |
| stitching-offset | integer (0:MAX) |  |  |  | MUST | MUST | [prod-print] §3.2.2.2 |
| stitching-locations | 1setOf integer(0:MAX) |  |  |  | MUST | MUST | [prod-print] §3.2.2.3 |
| force-front-side | 1setOf integer(1:MAX) | y | y | y | MAY | MAY | [prod-print] §3.3 |
| imposition-template | type2 keyword ¦ name(MAX) | y | y | y | MAY | MAY | [prod-print] §3.4 |
| insert-sheet | 1setOf collection | y10 | y | y | MAY | MAY | [prod-print] §3.5 |
| insert-after-page-number | integer(0:MAX) |  |  |  | MUST | MUST | [prod-print] §3.5.1 |
| insert-count | integer(0:MAX) |  |  |  | MAY | MUST | [prod-print] §3.5.2 |
| media | type3 keyword ¦ name(MAX) |  |  |  | MUST be one or the other, but NOT both | MUST | [prod-print] §3.5.3 |
| media-col | collection |  |  |  |  | MAY | [prod-print] §3.5.3 |
| job-account-id | name(MAX) | y | n | n | MAY | MAY | [prod-print] §3.6 |
| job-accounting-sheets | collection | y | n | n | MAY | MAY | [prod-print] §3.8 |
| job-accounting-sheets-type | type3 keyword ¦ name(MAX) |  |  |  | MUST | MUST | [prod-print] §3.8.1 |
| media | type3 keyword ¦ name(MAX) |  |  |  | MAY be neither or one of, but NOT both | MUST | [prod-print] §3.8.2 |
| media-col | collection |  |  |  |  | MAY | [prod-print] §3.8.2 |
| job-accounting-output-bin | type3 keyword ¦ name(MAX) |  |  |  | MAY | MAY | [prod-print] §3.8.3 |
| job-accounting-user-id | name(MAX) | y | n | n | MAY | MAY | [prod-print] §3.7 |
| job-error-sheet | collection | y | n | n | MAY | MAY | [prod-print] §3.9 |
| job-error-sheet-type | type3 keyword ¦ name(MAX) |  |  |  | MUST | MUST | [prod-print] §3.9.1 |
| job-error-sheet-when | type2 keyword |  |  |  | MAY | MAY | [prod-print] §3.9.2 |
| media | type3 keyword ¦ name(MAX) |  |  |  | MAY be neither or one of, but NOT both | MUST | [prod-print] §3.9.3 |
| media-col | collection |  |  |  |  | MAY | [prod-print] §3.9.3 |
| job-hold-until | type3 keyword ¦ name(MAX) | y | n | n | MAY | MAY | [RFC2911] §4.2.2 |
| job-message-to-operator | text(MAX) | y | n | n | MAY | MAY | [prod-print] §3.10 |
| job-priority | integer(1:100 | y | n | n | MAY | MAY | [RFC2911] §4.2.1 |
| job-sheet-message | text(MAX) | y | n | n | MAY | MAY | [prod-print] §3.12 |
| job-sheets | type3 keyword ¦ name(MAX) | y | n | n | MAY be neither or one of, but NOT both | MAY, MUST if “job-sheets-col” supported | [RFC2911] §4.2.3 |
| job-sheets-col | collection | y | n | n |  | MAY | [prod-print] §3.11 |
| job-sheets | type3 keyword ¦ name(MAX) |  |  |  | MUST | MUST | [prod-print] §3.11.1 |
| media | type3 keyword ¦ name(MAX) |  |  |  | MUST be one or the other, but NOT both | MUST | [prod-print] §3.11.2 |
| media-col | collection |  |  |  |  | MAY | [prod-print] §3.11.2 |
| media | type3 keyword ¦ name(MAX) | y | y | y | MAY be neither or one of, but NOT both | MAY, MUST if “media-col” supported | [RFC2911] §4.2.11 |
| media-col | collection | y | y | y |  | MAY | [prod-print] §3.13 |
| media-key | type3 keyword ¦ name(MAX) |  |  |  | MAY | MAY | [prod-print] §3.13.1 |
| media-type | type3 keyword ¦ name(MAX |  |  |  | MAY | MAY | [prod-print] §3.13.2 |
| media-info | text(255) |  |  |  | MAY | MAY | [prod-print] §3.13.3 |
| media-color | type3 keyword ¦ name(MAX) |  |  |  | MAY | MAY | [prod-print] §3.13.4 |
| media-preprinted | type3 keyword ¦ name(MAX) |  |  |  | MAY | MAY | [prod-print] §3.13.5 |
| media-hole-count | integer(0:MAX) |  |  |  | MAY | MAY | [prod-print] §3.13.6 |
| media-order-count | integer(1:MAX) |  |  |  | MAY | MAY | [prod-print] §3.13.7 |
| media-size | collection |  |  |  | MAY | MUST | [prod-print] §3.13.8 |
| x-dimension | integer(0:MAX) |  |  |  | MUST | MUST | [prod-print] §3.13.8.1 |
| y-dimension | integer(0:MAX) |  |  |  | MUST | MUST | [prod-print] §3.13.8.2 |
| media-weight-metric | integer(0:MAX) |  |  |  | MAY | MAY | [prod-print] §3.13.9 |
| media-front-coating | type3 keyword ¦ name(MAX) |  |  |  | MAY | MAY | [prod-print] §3.13.10 |
| media-back-coating | type3 keyword ¦ name(MAX) |  |  |  | MAY | MAY | [prod-print] §3.13.10 |
| media-recycled | type3 keyword ¦ name(MAX) |  |  |  | MAY | MAY | [prod-print] §3.13.11 |
| media-input-tray-check | type3 keyword ¦ name(MAX) | y | y | y | MAY | MAY | [prod-print] §3.14 |
| multiple-document-handling | type2 keyword | y | n | n | MAY | MUST if multiple document jobs supported | [RFC2911] §4.2.4 |
| number-up | integer(1:MAX) | y | y | y | MAY | MAY | [RFC2911] §4.2.9 |
| orientation-requested | type2 enum | y | y | y | MAY | MAY | [RFC2911] §4.2.10 |
| output-bin | type2 keyword ¦ name(MAX) | y | y | n | MAY | MAY | [output-bin] §2.1 |
| page-delivery | type2 keyword | y | y | n | MAY | MAY | [prod-print] §3.15 |
| page-order-received | type2 keyword | y | y | n | MAY | MAY | [prod-print] §3.16 |
| page-overrides | 1setOf collection | y | y | n | MAY | MAY | [override] §5.2 |
| input-documents | 1setOf rangeOfInteger(MAX) |  |  |  | MUST be one or the other, but NOT both | MUST | [override] §5.2.1 |
| output-documents | 1setOf rangeOfInteger(MAX) |  |  |  |  | MUST | [override] §5.2.2 |
| document-copies | 1setOf rangeOfInteger(MAX) |  |  |  | MAY | MAY | [override] §5.2.3 |
| pages | 1setOf rangeOfInteger(MAX) |  |  |  | MUST | MUST | [override] §5.2.4 |
| any other Job Template attributes that affect pages, such as “media” or “media-col” |  |  |  |  | MAY | MAY |  |
| page-ranges | 1setOf rangeOfInteger(1:MAX) | y11 | y | n | MAY | MAY | [RFC2911] §4.2.7 |
| pages-per-subset | 1setOf integer(1:MAX) | y | y | n | MAY | MAY | [prod-print] §5.3 |
| presentation-direction-number-up | type2 keyword | y | y | y | MAY | MAY | [prod-print] §3.17 |
| print-quality | type2 enum | y | y | y | MAY | MAY | [RFC2911] §4.2.13 |
| printer-resolution | resolution | y | y | y | MAY | MAY | [RFC2911] §4.2.12 |
| separator-sheets | collection | y | y | y | MAY | MAY | [prod-print] §3.18 |
| separator-sheets-type | type3 keyword ¦ name(MAX) |  |  |  | MUST | MUST | [prod-print] §3.18.1 |
| media | type3 keyword ¦ name(MAX) |  |  |  | MAY be neither or one of, but NOT both | MUST | [prod-print] §3.18.2 |
| media-col | collection |  |  |  |  | MAY | [prod-print] §3.18.2 |
| sheet-collate | type2 keyword | y | y | n | MAY | MAY | [job-prog] §3.1 |
| sides | type2 keyword | y | y | y | MAY | MAY | [RFC2911] §4.2.8 |
| x-image-position | type2 keyword | y | y | y | MAY | MAY | [prod-print] §3.19.2 |
| x-image-shift | integer(MIN:MAX) | y | y | y | MAY | MAY | [prod-print] §3.19.3 |
| x-side1-image-shift | integer(MIN:MAX) | y | y | y | MAY | MAY | [prod-print] §3.19.4 |
| x-side2-image-shift | integer(MIN:MAX) | y | y | y | MAY | MAY | [prod-print] §3.19.5 |
| y-image-position | type2 keyword | y | y | y | MAY | MAY | [prod-print] §3.19.6 |
| y-image-shift | integer(MIN:MAX) | y | y | y | MAY | MAY | [prod-print] §3.19.7 |
| y-side1-image-shift | integer(MIN:MAX) | y | y | y | MAY | MAY | [prod-print] §3.19.8 |
| y-side2-image-shift | integer(MIN:MAX) | y | y | y | MAY | MAY | [prod-print] §3.19.9 |

# Job Description and Document Description attributes

Table 6 lists all the Job Description defined in other IPP specifications and the corresponding Document Description attributes defined in this IPP specification.  Job Description and Document Description attributes are Job object and Document object attributes, respectively, that are filled in by the Printer.  Some Job/Document Description attribute values are supplied by the client in the Job/Document Creation operation using corresponding Operation attributes as flagged with “**”.

The column labeled “attribute syntax” indicates the attribute syntax (see [RFC2911] section 4.1).  The Printer conformance is indicated along with the reference to the specification in which the attribute is defined.  The conformance for Document Description attributes (if the Printer supports the Document object) is the same as the corresponding Job Description attributes.

Note:  There are no corresponding Page Range (or Page Override) Description attributes.  The Page Range is not an object with a state and it cannot be queried.

Legend:

[job-]	indicates an attribute that shouldn’t have had a “job-” prefix in its name in [RFC2911], so that the same attribute could also apply to the Document object as a Document Description attribute.  Note: For the PWG Semantic Model, the “job-” prefix is proposed to be dropped.

ISSUE 05:  Should we sort the attributes ignoring the [job-]?  Currently the “job-” is used in the sort for attributes.

ISSUE 06:  How will we register these attributes with IANA, since they will be registered for IPP use, with the “job-” prefix.  Do we add the ones without “job-” as aliases to the IANA registry as is done for charset registrations?

“-”	indicates that the Job or Document attribute is not defined.

“*”	indicates a fundamental Document Description that when the client supplies the value at the Job level, but not at the Document level, the Printer MUST appear to copy down the value to each Document object as returned in Get-Document-Attributes and Get-Documents operation responses.

**	indicates a Job and/or Document Description attribute that the client MAY supply the values for in a corresponding Operation attribute.

Table 6 - Job and Document Description attributes

| IPP Job Description / Document Description attribute name | IPP Attribute Syntax | Job | Doc | Printer conformance | Reference |
| --- | --- | --- | --- | --- | --- |
| attributes-charset | charset | y | y | MUST | [RFC2911] §4.3.19 |
| attributes-natural-language | naturalLanguage | y | y | MUST | [RFC2911] §4.3.20 |
| compression** | type3 keyword | y* | y* | MUST | [RFC2911] §3.2.1.1 |
| current-page-order | type2 keyword | y | y | MAY | [prod-print] §4.1 |
| date-time-at-completed | dateTime | y | y | MAY | [RFC2911] §4.3.14.7 |
| date-time-at-creation | dateTime | y | y | MAY | [RFC2911] §4.3.14.5 |
| date-time-at-processing | dateTime | y | y | MAY | [RFC2911] §4.3.14.6 |
| document-format** | documentFormat | y* | y* | MUST | [RFC2911] §3.2.1.1 |
| document-name** | name(MAX) | y* | y* | MUST | [RFC2911] §3.2.1.1 |
| document-natural-language** | naturalLanguage | y* | y* | MAY | [RFC2911] §3.2.1.1 |
| document-number | integer(0:MAX) | - | y | MUST | [this spec] §6.1 |
| document-state | type1 enum | - | y | MUST | [this spec] §6.3.2 |
| document-state-reasons | 1setOf type2 keyword | - | y | MUST | [this spec] §6.5 |
| impressions-completed-current-copy | integer(0:MAX) | y | y | MAY | [job-prog] §4.4 |
| job-collation-type | type2 enum | y | - | MAY | [job-prog] §4.1 |
| [job-]detailed-status-message | 1setOf text (MAX) | y | y | MAY | [RFC2911] §4.3.10 |
| [job-]document-access-errors | 1setOf text(MAX) | y | y | MAY | [RFC2911] §4.3.11 |
| job-id | integer(1:MAX) | y | - | MUST | [RFC2911] §4.3.2 |
| [job-]impressions ** | integer(0:MAX) | y | y | MAY | [RFC2911] §4.3.17.2 |
| [job-]impressions-completed | integer(0:MAX) | y | y | MAY | [RFC2911] §4.3.18.2 |
| [job-]k-octets ** | integer(0:MAX) | y | y | MAY | [RFC2911] §4.3.17.1 |
| [job-]k-octets-processed | integer(0:MAX) | y | y | MAY | [RFC2911] §4.3.18.1 |
| [job-]media-sheets ** | integer(0:MAX) | y | y | MAY | [RFC2911] §4.3.17.3 |
| [job-]media-sheets-completed | integer(0:MAX) | y | y | MAY | [RFC2911] §4.3.18.3 |
| job-message-from-operator | text(127) | y | - | MAY | [RFC2911] §4.3.16 |
| [job-]more-info | uri | y | y | MAY | [RFC2911] §4.3.4 |
| job-name | name(MAX) | y | - | MUST | [RFC2911] §4.3.5 |
| job-originating-user-name | name (MAX) | y | - | MUST | [RFC2911] §4.3.6 |
| [job-]printer-up-time | integer (1:MAX) | y | y | MUST | [RFC2911] §4.3.14.4 |
| job-printer-uri | uri | y | - | MUST | [RFC2911] §4.3.3 |
| job-state | type1 enum | y | - | MUST | [RFC2911] §4.3.7 |
| job-state-message | text(MAX) | y | - | MAY | [RFC2911] §4.3.9 |
| job-state-reasons | 1setOf type2 keyword | y | - | MUST | [RFC2911] §4.3.8 |
| job-uri | uri | y | - | MUST | [RFC2911] §4.3.1 |
| [job-]warnings-count | integer(0:MAX) | y | y | MAY | [override] §6.1 |
| last-document ** | boolean | - | y | MAY | [RFC2911] §4.3.12 |
| number-of-documents | integer(0:MAX) | y | - | MAY | [RFC2911] §4.3.15 |
| number-of-intervening-jobs | integer(0:MAX) | y | - | MAY | [RFC2911] §4.3.13 |
| output-device-assigned | name(127) | y | - | MAY | [RFC2911] §4.3.7.2 |
| sheet-completed-copy-number | integer(0:MAX) | y | y | MAY | [job-prog] §4.2 |
| sheet-completed-document-number | integer(0:MAX) | y | - | MAY | [job-prog] §4.3 |
| time-at-completed | integer (MIN:MAX) | y | y | MUST | [RFC2911] §4.3.14.3 |
| time-at-creation | integer (MIN:MAX) | y | y | MUST | [RFC2911] §4.3.14.1 |
| time-at-processing | integer (MIN:MAX) | y | y | MUST | [RFC2911] §4.3.14.2 |

“*” and “**”	See the description in the Legend before the table.

Most of the existing “job-state” and “job-state-reasons” Job Description attribute values apply equally well to individual documents in a multi-document job (with removal of the ‘job-’ prefix for those that apply to both Job and Document).  Furthermore, the job states are completely service-type neutral, so they can apply to scanning. faxing, formatting processes as well.  The job state reasons have a number of reasons that are print service specific, but most can apply to other types of service as well.  The reason for separating state from reasons, is so that the states can be the fundamental states that most implementations have, while the state reasons can vary depending on the sophistication of the system.  The states are the ones that clients are most likely to base actions upon, and the state reasons are more information, both for programs and people.

The descriptions of the new “document-state” and “document-state-reasons” attributes are merged in with the existing “job-state” and “job-state-reasons”.  Such merging shows the parallelism in the description to aid understanding and implementation and so that future maintenance will continue this strong parallelism.  The “/” character separates the words: Job from Document so that the same description can serve for both Job and Document objects.

## document-number (integer(1:MAX))	<Document>

This Document Description attribute is the ordinal number of the document within the job which determines its order of being printed in a multi-document job.  The Printer generates the values monotonically increasing from 1 for a job.  However, the client MAY supply the value in the “input-document-number” operation attribute.

## job-state (type1 enum)	<Job>

## document-state (type1 enum)	<Document>

The REQUIRED “job-state” and the REQUIRED “document-state” attribute identify the current state of the job and document, respectively.  Even though IPP defines seven values for job/document states (plus the out-of-band 'unknown' value - see [RFC2911] section 4.1), implementations only need to support those states which are appropriate for the particular implementation.  In other words, an IPP Printer object implementation supports only those job/document states implemented by the output device and available to the IPP Printer object implementation.

Table 7 lists the “job-state” and “document-state” attribute values.

Table 7 - "job-state" and "document-state" attribute values

| Values | “job-state” | “document-state” |
| --- | --- | --- |
| 3 pending | y | y |
| 4 pending-held | y | n/a |
| 5 processing | y | y |
| 6 processing-stopped | y | n/a |
| 7 canceled | y | y |
| 8 aborted | y | y |
| 9 completed | y | y |

The (JD) and (J) notation below indicates whether the keyword is defined for the “job-state” and “document-state” attributes or just the “job-state” attribute as indicated in Table 7.  Standard enum values are:

| Value |  | Symbolic Name and Description |
| --- | --- | --- |
| '3' |  | 'pending' (JD): The job is a candidate to start processing, but is not yet processing. The document has not started to be processed at all. |
| '4' |  | 'pending-held'' (J): The job is not a candidate for processing for any number of reasons but will return to the 'pending' state as soon as the reasons are no longer present. The job's "job-state-reason" attribute MUST indicate why the job is no longer a candidate for processing. The ‘pending-held’ state does not apply to the Document object’s “document-state” attribute. |
| '5' |  | 'processing' (JD): One or more of: - The job is using, or is attempting to use, one or more purely software processes that are analyzing, creating, or interpreting a PDL, etc., - The job is using, or is attempting to use, one or more hardware devices that are interpreting a PDL, making marks on a medium, and/or performing finishing, such as stapling, etc., - The Printer object has made the job ready for printing, but the output device is not yet printing it, either because the job hasn't reached the output device or because the job is queued in the output device or some other spooler, awaiting the output device to print it. When the job is in the 'processing' state, the entire job/document state includes the detailed status represented in the Printer object's "printer-state", "printer-state-reasons", and "printer-state-message" attributes. On the other hand, the ‘processing’ state for the Document indicates that the Document has begun to be processed. Even if the job stops being processed, the Document remains in the ‘processing’ state until it moves to one of the three terminal states. Implementations MAY, though they NEED NOT, include additional values in the job's/document’s "job-state-reasons"/“document-state-reasons” attribute to indicate the progress of the job/document, such as adding the 'job-printing'/‘printing’ value to indicate when the output device is actually making marks on paper and/or the 'processing-to-stop-point' value to indicate that the IPP object is in the process of canceling or aborting the job/document. Most implementations won't bother with this nuance. |
| '6' |  | 'processing-stopped' (J): The job has stopped while processing for any number of reasons and will return to the 'processing' state as soon as the reasons are no longer present. The job's "job-state-reason" attribute MAY indicate why the job has stopped processing. For example, if the output device is stopped, the 'printer-stopped' value MAY be included in the job's "job-state-reasons" attribute. Note: When an output device is stopped, the device usually indicates its condition in human readable form locally at the device. A client can obtain more complete device status remotely by querying the Printer object's "printer-state", "printer-state-reasons" and "printer-state-message" attributes. The ‘processing-stopped’ state does not apply to the Document object’s “document-state” attribute. |
| '7' |  | 'canceled' (JD): The job/document has been canceled by a Cancel-Job/Cancel-Document operation and the Printer object has completed canceling the job/document and all job/document status attributes have reached their final values for the job/document. While the Printer object is canceling the job/document, the job/document remains in its current state, but the job's/document’s "job-state-reasons"/“document-state-reasons” attribute SHOULD contain the 'processing-to-stop-point' value and one of the 'canceled-by-user', 'canceled-by-operator', or 'canceled-at-device' value. When the job/document moves to the 'canceled' state, the 'processing-to-stop-point' value, if present, MUST be removed, but the 'canceled-by-xxx', if present, MUST remain. |
| '8' |  | 'aborted' (JD): The job/document has been aborted by the system, usually while the job/document was in the 'processing' or 'processing-stopped' state and the Printer has completed aborting the job/document and all job/document status attributes have reached their final values for the job/document. While the Printer object is aborting the job/document, the job/document remains in its current state, but the job's/document’s "job-state-reasons"/“document-state-reasons” attribute SHOULD contain the 'processing-to-stop-point' and 'aborted-by-system' values. When the job/document moves to the 'aborted' state, the 'processing-to-stop-point' value, if present, MUST be removed, but the 'aborted-by-system' value, if present, MUST remain. |
| '9' |  | 'completed' (JD): The job/document has completed successfully or with warnings or errors after processing and all of the job/document media sheets have been successfully stacked in the appropriate output bin(s) and all job/document status attributes have reached their final values for the job/document. The job's/document’s "job-state-reasons"/“document-state-reasons” attribute SHOULD contain one of: 'completed-successfully', 'completed-with-warnings', or 'completed-with-errors' values. |

The final value for this attribute MUST be one of: 'completed', 'canceled', or 'aborted' before the Printer removes the job/document altogether.  The length of time that jobs/document remain in the 'canceled', 'aborted', and 'completed' states depends on implementation.

Jobs/documents reach one of the three terminal states: 'completed', 'canceled', or 'aborted', after the jobs/documents have completed all activity, including stacking output media, after the jobs/documents have completed all activity, and all job/document status attributes have reached their final values for the job/document.

### The “job-state” attribute and the Job Life Cycle

The “job-state” attribute is one of the most important Job Description attributes.  Figure 1 shows the values of the “job-state” attribute and the Job object life cycle as affected by actions on the Job, Printer, and job processing.

Figure 1 - The “job-state” Job Description attribute and the Job object life cycle

Normally a job progresses from left to right.  Other state transitions are unlikely, but are not forbidden.  Not shown are the transitions to the 'canceled' state from the 'pending', 'pending-held', and 'processing-stopped' states.

### The “document-state” Document Description attribute and the Document object life Cycle

The “document-state” attribute is one of the most important Document Description attributes.  Figure 2 shows the values of the “document-state” attribute and the Document object life cycle as affected by actions on the Job, Document, Printer, and job processing.  Documents are not active objects and their life cycle is closely tied to the lifecycle of a Job.  Documents basically have three states.   The first is waiting to be processed by a Job (i.e. pending).  The second state is from the time the Job first starts processing the Document (i.e., processing) and until it reaches one of its terminal states.  The third state for a Document is one of its terminal states (i.e., completed, canceled, or aborted).

Figure 2  The “document-state” Document Description attribute and the Document object life Cycle

A Document object has a much simpler life cycle than a Job object.  A Document always progresses from left to right in the state diagram.  No other state transitions are defined.

### Forwarding Servers

As with all other IPP attributes, if the implementation can not determine the correct value for this attribute, it SHOULD respond with the out-of-band value 'unknown' (see [RFC2911] Section 4.1) rather than try to guess at some possibly incorrect value and give the end user the wrong impression about the state of the Job/Document object.  For example, if the implementation is just a gateway into some printing system from which it can normally get status, but temporarily is unable, then the implementation should return the 'unknown' value.  However, if the implementation is a gateway to a printing system that never provides detailed status about the print job/document, the implementation MAY set the IPP Job/Document object's state to 'completed', provided that it also sets the 'queued-in-device' value in the job's/document’s "job-state-reasons"/“document-state-reasons” attribute (see “job-state-reasons”/“document-state-reasons” attribute below).

### Partitioning of Job/Document States

This section partitions the job/document states into phases:  Job/Document Not Completed, Job/Document Retention, Job/Document History, and Job/Document Removal.  This section also explains the 'job-restartable' value of the "job-state-reasons" Job Description attribute for use with the Restart-Job operation.

Job/Document Not Completed:  When a job is in the 'pending', 'pending-held', 'processing', or 'processing-stopped' states, the job is not completed.  When a Document is in the ‘pending’ or ‘processing’ state, it is not completed.

Job Retention:  When a job enters one of the three terminal job states:  'completed', 'canceled', or 'aborted', the IPP Printer object MAY "retain" the job in a restartable condition for an implementation-defined time period.  This time period MAY be zero seconds and MAY depend on the terminal job state.  This phase is called Job Retention.  While in the Job Retention phase, the job's document data is retained and a client may restart the job using the Restart-Job operation.  If the IPP object supports the Restart-Job operation, then it SHOULD indicate that the job is restartable by adding the 'job-restartable' value to the job's "job-state-reasons" attribute (see Section 6.4) during the Job Retention phase.  If the Printer supports the Document object, then the Get-Document-Attributes and Get-Documents operations MUST be supported while the job is in the Job Retention phase.

Job History:  After the Job Retention phase expires for a job, the Printer object deletes the document data for the job and the job becomes part of the Job History.  The Printer object MAY also delete any number of the job and document attributes.  Since the job is no longer restartable, the Printer object MUST remove the 'job-restartable' value from the job's "job-state-reasons" attribute, if present.  If the implementation supports the Document object, it SHOULD retain at least the “document-state” and “document-state-reasons” attributes on the Document object during the Job History phase which can be queried using the Get-Document-Attributes and Get-Documents operations.

Job Removal:  After the job has remained in the Job History for an implementation-defined time, such as when the number of jobs exceeds a fixed number or after a fixed time period (which MAY be zero seconds), the IPP Printer removes the Job and Document objects from the system.

Using the Get-Jobs/Get-Documents operation and supplying the 'not-completed' value for the "which-jobs" operation attribute, a client is requesting jobs/documents in the Job/Document Not Completed phase.  Using the Get-Jobs/Get-Documents operation and supplying the 'completed' value for the "which-jobs" operation attribute, a client is requesting jobs/documents in the Job Retention and Job History phases.  Using the Get-Job-Attributes/Get-Document-Attributes operation, a client is requesting a job/documents in any phase except Job Removal.  After Job Removal, the Get-Job-Attributes/Get-Document-Attributes and Get-Jobs/Get-Documents operations no longer are capable of returning any information about a job/document.

NOTE:  The semantics of this attribute or the set of valid values may be different for different document formats.

## job-state-reasons  (1setOf type2 keyword) 	<Job>

## document-state-reasons  (1setOf type2 keyword) 	<Document>

The REQUIRED “job-state-reasons” and the REQUIRED “document-state-reasons” attributes provide additional information about the job's and each document’s current state, i.e., information that augments the value of the job's "job-state" attribute and each Document’s “document-state” attribute, respectively.  The Printer SHOULD support the same values for both the “job-state-reasons” and the “document-state-reasons” attribute for those attributes that are defined for both as indicated in the “Job” and “Doc” columns in Table 8.

These values MAY be used with any job or document state or states for which the reason makes sense. Some of these value definitions indicate conformance requirements; the rest are OPTIONAL.  Furthermore, when implemented, the Printer MUST return these values when the reason applies and MUST NOT return them when the reason no longer applies whether the value of the Job's "job-state" attribute or the Document’s “document-state” attribute changed or not.  When the Job or Document does not have any reasons for being in its current state, the value of the Job's "job-state-reasons" attribute or Document’s “document-state-reasons” attribute MUST be 'none'.

Note: While values cannot be added to the “job-state” or “document-state” attributes without impacting deployed clients that take actions upon receiving "job-state" or “document-state” values, it is the intent that additional "job-state-reasons" and “document-state-reasons’ values can be defined and registered without impacting such deployed clients.  In other words, the "job-state-reasons" and the “document-state-reasons” attributes are intended to be extensible.

The following standard keyword values are defined for the “job-state-reasons” and/or “document-state-reasons” attributes as shown in Table 8.  In Table 8 the keywords are listed in alphabetic order (ignoring the “[job-]” prefix), while in their descriptions following, they are listed in the most likely order of occurrence.

When the Document object is supported, the Printer implementation SHOULD still support the corresponding Job level “job-state-reasons”, so that clients that do not support the Document object will receive the same information.  Also it is usually more convenient for the client to obtain just the Job level status.

Notation:

[job-]	indicates a state reason keyword value that shouldn’t have had a “job-” prefix in its name in [RFC2911], so that the same value could also apply at the Document level as a “document-state-reasons” attribute value.  Note: For the PWG Semantic Model, the “job-” prefix will be dropped.  For purposes of sorting in Table 8, the “[job-]” prefix is ignored, since the PWG Semantic Model name is proposed without the prefix.

ISSUE 07:  Should we sort the attributes values including the [job-]?  Currently the “job-” is ignored in the sort for attribute values.

ISSUE 08:  How will we register these attribute values with IANA, since they will be registered for IPP use, with the “job-” prefix.  Do we add the ones without “job-” as aliases to the IANA registry for as is done for charset registrations?

“-”	indicates that the Job or Document value is not defined.

Table 8 - "job-state-reasons" and "document-state-reasons" attribute values

| IPP Job Description / Document Description attribute name | IPP Attribute Syntax plus Values | Job | Doc | Printer conformance | Reference |
| --- | --- | --- | --- | --- | --- |
| job-state-reasons | 1setOf type2 keyword | y | - | MUST | [RFC2911] §4.3.8 |
| document-state-reasons | 1setOf type2 keyword | - | y | MUST | [this spec] §6.5 |
|  | ‘aborted-by-system’ | y | y | MAY | [RFC2911] §4.3.8 |
|  | ‘[job-]canceled-at-device’ | y | y | MAY | [RFC2911] §4.3.8 |
|  | ‘[job-]canceled-by-operator’ | y | y | MAY | [RFC2911] §4.3.8 |
|  | ‘[job-]canceled-by-user’ | y | y | MAY | [RFC2911] §4.3.8 |
|  | ‘[job-]completed-successfully’ | y | y | MAY | [RFC2911] §4.3.8 |
|  | ‘[job-]completed-with-errors’ | y | y | MAY | [RFC2911] §4.3.8 |
|  | ‘[job-]completed-with-warnings’ | y | y | MAY | [RFC2911] §4.3.8 |
|  | ‘compression-error | y | y | MAY | [RFC2911] §4.3.8 |
|  | ‘document-access-error’ | y | y | MAY | [RFC2911] §4.3.8 |
|  | ‘document-format-error’ | y | y | MAY | [RFC2911] §4.3.8 |
|  | ‘[job-]incoming’ | y | y | MAY | [RFC2911] §4.3.8 |
|  | ‘[job-]interpreting’ | y | y | MAY | [RFC2911] §4.3.8 |
|  | ‘job-data-insufficient’ | y | - | MAY | [RFC2911] §4.3.8 |
|  | ‘job-hold-until-specified’ | y | - | MAY | [RFC2911] §4.3.8 |
|  | ‘job-queued’ | y | - | MAY | [RFC2911] §4.3.8 |
|  | ‘job-restartable’ | y | - | MAY | [RFC2911] §4.3.8 |
|  | ‘job-resuming’ | y | - | MAY | [RFC2911] §4.3.8 |
|  | ‘job-suspended’ | y | - | MAY | [adm-ops] §9.1 |
|  | ‘none’ | y | y | MUST | [RFC2911] §4.3.8 |
|  | ‘[job-]outgoing’ | y | y | MAY | [RFC2911] §4.3.8 |
|  | ‘printer-stopped’ | y | - | MAY | [RFC2911] §4.3.8 |
|  | ‘printer-stopped-partly’ | y | - | MAY | [RFC2911] §4.3.8 |
|  | ‘[job-]printing’ | y | y | MAY | [RFC2911] §4.3.8 |
|  | ‘processing-to-stop-point’ | y | - | MAY | [RFC2911] §4.3.8 |
|  | ‘queued-in-device’ | y | y | MAY | [RFC2911] §4.3.8 |
|  | ‘[job-]queued-for-marker’ | y | y | MAY | [RFC2911] §4.3.8 |
|  | ‘resources-are-not-ready’ | y | y | MAY | [RFC2911] §4.3.8 |
|  | ‘resources-are-not-supported’ | y | y | MAY | [prod-print] §6.1 |
|  | ‘service-off-line’ | y | - | MAY | [RFC2911] §4.3.8 |
|  | ‘submission-interrupted’ | y | y | MAY | [RFC2911] §4.3.8 |
|  | ‘[job-]transforming’ | y | y | MAY | [RFC2911] §4.3.8 |
|  | ‘unsupported-compression’ | y | y | MUST | [RFC2911] §4.3.8 |
|  | ‘unsupported-document-format’ | y | y | MUST | [RFC2911] §4.3.8 |
|  | ‘[job-]warnings-detected’ | y | y | MAY | [override] §7.1 |

For ease of understanding, the following value descriptions are presented in the order in which the reasons are likely to occur (if implemented), starting with the ‘none’ value.  Note: The order is improved over that in [RFC2911].  The (JD), (J), and (D) notation is identical to the “Job” and “Doc” columns in Table 8 indicating whether the keyword is defined for the “job-state-reasons” and/or “document-state-reasons” attributes:

'none' (JD):  There are no reasons for the job's or document’s current state. This state reason is semantically equivalent to "job-state-reasons" or “document-state-reasons” attribute without any value and MUST be used when there is no other value, since the 1setOf attribute syntax requires at least one value.

'job-hold-until-specified' (J):  The value of the job's "job-hold-until" attribute was specified with a time period that is still in the future.  The job MUST NOT be a candidate for processing until this reason is removed and there are no other reasons to hold the job.  This value SHOULD be supported if the "job-hold-until" Job Template attribute is supported.

'resources-are-not-supported' (JD): At least one of the resources needed by the job/document, such as media, fonts, resource objects, etc., is not supported on any of the physical printers for which the job/document is a candidate.  This condition MAY be detected when the job/document is accepted, or subsequently while the job/document is pending or processing, depending on implementation.  The job/document may (1) remain in its current state, (2) be moved to the 'pending-held' state, depending on implementation and/or job scheduling policy, or (3) scheduled normally, but the Printer is put into the 'stopped' state when the job/document is attempted to be processed on the Printer.  This value is intended for use with an implementation that supports the "user-defined-values-supported" Printer attribute (see [prod-print] section 6.1) which allows a job to be accepted with an unsupported 'name' value.

'resources-are-not-ready' (JD):  At least one of the resources needed by the job/document, such as media, fonts, resource objects, etc., is not ready on any of the physical printer's for which the job/document is a candidate.  This condition MAY be detected when the job/document is accepted, or subsequently while the job/document is pending or processing, depending on implementation.  The job/document may remain in its current state or be moved to the 'pending-held' state, depending on implementation and/or job/document scheduling policy.

'[job-]incoming' (JD):  Either (1) the Printer has accepted the Create-Job operation and is expecting additional Send-Document and/or Send-URI operations or (2) the Printer is retrieving/accepting document data as a result of a Print-Job, Print-URI, Send-Document or Send-URI operation.

'job-data-insufficient' (J):  The Create-Job operation has been accepted by the Printer, but the Printer is expecting additional document data before it can move the job into the 'processing' state.  If a Printer starts processing before it has received all data, the Printer removes the  'job-data-insufficient' reason, but the 'job-incoming' remains.  If a Printer starts processing after it has received all data, the Printer removes the 'job-data-insufficient' reason and the 'job-incoming' at the same time.

'unsupported-compression' (JD): The job/document was aborted by the system because the Printer determined while attempting to decompress the document-data's that the compression is actually not among those supported by the Printer.  This value MUST be supported, since "compressions is a REQUIRED operation attribute.

'compression-error' (JD): The job/document was aborted by the system because the Printer encountered an error in the document-data while decompressing it.  If the Printer posts this reason, the document-data has already passed any tests that would have led to the 'unsupported-compression' job-state-reason/document-state-reason.

'document-access-error' (JD):  After accepting a Print-URI or Send-URI request, the Printer could not access one or more documents passed by reference.  This reason is intended to cover any file access problem, including file does not exist and access denied because of an access control problem.  The Printer MAY also indicate the document access error using the "job-document-access-errors" Job Description attribute (see the description of "job-document-access-errors" above).  Whether the Printer aborts the job/document and moves the job/document to the 'aborted' job state or prints all documents that are accessible and moves the job/document to the 'completed' job/document state and adds the 'completed-with-errors' value in the job's "job-state-reasons" or document’s “document-state-reasons” attribute depends on implementation and/or site policy.  This value SHOULD be supported if the Print-URI or Send-URI operations are supported.

'unsupported-document-format' (JD): The job/document was aborted by the system because the document-data's document-format is not among those supported by the Printer.  If the client specifies the document-format as 'application/octet-stream', the printer MAY abort the job/document and post this reason even though the format is a member of the "document-format-supported" printer attribute, but not among the auto-sensed document-formats.  This value MUST be supported, since "document-format" is a REQUIRED operation attribute.

'document-format-error' (JD): The job/document was aborted by the system because the Printer encountered an error in the document-data while processing it.  If the Printer posts this reason, the document-data has already passed any tests that would have led to the 'unsupported-document-format' job-state-reason/document-state-reason.

'submission-interrupted' (JD):  The job or document was not completely submitted for some unforeseen reason, such as: (1) the Printer has crashed before the job was closed by the client, (2) the Printer or the document transfer method has crashed in some non-recoverable way before the document data was entirely transferred to the Printer, (3) the client crashed or failed to close the job before the time-out period.  See the description of the “multiple-operation-time-out” Printer attribute in [RFC2911] Section 4.4.31.

'[job-]outgoing' (JD):  The Printer is transmitting the job/document to the output device.

'job-queued' (J): Job is in the 'processing' state, but more specifically, the Printer has queued the document data.

'[job-]transforming' (JD): Job/document is in the 'processing' state, but more specifically, the Printer is interpreting document data and producing another electronic representation.

'[job-]interpreting' (JD): Job/document is in the 'processing' state, but more specifically, the Printer is interpreting the document data.

'[job-]queued-for-marker' (JD): Job/document is in any of the 'pending-held', 'pending', or 'processing' states, but more specifically, the Printer has completed enough processing of the document to be able to start marking and the job/document is waiting for the marker.  Systems that require human intervention to release jobs using the Release-Job operation, put the job into the 'pending-held' job state.  Systems that automatically select a job to use the marker put the job into the  'pending' job state or keep the job in the 'processing' job state while waiting for the marker, depending on implementation.  All implementations put the job/document into (or back into) the 'processing' state when marking does begin.

'[job-]printing' (JD):  The output device is marking media. This value is useful for Printers which spend a great deal of time processing (1) when no marking is happening and then want to show that marking is now happening or (2) when the job/document is in the process of being canceled or aborted while the job/document remains in the 'processing' state, but the marking has not yet stopped so that impression or sheet counts are still increasing for the job/document.

'printer-stopped-partly' (J):  The value of the Printer's "printer-state-reasons" attribute contains the value 'stopped-partly'.

'printer-stopped' (J):  The value of the Printer's "printer-state" attribute is 'stopped'.

'job-suspended' (J): The job has been suspended while processing using the Suspend-Current-Job operation and other jobs can be processed on the Printer.  The Job can be resumed using the Resume-Job operation which removes this value.  Note: Documents cannot be suspended and there is no Suspend-Current-Document operation.

'[job-]canceled-by-user' (JD):  The job/document was canceled by the owner of the job using the Cancel-Job/Cancel-Document request, i.e., by a user whose authenticated identity is the same as the value of the originating user that created the Job object, or by some other authorized end-user, such as a member of the job owner's security group.  This ‘job-canceled’ value SHOULD be supported.  The ‘canceled-by-user’ value SHOULD be supported for the “document-state-reasons” attributes if the Cancel-Document operation is supported.

'[job-]canceled-by-operator' (JD):  The job/document was canceled by the operator using the Cancel-Job/Cancel-Document request, i.e., by a user who has been authenticated as having operator privileges (whether local or remote).  If the security policy is to allow anyone to cancel anyone's job/document, then these values may be used when the job/document is canceled by other than the owner of the job.  For such a security policy, in effect, everyone is an operator as far as canceling jobs/documents is concerned.  These values SHOULD be supported if the implementation permits canceling by other than the owner of the job.

'[job-]canceled-at-device' (JD):  The job/document was canceled by an unidentified local user, i.e., a user at a console at the device.  This value SHOULD be supported if the implementation supports canceling jobs/documents at the console.

'aborted-by-system' (JD):  The job/document (1) is in the process of being aborted, (2) has been aborted by the system and placed in the 'aborted' state, or (3) has been aborted by the system and placed in the 'pending-held' state, so that a user or operator can manually try the job/document again.  This value SHOULD be supported.

'processing-to-stop-point' (J):  The requester has issued a Cancel-Job/Cancel-Document operation or the Printer object has aborted the job/document, but is still performing some actions on the job/document until a specified stop point occurs or job/document termination/cleanup is completed.
If the implementation requires some measurable time to cancel the job/document in the 'processing' or 'processing-stopped' job/document states, the IPP object MUST use this value to indicate that the Printer object is still performing some actions on the job/document while the job/document remains in the 'processing' or 'processing-stopped' state.  After all the job's/document’s job/document description attributes have stopped incrementing, the Printer object moves the job/document from the 'processing' state to the 'canceled' or 'aborted' job/document states.

'service-off-line' (J):  The Printer is off-line and accepting no jobs.  All 'pending' jobs are put into the 'pending-held' state.  This situation could be true if the service's or document transform's input is impaired or broken.

'[job-]warnings-detected' (JD): If the Printer supports these values, the Printer MUST add the value to “job-state-reasons” / “document-state-reasons” when it generates the first warning message. That is, a single occurrence of this value is present in the “job-state-reasons” / “document-state-reasons” if the Printer has generated one or more warnings.

'[job-]completed-successfully' (JD):  The job/document completed successfully. There were no warnings or errors in printing.  This value SHOULD be supported.

'[job-]completed-with-warnings' (JD):  The job/document completed with warnings.  This value SHOULD be supported if the implementation detects warnings.

'[job-]completed-with-errors' (JD):  The job/document completed with errors (and possibly warnings too) .  This value SHOULD be supported if the implementation detects errors.

'job-restartable' (J): This job is retained (see the description of "job-state" above, specifically "Partitioning of Job States") and is currently able to be restarted using the Restart-Job operation (see [RFC2911] Section 3.3.7).  If 'job-restartable' is a value of the job's "job-state-reasons" attribute, then the IPP object MUST accept a Restart-Job operation for that job.  This value SHOULD be supported if the Restart-Job operation is supported.

'queued-in-device' (JD): The job/document has been forwarded to a device or print system that is unable to send back status.  The Printer sets the job's/document’s "job-state" / “document-state” attribute to 'completed'  and adds the 'queued-in-device' value to the job's/document’s "job-state-reasons" / “document-state-reasons” attribute to indicate that the Printer has no additional information about the job/document and never will have any better information.  See the description of the "job-state" / “document-state” attribute above, specifically the portion on "Forwarding Servers".

NOTE:  The semantics of this attribute or the set of valid values may be different for different document formats.

# Printer attributes

This section lists all of the Printer attributes defined in other IPP documents.  Printer attributes are divided into three disjoint groups:

| Job Template attributes (JT) | These Printer attributes are related directly to the “xxx” Job Template attributes that a client can supply in a Job Creation request as follows: - “xxx-supported” indicate the supported values - “xxx-default” indicate the values that will be used if the client and the PDL omit the attribute. - “xxx-ready” indicate the supported values that are available without human intervention, for those few attributes that require human intervention, such as “media” and possibly “finishing”. |
| --- | --- |
| Printer Description attributes (PD) | These Printer attributes describe the Printer, such as “printer-name” and “printer-location”. Some of them are “xxx-supported” and “xxx-default” for Operation attributes that the client can supply in a Job Creation request. |
| neither (-) | These Printer attributes describe supported and defaults for member attributes of collection Job Template attributes. |

In a Get-Printer-Attributes request, a client can request all of the Printer attributes in the Job Template (JT) or Printer Description (PD) groups by supplying the ‘job-template’ or ‘printer-description’ group name, respectively.  The column labeled “attribute syntax” indicates the attribute syntax (see [RFC2911] section 4.1).  Many “xxx-default” and “xxx-supported” attributes have the ‘collection’ syntax which consists of a number of member attributes.  These member attributes are not listed in this table when they are the same as the corresponding “xxx” attribute in Table 5.  Only those few Printer attribute collection attributes that do not have a corresponding Job Template attributes show the member attributes in this table.  The column labeled “Grp” indicates to which group the attribute belongs.  The column labeled “Printer Conf.” indicates the IPP/1.1 Printer conformance requirements for supporting the attribute.  Those attributes labeled as “MAY” may have conditional requirements that require their support, if the Printer supports a related attribute, operation, or feature.  For example, all Job Template attributes are OPTIONAL for a Printer to support.  However, if a Printer supports Job Template attribute “xxx”, then it MUST also support the corresponding “xxx-default” and “xxx-supported” attributes.  Similarly, if Event Notification is supported, then some of the attributes indicated as “MAY” in this table become MUST.  The Reference and Section columns list the IETF or IEEE-ISTO specification which defines the semantics of the attribute.

ISSUE 09: What other Printer Description attributes are needed, if any?

Table 9 - Printer attributes (Job Template and Printer Description)

| Printer attribute name (Job Template & Printer Description) | Attribute Syntax | Grp | Printer Conf. | Reference |
| --- | --- | --- | --- | --- |
| charset-configured | charset | PD | MUST | [RFC2911] §4.4.17 |
| charset-supported | 1setOf charset | PD | MUST | [RFC2911] §4.4.18 |
| color-supported | boolean | PD | MAY | [RFC2911] §4.4.26 |
| compression-supported | type3 keyword | PD | MUST | [RFC2911] §4.4.32 |
| copies-default | integer(1:MAX) | JT | MAY | [RFC2911] §4.2.5 |
| copies-supported | rangeOfInteger(1:MAX) | JT | MAY | [RFC2911] §4.2.5 |
| cover-back-default | collection | JT | MAY | [prod-print] §3.1.3 |
| cover-back-supported | 1setOf type2 keyword | JT | MAY | [prod-print] §3.1.4 |
| cover-front-default | collection | JT | MAY | [prod-print] §3.1.3 |
| cover-front-supported | 1setOf type2 keyword | JT | MAY | [prod-print] §3.1.4 |
| cover-type-supported | 1setOf type2 keyword | - | MAY | [prod-print] §3.1.2.1 |
| document-format-default | mimeMediaType | PD | MUST | [RFC2911] §4.4.21 |
| document-format-supported | 1setOf mimeMediaType | PD | MUST | [RFC2911] §4.4.22 |
| document-format-varying-attributes | 1setOf type2 keyword | PD | MAY | [set-ops] §6.3 |
| document-overrides-supported | 1setOf type2 keyword |  | MAY | [override] §5.1.14 |
| finishings-col-default | collection | JT | MAY | [prod-print] §3.2.3 |
| finishings-col-ready | 1setOf collection | JT | MAY | [prod-print] §3.2.4 |
| finishings-col-supported | 1setOf type2 keyword | JT | MAY | [prod-print] §3.2.5 |
| finishings-default | 1setOf type2 enum | JT | MAY | [RFC2911] §4.2.6 |
| finishings-ready | 1setOf type2 enum | JT | MAY | [RFC2911] §4.2.6 |
| finishings-supported | 1setOf type2 enum | JT | MAY | [RFC2911] §4.2.6 |
| finishing-template-supported | 1setOf name(MAX) | - | MAY | [prod-print] §3.2.1.1 |
| force-front-side-default | 1setOf integer(1:MAX) | JT | MAY | [prod-print] §3.3 |
| force-front-side-supported | rangeOfInteger(1:MAX) | JT | MAY | [prod-print] §3.3 |
| generated-natural-language-supported | 1setOf naturalLanguage | PD | MUST | [RFC2911] §4.4.20 |
| imposition-template-default | type3 keyword ¦ name(MAX) | - | MAY | [prod-print] §3.4 |
| imposition-template-supported | 1setOf (type3 keyword ¦ name(MAX)) | - | MAY | [prod-print] §3.4 |
| insert-after-page-number-supported | rangeOfInteger(0:MAX) | - | MAY | [prod-print] §3.5.1.1 |
| insert-count-supported | rangeOfInteger(0:MAX) | - | MAY | [prod-print] §3.5.2.1 |
| insert-sheet-default | 1setOf collection | JT | MAY | [prod-print] §3.5.4 |
| insert-sheet-supported | 1setOf type2 keyword | JT | MAY | [prod-print] §3.5.5 |
| ippget-event-life | integer(15:MAX) | PD | MAY | [get-method] §8.1 |
| ipp-versions-supported | 1setOf type2 keyword | PD | MUST | [RFC2911] §4.4.20 |
| job-account-id-default | name(MAX) | JT | MAY | [prod-print] §3.6 |
| job-account-id-supported | boolean | JT | MAY | [prod-print] §3.6 |
| job-accounting-output-bin-default | type3 keyword ¦ name(MAX) | - | MAY | [prod-print] §3.8.3.1 |
| job-accounting-output-bin-supported | 1setOf (type3 keyword ¦ name(MAX)) | - | MAY | [prod-print] §3.8.3.2 |
| job-accounting-sheets-default | collection | JT | MAY | [prod-print] §3.8.4 |
| job-accounting-sheets-supported | 1setOf type2 keyword | JT | MAY | [prod-print] §3.8.5 |
| job-accounting-sheets-type-supported | 1setOf (type3 keyword ¦ name(MAX)) | - | MAY | [prod-print] §3.8.1.1 |
| job-accounting-user-id-default | name(MAX) | JT | MAY | [prod-print] §3.7 |
| job-accounting-user-id-supported | boolean | JT | MAY | [prod-print] §3.7 |
| job-error-sheet-default | collection | JT | MAY | [prod-print] §3.7 |
| job-error-sheet-supported | 1setOf type2 keyword | JT | MAY | [prod-print] §3.9.5 |
| job-error-sheet-type-supported | 1setOf (type3 keyword ¦ name(MAX)) | - | MAY | [prod-print] §3.9.1.1 |
| job-error-sheet-when-supported | 1setOf type2 keyword | - | MAY | [prod-print] §3.9.2.1 |
| job-hold-until-default | type3 keyword ¦ name(MAX) | JT | MAY | [RFC2911] §4.2.2 |
| job-hold-until-supported | 1setOf (type3 keyword ¦ name(MAX)) | JT | MAY | [RFC2911] §4.2.2 |
| job-impressions-supported | rangeOfInteger(0:MAX) | PD | MAY | [RFC2911] §4.4.34 |
| job-k-octets-supported | rangeOfInteger(0:MAX) | PD | MAY | [RFC2911] §4.4.33 |
| job-media-sheets-supported | rangeOfInteger(0:MAX) | PD | MAY | [RFC2911] §4.4.35 |
| job-message-to-operator-default | text(MAX) | JT | MAY | [prod-print] §3.12 |
| job-message-to-operator-supported | boolean | JT | MAY | [prod-print] §3.12 |
| job-priority-default | integer(1:100) | JT | MAY | [RFC2911] §4.2.1 |
| job-priority-supported | 1setOf integer(1:100) | JT | MAY | [RFC2911] §4.2.1 |
| job-settable-attributes-supported | 1setOf type2 keyword | PD | MAY | [set-ops] §6.2 |
| job-sheet-message-default | text(MAX) |  | MAY | [prod-print] §3.12 |
| job-sheet-message-supported | boolean |  | MAY | [prod-print] §3.12 |
| job-sheets-col-default | collection | JT | MAY | [prod-print] §3.11.3 |
| job-sheets-col-supported | 1setOf type2 keyword | JT | MAY | [prod-print] §3.11.4 |
| job-sheets-default | type3 keyword ¦ name(MAX) | JT | MAY | [RFC2911] §4.2.3 |
| job-sheets-supported | 1setOf (type3 keyword ¦ name(MAX)) | JT | MAY | [RFC2911] §4.2.3 |
| max-stitching-locations-supported | integer(1:MAX) | PD | MAY | [prod-print] §5.2 |
| media-back-coating-supported | 1setOf (type3 keyword ¦ name(MAX)) | - | MAY | [prod-print] §3.13.10.2 |
| media-col-default | collection | JT | MAY | [prod-print] §3.13.12 |
| media-color-supported | 1setOf type2 keyword | - | MAY | [prod-print] §3.13.4.1 |
| media-col-ready | 1setOf collection | JT | MAY | [prod-print] §3.13.13 |
| media-col-supported | 1setOf type2 keyword | JT | MAY | [prod-print] §3.13.14 |
| media-default | type3 keyword ¦ name(MAX) | JT | MAY | [RFC2911] §4.2.11 |
| media-front-coating-supported | 1setOf (type3 keyword ¦ name(MAX)) | - | MAY | [prod-print] §3.13.10.1 |
| media-hole-count-supported | 1setOf rangeOfInteger(0:MAX) | - | MAY | [prod-print] §3.13.6.1 |
| media-info-supported | boolean | - | MAY | [prod-print] §3.13.3.1 |
| media-input-tray-check-default | type3 keyword ¦ name(MAX) | JT | MAY | [prod-print] §3.14 |
| media-input-tray-check-supported | 1setOf (type3 keyword ¦ name(MAX)) | JT | MAY | [prod-print] §3.14 |
| media-key-supported | 1setOf (type3 keyword ¦ name(MAX)) | - | MAY | [prod-print] §3.13.1.1 |
| media-order-count-supported | 1setOf rangeOfInteger(1:MAX) | - | MAY | [prod-print] §3.13.7.1 |
| media-pre-printed-supported | 1setOf (type3 keyword ¦ name(MAX)) | - | MAY | [prod-print] §3.13.5.1 |
| media-ready | 1setOf (type3 keyword ¦ name(MAX)) | JT | MAY | [RFC2911] §4.2.11 |
| media-recycled-supported | 1setOf (type3 keyword ¦ name(MAX)) | - | MAY | [prod-print] §3.13.11.1 |
| media-size-supported | 1setOf collection | - | MAY | [prod-print] §3.13.8.3 |
| media-supported | 1setOf (type3 keyword ¦ name(MAX)) | JT | MAY | [RFC2911] §4.2.11 |
| media-type-supported | 1setOf (type3 keyword ¦ name(MAX)) | - | MAY | [prod-print] §3.13.2.1 |
| media-weight-metric-supported | 1setOf rangeOfInteger(1:MAX) | - | MAY | [prod-print] §3.13.9.1 |
| multiple-document-handling-default | type2 keyword | JT | MAY | [RFC2911] §4.2.4 |
| multiple-document-handling-supported | 1setOf type2 keyword | JT | MAY | [RFC2911] §4.2.4 |
| multiple-document-jobs-supported | boolean | PD | MAY | [RFC2911] §4.4.16 |
| multiple-operation-time-out | integer(1:MAX) | PD | MAY | [RFC2911] §4.4.31 |
| natural-language-configured | naturalLanguage | PD | MUST | [RFC2911] §4.4.19 |
| notify-attributes-supported | 1setOf type2 keyword | PD | MAY | [ntfy] §5.3.4 |
| notify-events-default | 1setOf type2 keyword | PD | MAY | [ntfy] §5.3.3 |
| notify-events-supported | 1setOf type2 keyword | PD | MAY | [ntfy] §5.3.3 |
| notify-lease-duration-default | integer(0:67108863) | PD | MAY | [ntfy] §5.3.8 |
| notify-lease-duration-supported | 1setOf (integer(0: 67108863) ¦ rangeOfInteger(0:67108863)) | PD | MAY | [ntfy] §5.3.8 |
| notify-lease-expiration-time | integer(0:MAX) | PD | MAY | [ntfy] §5.4.3 |
| notify-max-events-supported | integer(2:MAX) | PD | MAY | [ntfy] §5.3.3 |
| notify-pull-method-supported | 1setOf type2 keyword | PD | MAY | [ntfy] §5.3.2 |
| notify-schemes-supported | 1setOf uriScheme | PD | MAY | [ntfy] §5.3.1 |
| number-up-default | integer(1:MAX) | JT | MAY | [RFC2911] §4.2.9 |
| number-up-supported | 1setOf (integer(1:MAX) ¦ rangeOfInteger(1:MAX)) | JT | MAY | [RFC2911] §4.2.9 |
| operations-supported | 1setOf type2 enum | PD | MUST | [RFC2911] §4.4.15 |
| orientation-requested-default | type2 enum | JT | MAY | [RFC2911] §4.2.10 |
| orientation-requested-supported | 1setOf type2 enum | JT | MAY | [RFC2911] §4.2.10 |
| output-bin-default | type2 keyword ¦ name(MAX) | JT | MAY | [output-bin] §2.1 |
| output-bin-supported | 1setOf (type2 keyword ¦ name(MAX)) | JT | MAY | [output-bin] §2.1 |
| page-delivery-default | type2 keyword | JT | MAY | [prod-print] §3.15 |
| page-delivery-supported | 1setOf type2 keyword | JT | MAY | [prod-print] §3.15 |
| page-order-received-default | type2 keyword | JT | MAY | [prod-print] §3.16 |
| page-order-received-supported | 1setOf type2 keyword | JT | MAY | [prod-print] §3.16 |
| page-overrides-supported | 1setOf type2 keyword | PD | MAY | [override] §5.2.9 |
| page-ranges-supported | boolean | JT | MAY | [RFC2911] §4.2.7 |
| pages-per-minute | integer(0:MAX) | PD | MAY | [RFC2911] §4.4.36 |
| pages-per-minute-color | integer(0:MAX) | PD | MAY | [RFC2911] §4.4.37 |
| pages-per-subset-supported | boolean | PD | MAY | [override] §5.3.2 |
| parent-printers-supported | 1setOf uri | PD | MAY | [ops-set2] §7.2 |
| pdl-override-supported | type2 keyword | PD | MUST | [RFC2911] §4.4.28 |
| presentation-direction-number-up-default | type2 keyword | JT | MAY | [prod-print] §3.17 |
| presentation-direction-number-up-supported | 1setOf type2 keyword | JT | MAY | [prod-print] §3.17 |
| printer-current-time | dateTime | PD | MAY | [RFC2911] §4.4.30 |
| printer-driver-installer | uri | PD | MAY | [RFC2911] §4.4.8 |
| printer-info | text(127) | PD | MAY | [RFC2911] §4.4.6 |
| printer-is-accepting-jobs | boolean | PD | MUST | [RFC2911] §4.4.23 |
| printer-location | text(127) | PD | MAY | [RFC2911] §4.4.5 |
| printer-make-and-model | text(127) | PD | MAY | [RFC2911] §4.4.9 |
| printer-message-date-time | dateTime | PD | MAY | [set-ops] §6.5 |
| printer-message-from-operator | text(127) | PD | MAY | [RFC2911] §4.4.25 |
| printer-message-time | integer(MIN:MAX) | PD | MAY | [set-ops] §6.4 |
| printer-more-info | uri | PD | MAY | [RFC2911] §4.4.7 |
| printer-more-info-manufacturer | uri | PD | MAY | [RFC2911] §4.4.10 |
| printer-name | name(127) | PD | MUST | [RFC2911] §4.4.4 |
| printer-resolution-default | resolution | JT | MAY | [RFC2911] §4.2.12 |
| printer-resolution-supported | 1setOf resolution | JT | MAY | [RFC2911] §4.2.12 |
| printer-settable-attributes-supported | 1setOf type2 keyword | PD | MAY | [set-ops] §6.1 |
| printer-state | type1 enum | PD | MUST | [RFC2911] §4.4.11 |
| printer-state-change-date-time | dateTime | PD | MAY | [ntfy] §6.2 |
| printer-state-change-time | integer(1:MAX) | PD | MAY | [ntfy] §6.1 |
| printer-state-message | text(MAX) | PD | MAY | [RFC2911] §4.4.13 |
| printer-state-reasons | 1setOf type2 keyword | PD | MUST | [RFC2911] §4.4.12 |
| printer-up-time | integer(1:MAX)) | PD | MUST | [RFC2911] §4.4.29 |
| printer-uri-supported | 1setOf uri | PD | MUST | [RFC2911] §4.4.1 |
| printer-xri-supported | 1setOf collection | PD | MAY | [set-ops] §6.6 |
| xri-uri | uri | PD | MUST | [set-ops] §6.6 |
| xri-authentication | 1setOf type2 keyword | PD | MUST | [set-ops] §6.6 |
| xri-security | 1setOf type2 keyword | PD | MUST | [set-ops] §6.6 |
| print-quality-default | type2 enum | JT | MAY | [RFC2911] §4.2.13 |
| print-quality-supported | 1setOf type2 enum | JT | MAY | [RFC2911] §4.2.13 |
| queued-job-count | integer(0:MAX) | PD | MUST | [RFC2911] §4.4.24 |
| reference-uri-schemes-supported | 1setOf uriScheme | PD | MAY | [RFC2911] §4.4.27 |
| separator-sheets-default | collection | JT | MAY | [prod-print] §3.18.3 |
| separator-sheets-supported | 1setOf type2 keyword | JT | MAY | [prod-print] §3.18.4 |
| separator-sheets-type-supported | 1setOf (type3 keyword ¦ name(MAX)) | - | MAY | [prod-print] §3.18.1.1 |
| sheet-collate-default | type2 keyword | JT | MAY | [job-prog] §3.1 |
| sheet-collate-supported | 1setOf type2 keyword | JT | MAY | [job-prog] §3.1 |
| sides-default | type2 keyword | JT | MAY | [RFC2911] §4.2.8 |
| sides-supported | 1setOf type2 keyword | JT | MAY | [RFC2911] §4.2.8 |
| stitching-locations-supported | 1setOf (integer(0:MAX) ¦ rangeOfInteger(0:MAX)) | - | MAY | [prod-print] §3.2.2.3.1 |
| stitching-offset-supported | 1setOf (integer (0:MAX) ¦ rangeOfInteger(0:MAX)) | - | MAY | [prod-print] §3.2.2.2.1 |
| stitching-reference-edge-supported | 1setOf type2 keyword |  | MAY | [prod-print] §3.2.2.1.1 |
| subordinate-printers-supported | 1setOf uri | PD | MAY | [ops-set2] §7.1 |
| uri-authentication-supported | 1setOf type2 keyword | PD | MUST | [RFC2911] §4.4.2 |
| uri-security-supported | 1setOf type2 keyword | PD | MUST | [RFC2911] §4.4.3 |
| user-defined-values-supported | 1setOf type2 keyword | PD | MAY | [prod-print] §5.1 |
| x-image-position-default | type2 keyword | JT | MAY | [prod-print] §3.19.2 |
| x-image-position-supported | 1setOf type2 keyword | JT | MAY | [prod-print] §3.19.2 |
| x-image-shift-default | integer(MIN:MAX) | JT | MAY | [prod-print] §3.19.3 |
| x-image-shift-supported | rangeOfInteger(MIN:MAX) | JT | MAY | [prod-print] §3.19.3 |
| xri-authentication-supported | 1setOf type2 keyword | PD | MAY | [set-ops] §6.8 |
| xri-security-supported | 1setOf type2 keyword | PD | MAY | [set-ops] §6.9 |
| xri-uri-scheme-supported | 1setOf uriScheme | PD | MAY | [set-ops] §6.7 |
| x-side1-image-shift-default | integer(MIN:MAX) | JT | MAY | [prod-print] §3.19.4 |
| x-side1-image-shift-supported | rangeOfInteger(MIN:MAX) | JT | MAY | [prod-print] §3.19.4 |
| x-side2-image-shift-default | integer(MIN:MAX) | JT | MAY | [prod-print] §3.19.5 |
| x-side2-image-shift-supported | rangeOfInteger(MIN:MAX) | JT | MAY | [prod-print] §3.19.5 |
| y-image-position-default | type2 keyword | JT | MAY | [prod-print] §3.19.6 |
| y-image-position-supported | 1setOf type2 keyword | JT | MAY | [prod-print] §3.19.6 |
| y-image-shift-default | integer(MIN:MAX) | JT | MAY | [prod-print] §3.19.7 |
| y-image-shift-supported | rangeOfInteger(MIN:MAX) | JT | MAY | [prod-print] §3.19.7 |
| y-side1-image-shift-default | integer(MIN:MAX) | JT | MAY | [prod-print] §3.19.8 |
| y-side1-image-shift-supported | rangeOfInteger(MIN:MAX) | JT | MAY | [prod-print] §3.19.8 |
| y-side2-image-shift-default | integer(MIN:MAX) | JT | MAY | [prod-print] §3.19.9 |
| y-side2-image-shift-supported | rangeOfInteger(MIN:MAX) | JT | MAY | [prod-print] §3.19.9 |

# New Values for Existing Printer Description Attributes

This section contains those attributes for which additional values are added.

## operations-supported (1setOf type2 enum)

The following “operation-id” values are added in order to support the new operations defined in this document:

Table 10 – Operation-id assignments

| Value | Operation Name | Reference |
| --- | --- | --- |
| 0x0032 | Cancel-Current-Document | [this-doc] §2 |
| 0x0033 | Cancel-Document | [this-doc] §2 |
| 0x0034 | Get-Document-Attributes | [this-doc] §2 |
| 0x0035 | Get-Documents | [this-doc] §2 |
| 0x0036 | Delete-Document | [this-doc] §2 |
| 0x0037 | Set-Document-Attributes | [this-doc] §2 |
| 0x0038 | Validate-Document | [this-doc] §2 |

# Conformance Requirements

This section specifies the conformance requirements for Printers and clients.

## Printer conformance requirements

In order to conform to this specification, a Printer:

- MUST meet the conformance requirements for Printers specified in [RFC2911].

- MUST support the Document operations that section 3 REQUIRES a Printer to support.

- MUST support the operation attributes in the Send-Document operation and, if supported, the Send-URI operation that section 4 REQUIRES a Printer to support.

- MAY support any of the Document Template attributes in Document Creation operations as indicated in section 5 and MUST NOT support them for contexts for where they are not defined as indicated in the Table 6 with an  “n”.

- MUST support the Document Description attributes that section 6 REQUIRES a Printer to support.

- MUST support the Printer attributes that section 7 REQUIRES a Printer to support.

- MUST support the values of Printer Descriptions attribute that section 8 REQUIRES a Printer to support.

## Client conformance requirements

In order to conform to this specification, a client:

- MUST meet the conformance requirements for clients specified in [RFC2911]

- MAY support any of the Document operations defined in section 3.  ISSUE 10: Or should the client be REQUIRED to support some of the Document operations?

- MUST supply the operation attributes that section 4 REQUIRES a client to supply in operation requests.

- MUST supply any Document Template attributes in the format that section 5 REQUIRES a client to supply in Document Creation operations and MUST NOT support them for contexts for where they are not defined as indicated in the Table 6 with an  “n”.

- SHOULD support any of the Document Description attributes that section 6 REQUIRES a Printer to support.

- MAY support any of the Printer attributes listed in section 7.

- MAY support any of the values of the Printer Descriptions attributes in section 8.

# Normative References

[adm-ops]

Kugler, C, Hastings, T., Lewis, H., "Internet Printing Protocol (IPP): Job and Printer Administrative Operations", <draft-ietf-ipp-ops-set2-03.txt>, July 17, 2001.

[coll]

deBry, R., Hastings, T., Herriot, R., Ocke, K., and P. Zehler, "Internet Printing Protocol (IPP): The ‘collection’ attribute syntax", <draft-ietf-ipp-collection-05.txt>, work in progress, July 17, 2001.

[finish]

Hastings, T., Fullman, D., "Internet Printing Protocol (IPP): “finishings” attribute values extension”, IEEE-ISTO 5100.1-2001, February 5, 2001, ftp://ftp.pwg.org/pub/pwg/standards/pwg5100.1.pdf

[get-method]

Herriot, R., and T. Hastings, "Internet Printing Protocol (IPP):  The ‘ippget’ Delivery Method for Event Notifications", <draft-ietf-ipp-notify-get-08.txt>, September 5, 2002.

[job-prog]

Hastings, T., Lewis, H., and R. Bergman, "Internet Printing Protocol (IPP): Job Progress Attributes", <draft-ietf-ipp-job-prog-03.txt>  work in progress, July 17, 2001.

[not-srv]

Hastings, T., Lewis, H, and Ira McDonald, “Distributed Notification Server”, PWG IEEE-ISTO standard to be started.

[ntfy]

Herriot, R., Hastings, T., Shepherd, M., deBry, R., Isaacson, S., Martin, J., and R. Bergman, "Internet Printing Protocol/1.1: Event Notifications and Subscriptions", <draft-ietf-ipp-not-spec-08.txt>, November 19, 2001.

[output-bin]

Hastings, T., and R. Bergman, “Internet Printing Protocol (IPP): output-bin attribute extension”, IEEE-ISTO 5100.2-2001, February 7, 2001, ftp://ftp.pwg.org/pub/pwg/standards/pwg5100.2.pdf.

[override]

Herriot, R., Ocke, K., "Internet Printing Protocol (IPP): Override Attributes for Documents and Pages", IEEE-ISTO 5100.4-2001, February 7, 2001, ftp://ftp.pwg.org/pub/pwg/standards/pwg5100.4.pdf.

[prod-print]

Ocke, K., Hastings, T., "Internet Printing Protocol (IPP): Production Printing Attributes - Set1", IEEE-ISTO 5100.3-2001, February 12, 2001, ftp://ftp.pwg.org/pub/pwg/standards/pwg5100.3.pdf.

[RFC2911]

R. deBry, T. Hastings, R. Herriot, S. Isaacson, P. Powell, "Internet Printing Protocol/1.1: Model and Semantics", RFC 2911,  September 2000.

# Informative References

[RFC2565]

Herriot, R., Butler, S., Moore, P., and R.  Turner, "Internet Printing Protocol/1.0: Encoding and Transport", RFC 2565, April 1999.

[RFC2566]

R.  deBry, T.  Hastings, R.  Herriot, S.  Isaacson, and P.  Powell, "Internet Printing Protocol/1.0: Model and Semantics", RFC 2566, April 1999.

[RFC2567]

Wright, D., "Design Goals for an Internet Printing Protocol", RFC 2567, April 1999.

[RFC2568]

Zilles, S., "Rationale for the Structure and Model and Protocol for the Internet Printing Protocol", RFC 2568, April 1999.

[RFC2569]

Herriot, R., Hastings, T., Jacobs, N., Martin, J., "Mapping between LPD and IPP Protocols", RFC 2569, April 1999.

[RFC2616]

R.  Fielding, J.  Gettys, J.  Mogul, H.  Frystyk, L.  Masinter, P.  Leach, T.  Berners-Lee, “Hypertext Transfer Protocol - HTTP/1.1”, RFC 2616, June 1999.

[RFC3196]

Hastings, T., Manros, C., Zehler, P., Kugler, C., and H. Holst, “Internet Printing Protocol/1.1: Implementer’s Guide”, RFC 3196, November 2001.

[RFC3239]

Kugler, C., Lewis, H. and T. Hastings, “Internet Printing Protocol (IPP): Requirements for Job, Printer, and Device Administrative Operations”, RFC 3239, February 2002.

# IANA Considerations

This section contains the registration information for IANA to add to the various IPP Registries according to the procedures defined in RFC 2911 [RFC2911] section 6 to cover the definitions in this document.

Note  Replace the References below with the RFC number for the appropriate document, so that it accurately reflects the content of the information for the IANA Registry.

## Attribute Registrations

The following table lists all the attributes defined in this document.   These are to be registered according to the procedures in RFC 2911 [RFC2911] section 6.2.

Document Description attributes:

Document Description attributes:                 Reference:   Section

attributes-charset (charset)                     [RFC2911]    4.3.19

attributes-natural-language (naturalLanguage)    [RFC2911]    4.3.20

compression (type3 keyword)                      [RFC2911]    3.2.1.1

current-page-order (type2 keyword)               [prod-print] 4.1

date-time-at-completed (dateTime)                [RFC2911]    4.3.14.7

date-time-at-creation (dateTime)                 [RFC2911]    4.3.14.5

date-time-at-processing (dateTime)               [RFC2911]    4.3.14.6

detailed-status-message (1setOf text (MAX))      [RFC2911]    4.3.10

document-access-errors (1setOf text(MAX))        [RFC2911]    4.3.11

document-format (documentFormat)                 [RFC2911]    3.2.1.1

document-name (name(MAX))                        [RFC2911]    3.2.1.1

document-natural-language (naturalLanguage)      [RFC2911]    3.2.1.1

document-number (integer(0:MAX))                 [this spec]  6.1

document-state (type1 enum)                      [this spec]  6.3

document-state-reasons (1setOf type2 keyword)    [this spec]  6.5!

impressions  (integer(0:MAX))                    [RFC2911]    4.3.17.2

impressions-completed (integer(0:MAX))           [RFC2911]    4.3.18.2

impressions-completed-current-copy               [job-prog]   4.4

(integer(0:MAX))

k-octets  (integer(0:MAX))                       [RFC2911]    4.3.17.1

k-octets-processed (integer(0:MAX))              [RFC2911]    4.3.18.1

last-document  (boolean)                         [RFC2911]    4.3.12

media-sheets  (integer(0:MAX))                   [RFC2911]    4.3.17.3

media-sheets-completed (integer(0:MAX))          [RFC2911]    4.3.18.3

more-info (uri)                                  [RFC2911]    4.3.4

printer-up-time (integer (1:MAX))                [RFC2911]    4.3.14.4

sheet-completed-copy-number (integer(0:MAX))     [job-prog]   4.2

time-at-completed (integer (MIN:MAX))            [RFC2911]    4.3.14.3

time-at-creation (integer (MIN:MAX))             [RFC2911]    4.3.14.1

time-at-processing (integer (MIN:MAX))           [RFC2911]    4.3.14.2

warnings-count (integer(0:MAX))                  [override]   6.1

Document Templates attributes:                   Reference:   Section:

copies (integer(1:MAX))                          [RFC2911]    4.2.5

cover-back (collection)                          [prod-print] 3.1

media (type3 keyword | name(MAX))                [prod-print] 3.1.1

media-col (collection)                           [prod-print] 3.1.1

cover-front (collection)                       [prod-print] 3.1

cover-type (type2 keyword)                     [prod-print] 3.1.2

media (type3 keyword | name(MAX))              [prod-print] 3.1.1

media-col (collection)                         [prod-print] 3.1.1

cover-type (type2 keyword)                       [prod-print] 3.1.2

finishings (1setOf type2 enum)                   [RFC2911]    4.2.6

[finish]     2

finishings-col (collection)                      [prod-print] 3.2

finishing-template (name(MAX))                 [prod-print] 3.2.1

stitching (collection)                         [prod-print] 3.2.2

stitching-locations (1setOf integer(0:MAX))  [prod-print] 3.2.2.3

stitching-offset (integer (0:MAX))           [prod-print] 3.2.2.2

stitching-reference-edge (type2 keyword)     [prod-print] 3.2.2.1

force-front-side (1setOf integer(1:MAX))         [prod-print] 3.3

imposition-template (type2 keyword | name(MAX))  [prod-print] 3.4

insert-sheet (1setOf collection)                 [prod-print] 3.5

insert-after-page-number (integer(0:MAX))      [prod-print] 3.5.1

insert-count (integer(0:MAX))                  [prod-print] 3.5.2

media (type3 keyword | name(MAX))              [prod-print] 3.5.3

media-col (collection)                         [prod-print] 3.5.3

media (type3 keyword | name(MAX))                [RFC2911]    4.2.11

media-col (collection)                           [prod-print] 3.13

media-back-coating (type3 keyword |            [prod-print] 3.13.10

name(MAX))

media-color (type3 keyword | name(MAX))        [prod-print] 3.13.4

media-front-coating (type3 keyword |           [prod-print] 3.13.10

name(MAX))

media-hole-count (integer(0:MAX))              [prod-print] 3.13.6

media-info (text(255))                         [prod-print] 3.13.3

media-key (type3 keyword | name(MAX))          [prod-print] 3.13.1

media-order-count (integer(1:MAX))             [prod-print] 3.13.7

media-preprinted (type3 keyword | name(MAX))   [prod-print] 3.13.5

media-recycled (type3 keyword | name(MAX))     [prod-print] 3.13.11

media-size (collection)                        [prod-print] 3.13.8

x-dimension (integer(0:MAX))                 [prod-print] 3.13.8.1

y-dimension (integer(0:MAX))                 [prod-print] 3.13.8.2

media-type (type3 keyword | name(MAX)          [prod-print] 3.13.2

media-weight-metric (integer(0:MAX))           [prod-print] 3.13.9

media-input-tray-check (type3 keyword |          [prod-print] 3.14

name(MAX))

number-up (integer(1:MAX))                       [RFC2911]    4.2.9

orientation-requested (type2 enum)               [RFC2911]    4.2.10

output-bin (type2 keyword | name(MAX))           [output-bin] 2.1

page-delivery (type2 keyword)                    [prod-print] 3.15

page-order-received (type2 keyword)              [prod-print] 3.16

page-overrides (1setOf collection)               [override]   5.2

document-copies (1setOf rangeOfInteger(MAX))   [override]   5.2.3

input-documents (1setOf rangeOfInteger(MAX))   [override]   5.2.1

output-documents (1setOf rangeOfInteger(MAX))  [override]   5.2.2

pages (1setOf rangeOfInteger(MAX))             [override]   5.2.4

<any other Job Template attributes that

affect pages, such as media or media-col (

page-ranges (1setOf rangeOfInteger(1:MAX))       [RFC2911]    4.2.7

pages-per-subset (1setOf integer(1:MAX))         [prod-print] 5.3

presentation-direction-number-up (type2          [prod-print] 3.17

keyword)

print-quality (type2 enum)                       [RFC2911]    4.2.13

printer-resolution (resolution)                  [RFC2911]    4.2.12

separator-sheets (collection)                    [prod-print] 3.18

media (type3 keyword | name(MAX))              [prod-print] 3.18.2

media-col (collection)                         [prod-print] 3.18.2

separator-sheets-type (type3 keyword |         [prod-print] 3.18.1

name(MAX))

sheet-collate (type2 keyword)                    [job-prog]   3.1

sides (type2 keyword)                            [RFC2911]    4.2.8

x-image-position (type2 keyword)                 [prod-print] 3.19.2

x-image-shift (integer(MIN:MAX))                 [prod-print] 3.19.3

x-side1-image-shift (integer(MIN:MAX))           [prod-print] 3.19.4

x-side2-image-shift (integer(MIN:MAX))           [prod-print] 3.19.5

y-image-position (type2 keyword)                 [prod-print] 3.19.6

y-image-shift (integer(MIN:MAX))                 [prod-print] 3.19.7

y-side1-image-shift (integer(MIN:MAX))           [prod-print] 3.19.8

y-side2-image-shift (integer(MIN:MAX))           [prod-print] 3.19.9

Operation attributes:

document-number                                    [this spec]   3

## Additional Enum Attribute Value Registrations for the “operations-supported” Printer Attribute

The following table lists all the new enum attribute values defined in this document as additional type2 enum values for use with the “operations-supported” Printer Description attribute.  These are to be registered according to the procedures in RFC 2911 [RFC2911] section 6.1.

Value      Name                                Reference   Section:

--------   ----------------------              ---------   -------

operations-supported (type2 enum):

0x0032      Cancel-Current-Document              [this-doc]    §2

0x0033      Cancel-Document                      [this-doc]    §2

0x0034      Get-Document-Attributes              [this-doc]    §2

0x0035      Get-Documents                        [this-doc]    §2

0x0036      Delete-Document                      [this-doc]    §2

0x0037      Set-Document-Attributes              [this-doc]    §2

0x0038      Validate-Document                    [this-doc]    §2

The resulting enum attribute value registrations will be published in the

http://www.iana.org/assignments/ipp-registrations file.

## Operation Registrations

The following table lists all of the operations defined in this document.  These are to be registered according to the procedures in RFC 2911 [RFC2911] section 6.4.

Operations:                                    Reference:   Section:

Cancel-Current-Document                        [this-doc]    §3

Cancel-Document                                [this-doc]    §3

Get-Document-Attributes                        [this-doc]    §3

Get-Documents                                  [this-doc]    §3

Delete-Document                                [this-doc]    §3

Set-Document-Attributes                        [this-doc]    §3

Validate-Document                              [this-doc]    §3

# Internationalization Considerations

The internationalization considerations for this extension for handling the Document object are the same as for handling the Job object in [RFC2911].

# Security Considerations

The Document object should provide no more increased security considerations than the Job object.  The Printer MUST provide the same access control for Document objects as for Job objects.

# Contributors

# Acknowledgments

# Author’s Address

Tom Hastings

Xerox Corporation

737 Hawaii St.

El Segundo, CA 90245

Phone: 310 333-6413

Fax:  310 333-5514

e-mail: hastings@cp10.es.xerox.com

Peter Zehler

Xerox Corporation

MS: 0128-30E

800 Phillips Road

Webster, NY 14580-9701

Phone: 1+585+2658755

Fax:

e-mail: PZehler@crt.xerox.com

Additional contributors:

Don Fullman, Xerox

Ira McDonald, High North

# Annex B: Change Log (informative)

Version 0.3, 9 September 2002:

- Reformatted as an IEEE-ISTO standard and added the usual sections for a standard.

- Added “original-requesting-user-name” Operation attribute to Table 3.

- Added “y” to “output-bin” in Table 5 so it MAY be a Document Template attribute.

- Clarified the * and ** in Table 6 - Job and Document Description attributes.

- Added section 7 all of the Printer attributes defined In any IPP standard.

- Allocated operation-id enums in section 8 to the 7 Document operations.

- Added the conformance section 9

- Completed the IANA Considerations section 12.

The Document object is not a sub-job.  If Job trees are desired where sub-jobs can be on different network nodes, such as in JDF, then a separate specification should be generated for Job trees.

The Send-Document operation is OPTIONAL for a Printer to conform to [RFC2911], but is REQUIRED for a Printer to conform to this Document object specification.

The “compression”, “document-format”, “document-name”, and “document-natural-language” MAY be supplied in the Create-Job Job Creation operation as an extension to [RFC2911] as specified in [override].  If supplied in any Job Creation operation, the Printer copies them to corresponding Job Description attributes for subsequent Job query.  These supplied attributes become defaults for the Documents in the job and used if the client does not supply the corresponding operation attribute in the Send-Document or Send-URI Document Creation operation.

The “document-overrides” (collection) attribute MAY be supplied by the client in a Send-Document or Send-URI request as an Operation attribute to apply document overrides to this and/or subsequent documents in the job.  See the “page-overrides” Job Template attribute in Table 5 for the listing of the member attributes.  However, with the introduction of the Document object, the “document-overrides” (collection) attribute SHOULD NOT be used (either as a Job Template attribute or an Operation attribute).  Instead, the client simply supplies the Document Template attributes (see Table 5) for each Document Creation request (in a new Document Template attribute group) without needing a collection.

The “input-document-number” operation attribute sets the Document’s “document-number” Document Description attribute (see section 6.1).

The “page-overrides” (collection) attribute MAY be supplied by the client in a Send-Document or Send-URI request as an Operation attribute to apply page overrides to this and/or subsequent documents in the job.  See the “page-overrides” Job Template attribute in Table 5 for the listing of the member attributes.  However, with the introduction of the Document object, the “page-overrides” (collection) attribute SHOULD be more simply supplied as one of the Document Template attributes for this document only.

The Printer returns the “redirect-other-site” operation attribute as a hint that it will redirect any future Get-Notification operation requests to a separate Notification Server.

The IPP “copies” attribute is an extensive attribute, so its effect when supplied at the job level is not always inherited by the documents in a multi-document job.  Instead, its effect depends on the value of the “multiple-document-handling” Job Template attribute.  The ‘single-document’ and ‘single-document-new-sheet’ values produce copies of the job as a whole with the multiple input documents concatenated into a single output document for each job copy.  The ‘separate-document-uncollated-copies’ value produce N copies of the first input document followed by N copies of the second input document, etc.  The ‘separate-document-collated-copies’ produce N successive job copies, each job copy consisting of 1 copy of the first document followed by 1 copy of the second document, etc.

The IPP “finishings” and “finishings-col” attributes are extensive attributes, so their effect when supplied at the job level is not always inherited by the documents in a multi-document job.  Instead, their effect depends on the value of the “multiple-document-handling” Job Template attribute.  The ‘single-document’ and ‘single-document-new-sheet’ cause the finishing to be applied to each job copy as a whole.  The ‘separate-documents-collated-copies’ and ‘separate-document-uncollated-copies’ values cause the finishing to be applied to each document.

The effect of the IPP “force-front-side” and “insert-sheet” attributes when supplied at the job level of a multi-document job depends on the value of the “multiple-document-handling” Job Template attribute.  For the ‘single-document’ and ‘single-document-new-sheet’ values, the pages are numbered as a single set from 1 to n for the job as a whole.  For the ‘separate-documents-collated-copies’ and ‘separate-document-uncollated-copies’ values, the pages are numbered from 1 to n for each document separately.

The effect of the IPP “page-overrides” and “page-ranges”, attributes when supplied at the job level of a multi-document job depends on the value of the “multiple-document-handling” Job Template attribute.  For the ‘single-document’ and ‘single-document-new-sheet’ values, the pages are numbered as a single set from 1 to n for the job as a whole.  For the ‘separate-documents-collated-copies’ and ‘separate-document-uncollated-copies’ values, the pages are numbered from 1 to n for each document separately.

The IPP “pages-per-subset” attribute can only be used when the “multiple-document-handling” Job Template attribute is ‘separate-documents-collated-copies’ or ‘separate-documents-uncollated-copes’, since the purpose of the “pages-per-subset” attribute is to produce separate output documents for each subset.  Otherwise, the Printer MUST ignore the “pages-per-subset” attribute.

When documents in a multiple document job have the same page order, the Printer populates both the Job and Document “current-page-order” with the same value, so that clients that only know about the Job Description attribute will get a value.  When the page orders differ, the Printer doesn’t populate the “current-page-order” Job Description attribute, but only the Document Description attribute of each Document.

The Document Description attribute that corresponds to the “job-id” Job Description attribute is the (new) “document-number” attribute.

The Document Description attribute that corresponds to the “job-name” Job Description attribute is the (existing) “document-name” attribute defined in [RFC2911].

The Document Description attribute that corresponds to the “job-state” Job Description attribute is the  (new) “document-state” attribute.

There is no Document Description attribute that corresponds to the “job-state-message”.  Printers can use the “job-state-message” to describe any document level conditions.

The Document Description attribute that corresponds to the “job-state-reasons” Job Description attribute is the  (new) “document-state-reasons” attribute.

The Document object is not really a sub-job.  If the Document object were its own independent active object, i.e., were a sub-job, then the Document state would affect the job state and there would be complex and debatable roll-up rules for how the various “document-state” values of all of the Job’s document are rolled up to give a single value for the Job’s “job-state” attribute.  Instead, the Job state reflects the overall processing of the job and is the same whether or not the Document object is supported.

The Document Description attribute that corresponds to the “job-state-reasons” Job Description attribute is the  (new) “document-state-reasons” attribute.