Rules for Values Allowed in Set-XXX-Attributes

Robert Herriot

The following table shows what values a Printer accepts as values for each attribute. However, a Printer MAY make a value read-only, especially if it supports only one possible value. For example, “color-supported” is “false” and read-only for a monochrome printer.

There are only a few basic themes that appear in the table:

- Always read-only

- Any value allowed by Printer’s xxx-supported according to the rules of IPP/1.1

- Any value allowed by Reset-Printer’s xxx-supported according to the rules of IPP/1.1 (see Get-Reset-Printer-Attributes below)

- Any value of the proper syntax type:

- Any text(127)

- Any name(127)

- Any naturalLanguage

- Any uri

- Any boolean

- Any positive integer

- Any datetime

- 1setOf any uri

- Combination of #3 or #4b.

There needs to be an attribute which specifies those attributes which can have name values added to the xxx-supported attributes, e.g. “attributes-with-names” and its value would contain a subset of the three attributes names: “media”, “job-hold-until” and “job-sheets”.

- A new attribute that specifies the allowed values for “job-priority-supported”, e.g. “job-priority-supported-values(1setOf (integer(1:MAX) | rangeOfInteger(1:MAX))).  ISSUE: do we mandate that all implementation support either a single read-only value or any value between 1 and 100.

There is an operation Get-Reset-Printer-Attributes (or some other name) which returns the value that an attribute has after the Reset operation. It behaves like Get-Printer-Attributes except that the value for each attribute is the value that Get-Printer-Attributes would return after a “Reset” operation.  The value of an attribute returned by Get-Printer-Attributes and Get-Reset-Printer-Attributes may differ if a client has performed a Set-Printer-Attributes operation.  The Get-Reset-Printer-Attributes operation returns any Printer attribute defined in IPP/1.1 except for the following attributes which are unaffected by a “Reset” operation.

- printer-uri-supported (1setOf uri)

- printer-name (name(127))

- printer-location (text(127))

- printer-info (text(127))

- printer-state (type1 enum)

- printer-state-reasons (1setOf type2 keyword)

- printer-state-message (text(MAX))

- printer-is-accepting-jobs (boolean) (if Reset always sets this attribute to true, this attribute doesn’t belong in this list)

- printer-up-time (integer(1:MAX)) (if Reset sets this attribute to 0, then this attribute doesn’t belong in this list)

- printer-current-time (dateTime)

- queued-job-count (integer(0:MAX))

Issue: Is this right list of attributes?

Issue: What do we by reset? Is it a soft reset, hard reset or something else.

| # JobTemplate Attributes | # Values allowed for Set |
| --- | --- |
| job-priority (integer(1:100)) | Any value allowed by Printer’s xxx-supported |
| job-hold-until (type3 keyword ¦ name (MAX)) | Any value allowed by Printer’s xxx-supported |
| job-sheets (type3 keyword ¦ name(MAX)) | Any value allowed by Printer’s xxx-supported |
| multiple-document-handling (type2 keyword) | Any value allowed by Printer’s xxx-supported |
| copies (integer(1:MAX)) | Any value allowed by Printer’s xxx-supported |
| finishings (1setOf type2 enum) | Any value allowed by Printer’s xxx-supported |
| page-ranges (1setOf rangeOfInteger (1:MAX)) | Any value allowed by Printer’s xxx-supported |
| sides (type2 keyword) | Any value allowed by Printer’s xxx-supported |
| number-up (integer(1:MAX)) | Any value allowed by Printer’s xxx-supported |
| orientation-requested (type2 enum) | Any value allowed by Printer’s xxx-supported |
| media (type3 keyword ¦ name(MAX)) | Any value allowed by Printer’s xxx-supported |
| printer-resolution (resolution) | Any value allowed by Printer’s xxx-supported |
| print-quality (type2 enum) | Any value allowed by Printer’s xxx-supported |

| # Job Description Attributes | # Values allowed for Set |
| --- | --- |
| job-uri (uri) | read-only |
| job-id (integer(1:MAX)) | read-only |
| job-printer-uri (uri) | read-only |
| job-more-info (uri) | read-only |
| job-name (name(MAX)) | read-only |
| job-originating-user-name (name(MAX)) | read-only |
| job-state (type1 enum) | read-only |
| Forwarding Servers | read-only |
| job-state-reasons (1setOf type2 keyword) | read-only |
| job-state-message (text(MAX)) | read-only |
| job-detailed-status-messages (1setOf text(MAX)) | read-only |
| job-document-access-errors (1setOf text(MAX)) | read-only |
| number-of-documents (integer(0:MAX)) | read-only |
| output-device-assigned (name(127)) | read-only |
| time-at-creation (integer(MIN:MAX)) | read-only |
| time-at-processing (integer(MIN:MAX)) | read-only |
| time-at-completed (integer(MIN:MAX)) | read-only |
| job-printer-up-time (integer(1:MAX)) | read-only |
| date-time-at-creation (dateTime) | read-only |
| date-time-at-processing (dateTime) | read-only |
| date-time-at-completed (dateTime) | read-only |
| number-of-intervening-jobs (integer(0:MAX)) | read-only |
| job-message-from-operator (text(127)) | Any text(127) |
| job-k-octets (integer(0:MAX)) | read-only |
| job-impressions (integer(0:MAX)) | read-only |
| job-media-sheets (integer(0:MAX)) |  |
| job-k-octets-processed (integer(0:MAX)) | read-only |
| job-impressions-completed (integer(0:MAX)) | read-only |
| job-media-sheets-completed (integer(0:MAX)) | read-only |
| attributes-charset (charset) | read-only |
| attributes-natural-language (naturalLanguage) | Any naturalLanguage |

| # Printer JobTemplate Attributes | # Values allowed for Set |
| --- | --- |
| job-priority-default (integer(1:100)) | Any value allowed by Printer’s xxx-supported |
| job-hold-until-default (type3 keyword ¦ name (MAX)) | Any value allowed by Printer’s xxx-supported |
| job-sheets-default (type3 keyword ¦ name(MAX)) | Any value allowed by Printer’s xxx-supported |
| multiple-document-handling-default (type2 keyword) | Any value allowed by Printer’s xxx-supported |
| copies-default (integer(1:MAX)) | Any value allowed by Printer’s xxx-supported |
| finishings-default (1setOf type2 enum) | Any value allowed by Printer’s xxx-supported |
| sides-default (type2 keyword) | Any value allowed by Printer’s xxx-supported |
| number-up-default (integer(1:MAX)) | Any value allowed by Printer’s xxx-supported |
| orientation-requested-default (type2 enum) | Any value allowed by Printer’s xxx-supported |
| media-default (type3 keyword ¦ name(MAX)) | Any value allowed by Printer’s xxx-supported |
| printer-resolution-default (resolution) | Any value allowed by Printer’s xxx-supported |
| print-quality-default (type2 enum) | Any value allowed by Printer’s xxx-supported |
| job-priority-supported (integer(1:100)) | Need new attribute that specifies allowed values or any integer(1:100) |
| job-hold-until-supported (1setOf(type3 keyword ¦ name (MAX))) | Any value allowed by Reset-Printer’s xxx-supported or any name |
| job-sheets-supported (1setOf(type3 keyword ¦ name(MAX))) | Any value allowed by Reset-Printer’s xxx-supported or any name |
| multiple-document-handling-supported (1setOf type2 keyword) | Any value allowed by Reset-Printer’s xxx-supported |
| copies-supported (rangeOfInteger(1:MAX)) | Any value allowed by Reset-Printer’s xxx-supported |
| finishings-supported (1setOf type2 enum) | Any value allowed by Reset-Printer’s xxx-supported |
| page-ranges-supported (boolean) | Any boolean |
| sides-supported (1setOf type2 keyword) | Any value allowed by Reset-Printer’s xxx-supported |
| number-up-supported (1setOf (integer(1:MAX) ¦ rangeOfInteger(1:MAX))) | Any value allowed by Reset-Printer’s xxx-supported |
| orientation-requested-supported (1setOf type2 enum) | Any value allowed by Reset-Printer’s xxx-supported |
| media-supported (1setOf (type3 keyword ¦ name(MAX))) | Any value allowed by Reset-Printer’s xxx-supported |
| printer-resolution-supported (1setOf resolution) | Any value allowed by Reset-Printer’s xxx-supported |
| print-quality-supported (1setOf type2 enum) | Any value allowed by Reset-Printer’s xxx-supported |

| # Printer Description Attributes | # Values allowed for Set |
| --- | --- |
| printer-uri-supported (1setOf uri) | 1setOf Any uri |
| uri-authentication-supported (1setOf type2 keyword) | Any value allowed by Reset-Printer’s xxx-supported |
| uri-security-supported (1setOf type2 keyword) | Any value allowed by Reset-Printer’s xxx-supported |
| printer-name (name(127)) | Any name(127) |
| printer-location (text(127)) | Any text(127) |
| printer-info (text(127)) | Any text(127) |
| printer-more-info (uri) | Any uri |
| printer-driver-installer (uri) | Any uri |
| printer-make-and-model (text(127)) | Any text(127) |
| printer-more-info-manufacturer (uri) | Any uri |
| printer-state (type1 enum) | ## Read-only |
| printer-state-reasons (1setOf type2 keyword) | Read-only |
| printer-state-message (text(MAX)) | Read-only |
| ipp-versions-supported (1setOf type2 keyword) | Read-only |
| operations-supported (1setOf type2 enum) | Any value allowed by Reset-Printer’s xxx-supported |
| multiple-document-jobs-supported (boolean) | Any value allowed by Reset-Printer’s xxx-supported |
| charset-configured (charset) | Any value allowed by Printer’s xxx-supported |
| charset-supported (1setOf charset) | Any value allowed by Reset-Printer’s xxx-supported |
| natural-language-configured (naturalLanguage) | Any value allowed by Printer’s xxx-supported, i.e. xxx-supported is generated-natural-language-supported |
| generated-natural-language-supported (1setOf naturalLanguage) | Any value allowed by Reset-Printer’s xxx-supported |
| document-format-default (mimeMediaType) | Any value allowed by Printer’s xxx-supported |
| document-format-supported (1setOf mimeMediaType) | Any value allowed by Reset-Printer’s xxx-supported |
| printer-is-accepting-jobs (boolean) | ## Read-only |
| queued-job-count (integer(0:MAX)) | ## Read-only |
| printer-message-from-operator (text(127)) | Any text(127) |
| color-supported (boolean) | Any boolean; false and read-only for B&W |
| reference-uri-schemes-supported (1setOf uriScheme) | Any value allowed by Reset-Printer’s xxx-supported |
| pdl-override-supported (type2 keyword) | Any value allowed by Reset-Printer’s xxx-supported |
| printer-up-time (integer(1:MAX)) | Read-only or any positive integer |
| printer-current-time (dateTime) | Any dateTime |
| multiple-operation-time-out (integer(1:MAX)) | any positive integer |
| compression-supported (1setOf type3 keyword) | Any value allowed by Reset-Printer’s xxx-supported |
| job-k-octets-supported (rangeOfInteger(0:MAX)) | Any value allowed by Reset-Printer’s xxx-supported |
| job-impressions-supported (rangeOfInteger(0:MAX)) | Any value allowed by Reset-Printer’s xxx-supported |
| job-media-sheets-supported (rangeOfInteger(0:MAX)) | Any value allowed by Reset-Printer’s xxx-supported |
| pages-per-minute (integer(0:MAX)) | Read-only |
| pages-per-minute-color (integer(0:MAX)) | Read-only |