IPP Test Plan Outline

Table of Contents

1	General	4

1.1	Common restrictions for test purposes	4

1.2	Test print files	4

1.3	Result processing and documentation	4

1.3.1	Anonymity	4

1.3.2	Test data collection method	4

1.3.3	Results determination	5

1.3.4	Output	5

2	Procedure	5

2.1	Send request from Client to Printer	5

2.1.1	Get-printer-attributes	6

2.1.2	Mandatory operations coverage	6

2.1.3	Optional operations coverage	6

2.2	Find out the capabilities of the printer	6

2.2.1	Mandatory printer attributes coverage	6

2.2.2	Optional printer attribute coverage	6

2.3	Print Job interoperability matrix	6

2.4	Print  job validation	6

2.4.1	Simple Job	6

2.4.2	Unsupported attribute job/best effort	7

2.4.3	Unsupported attribute job/fidelity	7

2.5	Print jobs submission	7

2.5.1	Simple Job	7

2.5.2	Unsupported attribute job/best effort	7

2.5.3	Unsupported attribute job/fidelity	7

2.6	List all the jobs	7

2.6.1	Default	7

2.6.2	Requested-attributes: groups	7

2.6.3	Requested-attributes: attributes	8

2.6.4	Which-jobs	8

2.6.5	Limit	8

2.6.6	My-jobs	8

2.7	List specific job	8

2.7.1	Default	8

2.8	get-printer-attributes	8

2.8.1	Default	8

2.8.2	All	8

2.8.3	Printer-description	8

2.8.4	Job-template	9

2.9	Cancel a job	9

2.9.1	Cancel a pending job (owner)	9

2.9.2	Cancel a pending job (not owner)	9

2.9.3	Cancel a completed job	9

2.9.4	Cancel a processing job	9

2.10	Printer State and Status	9

2.10.1	State of the printer	9

2.10.2	Out of paper	9

2.10.3	Cover open	9

2.10.4	Tray missing	10

2.10.5	Pausing a printer	10

2.10.6	Paper jam	10

2.11	Operation status code sampling	10

2.11.1	successful-ok (0x0000)	10

2.11.2	successful-ok-ignored-or-substituted-attributes (0x0001)	10

2.11.3	client-error-bad-request (0x0400)	10

2.11.4	client-error-not-found (0x0406)	10

2.11.5	client-error-request-value-too-long (0x0409)	10

2.11.6	client-error-document-format-not-supported (0x040A)	10

2.11.7	client-error-attributes-or-values-not-supported (0x040B)	10

2.11.8	client-error-charset-not-supported (0x040D)	11

2.11.9	server-error-operation-not-supported (0x0501)	11

2.11.10	server-error-version-not-supported (0x0503)	11

2.12	Printer Contention	11

2.13	Large Jobs	11

3	Optional Testing	11

3.1	Security	11

3.1.1	Basic	11

3.1.2	Digest	11

3.2	Get-printer-attribute/Document-format Test	12

3.3	Chunking Request/Response Test	12

3.4	HTTP version compatibility Test	12

4	Procedure Checklists	12

4.1	IPP Components matrix	12

4.2	Checklists for section 2.1	13

4.2.1	Checklist for section 2.1.1	13

4.2.2	Checklist for section 2.1.2	14

4.2.3	Checklist for section 2.1.3	14

4.3	Checklist for section 2.2	15

4.3.1	Checklist for section 2.2.1	15

4.3.2	Checklist for section 2.2.2	15

4.4	Checklist for section 2.3	16

4.5	Checklist for section 2.4	17

4.6	Checklist for section 2.5	18

4.7	Checklist for section 2.6 and 2.7	19

4.8	Checklist for section 2.9	19

4.9	Checklist for section 2.10	20

4.10	Checklist for section 2.11	20

4.11	Checklist for section 2.12	21

4.12	Checklist for section 2.13	22

4.13	Checklist for section 3.1	22

4.14	Checklist for section 3.2	23

4.15	Checklist for section 3.3	23

4.16	Checklist for section 3.4	24

5	Summary Checklists	24

5.1	Operation Coverage	24

5.2	Operational Attributes Coverage	25

5.3	Attribute Coverage	27

5.3.1	Printer Description	27

5.3.2	Job Template	28

5.3.3	Job Description	29

5.4	Syntax Coverage	29

6	Issues	30

# General

## Common restrictions for test purposes

To facilitate testing and analysis of results we should conduct our tests under certain restrictions.  If time permits specific restrictions may be lifted for specific tests.

- Security.  No authentication, authorization or privacy will be used.  There will be a series of test specifically targeting interoperability of security implementations.  The core protocol testing will eliminate these variables.

- Character set.  We will use the UTF-8 character encoding of ISO646.  This will simplify the comparison of results.

- Language.  We will use en-us.  This will simplify the comparison of results.

- IPP Scheme.  The URLs for printers will be http://* as specified in the June 30 IPP specification.

- Http Chunking.  No IPP component being tested will chunk its requests or responses.  If time permits testing of chunking IPP requests and responses will specifically be performed.

- Set 1 operations.  Any implementations that support the set 1 extensions will use the PWG OP codes.  The PWG OP codes are from 0x0C to 0X12 inclusive.

- HTTP Version 1.1 will be used.  If time permits compatibility tests with 1.1 and 1.0 components will be performed.

## Test print files

Where possible a small standard file will be used.

## Result processing and documentation

### Anonymity

The participants of the tests will be aware of the identity of the participants.  Any posting or discussion of results will not identify the participants demonstrating the issue at hand.  The discussions will focus on issues regarding the IPP specification.  The objective is to clarify the specification’s language to resolve the issue.

### Test data collection method

Developers of IPP test tools should operate at the IPP application layer.  This will simplify the comparison of results.  The format of the traces will be test tool specific.   It is recommended that there be an indication of success or failure.  The tool should also report expected and received results.

Network analyzers are suggested to keep a detailed trace of various tests.  All participants are encouraged to bring their favorite.

### Results determination

Humans can determine the results for many of the tests.  An IPP Client submitting a job that is printed by an IPP printer would seem to indicate interoperability.  We will submit print requests with various attributes.  Subsequently we will list the attributes of the job and insure the expected attributes are present.

Some clients or tools will allow us to examine the attribute groups, tags, attributes and values.  The examination of these may be automated to some extent.

If all else fails the network analyzers will allow us to examine the bits on the wire.

### Output

#### Detailed results

The detailed results of the bake-off will be emailed directly to the participants of the test. Some of the detailed results will be made available during the bake-off.  All participants are encouraged to make their traces available to interested participants.  Implementers of IPP components involved in a test are entitled to the results of the test.

#### Summary of results

A summary of the test results will be circulated among the participants prior to posting on the IPP distribution list.  The results will be sanitized of any vendor specific information.  The object of the summary results is to give a high level overview of the bake-off results.

#### Issues

Issues will be recorded and tracked by the TES whip or designate.

# Procedure

After setting up and establishing basic IP connectivity every IPP component will be assigned a designator.  The IP address and Ethernet address will be recorded in the table in section 4.1.  During each step of the test, each IPP component involved will be recorded.

The tests are performed against IPP components.  In the tables in section 4 columns marked “supported” indicates the IPP component supports that operation, attribute or attribute value.  The columns marked “interoperate” are used to indicate the peer IPP component correctly understands the operation, attribute or attribute value.

## Send request from Client to Printer

For each IPP Printer send a get-printer-attributes request to the Printer to get default response (i.e. no requested attributes or groups).  Any IPP Client or Test Tool may be used.  The objective of this initial test is basic IPP connectivity.  It is preferable to use an independently implemented Client/Test Tool.  Use the results of the IPP operation below.

### Get-printer-attributes

Use the test tool and/or a network analyzer and insure the appropriate operational attributes are returned.  Record results in section 4.2.1

### Mandatory operations coverage

Check off the operations supported in the table in section 4.2.2.

### Optional operations coverage

Check off the attribute supported in the table in section 4.2.3

## Find out the capabilities of the printer

Use the results from 2.1

### Mandatory printer attributes coverage

Check off the attribute supported in the table in section 4.3.1.

### Optional printer attribute coverage

Check off the attributes supported in the table in section 4.3.2

NOTE: The remainder of the testing should be performed using a pair of independent implementations of the IPP protocol.

## Print Job interoperability matrix

Each IPP Client/Test Tool will submit a simple print job to all of the IPP Printers.  The attributes in the request will be the entire mandatory attributes and one optional attribute.   The mandatory attributes are attributes-charset, attributes-natural-language and printer-uri.  The optional attribute will be job-name.  For each IPP Client/Test Tool, the results will be recorded in the table in section 4.4.

## Print  job validation

This will test the mandatory validate-job operation.  Implementations that do not implement validate job can substitute the results from section 2.5 in the table in section 4.5.  A note should be made in the table indicating this substitution has bee made.

### Simple Job

Send a validate-job to the printer.  Record the job attributes that are being sent.  Try various supported attributes.  Record combinations in the table in section 4.5 for comparison with the results in section 2.5.1.

### Unsupported attribute job/best effort

Send a validate-job to the printer.  The attribute-fidelity attribute should be set to false and/or not included.  Record the job attributes that are being sent.  Try various unsupported and supported attribute combinations.  Record combinations in the table in section 4.5 for comparison with the results in section 2.5.2.

### Unsupported attribute job/fidelity

Send a validate-job to the printer.  The attribute-fidelity attribute should be set to true.  Record the job attributes that are being sent.  Try various unsupported and supported attribute combinations.  Record combinations in the table in section 4.5 for comparison with the results in section 2.5.3.

## Print jobs submission

This step tests the print-job operation.  The results of this test can be checked by the examining the response and performing a subsequent operation as in sections 2.6 and 2.7.  List any results that do not match the results from section 2.4 in the table in section 4.6

### Simple Job

Submit some jobs using the print-job operation.  Use the attribute combinations used in section 2.4.1.  If the results do not agree, make a note in section 4.6.

### Unsupported attribute job/best effort

Submit some jobs using the print-job operation.  Use the attribute combinations used in section 2.4.2.  If the results do not agree, make a note in section 4.6.

### Unsupported attribute job/fidelity

Submit some jobs using the print-job operation.  Use the attribute combinations used in section 2.4.3.  If the results do not agree, make a note in section 4.6.

## List all the jobs

This will test the get-jobs operation.  If job information is not retained once a job is completed the printer should be paused.  This may be done with the optional set 1 operation “pause-printer” or by preventing the printer by some other means from processing the job.  This test can be done in combination with the section 2.5 to examine the results.   For each job the results from this test should match the results from section 2.7.

For this section, submit a job(s) with known attributes.

### Default

The list of jobs should only have the 'job-uri' and 'job-id' attributes.  Record results in section 4.7.

### Requested-attributes: groups

The groups to be tested are ‘all’, ‘job-description’ and ‘job-template’.    Record results in section 4.7.

### Requested-attributes: attributes

Pick an attribute that is in every job.  Pick an attribute that is not in every job.  Record results in section 4.7.

### Which-jobs

List ‘completed’ and ‘not-completed’ jobs.  Record results in section 4.7.

### Limit

Send a request with the ‘limit’ set to less than the number of jobs on the printer.  Try again with ‘limit’ greater than the number of jobs on the printer.  Record results in section 4.7.

### My-jobs

Submit a job from two different users.  Each user should then submit a ‘get-jobs’ request.  Record results in section 4.7.

## List specific job

This will test the Get-Job-Attributes operation.  If job information is not retained once a job is completed the printer should be paused.  This may be done with the optional set 1 operation “pause-printer” or by preventing the printer by some other means from processing the job.  This test can be done in combination with the section 2.5 to examine the results.   This test should also be done in combination with section 2.6

For this section, submit a job(s) with known attributes.

### Default

Send get-jobs-attributes and compare with section Error: Reference source not found.  Note that the default behavior of get-jobs-attributes is different from that of get-jobs.  Record results in section 4.7.

## get-printer-attributes

Test the attribute groups in the get-printer-attributes

### Default

Make sure output is the same as the results for the printer in section 2.1 as listed in the tables in section 4.2

### All

Make sure output is the same as 2.8.1

### Printer-description

Make sure all the printer-description attributes listed in section 4.3 for the target printer are reported.  The list of attributes for the target printer may be obtained from performing section 2.2 on the Printer.  The Printer implementers will have these results.  The attribute groupings are listed in section 5.3.

### Job-template

Make sure all the job-template attributes listed in section 4.3 for the target printer are reported.  The list of attributes for the target printer may be obtained from performing section 2.2 on the Printer.  The Printer implementers will have these results.  The attribute groupings are listed in section 5.3.

## Cancel a job

The printers tested must have the ability to prevent jobs from being scheduled.  This may be accomplished by the Set 1 IPP operations, opening a cover or by other printer specific means.   Record the results and behavior in section  4.8

### Cancel a pending job (owner)

### Cancel a pending job (not owner)

### Cancel a completed job

### Cancel a processing job

## Printer State and Status

The objective of this step is to compare the state of printers under various conditions.  The get-printer-attributes operation will be used to obtain the printer-state and printer-state-reasons attributes.   The printer-state-reasons attribute is optional.  Therefore, it may be unsupported in some implementations.

Record the results in section 4.9

### State of the printer

Poll the printer before, during and after a job has been processed. The printer-state should be ‘idle’, ‘processing’ and ‘idle’ respectively.  The printer-state of ‘paused’ will be tested in section 2.10.5.

### Out of paper

Submit a job of a known size to a printer with the input tray(s) empty.  The printer-state should be stopped and the printer-state-reasons should be 'media-empty'.

Put some paper into the input tray.  There should not be enough to complete the job.  While the printer is printing, examine the printer-state and printer-state-reasons.  The printer-state should be processing and the printer-state-reasons may contain media-low.

When the printer runs out of paper, examine the two attributes again.  The printer-state should be stopped.  The printer-state-reasons may contain media-needed

### Cover open

Open the appropriate covers and doors.  The printer-state should move to ‘stopped’.  The printer-state-reasons may contain 'cover-open’ and/or 'door-open'

### Tray missing

Remove an input tray and record the printer-state and printer-state-reasons.  The printer-state-reasons may contain 'input-tray-missing'.

### Pausing a printer

If possible, pause the printer.  The printer-state should be stopped.  The printer-state-reasons may contain paused.

### Paper jam

If possible, cause the printer to jam.  The printer-state should be stopped.  The printer-state-reasons may contain media-jam.

## Operation status code sampling

Below are some statuses codes that will be tested.  The list is not all-inclusive.  It is intended to touch on some of the important conditions.

For each of these scenarios record the status-code in section 4.10.  The status-code-message should also be recorded.

### successful-ok (0x0000)

This code should be returned in any of the successful operations performed so far.

### successful-ok-ignored-or-substituted-attributes (0x0001)

This code should have been returned in the test in section 2.4.2 and  section 2.5.2.

### client-error-bad-request (0x0400)

Submit a request that is syntactically incorrect.

### client-error-not-found (0x0406)

Submit a get-jobs-attribute request for a job that does not exist.

### client-error-request-value-too-long (0x0409)

Submit a request with an attribute value whose length exceeds the specified maximum.

### client-error-document-format-not-supported (0x040A)

Submit a request specifying a document format not listed in the printer attribute document-format-supported.

### client-error-attributes-or-values-not-supported (0x040B)

Submit a get-printer-attribute request with a requested attribute that is not supported by the printer.  The attribute may be an attribute not defined in the IPP model specification.  The test should be repeated for an invalid attribute value for a supported attribute.

### client-error-charset-not-supported (0x040D)

Submit a request where the value of the operational attribute attributes-charset is set to an unsupported value.  The value may be something other than the valid values from the IPP specification.

### server-error-operation-not-supported (0x0501)

Submit a request with a value in the operation field for a nonexistent operation.

### server-error-version-not-supported (0x0503)

Submit a request with an invalid value for the IPP version.

## Printer Contention

Have multiple clients submit print jobs to a single printer.  Record the status codes, status code message from the responses.  Describe the behavior of the client/printer interactions.   Record in section 4.11

Compare the results with other printer contention tests.

## Large Jobs

Have a clients submit a large print jobs to several printers.  Watch the data flow using a network analyzer.  Record the status codes, status code message from the responses.  Describe the behavior of the client/printer interactions.   Record in section 4.12

Compare the results with other printer contention tests.

# Optional Testing

If time permits some additional tests may be performed.

## Security

Security that is supported by the HTTP layer will be tested for interoperability.  The IPP printers participating will all turn on the security mentioned below.  The results will be summarized and recorded in section.

### Basic

Turn on basic authentication at the printer.  Have the client submit an request.  Record the participating components, the operation and the results.

### Digest

Turn on digest authentication at the printer.  Have the client submit an request.  Record the participating components, the operation and the results.

## Get-printer-attribute/Document-format Test

Any printer that supports the behavior of returning different printer attributes based on the document-format attribute will be tested.  The test will be similar to section 2.2 except the procedure will be repeated for each document-format-supported.  Record the results in section 4.14

## Chunking Request/Response Test

Select Clients and Printers that support chunking.  Try operations that will cause the client to chunk a request and record the results.  Try operations that will cause the Printer to chunk a response and record the results.  Record the results in section 4.15.

## HTTP version compatibility Test

Select Clients and Printers that use different versions of HTTP.  Record the results of their interactions.  Record the results in section 4.16

# Procedure Checklists

## IPP Components matrix

| Section 2 (Procedure) |  |  |  |
| --- | --- | --- | --- |
| ID | IP Address | Ethernet Address | URL |
| Clients |  |  |  |
| CA |  |  |  |
| CB |  |  |  |
| CC |  |  |  |
| CD |  |  |  |
| CE |  |  |  |
| CF |  |  |  |
| CG |  |  |  |
| Printers |  |  |  |
| PA |  |  |  |
| PB |  |  |  |
| PC |  |  |  |
| PD |  |  |  |
| PE |  |  |  |
| PF |  |  |  |
| PG |  |  |  |
| PH |  |  |  |
| PI |  |  |  |
| PJ |  |  |  |
| PK |  |  |  |
| PL |  |  |  |
| PM |  |  |  |
| PN |  |  |  |
| PM |  |  |  |
| Test Suites |  |  |  |
| TS A |  |  |  |
| TS B |  |  |  |
| TS C |  |  |  |

## Checklists for section 2.1

### Checklist for section 2.1.1

| Operational attributes |  |  |  |  |
| --- | --- | --- | --- | --- |
| Printer ID |  |  |  |  |
| Client(s) ID |  |  |  |  |
| Test Tool(s) ID |  |  |  |  |
| Attr ID | Operational Attribute | Group | Supported | Interoperate |
| OA05 | version-number | preamble |  |  |
| OA06 | operation-id | preamble |  |  |
| OA07 | request-id | preamble |  |  |
| OA08 | attributes-charset | operational-attribute |  |  |
| OA09 | attributes-natural-language | operational-attribute |  |  |
| OA10 | printer-uri | operational-attribute |  |  |
| OA14 | requesting-user-name | operational-attribute |  |  |
| OA17 | status code | preamble |  |  |
| OA18 | Status-message | operational-attribute |  |  |

### Checklist for section 2.1.2

| Mandatory Operations |  |  |  |
| --- | --- | --- | --- |
| Printer ID |  |  |  |
| Client(s) ID |  |  |  |
| Test Tool(s) ID |  |  |  |
| Attr ID | Operation |  | Supported |
| OC01 | print-job |  |  |
| OC04 | validate-job |  |  |
| OC05 | get-printer-attributes |  |  |
| OC06 | get-jobs |  |  |
| OC07 | get-job-attributes |  |  |
| OC08 | cancel-job |  |  |

### Checklist for section 2.1.3

| Optional Operations |  |  |  |
| --- | --- | --- | --- |
| Printer ID |  |  |  |
| Client(s) ID |  |  |  |
| Test Tool(s) ID |  |  |  |
| Attr ID | Operation |  | Supported |
| OC09 | print-uri |  |  |
| OC10 | create-job |  |  |
| OC11 | send-document |  |  |
| OC12 | send-uri |  |  |
| OC13 | hold-job |  |  |
| OC14 | release-job |  |  |
| OC15 | pause-printer |  |  |
| OC16 | resume-printer |  |  |
| OC17 | purge-printer |  |  |
| OC18 | reprint-job |  |  |

## Checklist for section 2.2

### Checklist for section 2.2.1

| Mandatory Printer Attributes |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ID | Attribute |  | Supported |  |  |
| PD01 | printer-uri-supported |  |  |  |  |
| PD01 | uri-security-supported |  |  |  |  |
| PD03 | printer-name |  |  |  |  |
| PD04 | printer-state |  |  |  |  |
| PD05 | operations-supported |  |  |  |  |
| PD06 | charset-configured |  |  |  |  |
| PD07 | charset-supported |  |  |  |  |
| PD08 | natural-language-configured |  |  |  |  |
| PD09 | generated-natural-language-supported |  |  |  |  |
| PD10 | printer-is-accepting-jobs |  |  |  |  |
| PD11 | pdl-override-supported |  |  |  |  |
| PD21 |  | document-format-default |  |  |  |
| PD22 |  | document-format-supported |  |  |  |
| PD12 | printer-up-time |  |  |  |  |

### Checklist for section 2.2.2

| Optional Printer Attributes |  |  |
| --- | --- | --- |
| Attr ID | Attribute | Supported |
| PD13 | printer-location |  |
| PD14 | printer-info |  |
| PD15 | printer-more-info |  |
| PD16 | printer-driver-installer |  |
| PD17 | printer-make-and-model |  |
| PD18 | printer-more-info-manufacturer |  |
| PD19 | printer-state-reasons |  |
| PD20 | printer-state-message |  |
| PD23 | queued-job-count |  |
| PD24 | printer-message-from-operator |  |
| PD25 | color-supported |  |
| PD26 | reference-uri-schemes-supported |  |
| PD27 | printer-current-time |  |
| PD28 | multiple-operation-time-out |  |
| PD29 | compression-supported |  |
| PD30 | job-k-octets-supported |  |
| PD31 | job-impressions-supported |  |
| PD32 | job-media-sheets-supported |  |
| JT02 | job-priority-default |  |
| JT03 | job-priority-supported |  |
| JT05 | job-hold-until-default |  |
| JT06 | job-hold-until-supported |  |
| JT08 | job-sheets-default |  |
| JT09 | job-sheets-supported |  |
| JT11 | multiple-document-handling-default |  |
| JT12 | multiple-document-handling-supported |  |
| JT14 | copies-default |  |
| JT15 | copies-supported |  |
| JT17 | finishings-default |  |
| JT18 | finishings-supported |  |
| JT20 | page-ranges-supported |  |
| JT22 | sides-default |  |
| JT23 | sides-supported |  |
| JT25 | number-up-default |  |
| JT26 | number-up-supported |  |
| JT28 | orientation-requested-default |  |
| JT29 | orientation-requested-supported |  |
| JT31 | media-default |  |
| JT32 | media-supported |  |
| JT33 | media-ready |  |
| JT35 | printer-resolution-default |  |
| JT36 | printer-resolution-supported |  |
| JT38 | print-quality-default |  |
| JT39 | print-quality-supported |  |

## Checklist for section 2.3

Print Job interoperability matrix

| Client/Test Tool ID | Successful Printer ID (s) | Unsuccessful Printer ID (s) |
| --- | --- | --- |

## Checklist for section 2.4

Print  job validation

| Test # | Client/ Test Tool ID | Printer ID | Status Code | Requested Attributes | Accepted Attributes | Rejected Attributes |
| --- | --- | --- | --- | --- | --- | --- |
| A |  |  |  |  |  |  |
| B |  |  |  |  |  |  |
| C |  |  |  |  |  |  |
| D |  |  |  |  |  |  |
| E |  |  |  |  |  |  |
| F |  |  |  |  |  |  |
| G |  |  |  |  |  |  |
| H |  |  |  |  |  |  |
| I |  |  |  |  |  |  |
| J |  |  |  |  |  |  |
| K |  |  |  |  |  |  |
| L |  |  |  |  |  |  |
| M |  |  |  |  |  |  |
| N |  |  |  |  |  |  |
| O |  |  |  |  |  |  |

## Checklist for section 2.5

Print jobs submission

| Test # | Description of difference |
| --- | --- |

## Checklist for section 2.6 and 2.7

List all the jobs and List specific job

| Operation and description | results |
| --- | --- |
| List-jobs default |  |
| List-jobs requested-attributes = all |  |
| List-jobs requested-attributes = job-description |  |
| List-jobs requested-attributes = job-template |  |
| List-jobs attribute in every job |  |
| List-jobs attribute not in every job |  |
| List-jobs which-jobs = completed’ |  |
| List-jobs which-jobs = not-completed’ |  |
| List-jobs limit is less than the number of printer jobs |  |
| List-jobs limit is greater than the number of printer jobs |  |
| List-jobs my-jobs |  |

## Checklist for section 2.9

Cancel a job

| Scenerio | status code | status-message | Describe behavior |
| --- | --- | --- | --- |
| Cancel a pending job (owner) |  |  |  |
| Cancel a pending job (not owner) |  |  |  |
| Cancel a completed job |  |  |  |
| Cancel a processing job |  |  |  |

## Checklist for section 2.10

Printer State and Status

| Scenario | Printer-state | Printer-state-reasons | Describe behavior |
| --- | --- | --- | --- |
| State of the printer before job |  |  |  |
| State of the printer during job |  |  |  |
| State of the printer after job |  |  |  |
| Out of paper |  |  |  |
| Cover open |  |  |  |
| Tray missing |  |  |  |
| Pausing a printer |  |  |  |
| Paper jam |  |  |  |

## Checklist for section 2.11

Operation status code sampling

| status code | status-message | Operation | Description of conditions/behavior |
| --- | --- | --- | --- |
| successful-ok (0x0000) |  |  |  |
| successful-ok-ignored-or-substituted-attributes (0x0001) |  |  |  |
| client-error-bad-request (0x0400) |  |  |  |
| client-error-not-found (0x0406) |  |  |  |
| client-error-request-value-too-long (0x0409) |  |  |  |
| client-error-document-format-not-supported (0x040A) |  |  |  |
| client-error-attributes-or-values-not-supported (0x040B) |  |  |  |
| client-error-charset-not-supported (0x040D) |  |  |  |
| server-error-operation-not-supported (0x0501) |  |  |  |
| server-error-version-not-supported (0x0503) |  |  |  |

## Checklist for section 2.12

Printer Contention

| status code | status-message | Description of behavior |
| --- | --- | --- |

## Checklist for section 2.13

Large Jobs

| status code | status-message | Description of behavior |
| --- | --- | --- |

## Checklist for section 3.1

Security

| Client ID | Printer ID | Security | Description of results |
| --- | --- | --- | --- |

## Checklist for section 3.2

Get-printer-attribute/Document-format Test

| 1st document-format | 2nd document-format | Description difference |
| --- | --- | --- |

## Checklist for section 3.3

Chunking Request/Response Test

| Component IDs | Ids of components that support chunking | Describe results |
| --- | --- | --- |

## Checklist for section 3.4

HTTP version compatibility Test

| Client ID & HTTP Version | Printer ID & HTTP Version | Describe results |
| --- | --- | --- |

# Summary Checklists

The purpose of the set of tables in this section is to assign a shorthand ID to all operations, attributes and syntaxes .  These tables may also be used during and after the test to document the IPP Model coverage.

## Operation Coverage

| ID | Operation | Mandatory | Tested |
| --- | --- | --- | --- |
| OC01 | print-job | Yes |  |
| OC04 | validate-job | Yes |  |
| OC05 | get-printer-attributes | Yes |  |
| OC06 | get-jobs | Yes |  |
| OC07 | get-job-attributes | Yes |  |
| OC08 | cancel-job | Yes |  |
| OC09 | print-uri | No |  |
| OC10 | create-job | No |  |
| OC11 | send-document | No |  |
| OC12 | send-uri | No |  |
| OC13 | hold-job | No |  |
| OC14 | release-job | No |  |
| OC15 | pause-printer | No |  |
| OC16 | resume-printer | No |  |
| OC17 | purge-printer | No |  |
| OC18 | restart-job | No |  |

## Operational Attributes Coverage

| Operational Attribute Coverage |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ID | Operational Attribute | Group | Comment | Mandatory | Tested |
| OA01 |  | job-attribute | Tested by get-jobs | Yes |  |
| OA02 |  | printer-attribute | Tested by get-printer-attributes | Yes |  |
| OA03 |  | unsupported-attributes | Tested by print-job with unsupported attributes | Yes |  |
| OA04 |  | operational-attribute | See OA08 to OAZZ |  |  |
| OA05 | version-number | preamble | Test with any operation | Yes |  |
| OA06 | operation-id | preamble | See section 5.1 | Yes |  |
| OA07 | request-id | preamble | Test with any operation | Yes |  |
| OA08 | attributes-charset | operational-attribute | Must support utf-8 | Yes |  |
| OA09 | attributes-natural-language | operational-attribute | Test imposes en-us requirement | Yes |  |
| OA10 | printer-uri | operational-attribute | This or OA11 must be 3rd attribute | Conditional |  |
| OA11 | job-uri | operational-attribute | This or OA10 must be 3rd attribute | Conditional |  |
| OA12 | job-id | operational-attribute | This must be 4th attribute | Conditional |  |
| OA13 | job-name | operational-attribute | Only for print & validate operations | Optional |  |
| OA14 | requesting-user-name | operational-attribute |  | Optional |  |
| OA15 | document-uri | operational-attribute | Only for print-uri | Conditional |  |
| OA16 | last-document | operational-attribute | Only for send-uri and send-document | Conditional |  |
| OA17 | status code | preamble | Test with any response | Yes |  |
| OA18 | status-message | operational-attribute | Test with any response | Optional |  |
| OA19 | compression | operational-attribute | Only for print & validate operations | Optional |  |
| OA20 | document-natural-language | operational-attribute | Only for print & validate operations | Optional |  |
| OA21 | ipp-attribute-fidelity | operational-attribute | Only for print & validate operations | Optional |  |
| OA22 | job-impressions | operational-attribute | Only for print & validate operations | Optional |  |
| OA23 | job-k-octets | operational-attribute | Only for print & validate operations | Optional |  |
| OA24 | job-media-sheets | operational-attribute | Only for print & validate operations | Optional |  |
| OA25 | limit | operational-attribute | Only for get-jobs operations | Optional |  |
| OA26 | message | operational-attribute | Only for cancel operations | Optional |  |
| OA27 | my-jobs | operational-attribute | Only for get-jobs operations | Optional |  |
| OA28 | requested-attributes | operational-attribute | Only for get-* operations | Optional |  |
| OA29 | Document-format | operational-attribute | Only for print operations | Optional |  |
| OA30 | which-jobs | operational-attribute | Only for get-jobs operations | Optional |  |

## Attribute Coverage

### Printer Description

| Printer Description |  |  |  |  |
| --- | --- | --- | --- | --- |
| ID | Attribute | Syntax | Mandatory | Tested |
| PD01 | printer-uri-supported | uri | Yes |  |
| PD01 | uri-security-supported | 1setOf type2 keyword | Yes |  |
| PD03 | printer-name | name | Yes |  |
| PD04 | printer-state | type1 enum | Yes |  |
| PD05 | operations-supported | 1setOf type2 enum | Yes |  |
| PD06 | charset-configured | charset | Yes |  |
| PD07 | charset-supported | 1setOf charset | Yes |  |
| PD08 | natural-language-configured | naturalLanguage | Yes |  |
| PD09 | generated-natural-language-supported | 1setOf naturalLanguage | Yes |  |
| PD10 | printer-is-accepting-jobs | boolean | Yes |  |
| PD11 | pdl-override-supported | type2 keyword | Yes |  |
| PD12 | printer-up-time | integer | Yes |  |
| PD13 | printer-location | text | No |  |
| PD14 | printer-info | text | No |  |
| PD15 | printer-more-info | uri | No |  |
| PD16 | printer-driver-installer | uri | No |  |
| PD17 | printer-make-and-model | text | No |  |
| PD18 | printer-more-info-manufacturer | uri | No |  |
| PD19 | printer-state-reasons | 1setOf type2 keyword | No |  |
| PD20 | printer-state-message | text | No |  |
| PD21 | document-format-default | mimeMediaType | No |  |
| PD22 | document-format-supported | 1setOf mimeMediaType | No |  |
| PD23 | queued-job-count | integer | No |  |
| PD24 | printer-message-from-operator | text | No |  |
| PD25 | color-supported | boolean | No |  |
| PD26 | reference-uri-schemes-supported | 1setOf uriScheme | No |  |
| PD27 | printer-current-time | dateTime | No |  |
| PD28 | multiple-operation-time-out | integer | No |  |
| PD29 | compression-supported | 1setOf type3 keyword | No |  |
| PD30 | job-k-octets-supported | rangeOfInteger | No |  |
| PD31 | job-impressions-supported | rangeOfInteger | No |  |
| PD32 | job-media-sheets-supported | rangeOfInteger | No |  |

### Job Template

| Job Template |  |  |  |  |
| --- | --- | --- | --- | --- |
| ID | Attribute | Syntax | Mandatory | Tested |
| JT01 | job-priority | integer | No |  |
| JT02 | job-priority-default | integer | No |  |
| JT03 | job-priority-supported | integer | No |  |
| JT04 | job-hold-until | type4 keyword ¦ name | No |  |
| JT05 | job-hold-until-default | type4 keyword ¦ name | No |  |
| JT06 | job-hold-until-supported | 1setOf type4 keyword ¦ name | No |  |
| JT07 | job-sheets | type4 keyword ¦ name | No |  |
| JT08 | job-sheets-default | type4 keyword ¦ name | No |  |
| JT09 | job-sheets-supported | 1setOf type4 keyword ¦ name | No |  |
| JT10 | multiple-document-handling | type2 keyword | No |  |
| JT11 | multiple-document-handling-default | type2 keyword | No |  |
| JT12 | multiple-document-handling-supported | 1setOf type2 keyword | No |  |
| JT13 | copies | integer | No |  |
| JT14 | copies-default | integer | No |  |
| JT15 | copies-supported | integer | No |  |
| JT16 | finishings | 1setOf type2 enum | No |  |
| JT17 | finishings-default | 1setOf type2 enum | No |  |
| JT18 | finishings-supported | 1setOf type2 enum | No |  |
| JT19 | page-ranges | 1setOf rangeOfInteger | No |  |
| JT20 | page-ranges-supported | boolean | No |  |
| JT21 | sides | type2 keyword | No |  |
| JT22 | sides-default | type2 keyword | No |  |
| JT23 | sides-supported | 1setOf type2 keyword | No |  |
| JT24 | number-up | integer | No |  |
| JT25 | number-up-default | integer | No |  |
| JT26 | number-up-supported | 1setOf integer ¦ rangeOfInteger | No |  |
| JT27 | orientation-requested | type2 | No |  |
| JT28 | orientation-requested-default | type2 | No |  |
| JT29 | orientation-requested-supported | 1setOf type2 | No |  |
| JT30 | media | type4 keyword ¦ name | No |  |
| JT31 | media-default | type4 keyword ¦ name | No |  |
| JT32 | media-supported | 1setOf type4 keyword ¦ name | No |  |
| JT33 | media-ready | 1setOf type4 keyword ¦ name | No |  |
| JT34 | printer-resolution | resolution | No |  |
| JT35 | printer-resolution-default | resolution | No |  |
| JT36 | printer-resolution-supported | 1setOf resolution | No |  |
| JT37 | print-quality | type2 enum | No |  |
| JT38 | print-quality-default | type2 enum | No |  |
| JT39 | print-quality-supported | 1setOf type2 enum | No |  |

### Job Description

| Job Description |  |  |  |  |
| --- | --- | --- | --- | --- |
| ID | Attribute | Syntax | Mandatory | Tested |
| JD01 | job-uri | uri | Yes |  |
| JD02 | job-id | integer | Yes |  |
| JD03 | job-printer-uri | uri | Yes |  |
| JD04 | job-name | name | Yes |  |
| JD05 | job-originating-user-name | name | Yes |  |
| JD06 | job-state | type1 enum | Yes |  |
| JD07 | attributes-charset | charset | Yes |  |
| JD08 | attributes-natural-language | naturalLanguage | Yes |  |
| JD09 | job-more-info | uri | No |  |
| JD10 | job-state-reasons | 1setOf type2 keyword | No |  |
| JD11 | job-state-message | text | No |  |
| JD12 | number-of-documents | integer | No |  |
| JD13 | output-device-assigned | name | No |  |
| JD14 | time-at-creation | integer | No |  |
| JD15 | time-at-processing | integer | No |  |
| JD16 | time-at-completed | integer | No |  |
| JD17 | number-of-intervening-jobs | integer | No |  |
| JD18 | job-message-from-operator | text | No |  |
| JD19 | job-k-octets | integer | No |  |
| JD20 | job-impressions | integer | No |  |
| JD21 | job-media-sheets | integer | No |  |
| JD22 | job-k-octets-processed | integer | No |  |
| JD23 | job-impressions-completed | integer | No |  |
| JD24 | job-media-sheets-completed | integer | No |  |

## Syntax Coverage

| Syntax Coverage |  |  |  |  |
| --- | --- | --- | --- | --- |
| ID | Syntax | Attribute | Mandatory | Tested |
| SC01 | text | printer-location | No |  |
| SC02 | textWithLanguage |  |  |  |
| SC03 | name | printer-name | Yes |  |
| SC04 | nameWithLanguage |  |  |  |
| SC05 | keyword | pdl-override-supported | Yes |  |
| SC06 | enum | printer-state | Yes |  |
| SC07 | uri | printer-uri-supported | Yes |  |
| SC08 | UriScheme (1 setOf) | reference-uri-schemes-supported | No |  |
| SC09 | charset | charset-configured | Yes |  |
| SC10 | naturalLanguage | natural-language-configured | Yes |  |
| SC11 | mimeMediaType | document-format-default | No |  |
| SC12 | octetString |  |  |  |
| SC13 | boolean | printer-is-accepting-jobs | Yes |  |
| SC14 | integer | printer-up-time | Yes |  |
| SC15 | rangeOfInteger | job-k-octets-supported | No |  |
| SC16 | dateTime | printer-current-time | No |  |
| SC17 | resolution | printer-resolution | No |  |
| SC18 | 1setOf X (1setOf type2 enum) | operations-supported | Yes |  |

# Issues

Use this area to record any issues, comments or concerns.  The items should relate to the IPP specification or interoperability testing.

- There is no on-line/off-line.

- Requested attribute groups are not well explained in MOD document