PWG-DRAFT  - 13 ISSUES are numbered and highlighted like this	Carl Kugler

ipp-ops-admin-990630.txt	IBM Corporation

T. Hastings

Xerox Corporation

June 30, 1999

Internet Printing Protocol/1.0 & 1.1: Additional Administrative Operations

Status of this Memo

This document is a PWG-Draft.  It is intended for registration following the registration procedures of IPP/1.0 [RFC2566] and IPP/1.1 [ipp-mod].  This version includes the comments discussed at the IPP telecon, on 6/23/1999 and 6/30/1999.

Abstract

This document specifies 15 additional OPTIONAL administrative operations for use with the Internet Printing Protocol/1.0 (IPP) [RFC2565, RFC2566] and IPP/1.1 [ipp-mod, ipp-pro].  These operations are 7 Printer object operations that operators/administrators may perform on a Printer object:

Set-Printer-Attributes

Enable-Printer

Disable-Printer

Reset-Printer

Restart-Printer

Space-Printer

Shutdown-Printer

and 8 Job object operations that end-users may perform on their jobs and operators/administrators may perform on any job:

Set-Job-Attributes

Reprocess-Job

Cancel-Current-Job  (though the target is the Printer object)

Pause-Job

Pause-Current-Job  (though the target is the Printer object)

Resume-Job

Promote-Job

Space-Current-Job  (though the target is the Printer object)

In addition, two operation attributes are defined:  "job-message-from-operator" and "printer-message-from-operator" are included to set the corresponding Job and Printer Description attributes with the same names.  And the "when" operation attribute is added to the Pause-Printer operation.

Finally, a new 'client-error-attributes-not-settable' is defined for use with the two Set operations.

The full set of IPP documents includes:

Design Goals for an Internet Printing Protocol [RFC2567]

Rationale for the Structure and Model and Protocol for the Internet Printing Protocol [RFC2568]

Internet Printing Protocol/1.1: Model and Semantics (this document)

Internet Printing Protocol/1.1: Encoding and Transport [IPP-PRO]

Internet Printing Protocol/1.1: Implementer's Guide [IPP-IIG]

Mapping between LPD and IPP Protocols [RFC2569]

The "Design Goals for an Internet Printing Protocol" document takes a broad look at distributed printing functionality, and it enumerates real-life scenarios that help to clarify the features that need to be included in a printing protocol for the Internet.  It identifies requirements for three types of users: end users, operators, and administrators.  It calls out a subset of end user requirements that are satisfied in IPP/1.0.  A few OPTIONAL operator operations have been added to IPP/1.1.

The "Rationale for the Structure and Model and Protocol for the Internet Printing Protocol" document describes IPP from a high level view, defines a roadmap for the various documents that form the suite of IPP specification documents, and gives background and rationale for the IETF working group's major decisions.

The "Internet Printing Protocol/1.1: Encoding and Transport" document is a formal mapping of the abstract operations and attributes defined in the model document onto HTTP/1.1 [RFC2616].  It defines the encoding rules for a new Internet MIME media type called "application/ipp".  This document also defines the rules for transporting over HTTP a message body whose Content-Type is "application/ipp".  This document defines a new scheme named 'ipp' for identifying IPP printers and jobs.

The "Internet Printing Protocol/1.1: Implementer's Guide" document gives insight and advice to implementers of IPP clients and IPP objects.  It is intended to help them understand IPP/1.1 and some of the considerations that may assist them in the design of their client and/or IPP object implementations.  For example, a typical order of processing requests is given, including error checking.  Motivation for some of the specification decisions is also included.

The "Mapping between LPD and IPP Protocols" document gives some advice to implementers of gateways between IPP and LPD (Line Printer Daemon) implementations.

Table of Contents

1.	Introduction	4

2.	New Operation attributes	4

2.1	New operation attribute:  "printer-message-from-operator" (text(127))	4

2.2	New operation attribute:  "job-message-from-operator" (text(127))	5

2.3	New operation attribute for Pause-Printer:  "when" (type2 keyword)	6

3.	New Attributes	7

3.1	New Printer Description attribute:  "printer-message-date-time" (dateTime)	7

3.2	New Printer Description attribute:  "printer-message-operation" (type2 keyword)	7

4.	Additional values for "printer-state-reasons" and "job-state-reasons" attributes	8

4.1	Value for "printer-state-reasons":  'standby'	8

4.2	Value for "job-state-reasons":  'job-paused'	8

4.3	Value for "job-state-reasons":  'process-next'	8

5.	New status codes	9

5.1	New status code: 'client-error-attributes-not-settable'	9

5.2	New status code: 'server-error-printer-is-in-standby-mode'	9

6.	Summary of Administrative operations and Operation-Id Assignments	9

6.1	Printer Operations	10

6.1.1	Set-Printer-Attributes Operation	10

6.1.2	Disable-Printer Operation	15

6.1.3	Enable-Printer Operation	16

6.1.4	Reset-Printer operation	16

6.1.5	Restart-Printer operation	18

6.1.6	Space-Printer operation	18

6.1.7	Shutdown-Printer Operation	19

6.2	Job Operations	21

6.2.1	Set-Job-Attributes	21

6.2.2	Reprocess-Job Operation	25

6.2.3	Cancel-Current-Job Operation	25

6.2.4	Pause-Job operation	27

6.2.5	Pause-Current-Job operation	28

6.2.6	Resume-Job operation	29

6.2.7	Promote-Job operation	30

6.2.8	Space-Current-Job operation	30

# Introduction

The Internet Printing Protocol (IPP) is an application level protocol that can be used for distributed printing using Internet tools and technologies.  IPP version 1.1 (IPP/1.1) focuses only on end user functionality with a few administrative operations included.  This document defines additional OPTIONAL end user and administrative operations used to control Jobs and Printers. This document is a registration proposal for an extension to IPP/1.0 and IPP/1.1 following the registration procedures in those documents.

# New Operation attributes

This section defines the new "printer-message-from-operation" and "job-message-from-operator" operation attributes that set the corresponding Printer and Job Description attributes.

## New operation attribute:  "printer-message-from-operator" (text(127))

Type of registration:  attribute

Proposed keyword name of this attribute:  "printer-message-from-operator"

Types of attribute (Operation, Job Template, Job Description, Printer Description):  Operation

Operations to be used with if the attribute is an operation attribute:  See below

Object (Job, Printer, etc. if bound to an object):   Printer (already in IPP/1.0 and IPP/1.1)

Attribute syntax(es) (include 1setOf and range as in Section 4.2):  text(127)

Specification of this attribute (follow the style of IPP Model Section 4.2):

"printer-message-from-operator" (text(127))

The client OPTIONALLY supplies this attribute.  The Printer object SHOULD supports this operation attribute if it supports the corresponding Printer Description attribute.  The value of this attribute is a message from the operator about the Printer object on which the operator has just performed an operation.  If supported, the Printer copies the value to the Printer's "printer-message-from-operator" Printer Description attribute (see [ipp-mod] section 4.4.25), automatically sets the value of the Printer's "printer-message-date-time to the current date and time, and automatically sets the value of the Printer's "printer-message-operation" to the operation-id value of this operation.  If the client omits this attribute, the Printer does not change the value of its "printer-message-from-operator", "printer-message-date-time", and "printer-message-operation" Printer Description attributes.

This operation attribute is defined for use with the following operator operations on the Printer object:

Pause-Printer - see [ipp-mod] section 3.2.7

Resume-Printer - see [ipp-mod] section 3.2.8

Purge-Jobs - see [ipp-mod] section 3.2.9

Disable-Printer - see section 6.1.2

Enable-Printer - see section 6.1.3

Reset-Printer - see section 6.1.4

Restart-Printer - see section 6.1.5

Space-Printer - see section 6.1.6

Shutdown-Printer - see section 6.1.7

The "printer-message-from-operator" operation attribute MUST NOT be supported as an operation attribute for the Set-Printer-Attributes operation.  If the operator wants to set the Printer's "printer-message-from-operator" Printer Description attribute when issuing the Set-Printer-Attributes operation, the client supplies the "printer-message-from-operator" with its new value as one of the Printer Description attributes in Group 2 in the request.  Otherwise the Printer leaves the value of the Printer's "printer-message-from-operator" Printer Description attribute unchanged by not explicitly setting the attribute.

## New operation attribute:  "job-message-from-operator" (text(127))

Type of registration:  attribute

Proposed keyword name of this attribute:  "job-message-from-operator"

Types of attribute (Operation, Job Template, Job Description, Printer Description):  Operation

Operations to be used with if the attribute is an operation attribute:  See below

Object (Job, Printer, etc. if bound to an object):   Job (already in IPP/1.0 and IPP/1.1)

Attribute syntax(es) (include 1setOf and range as in Section 4.2):  text(127)

Specification of this attribute (follow the style of IPP Model Section 4.2):

"job-message-from-operator" (text(127))

The client OPTIONALLY supplies this attribute.  The Printer object SHOULD supports this operation attribute if it supports the corresponding Job Description attribute.  The value of this attribute is a message from the operator about the Job object on which the operator has just performed an operation.  If supported, the Printer copies the value to the Job's "job-message-from-operator" Job Description attribute (see [ipp-mod] section 4.3.16).  If the client omits this attribute, the Printer does not change the value of its "printer-message-from-operator" Job Description attribute.

Note:  There are no corresponding "job-message-date-time" and "job-message-operation" Job Description attributes, since the usual lifetime of a job is limited.

This operation attribute is defined for use with the following operator operations on the Job object:

Cancel-Job - see [ipp-mod] section 3.2.4

Hold-Job - see [ipp-mod] section 3.3.5

Release-Job - see [ipp-mod] section 3.3.6

Restart-Job - see [ipp-mod] section 3.3.7

Reprocess-Job - see section 6.2.2

Cancel-Current-Job - see section 6.2.3

Pause-Job - see section 6.2.4

Pause-Current-Job - see section 6.2.5

Resume-Job - see section 6.2.6

Promote-Job - see section 6.2.7

Space-Current-Job - see section 6.2.8

The "job-message-from-operator" operation attribute MUST NOT be supported as an operation attribute for the Set-Job-Attributes operation.  If the operator wants to set the Job's "job-message-from-operator" Job Description attribute when issuing the Set-Job-Attributes operation, the client supplies the "job-message-from-operator" with its new value as one of the Job Description attributes in Group 2 in the request.  Otherwise the Printer leaves the value of the Job's "job-message-from-operator" Job Description attribute unchanged by not explicitly setting the attribute.

## New operation attribute for Pause-Printer:  "when" (type2 keyword)

Type of registration:  attribute

Proposed keyword name of this attribute:  "when"

Types of attribute (Operation, Job Template, Job Description, Printer Description):  Operation

Operations to be used with if the attribute is an operation attribute:  Pause-Printer, Reset-Printer, Shutdown-Printer

Object (Job, Printer, etc. if bound to an object):   Job (already in IPP/1.0 and IPP/1.1)

Attribute syntax(es) (include 1setOf and range as in Section 4.2):  type2 keyword

Specification of this attribute (follow the style of IPP Model Section 4.2):

"when" (type2 keyword)

The client OPTIONALLY supplies this attribute.  The Printer object MUST support this attribute, if it supports this operation.  The value of this attribute indicates when to shutdown the printer.  If the client omits this attribute, the Printer assumes the 'after-current-job' value.

Standard keyword values are:

'now' - cancel the currently printing job(s) and shut down the Printer.  Jobs in the 'held' and 'pending' state remain in those states.

'after-current-copy' - shutdown the Printer after the current job finishes printing its current copy.  Jobs in the 'held' and 'pending' state remain in those states.

'after-current-job' - shutdown the Printer after the current job finishes printing (all its copies).  Jobs in the 'held' and 'pending' state remain in those states.

'after-all' - shutdown the Printer after all 'pending' jobs finish printing.  Jobs in the 'held' state remain in the 'held' state.

ISSUE 1:  Which values of the "when" operation attribute are required to be supported?

# New Attributes

The following new attributes are needed to support the new operations defined in this document.

## New Printer Description attribute:  "printer-message-date-time" (dateTime)

Type of registration:  attribute

Proposed keyword name of this attribute:  printer-message-date-time

Types of attribute (Operation, Job Template, Job Description, Printer Description):

Operations to be used with if the attribute is an operation attribute:  N/A

Object (Job, Printer, etc. if bound to an object):  Printer

Attribute syntax(es) (include 1setOf and range as in Section 4.2):  dateTime

If attribute syntax is 'keyword' or 'enum', is it type2 or type3:  N/A

If this is a Printer attribute, MAY the value returned depend on "document-format" (See Section 6.2):  No

If this is a Job Template attribute, how does its specification depend on the value of the "multiple-document-handling" attribute:  N/A

Specification of this attribute (follow the style of IPP Model Section 4.2):

4.2.? printer-message-date-time (dateTime)

The date and time that the Printer's "printer-message-from-operator" was changed by the operator using any operation where the client supplied the "printer-message-from-operator" operation attribute (see section 2.1) or was explicitly set using the Set-Printer-Attributes operation (see section 6.1.1).  This attribute allows the users to know when the "printer-message-from-operator" attribute was last set.

Note:  This attribute helps users better understand the context for the "printer-message-from-operator" message.

## New Printer Description attribute:  "printer-message-operation" (type2 keyword)

Type of registration:  attribute

Proposed keyword name of this attribute:  printer-message-operation

Types of attribute (Operation, Job Template, Job Description, Printer Description):

Operations to be used with if the attribute is an operation attribute:  N/A

Object (Job, Printer, etc. if bound to an object):  Printer

Attribute syntax(es) (include 1setOf and range as in Section 4.2):  type2 enum

If attribute syntax is 'keyword' or 'enum', is it type2 or type3:  type2

If this is a Printer attribute, MAY the value returned depend on "document-format" (See Section 6.2):  No

If this is a Job Template attribute, how does its specification depend on the value of the "multiple-document-handling" attribute:  N/A

Specification of this attribute (follow the style of IPP Model Section 4.2):

4.2.? printer-message-attribute (type2 enum)

The operation that was used to changed the Printer's "printer-message-from-operator" by the operator using any operation where the client supplied the "printer-message-from-operator" operation attribute (see section 2.1) or explicitly set using the Set-Printer-Attributes operation (see section 6.1.1).  This attribute allows the users to know which operation was used to change the "printer-message-from-operator" attribute when it was last set.

Note:  This attribute helps users better understand the context for the "printer-message-from-operator" message.

# Additional values for "printer-state-reasons" and "job-state-reasons" attributes

The following values are added to the "printer-state-reasons" and "job-state-reasons" for use with the operations defined in this document.

## Value for "printer-state-reasons":  'standby'

Type of registration:  type2 keyword attribute value

Name of attribute to which this keyword specification is to be added:  printer-state-reasons

Proposed keyword name of this keyword value:  standby

Specification of this keyword value (follow the style of IPP Model Section 4.1.2.3):

'standby':  The Printer has been shutdown in standby mode.  Only Restart-Printer and Get-Printer-Attributes operations are accepted in this state;  all other operations are rejected with the 'server-error-printer-is-in-standby-mode'.

## Value for "job-state-reasons":  'job-paused'

Type of registration:  type2 keyword attribute value

Name of attribute to which this keyword specification is to be added:  job-state-reasons

Proposed keyword name of this keyword value:  job-paused

Specification of this keyword value (follow the style of IPP Model Section 4.1.2.3):

'job-paused':  The job has been paused while processing using the Pause-Job or Pause-Current-Job operations and other jobs can be processed on the Printer.  The Job can be resumed using the Resume-Job operation which removes this value.

## Value for "job-state-reasons":  'process-next'

Type of registration:  type2 keyword attribute value

Name of attribute to which this keyword specification is to be added:  job-state-reasons

Proposed keyword name of this keyword value:  process-next

Specification of this keyword value (follow the style of IPP Model Section 4.1.2.3):

'process-next':  The job has been promoted to be the next job processed using the Promote-Job operation (see section 6.2.7).

# New status codes

This section defines new status codes used by the operations defined in this document.

## New status code: 'client-error-attributes-not-settable'

Type of registration:  status code

Keyword symbolic name of this status code value:  'client-error-attributes-not-settable'

Numeric value (to be assigned by the IPP Designated Expert in consultation with IANA):

Operations that this status code may be used with:  Set-Printer-Attributes, Set-Job-Attributes

Specification of this status code (follow the style of IPP Model Section 13 APPENDIX B:  Status Codes and Suggested Status Code Messages):

#### 13.1.4.20 client-error-attributes-not-settable (0x0413)

In a Set-Printer-Attributes or Set-Job-Attributes request, if the Printer object does not support one or more attributes as settable, and the client supplied the "ipp-attributes-fidelity" operation attribute with the 'true' value, the Printer object MUST return this status code.  The Printer object MUST also return in the Unsupported Attributes Group all the attributes and/or values supplied by the client that are not settable.  See [ipp-mod] section 3.1.7.  For example, if the request indicates 'job-state', all implementations MUST reject the request.  As another example, if the request indicates an attribute that is supported, but not settable by this implementation, such as, say, "printer-name", the implementation rejects the request.  If the "ipp-attribute-fidelity" attribute is 'false', the Printer MUST ignore attributes that are not settable, rather than reject the request and return this status code.

## New status code: 'server-error-printer-is-in-standby-mode'

Type of registration:  status code

Keyword symbolic name of this status code value:  'server-error-printer-is-in-standby-mode'

Numeric value (to be assigned by the IPP Designated Expert in consultation with IANA):

Operations that this status code may be used with:  Shutdown-Printer

Specification of this status code (follow the style of IPP Model Section 13 APPENDIX B:  Status Codes and Suggested Status Code Messages):

13.1.5.8 server-error-printer-is-in-standby-mode

The Printer has been shutdown and is only accepting the Restart-Printer (see section 6.1.5) and Get-Printer-Attributes operations.  An operator can perform the Restart-Printer operation to allow the Printer to accept other operations.

# Summary of Administrative operations and Operation-Id Assignments

The Administrative operations are summarized in the following table:

| Operation Name | Operation-Id | Brief description |
| --- | --- | --- |
| Set-Printer-Attributes | 0x?? | Sets attribute values of the target Printer object |
| Enable-Printer | 0x?? | Allows the target Printer to accept create job operations |
| Disable-Printer | 0x?? | Prevents the target Printer from accepting create job operations |
| Reset-Printer | 0x?? | Resets the target Printer to one of several indicated ways |
| Restart-Printer | 0x?? | Restarts the target Printer from a standby shutdown mode |
| Space-Printer | 0x?? | Skips or repeats the specified number of impressions for the target job |
| Shutdown-Printer | 0x?? | Shuts down the target Printer in several ways |
| Set-Job-Attributes | 0x?? | Sets attribute values of the target Job object |
| Reprocess-Job | 0x?? | Creates a copy of a completed target job with a new Job ID and processes it |
| Cancel-Current-Job | 0x?? | Cancels the current job on the target Printer |
| Pause-Job | 0x?? | Pauses the indicated processing job, allowing other jobs to be processed instead |
| Pause-Current-Job | 0x?? | Pauses the current processing job on the target Printer |
| Resume-Job | 0x?? | Resume the paused target job |
| Promote-Job | 0x?? | Promote the pending target job to be next after the current job(s) complete |
| Space-Current-Job | 0x?? | Skips or repeats some number of impressions for the current job on the target Printer |

All of the operations in this registration proposal specification are OPTIONAL for an IPP object to support.  Unless the specification of an OPTIONAL operation requires support of another OPTIONAL operation, conforming implementations may support any combination of these operations.

## Printer Operations

All Printer operations are directed at Printer objects.  A client MUST always supply the "printer-uri" operation attribute in order to identify the correct target of the operation.  These descriptions assume all of the common semantics of IPP/1.1 Model and Semantics document [ipp-mod] section 3.1.

### Set-Printer-Attributes Operation

Type of registration:  operation

Proposed name of this operation:  Set-Printer-Attributes

Object Target:  Printer

Specification of this operation:

This OPTIONAL operation allows a client to set the values of the attributes of a Printer object.   In the request, the client supplies the set of Printer attribute names and values that are to be set.  In the response, the Printer object returns success or rejects the request with indications of which attribute or attributes could not be set.

How the Printer object validates the client-supplied attributes in the Set-Printer-Attributes request is implementation-dependent, since there are no corresponding Printer attributes that specify the allowed values that may be set on the Printer object.

The Printer MUST accept this operation in any state, i.e., for any of the values of the Printer object's "printer-state" attribute.

For Printer attributes that are supported, some MUST be settable, some SHOULD be settable, some MAY be settable, and some MUST NOT be settable as indicated in the following table:

| IPP Printer Attributes | Settable? |
| --- | --- |
| all Job Template Default attributes ("xxx-default" [ipp-mod] section 4.2 and extensions) | SHOULD if attribute supported |
| all Job Template Supported attributes ("xxx-supported" [ipp-mod] section 4.2 and extensions) | SHOULD if attribute supported |
| all Job Template Ready attributes ("xxx-ready" [ipp-mod] section 4.2 and extensions) | SHOULD if attribute supported |
| printer-uri-supported | SHOULD |
| uri-security-supported | SHOULD |
| uri-authentication-supported | SHOULD |
| printer-name | MAY |
| printer-location | SHOULD if attribute supported |
| printer-info | SHOULD if attribute supported |
| printer-driver-installer | SHOULD if attribute supported |
| printer-make-and-model | MAY |
| printer-more-info-manufacturer | MAY |
| printer-state | MUST NOT |
| printer-state-reasons | MUST NOT |
| printer-state-message | MUST NOT if attribute supported |
| ipp-versions-supported | MAY |
| operations-supported | SHOULD |
| ipp-multiple-document-jobs-supported | MUST NOT if attribute supported |
| charset-configured | SHOULD |
| charset-supported | MAY |
| natural-language-configured | SHOULD |
| generated-natural-language-supported | MAY |
| document-format-default | SHOULD |
| document-format-supported | SHOULD |
| printer-is-accepting-jobs | MUST NOT - see Enable-Printer/Disable-Printer |
| queued-job-count | MUST NOT |
| printer-message-from-operator | MUST if attribute supported Note: There is no "printer-message-from-operator" operation attribute for the Set-Printer-Attributes operation. |
| printer-message-date-time | MUST NOT if attribute supported |
| printer-message-operation | MUST NOT if attribute supported |
| color-supported | MAY if attribute supported |
| reference-uri-schemes-supported | MAY if attribute supported |
| pdl-override-supported | MUST NOT |
| printer-up-time | MUST NOT |
| printer-current-time | MAY if attribute supported |
| multiple-operation-time-out | SHOULD if attribute supported |
| compression-supported | MAY |
| job-k-octets-supported | SHOULD if attribute supported |
| job-impressions-supported | SHOULD if attribute supported |
| job-media-sheets-supported | SHOULD if attribute supported |
| pages-per-minute | MAY if attribute supported |
| pages-per-minute-color | MAY if attribute supported |

Access Rights: The authenticated user (see [ipp-mod] section 8.3) performing this operation must be an operator or administrator of the Printer object (see [ipp-mod] sections 1 and 8.5).  Otherwise, the IPP Printer MUST reject the operation and return:  'client-error-forbidden', 'client-error-not-authenticated', or 'client-error-not-authorized' as appropriate.

Most Printer attributes will require administrator privileges to set, such as "xxx-supported", while some will require operator privileges only, such as "media-ready" and "printer-message-from-operator".  Which attributes require which privileges depends on implementation and MAY depend on site policy.

#### Set-Printer-Attributes Request

The following sets of attributes are part of the Set-Printer-Attributes Request:

Group 1: Operation Attributes

Natural Language and Character Set:

The "attributes-charset" and "attributes-natural-language" attributes as described in [ipp-mod] section 3.1.4.1.

Target:

The "printer-uri" (uri) operation attribute which is the target for this operation as described in [ipp-mod] section 3.1.5.

Requesting User Name:

The "requesting-user-name" (name(MAX)) attribute SHOULD be supplied by the client as described in [ipp-mod] section 8.3.

"ipp-attribute-fidelity" (boolean):

The client OPTIONALLY supplies this attribute.  The Printer object MUST support this attribute.  This operation attribute is used to specify whether or not the sets are to be atomic.  If 'false', or not present, Printer sets what it can and returns the unset attribute/value pairs in the Unsupported Attributes group.  If 'true', either all supplied attributes MUST be set or the operation MUST be rejected.  All Printer objects MUST support both types of setting.

"document-format" (mimeMediaType):

The client SHOULD supply this attribute.  The Printer object MUST support this attribute.  This attribute is useful for a client to select the interpreter to which the attribute modification should be applied.  See [ipp-mod] section 3.2.5.1 "Get-Printer-Attributes Request".

If the Printer object does not distinguish between different sets of supported values for each different document format when validating jobs in the create and Validate-Job operations, it MUST NOT distinguish between different document formats in the Set-Printer-Attributes operation.  If the Printer object does distinguish between different sets of supported values for each different document format specified by the client, this specialization applies only to the same Printer attributes as the Get-Printer-Attributes operation (see [ipp-mod] section 3.2.5.1).

If the client omits this "document-format" operation attribute, the Printer object MUST respond as if the attribute had been supplied with the value of the Printer object's "document-format-default" attribute.  It is recommended that the client always supply a value for "document-format", since the Printer object's "document-format-default" may be 'application/octet-stream', in which case the set attributes and values are for the union of the document formats that the Printer can automatically sense.  For more details, see the description of the 'mimeMediaType' attribute syntax in [ipp-mod] section 4.1.9.

If the client supplies a value for the "document-format" Operation attribute that is not supported by the Printer, i.e., is not among the values of the Printer object's "document-format-supported" attribute, the Printer object MUST reject the operation and return the 'client-error-document-format-not-supported' status code.

Group 2: Printer Attributes

The client MUST supply a set of Printer attributes as defined in [ipp-mod] section 4.2 Job Template Attributes ("xxx-default", "xxx-supported", and "xxx-ready" attributes) and section 4.4 Printer Description Attributes.  Each Printer attribute supplied in Group 2 replaces the value(s) of the corresponding Printer attribute on the target Printer object.  If a Printer object attribute had not been configured yet and so had the 'no-value' out-of-band value (see [ipp-mod] 4.1), the supplied value(s) replace the 'no-value' value.  For attributes that can have multiple values (1setOf), all values supplied by the client replace all values of the corresponding Printer object attribute.

#### Set-Printer-Attributes Response

The Printer object returns the following sets of attributes as part of the Get-Printer-Attributes Response:

Group 1: Operation Attributes

Status Message:

In addition to the REQUIRED status code returned in every response, the response OPTIONALLY includes a "status-message" (text(255)) and/or a "detailed-status-message" (text(MAX)) operation attribute as described in [ipp-mod] sections 13 and 3.1.6.

Natural Language and Character Set:

The "attributes-charset" and "attributes-natural-language" attributes as described in [ipp-mod] section 3.1.4.2.

Group 2: Unsupported Attributes

See [ipp-mod] section 3.1.7 for details on returning Unsupported Attributes.

In the case of attributes that are supported, but are not settable by the implementation (see table above), the Printer object returns the client-supplied attribute(s) with a substituted value of 'not-settable'.  This value's syntax type is "out-of-band" and its encoding is defined by special rules for "out-of-band" values in the "Encoding and Transport" document [IPP-PRO].  Its value indicates that the attribute is not settable.  Group 2 is returned independent of the value for the "ipp-attribute-fidelity" attribute supplied by the client.

Disable-Printer Operation

Type of registration:  operation

Proposed name of this operation:  Disable-Printer

Object Target:  Printer

Specification of this operation:

This OPTIONAL operation allows a client to stop the Printer object from accepting jobs, i.e., cause the Printer to reject subsequent create job operations (Print-Job, Print-URI, and Create-Job operation) and return the 'server-error-not-accepting-jobs' status code.  The Printer still accepts all other operations.  All previously submitted Jobs and currently processing Jobs continue unaffected.  The Printer sets the value of its "printer-is-accepting-jobs" read-only Printer Description attribute to 'false' (see [ipp-mod] section 4.4.20), no matter what the previous value was.  This operation has no immediate effect on the Printer's "printer-state" and "printer-state-reasons" attributes.

Note:  Use the Enable-Printer operation (section 6.1.3) to enable a Printer to accept Jobs again.

If the Disable-Printer operation is supported, then the Enable-Printer operation MUST be supported, and vice-versa.

Note:  Use the Enable-Printer and Disable-Printer operations to allow or prevent input to a Printer.  Use the Pause-Printer and Resume-Printer operations to prevent or allow output from the Printer.

Whether or not the Disable-Printer operation stops jobs that are submitted using job submission protocols other than IPP, depends on implementation, i.e., on whether the IPP protocol is being used as a universal management protocol or just to manage IPP jobs, respectively.

Access Rights: The authenticated user (see [ipp-mod] section 8.3) performing this operation must be an operator or administrator of the Printer object (see [ipp-mod] Sections 1 and 8.5).  Otherwise, the IPP Printer MUST reject the operation and return:  'client-error-forbidden', 'client-error-not-authenticated', or 'client-error-not-authorized' as appropriate.

The Disable-Printer Request and Disable-Printer Response have the same attribute groups and attributes as the Pause-Printer operation (see [ipp-mod] sections 3.2.7.1 and 3.2.7.2), including the new "printer-message-from-operator" operation attribute (see section 2.1).

### Enable-Printer Operation

Type of registration:  operation

Proposed name of this operation:  Enable-Printer

Object Target:  Printer

Specification of this operation:

This OPTIONAL operation allows a client to start the Printer object accepting jobs, i.e., cause the Printer to accept subsequent create job operations (Print-Job, Print-URI, and Create-Job operation).  The Printer still accepts all other operations.  All previously submitted Jobs and currently processing Jobs continue unaffected.  The Printer sets the value of its "printer-is-accepting-jobs" read-only Printer Description attribute to 'true' (see [ipp-mod] section 4.4.20), no matter what the previous value was.  This operation has no immediate effect on the Printer's "printer-state" and "printer-state-reasons" attributes.

Note:  Use the Disable-Printer operation (section 6.1.2) to stop a Printer from accepting Jobs.

If the Enable-Printer operation is supported, then the Disable-Printer operation MUST be supported, and vice-versa.

Note:  Use the Enable-Printer and Disable-Printer operations to allow or prevent input to a Printer.  Use the Pause-Printer and Resume-Printer operations to prevent or allow output from the Printer.

Whether or not the Enable-Printer operation allows acceptance of  jobs that are submitted using job submission protocols other than IPP, depends on implementation, i.e., on whether the IPP protocol is being used as a universal management protocol or just to manage IPP jobs, respectively.

Access Rights: The authenticated user (see [ipp-mod] section 8.3) performing this operation must be an operator or administrator of the Printer object (see [ipp-mod] Sections 1 and 8.5).  Otherwise, the IPP Printer MUST reject the operation and return:  'client-error-forbidden', 'client-error-not-authenticated', or 'client-error-not-authorized' as appropriate.

The Enable-Printer Request and Enable-Printer Response have the same attribute groups and attributes as the Pause-Printer operation (see [ipp-mod] sections 3.2.7.1 and 3.2.7.2), including the new "printer-message-from-operator" operation attribute (see section 2.1).

### Reset-Printer operation

Type of registration:  operation

Proposed name of this operation:  Reset-Printer

Object Target:  Printer

Specification of this operation:

This OPTIONAL operation allows a client to reset the Printer in a number of ways, depending on the "reset-function" operation attribute.  The keyword values of this attribute map one-to-one to the enum values that the NMS writes into the prtGeneralReset object in the Printer MIB [RFC1759] to affect a reset operation.  As in the Printer MIB, the 'reset-to-nvram' (soft reset) value MUST be supported, if this operation is supported.  The other values are OPTIONAL.

As is says in the Printer MIB specification, if a device does not have NVRAM (non-volatile RAM), the device MUST none-the-less respond to this operation for the 'reset-to-nvram' value with some sort of warm reset that resets the device to some implementation-defined state that is preferably under control of the system administrator by some means outside the scope of the Printer MIB and this document.

The effect of this operation on the currently processing job(s), if any, is not specified by this document.  Note:  If this operation does affect the current job(s), it is expected that the operator would issue this operation on a Printer in the 'idle' state after disabling the Printer with the Disable operation in order to prevent a job from inadvertently being affected by this operation.

The Printer object MUST accept this operation in any state and transition the Printer object to the 'idle' state.

Access Rights: The authenticated user (see [ipp-mod] section 8.3) performing this operation must be an operator or administrator of the Printer object (see [ipp-mod] sections 1 and 8.5).  Otherwise, the IPP object MUST reject the operation and return: client-error-forbidden, client-error-not-authenticated, and client-error-not-authorized as appropriate.

The Reset-Printer Request and Reset-Printer Response have the same attribute groups and attributes as the Pause-Printer operation (see [ipp-mod] sections 3.2.7.1 and 3.2.7.2), including the new "printer-message-from-operator" operation attribute (see section 2.1), the new "when" operation attribute (see section 2.3), and with the addition of the following Group 1 operation attributes in the request:

"reset-function" (type3 keyword)

The client OPTIONALLY supplies this attribute.  The Printer object MUST support this attribute, if it supports this operation.  The value of this attribute indicates the reset function to be performed.  If the client omits this attribute, the Printer assumes the 'reset-to-nvram' value.

Standard keyword values are:

'power-cycle-reset' - Cold start, i.e., to the state when the device is powered up.

'reset-to-nvram' - Warm start.

'reset-to-factory-defaults' - reset NVRAM to factory defaults.

non-process-run-out" (boolean):

The client OPTIONALLY supplies this operation attribute.  The IPP object OPTIONALLY supports this operation attribute, if it is able to perform a non-process-run-out, i.e., remove the last few printed sheets of a job that is finished printing on a continuous-forms printer.  Normally, on such a printer, the last sheets of a job remain in the printer and are forced out by the next job, thereby saving time when printing one job after another.  This attribute indicates whether or not the printer is to perform "non-process run-out", i.e., move the last printed sheet to the stacker, before resetting the Printer.

If the client does not supply this attribute and the attribute is supported, the Printer does not perform non-process run-out ('false' behavior).  If the Printer does not support this attribute, the Printer is assumed to perform non-process run-out ('true' behavior).  ISSUE 2:  Is the "non-process-run-out" operation attribute really needed at all or can the default behavior for Reset-Printer be defined to be to perform non-process run out (for continuous and cut sheet printers)?

### Restart-Printer operation

Type of registration:  operation

Proposed name of this operation:  Restart-Printer

Object Target:  Printer

Specification of this operation:

This OPTIONAL operation allows a client to restart a Printer that has previously been shutdown in standby mode (see section 6.1.7).  Standby mode is indicated by the Printer's "printer-state" being 'idle' and its "printer-state-reasons" including the 'standby' value.  If the Printer is not in standby mode, the Printer MUST reject this operation and return the 'client-error-not-possible' status code.

Access Rights: The authenticated user (see [ipp-mod] section 8.3) performing this operation must be an operator or administrator of the Printer object (see [ipp-mod] sections 1 and 8.5).  Otherwise, the IPP object MUST reject the operation and return: client-error-forbidden, client-error-not-authenticated, and client-error-not-authorized as appropriate.

The Restart-Printer Request and Restart-Printer Response have the same attribute groups and attributes as the Pause-Printer operation (see [ipp-mod] sections 3.2.7.1 and 3.2.7.2), including the new "printer-message-from-operator" operation attribute (see section 2.1) and the following Group 1 operation attribute:

"non-process-run-out" (boolean):

The client OPTIONALLY supplies this operation attribute.  The IPP object OPTIONALLY supports this operation attribute, if it is able to perform a non-process-run-out, i.e., remove the last few printed sheets of a job that is finished printing on a continuous-forms printer.  Normally, on such a printer, the last sheets of a job remain in the printer and are forced out by the next job, thereby saving time when printing one job after another.  This attribute indicates whether or not the printer is to perform "non-process run-out", i.e., move the last printed sheet to the stacker, as part of restarting the Printer.

If the client does not supply this attribute and the attribute is supported, the Printer does not perform non-process run-out ('false' behavior).  If the Printer does not support this attribute, the Printer is assumed to perform non-process run-out ('true' behavior).  ISSUE 3:  Is the "non-process-run-out" operation attribute really needed at all or can the default behavior for Restart-Printer be defined to be to perform non-process run out (for continuous and cut sheet printers)?

### Space-Printer operation

Type of registration:  operation

Proposed name of this operation:  Space-Printer

Object Target:  Printer

Specification of this operation:

This OPTIONAL operation allows a client to repeat or skip a specified number of impressions on the target Printer.  The Printer repeats or skips the indicated number of impressions specified by the "back-space" or "forward-space" operation attribute, respectively.  This operation is typically supported in a continuous forms implementation for synchronizing the web after forms run out or media change.

The Printer MUST accept this request in any state and whether or not there is a current job.

Access Rights: The authenticated user (see [ipp-mod] section 8.3) performing this operation must be an operator or administrator of the Printer object (see [ipp-mod] sections 1 and 8.5).  Otherwise, the IPP object MUST reject the operation and return: client-error-forbidden, client-error-not-authenticated, and client-error-not-authorized as appropriate.

The Reset-Printer Request and Reset-Printer Response have the same attribute groups and attributes as the Pause-Printer operation (see [ipp-mod] sections 3.2.7.1 and 3.2.7.2), including the new "printer-message-from-operator" operation attribute (see section 2.1), and with the addition of the following Group 1 operation attributes in the request:

"back-space" (integer(1:MAX)):

The client OPTIONALLY supplies this Operation attribute.  The IPP object OPTIONALLY supports this operation attribute, if it is able to repeat impressions.

If the client supplies a value that specifies more impressions than the job has performed, the job is positioned at the beginning without any indication.

"forward-space" (integer(1:MAX)):

The client OPTIONALLY supplies this Operation attribute.  The IPP object OPTIONALLY supports this operation attribute, if it is able to skip impressions.

If the client supplies a value that specifies more impressions than remain in the job, the job is positioned at the end without any indication.

"non-process-run-out" (boolean):

The client OPTIONALLY supplies this operation attribute.  The IPP object OPTIONALLY supports this operation attribute, if it is able to perform a non-process-run-out, i.e., remove the last few printed sheets of a job that is finished printing on a continuous-forms printer.  Normally, on such a printer, the last sheets of a job remain in the printer and are forced out by the next job, thereby saving time when printing one job after another.  This attribute indicates whether or not the printer is to perform "non-process run-out", i.e., move the last printed sheet to the stacker, after spacing the Printer.

If the client does not supply this attribute and the attribute is supported, the Printer does not perform non-process run-out ('false' behavior).  If the Printer does not support this attribute, the Printer is assumed to perform non-process run-out ('true' behavior).  ISSUE 4:  Is the "non-process-run-out" operation attribute really needed at all or can the default behavior for Space-Printer be defined to be to perform non-process run out (for continuous and cut sheet printers)?

### Shutdown-Printer Operation

Type of registration:  operation

Proposed name of this operation:  Shutdown-Printer

Object Target:  Printer

Specification of this operation:

This OPTIONAL operation allows a client to shut down a Printer, i.e., stop processing jobs.  Whether the device is powered down or put into standby mode, depends on the value of the "shutdown-function" operation attribute.  If the value is 'power-off', or the "shutdown-function" attribute is omitted, the Printer adds the 'shutdown' value (see [ipp-mod] section 4.4.11) to its "printer-state-reasons" Printer Description attribute immediately.  If the value is 'standby', the Printer adds the 'standby' value to its  "printer-state-reasons" Printer Description attribute immediately.  The execution of the rest of the operation is the same for either value:  The Printer is disabled immediately (see the Disable-Printer operation in section 6.1.2).  The "when" operation attribute specifies how much processing occurs before the shutdown takes place.  All other requests continue to be accepted until the printer is powered down or enters standby mode.  In standby mode, the Printer MUST continue to accept the Restart-Printer and Get-Printer-Attribute requests.

Whether or not the Shutdown-Printer operation affects jobs that were submitted to the device using job submission protocols other than IPP, depends on implementation, i.e., on whether the IPP protocol is being used as a universal management protocol or just to manage IPP jobs, respectively.

The Printer object MUST accept this operation in any state and transition the Printer object to the 'idle' state.

Access Rights: The authenticated user (see [ipp-mod] section 8.3) performing this operation must be an operator or administrator of the Printer object (see [ipp-mod] sections 1 and 8.5).  Otherwise, the IPP object MUST reject the operation and return: client-error-forbidden, client-error-not-authenticated, and client-error-not-authorized as appropriate.

The Shutdown-Printer Request and Shutdown-Printer Response have the same attribute groups and attributes as the Pause-Printer operation (see [ipp-mod] sections 3.2.7.1 and 3.2.7.2), including the new "printer-message-from-operator" operation attribute (see section 2.1), the new "when" operation attribute (see section 2.3), and with the addition of the following Group 1 operation attributes in the request:

"non-process-run-out" (boolean)

The client OPTIONALLY supplies this attribute.  The Printer object OPTIONALLY supports this attribute, if it is able to perform a non-process-run-out, i.e., remove the last few printed sheets of a job that is finished printing on a continuous-forms printer.  Normally, on such a printer, the last sheets of a job remain in the printer and are forced out by the next job, thereby saving time when printing one job after another.  This attribute indicates whether or not the printer is to perform "non-process run-out", i.e., move the last printed sheet to the stacker, before shutting down.

If the client does not supply this attribute and the attribute is supported, the Printer does not perform non-process run-out ('false' behavior).  If the Printer does not support this attribute, the Printer is assumed to perform non-process run-out ('true' behavior).

ISSUE 5:  Is the "non-process-run-out" operation attribute really needed at all or can the default behavior for Shutdown-Printer be defined to be to perform non-process run out (for continuous and cut sheet printers)?

"synchronize" (boolean)

The client OPTIONALLY supplies this attribute.  The Printer object OPTIONALLY supports this attribute.  This attribute indicates whether or not the printer is to synchronize the checkpoint data for the current job ("when" = 'now') with the pages that have actually printed.  If the value of the "when" attribute is not 'now' or the "when" attribute is not supplied, then the "synchronize" attribute has no meaning and the Printer MUST ignore it.  If this attribute is supported, then a value of 'true' implies that the Printer will be able to resume the job at the point of synchronization when the Printer is restarted.

If the client does not supply this attribute and the attribute is supported, the Printer does not perform synchronization ('false' behavior).  If the Printer does not support this attribute, the Printer is assumed to not synchronize ('false' behavior).

ISSUE 6:  Do we really need the "synchronize" operation attribute or can synchronization be the default behavior of the Shutdown-Printer operation?

ISSUE 7:  Is the current job automatically restarted when the Printer is restarted?  Or does some client have to issue a Restart-Job operation?

## Job Operations

All Job operations are directed at Job objects.  A client MUST always supply some means of identifying the Job object in order to identify the correct target of the operation.  That job identification MAY either be a single Job URI or a combination of a Printer URI with a Job ID.  The IPP object implementation MUST support both forms of identification for every job.

### Set-Job-Attributes

Type of registration:  operation

Proposed name of this operation:  Set-Job-Attributes

Object Target:  Job

Specification of this operation:

This OPTIONAL operation allows a client to set the values of the attributes of a Job object.   In the request, the client supplies the set of Job attribute names and values that are to be set.  In the response, the IPP object returns success or rejects the request with indications of which attribute or attributes could not be set.

This operation is almost identical to the Set-Printer-Attributes operation (see section 6.1.1).  The only differences are that the Set-Job-Attributes operation is directed at a Job object rather than a Printer object, there is no "document-format" operation attribute used when setting a Job object, and the validation is the same as the create job operations, i.e., depends on the "xxx-supported" Printer Description attributes.

The validation of the Set-Job-Attributes request is performed as if the job had been submitted originally with the new values.  If such a create job operation would have been accepted, then the Set-Job-Attributes MUST be accepted.  If such a create job operation would have been rejected, then the Set-Job-Attributes MUST be rejected and the Job MUST be unchanged.

The IPP object MUST accept or reject the request based on the job's current state and transition the job to the indicated new state as follows:

| Current "job-state" | New "job-state" | IPP object's response status code and action: |
| --- | --- | --- |
| 'pending' | 'pending' | 'successful-ok' |
| 'pending' | 'pending-held' | 'successful-ok' - needed resources are not ready |
| 'pending-held' | 'pending-held' | 'successful-ok' |
| 'pending-held' | 'pending' | 'successful-ok' - needed resources are ready |
| 'processing' | 'processing' | 'successful-ok' or 'client-error-not-possible' depending on the attributes being set, whether the job has started marking media, and/or implementation |
| 'processing-stopped' | 'processing-stopped' | 'successful-ok' or 'client-error-not-possible' depending on the attributes being set, whether the job has started marking media, and/or implementation |
| 'completed' | 'completed' | 'client-error-not-possible' |
| 'canceled' | 'canceled' | 'client-error-not-possible' |
| 'aborted' | 'aborted' | 'client-error-not-possible' |

For Job attributes that are supported, some MUST be settable, some SHOULD be settable, some MAY be settable, and some MUST NOT be settable as indicated in the following table:

| IPP Job Attributes | Settable? |
| --- | --- |
| all Job Template Job attributes ("xxx" [ipp-mod] section 4.2 and extensions) | SHOULD if attribute supported |
| job-uri | MUST NOT |
| job-id | MUST NOT |
| job-printer-uri | MUST NOT |
| job-more-info | MUST NOT if attribute supported |
| job-name | MUST - also set in create operation |
| job-originating-user-name | MUST NOT - set in create operation |
| job-state | MUST NOT |
| job-state-reasons | MUST NOT |
| job-state-message | MUST NOT if attribute supported |
| number-of-documents | MUST NOT |
| output-device-assigned | MUST NOT |
| time-at-creation | MUST NOT |
| time-at-processing | MUST NOT |
| time-at-completed | MUST NOT |
| job-printer-up-time | MUST NOT |
| date-time-at-creation | MUST NOT if attribute supported |
| date-time-at-processing | MUST NOT if attribute supported |
| date-time-at-completed | MUST NOT if attribute supported |
| number-of-intervening-jobs | MUST NOT if attribute supported |
| job-message-from-operator | MUST if attribute supported Note: There is no "job-message-from-operator" operation attribute for the Set-Job-Attributes operation. |
| job-k-octets | MUST NOT if attribute supported |
| job-impressions | MAY if attribute supported - see Rule 1 |
| job-media-sheets | MAY if attribute supported - see Rule 1 |
| job-k-octets-processed | MUST NOT if attribute supported |
| job-impressions-completed | MUST NOT if attribute supported |
| job-media-sheets-completed | MUST NOT if attribute supported |
| attributes-charset | MUST NOT - set in create operation |
| attributes-natural-language | MUST NOT - set in create operation |

Rule 1 - However, an implementation MAY refuse to allow this Job Description attribute to be set, if it has determined a more reliable value from the document content.

Whether or not the Set-Job-Attributes operation affects jobs that are submitted using job submission protocols other than IPP, depends on implementation, i.e., on whether the IPP protocol is being used as a universal management protocol or just to manage IPP jobs, respectively.

Access Rights: The authenticated user (see [ipp-mod] section 8.3) performing this operation must either be the job owner or an operator or administrator of the Printer object (see [ipp-mod] sections 1 and 8.5).  Otherwise, the IPP object MUST reject the operation and return: 'client-error-forbidden', 'client-error-not-authenticated', or 'client-error-not-authorized' as appropriate.

#### Set-Job-Attributes Request

The following sets of attributes are part of the Set-Job-Attributes Request:

Group 1: Operation Attributes

Natural Language and Character Set:

The "attributes-charset" and "attributes-natural-language" attributes as described in [ipp-mod] section 3.1.4.1.

Target:

Either (1) the "printer-uri" (uri) plus "job-id" (integer(1:MAX)) or (2) the "job-uri" (uri) operation attribute(s) which define the target for this operation as described in [ipp-mod] section 3.1.5.

Requesting User Name:

The "requesting-user-name" (name(MAX)) attribute SHOULD be supplied by the client as described in [ipp-mod] section 8.3.

"ipp-attribute-fidelity" (boolean):

The client OPTIONALLY supplies this attribute.  The Printer object MUST support this attribute if it supports this operation.  This operation attribute is used to specify whether or not the sets are to be atomic.  If 'false', or not present, the Printer sets what it can and returns the unset attribute/value pairs in the Unsupported Attributes group.  If 'true', either all supplied attributes MUST be set or the operation MUST be rejected.  All Printer objects MUST support both types of setting.  See [ipp-mod] section 15 for a full description of "ipp-attribute-fidelity" and its relationship to other attributes, especially the Printer object's "pdl-override-supported" attribute.

Group 2: Job Attributes

The client MUST supply a set of Job attributes as defined in [ipp-mod] section 4.2 Job Template Attributes ("xxx" attributes) and section 4.3 Job Description Attributes.  Each Job attribute supplied in Group 2 replaces the value(s) of the corresponding Job attribute on the target Job object.  For attributes that can have multiple values (1setOf), all values supplied by the client replace all values of the corresponding Job object attribute.

#### Set-Job-Attributes Response

The Printer object returns the following sets of attributes as part of the Set-Job-Attributes Response:

Group 1: Operation Attributes

Status Message:

In addition to the REQUIRED status code returned in every response, the response OPTIONALLY includes a "status-message" (text(255)) and/or a "detailed-status-message" (text(MAX)) operation attribute as described in [ipp-mod] sections 13 and 3.1.6.

Natural Language and Character Set:

The "attributes-charset" and "attributes-natural-language" attributes as described in [ipp-mod] section 3.1.4.2.

Group 2: Unsupported Attributes

See [ipp-mod] section 3.1.7 for details on returning Unsupported Attributes.

The attributes returned are the same as for the create operation with the same new attribute values.  However, in the case of attributes that are supported, but are not settable by the implementation (see table above), the Printer object returns the client-supplied attribute(s) with a substituted value of 'not-settable'.  This value's syntax type is "out-of-band" and its encoding is defined by special rules for "out-of-band" values in the "Encoding and Transport" document [IPP-PRO].  Its value indicates that the attribute is not settable.

Group 2 is returned independent of the value for the "ipp-attribute-fidelity" attribute supplied by the client.  However, if "ipp-attribute-fidelity" is 'true', the 'client-error-attributes-not-settable' status code is returned.  If "ipp-attribute-fidelity" is 'false', the 'successful-ok-ignored-or-substituted-attributes' status code is returned.

### Reprocess-Job Operation

This OPTIONAL operation is a create job operation that allows a client to re-process a copy of a job that had been retained in the queue after processing completed, was canceled, or was aborted (see [ipp-mod] section 4.3.7.2).  This operation is the same as the Restart-Job operation (see [ipp-mod] section 3.3.7), except that the Printer creates a new job that is a copy of the target job and the target job is unchanged.  The new job is assigned new values to the "job-uri" and "job-id" attributes and the new job's Job Description attributes that accumulate job progress, such as "job-impressions-completed", "job-media-sheets-completed", and "job-k-octets-processed", are initialized to 0 as with any create job operation.  The target job moves to the Job History after a suitable period, independent of whether one or more Reprocess-Job operations have been performed on it.

If the Set-Job-Attributes operation is supported, then the "job-hold-until" operation attribute MUST be supported with at least the 'indefinite' value, so that a client can modify the new job before it is scheduled for processing using the Set-Job-Attributes operation.  After modifying the job, the client can release the job for processing, by using the Release-Job operation specifying the newly assigned "job-uri" or "job-id" for the new job.

### Cancel-Current-Job Operation

This OPTIONAL operation allows a client to cancel the current job on the target Printer.  Since a Job might already be marking by the time a Cancel-Current-Job is received, some media sheet pages might be printed before the job is actually terminated.

If there are no jobs in the 'processing' or 'processing-stopped' states, the Printer rejects the request and returns the 'client-error-not-possible' status code.  Otherwise, the Printer MUST perform the Cancel-Current-Job request, as if the client had supplied the "job-uri" or the "job-id" of the current job in a Cancel-Job operation.  If more than one job is in the 'processing' or 'processing-stopped' states, the one that is marking is canceled and the others are unaffected.

Warning:  On a shared printer, there is a race condition.  Between the time that a user issues this operation and its acceptance, the current job might change to a different job.  If the user is authenticated to cancel the new job, the wrong job is canceled.  To prevent this race from canceling the wrong job, the client can supply the "job-id" operation attribute which is checked against the current job's job-id.  If the job identified by the "job-id" attribute is not the current job on the Printer, the Printer MUST reject this operation and return the 'client-error-not-possible' status code.

Access Rights: The authenticated user (see [ipp-mod] section 8.3) performing this operation must either be the job owner of the current job or an operator or administrator of the Printer object (see [ipp-mod] sections 1 and 8.5).  Otherwise, the IPP object MUST reject the operation and return: 'client-error-forbidden', 'client-error-not-authenticated', or 'client-error-not-authorized' as appropriate.

The Cancel-Current-Job Request and Cancel-Current-Job Response have the same attribute groups and attributes as the Pause-Printer operation (see [ipp-mod] sections 3.2.7.1 and 3.2.7.2), including the new "job-message-from-operator" operation attribute (see section 2.2), with the addition of the following Group 1 Operation attributes in the request:

"job-id" (integer(1:MAX)):

The client OPTIONALLY supplies this Operation attribute in order to verify that the identified job is still the current job on the target Printer object.  The IPP object MUST supports this operation attribute, if it supports this operation.

"non-process-run-out" (boolean):

The client OPTIONALLY supplies this Operation attribute.  The IPP object OPTIONALLY supports this operation attribute, if it is able to perform a non-process-run-out, i.e., remove the last few printed sheets of a job that is finished printing on a continuous-forms printer.  Normally, on such a printer, the last sheets of a job remain in the printer and are forced out by the next job, thereby saving time when printing one job after another.  However, when canceling the current job it is desirable to move the end of the canceled job to the stacker.  This attribute indicates whether or not the printer is to perform "non-process run-out", i.e., move the last printed sheet to the stacker, after canceling the current job.

If the client does not supply this attribute and the attribute is supported, the Printer does not perform non-process run-out ('false' behavior).  If the Printer does not support this attribute, the Printer is assumed to perform non-process run-out ('true' behavior).

ISSUE 8:  Why isn't non-process-run-out automatic on a continuous form printer?  When would an operator want to cancel the job and NOT run out the last sheets.?  It would be simpler to require process-run-out when canceling the current job (for continuous and cut sheet printers).

### Pause-Job operation

This OPTIONAL operation allows a client to stop the target job and allow other jobs to be processed instead.  The Printer moves the target job to the 'pending-held' state and sets the 'job-paused' value in the job's "job-state-reasons" attribute.  ISSUE 9:  Or should the job's state be 'processing-stopped' after a Pause-Job or Pause-Current-Job operation in order to indicate that it has processed some with the 'job-paused' value added to the job's "job-state-reasons" attribute?  Then the 'pending-held' job state is not partitioned into two parts depending on whether the Hold-Job or Pause-Job operation was performed.

If the target job is not in the 'processing' or 'processing-stopped' states, the Printer rejects the request and returns the 'client-error-not-possible' status code.  Otherwise, the Printer stops the target job and processes other jobs.

The Printer MUST reject a Release-Job request (and return the 'client-error-not-possible') for a job that has been paused , i.e., for a job in the 'pending-held' state, with the 'job-paused' value in its "job-state-reasons" attribute.

If the Pause-Job operation is supported, then the Pause-Current-Job and Resume-Job operations MUST be supported, and vice-versa.

Access Rights: The authenticated user (see [ipp-mod] section 8.3) performing this operation must either be the job owner of the target job or an operator or administrator of the Printer object (see [ipp-mod] sections 1 and 8.5).  Otherwise, the IPP object MUST reject the operation and return: 'client-error-forbidden', 'client-error-not-authenticated', or 'client-error-not-authorized' as appropriate.

The Pause-Job Request and Pause-Job Response have the same attribute groups and attributes as the Cancel-Job operation (see [ipp-mod] section 3.3.3.1 ), including the new "job-message-from-operator" operation attribute (see section 2.2), the new "when" operation attribute (see section 2.3), with the addition of the following Group 1 Operation attribute in the request:

"non-process-run-out" (boolean)

The client OPTIONALLY supplies this attribute.  The Printer object OPTIONALLY supports this attribute, if it is able to perform a non-process-run-out, i.e., remove the last few printed sheets of a job that is finished printing on a continuous-forms printer.  Normally, on such a printer, the last sheets of a job remain in the printer and are forced out by the next job, thereby saving time when printing one job after another.  This attribute indicates whether or not the printer is to perform "non-process run-out", i.e., move the last printed sheet to the stacker, as part of pausing the job.

If the client does not supply this attribute and the attribute is supported, the Printer does not perform non-process run-out ('false' behavior).  If the Printer does not support this attribute, the Printer is assumed to perform non-process run-out ('true' behavior).

ISSUE 10:  Is the "non-process-run-out" operation attribute really needed at all or can the default behavior for Pause-Job be defined to be to perform non-process run out (for continuous and cut sheet printers)?

"synchronize" (boolean)

The client OPTIONALLY supplies this attribute.  The Printer object OPTIONALLY supports this attribute.  This attribute indicates whether or not the printer is to synchronize the checkpoint data for the current job being paused with the pages that have actually printed.  If this attribute is supported, then a value of 'true' implies that the Printer will be able to resume the job at the point of synchronization when the job is resumed.

If the client does not supply this attribute and the attribute is supported, the Printer does not perform synchronization ('false' behavior).  If the Printer does not support this attribute, the Printer is assumed to not synchronize ('false' behavior).

ISSUE 11:  Do we really need the "synchronize" operation attribute or can synchronization be the default behavior of the Pause-Job operation?

### Pause-Current-Job operation

This OPTIONAL operation allows a client to stop the current job on the target Printer and allow other jobs to be processed instead.  The Printer moves the current job to the 'pending-held' state and sets the 'job-paused' value in the job's "job-state-reasons" attribute.   ISSUE 12:  Or should the job's state be 'processing-stopped' after a Pause-Job or Pause-Current-Job operation in order to indicate that it has processed some?  Then the 'pending-held' job state is not partitioned into two parts depending on whether the Hold-Job or Pause-Current-Job operation was performed.

If there are no jobs in the 'processing' or 'processing-stopped' states, the Printer rejects the request and returns the 'client-error-not-possible' status code.  Otherwise, the Printer stops the current job and processes other jobs.

Warning:  On a shared printer, there is a race condition.  Between the time that a user issues this operation and its acceptance, the current job might change to a different job.  If the user is authenticated to pause the new job, the wrong job is paused.  To prevent this race from pausing the wrong job, the client can supply the "job-id" operation attribute which is checked against the current job's job-id.  If the job identified by the "job-id" attribute is not the current job on the Printer, the Printer MUST reject this operation and return the 'client-error-not-possible' status code.

If the Pause-Current-Job operation is supported, then the Pause-Job and Resume-Job operations MUST be supported, and vice-versa.

Access Rights: The authenticated user (see [ipp-mod] section 8.3) performing this operation must either be the job owner of the current job or an operator or administrator of the target Printer object (see [ipp-mod] sections 1 and 8.5).  Otherwise, the IPP object MUST reject the operation and return: 'client-error-forbidden', 'client-error-not-authenticated', or 'client-error-not-authorized' as appropriate.

The Pause-Current-Job Request and Pause-Current-Job Response have the same attribute groups and attributes as the Pause-Job operation (see 6.2.4), including the new "job-message-from-operator" operation attribute (see section 2.2), with the addition of the following Group 1 Operation attribute in the request, but not including the new "when" operation attribute (see section 2.3):

"job-id" (integer(1:MAX)):

The client OPTIONALLY supplies this Operation attribute in order to verify that the identified job is still the current job on the target Printer object.  The IPP object MUST supports this operation attribute, if it supports this operation.

### Resume-Job operation

This OPTIONAL operation allows a client to stop the target job and allow other jobs to be processed instead.  The Printer moves the target job to the 'pending' state and removes the 'job-paused' value from the job's "job-state-reasons" attribute.  ISSUE 13:  Or should the job's state be 'processing-stopped' after a Pause-Job or Pause-Current-Job operation in order to indicate that it has processed some?  Then the 'pending-held' job state is not partitioned into two parts depending on whether the Hold-Job or Pause-Job operation was performed.

If the target job is not in the 'pending-held' state with the 'job-paused' value in the job's "job-state-reasons" attribute, the Printer rejects the request and returns the 'client-error-not-possible' status code.

If the Resume-Job operation is supported, then the Pause-Job and Pause-Current-Job operations MUST be supported, and vice-versa.

Access Rights: The authenticated user (see [ipp-mod] section 8.3) performing this operation must either be the job owner of the target job or an operator or administrator of the Printer object (see [ipp-mod] sections 1 and 8.5).  Otherwise, the IPP object MUST reject the operation and return: 'client-error-forbidden', 'client-error-not-authenticated', or 'client-error-not-authorized' as appropriate.

The Resume-Job Request and Resume-Job Response have the same attribute groups and attributes as the Cancel-Job operation (see [ipp-mod] section 3.3.3), including the new "job-message-from-operator" operation attribute (see section 2.2).

### Promote-Job operation

This OPTIONAL operation allows a client to make the pending target job be processed next after the current job completes.

If the target job is in the 'pending' state, this operation does not change the job's state, but the Printer adds the 'process-next' value to the job's "job-state-reasons" attribute.  If the target job is not in the 'pending' state, the Printer rejects the request and returns the 'client-error-not-possible' status code.  The Printer returns the target job immediately after the current job(s) in a Get-Jobs response (see [ipp-mod] section 3.2.6) for the 'not-completed' jobs.

When the current job completes, is canceled, paused, or aborted, the target of this operation is processed next, at which time the Printer removes the 'process-next' value from the job's "job-state-reasons" attribute.

If a client issues this request (again) before the target of the operation of the original request started processing, the target of this new request replaces the previous job that was to be processed next.  The Printer removes the 'process-next' from the previous job and adds the value to the target of the subsequent request.  In other words, only one job at a time has the property to be the next job and no more than one job has the 'process-next' value in its "job-state-reasons" attribute at the same time.

If a client issues a Hold-Job operation with the target being the job that is to be processed next, the Printer removes the 'process-next' value from the target job's "job-state-reasons" attribute and moves it to the 'held' state.

Access Rights: The authenticated user (see [ipp-mod] section 8.3) performing this operation must be an operator or administrator of the Printer object (see [ipp-mod] sections 1 and 8.5).  Otherwise, the IPP object MUST reject the operation and return: 'client-error-forbidden', 'client-error-not-authenticated', or 'client-error-not-authorized' as appropriate.

The Promote-Job Request and Promote-Job Response have the same attribute groups and attributes as the Cancel-Job operation (see [ipp-mod] section 3.3.3), including the new "job-message-from-operator" operation attribute (see section 2.2).

### Space-Current-Job operation

This OPTIONAL operation allows a client to repeat or skip a specified number of impressions for the current job on the target Printer.  This operation is typically supported in a continuous forms implementation for synchronizing the web after forms run out or media change.

If there are no jobs in the 'processing' or 'processing-stopped' states, the Printer rejects the request and returns the 'client-error-not-possible' status code.  Otherwise, the Printer repeats or skips the indicated number of impressions specified by the "back-space" or "forward-space" operation attribute, respectively.

Warning:  On a shared printer, there is a race condition.  Between the time that a user issues this operation and its acceptance, the current job might change to a different job.  If the user is authenticated to space the new job, the wrong job is spaced.  To prevent this race from spacing the wrong job, the client can supply the "job-id" operation attribute which is checked against the current job's job-id.  If the job identified by the "job-id" attribute is not the current job on the Printer, the Printer MUST reject this operation and return the 'client-error-not-possible' status code.

Access Rights: The authenticated user (see [ipp-mod] section 8.3) performing this operation must either be the job owner of the current job or an operator or administrator of the Printer object (see [ipp-mod] sections 1 and 8.5).  Otherwise, the IPP object MUST reject the operation and return: 'client-error-forbidden', 'client-error-not-authenticated', or 'client-error-not-authorized' as appropriate.

The Space-Current-Job Request and Space-Current-Job Response have the same attribute groups and attributes as the Space-Printer operation (see section 6.1.6), including the new "job-message-from-operator" operation attribute (see section  2.2), with the addition of the following Group 1 Operation attribute in the request:

"job-id" (integer(1:MAX)):

The client OPTIONALLY supplies this Operation attribute in order to verify that the identified job is still the current job on the target Printer object.  The IPP object MUST supports this operation attribute, if it supports this operation.