INTERNET-DRAFT  - 14 ISSUES are numbered and highlighted like this	Carl Kugler

<draft-ietf-ipp-ops-set2-01.txt>	IBM Corporation

T. Hastings

Xerox Corporation

H. Lewis

IBM Corporation

October 29, 1999

Internet Printing Protocol/1.1: Set2 Operations

Status of this Memo

This document is an Internet-Draft and is in full conformance with all provisions of Section 10 of [RFC2026].  Internet-Drafts are working documents of the Internet Engineering Task Force (IETF), its areas, and its working groups.  Note that other groups may also distribute working documents as Internet-Drafts.

Internet-Drafts are draft documents valid for a maximum of six months and may be updated, replaced, or obsoleted by other documents at any time.  It is inappropriate to use Internet-Drafts as reference material or to cite them other than as "work in progress".

The list of current Internet-Drafts can be accessed at http://www.ietf.org/ietf/1id-abstracts.txt

The list of Internet-Draft Shadow Directories can be accessed as http://www.ietf.org/shadow.html.

Abstract

This document specifies 14 additional OPTIONAL operations for use with the Internet Printing Protocol/1.0 (IPP) [RFC2565, RFC2566] and IPP/1.1 [ipp-mod, ipp-pro].  These operations are 7 Printer object operations that operators/administrators may perform on a Printer object:

Set-Printer-Attributes

Enable-Printer

Disable-Printer

Restart-Printer

Shutdown-Printer

and 7 Job object operations that end-users may perform on their jobs and operators/administrators may perform on any job, depending on circumstances:

Set-Job-Attributes

Reprocess-Job

Cancel-Current-Job  (though the target is the Printer object)

Pause-Current-Job  (though the target is the Printer object)

Resume-Job

Promote-Job

The Interpreter object is added for those implementations that make a distinction on the values of some Printer attributes depending on the document-format, such as "resolution-supported".

In addition, two operation attributes are defined:  "printer-message-from-operator" and "job-message-from-operator" are included to set the corresponding Printer and Job Description attributes with the same names.  And the "when" operation attribute is added to the IPP/1.1 Pause-Printer operation as well as some of the Set2 Printer operations.

New Printer Description attributes are added, along with additional values for the "printer-state-reasons" and "job-state-reasons" attributes.

New status codes: 'client-error-attributes-not-settable' and 'server-error-printer-is-in-standby-mode' are added.

Finally, the 'not-settable' out-of-band value is added for use with the Set-Printer-Attributes and Set-Job-Attributes operations.

The scope of IPP, is characterized in RFC2526 "Design Goals for an Internet Printing Protocol".  It is not the intent of this document to revise or clarify this scope or conjecture as to the degree of industry adoption or trends related to IPP within printing systems.  It is the intent of this document to extend the original set of operations - in a similar fashion to the Set1 extensions which referred to IPP/1.0 and were later incorporated into IPP/1.1.

This document is intended for registration following the registration procedures of IPP/1.0 [RFC2566] and IPP/1.1 [ipp-mod].  This version includes the comments discussed at the IPP telecon, on 6/23/1999, 6/30/1999, at the IETF IPP WG meeting, 7/7/99-7/8/99, in Copenhagen, and the IPP telecon, 7/17/1999, the August, 1999 IPP meeting in Alaska and subsequent phone conferences and discussions.  Specifically, the 9/16 update refers to this set of extensions simply as "Set2" rather than using the term "Administrative" which was misleading, controversial and incorrect as an overall description. Also, two new attributes have been proposed to clarify the intent of each operation in terms of its target, the Printer vs. the Print Job.  Unresolved ISSUES are highlighted like this in the .doc and .pdf files.

The full set of IPP documents includes:

Design Goals for an Internet Printing Protocol [RFC2567]

Rationale for the Structure and Model and Protocol for the Internet Printing Protocol [RFC2568]

Internet Printing Protocol/1.1: Model and Semantics (this document)

Internet Printing Protocol/1.1: Encoding and Transport [IPP-PRO]

Internet Printing Protocol/1.1: Implementer's Guide [IPP-IIG]

Mapping between LPD and IPP Protocols [RFC2569]

The "Design Goals for an Internet Printing Protocol" document takes a broad look at distributed printing functionality, and it enumerates real-life scenarios that help to clarify the features that need to be included in a printing protocol for the Internet.  It identifies requirements for three types of users: end users, operators, and administrators.  It calls out a subset of end user requirements that are satisfied in IPP/1.0.  A few OPTIONAL operator operations have been added to IPP/1.1.

The "Rationale for the Structure and Model and Protocol for the Internet Printing Protocol" document describes IPP from a high level view, defines a roadmap for the various documents that form the suite of IPP specification documents, and gives background and rationale for the IETF working group's major decisions.

The "Internet Printing Protocol/1.1: Encoding and Transport" document is a formal mapping of the abstract operations and attributes defined in the model document onto HTTP/1.1 [RFC2616].  It defines the encoding rules for a new Internet MIME media type called "application/ipp".  This document also defines the rules for transporting over HTTP a message body whose Content-Type is "application/ipp".  This document defines a new scheme named 'ipp' for identifying IPP printers and jobs.

The "Internet Printing Protocol/1.1: Implementer's Guide" document gives insight and advice to implementers of IPP clients and IPP objects.  It is intended to help them understand IPP/1.1 and some of the considerations that may assist them in the design of their client and/or IPP object implementations.  For example, a typical order of processing requests is given, including error checking.  Motivation for some of the specification decisions is also included.

The "Mapping between LPD and IPP Protocols" document gives some advice to implementers of gateways between IPP and LPD (Line Printer Daemon) implementations.

Table of Contents

1.	Introduction	6

2.	New objects	6

2.1	Interpreter object	6

2.2	Use of the Printer object to represent fan-out	7

2.3	Device object	8

3.	New Operation attributes	8

3.1	New operation attribute:  "printer-message-from-operator" (text(127))	8

3.2	New operation attribute:  "job-message-from-operator" (text(127))	9

3.3	New operation attribute for Get-Printer-Attributes:  "factory-settings" (boolean)	10

12

13

3.6	Summary of the operation attributes for the Printer operations	13

3.7	Summary of the operation attributes for the Job operations	14

4.	New Printer Description Attributes	14

4.1	printer-settable-attributes (1setOf type2 keyword)	14

15

4.3	job-settable-attributes (1setOf type2 keyword)	16

4.4	printer-controls-other-protocols (boolean)	16

4.5	printer-message-time (integer(MIN:MAX))	17

4.6	printer-message-date-time (dateTime)	18

4.7	printer-message-operation (type2 keyword)	18

19

19

4.8	subordinate-printers-supported (1setOf uri)	19

5.	Additional values for "printer-state-reasons" and "job-state-reasons" attributes	20

5.1	Value for "printer-state-reasons":  'standby'	20

5.2	Value for "job-state-reasons":  'job-paused'	20

6.	New status codes	21

6.1	New status code: 'client-error-attributes-not-settable'	21

6.2	New status code: 'server-error-printer-is-in-standby-mode'	21

7.	New out-of-band values	22

7.1	'not-settable' out of band value	22

8.	Summary of Set 2 operations and Operation-Id Assignments	22

8.1	Printer Operations	23

8.1.1	Set-Printer-Attributes Operation	23

8.1.2	Disable-Printer Operation	28

8.1.3	Enable-Printer Operation	29

30

8.1.5	Restart-Printer operation	31

32

8.1.7	Shutdown-Printer Operation	33

8.2	Job Operations	36

8.2.1	Set-Job-Attributes	36

8.2.2	Reprocess-Job Operation	40

8.2.3	Cancel-Current-Job Operation	41

8.2.4	Pause-Current-Job operation	42

8.2.5	Resume-Job operation	44

8.2.6	Promote-Job operation	45

45

9.	IANA Considerations	47

10.	Internationalization Considerations	47

11.	Security Considerations	47

12.	Author's Addresses	47

13.	References	48

14.	Change History	48

14.1	Changes to the October 22, 1999 version to make the October 29, 1999 version	48

14.2	Changes to the September 19, 1999 version to make the October 22, 1999 version	48

14.3	Changes to the July 19, 1999 version to make the September 19, 1999 version	49

14.4	Changes to the June 30, 1999 version to make the July 19, 1999 version	50

15.	Appendix A: Full Copyright Statement	51

# Introduction

The Internet Printing Protocol (IPP) is an application level protocol that can be used for distributed printing using Internet tools and technologies.  IPP version 1.1 (IPP/1.1) focuses on end user functionality with a few administrative operations included.  This document defines additional OPTIONAL end user, operator, and administrator operations used to control Jobs and Printers. This document is a registration proposal for an extension to IPP/1.0 and IPP/1.1 following the registration procedures in those documents.

# New objects

This section defines the new Interpreter object and the use of the Printer object to represent fan-out.

## Interpreter object

The Interpreter object models the document format interpreters that are contained in the Printer object.  The purpose of the Interpreter object is to model those Printer attributes whose value depends on which interpreter is being used to process the document.  Depending on implementation, the Printer object attributes whose values ("xxx-supported" and "xxx-default") depend on the interpreter, i.e., on the "document-format" of the document being processed, are considered to be Interpreter object attributes instead.  A Get-Printer-Attributes operation returns Printer and Interpreter attributes as specified in the "requested-attributes" operation attribute supplied by the client.  Depending on the value of the "document-format" attribute supplied by the client in the Get-Printer-Attributes request (or the "document-format-default", if the client omits the "document-format" attribute), selects the corresponding Interpreter object.

If an implementation does not make a distinction on the values of Printer attributes by document format, say, for purposes of job validation (see [ipp-mod] Get-Printer-Attributes), there is no need to implement or support the Interpreter object.  The Interpreter object is introduced to provide a means to model an implementation in which some attributes do depend on the document format.  Those attributes are then sub-classed to be Interpreter object attributes.

Note:  the addition of the Interpreter object is completely compatible with IPP/1.0 and IPP/1.1 (see the description of the "document-format" operation attribute in [ipp-mod] section 3.2.5.1 Get-Printer-Attributes request).  The protocol and semantics are the same whether or not the Interpreter object is used to distinguish attributes that depend on the "document-format".  Consequently, there is no "interpreters-supported" Printer Description attribute.  In order to determine which Interpreter objects there MAY be, the client can request the values of the Printer's "document-format-supported" attribute.

Note:  The Interpreter object is really a sub-class of the Printer object, rather than being a full fledged object in the sense that the Job and Printer objects are.  There are no operations defined solely for the Interpreter object.  The Get-Printer-Attributes and Set-Printer-Attributes operations operate on the Interpreter object if the implementation has the concept of an Interpreter object.  However, if the IPP Printer implementation does not contain Interpreter objects, the same Interpreter object attributes are considered Printer object attributes, instead.

## Use of the Printer object to represent fan-out

The IPP/1.1 Model and Semantics introduces the semantic concept of an IPP Printer object that represents more than one output device (see [ipp-mod] section 2.1).  However, there was no way to represent the individual states of the devices or to perform operations on selected devices.  This specification generalizes the semantics of the Printer object to represent such "subordinate" fan-out devices as IPP Printer objects.  Thus a "parent" Printer object MAY support two or more subordinate Printer objects in order to represent fan-out.  Such a parent Printer object MUST support the "subordinate-printers-supported" (1setOf uri) Printer Description attribute.

A conforming subordinate Printer object NEED NOT support all of the REQUIRED operations.  For example a subordinate Printer NEED NOT support a Print-Job operation, unless it is a networked printer that supports the IPP Protocol.  In such a case, the subordinate Printer MAY be configured so that its access control list only permits the parent Printer to submit jobs to it.  A subordinate Printer MAY, in turn, have subordinate Printers.

ISSUE 01 - What sub-set of the REQUIRED IPP/1.1 operations MUST a subordinate Printer object support?  Get-Printer-Attributes?  Any others?

ISSUE 02 - Are there any IPP Printer or Job operations that a subordinate Printer MUST NOT support?

ISSUE 03 - Are all the REQUIRED attributes of a Printer object REQUIRED for a subordinate Printer object?

The degenerate case of a Printer having only a single subordinate Printer is not allowed.

Each subordinate Printer object has a URI which is used as the target of each operation on the subordinate Printer.  The means for configuring URIs for subordinate Printer objects is implementation-dependent as are all URIs.  However, there are two distinct approaches:

a. When the implementation wants to make sure that no operation on a subordinate Printer object as a target "sneaks by" the parent Printer object (or the subordinate Printer is fronting for a device that is not networked), the host part of the URI specifies the host of the parent Printer.  Then the parent Printer object can easily reflect the state of the subordinate Printer objects in the parent's Printer object state and state reasons as the operation passes "through" the parent Printer object.

b. When the subordinate Printer is networked and the implementation allows operations to go directly to the subordinate Printer (with proper access control) without knowledge of the parent Printer object, the host part of the URI is different than the host part of the parent Printer object.  In such a case, the parent Printer object MUST keep its "printer-state" and "printer-state-reasons" up to date, either by polling the subordinate Printer object or by subscribing to events with the subordinate Printer object (see [ipp-not-spec] for means to do this when the subordinate Printer object supports IPP notification).

## Device object

In order to control the physical device, the Device object is introduced.  Its semantics are defined in [ipp-set3].  The Device object has attributes that represent physical characteristics of the device, such as input trays, output bins, and finishing equipment.  Thus the Printer object remains as the representative of a logical entity or service, while the Device object represents a physical entity.  The operations in this Set2 document operate only on Printer objects, not on Device objects.  However, some of the Set2 Printer operations have analogous Device operations.  For example, the Shutdown-Printer and Shutdown-Device operations are defined in Set2 and Set3, respectively.

# New Operation attributes

This section defines the new "printer-message-from-operation" and "job-message-from-operator" operation attributes that set the corresponding Printer and Job Description attributes.

## New operation attribute:  "printer-message-from-operator" (text(127))

Type of registration:  attribute

Proposed keyword name of this attribute:  "printer-message-from-operator"

Types of attribute (Operation, Job Template, Job Description, Printer Description):  Operation

Operations to be used with if the attribute is an operation attribute:  See below

Object (Job, Printer, etc. if bound to an object):   Printer (already in IPP/1.0 and IPP/1.1)

Attribute syntax(es) (include 1setOf and range as in Section 4.2):  text(127)

Specification of this attribute (follow the style of IPP Model Section 4.2):

"printer-message-from-operator" (text(127))

The client OPTIONALLY supplies this attribute.  The Printer object SHOULD supports this operation attribute if it supports the corresponding Printer Description attribute.  The value of this attribute is a message from the operator about the Printer object on which the operator is performing the operation.  If supported, the Printer copies the value to the Printer's "printer-message-from-operator" Printer Description attribute (see [ipp-mod] section 4.4.25), automatically sets the value of the Printer's "printer-message-time" with the current value of the Printer's "printer-up-time" attribute, the value of the "printer-message-date-time" with the current value of the Printer's printer-current-date-time" attribute, and the value of the Printer's "printer-message-operation" with the operation-id value of this operation (see [ipp-mod] section 4.4.15).  

If the client omits this attribute, the Printer does not change the value of its "printer-message-from-operator", "printer-message-time", "printer-message-date-time", and "printer-message-operation" Printer Description attributes.

If the client supplies this attribute with a zero-length text value or with a value consisting solely of white space, the Printer copies that value as any other value to the Printer's "printer-message-from-operator" and sets the "printer-message-time", "printer-message-date-time", and "printer-message-operation" attributes.  Supplying such a value is the way that the operator indicates that there is no longer a printer message from the operator (rather than using the "out-of-band" 'no-value' value).

This operation attribute is defined for use with the following operator operations on the Printer object:

Pause-Printer - see [ipp-mod] section 3.2.7

Resume-Printer - see [ipp-mod] section 3.2.8

Purge-Jobs - see [ipp-mod] section 3.2.9

Disable-Printer - see section 8.1.2

Enable-Printer - see section 8.1.3

Restart-Printer - see section 8.1.5

Shutdown-Printer - see section 8.1.7

The "printer-message-from-operator" operation attribute MUST NOT be supported as an operation attribute for the Set-Printer-Attributes operation.  If the operator wants to set the Printer's "printer-message-from-operator" Printer Description attribute when issuing the Set-Printer-Attributes operation, the client supplies the "printer-message-from-operator" with its new value as one of the Printer Description attributes in Group 2 in the request.  The Printer also updates the Printer's "printer-message-date-time" and "printer-message-operation" Printer Description attributes.  If the client does not explicitly supply the "printer-message-from-operator" with its new value, the Printer leaves the value of the Printer's "printer-message-from-operator" Printer Description attribute unchanged.

## New operation attribute:  "job-message-from-operator" (text(127))

Type of registration:  attribute

Proposed keyword name of this attribute:  "job-message-from-operator"

Types of attribute (Operation, Job Template, Job Description, Printer Description):  Operation

Operations to be used with if the attribute is an operation attribute:  See below

Object (Job, Printer, etc. if bound to an object):   Job (already in IPP/1.0 and IPP/1.1)

Attribute syntax(es) (include 1setOf and range as in Section 4.2):  text(127)

Specification of this attribute (follow the style of IPP Model Section 4.2):

"job-message-from-operator" (text(127))

The client OPTIONALLY supplies this attribute.  The Printer object SHOULD supports this operation attribute if it supports the corresponding Job Description attribute.  The value of this attribute is a message from the operator about the Job object on which the operator has just performed an operation.  If supported, the Printer copies the value to the Job's "job-message-from-operator" Job Description attribute (see [ipp-mod] section 4.3.16).  

If the client omits this attribute, the Printer does not change the value of its "printer-message-from-operator" Job Description attribute.

If the client supplies this attribute with a zero-length text value or with a value consisting solely of white space, the Printer copies that value as any other value to the job's "job-message-from-operator".  Supplying such a value is the way that the operator indicates that there is no longer a job message from the operator (rather than using the "out-of-band" 'no-value' value).

Note:  There are no corresponding 'job-message-time", "job-message-date-time", and "job-message-operation" Job Description attributes, since the usual lifetime of a job is limited.

This operation attribute is defined for use with the following operator operations on the Job object:

Cancel-Job - see [ipp-mod] section 3.2.4

Hold-Job - see [ipp-mod] section 3.3.5

Release-Job - see [ipp-mod] section 3.3.6

Restart-Job - see [ipp-mod] section 3.3.7

Reprocess-Job - see section 8.2.2

Cancel-Current-Job - see section 8.2.3

Pause-Current-Job - see section 8.2.4

Resume-Job - see section 8.2.5

Promote-Job - see section 8.2.6

The "job-message-from-operator" operation attribute MUST NOT be supported as an operation attribute for the Set-Job-Attributes operation.  If the operator wants to set the Job's "job-message-from-operator" Job Description attribute when issuing the Set-Job-Attributes operation, the client supplies the "job-message-from-operator" with its new value as one of the Job Description attributes in Group 2 in the request.  Otherwise, the Printer leaves the value of the Job's "job-message-from-operator" Job Description attribute unchanged by not explicitly setting the attribute.

## New operation attribute for Get-Printer-Attributes:  "factory-settings" (boolean)

Type of registration:  attribute

Proposed keyword name of this attribute:  "factory-settings"

Types of attribute (Operation, Job Template, Job Description, Printer Description):  Operation

Operations to be used with if the attribute is an operation attribute:  Get-Printer-Attributes

Object (Job, Printer, etc. if bound to an object):   N/A

Attribute syntax(es) (include 1setOf and range as in Section 4.2):  boolean

Specification of this attribute (follow the style of IPP Model Section 4.2):

"factory-settings" (boolean)

The client OPTIONALLY supplies this attribute.  The Printer object OPTIONALLY supports this attribute, if it supports the Set-Printer-Attributes operation.  If the client omits this attribute or supplies the 'false' value, the Printer returns the current values of the requested attributes that are settable, i.e., the values that have been set by previous Set-Printer-Attributes.  If the client supplies the 'true' value, the Printer returns the factory settings, i.e., the inherent values supported by the implementation as shipped from the manufacturer or established at install time.  This operation attribute allows an operator to determine which values are supported in an implementation after having modified a settable attribute.  Attributes that are not settable are not affected by this operation attribute, so that the Printer returns the same values for non-settable attribute when either the 'true' or 'false' value has been supplied.  If this operation attribute is supported, both 'true' and 'false' values MUST be supported.

## New operation attribute for Pause-Printer:  "when" (type2 keyword)

Type of registration:  attribute

Proposed keyword name of this attribute:  "when"

Types of attribute (Operation, Job Template, Job Description, Printer Description):  Operation

Operations to be used with if the attribute is an operation attribute:  Pause-Printer, Shutdown-Printer, and Pause-Current-Job

Object (Job, Printer, etc. if bound to an object):   N/A

Attribute syntax(es) (include 1setOf and range as in Section 4.2):  type2 keyword

Specification of this attribute (follow the style of IPP Model Section 4.2):

"when" (type2 keyword)

The client OPTIONALLY supplies this attribute.  The Printer object OPTIONALLY supports this attribute, if it supports this operation.  The value of this attribute indicates when to pause the printer.  If the client omits this attribute, the Printer assumes the 'after-current-job' value.  The 'after-current-job' value is REQUIRED to be supported if the "when" attribute is supported; the remaining values are OPTIONAL.

Note:  There is no way to query to see which values of the "when" attribute are supported for this or any other operation.  Since this operation is intended for Operators, rather than End-Users, it did not seem necessary to provide the means to query the supported values.

Standard keyword values are:

'now' - pause the Printer immediately, while the current job is processing.  See [ipp-mod] section 3.2.7) for the effect on the Printer's "printer-state" and "printer-state-reasons" attributes.  Jobs in the 'held' and 'pending' state remain in those states.

'after-current-copy' - pause the Printer after the current job finishes printing its current copy.  Jobs in the 'held' and 'pending' state remain in those states.

'after-current-job' - pause the Printer after the current job finishes printing (all its copies).  Jobs in the 'held' and 'pending' state remain in those states.

'after-all' - pause the Printer after all 'pending' jobs finish printing.  Jobs in the 'held' state remain in the 'held' state.

Note:  The "when" operation attribute is also defined for use with the Shutdown-Printer (see section 8.1.7) and the Pause-Current-Job (see section 8.2.4) operations, but with a subset of these values.

## Summary of the operation attributes for the Printer operations

The following table shows the operation attributes for the Printer operations that a client MAY supply in a request.  Operation parameters and attributes that a client MUST supply are not shown.  Also "requesting-user-name" is not shown, though it is RECOMMENDED that a client supply it in every request.

Legend:

R - REQUIRED for a Printer to support

O - OPTIONAL for a Printer to support; the Printer ignores the attribute if not supported

| Operation Attribute | Pause-Printer | Resume-Printer | Purge-Jobs | Get-Printer-Attributes | Set-Printer-Attributes | Enable-Print | Disable-Printer |  | Restart-Printer |  | Shut down-Printer |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| document-format |  |  |  | R | R |  |  |  |  |  |  |
| factory-settings |  |  |  | O |  |  |  |  |  |  |  |
| printer-message-from-operator | O | O | O |  |  | O | O |  | O |  | O |
| job-type |  |  |  |  |  | O | O |  |  |  |  |
| when | O |  |  |  |  |  |  |  |  |  | O |
| reset-function |  |  |  |  |  |  |  |  |  |  |  |
| synchronize |  |  |  |  |  |  |  |  |  |  | O |
| shutdown-function |  |  |  |  |  |  |  |  |  |  | R |

## Summary of the operation attributes for the Job operations

The following table shows the operation attributes for the Job operations that a client MAY supply in a request.  Operation attributes that specify the Printer or Job object as the target are shown in the first two rows, respectively.  Other operation attributes and parameters that a client MUST supply are not shown.  Also "requesting-user-name" is not shown, though it is RECOMMENDED that a client supply it in every request.

Legend:

R - REQUIRED for a Printer to support

O - OPTIONAL for a Printer to support; the Printer ignores the attribute if not supported

| Operation Attribute | Cancel-Job | Cancel-Current-Job | Hold-Job | Release-Job |  | Pause-Current-Job | Resume-Job | Get-Job-Attributes | Set-Job-Attributes | Restart-Job | Reprocess-Job | Promote-Job |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| printer-uri |  | R |  |  |  | R |  |  |  |  |  |  |
| printer-uri/job-id or job-uri | R |  | R | R |  |  | R | R | R | R | R | R |
| job-id |  | R |  |  |  | R |  |  |  |  |  |  |
| requested-attributes |  |  |  |  |  |  |  | R |  |  |  |  |
| back-space |  |  |  |  |  |  |  |  |  |  |  |  |
| forward-space |  |  |  |  |  |  |  |  |  |  |  |  |
| synchronize |  |  |  |  |  | O |  |  |  |  |  |  |
| when |  |  |  |  |  | O |  |  |  |  |  |  |
| job-message-from-operator | O | O | O | O |  | O | O |  |  | O | O | O |
| message [to-operator] | O |  | O | O |  | O | O |  |  | O | O | O |
| job-hold-until |  |  | O* | O* |  |  |  |  |  |  | O** |  |

*    The Printer MUST support the "job-hold-until" operation attribute if it supports the "job-hold-until" Job Template attribute.

**  The Printer MUST support the "job-hold-until" operation attribute if it supports the Set-Job-Attributes operation, so that the client can hold the job with the Reprocess-Job operation and the modify the job before releasing it to be processed.

# New Printer Description Attributes

The following new Printer Description attributes are needed to support the new operations defined in this document.

## printer-settable-attributes (1setOf type2 keyword)

Type of registration:  attribute

Proposed keyword name of this attribute:  printer-settable-attributes

Types of attribute (Operation, Job Template, Job Description, Printer Description): Printer Description

Operations to be used with if the attribute is an operation attribute:  N/A

Object (Job, Printer, etc. if bound to an object):  Printer

Attribute syntax(es) (include 1setOf and range as in Section 4.2):  1setOf type2 keyword

If attribute syntax is 'keyword' or 'enum', is it type2 or type3:  type2

If this is a Printer attribute, MAY the value returned depend on "document-format" (See Section 6.2):  Yes

If this is a Job Template attribute, how does its specification depend on the value of the "multiple-document-handling" attribute:  N/A

Specification of this attribute (follow the style of IPP Model Section 4.2):

4.4.? printer-settable-attributes (1setOf type2 keyword)

- This READ-ONLY Printer attribute identifies the Printer object attributes that are settable in this implementation, i.e., that are settable using the Set-Printer-Attributes operations.  This attribute MUST be supported if the Set-Printer-Attributes operations is supported.  The Printer MUST reject attempts to set any Printer attributes that are not in this list, returning the 'client-error-attributes-not-settable' status code (see section 6.1).  The value of this attribute MAY depend on the value of the "document-format" operation attribute supplied in the Get-Printer-Attributes operation (see [ipp-mod] section 3.2.5.1).

## job-settable-attributes (1setOf type2 keyword)

Type of registration:  attribute

Proposed keyword name of this attribute:  job-settable-attributes

Types of attribute (Operation, Job Template, Job Description, Printer Description): Printer Description

Operations to be used with if the attribute is an operation attribute:  N/A

Object (Job, Printer, etc. if bound to an object):  Printer

Attribute syntax(es) (include 1setOf and range as in Section 4.2):  1setOf type2 keyword

If attribute syntax is 'keyword' or 'enum', is it type2 or type3:  type2

If this is a Printer attribute, MAY the value returned depend on "document-format" (See Section 6.2):  Yes

If this is a Job Template attribute, how does its specification depend on the value of the "multiple-document-handling" attribute:  N/A

Specification of this attribute (follow the style of IPP Model Section 4.2):

4.4.? job-settable-attributes (1setOf type2 keyword)

This READ-ONLY Printer attribute identifies the Job object attributes that are settable in this implementation, i.e., that are settable using the Set-Job-Attributes operations.  This attribute MUST be supported if the Set-Job-Attributes operations is supported.  The Printer MUST reject attempts to set any Job attributes that are not in this list, returning the 'client-error-attributes-not-settable' status code (see section 6.1).

## printer-controls-other-protocols (boolean)

Type of registration:  attribute

Proposed keyword name of this attribute:  printer-controls-other-protocols

Types of attribute (Operation, Job Template, Job Description, Printer Description): Printer Description

Operations to be used with if the attribute is an operation attribute:  N/A

Object (Job, Printer, etc. if bound to an object):  Printer

Attribute syntax(es) (include 1setOf and range as in Section 4.2):  boolean

If attribute syntax is 'keyword' or 'enum', is it type2 or type3:  N/A

If this is a Printer attribute, MAY the value returned depend on "document-format" (See Section 6.2):  No

If this is a Job Template attribute, how does its specification depend on the value of the "multiple-document-handling" attribute:  N/A

Specification of this attribute (follow the style of IPP Model Section 4.2):

4.4.? printer-controls-other-protocols (boolean)

This Printer attribute indicates whether all IPP printer and job operations, such as Disable-Printer, Pause-Printer, Cancel-Job, etc., affect non-IPP protocols that may be supported.  It is RECOMMENDED that IPP control other non-IPP protocols.  However, this attribute permits an implementation to indicate explicitly whether it does affect other protocols or not.

A 'false' value indicates that IPP operations only affect jobs submitted by the IPP Protocol.  For example, a 'true' value indicates that a Disable-Printer operation prevents all protocols from submitting jobs, not just the IPP protocol.  A 'true' value indicates that the Pause-Printer operation would pause the current job, no matter what job submission protocol had submitted it.  A 'true' value indicates that the Cancel-Job operation will cancel the specified job even if it was submitted using a different job submission protocol, such as LPR.

As with any Printer Description attribute that this specification does not list as READ-ONLY, an implementation MAY allow a client to change the value of this attribute using Set-Printer-Attributes, thereby changing the way that the IPP operations affect other non-IPP protocols.  An implementation MAY also support other means to change the value of this attribute, such as via the console or at installation time.

## printer-message-time (integer(MIN:MAX))

Type of registration:  attribute

Proposed keyword name of this attribute:  printer-message-time

Types of attribute (Operation, Job Template, Job Description, Printer Description):  Printer Description

Operations to be used with if the attribute is an operation attribute:  N/A

Object (Job, Printer, etc. if bound to an object):  Printer

Attribute syntax(es) (include 1setOf and range as in Section 4.2):  integer(MIN:MAX)

If attribute syntax is 'keyword' or 'enum', is it type2 or type3:  N/A

If this is a Printer attribute, MAY the value returned depend on "document-format" (See Section 6.2):  No

If this is a Job Template attribute, how does its specification depend on the value of the "multiple-document-handling" attribute:  N/A

Specification of this attribute (follow the style of IPP Model Section 4.2):

4.4.? printer-message-time (integer(MIN:MAX))

This READ-ONLY Printer Description attribute contains the time that the Printer's "printer-message-from-operator" was changed by the operator using any operation where the client supplied the "printer-message-from-operator" operation attribute (see section 3.1) or was explicitly set using the Set-Printer-Attributes operation (see section 8.1.1).  This attribute allows the users to know when the "printer-message-from-operator" attribute was last set.

The Printer sets the value of this attribute by copying the value of the Printer's "printer-up-time" attribute (see [ipp-mod] section 4.3.14).  If the Printer resets its "printer-up-time" attribute to 1 on power-up, then it MUST change the value of the "printer-message-time" to 0 or a negative number as specified in [ipp-mod] section 4.3.14.

Note:  This attribute helps users better understand the context for the "printer-message-from-operator" message.

## printer-message-date-time (dateTime)

Type of registration:  attribute

Proposed keyword name of this attribute:  printer-message-date-time

Types of attribute (Operation, Job Template, Job Description, Printer Description):  Printer Description

Operations to be used with if the attribute is an operation attribute:  N/A

Object (Job, Printer, etc. if bound to an object):  Printer

Attribute syntax(es) (include 1setOf and range as in Section 4.2):  dateTime

If attribute syntax is 'keyword' or 'enum', is it type2 or type3:  N/A

If this is a Printer attribute, MAY the value returned depend on "document-format" (See Section 6.2):  No

If this is a Job Template attribute, how does its specification depend on the value of the "multiple-document-handling" attribute:  N/A

Specification of this attribute (follow the style of IPP Model Section 4.2):

4.4.? printer-message-date-time (dateTime)

This READ-ONLY Printer Description attribute contains the date and time that the Printer's "printer-message-from-operator" was changed by the operator using any operation where the client supplied the "printer-message-from-operator" operation attribute (see section 3.1) or was explicitly set using the Set-Printer-Attributes operation (see section 8.1.1).  This attribute allows the users to know when the "printer-message-from-operator" attribute was last set.  This attribute MUST be supported if the Printer supports both the "printer-message-from-operator" and the "printer-current-date-time" attributes.

Note:  This attribute helps users better understand the context for the "printer-message-from-operator" message.

## printer-message-operation (type2 keyword)

Type of registration:  attribute

Proposed keyword name of this attribute:  printer-message-operation

Types of attribute (Operation, Job Template, Job Description, Printer Description):  Printer Description

Operations to be used with if the attribute is an operation attribute:  N/A

Object (Job, Printer, etc. if bound to an object):  Printer

Attribute syntax(es) (include 1setOf and range as in Section 4.2):  type2 enum

If attribute syntax is 'keyword' or 'enum', is it type2 or type3:  type2

If this is a Printer attribute, MAY the value returned depend on "document-format" (See Section 6.2):  No

If this is a Job Template attribute, how does its specification depend on the value of the "multiple-document-handling" attribute:  N/A

Specification of this attribute (follow the style of IPP Model Section 4.2):

4.4.? printer-message-operation (type2 enum)

This READ-ONLY Printer Description attribute contains the operation that was used to changed the Printer's "printer-message-from-operator" by the operator using any operation where the client supplied the "printer-message-from-operator" operation attribute (see section 3.1) or explicitly set using the Set-Printer-Attributes operation (see section 8.1.1).  This attribute allows the users to know which operation was used to change the "printer-message-from-operator" attribute when it was last set.

Note:  This attribute helps users better understand the context for the "printer-message-from-operator".

## subordinate-printers-supported (1setOf uri)

This Printer attribute is REQUIRED if an implementation supports subordinate Printers (see section 2.2) and contains the URIs of the subordinate Printer objects associated with this Printer object.  In order to support subordinate Printer objects, there MUST be at least two subordinate Printer objects.

The precise format of the subordinate Printer URIs is implementation dependent (see section 2.2).

If the Printer object does not have an associated Device object (see section 2.3), the value of the subordinate Printer object's "printer-name" MAY be used to populate the Job object's  "output-device-assigned" attribute (see [ipp-mod] section 4.3.13).  The "output-device-assigned" Job attribute identifies the output device to which the Printer object has assigned a job, for example, when a single Printer object is supporting fan-out.

# Additional values for "printer-state-reasons" and "job-state-reasons" attributes

The following values are added to the "printer-state-reasons" and "job-state-reasons" for use with the operations defined in this document.

## Value for "printer-state-reasons":  'standby'

Type of registration:  type2 keyword attribute value

Name of attribute to which this keyword specification is to be added:  printer-state-reasons

Proposed keyword name of this keyword value:  standby

Specification of this keyword value (follow the style of IPP Model Section 4.1.2.3):

'standby':  The Printer has been shutdown in standby mode.  Only Restart-Printer and Get-Printer-Attributes operations are accepted in this state;  all other operations are rejected with the 'server-error-printer-is-in-standby-mode'.

## Value for "job-state-reasons":  'job-paused'

Type of registration:  type2 keyword attribute value

Name of attribute to which this keyword specification is to be added:  job-state-reasons

Proposed keyword name of this keyword value:  job-paused

Specification of this keyword value (follow the style of IPP Model Section 4.1.2.3):

'job-paused':  The job has been paused while processing using the Pause-Current-Job operations and other jobs can be processed on the Printer.  The Job can be resumed using the Resume-Job operation which removes this value.

# New status codes

This section defines new status codes used by the operations defined in this document.

## New status code: 'client-error-attributes-not-settable'

Type of registration:  status code

Keyword symbolic name of this status code value:  'client-error-attributes-not-settable'

Numeric value (to be assigned by the IPP Designated Expert in consultation with IANA):

Operations that this status code may be used with:  Set-Printer-Attributes, Set-Job-Attributes

Specification of this status code (follow the style of IPP Model Section 13 APPENDIX B:  Status Codes and Suggested Status Code Messages):

#### 13.1.4.20 client-error-attributes-not-settable (0x0413)

In a Set-Printer-Attributes or Set-Job-Attributes request, if the Printer object does not support one or more attributes as settable, the Printer object MUST return this status code.  The Printer object MUST also return in the Unsupported Attributes Group all the attributes and/or values supplied by the client that are not settable.  See [ipp-mod] section 3.1.7.  For example, if the request indicates 'job-state', all implementations MUST reject the request.  As another example, if the request indicates an attribute that is supported, but not settable by this implementation, such as, say, "printer-name", the implementation rejects the request.

## New status code: 'server-error-printer-is-in-standby-mode'

Type of registration:  status code

Keyword symbolic name of this status code value:  'server-error-printer-is-in-standby-mode'

Numeric value (to be assigned by the IPP Designated Expert in consultation with IANA):

Operations that this status code may be used with:  Shutdown-Printer

Specification of this status code (follow the style of IPP Model Section 13 APPENDIX B:  Status Codes and Suggested Status Code Messages):

13.1.5.8 server-error-printer-is-in-standby-mode

The Printer has been shutdown and is only accepting the Restart-Printer (see section 8.1.5) and Get-Printer-Attributes operations.  An operator can perform the Restart-Printer operation to allow the Printer to accept other operations.

# New out-of-band values

This section defines additional out-of-band values that can be used with any attribute in principle.  See the beginning of [ipp-mod] section 4.1.

## 'not-settable' out of band value

The 'not-settable' out of band value is used by the IPP Printer in a Set-Job-Attributes or Set-Printer-Attributes response to indicate an attribute that was supplied, but is not settable in that implementation or as defined by the policy for that system, because it is not in the "job-settable-attributes" on the one hand, or the "interpreter-settable-attributes" or "printer-settable-attributes", on the other hand.

# Summary of Set 2 operations and Operation-Id Assignments

The Set 2 operations are summarized in the following table:

| Operation Name | Operation-Id | Brief description |
| --- | --- | --- |
| Set-Printer-Attributes | 0x?? | Sets attribute values of the target Printer object |
| Enable-Printer | 0x?? | Allows the target Printer to accept create job operations |
| Disable-Printer | 0x?? | Prevents the target Printer from accepting create job operations |
| Restart-Printer | 0x?? | Restarts the target Printer from a standby shutdown mode |
| Shutdown-Printer | 0x?? | Shuts down the target Printer in several ways |
| Set-Job-Attributes | 0x?? | Sets attribute values of the target Job object |
| Reprocess-Job | 0x?? | Creates a copy of a completed target job with a new Job ID and processes it |
| Cancel-Current-Job | 0x?? | Cancels the current job on the target Printer or the specified job if it is the current job |
| Pause-Current-Job | 0x?? | Pauses the current processing job on the target Printer or the specified job if it is the current job, allowing other jobs to be processed instead |
| Resume-Job | 0x?? | Resume the paused target job |
| Promote-Job | 0x?? | Promote the pending target job to be next after the current job(s) complete |

All of the operations in this registration proposal specification are OPTIONAL for an IPP object to support.  Unless the specification of an OPTIONAL operation requires support of another OPTIONAL operation, conforming implementations may support any combination of these operations.

## Printer Operations

All Printer operations are directed at Printer objects.  A client MUST always supply the "printer-uri" operation attribute in order to identify the correct target of the operation.  These descriptions assume all of the common semantics of IPP/1.1 Model and Semantics document [ipp-mod] section 3.1.

### Set-Printer-Attributes Operation

Type of registration:  operation

Proposed name of this operation:  Set-Printer-Attributes

Object Target:  Printer

Specification of this operation:

This OPTIONAL operation allows a client to set the values of the attributes of a Printer object.   In the request, the client supplies the set of Printer attribute names and values that are to be set.  In the response, the Printer object returns success or rejects the request with indications of which attribute or attributes could not be set.

How the Printer object validates the client-supplied attributes in the Set-Printer-Attributes request is implementation-dependent, since there are no corresponding Printer attributes that specify the allowed values that may be set on the Printer object.

The Printer MUST accept this operation in any state, i.e., for any of the values of the Printer object's "printer-state" attribute.

#### Settable and READ-ONLY Printer Description attributes

If the Printer supports the "printer-message-from-operator" Printer Description attribute (see [ipp-mod] section 4.4.25) and the client explicitly supplies a new value for the "printer-message-from-operator" in the request, then the Printer MUST set the "printer-message-from-operator" Printer attribute to this new value and MUST also set the "printer-message-time", "printer-message-date-time", and "printer-message-operation" attributes, if supported (see sections 4.5, 4.6, and 4.7).

If the Printer supports the Set-Printer-Attributes operation, then it SHOULD support setting of:

all Job Template Default ("xxx-default") attributes

all Job Template Supported ("xxx-supported") attributes

all Job Template Ready ("xxx-ready") attributes

that the implementation supports (see [ipp-mod] section 4.2 and extensions).

The following Printer Description attributes (see [ipp-mod] section 4.4) MUST NOT be settable, i.e., they are READ-ONLY:

printer-state

printer-state-reasons

printer-state-message

printer-is-accepting-jobs - see Enable-Printer/Disable-Printer

queued-job-count

printer-message-time - see Set-Printer-Attributes when setting "printer-message-from-operator"

printer-message-date-time - see Set-Printer-Attributes when setting "printer-message-from-operator"

printer-message-operation - see Set-Printer-Attributes when setting "printer-message-from-operator"

printer-up-time

The remaining Printer Description attributes MAY be settable using the Set-Printer-Attributes operation, depending on implementation.  If "xxx-supported" Printer Description attribute are settable, then they MUST affect the behavior of the implementation.  If they are READ-ONLY then they reflect the implementation and cannot be changed using the Set-Printer-Attributes operation.  Consider the following examples:

For example, if the "operations-supported" Printer Description attribute (see [ipp-mod] section 4.4.15) is settable, then changing its value MUST affect the operations that the implementation accepts or rejects.  Such an implementation will need to be able to reject values for operations that it contains no code support for.

As another example, if the "ipp-versions-supported" Printer Description attribute (see [ipp-mod] section 4.4.14) is settable, then changing its value MUST affect the protocol versions that are accepted or rejected.  Such an implementation will need to be able to reject values for operations that it contains no code support for.

See the "factory-settings" operation attribute (see section 3.3) for a way to query the implementation supported values using the Get-Printer-Attributes operation.  See the "reset-function" operation attribute of the Reset-Device operation (see [ipp-set3]) for a way to restore the values of the Device object to the factory settings.

ISSUE 04 - Ok not to have a way to reset the values of the Printer object to the factory settings, but only have a way to reset the factory default settings for the Device object?

Access Rights: Authentication and access control (see [ipp-mod] sections 1, 8.3, and 8.5) apply to this operation.

Most Printer attributes will require administrator privileges to set, such as "xxx-supported", while some will require operator privileges only, such as "media-ready" and "printer-message-from-operator".  Which attributes require which privileges depends on implementation and MAY depend on site policy.

#### Set-Printer-Attributes Request

The following sets of attributes are part of the Set-Printer-Attributes Request:

Group 1: Operation Attributes

Natural Language and Character Set:

The "attributes-charset" and "attributes-natural-language" attributes as described in [ipp-mod] section 3.1.4.1.

Target:

The "printer-uri" (uri) operation attribute which is the target for this operation as described in [ipp-mod] section 3.1.5.

Requesting User Name:

The "requesting-user-name" (name(MAX)) attribute SHOULD be supplied by the client as described in [ipp-mod] section 8.3.

"document-format" (mimeMediaType):

The client SHOULD supply this attribute.  The Printer object MUST support this attribute.  This attribute is useful for a client to select the Interpreter object to which the attribute modification should be applied.  Each Printer object is modeled to contain one or more Interpreter objects.  Those Printer attributes whose values vary from Interpreter to Interpreter, are modeled as Interpreter objects, while those that do not are Printer object attributes.  Thus the target of a Get-Printer-Attributes or Set-Printer-Attributes operation is either the Printer object or the Interpreter object identified by the "document-format" operation attribute supplied by the client.  Except for Get-Printer-Attributes and Set-Printer-Attributes, there are no other operations with the Interpreter object as a target.  See [ipp-mod] section 3.2.5.1 "Get-Printer-Attributes Request".

If a client wants to set an attribute of all of the Interpreter objects to the same value, it can query the Printer's "document-format-supported" Printer Description attribute and perform separate Set-Printer-Attributes for each document format supported.

If the Printer object does not distinguish between different sets of supported values for each different document format when validating jobs in the create and Validate-Job operations, it MUST NOT distinguish between different document formats in the Set-Printer-Attributes operation.  If the Printer object does distinguish between different sets of supported values for each different document format specified by the client, this specialization applies only to the same Printer attributes as the Get-Printer-Attributes operation (see [ipp-mod] section 3.2.5.1).

If the client omits this "document-format" operation attribute, the Printer object MUST respond as if the attribute had been supplied with the value of the Printer object's "document-format-default" attribute.  It is recommended that the client always supply a value for "document-format", since the Printer object's "document-format-default" may be 'application/octet-stream', in which case the set attributes and values are for the union of the document formats that the Printer can automatically sense.  For more details, see the description of the 'mimeMediaType' attribute syntax in [ipp-mod] section 4.1.9.

If the client supplies a value for the "document-format" Operation attribute that is not supported by the Printer, i.e., is not among the values of the Printer object's "document-format-supported" attribute, the Printer object MUST reject the operation and return the 'client-error-document-format-not-supported' status code.

Group 2: Printer Attributes

The client MUST supply a set of Printer attributes as defined in [ipp-mod] section 4.2 Job Template Attributes ("xxx-default", "xxx-supported", and "xxx-ready" attributes) and section 4.4 Printer Description Attributes.  Each Printer attribute supplied in Group 2 replaces the value(s) of the corresponding Printer attribute on the target Printer object.  If a Printer object attribute had not been configured yet and so had the 'no-value' out-of-band value (see [ipp-mod] 4.1), the supplied value(s) replace the 'no-value' value.  For attributes that can have multiple values (1setOf), all values supplied by the client replace all values of the corresponding Printer object attribute.

#### Set-Printer-Attributes Response

The Printer object returns the following sets of attributes as part of the Get-Printer-Attributes Response:

Group 1: Operation Attributes

Status Message:

In addition to the REQUIRED status code returned in every response, the response OPTIONALLY includes a "status-message" (text(255)) and/or a "detailed-status-message" (text(MAX)) operation attribute as described in [ipp-mod] sections 13 and 3.1.6.

Natural Language and Character Set:

The "attributes-charset" and "attributes-natural-language" attributes as described in [ipp-mod] section 3.1.4.2.

Group 2: Unsupported Attributes

See [ipp-mod] section 3.1.7 for details on returning Unsupported Attributes.

In the case of attributes that are supported, but are not settable by the implementation, i.e., are not among the values of the Printer's "settable-attributes" Printer Description attribute (see section 4.1), the Printer object returns the client-supplied attribute(s) with a substituted value of 'not-supported' (same out-of-band value as for attributes that are not supported).  This value's syntax type is "out-of-band" and its encoding is defined by special rules for "out-of-band" values in the "Encoding and Transport" document [IPP-PRO].  Its value indicates that the attribute is either not settable or is not supported at all.

### Disable-Printer Operation

Type of registration:  operation

Proposed name of this operation:  Disable-Printer

Object Target:  Printer

Specification of this operation:

This OPTIONAL operation allows a client to stop the Printer object from accepting jobs, i.e., cause the Printer to reject subsequent create job operations (Print-Job, Print-URI, and Create-Job operation) and return the 'server-error-not-accepting-jobs' status code.  The Printer still accepts all other operations.  All previously submitted Jobs and currently processing Jobs continue unaffected.  The Printer sets the value of its "printer-is-accepting-jobs" READ-ONLY Printer Description attribute to 'false' (see [ipp-mod] section 4.4.20), no matter what the previous value was.  This operation has no immediate effect on the Printer's "printer-state" and "printer-state-reasons" attributes.

Note:  Use the Enable-Printer operation (section 8.1.3) to enable a Printer to accept Jobs again.

If the Disable-Printer operation is supported, then the Enable-Printer operation MUST be supported, and vice-versa.

Note:  Use the Enable-Printer and Disable-Printer operations to allow or prevent input to a Printer.  Use the Pause-Printer and Resume-Printer operations to prevent or allow output from the Printer.

Whether or not the Disable-Printer operation stops jobs that are submitted using job submission protocols other than IPP, depends on implementation, i.e., on whether the IPP protocol is being used as a universal management protocol or just to manage IPP jobs, respectively.  See "printer-controls-other-protocols"  (section 4.4).

Access Rights: Authentication and access control (see [ipp-mod] sections 1, 8.3, and 8.5) apply to this operation.

The Disable-Printer Request and Disable-Printer Response have the same attribute groups and attributes as the Resume-Printer operation (see [ipp-mod] sections 3.2.7.1 and 3.2.7.2), including the new "printer-message-from-operator" operation attribute (see section 3.1), and with the addition of the following Group 1 operation attribute in the request:

"job-type" (type2 keyword):

The client OPTIONALLY supplies this attribute.  The Printer object OPTIONALLY supports this attribute.  The value of this attribute indicates the type of job to be disabled.  If the client omits this attribute, the Printer assumes the 'network-jobs' value.

Standard keyword values are:

'network-jobs' - disable jobs submitted using the create job operations.

'walk-up-jobs' - disable jobs submitted locally, such as walkup copy jobs

'all-jobs' - disable all type of jobs

### Enable-Printer Operation

Type of registration:  operation

Proposed name of this operation:  Enable-Printer

Object Target:  Printer

Specification of this operation:

This OPTIONAL operation allows a client to start the Printer object accepting jobs, i.e., cause the Printer to accept subsequent create job operations (Print-Job, Print-URI, and Create-Job operation).  The Printer still accepts all other operations.  All previously submitted Jobs and currently processing Jobs continue unaffected.  The Printer sets the value of its "printer-is-accepting-jobs" READ-ONLY Printer Description attribute to 'true' (see [ipp-mod] section 4.4.20), no matter what the previous value was.  This operation has no immediate effect on the Printer's "printer-state" and "printer-state-reasons" attributes.

Note:  Use the Disable-Printer operation (section 8.1.2) to stop a Printer from accepting Jobs.

If the Enable-Printer operation is supported, then the Disable-Printer operation MUST be supported, and vice-versa.

Note:  Use the Enable-Printer and Disable-Printer operations to allow or prevent input to a Printer.  Use the Pause-Printer and Resume-Printer operations to prevent or allow output from the Printer.

Whether or not the Enable-Printer operation allows acceptance of  jobs that are submitted using job submission protocols other than IPP, depends on implementation, i.e., on whether the IPP protocol is being used as a universal management protocol or just to manage IPP jobs, respectively.  See "printer-controls-other-protocols" (section 4.4).

Access Rights: Authentication and access control (see [ipp-mod] sections 1, 8.3, and 8.5) apply to this operation.

The Enable-Printer Request and Enable-Printer Response have the same attribute groups and attributes as the Resume-Printer operation (see [ipp-mod] sections 3.2.8.1 and 3.2.8.2), including the new "printer-message-from-operator" operation attribute (see section 3.1), and with the addition of the following Group 1 operation attribute in the request:

"job-type" (type2 keyword):

The client OPTIONALLY supplies this attribute.  The Printer object OPTIONALLY supports this attribute.  The value of this attribute indicates the type of job to be enabled.  If the client omits this attribute, the Printer assumes the 'network-jobs' value.

Standard keyword values are:

'network-jobs' - enable jobs submitted using the create job operations.

'walk-up-jobs' - enable jobs submitted locally, such as walkup copy jobs

'all-jobs' - enable all types of jobs

### Restart-Printer operation

Type of registration:  operation

Proposed name of this operation:  Restart-Printer

Object Target:  Printer

Specification of this operation:

This OPTIONAL operation allows a client to restart a Printer that has previously been shutdown in standby mode (see section 8.1.7).  Standby mode is indicated by the Printer's "printer-state" being 'stopped' and its "printer-state-reasons" including the 'standby' and 'paused' values.  If the Printer is not in standby mode, the Printer MUST reject this operation and return the 'client-error-not-possible' status code.

ISSUE 05:   What state does the Printer comes back up on Restart-Printer and how can the client control?
Possible alternatives:

a. Restart-Printer always brings the Printer up Disabled ("printer-is-accepting-jobs" = 'false') and Paused ("printer-state" = 'stopped', and "printer-state-reasons" = 'paused') which is the state that Shutdown-Printer leaves the Printer in.  Then the operator issues Enable-Printer and Resume-Printer when want to restore normal operation.  The client can automatically issues these addition 2 operations depending on GUI options.  Advantages:  This is the simplest to implement, allows new states to be added without changing the Restart-Printer operation, and can have the same GUI interface as b:

b. Add a REQUIRED operation attribute to Restart-Printer, something like "printer-condition" with values: 'disabled-and-paused', 'enabled-and-paused', and 'enabled-and-idle'.

DENVER MEETING> There was significant confusion as to the meaning of shutdown.  If a shutdown operation halts communications to the printer a restart is not possible.  It was agreed to include restart attributes ‘restart-to-standby’ (requires a two step process), ‘restart-fully’, and ‘restart-sync’.

ACTION ITEM (Harry Lewis):  make a complete proposal and submit for approval.

If the Restart-Printer operation is supported, then the Shutdown-Printer operation MUST be supported, since the Restart-Printer operation is meaningful only after a Shutdown-Printer operation has been performed.  However, if the Shutdown-Printer operation is supported, the Restart-Printer NEED NOT be supported.

Issue 06: Why? This is backward, if you ask me (HRL).

Denver meeting> Consider "shutdown-attributes" similar to Windows "shutdown". Like:

- Completely power off (walk to printer to bring it back up

- Power off but listen for "restart" - shutdown to standby

Denver meeting> Also consider restart attributes.

- Restart to Standby (disabled and paused) (two step bring up)

- Restart (fully).

- Restart sych

Access Rights: Authentication and access control (see [ipp-mod] sections 1, 8.3, and 8.5) apply to this operation.

The Restart-Printer Request and Restart-Printer Response have the same attribute groups and attributes as the Resume-Printer operation (see [ipp-mod] sections 3.2.8.1 and 3.2.8.2), including the new "printer-message-from-operator" operation attribute (see section 3.1) and the following Group 1 operation attribute:

Shutdown-Printer Operation

Type of registration:  operation

Proposed name of this operation:  Shutdown-Printer

Object Target:  Printer

Specification of this operation:

This OPTIONAL operation allows a client to shutdown a Printer, i.e., stop processing jobs and power off in some implementation-dependent manner.  The purpose of Shutdown-Printer is to shutdown the Printer for an extended period, not to reset the device(s) or modify a Printer attribute.  See Reset-Device [ipp-set3] for the way to reset the device(s).  See the Disable-Printer operation (section 8.1.2) for a way for the client to stop the Printer from accepting Job Creation requests without stopping processing or shutting down.

The Printer is disabled immediately (see the Disable-Printer operation in section 8.1.2).  The Printer adds the 'shutdown' value (see [ipp-mod] section 4.4.11) to its "printer-state-reasons" Printer Description attribute.  The "when" operation attribute specifies how much processing occurs before the Printer is paused (see [ipp-mod] section 3.2.7) and the shutdown is complete.  All other requests continue to be accepted until the printer is powered down.

ISSUE 07 - Need to look at life cycle of the Printer versus standby/power-down and the other operations that can be accepted.  There can be appreciable time between acceptance of this operation and when the final state of the printer, either standby or powered down is reached.  Is it ok for non-submission operations to continue to be accepted during this time?  May need 'moving-to-shutdown'.  What about 'moving-to-standby'?

TH>  Add 'moving-to-shutdown' which the Shutdown-Printer sets

immediately (analogous to 'moving-to-paused').  Then the 'shutdown'

values means that the shutdown has completed (and is only meaningful to

a server implementation that hosts the Printer object).  Thus the server

can still respond to a Get-Printer-Attributes operation after the

Printer is shutdown as stated in IPP/1.1.

HRL> Is this granularity really achievable enough of a wide enough

variety of environments to be reliable or, in reality, will this be

implementation dependent?

Whether or not the Shutdown-Printer operation affects jobs that were submitted to the device using job submission protocols other than IPP, depends on implementation, i.e., on whether the IPP protocol is being used as a universal management protocol or just to manage IPP jobs, respectively.  See "printer-controls-other-protocols" (section 4.4).

The Printer object MUST accept this operation in any state and transition the Printer object to the 'idle' state.

Access Rights: Authentication and access control (see [ipp-mod] sections 1, 8.3, and 8.5) apply to this operation.

The Shutdown-Printer Request and Shutdown-Printer Response have the same attribute groups and attributes as the Pause-Printer operation (see [ipp-mod] sections 3.2.7.1 and 3.2.7.2), including the new "printer-message-from-operator" operation attribute (see section 3.1), and with the addition of the following Group 1 operation attributes in the request:

"when" (type2 keyword)

The client OPTIONALLY supplies this attribute.  The Printer object OPTIONALLY supports this attribute, if it supports this operation.  The value of this attribute indicates when to pause the printer.  If the client omits this attribute, the Printer assumes the 'after-current-job' value.  The 'after-current-job' value is REQUIRED to be supported if the "when" attribute is supported; the remaining values are OPTIONAL.

Note:  There is no way to query to see which values of the "when" attribute are supported for this or any other operation.  Since this operation is intended for Operators, rather than End-Users, it did not seem necessary to provide the means to query the supported values.

Standard keyword values are:

'now' - cancel the currently printing job(s) and shutdown the Printer.  Jobs in the 'held' and 'pending' state remain in those states.

'after-current-job' - shutdown the Printer after the current job finishes printing (all its copies).  Jobs in the 'held' and 'pending' state remain in those states.

'after-all' - shutdown the Printer after all 'pending' jobs finish printing.  Jobs in the 'held' state remain in the 'held' state.

ISSUE 08 - Ok to remove the 'after-current-copy' value from Shutdown-Printer?

ISSUE 09 - Ok to have removed the "shutdown-function" operation attribute with values 'stand-by' vs. 'power-off' Shutdown-Printer on the grounds that for the Printer object they have no meaning.  Power-off is for the Shutdown-Device operation.

"synchronize" (boolean)

The client OPTIONALLY supplies this attribute.  The Printer object OPTIONALLY supports this attribute.  This attribute indicates whether or not the printer is to synchronize the checkpoint data for the current job ("when" = 'now') with the pages that have actually printed.  If the value of the "when" attribute is not 'now' or the "when" attribute is not supplied, then the "synchronize" attribute has no meaning and the Printer MUST ignore it.  If this attribute is supported, then a value of 'true' implies that the Printer will be able to resume the job at the point of synchronization when the Printer is restarted.  If the implementation does not support resuming a job (either automatically or with the Resume-Job operation) after a Shutdown-Printer with "when" = 'now', then it does not implement this attribute.  In this case, check-pointing implies that the job may be resumed in the future, exactly from the point and in the state and resource context from which it left off.

If the Printer supports this attribute but the client does not supply it, the Printer is assumed to  perform synchronization ('true' behavior).  If the Printer does not support this attribute, the Printer is assumed to not synchronize ('false' behavior).

ISSUE 10:  Is the current job automatically restarted when the Printer is restarted?  Or does some client have to issue a Restart-Job operation?

## Job Operations

All Job operations are directed at Job objects.  A client MUST always supply some means of identifying the Job object in order to identify the correct target of the operation.  That job identification MAY either be a single Job URI or a combination of a Printer URI with a Job ID.  The IPP object implementation MUST support both forms of identification for every job.

### Set-Job-Attributes

Type of registration:  operation

Proposed name of this operation:  Set-Job-Attributes

Object Target:  Job

Specification of this operation:

This OPTIONAL operation allows a client to set the values of the attributes of a Job object.   In the request, the client supplies the set of Job attribute names and values that are to be set.  In the response, the IPP object returns success or rejects the request with indications of which attribute or attributes could not be set.

This operation is almost identical to the Set-Printer-Attributes operation (see section 8.1.1).  The only differences are that the Set-Job-Attributes operation is directed at a Job object rather than a Printer object, there is no "document-format" operation attribute used when setting a Job object, and the validation is the same as the create job operations, i.e., depends on the "xxx-supported" Printer Description attributes.

The validation of the Set-Job-Attributes request is performed as if the job had been submitted originally with the new values and with "ipp-attribute-fidelity" set to 'true', i.e., all modified attributes MUST be supported along with the attributes not modified.  If such a create job operation would have been accepted, then the Set-Job-Attributes MUST be accepted.  If such a create job operation would have been rejected, then the Set-Job-Attributes MUST be rejected and the Job MUST be unchanged.

The IPP object MUST accept or reject the request based on the job's current state and transition the job to the indicated new state as follows:

| Current "job-state" | New "job-state" | IPP object's response status code and action: |
| --- | --- | --- |
| 'pending' | 'pending' | 'successful-ok' |
| 'pending' | 'pending-held' | 'successful-ok' - needed resources are not ready |
| 'pending-held' | 'pending-held' | 'successful-ok' |
| 'pending-held' | 'pending' | 'successful-ok' - needed resources are ready |
| 'processing' | 'processing' | 'successful-ok' or 'client-error-not-possible' depending on the attributes being set, whether the job has started marking media, and/or implementation |
| 'processing-stopped' | 'processing-stopped' | 'successful-ok' or 'client-error-not-possible' depending on the attributes being set, whether the job has started marking media, and/or implementation |
| 'completed' | 'completed' | 'client-error-not-possible' |
| 'canceled' | 'canceled' | 'client-error-not-possible' |
| 'aborted' | 'aborted' | 'client-error-not-possible' |

#### Settable and READ-ONLY Job Description attributes

If the Printer supports the "job-message-from-operator" Job Description attribute (see [ipp-mod] section 4.3.16) and the client explicitly supplies a new value for the "job-message-from-operator" in the request, then the Printer MUST set the "job-message-from-operator" Job attribute to this new value.

If the Printer supports the Set-Job-Attributes operation, then it SHOULD support setting of:

all Job Template ("xxx") attributes (see [ipp-mod] section 4.2 and extensions)

that the implementation supports.

The following Job Description attributes (see [ipp-mod] section 4.3) MUST NOT be settable, i.e., they are READ-ONLY:

job-uri

job-id

job-printer-uri

job-more-info

job-originating-user-name - set in create operation

job-state

job-state-reasons

job-state-message

number-of-documents

output-device-assigned

time-at-creation

time-at-processing

time-at-completed

job-printer-up-time

date-time-at-creation

date-time-at-processing

date-time-at-completed

number-of-intervening-jobs

job-k-octets

job-k-octets-processed

job-impressions-completed

job-media-sheets-completed

attributes-charset - set in create operation

attributes-natural-language - set in create operation

The remaining Job Description attributes MAY be settable using the Set-Job-Attributes operation, depending on implementation.

Whether or not the Set-Job-Attributes operation affects jobs that are submitted using job submission protocols other than IPP, depends on implementation, i.e., on whether the IPP protocol is being used as a universal management protocol or just to manage IPP jobs, respectively.  See "printer-controls-other-protocols" (section 4.4).

Access Rights: Authentication and access control (see [ipp-mod] sections 1, 8.3, and 8.5) apply to this operation.

#### Set-Job-Attributes Request

The following sets of attributes are part of the Set-Job-Attributes Request:

Group 1: Operation Attributes

Natural Language and Character Set:

The "attributes-charset" and "attributes-natural-language" attributes as described in [ipp-mod] section 3.1.4.1.

Target:

Either (1) the "printer-uri" (uri) plus "job-id" (integer(1:MAX)) or (2) the "job-uri" (uri) operation attribute(s) which define the target for this operation as described in [ipp-mod] section 3.1.5.

Requesting User Name:

The "requesting-user-name" (name(MAX)) attribute SHOULD be supplied by the client as described in [ipp-mod] section 8.3.

Group 2: Job Attributes

The client MUST supply a set of Job attributes as defined in [ipp-mod] section 4.2 Job Template Attributes ("xxx" attributes) and section 4.3 Job Description Attributes.  Each Job attribute supplied in Group 2 replaces the value(s) of the corresponding Job attribute on the target Job object.  For attributes that can have multiple values (1setOf), all values supplied by the client replace all values of the corresponding Job object attribute.

#### Set-Job-Attributes Response

The Printer object returns the following sets of attributes as part of the Set-Job-Attributes Response:

Group 1: Operation Attributes

Status Message:

In addition to the REQUIRED status code returned in every response, the response OPTIONALLY includes a "status-message" (text(255)) and/or a "detailed-status-message" (text(MAX)) operation attribute as described in [ipp-mod] sections 13 and 3.1.6.

Natural Language and Character Set:

The "attributes-charset" and "attributes-natural-language" attributes as described in [ipp-mod] section 3.1.4.2.

Group 2: Unsupported Attributes

See [ipp-mod] section 3.1.7 for details on returning Unsupported Attributes.

The attributes returned are the same as for the create operation with the same new attribute values.  In the case of attributes that are supported, but are not settable by the implementation, i.e., are not among the values of the Printer's "settable-attributes" Printer Description attribute (see section 4.1), the Printer object returns the client-supplied attribute(s) with a substituted value of 'not-supported' (same out-of-band value as for attributes that are not supported).  This value's syntax type is "out-of-band" and its encoding is defined by special rules for "out-of-band" values in the "Encoding and Transport" document [IPP-PRO].  Its value indicates that the attribute is either not settable or is not supported at all.

### Reprocess-Job Operation

This OPTIONAL operation is a create job operation that allows a client to re-process a copy of a job that had been retained in the queue after processing completed, was canceled, or was aborted (see [ipp-mod] section 4.3.7.2).  This operation is the same as the Restart-Job operation (see [ipp-mod] section 3.3.7), except that the Printer creates a new job that is a copy of the target job and the target job is unchanged.  The new job is assigned new values to the "job-uri" and "job-id" attributes and the new job's Job Description attributes that accumulate job progress, such as "job-impressions-completed", "job-media-sheets-completed", and "job-k-octets-processed", are initialized to 0 as with any create job operation.  The target job moves to the Job History after a suitable period, independent of whether one or more Reprocess-Job operations have been performed on it.

If the Set-Job-Attributes operation is supported, then the "job-hold-until" operation attribute MUST be supported with at least the 'indefinite' value, so that a client can modify the new job before it is scheduled for processing using the Set-Job-Attributes operation.  After modifying the job, the client can release the job for processing, by using the Release-Job operation specifying the newly assigned "job-uri" or "job-id" for the new job.

### Cancel-Current-Job Operation

This OPTIONAL operation allows a client to cancel the current job on the target Printer or the specified job if it is the current job on the Printer.  See [ipp-mod] section 3.3.3 for the semantics of canceling a job.  Since a Job might already be marking by the time a Cancel-Current-Job is received, some media sheet pages might be printed before the job is actually terminated.

If the client does not supply a "job-id" operation attribute, the Printer MUST accept the request and cancel the current job if there is a current job in the 'processing' or 'processing-stopped' state; otherwise, it MUST reject the request and return the 'client-error-not-possible' status code.  If more than one job is in the 'processing' or 'processing-stopped' states, the one that is marking is canceled and the others are unaffected.

Warning:  On a shared printer, there is a race condition.  Between the time that a user issues this operation and its acceptance, the current job might change to a different job.  If the user or operator is authenticated to cancel the new job, the wrong job is canceled.  To prevent this race from canceling the wrong job, the client MAY supply the "job-id" operation attribute which is checked against the current job's job-id.  If the job identified by the "job-id" attribute is not the current job on the Printer, i.e., is not in the 'processing' or 'processing-stopped' states, the Printer MUST reject this operation and return the 'client-error-not-possible' status code.  Otherwise, the Printer cancels the specified job.

Access Rights: Authentication and access control (see [ipp-mod] sections 1, 8.3, and 8.5) apply to this operation.

The Cancel-Current-Job Request and Cancel-Current-Job Response have the same attribute groups and attributes as the Resume-Printer operation (see [ipp-mod] section 3.2.8), including the new "job-message-from-operator" operation attribute (see section 3.2), with the addition of the following Group 1 Operation attributes in the request:

"job-id" (integer(1:MAX)):

The client OPTIONALLY supplies this Operation attribute in order to verify that the identified job is still the current job on the target Printer object.  The IPP object MUST supports this operation attribute, if it supports this operation.

### Pause-Current-Job operation

ISSUE 11 - Since the semantics of Pause-Current-Job is different from Pause-Printer, in that other jobs are processed, while this job is stopped, should we have a different name for the verb, such as Suspend-Current-Job, instead of Pause-Current-Job.  Otherwise, users may be mistakenly think that the printer is paused when the job is paused.  If the name is changes, then the 'job-paused' job state reason would also be changed to 'job-suspended'.

This OPTIONAL operation allows a client to stop the current job on the target Printer or the specified job if it is the current job on the Printer, and allow other jobs to be processed instead.  The Printer moves the current job or the target job to the 'processing-stopped' state and sets the 'job-paused' value in the job's "job-state-reasons" attribute and processes other jobs.

If the client does not supply a "job-id" operation attribute, the Printer MUST accept the request and pause the current job if there is a current job in the 'processing' or 'processing-stopped' state; otherwise, it MUST reject the request and return the 'client-error-not-possible' status code.  If more than one job is in the 'processing' or 'processing-stopped' states, all of them are paused.

Warning:  On a shared printer, there is a race condition.  Between the time that a user issues this operation and its acceptance, the current job might change to a different job.  If the user or operator is authenticated to pause the new job, the wrong job is paused.  To prevent this race from pausing the wrong job, the client MAY supply the "job-id" operation attribute which is checked against the current job's job-id.  If the job identified by the "job-id" attribute is not the current job on the Printer, i.e., is not in the 'processing' or 'processing-stopped' states, the Printer MUST reject this operation and return the 'client-error-not-possible' status code.  Otherwise, the Printer pauses the specified job and processed other jobs.

A paused job is resumed using the Resume-Job operation (see section 8.2.5).  If the Pause-Current-Job operation is supported, then the Resume-Job operation MUST be supported, and vice-versa.

The Printer MUST reject a Release-Job request (and return the 'client-error-not-possible') for a job that has been paused , i.e., for a job in the 'processing-stopped' state, with the 'job-paused' value in its "job-state-reasons" attribute.  The Hold-Job and Release-Job operations are for holding and releasing held jobs, not pausing and resuming paused jobs.

ISSUE 12 - Ok for Pause-Job to reject a request for a Job that is already paused (like Cancel-Job), instead of accepting the request?

Access Rights: Authentication and access control (see [ipp-mod] sections 1, 8.3, and 8.5) apply to this operation.

The Pause-Current-Job Request and Pause-Current-Job Response have the same attribute groups and attributes as the Resume-Printer operation (see [ipp-mod] section 3.2.8 ), including the new "job-message-from-operator" operation attribute (see section 3.2), with the addition of the following Group 1 Operation attributes in the request:

"job-id" (integer(1:MAX)):

The client OPTIONALLY supplies this Operation attribute in order to verify that the identified job is still the current job on the target Printer object.  The IPP object MUST supports this operation attribute, if it supports this operation.

"when" (type2 keyword)

The client OPTIONALLY supplies this attribute.  The Printer object OPTIONALLY supports this attribute, if it supports this operation.  The value of this attribute indicates when to pause the printer.  If the client omits this attribute, the Printer assumes the 'after-current-copy' value.  The 'after-current-copy' value is REQUIRED to be supported if the "when" attribute is supported; the remaining values are OPTIONAL.

Note:  There is no way to query to see which values of the "when" attribute are supported for this or any other operation.  Since this operation is intended for Operators, rather than End-Users, it did not seem necessary to provide the means to query the supported values.

Standard keyword values are:

'now' - pause the current Job immediately, while the current Job is processing.

'after-current-copy' - pause the current Job after the current Job finishes printing its current copy.

ISSUE 13 - Ok that the only values for the "when" operation attribute for the Pause-Current-Job are 'now' and 'after-current-copy' with 'now' being the default and REQUIRED?

"synchronize" (boolean)

The client OPTIONALLY supplies this attribute.  The Printer object OPTIONALLY supports this attribute.  This attribute indicates whether or not the printer is to synchronize the checkpoint data for the current job being paused with the pages that have actually printed.  If this attribute is supported, then a value of 'true' implies that the Printer will be able to resume the job at the point of synchronization when the job is resumed.

If the Printer supports this attribute but the client does not supply it, the Printer is assumed to perform synchronization ('true' behavior).  If the Printer does not support this attribute, the Printer is assumed to not synchronize ('false' behavior).

### Resume-Job operation

This OPTIONAL operation allows a client to resume the target job at the point where it was paused.  The Printer moves the target job to the 'pending' state and removes the 'job-paused' value from the job's "job-state-reasons" attribute.

If the target job is not in the 'processing-stopped' state with the 'job-paused' value in the job's "job-state-reasons" attribute, the Printer rejects the request and returns the 'client-error-not-possible' status code, since the job was not paused.

ISSUE 14 - Ok to return an error (like Cancel-Job), rather than having Resume-Job accept the request even if the Job is already resumed (i.e., not paused)?

If the Resume-Job operation is supported, then the Pause-Current-Job operation MUST be supported, and vice-versa.

Access Rights: Authentication and access control (see [ipp-mod] sections 1, 8.3, and 8.5) apply to this operation.

The Resume-Job Request and Resume-Job Response have the same attribute groups and attributes as the Release-Job operation (see [ipp-mod] section 3.3.6), including the new "job-message-from-operator" operation attribute (see section 3.2).

### Promote-Job operation

This OPTIONAL operation allows a client to make the pending target job be processed next after the current job completes.  This operation is specially useful in a production printing environment where the operator is involved in job scheduling.

If the target job is in the 'pending' state, this operation does not change the job's state, but causes the job to be processed after the current job(s) complete.  If the target job is not in the 'pending' state, the Printer rejects the request and returns the 'client-error-not-possible' status code.  The Printer returns the target job immediately after the current job(s) in a Get-Jobs response (see [ipp-mod] section 3.2.6) for the 'not-completed' jobs.

When the current job completes, is canceled, paused, or aborted, the target of this operation is processed next.

If a client issues this request (again) before the target of the operation of the original request started processing, the target of this new request is scheduled before the previous job that was to be processed next.

Note:  IPP is specified not to require queues for job scheduling, since there are other implementations for scheduling multiple jobs.  However, if an implementation does implement queues for jobs, then the Promote-Job puts the specified job at the front of the queue.  A subsequent Promote-Job before the first job starts processing puts that specified job at the front of the queue, so that it is "in front" of the previously promoted job.

Access Rights: Authentication and access control (see [ipp-mod] sections 1, 8.3, and 8.5) apply to this operation.

The Promote-Job Request and Promote-Job Response have the same attribute groups and attributes as the Cancel-Job operation (see [ipp-mod] section 3.3.3), including the new "job-message-from-operator" operation attribute (see section 3.2).

# IANA Considerations

The operations and attributes in this registration proposal will be published by IANA according to the procedures in RFC 2566 [rfc2566] section 6.4 for operations with the following URL:

ftp.isi.edu/iana/assignments/ipp/operations/set2.txt

# Internationalization Considerations

This document has the same localization considerations as the [ipp-mod].

# Security Considerations

The IPP Model and Semantics document [ipp-mod] discusses high level security requirements (Client Authentication, Server Authentication and Operation Privacy). Client Authentication is the mechanism by which the client proves its identity to the server in a secure manner. Server Authentication is the mechanism by which the server proves its identity to the client in a secure manner. Operation Privacy is defined as a mechanism for protecting operations from eavesdropping.

# Author's Addresses

Carl Kugler

IBM

Boulder CA

Phone: (303) 924-5060

FAX:

e-mail:  kugler@us.ibm.com

Tom Hastings

Xerox Corporation

737 Hawaii St.  ESAE 231

El Segundo, CA  90245

Phone: 310-333-6413

Fax: 310-333-5514

e-mail: hastings@cp10.es.xerox.com

Harry Lewis

IBM

Boulder CA

Phone: (303) 924-5337

FAX:

e-mail:  harryl@us.ibm.com

# References

[ipp-mod]

R. deBry, T. Hastings, R. Herriot, S. Isaacson, P. Powell, "Internet Printing Protocol/1.0: Model and Semantics", <draft-ietf-ipp-model-v11-03.txt>, June, 1999.

[RFC2566]

R. deBry, T. Hastings, R. Herriot, S. Isaacson, P. Powell, "Internet Printing Protocol/1.0: Model and Semantics", RFC 2566, April 1999.

# Change History

This section summarizes the changes.  Each sub-section is in reverse chronological order.

## Changes to the October 22, 1999 version to make the November 1, 1999 version

Adding or removing ISSUES that don't change the document are not listed here.  The following changes to the October 22, 1999 version to make the November 1, 1999 version as a result of the IPP WG meeting in Durham, 10/99:

- Removed the Reset-Printer, Non-Process-Run-Out, and Space-Current-Job operations from this Set2 spec and moved them to a new Set3 spec for use with the new Device object, renaming them appropriately, to Reset-Device, Non-Process-Run-Out-Device, and Space-Device.

- Add the concept of parent and subordinate Printer objects to formally represent fan-out.  Mentioned the Device object that is in a new [ipp-set3] spec.

- Distributed the definition of the "when" operation attribute to the Pause-Printer (IPP/1.1), Shutdown-Printer, and Pause-Current-Job operations and listed the values that are appropriate to that operation only:  
Pause-Printer: 'now', 'after-current-copy', 'after-current-job' (default), and 'after-all'.
Shutdown-Printer:  'now', 'after-current-job' (default), and 'after-all'
Pause-Current-Job: 'now', 'after-current-copy' (default)

- Deleted the "device-name" operation attribute and the "device-names-supported" (1setOf name(127)) Printer Description attribute.  The latter will be part of the [ipp-set3] document.

- Kept the "job-settable-attributes" (1setOf type2 keyword) and "printer-settable-attributes" (1setOf type2 keyword), but deleted the "interpreter-settable-attributes (1setOf type2 keyword), since the Interpreter object and its attributes are really a sub-class of the Printer object.

- Deleted the "when-values-supported" (1setOf type2 keyword) Printer Description attribute.

- Added the "subordinate-printers-supported" (1setOf uri) Printer Description attribute.

## Changes to the September 19, 1999 version to make the October 22, 1999 version

Adding or removing ISSUES that don't change the document are not listed here.  The following changes to the September 19, 1999 version to make the October 22, 1999 version as a result of the IPP WG meeting in Denver, 9/99:

- Added the Interpreter object.

- Added the "device-name" operation attribute to handle passing operations through the IPP Printer object to the device.

- Added the out-of-band 'not-settable' to allow the Set-Job-Attributes and Set-Printer-Attributes response to indicate the difference between an unsupported attribute and a supported, but not settable, attribute in the Unsupported Attributes Group.

- Removed "when-values-supported" and "job-settable-attributes" and "printer-settable-attributes" and "interpreter-settable-attributes" from the list of attributes that MUST be read-only.  So an administrator could sub-set the policy on what when values are supported or which attributes can be set.

## Changes to the July 19, 1999 version to make the September 19, 1999 version

The following changes to the July 19, 1999 version to make the September 19, 1999 version as a result of the IPP WG meeting in Alaska, 8/99:

- Refer to proposal as "Set2" rather than "Administrative" operations.

- Revise the emphasis on administrator throughout the document, although the word administrator remains wherever appropriate.

- Convert non-process-run-out from an operations attribute to an operation.

- Added  Issue 21: For all these "access" caveats, why not just say... 'authentication and access control (see ipp-mod sections 1, 8.3 and 8.5) applies to this operation".?

- Added Issue 22: Why? This is backward, if you ask me (HRL).

- Per resolution of Issue 2, the "settable-attributes" Printer Description attribute, was replaced with three Printer Description attributes:  "printer-settable-attributes", "job-settable-attributes", and "interpreter-settable-attributes".  The latter for those implementations that have different values for Printer attributes in the Get-Printer-Attributes and Set-Printer-Attributes operations, depending on the value of the "document-format" operation attribute supplied by the client.  If and when we get a Document object, then we can add a "document-settable-attributes" Printer Description attribute.

## Changes to the June 30, 1999 version to make the July 19, 1999 version

The following changes to the June 30, 1999 version to make the July 19, 1999 version as a result of the IPP WG meeting in Copenhagen, 7/7/99-7/8/99, and the IPP telecon, 7/14/1999:

- Sections 2.1 and 2.2:  Clarified that the way to remove a message from the operator was for the client to supply a zero-length or all white space text string which is copied as usual to the "xxx-message-from-operator" attribute.

- Section 2.3:  Added "factory-settings" (boolean) operation attribute to the Get-Printer-Attributes operation.

- Section 2.4:  Added the "when" operation attribute to the Pause-Current-Job operation.

- Section 2.4:  Made the "when" operation attribute OPTIONAL for  use in operations (Pause-Printer, Reset-Printer, Shutdown-Printer, and Pause-Current-Job operations).

- Sections 2.5:  Added table of operation attributes for the Printer operations to make it easy to compare.

- Sections 2.6:  Added table of operation attributes for the Job operations to make it easy to compare.

- Section 3.1:  Added "settable-attributes" (1setOf type2 keyword) READ-ONLY Printer Description attribute.

- Section 3.2:  Added "printer-controls-other-protocols" (boolean) Printer Description attribute

- Section 3.3:  Added the READ-ONLY "printer-message-time" (integer(MIN:MAX)) Printer Description attribute to keep time message updated in time ticks.

- Section 4.2:  Deleted the 'process-next' "job-state-reasons" value, so that repeated Promote-Job operations promote each job "to the front of the queue".

- Sections 6.1.1.1 and 6.2.1.1:  Replaced the table that listed all attributes with one that lists only the attributes that MUST be READ-ONLY.

- Section 6.1.1.1:  Indicated that attributes that are not specified as READ-ONLY in this document MAY be settable.  If they control behavior, that changing their values MUST change the behavior.

- Section 6.1.1.2 and 6.2.1.2:  Deleted the "ipp-attribute-fidelity" operation attribute from the Set-Printer-Attributes and Set-Job-Attributes operations.  All set operations are atomic.

- Section 6.1.1.2:  Add the concept of the Interpreter object to handle attributes whose values vary in the Set-Printer-Attributes and Get-Printer-Attributes, depending on the value of the "document-format" operation attribute.

- Sections 6.1.1.3 and 6.2.1.2:  Changed the "out-of-band" 'not-settable' value back to the existing 'not-supported' value.

- Section 6.1.2 and 6.1.3:  Added "job-type" operation attribute to Disable-Printer and Enable-Printer operations with values: 'network-jobs', 'walk-up-jobs', and 'all-jobs'.

- Section 6.1.5:  Clarified that Restart-Printer brings up the Printer disabled and paused, since that is the eventual state that Shutdown-Printer leaves the printer in.

- Section 6.1.5:  Indicated that if Restart-Printer is supported, then Shutdown-Printer MUST be supported.

- Section 6.1.6:  Deleted Space-Printer operation.  Keep Space-Current-Job operation only which has a "job-id" operation attribute that a client MAY supply.

- Section 6.1.6:  Clarified that Shutdown-Printer is for a long period of time, not just to reset the device or change attribute values.  Also that Shutdown performs an immediate Disable-Printer and an eventual Pause-Printer.

- Sections 6.2.3, 6.2.4, and 6.2.7 :  Added a "job-id" operation attribute to Cancel-Current-Job, Pause-Current-Job, and Space-Current-Job that a client MAY supply to check for race condition where current job changes

- Section 6.2.4:  Combined Pause-Job into Pause-Current-Job operation.

- Sections 6.2.4 and 6.2.5:  Pause-Current-Job puts job in 'processing-stopped' state, not 'pending-held' state.

- Section 6.2.6:  Simplified Promote-Job, so that it behaves as if the job were put at the front of the queue.

# Appendix A: Full Copyright Statement

Copyright (C) The Internet Society (1998,1999). All Rights Reserved

This document and translations of it may be copied and furnished to others, and derivative works that comment on or otherwise explain it or assist in its implementation may be prepared, copied, published and distributed, in whole or in part, without restriction of any kind, provided that the above copyright notice and this paragraph are included on all such copies and derivative works.  However, this document itself may not be modified in any way, such as by removing the copyright notice or references to the Internet Society or other Internet organizations, except as needed for the purpose of developing Internet standards in which case the procedures for copyrights defined in the Internet Standards process must be followed, or as required to translate it into languages other than English.

The limited permissions granted above are perpetual and will not be revoked by the Internet Society or its successors or assigns.

This document and the information contained herein is provided on an "AS IS" basis and THE INTERNET SOCIETY AND THE INTERNET ENGINEERING TASK FORCE DISCLAIMS ALL WARRANTIES, EXPRESS OR IMPLIED, INCLUDING BUT NOT LIMITED TO ANY WARRANTY THAT THE USE OF THE INFORMATION HEREIN WILL NOT INFRINGE ANY RIGHTS OR ANY IMPLIED WARRANTIES OF MERCHANTABILITY OR FITNESS FOR A PARTICULAR PURPOSE.