-- Module:  PRINTER-TC

-- Editor:  Tom Hastings

-- File:    prttc.doc, .psr, .ps, .mib, .txt

-- Date:    October 25, 1995

-- Version: 0.1

This specification contains the Printer MIB textual-conventions which are a companion specification to the Printer MIB (see RFC 1759).  These textual-conventions also includes the explanatory information about the Printer MIB and should be read first.  It is intended that the Printer MIB textual-conventions and explanation can be updated and re-issued as needed with more textual-conventions and/or explanatory material, while the Printer MIB remains with no change, making the Printer MIB more stable.

NOTE: I haven’t finished copying in the enums from RFC 1759 and creating textual-convention data types for them.  So far, I have only defined the textual-convention data types needed to compile the Job Monitoring MIB.  However, this should give us an idea of what separating the textual-conventions into a separate module (and RFC) from the rest of the Printer MIB when we progress the Printer MIB to draft state (with two RFCs).

1.  Introduction

1.1.  Network Printing Environment

The management of producing a printed document, in any computer

environment, is a complex subject. Basically, the task can be divided

into two overlapping pieces, the management of printing and the

management of the printer. Printing encompasses the entire process of

producing a printed document from generation of the file to be

printed, selection of a printer, choosing printing properties,

routing, queuing, resource management, scheduling, and final printing

including notifying the user.  Most of the printing process is

outside the scope of the model presented here; only the management of

the printer is covered.

Figure 1 - One Printer's View of the Network

system   printer    asset     user          user           user

manager  operator   manager

O         O         O         O             O              O

/|\       /|\       /|\       /|\           /|\            /|\

/ \       / \       / \       / \           / \            / \

|         |         |         |             |              |

+---------+ +-------+ +-------+ +-------+   +-----------+ +-----------+

|configur-| |printer| | asset | |printer|   |   user    | |   user    |

|ator     | |manager| |manager| |browser|   |application| |application|

+---------+ +-------+ +-------+ +-------+   +-----------+ +-----------+

^            ^         ^         ^             |             |

|R/W         |R/W      |R        |R      +-----------+ +-----------+

|            |         |         |       |  spooler  | |  spooler  |

|            |         |         |       +-----------+ +-----------+

|            |         |         |             |             |

|            |         |         |       +-----------+ +-----------+

|            |         |         |       |supervisor | |supervisor |

|            |         |         |       +-----------+ +-----------+

|            |         |         |        ^       ^     ^       ^

|            |         |         |        |R      |R/W  |R      |R/W

v            v         |         |        |       |     |       |

==================================================   |   =====     |

|                          print|        print|

|SNMP                       data|         data|

+-----+        +-------+                        PCL|          PCL|

| MIB |<------>| agent |                 PostScript|   PostScript|

+-----+        +-------+                       NPAP|         NPAP|

|unspecified                etc.|         etc.|

+=============+  +-----------------+   |             |

|             |--|channel/interface|<--+             |

|             |  +-----------------+                 |

|   PRINTER   |                                      |

|             |  +-----------------+                 |

|             |--|channel/interface|<----------------+

+=============+  +-----------------+

1.2.  Printer Device Overview

A printer is the physical device that takes media from an input

source, produces marks on that media according to some page

description or page control language and puts the result in some

output destination, possibly with finishing applied. Printers are

complex devices that consume supplies, produce waste and have

mechanical problems. In the management of the physical printing

device the description, status and alert information concerning the

printer and its various subparts has to be made available to the

management application so that it can be reported to the end user,

key operators for the replenishment of supplies or the repair or

maintenance of the device. The information needed in the management

of the physical printer and the management of a printing job overlap

highly and many of the tasks in each management area require the same

or similar information.

1.3.  Categories of Printer Information

Information about printers is classified into three basic categories,

descriptions, status and alerts.

1.3.1.  Descriptions

Descriptions convey information about the configuration and

capabilities of the printer and its various sub-units. This

information is largely static information and does not generally

change during the operation of the system but may change as the

printer is repaired, reconfigured or upgraded. The descriptions are

one part of the visible state of the printer where state means the

condition of being of the printer at any point in time.

1.3.2.  Status

Status is the information regarding the current operating state of

the printer and its various sub-units. Status is the rest of the

visible state of the printer. As an example of the use of status, a

management application must be able to determine if the various sub-

units are ready to print or are in some state that prevents printing

or may prevent printing in the future.

1.3.3.  Alerts

An Alert is the representation of a reportable event in the printer.

An event is a change in the state of the printer. Some of those state

changes are of interest to a management application and are therefore

reportable. Typically, these are the events that affect the printer's

ability to print. Alerts usually occur asynchronously to the

operation of the computer system(s) to which the printer is attached.

For convenience below, "alert" will be used for both the event caused

by a change in the printer's state and for the representation of that

event.

Alerts can be classified into two basic categories, critical and

non-critical.  A critical alert is one that is triggered by entry

into a state in which the printer is stopped and printing can not

continue until the condition that caused critical alert is

eliminated. "Out of paper", "toner empty" and "output bin full" are

examples of critical alerts. Non-critical alerts are triggered by

those events that enter a state in which printing is not stopped.

Such a non-critical state may, at some future time, lead to a state

in which printing may be stopped.  Examples of this kind of non-

critical alerts are "input media low", "toner low" and "output bin

nearly full". Or, a non-critical alert may simply provide

information, such as signaling a configuration changed in the

printer.

Description, status and alert information about printer can be

thought of as a data base describing the printer. The management

application for a printer will want to view the printer data base

differently depending on how and for what purposes the information in

the data base is needed.

2.  Printer Model

In order to accomplish the management of the printer, an abstract

model of the printer is needed to represent the sub-units from which

the printer is composed. A printer can be described as consisting of

13 types of sub-units. It is important to note that the sub-units of

a printer do not necessarily relate directly to any physically

identifiable mechanism. Sub-units can also be a set of definable

logical processes, such as interpreters for page description

languages or command processors that set various operating modes of

the printer.

Figure 2 shows a block diagram of the printer and its basic 13 sub-

units.

Figure 2 - Printer  Block Diagram

Physical Connections

|

+-----------+

|           |

+-------------+ |

| Interface   |-+

| (RFC1213)   |

+-------------+

|

+-----------+

|           |

+-------------+ |    +-----------+

| Channel     |-+    | Operator  |

|             |      |  Console  |

+-------------+      +-----------+

|

+-----------+        +---------+

|           |        |         |

+-----------+       +-------------+ |    +-----------+ |

|  General  |       | Interpreter |-+    |  Alerts   |-+

|  Printer  |       |             |      |           |

+-----------+       +-------------+      +-----------+

|

+-------------------------------+

|        System Controller      |

|     (This is the Host MIB)    |

+-------------------------------+

+------+                    +--------+                  +--------+

|      |                    |        |                  |        |

+-------+ |    +-------+    +---------+ |    +-------+   +--------+ |

| Input |-+  +--------+|    |  Marker |-+  +--------+|   | Output |-+

|       |===>|        |+<==>|         |<==>|        |+==>|        |

+-------+    +--+  +--+     +---------+    +--+  +--+    +--------+

\            |  ||                         |  ||         \

\           |  ||                         |  ||          \

\          |  ||                         |  ||           \

+--------+  |  |+-------------------------|  ||         +---------+

|        |  |  +--------------------------+  ||         |         |

+----------+ |  |            Media Path          |+      +----------+ |

|  Media   |-+  +--------------------------------+       | Finisher |-+

|(optional)|                                             |(optional)|

+----------+                                             +----------+

2.1.  Overview of the Printer Model

The model has three basic parts: (1) the flow of a print file into an

interpreter and onto the marker, (2) the flow of media through the

marker and (3) the auxiliary sub-units that control and facilitate

the two prior flows.  The flow of the print data comes through a

physical connection on which some form of transport protocol stack is

running.  The data provided by the transport protocol (interface)

appears on a channel which is the input to an interpreter. The

interpreter converts the print data into a form suitable for marking

on the media.

The media resides in Input sub-units from which the media is selected

and then transported via a Media Path first to a Marking sub-unit and

then onto an Output sub-unit with (optionally) some finishing

operations being performed.  The auxiliary sub-units facilitate

control of the printer, inquiry/control of the operator panel,

reporting of alerts, and the adaptation of the printer to various

natural languages and characters sets. All the software sub-units run

on the System Controller which represents the processor, memory and

storage systems of the Printer.  Each of the sub-units is discussed

in more detail below.

All of the sub-units other than the Alerts report only state

information, either a description or a status. The Alerts sub-unit

reports event information.

2.2.  Printer Sub-Units

A printer is composed of 13 types of sub-units, called groups.  The

following sections describe the different types of sub-units.

2.2.1.  General Printer

The general printer sub-unit is responsible for the overall control

and status of the printer. There is exactly one general printer sub-

unit in a printer. The general printer sub-unit is represented by the

General Printer Group in the model. In addition to the providing the

status of the whole printer and allowing the printer to be reset,

this Group provides information on the status of the packaging of the

printer, in particular, the covers. The general printer sub-unit is

usually implemented on the system controller.

The localization portion of the general printer sub-unit is

responsible for identifying the natural language, country, and

character set in which character strings are expressed. There may be

one or more localizations supported per printer. The available

localizations are represented by the Localization table.

Localization is only performed on those strings in the MIB that are

explicitely marked as being localized.  All other character strings

are returned in ASCII.

The character set portion of the general printer sub-unit is

responsible for identifying the possible character sets that are used

by the interpreters, the operator console, and in network management

requests for display objects. There may be one or more character sets

per printer.  The understood character sets are represented by the

Character Set Table.

2.2.2.  Inputs

Input sub-units are mechanisms that feed media to be marked on into

the printer. A printer contains one or more input sub-units. These

are represented by the Input Group in the model. The model does not

distinguish fixed input bins from removable trays, except to report

when a removable tray has been removed.

There are as many input sub-units as there are distinctly selectable

input "addresses".  For example, if a tray has an option for manually

feeding paper as well as automatically feeding from the tray, then

this is two input sub-units if these two sources can be (must be)

separately selected and is one input sub-unit if putting a sheet in

the manual feed slot overrides feeding from the contents of the tray;

that is, in the second case there is no way to separately select or

address the manual feed slot.

2.2.3.  Media

An input sub-unit can hold one or more instances of the media on

which marking is to be done. Typically, there is a large set of

possible media that can be associated with an input. The Media Group

is an extension of the Input Group which represents that media that

is in an input sub-unit. The Media Group only describes the current

contents of each input and not the possible content of the input

sub-unit.

2.2.4.  Outputs

Output sub-units are mechanisms that receive media that has been

marked on. A printer contains one or more output mechanisms. These

are represented by the Output Group in the model. The model does not

distinguish fixed output bins from removable output bins, except to

report when a removable bin has been removed.

There are as many output sub-units as there are distinctly selectable

output "addresses".  Output sub-units can be addressed in two

different ways: (1) as a set of "mailboxes" which are addressed by a

specific mailbox selector such as a bin number or a bin name, or (2)

as a set of "slots" into which multiple copies are collated.

Sometimes both modes of using the output sub-units can be used on the

same printer.  All that is important from the viewpoint of the model

is that the output units can be separately selected.

2.2.5.  Finishers

A finisher is a sub-unit that performs some operations on the media

other than marking.  The finisher sub-units are represented by the

Finisher Group in the model.  Some examples of finishing processes

are stapling, punching, binding, inserting, or folding.  Finishing

processes may have supplies asssociated with the process.  Stapling,

binding, and punching are examples of processes that have supplies. A

printer may have more than one finishing sub-unit and each finishing

sub-unit may be associated with one or more output sub-units.

Finishers are not described in this MIB.

The exact interaction and sequencing between an output device and its

associated finisher is not specified by the model. It depends on the

type of finishing process and the exact implementation of the printer

system. This standard allows for the logical association of a

finishing process with an output device but does not put any

restrictions on the exact sequence or interaction with the associated

output device. The output and finisher sub-units may or may not be

separate identifiable physical mechanisms depending on the exact

implementation of a printer.  In addition, a single output device may

be associated with multiple finishing sub-units and a single

finishing sub-unit may be associated with multiple output devices.

2.2.6.  Markers

A marker is the mechanism that produces marks on the print media. The

marker sub-units and their associated supplies are represented by the

Marker Group in the model. A printer can contain one or more marking

mechanisms.  Some examples of multiple marker sub-units are: a

printer with separate markers for normal and magnetic ink or an

imagesetter that can output to both a proofing device and final film.

Each marking device can have its own set of  characteristics

associated with it, such as marking technology and resolution.

In this model the marker sub-unit is viewed as very generalized and

encompasses all aspects of a marking process. For example, in a

xero-graphic process, the marking process as well as the fusing

process would be included in the generalized concept of the marker.

With the generalized concept of a marking process, the concept of

multiple marking supplies associated with a single marking sub-unit

results. For example, in the xerographic process, there is not only a

supply of toner, but there can also be other supplies such as a fuser

supply that can be consumed and replaced separately. In addition

there can be multiple supplies of toner for a single marker device,

as in a color process.

2.2.7.  Media Paths

The media paths encompass the mechanisms in the printer that move the

media through the printer and connect all other media related sub-

units: inputs, outputs, markers and finishers. A printer contains one

or more media paths. These are represented by the Media Path Group in

the model.  The Media Path group has some objects that apply to all

paths plus a table of the separate media paths.

In general, the design of the media paths determines the maximum

speed of the printer as well as the maximum media size that the

printer can handle. Media paths are complex mechanisms and can

contain many different identifiable sub-mechanisms such as media

movement devices, media buffers, duplexing units and interlocks. Not

all of the various sub-mechanisms reside on every media path.  For

example, one media path may provide printing only on one surface of

the media (a simplex path) and another media path may have a sub-

mechanism that turns the media over and feeds it a second time

through the marker sub-unit (a duplex path).  The duplex path may

even have a buffer sub-mechanism that allows multiple copies of the

obverse side to be held before the reverse side of all the copies are

marked.

2.2.8.  System Controller

The System Controller is the sub-unit upon which the software

components of the Printer run. The System Controller is represented

in the model by the Host MIB. This MIB allows for the specification

of the processor(s), memory, disk storage, file system and other

underlying sub-mechanisms of the printer. The controller can range

from simple single processor systems to multiprocessor systems. In

addition, controllers can have a full range of resources such as hard

disks. The printer is modeled to have one system controller even

though it may have more than one processor and multiple other

resources associated with it.

2.2.9.  Interfaces

An interface is the communications port and associated protocols that

are responsible for the transport of data to the printer. A printer

has one or more interface sub-units. The interfaces are represented

by the Interfaces Group of MIB-II (RFC 1213). Some examples of

interfaces are serial ports (with little or no protocol) and EtherNet

ports on which one might run InterNet IP, Novell IPX, etc.

2.2.10.  Channels

The channel sub-units identify the independent sources of print data

(here print data is the information that is used to construct printed

pages and may have both data and control aspects).  A printer may

have one or more channels. The channel sub-units are represented by

the Channel Group in the Model. Each channel is typically identified

by the electronic path and service protocol used to deliver print

data to the printer. A channel sub-unit may be independently enabled

(allowing print data to flow) or disabled (stopping the flow of print

data). It has a current Control Language which can be used to specify

which interpreter is to be used for the print data and to query and

change environment variables used by the interpreters (and SNMP).

There is also a default interpreter that is to be used if an

interpreter is not explicitly specified using the Control Language.

Channel sub-units are based on an underlying interface.

2.2.11.  Interpreters

The interpreter sub-units are responsible for the conversion of a

description of intended print instances into images that are to be

marked on the media. A printer may have one or more interpreters. The

interpreter sub-units are represented by the Interpreter Group in the

Model. Each interpreter is generally implemented with software

running on the System Controller sub-unit. The Interpreter Table has

one entry per interpreter where the interpreters include both Page

Description Language (PDL) Interpreters and Control Language

Interpreters.

2.2.12.  Console

Many printers have a console on the printer, the operator console,

that is used to display and modify the state of the printer.  The

console can be as simple as a few indicators and switches or as

complicated as full screen displays and keyboards. There can be at

most one such console.  This console sub-unit is represented by the

Console Group in the model.  Although most of the information

displayed there is also available in the state of the printer as

represented by the various Groups, it is useful to be able to query

and modify the operator console remotely.  For example, a management

application might like to display to its user the current message on

the operator console of the remote printer or the management

application user might like to modify the current message on the

operators console of the remote printer.  As another example, one

might have a remote application that puts up a pseudo console on a

workstation screen. Since the rules by which the printer state is

mapped onto the console and vice versa are not standardized, it is

not possible to reproduce the console state or the action of console

buttons and menus. Therefore, the Console Group provides access to

the console. The operator console is usually implemented on the

system controller with additional hardware for input and display.

2.2.13.  Alerts

The alert sub-unit is responsible for detecting reportable events,

making an entry in the alert table and, if and only if the event is a

critical event, initiating a trap. The alert sub-unit is represented

by the Alerts Group and, in particular, the Alert Table. This table

contains information on the severity, sub-unit, detailed location

within the sub-unit, alert code and description of each critical

alert that is currently active within the printer. Each reportable

event causes an entry to be made in the Alert Table.

2.2.13.1.  Status and Alerts

Summary information about the state of the printer is reported at

three separate levels: (1) there is the status of the printer as a

whole reported in the Host MIB, (2) there is the status of various

sub-units reported in the principle table of the Group that

represents the sub-unit, and (3) there are alert codes reported in

the Alert Table.

2.2.13.2.  Overall Printer Status

Of the many states a printer can be in, certain states are more

"interesting" because of the distinct actions they are likely to

provoke in the administrator.  These states may be applied to the

printer as a whole, or to a particular sub-unit of the printer.

These named states are:

Non Critical Alert Active - For the printer this means that one or

more sub-units have a non-critical alert active.  For a sub-unit,

this means that the sub-unit has a non-critical alert active.

Critical Alert Active - For the printer this means that one or more

sub-units have a critical alert active.  For a sub-unit, this means

that the sub-unit has a critical alert active.

Unavailable - The printer or sub-unit is unavailable for use (this is

the same as "broken" or "down" in other terminologies).  A trained

service person is typically necessary to make it available.

Busy / Temporarily Unavailable - The printer or sub-unit is

operational but currently occupied with a request for activity. The

sub-unit will become available without the need of human interaction.

Moving on-line or off-line - The printer is either off-line, in the

process of moving off-line or in the process of moving back on-line;

for example on high end printers reloading paper involves a

transition to off-line to open the paper bin, it is then filled and,

finally, there is a transition back to on-line as the paper bin is

repositioned for printing.

Standby - The printer or sub-unit is unavailable for use because it

is partially powered down and may need some period of time to become

fully operational again.  A unit in Standby state shall respond to

network management requests.

The Host MIB provides three status objects that can be used to

describe the status of a printer: (1) hrDeviceStatus in the entry in

the Host MIB hrDeviceTable; (2) hrPrinterStatus in the

hrPrinterTable; and (3) hrPrinterDetectedErrorState in the

hrPrinterTable.  These objects describe many of the states that a

printer can be in.  The following table shows how the "interesting"

states named above can be recognized by inspecting the values of the

three printer-related objects in the Host MIB:

Printer     hrDeviceStatus  hrPrinterStatus  hrPrinterDetectedErrorState

Status

Normal         running(2)     idle(3)        none set

Busy/          running(2)     printing(4)

Temporarily

Unavailable

Non Critical   warning(3)     idle(3) or     could be: lowPaper,

Alert Active                  printing(4)    lowToner, or

serviceRequested

Critical       down(5)        other(1)       could be: jammed,

Alert Active                                 noPaper, noToner,

coverOpen, or

serviceRequested

Unavailable    down(5)        other(1)

Moving off-    warning(3)     idle(3) or     offline

line                          printing(4)

Off-line       down(5)        other(1)       offline

Moving         down(5)        warmup(5)

on-line

Standby        running(2)     other(1)

These named states are only a subset of the possible states - they

are not an exhaustive list of the possible states.  Nevertheless,

several things should be noted.  When using these states, it is not

possible to detect when both critical and non-critical alerts are

pending - if both are pending, the Critical Alert Active state will

prevail.  In addition, a printer in the Standby state will be

represented in the Host MIB with a device status of running(2) and a

printer status of other(1), a set of states that don't uniquely

distinguish this important printer state.

Although the above mapping is workable, it would be improved with a

few additions to hrDeviceStatus and hrPrinterStatus in the Host

Resources MIB. In particular, it would be appropriate to add a

"standby" enumeration to hrDeviceStatus.  Similarly, it would be

useful to add the following states to hrPrinterStatus: "offline" to

indicate that reason for the printer being down (instead of having to

use "other") which allows both "warning" and "offline" to indicate

going offline and "down" and "offline" to indicate offline and

"notApplicable" to cover cases, such as "standby", where the device

state completely describes the state of the device.

Detailed status per sub-unit is reported in the sub-unit status

fields.

2.2.13.2.1.  Host MIB Printer Status

For completeness, the definitions of the Printer Status objects of

the Host MIB are given below:

hrDeviceStatus OBJECT-TYPE

SYNTAX  INTEGER {

unknown(1),

running(2),

warning(3),

testing(4),

down(5)

}

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The current operational state of the device

described by this row of the table.  A value

unknown(1) indicates that the current state of the

device is unknown.  running(2) indicates that the

device is up and running and that no unusual error

conditions are known.  The warning(3) state

indicates that agent has been informed of an

unusual error condition by the operational software

(e.g., a disk device driver) but that the device is

still 'operational'.  An example would be high

number of soft errors on a disk.  A value of

testing(4), indicates that the device is not

available for use because it is in the testing

state.  The state of down(5) is used only when the

agent has been informed that the device is not

available for any use."

::= { hrDeviceEntry 5 }

hrPrinterStatus OBJECT-TYPE

SYNTAX INTEGER {

other(1),

unknown(2),

idle(3),

printing(4),

warmup(5)

}

ACCESS read-only

STATUS mandatory

DESCRIPTION

"The current status of this printer device.  When

in the idle(1), printing(2), or warmup(3) state,

the corresponding hrDeviceStatus should be

running(2) or warning(3).  When in the unknown

state, the corresponding hrDeviceStatus should be

unknown(1)."

::= { hrPrinterEntry 1 }

hrPrinterDetectedErrorState OBJECT-TYPE

SYNTAX OCTET STRING

ACCESS read-only

STATUS mandatory

DESCRIPTION

"This object represents any error conditions

detected by the printer.  The error conditions are

encoded as bits in an octet string, with the

following definitions:

Condition         Bit #    hrDeviceStatus

lowPaper          0        warning(3)

noPaper           1        down(5)

lowToner          2        warning(3)

noToner           3        down(5)

doorOpen          4        down(5)

jammed            5        down(5)

offline           6        down(5)

serviceRequested  7        warning(3)

If multiple conditions are currently detected and

the hrDeviceStatus would not otherwise be

unknown(1) or testing(4), the hrDeviceStatus shall

correspond to the worst state of those indicated,

where down(5) is worse than warning(3) which is

worse than running(2).

Bits are numbered starting with the most

significant bit of the first byte being bit 0, the

least significant bit of the first byte being bit

7, the most significant bit of the second byte

being bit 8, and so on.  A one bit encodes that

the condition was detected, while a zero bit

encodes that the condition was not detected.

This object is useful for alerting an operator to

specific warning or error conditions that may

occur, especially those requiring human

intervention."

::= { hrPrinterEntry 2 }

2.2.13.2.2.  Sub-unit Status

Sub-unit status is reported in the entries of the principle table in

the Group that represents the sub-unit. For sub-units that report a

status, there is a status column in the table and the value of this

column is always an integer formed in the following way.

The SubUnitStatus is an integer that is the sum of 5 distinct values,

Availability, Non-Critical, Critical, On-line, and Transitioning.

These values are:

Availability                           value

Available and Idle              0       000'b

Available and Standby           2       010'b

Available and Active            4       100'b

Available and Busy              6       110'b

Unavailable and OnRequest       1       001'b

Unavailable because Broken      3       011'b

Unknown                         5       101'b

Non-Critical

No Non-Critical Alerts          0

Non-Critical Alerts             8

Critical

No Critical Alerts              0

Critical Alerts                 16

On-Line

Intended state is On-Line       0

Intended state is Off-Line      32

Transitioning

At intended state               0

Transitioning to intended state 64

For example, an input (tray) that jammed on the next to the last page

may show a status of 27 (unavailable because broken (3) + a critical

state (16), jammed, and a noncritical state (8), low paper).

2.2.13.3.  Alert Tables

The Alert Group consists of a single table in which all active alerts

are represented.  This section provides and overview of the table and

a description of how it is managed.  The basic content of the alert

table is the severity (critical or non-critical) of the alert, the

Group and entry where a state change caused the alert, additional

information about the alert (a more detailed location, an alert code,

and a description), and an indication of the level of training needed

to service the alert.

The Alert Table contains some information that is redundant, for

example that an event has occurred, and some information that is only

represented in the Alert Table, for example the additional

information.  A single table was used because a single entry in a

Group could cause more than one alert, for example paper jams in more

than one place in a media path. Associating the additional

information with the entry in the affected group would only allow one

report where associating the additional information with the alert

makes multiple reports possible.

Every time an alert occurs in the printer, the printer makes one or

more entries into the Alert Table. The printer determines if an event

is to be classified as critical or non-critical. If the severity of

the Alert is "critical", the printer sends a trap or event

notification to the host indicating that the table has changed.

Whether or not a trap is sent, the management application is expected

to poll the printer on a regular basis and to read and parse the

table to determine what conditions have changed, in order to provide

reliable information to the management application user.

2.2.13.4.  Alert Table Management

The alert tables are sparsely populated tables. This means the tables

will only contain entries of the alerts that are currently active and

the number of rows, or entries in the table will be dynamic. More

than one event can be added or removed from the event tables at a

time depending on the implementation of the printer.

There are basically two kinds of events that produce alerts: binary

change events and simple change events. Binary change events come in

pairs: the leading edge event and the trailing edge event. The

leading edge event enters a state from which there is only one exit;

for example, going from running to stopped with a paper jam. The only

exit from this state is fixing the paper jam and it is clear when

that is accomplished.  The trailing edge event is the event which

exits the state the was entered by the leading edge event; in the

example above fixing the paper jam is the trailing edge event.

It is relatively straightforward to manage binary change events in

the Alert Table. Only the leading edge event makes an entry in the

alert table.  This entry persists in the Alert Table until the

trailing edge event occurs at which point this event is signal by the

removal of the leading edge event entry in the Alert Table.  That is,

a trailing edge event does not create an entry; it removes the

corresponding leading edge event. With binary events it is possible

to compute the maximum number that can occur at the same time and

construct an Alert Table that would hold that many events. There

would be no possibility of table overflow and no information about

outstanding events would be lost.

Unfortunately, there are some events that are not binary changes.

This other category of event, the simple change event,  is

illustrated by the configuration change event. With this kind of

event the state of the machine has changed, but to a state which is

(often) just as valid as the state that was left and from which no

return is necessary.  For example, an operator may change the paper

that is in the primary input source from letter to legal. At some

time in the future the paper may be changed back to letter, but it

might be changed to executive instead.  This is where the problem

occurs. It is not obvious how long to keep simple change event

entries in the Alert Table. It they were never removed, the Alert

Table would continue to grow indefinitely.

The agent needs to have an algorithm implemented for the management

of the alert table, especially in the face of combinations of binary

and simple alerts that would overflow the storage capaciity of the

table.  When the table is full and a new alert needs to be added, an

old alert needs to be deleted.  The alert to be deleted should be

chosen using the following rules:

1. Find a non-critical simple alert and delete it.  If there are

multiple non-critical simple alerts, it is suggested that the

oldest one be chosen.  If there are no non-critical simple

alerts, then,

2. Find a non-critical binary alert and delete it.  If there are

multiple non-critical binary alerts, it is suggested that the

oldest one be chosen.  If there are no non-critical binary

alerts, then,

3. Find a critical (binary) alert and delete it.  If there are

multiple critical alerts, it is suggested that the

oldest one be chosen.  Agent implementors are encouraged to

provide at least enough storage space for the maximum number

of critical alerts that could occur simultaneously.  Note that

all critical alerts are binary.

Note that because the Alert Index is a monotonically increasing

integer there will be gaps in the values in the table when an alert

is deleted.  Such gaps can be detected by the management application

to indicate that the management application may want to re-acquire

the Printer state and check for state changes it did not observe in

the Alert Table.

2.3.  Read-Write Objects

Some of the objects in the printer MIB report on the existence of or

amount of a given resource used with the printer.  Some examples of

such resources are the size and number of sheets of paper in a paper

tray or the existence of certain output options.  On some printers

there are sensors that allow these resources to be sensed.  Other

printers, however, lack sensors that can detect (all of) the

properties of the resource.  Because the printer needs to know of the

existence or properties of these resources for the printer to

function properly some other way of providing this information is

needed.  The chosen way to solve this problem is to allow a

management application to write into objects which hold the

descriptive or existence values for printers that cannot sense the

values.  Thus many of the objects in the MIB are given read-write

access, but a printer implementation might only permit a management

operation to change the value if the printer could not sense the

value itself.  Therefore, the ability to change the value of a read-

write object may depend on the implementation of the agent.  Note

that even though some objects explicitely state the behaviour of

conditional ability to change values, any read-write object may act

that way.

Generally, an object is given read-write access in the Printer MIB

specification if:

1.The object involves installation of a resource that some

printers cannot themselves detect.  Therefore, external means are

needed to inform the printer of the installation.  (Here external

means include using the operator console, or remote management

application) and

2.The printer will behave differently if the installation of the

resource is reported than the printer would if the installation

were not reported; that is, the object is not to be used

as a place to put information not used by the printer, i.e., not a

"PostIt".  Another way of saying this is that the printer believes

that information given it and acts as if the information were

true.  For example, on a printer that cannot sense the size, if

one paper size is loaded, but another size is set into the paper

size object, then the printer will use the size that was

set as its current paper size in its imaging and paper handling.

The printer may get hints that it may not know about the existence or

properties of certain resources.  For example, a paper tray may be

removed and re-inserted.  When this removal and insertion happens,

the printer may either assume that a property, such as the size of

paper in the tray, has not changed or the printer may change the

value of the associated object to "unknown", as might be done for the

amount of paper in the tray.  As long as the printer acts according

to the value in  the object either strategy is acceptable.

It is an implementation-specific matter as to whether or not MIB

object values are persistent across power cycles or cold starts.  It

is particularly important that the values of the prtMarkerLifeCount

object persist throughout the lifetime of the printer.  Therefore, if

the value of any MIB object persists across power cycles, then the

prtMarkerLifeCount object must also persist.

2.4.  Enumerations

Enumerations (enums) are sets of symbolic values defined for use with

one or more objects.  Some common enumeration sets are assigned a

symbolic data type name (textual convention).  These enumerations are

listed at the beginning of this specification.

2.4.1.  Registering Additional Enumerated Values

This working group has defined several type of enumerations.  These

enumerations differ in the method employed to control the addition of

new enumerations.  Throughout this document, references to

"enumeration (n)", where n can be 1, 2 or 3 can be found in the

various tables.  The definitions of these types of enumerations are:

enumeration (1)  All the values are defined in the Printer MIB

specification (RFC for the Printer MIB).  Additional enumerated

values require a new RFC.

enumeration (2)  An initial set of values are defined in the Printer

MIB specification.  Additional enumerated values are

registered after review by this working group. The initial

versions of the MIB will contain the values registered so far.

After the MIB is approved, additional values will be

registered through IANA after approval by this working group.

enumeration (3)  An initial set of values are defined in the Printer

MIB specification.  Additional enumerated values are

registered without working group review.  The initial versions of

the MIB will contain the values registered so far.  After the MIB

is approved, additional values will be registered

through IANA without approval by this working group.

3.  Objects from other MIB Specifications

This section lists the objects from other IETF MIB specifications

that are mandatory for conformance to this Printer MIB specification.

3.1.  System Group objects

All objects in the system group of MIB-II (RFC 1213) must be

implemented.

3.2.  System Controller

The System Controller is represented by the Storage and Device Groups

of the Host Resources MIB (RFC 1514).  These are the only groups that

are required to be implemented.  Other Groups (System, Running

Software, Running Software Performance, and Installed Software) may

be implemented at the discretion of the implementor.

3.3.  Interface Group objects

All objects in the Interfaces Group of MIB-II (RFC 1213) shall be

implemented.

PRINTER-TC DEFINITIONS ::= BEGIN

IMPORTS

MODULE-IDENTITY, experimental

FROM SNMPv2-SMI                    -- RFC 1442

TEXTUAL-CONVENTION

FROM SNMPv2-TC;                    -- RFC 1443

-- Upon publication as RFC, delete this comment and the line following

-- this comment and change the reference of { printmib 102 }

-- (below) to { mib-2 X }.

-- This will result in changing:

-- 1 3 6 1 3 54 printTC(102)    to:

-- 1 3 6 1 2  1 printTC(X)

-- This will make it easier to translate prototypes to

-- the standard namespace because the lengths of the OID's won't

-- change.

printmib OBJECT IDENTIFIER ::= { experimental 54 }

printerTC MODULE-IDENTITY

LAST-UPDATED "9510250000Z"

ORGANIZATION "IETF/DMTF Printer Working Group (PWG)"

CONTACT-INFO

"       Thomas N. Hastings

Xerox Corporation, MS ES-AE 242

701 S. Aviation Blvd.

El Segundo, CA 90245

Phone:        1+ (310)333-6413

FAX:          1+ (310)333-6342

E-Mail:       hastings@cp10.es.xerox.com"

DESCRIPTION

"File: prttc.doc, .psr, .ps, .mib, .txt

Version: 0.1

This textual-convention module defines textual-

conventions for use with the Printer MIB,

module:  Printer-MIB.  Also the explanatory

material with this module explains the Printer

MIB.  These textual-conventions and explanations are in

a separate module from the Printer MIB, so that

they may be republished when additional enums are added

or more explanatory material is added without needing to

republish the Printer MIB, thus increasing the

stability of the Printer MIB."

::= { printmib 102 }

-- Textual conventions used in more than one group

MediaUnit ::= TEXTUAL-CONVENTION

STATUS       current

DESCRIPTION

"Units of measure for media dimensions.

This is a type 1 enumeration."

SYNTAX       INTEGER {

tenThousandthsOfInches(3),  -- .0001

micrometers(4)

}

CapacityUnit ::= TEXTUAL-CONVENTION

STATUS       current

DESCRIPTION

"Units of measure for media capacity.

This is a type 1 enumeration."

SYNTAX       INTEGER {

tenThousandthsOfInches(3),  -- .0001

micrometers(4),

sheets(8),

feet(16),

meters(17)

}

SubUnitStatus ::= TEXTUAL-CONVENTION

STATUS       current

DESCRIPTION

"Status of a printer sub-unit.

The SubUnitStatus is an integer that is the sum of 5

distinct values, Availability, Non-Critical, Critical,

On-line, and Transitioning. These values are:

Availability                           value

Available and Idle              0       000'b

Available and Standby           2       010'b

Available and Active            4       100'b

Available and Busy              6       110'b

Unavailable and OnRequest       1       001'b

Unavailable because Broken      3       011'b

Unknown                         5       101'b

Non-Critical

No Non-Critical Alerts          0

Non-Critical Alerts             8

Critical

No Critical Alerts              0

Critical Alerts                 16

On-Line

Intended state is On-Line       0

Intended state is Off-Line      32

Transitioning

At intended state               0

Transitioning to intended state 64

"

SYNTAX       INTEGER (0..126)

PresentOnOff ::= TEXTUAL-CONVENTION

STATUS       current

DESCRIPTION

"Presence and configuration of a device or feature.

This is a type 1 enumeration."

SYNTAX       INTEGER {

other(1),

on(3),

off(4),

notPresent(5)

}

-- General Group textual-conventions

PrtGeneralReset ::= TEXTUAL-CONVENTION

STATUS     current

DESCRIPTION

"Values for reading and writing the prtGeneralReset object.

This value is a type 3 enumeration."

SYNTAX     INTEGER {

notResetting(3),

powerCycleReset(4), -- Cold Start

resetToNVRAM(5), -- Warm Start

resetToFactoryDefaults(6) -- Reset contents of

-- NVRAM to factory

-- defaults

}

CodedCharSet ::= TEXTUAL-CONVENTION

STATUS     current

DESCRIPTION

"A coded character set value that specifies both a set of

characters that may be used and an encoding (as one or more

octets) that is used to represent the characters in the

set. These values are to be used to identify the encoding

employed for strings in the MIB where this is not fixed by

the MIB.

Some objects that allow a choice of coded character set

are: the prtLocalizationCharacterSet object in the

LocalizationTable and prtInterpreterDefaultCharSetIn.

The prtGeneralCurrentLocalization and prtConsoleLocalization

objects in turn contain the index in the LocalizationTable

of the current localization (country, language, and coded

character set) of the `description' objects and the console,

respectively.

The space of the coded character set enumeration has been

divide into three regions. The first region (3-999) consists

of coded character sets that have been standardized by some

standard setting organization. This region is intended for

standards that do not have subset implementations. The

second region (1000-1999) is for the Unicode and ISO/IEC 10646

coded character sets together with a specification of a (set

of) sub-repetoires that may occur.  The third region (>1999)

is intended for vendor specific coded character sets.

NOTE: Unicode and ISO 10646 character coded data may be

processed and stored in either Big Endian (most significant

octet first) or Little Endian (least significant octet

first) order.  Intel x86, VAX, and Alpha/AXP architectures are

examples of Little Endian processor architectures.

Furthermore, in environments where either order may occur,

so-called Unicode BYTE ORDER MARK (BOM) character (which is

ISO 10646 ZERO WIDTH NO BREAK SPACE), coded as FEFF in two

octets and 0000FEFF in four octets is used at the beginning

of the data as a signature to indicate the order of the

following data (See ISO 10646 Annex F).  Thus either

ordering and BOM may occur in print data streams sent to the

interpreter.  However, ISO 8824/8825 (ASN.1/BER) used by

SNMP is quite clear that Big Endian order shall be used and

BOM shall NOT be used in transmission in the protocol.

Transmitting Unicode in Big Endian order in SNMP should

not prove to be a hardship for Little Endian machines,

since SNMP ASN.1/BER requires integers to be transmitted

in Big Endian order as well.  So SNMP implementations on

Little Endian machines are already reversing the order of

integers to make them Big Endian for transmission via

SNMP.  Also Unicode characters are usually treated as

two-octet integers, not short text strings, so that it will

be straightforward for Little Endian machines to reverse the

order of Unicode character octets as well before

transmitting them and after receiving them via the SNMP

protocol.

Where a given coded character set may be known by more than

one name, the most commonly known name is used as the name

of the enumeration and other names are shown in the

comments.  The comments also indicate where to find detailed

information on the coded character set and briefly

characterize its relationship to other similar coded

character sets.

The current list of character sets and their enumerated

values used to reference them is contained in the IANA

Character Set registry.  The enum value is indicated by

the MIBenum entry in the registry.  The enum symbol is

indicated by the Alias that starts with `cs' for character

set.

The IANA character sets registry is available via

anonymous ftp.

The ftp server is ftp.isi.edu.

The subdirectory is /in-notes/iana/assignments/.

The file name is character-sets.

To add a character set to the IANA Registry:

1. Format an entry like those in the current list,

omitting the MIBenum value.

2. Send the entry with a request to add the entry

to the character set list to iana@ISI.EDU.

3. The IANA will supply a unique MIBenum value

and update the list.

This is a type 3 enumeration."

SYNTAX     INTEGER {

other(1)               -- used if the designated coded

-- character set is not currently in

-- the enumeration

-- See IANA Registry for standard character sets in the

-- MIBenum range of 3-999.

-- See IANA Registry for Unicode and vendor-supplied

-- combinations of ISO collections and character sets based

-- on Unicode in the MIBenum range of 1000-1999.

-- See IANA Registry for vendor developed character sets

-- in the MIBenum range of 2000-xxxx.

}

-- Channel Group textual-conventions

PrtChannelType ::= TEXTUAL-CONVENTION

STATUS     current

DESCRIPTION

"This enumeration indicates the type of channel that is

receiving jobs.

This is a type 2 enum."

SYNTAX     INTEGER {

other(1),

chSerialPort(3),

chParallelPort(4),

chIEEE1284Port(5),

chSCSIPort(6),

chAppleTalkPAP(7),     -- AppleTalk Printer Acess

-- Protocol

chLPDServer(8),

chNetwareRPrinter(9),  -- Netware

chNetwarePServer(10),  -- Netware

chPort9100(11),

chAppSocket(12),       -- a bi-directional, LPD-like

-- protocol using 9101 for

-- control and 9100 for data.

-- Adobe Systems, Inc.

chFTP(13),             -- FTP "PUT" to printer

chTFTP(14),

chDLCLLCPort(15),

chIBM3270(16),

chIBM5250(17),

chFax(18),

chIEEE1394(19),

chTransport1(20),        -- port 35

chCPAP(21),              -- port 170

chDCERemoteProcCall(22), -- OSF

chONCRemoteProcCall(23), -- Sun Microsystems

chOLE(24),               -- Microsoft

chNamedPipe(25),

chPCPrint(26),           -- Banyan

chServerMessageBlock(27),

-- File/Print sharing protocol used by

-- various network operating systems

-- from IBM 3Com, Microsoft and others

chDPMF(28),  -- Distributed Print Mgt. Framework, IBM

chDLLAPI(29), -- Microsoft

chVxDAPI(30), -- Microsoft

chSystemObjectManager(31), -- IBM

chDECLAT(32),              -- Digital Equipment Corp.

chNPAP(33)

}

-- Interpreter Group textual conventions

PrtInterpreterLangFamily ::= TEXTUAL-CONVENTION

STATUS     current

DESCRIPTION

"This enumeration indicates the type of interpreter that is

receiving jobs.

This value is a type 2 enumeration."

SYNTAX     INTEGER {

other(1),

langPCL(3),        -- PCL.  Starting with PCL version 5,

-- HP-GL/2 is included as part of the

-- PCL language.

-- PCL and HP-GL/2 are registered

-- trademarks of Hewlett-Packard Company.

langHPGL(4),       -- Hewlett-Packard Graphics Language.

-- HP-GL is a registered trademark of

-- Hewlett-Packard Company.

langPJL(5),        -- Peripheral Job Language. Appears in the

-- data stream between data intended for a

-- page description language.

-- Hewlett-Packard Co.

langPS(6),         -- PostScript Language (tm)

-- Postscript - a trademark of Adobe

-- Systems Incorporated which may be

-- registered in certain jurisdictions

langPSPrinter(42), -- The PostScript Language used for

-- control (with any PDLs)

-- Adobe Systems Incorporated

langIPDS(7),       -- Intelligent Printer Data Stream

-- Bi-directional print data stream for

-- documents consisting of data objects

-- (text, image, graphics, bar codes),

-- resources (fonts, overlays) and page,

-- form and finishing instructions.

-- Facilitates system level device

-- control, document tracking and error

-- recovery throughout the print process.

-- Pennant Systems, IBM

langPPDS(8),       -- IBM Personal Printer Data Stream.

-- Originally called IBM ASCII, the name

-- was changed to PPDS when the Laser

-- Printer was introduced in 1989.

-- Lexmark International, Inc.

langEscapeP(9),

langEpson(10),

langDDIF(11),      -- Digital Document Interchange Format

-- Digital Equipment Corp., Maynard MA

langInterpress(12),

langISO6429(13),   -- ISO 6429.  Control functions for Coded

-- Character Sets (has ASCII control

-- characters, plus additional controls for

-- character imaging devices.)

-- ISO Standard, Geneva, Switzerland

langLineData(14),  -- line-data:  Lines of data as separate

-- ASCII or EBCDIC records and containing

-- no control functions (no CR, LF, HT, FF,

-- etc.). For use with traditional line

-- printers.  May use CR and/or LF to

-- delimit lines, instead of records.  See

-- ISO 10175 Document Printing Application

-- (DPA)

-- ISO standard, Geneva, Switzerland

langMODCA(15),     -- Mixed Object Document Content

-- Architecture

-- Definitions that allow the composition,

-- interchange, and presentation of final

-- form documents as a collection of data

-- objects (text, image, graphics, bar

-- codes), resources (fonts, overlays) and

-- page, form and finishing instructions.

-- Pennant Systems, IBM

langREGIS(16),     -- Remote Graphics Instruction Set,

-- Digital Equipment Corp., Maynard MA

langSCS(17),       -- SNA Character String

-- Bi-directional print data stream for SNA

-- LU-1 mode of communications

-- IBM

langSPDL(18),      -- ISO 10180 Standard Page Description

-- Language

-- ISO Standard

langTEK4014(19),

langPDS(20),

langIGP(21),

langCodeV(22),     -- Magnum Code-V, Image and printer control

-- language used to control impact/dot-

-- matrix printers.

-- QMS, Inc., Mobile AL

langDSCDSE(23),    -- DSC-DSE:  Data Stream Compatible and

-- Emulation Bi-directional print data

-- stream for non-SNA (DSC) and SNA LU-3

-- 3270 controller (DSE) communications

-- IBM

langWPS(24),       -- Windows Printing System, Resource based

-- command/data stream used by Microsoft At

-- Work Peripherals.

-- Developed by the Microsoft Corporation.

langLN03(25),      -- Early DEC-PPL3, Digital Equipment Corp.

langCCITT(26),

langQUIC(27),      -- QUIC (Quality Information Code), Page

-- Description Language for laser printers.

-- Included graphics, printer control

-- capability and emulation of other well-

-- known printer .

-- QMS, Inc.

langCPAP(28),      -- Common Printer Access Protocol

-- Digital Equipment Corp.

langDecPPL(29),    -- Digital ANSI-Compliant Printing Protocol

-- (DEC-PPL)

-- Digital Equipment Corp.

langSimpleText(30),-- simple-text:  character coded data,

-- including NUL, CR , LF, HT, and FF

-- control characters.  See ISO 10175

-- Document Printing Application (DPA)

-- ISO standard, Geneva, Switzerland

langNPAP(31),      -- Network Printer Alliance Protocol

-- IEEE 1284.1

langDOC(32),       -- Document Option Commands, Appears in the

-- data stream between data intended for a

-- page description .

-- QMS, Inc.

langimPress(33),   -- imPRESS, Page description language

-- originally developed for the ImageServer

-- line of systems. A binary language

-- providing representations for text,

-- simple graphics (rules, lines, conic

-- sections), and some large forms (simple

-- bit-map and CCITT group 3/4 encoded).The

-- language was intended to be sent over an

-- 8-bit channel and supported early

-- document preparation languages (e.g. TeX

-- and TROFF).

-- QMS, Inc.

langPinwriter(34), -- 24 wire dot matrix printer  for

-- USA, Europe, and Asia except Japan.

-- More widely used in Germany, and some

-- Asian countries than in US.

-- NEC

langNPDL(35),      -- Page printer  for Japanese

-- market.

-- NEC

langNEC201PL(36),  -- Serial printer language used in the

-- Japanese market.

-- NEC

langAutomatic(37), -- Automatic PDL sensing.  Automatic

-- sensing of the interpreter language

-- family by the printer examining the

-- document content.  Which actual

-- interpreter language families are sensed

-- depends on the printer implementation.

langPages(38),     -- Page printer Advanced Graphic Escape Set

-- IBM Japan

langLIPS(39),      -- LBP Image Processing System

langTIFF(40),      -- Tagged Image File Format (Aldus)

langDiagnostic(41),-- A hex dump of the input to the

-- interpreter

langCaPSL(43),     -- Canon Print Systems Language

langEXCL(44),      -- Extended Command Language

-- Talaris Systems Inc.

langLCDS(45),      -- Line Conditioned Data Stream

-- Xerox Corporation

langXES(46)        -- Xerox Escape Sequences

-- Xerox Corporation

}

END