INTERNET-DRAFT                                                                                            Ron Bergman

Dataproducts Corp.

January 23, 1998

Job Submission Protocol Mapping Recommendations

for the Job Monitoring MIB

<draft-bergman-printmib-job-protomap-01.txt>

Expires July 23, 1998

Status of this Memo

This document is an Internet-Draft.    Internet-Drafts are working

documents of the Internet Engineering Task Force (IETF), its areas,

and its working groups.    Note that other groups may also distribute

working documents as Internet-Drafts.

Internet-Drafts are draft documents valid for a maximum of six

months and may be updated, replaced, or obsoleted by other

documents at any time.    It is inappropriate to use Internet-Drafts

as reference material or to cite them other than as "work in

progress".

To learn the current status of any Internet-Draft, please check the

"1id-abstracts.txt" listing contained in the Internet-Drafts Shadow

Directories on ftp.is.co.za (Africa), nic.nordu.net (Europe),

munnari.oz.au (Pacific Rim), ds.internic.net (US East Coast), or

ftp.isi.edu (US West Coast).

Abstract

This Internet-Draft defines the recommended mapping for many

currently popular Job submission protocols to objects and

attributes in the Job Monitoring MIB.

TABLE OF CONTENTS

1.0  INTRODUCTION	3

2.0  LINE PRINTER DAEMON (LPR/LPD) PROTOCOL	4

2.1  jmJobSubmissionId Mapped to LPR/LPD	4

2.2  jmJobIndex Mapped to LPR/LPD	5

2.3  Other MIB Objects Mapped to LPR/LPD	5

2.4  The Attribute Group Mapped to LPD	5

3.0  APPLETALK PROTOCOL	6

3.1  jmJobSubmissionId Mapped to AppleTalk	6

3.2  Other AppleTalk Mappings	6

4.0  INTERNET PRINTING PROTOCOL (IPP)	6

4.1  jmJobSubmissionId Mapped to IPP	7

4.2  jmJobIndex Mapped to IPP	7

4.3  Other MIB Objects Mapped to IPP	7

4.4  The Attribute Group Mapped to IPP	8

5.0  INTELLIGENT PRINTER DATA STREAM (IPDS)	8

6.0  DOCUMENT PRINTING APPLICATION (DPA)	9

6.1  jmJobSubmissionId Mapped to DPA	9

6.2  jmJobIndex Mapped to DPA	9

6.3  Other MIB Objects Mapped to DPA	9

6.4  The Attribute Group Mapped to DPA	10

7.0  NOVELL DISTRIBUTED PRINT SERVICE (NDPS)	11

7.1  jmJobSubmissionId Mapped to NDPS	11

7.2  jmJobIndex Mapped to NDPS	11

7.3  Other MIB Objects Mapped to NDPS	12

7.4  The Attribute Group Mapped to NDPS	12

8.0  PRINTER JOB LANGUAGE (PJL)	13

8.1  jmJobSubmissionId Mapped to PJL	13

8.2  jmJobIndex Mapped to PJL	14

8.3  The Attribute Group Mapped to PJL	14

9.0  POSTSCRIPT	15

9.1  jmJobSubmissionId Mapped to PostScript	15

9.2  Other MIB Objects and Attributes Mapped to PostScript	15

10.0  NETWARE PSERVER	15

10.1  jmJobSubmissionId Mapped to PServer	15

10.2  jmJobIndex Mapped to PServer	16

10.3  The Attribute Group Mapped to PServer	16

11.0  NETWARE NPRINTER or RPRINTER	16

12.0  SERVER MESSAGE BLOCK (SMB) PROTOCOL	17

12.1  jmJobSubmissionId Mapped to SMB	17

12.2  jmJobIndex Mapped to SMB	17

12.3  Other MIB objects Mapped to SMB	17

13.0  TRANSPORT INDEPENDENT PRINTER/SYSTEM INTERFACE (TIP/SI)	18

13.1  jmJobSubmissionId Mapped to TIP/SI	18

13.2  jmJobIndex Mapped to TIP/SI	18

13.3  Other MIB Objects Mapped to TIP/SI	18

13.4  The Attribute Group Mapped to TIP/SI	18

14.0  REFERENCES	19

15.0  AUTHORS	19

1.0    INTRODUCTION

The Job Monitoring MIB [JobMIB] is intended to be implemented in a device or server that supports any job submission protocol.    However, the information available and the method of presentation varies significantly by job submission protocol.    A common method of mapping job submission information to the Job Monitoring MIB is essential for interoperability of Job MIB agents and monitoring applications.    This document defines recommended mappings for most popular job submission protocols to insure this compatibility.

All mappings are unidirectional from the job submission protocol to the MIB.    It is assumed that support of the job submission protocol in the printer implies that the reverse information flow is presently defined and does not require interaction from the MIB.    This mapping is not defined in this document as it should be obvious.

This document refers to system configurations that are defined in the Job Monitoring MIB [JobMIB].    For those readers that are familiar with the configuration descriptions, a short summary appears here.    Please see the Job MIB document for further details.

Configuration 1:    This is a simple peer-to-peer system which contains

only a client and a printer.    The Job MIB agent is resident in

the printer.

Configuration 2:    This system contains a client, server, and a

printer.    The Jib MIB agent is resident in the server.

Configuration 3:    This system, as in configuration 2, contains a

client, server, and a printer.    In this case the Job MIB agent is

implemented within the printer.

The most important object to be mapped is jmJobSubmissionID, since this is a method for the user or client to determine the jmJobIndex for a submitted job.    Therefore, jmJobSubmissionID is specified for all job submission protocols defined in this document.    The remaining objects mapped include only those items that have the equivalent information presented to the printer by the job submission protocol.

While this document places a strong emphasis on jmJobSubmissionID mapping to obtain jmJobIndex, the preferred method is through the use of a bi-directional protocol that returns the value of jmJobIndex to the client, such as IPP.    When a bi-directional protocol that returns jmJobIndex is in use, the jmJobSubmissionID object has no value to the client.    When the jmJobIndex cannot be returned, the use of a client defined jmJobSubmissionID is preferred over an agent derived value.    The client defined version allows for retrieval of jmJobIndex using a single SNMP Get operation, since jmJobSubmissionID is the index into the jmJobIDTable.    An agent derived value will require a search through multiple entries in the jmJobIDTable.

The majority of the protocols mapped in this document are oriented towards network job submission.    However, the Job Monitoring MIB is also intended to monitor print jobs received from other than network ports, such as parallel and serial ports.    Some of the job submission protocols included that are used with non-networked ports are PJL, PostScript, and TIP/SI.    In addition, the Job Monitoring MIB can be used with print jobs that are internally generated, such as self test pages.    In this latter case, no mapping is required since all job submission protocols are bypassed.

2.0    LINE PRINTER DAEMON (LPR/LPD) PROTOCOL

The LPR/LPD printing protocol [LPD] is used with BSD UNIX systems in the client-server-printer configuration.    Usage of the Job Monitoring MIB with LPR/LPD will most likely conform to Configuration 3, where the monitor application or the server uses SNMP to obtain job information from the printer.    The client communicates with the UNIX server using the existing LPD protocol to obtain job information.

The LPR/LPD protocol is also used in the Windows environment to implement peer-to-peer printing, as shown in configuration 1.    In this case, SNMP is used by the client and/or the monitor application to obtain the job information.

One of the major problems of LPR/LPD is the large number of vendor unique extensions currently used with the protocol and the resulting compatibility issues between available implementations.    To avoid these issues, this mapping of LPR/LPD is restricted to the protocol as defined by RFC 1179.

The LPR/LPD protocol transfers print job data and control information in separate files, known as the Data File and Control File, respectively.    Most of the information concerning the print job is contained in the Control File.    In many LPD implementations, the Control File is transferred following the Data File.    Thus much of the information concerning the job may not be available until the completion of the data transmission.

2.1    jmJobSubmissionID Mapped to LPR/LPD

The LPR/LPD Receive Data File command contains a parameter which defines the name of the data file.    This name field is structured as follows:

dfaXXX<host-name>    or    daXXXX<host-name>

Where XXX or XXXX is the numeric job number assigned by the LPR/LPD client submitting the print job.    The recommended mapping of this name field to jmJobSubmissionID is:

octet 1:      '9'

octets 2-40:    Contains the <host-name> portion of the name field.    If

the <host-name> portion is less than 40 octets, the

left-most character in the string shall appear in octet

position 2.    Any unused portion of this field shall be

filled with spaces.    Otherwise, only the last 39 bytes

shall be included.

octets 41-48:    '00000XXX' or '0000XXXX', where XXX or XXXX is the

decimal (ASCII coded) representation of the LPR/LPD

job number.

2.2    jmJobIndex Mapped to LPR/LPD

The job index (jmJobIndex) is assigned by the SNMP job monitoring agent and is independent of the XXX (or XXXX) index assigned by the LPR/LPD client.    This will allow the SNMP agent to track jobs received from multiple sources.

2.3    Other MIB Objects Mapped to LPR/LPD

MIB Object                                        | LPR/LPD Parameter

------------------------------+---------------------------------------

jmJobKOctetsPerCopyRequested    | Number of bytes as defined in the Data

| File

jmJobOwner                                        | Control file command code = P (User Id)

2.4    The Attribute Group Mapped to LPD

Other attributes that are applicable, but not defined in this section such as attributes that map to a vendor unique extension, may also be included.

MIB attribute                  | LPR/LPD information                          | Data type

----------------------+---------------------------------+--------------

jobName                              | Name of the data file (note 1)    | Octet String

queueNameRequested        | Queue name from the Data File      | Octet String

fileName                            | Source File Name (notes 2, 3)      | Octet String

documentName                    | Document title (notes 2, 4)          | Octet String

Notes:

------

1. See section 2.1 (jmJobSubmissionID).

2. The information is optional in the Control File.    The attribute

should be included if present in the Control File.

3. Control file command code = N.

4. Control file command code = J.

3.0    APPLETALK PROTOCOL

AppleTalk was originally developed as a peer-to-peer network protocol, as described in configuration 1, for use with Apple Macintosh computers.    Today, print spoolers are also available for use with Macintosh computer networks that conform to configurations 2/3.    In addition, printing with the AppleTalk protocol is supported from both Windows NT servers and Novell servers also per configurations 2/3.

The AppleTalk protocol provides very little information that can be used with the Job Monitoring MIB.    The Macintosh print drivers are able to provide information concerning the user and document name but imbed this information in the PDL, which is typically PostScript.    The preferred jmJobSubmissionID is constructed from the information in the PostScript file, as defined in section 9.0.

3.1    jmJobSubmissionID Mapped to AppleTalk

An alternative jmJobSubmissionID may be constructed from the Connection Identifier contained in the AppleTalk Printer Access Protocol (PAP) header.    Since the Connection Id is not readily available in any of the defined AppleTalk implementations, this approach may be of little utility.

octet 1:      'A'

octets 2-40:    Contains the AppleTalk printer name, with the first

character of the name in octet 2.    AppleTalk printer

names are a maximum of 31 characters.    Any unused

portion of this field shall be filled with spaces.

octets 41-48:    '00000XXX', where 'XXX' is the decimal (ASCII coded)

representation of the Connection Id.

3.2    Other AppleTalk Mappings

No other Job MIB objects or parameters can be derived from information available in the AppleTalk headers

4.0    INTERNET PRINTING PROTOCOL (IPP)

The Internet Printing Protocol [IPP] supports printing using any one of the three possible configurations.    For configuration 2, the mapping defined herein is performed on an agent within the server.    Otherwise, the mapping is performed on an agent within the printer.

4.1    jmJobSubmissionID Mapped to IPP

IPP contains a rich set of parameters which allow several methods of creating the jmJobSubmissionID object.    To prevent interoperability problems, the preferred method is to use the IPP job-uri attribute as follows:

octet 1:      '4'

octets 2-40:    Contains the IPP job-uri job description attribute

generated by the printer.    (The job-uri is returned to

the client by IPP.)    If the job-uri is less than 40

octets, the left-most character in the string shall

appear in octet position 2.    Any unused portion of this

field shall be filled with spaces.    Otherwise, only the

last 39 bytes shall be included.

octets 41-48:    Contains the decimal (ASCII coded) representation of

the job-id job description attribute.    Leading zeros

shall be inserted to fill the entire 8 octet field.

4.2    jmJobIndex Mapped to IPP

The job index (jmJobIndex) assigned by the SNMP job monitoring agent is returned to the client by IPP as the job-id job description attribute.    (Since IPP does not require consecutively generated job-ids, the agent may receive jobs from multiple clients and can assign jmJobIndex in an ascending sequence independent of the submitting job client.)    The IPP job-id must be restricted to the range of 1 to 99,999,999 (decimal) to allow the value to be properly represented in jmJobSubmissionID.

4.3    Other MIB Objects Mapped to IPP

MIB Object                                              | IPP Job attribute

---------------------------------+-----------------------------------

jmJobState                                              | job-state

jmJobStateReasons1                              | job-state-reasons (note 1)

jmNumberOfInterveningJobs                | number-of-intervening-jobs

jmJobKOctetsPerCopyRequested          | job-k-octets

jmJobKOctetsProcessed                        | job-k-octets-processed

jmJobImpressionsPerCopyRequested | job-impressions

jmJobImpressionsCompleted                | job-impressions-completed

jmJobOwner                                              | job-originating-user-name

Notes:

------

1. jmJobStateReasons1 is a bit map described in one object and three

attributes.    The IPP condition may change one or more of the bits

in one or more of these Job MIB items.

4.4    The Attribute Group Mapped to IPP

The following mappings are required if the listed IPP job template attribute is provided.

MIB attribute                            | IPP job attribute                        | Data type

---------------------------+------------------------------+-------------

jobStateReasonsN                      | job-state-reasons (note 3)      | Integer

jobCodedCharSet                        | attributes-charset (note 1)    | Octet String

jobNaturalLanguageTag            | attributes-natural-language    | Octet String

jobURI                                          | job-uri                                            | Octet String

jobName                                        | job-name                                          | Octet String

physicalDevice                          | output-device-assigned              | Octet String

numberOfDocuments                    | number-of-documents                    | Integer

jobPriority                                | job-priority                                  | Integer

jobHoldUntil                              | job-hold-until                              | Octet String

sides                                            | sides (note 2)                              | Integer

finishing                                    | finishings                                      | Integer

printQualityRequested            | print-quality                                | Integer

printerResolutionRequested | printer-resolution                      | Integer

jobCopiesRequested                  | copies (note 4)                            | Integer

documentCopiesRequested       | copies (note 4)                            | Integer

jobCollationType                      | multiple-document-handling      | Integer

sheetsRequested                        | job-media-sheets                          | Integer

sheetsCompleted                        | job-media-sheets-completed      | Integer

mediumRequested                        | media                                                | Octet String

jobSubmissionTime                    | time-at-submission                      | Integer

jobStartedProcessingTime      | time-at-processing                      | Integer

jobCompletionTime                    | time-at-completed                        | Integer

Notes:

------

1. jobCodedCharSet is an enum from the IANA registry which is also

used in the Printer MIB.    The IPP attributes-charset is the name

(MIME preferred name) of the character set.

2. The Job MIB sides attribute uses the integer values "1" and "2".

The IPP sides attribute uses three keywords.

3. jobStateReasonsN is a bit map described in one object and three

attributes.    The IPP condition may change one or more of the bits

in one or more of these Job MIB items.

4. The IPP "copies" attribute maps to the Job MIB:

(1) jobCopiesRequested when the job has only one document OR

IPP "multiple-document-handling" is 'single-valued'

(2) documentCopiesRequested, in which case the MIB value is the

total number of document copies that the job will produce as a

whole.

5.0    INTELLIGENT PRINTER DATA STREAM (IPDS)

6.0    DOCUMENT PRINTING APPLICATION (DPA)

The ISO 10175 Document Printing Application (DPA) [DPA] supports printing using any one of the three possible configurations.    For configuration 2, the mapping defined herein is performed on a server.    Otherwise, the mapping is performed on an agent within the printer.

6.1    jmJobSubmissionID Mapped to DPA

DPA contains a rich set of parameters which allow several methods of creating the jmJobSubmissionID object.    To prevent interoperability problems, the preferred method is to use the DPA job-originating-user attribute as follows:

octet 1:      '0'

octets 2-40:    Contains the DPA job-owner attribute

supplied by the submitter.    If the job-owner

is less than 40 octets, the left-most character in the

string shall appear in octet position 2.    Any unused

portion of this field shall be filled with spaces.

Otherwise, only the last 39 bytes shall be included.

octets 41-48:    Contains an 8-digit sequential decimal number.

6.2    jmJobIndex Mapped to DPA

The job index (jmJobIndex) assigned by the SNMP job monitoring agent is returned to the client by DPA as a decimal digit string as the value of the DPA job-identifier attribute.    (Since DPA does not require consecutively generated job-identifiers, the agent may receive jobs from multiple clients and can assign the jmJobIndex in an ascending sequence independent of the submitting job client.)    The DPA job-identifier must be restricted to the range of 1 to 99,999,999 (decimal) to allow the value to be properly represented in jmJobSubmissionID.

6.3    Other MIB Objects Mapped to DPA

MIB Object                                              | DPA Job attribute

---------------------------------+------------------------------------

jmJobState                                              | job-state

jmJobStateReasons1                              | job-state-reasons (note 2)

jmNumberOfInterveningJobs                | intervening-jobs

jmJobKOctetsPerCopyRequested          | total-job-octets (notes 1, 3)

jmJobKOctetsProcessed                        | job-octets-completed (note 1)

jmJobImpressionsPerCopyRequested | job-impression-count (note 3)

jmJobImpressionsCompleted                | impressions-completed

jmJobOwner                                              | job-owner

Notes:

------

1. jmJobKOctetsPerCopyRequested and jmJobKOctetsProcessed is in K

octets while the DPA job-total-octets and job-octets-completed is

in octets and is 63-bits of significance.

2. jobStateReasonsN is a bit map described in one object and three

attributes.    The DPA condition may change one or more of the bits

in one or more of these Job MIB items.    Also the DPA

job-state-reasons is a multi-valued attribute with each value being

an OBJECT IDENTIFIER (OID).

3. DPA octets include the multiplication factor due to job and

document copies, while the MIB values do not.

6.4    The Attribute Group Mapped to DPA

The following mappings are required if the listed DPA job attribute is provided.

MIB attribute                            | DPA job attribute                        |IPP Data type

---------------------------+------------------------------+-------------

jobStateReasonsN                      | job-state-reasons (note 2)      | Integer

jobCodedCharSet                        | (note 1)                                          | Octet String

jobAccountName                          | accounting-information              | Octet String

jobName                                        | job-name                                          | Octet String

deviceNameRequested                | printer-name-requested              | Octet String

physicalDevice                          | printers-assigned                        | Octet String

numberOfDocuments                    | number-of-documents                    | Integer

fileName                                      | file-name                                        | Octet String

documentName                              | document-name                                | Octet String

jobComment                                  | job-comment                                    | Octet String

documentFormat                          | document-format                            | Octet String

jobPriority                                | job-priority                                  | Integer

jobProcessAfterDateAndTime | job-print-after                            | Octet String

outputBin                                    | results-profile.output-bin      | Octet String

sides                                            | sides (note 3)                              | Integer

finishing                                    | job-finishing, finishing          | Integer

printQualityRequested            | print-quality                                | Integer

printerResolutionRequested | default-printer-resolution      | Integer

|    (note 4)                                        |

jobCopiesRequested                  | results-profile.job-copies      | Integer

jobCopiesCompleted                  | job-copies-completed                  | Integer

documentCopiesRequested       | copy-count (note 5)                    | Integer

documentCopiesCompleted        | copies-completed (note 6)        | Integer

sheetsRequested                        | job-media-sheet-count                | Integer

sheetsCompleted                        | job-media-sheets-completed      | Integer

pagesRequested                          | job-page-count                              | Integer

pagesCompleted                          | pages-completed                            | Integer

mediumRequested                        | page-media-select,                      | Octet String

|    default-medium                            |

jobSubmissionTime                    | submission-time (note 7)          | Octet String

jobStartedProcessingTime      | started-printing-time (note 7) Octet String

jobCompletionTime                    | completion-time (note 7)          | Octet String

Notes:

------

1. Every DPA attribute is tagged indicating the coded character set

to be used for that attribute.

2. jobStateReasonsN is a bit map described in one object and three

attributes.    The DPA condition may change one or more of the bits

in one or more of these Job MIB items.    Also the DPA

job-state-reasons is a multi-valued attribute with each value being

an OBJECT IDENTIFIER (OID).

3. The Job MIB sides attribute is an integer '1' or '2' while the DPA

sides attribute has one of six OID values that includes plex.

4. printerResolutionRequested has x and y resolution and is intended

to override the resolution instruction in the document, if any,

while the DPA default-printer-resolution is the same in x and y and

only takes effect if the document does not contain a resolution

instruction

5. The DPA "copy-count" attribute is a per-document attribute, so the

MIB value is the sum of the documents' "copy-count" values times

the job's "results-profile.job-copies" value.

6. The DPA "copies-completed" attribute is a per-document attribute,

so the MIB value is the sum of the documents' "copies-completed"

values times the job's "results-profile.job-copies" value.

7. The DPA GeneratlizedTime data type is defined by ISO 8824

(ISO-8824) while the MIB DateAndTime is defined by SNMPv2-TC.

7.0    NOVELL DISTRIBUTED PRINT SERVICE (NDPS)

Novell Distributed Print Services is a DPA based job submission protocol that conforms to configuration 3.

7.1    jmJobSubmissionID Mapped to NDPS

NDPS supports the generation of a properly formatted jmJobSubmissionID for use in the Job MIB, via the attribute ndps-att-job-identifier.

ISSUE: Is this the proper NDPS attribute or should the attribute ndps-att-identifier-on-client or ndps-att-new-job-identifier to be used?

7.2    jmJobIndex Mapped to NDPS

NDPS defines the attribute ndps-att-job-identifier-on-printer that can be used to return the value of jmJobIndex to the NDPS client.

7.3    Other MIB Objects Mapped to NDPS

MIB Object                                              | NDPS Parameter

---------------------------------+--------------------------------------

jmJobState                                              | ndps-att-current-job-state (note 1)

jmJobStateReasons1                              | ndps-att-job-state-reasons (note 2)

jmNumberOfInterveningJobs                | ndps-att-intervening-jobs

jmJobKOctetsPerCopyRequested          | ndps-att-total-job-octets

|    (notes 3, 4)

jmJobKOctetsProcessed                        | ndps-att-octets-completed (note 3)

jmJobImpressionsPerCopyRequested | ndps-att-job-impressions-count

jmJobImpressionsCompleted                | ndps-att-impressions-completed

jmJobOwner                                              | ndps-att-job-owner

|    (note 5)

Notes:

------

1. Some of the NDPS job states must be represented by both a

jmJobState and a jmJobStateReasons1 object or a jobStateReasonsN

attribute.

2. The NDPS job state reasons may be mapped to either the object

jmJobStateReasons1 or the attribute jobStateReasonsN.

3. jmJobKOctetsPerCopyRequested and jmJobKOctetsProcessed is in K

octets while the NDPS ndps-att-job-total-octets and

ndps-att-job-octets-completed is in octets and is 63-bits of

significance.

4. NDPS octets include the multiplication factor due to job and

document copies, while the MIB values do not.

5. The Job MIB object must be multiplied by the attribute

jobCopiesRequested to obtain the NDPS attribute value, if multiple

copies have been requested.

7.4    The Attribute Group Mapped to NDPS

The following mappings are required if the listed PJL attribute or command option is provided.

MIB attribute                            | NDPS parameter                              | Data type

---------------------------+------------------------------+-------------

jobAccountName                          | ndps-att-job-owner                      | Octet String

jobName                                        | ndps-att-job-name                        | Octet String

jobOriginatingHost                  | ndps-att-job-originator            | Octet String

deviceNameRequested                | ndps-att-printer-name--            | Octet String

|                    requested                      |

numberOfDocuments                    | ndps-att-number-of-documents | Integer

fileName                                      | ndps-att-document-file-name    | Octet String

documentName                              | ndps-att-document-name              | Octet String

jobComment                                  | ndps-att-job-comment                  | Octet String

documentFormatIndex                | ndps-att-prtInterpreterIndex | Integer

documentFormat                          | ndps-att-document-format          | Integer

jobPriority                                | ndps-att-job-priority                | Integer

jobProcessAfterDateAndTime | ndps-att-job-print-after          | Octet String

outputBin                                    | ndps-att-results-profile          | Integer

|    (note 1)                                        |

sides                                            | ndps-att-sides (note 2)            | Integer

finishing                                    | ndps-att-job-finishing              | Integer

printQualityRequested            | ndps-att-print-quality              | Integer

printerResolutionRequested | ndps-att-default-printer--      |

|    resolution (note 3)                  | Integer

printerResolutionUsed            | ndps-att-default-resolutions--

|    used                                                | Integer

jobCopiesRequested                  | ndps-att-results-profile          | Integer

|    (note 4)                                        |

jobCopiesCompleted                  | ndps-att-job-copies-completed| Integer

documentCopiesRequested        | ndps-att-copy-count                    | Integer

documentCopiesCompleted        | ndps-att-copies-completed        | Integer

|    (note 3)

sheetsRequested                        | ndps-att-job-media--                  |

|                                    sheet-count | Integer

sheetsCompleted                        | ndps-att-media-sheets--            |

|                                        completed | Integer

mediumConsumed                          | ndps-att-media-used                    | Integer

jobSubmissionToServerTime    | ndps-att-submission-time          | Octet String

jobSubmissionTime                    | ndps-att-started-printing-time Octet String

jobCompletionTime                    | ndps-att-completion-time          | Octet String

Notes:

------

1. The output-bin field in ndps-att-results-profile is to be used.

2. The Job MIB sides attribute is an integer '1' or '2' while the NDPS

sides attribute has one of six OID values that includes plex.

3. printerResolutionRequested has x and y resolution and is intended

to override the resolution instruction in the document, if any,

while the ndps-att-default-printer-resolution is the same in x and

y and only takes effect if the document does not contain a

resolution instruction

4. The job-copies field in ndps-att-results-profile is to be used.

8.0    PRINTER JOB LANGUAGE (PJL)

PJL [PJL] has been developed by Hewlett-Packard to provide job control information to the printer and status information to applications, independent of the PDL.

8.1    jmJobSubmissionID Mapped to PJL

PJL has defined the SUBMISSIONID option for the JOB command which indicates a properly formatted jmJobSubmissionID for use in the Job MIB.    The PJL JOB command is presented at the start of a print job with options that apply only the attached job.    The syntax for this command option is:

@PJL JOB SUBMISSIONID = "id string"

Driver software that implements this PJL command option must provide the "id string" in one of the client version formats specified in the Job MIB for jmJobSubmissionID.

For drivers that are not able to create the SUBMISSIONID option, it is recommended that jmJobSubmissionID format 0 be created by the agent using the PJL attribute DocOwner or DocOwnerId.

octet 1:      '0'

octets 2-40:    Contains the string associated with DocOwner or

DocOwnerId.    If the string is less than 40 octets, the

left-most character in the string shall appear in octet

position 2.    Otherwise, only the last 39 bytes shall be

included.    Any unused portion of this field shall be

filled with spaces.    If DocOwner or DocOwnerId cannot be

obtained, this field shall be blank.

octets 41-48:    Contains the value of jmJobIndex associated with the

job.    Leading zeros shall be inserted to fill the

entire 8 octet field.

8.2    jmJobIndex Mapped to PJL

PJL does not provide a value that can be mapped to jmJobIndex.

8.3    Other MIB Objects Mapped to PJL

MIB Object                        | PJL Job attribute

----------------------+------------------------------------

jobOwner                            | DocOwner or DocOwnerId attribute

8.4    The Attribute Group Mapped to PJL

The following mappings are required if the listed PJL attribute or command option is provided.

MIB attribute                  | PJL attribute or command option    | Data type

----------------------+----------------------------------+--------------

serverAssignedJobName | DocName attribute or the command | Octet String

|    @PJL JOB Name = "string"                | Octet String

submittingServerName    | SrcServerName attribute                    | Octet String

jobOriginatingHost        | SrcPort attribute                                | Octet String

queueNameRequested        | SrcQ attribute                                      | Octet String

fileName                            | JobFName attribute                              | Octet String

jobComment                        | JobDesc attribute                                | Octet String

jobSubmissionTime          | TimeSubmit attribute                          | Octet String

9.0    POSTSCRIPT

The PostScript PDL permits comment fields which can be used by application drivers to include job information.    Although there are no restrictions or requirements as to what information may be included, many drivers include job owner and/or document name.

9.1    jmJobSubmissionID Mapped to PostScript

The use of a standard format job submission id comment string will allow interoperability of printers and drivers from multiple vendors.    The following comment string format is recommended for use with PostScript level 1 and level 2 data streams.

%%JMPJobSubmissionId:(id-string)

where "id string" can be any jmJobSubmissionID format reserved for clients.

9.2    Other MIB Objects and Attributes Mapped to PostScript

No Other mappings from PostScript comment strings are recommended, but many Job MIB objects and attributes can be defined using vendor unique comment strings.

10.0    NETWARE PSERVER

The NetWare PServer job submission protocol is implemented in a client-server-printer system on the server to printer link as defined in configuration 3.

10.1    jmJobSubmissionID Mapped to PServer

octet 1:      'B'

octets 2-40:    Contains the Directory Path Name of the agent as

recorded by the Novell File Server in the queue

directory.    If the string is less than 40 octets, the

left-most character in the string shall appear in octet

position 2.    Otherwise, only the last 39 bytes shall be

included.    Any unused portion of this field shall be

filled with spaces.

octets 41-48:    '000XXXXX'    The decimal (ASCII coded) representation of

the Job Number as per the NetWare File Server Queue

Management Services.

10.2    jmJobIndex Mapped to PServer

The job index (jmJobIndex) is assigned by the SNMP job monitoring agent and is independent of the Job Number assigned by the NetWare File Server Queue Management Services.    This will allow the SNMP agent to track jobs received from multiple sources.

10.3    Other MIB Objects Mapped to PJL

MIB Object                        | PServer Job attribute

----------------------+------------------------------------

jobOwner                            | Client Id Number                        | Octet String

10.4    The Attribute Group Mapped to PServer

The following mappings are required if the listed PServer parameter is provided in the Novell File Server queue directory.

MIB attribute                            | PServer parameter                      | Data type

---------------------------+-----------------------------+--------------

serverAssignedJobName            | Job File Name                              | Octet String

queueNameRequested                  | Queue Id                                        | Integer

physicalDevice                          | Server Id Number                        | Integer

jobComment                                  | Job Description                          | Octet String

jobPriority                                | (note 1)                                        | Integer

jobProcessAfterDateAndTime | Target Execution Time              | Octet String

jobCopiesRequested                  | Number of Copies                        | Integer

mediumRequested                        | Form Name                                      | Octet String

jobSubmissionToServerTime    | Job Entry Time                            | Octet String

Notes:

------

1. The job priority is determined by the priority assigned to the queue

that contains the job.    Each queue can be assigned a unique priority

and the priority of the job is inherited from the queue.

11.0    NETWARE NPRINTER or RPRINTER

The NetWare NPrinter/RPrinter protocol was designed to transfer print data from a Novell File Server to a printer attached directly to a local port (e.g. parallel or serial) on a PC.    NPrinter/RPrinter is an extremely lightweight printing protocol.    Consequently, no information required by the Job Monitoring MIB is provided and a meaningful jmJobSubmissionID cannot be generated.

It is recommended that an additional job submission layer, such as PJL or another vendor private protocol,    be included on top of NPrinter/RPrinter to provide the required information.    The mapping should then be performed according to the recommendations of the higher layer submission protocol.

12.0    SERVER MESSAGE BLOCK (SMB) PROTOCOL

The Server Message Block protocol is used with several PC Network operating systems, such as Microsoft Windows for Workgroups, IBM LAN Server, and Artisoft Lantastic.    SMB systems supporting the Job Monitoring MIB will conform to either configuration 1 or 3.

12.1    jmJobSubmissionID Mapped to SMB

octet 1:      'C'

octets 2-40:    Contains a decimal (ASCII coded) representation of the

16 bit SMB Tree Id field, which uniquely identifies the

connection that submitted the job to the printer.    The

most significant digit of the numeric string shall be

placed in octet position 2.    All unused portions of this

field shall be filled with spaces.    The SMB Tree Id has

a maximum value of 65,535.

octets 41-48:    Contains a decimal (ASCII coded) representation of the

File Handle returned from the printer agent to the

client in response to a Create Print File command.

Leading zeros shall be inserted to fill the entire 8

octet field.

12.2    jmJobIndex Mapped to SMB

It is strongly recommended that the File Handle returned from the printer agent be identical to jmJobIndex.    If these items are identical,    there is no need for the client application to perform a search on jmJobSubmissionID.    To be compatible with the 16 bit field allocated to this value by SMB, the maximum jmJobIndex is 65,535.

12.3    Other MIB objects Mapped to SMB

MIB Object            | SMB Parameter

----------------+------------------------------------------------

jmJobOwner            | SMB User Id field (note 1)

Notes:

------

1. A decimal (ASCII coded) representation of the SMB User Id numeric

shall be presented as jmJobOwner.

13.0    TRANSPORT INDEPENDENT PRINTER/SYSTEM INTERFACE (TIP/SI)

The TIP/SI protocol, although currently specified as a part of the IEEE 1284 parallel port standards [TIP/SI], was originally developed as a network protocol.    TIP/SI thus has the potential of being integrated into any network or non-network configuration.

13.1    jmJobSubmissionID Mapped to TIP/SI

octet 1:      'D'

octets 2-40:    Contains the Job Name from the Job Control-Start Job

(JC-SJ) command.    If the Job Name portion is less than

40 octets, the left-most character in the string shall

appear in octet position 2.    Any unused portion of this

field shall be filled with spaces.    Otherwise, only the

last 39 bytes shall be included.

octets 41-48:    Contains a decimal (ASCII coded) representation of the

jmJobIndex assigned by the agent.    Leading zeros shall

be inserted to fill the entire 8 octet field.

13.2    jmJobIndex Mapped to TIP/SI

jmJobIndex is returned to the client as the Printer Assigned Job Id in a Job Control-Start Job (JC-SJ) response packet.    To be compatible with the 16 bit field allocated to this value by TIP/SI, the maximum jmJobIndex is 65,535.

13.3    Other MIB Objects Mapped to TIP/SI

MIB Object                          | TIP/SI Parameter

-----------------------+------------------------------------------------

jmJobOwner                          | User string

13.4    The Attribute Group Mapped to TIP/SI

MIB attribute                  | TIP/SI information                            | Data type

----------------------+---------------------------------+--------------

jobName                              | Job Name string                                  | Octet String

jobComment                        | Additional Information string      | Octet String

14.0    REFERENCES

[DPA]    ISO/IEC 10175-1:1996(E), "Information technology - Text and office systems - Document Printing Application (DPA) - Part 1: Abstract service definition and procedures", JTC1/SC18.

[IPP]    The Internet Printing Protocol RFC XXXX, Model RFC XXXX

[ISO-8824]    ISO/IEC 8824:1990, "Information technology - Open Systems Interconnection - Specification of Abstract Syntax Notation (ASN.1)".

[JobMIB]    The Job Monitoring MIB, work in progress, <draft-ietf-printmib-job-monitoring-07.txt>, to be published as an Informational RFC as a Printer Working Group (PWG) standard.

[LPD]    Line Printer Daemon Protocol, RFC 1179, IETF informational document.

[PJL]    Printer Job Language Technical Reference Manual, Hewlett-Packard part number 5021-0328.

[PrtMIB]    The Printer MIB, RFC 1759, IETF standards track document.

[TIP/SI]    IEEE Standard 1284.1, Transport Independent Printer/System Interface.

15.0    AUTHORS

This document was created with significant contributions from the following individuals.

Ron Bergman (Editor)

Dataproducts Corp.

1757 Tapo Canyon Road

Simi Valley, CA 93063-3394

Phone: 805-578-4421

Fax:    805-578-4001

Email: rbergman@dpc.com

Tom Hastings

Xerox Corporation, ESAE-231

701 S. Aviation Blvd.

El Segundo, CA      90245

Phone: 310-333-6413

Fax:      310-333-5514

EMail: hastings@cp10.es.xerox.com

Scott A. Isaacson

Novell, Inc.

122 E 1700 S

Provo, UT      84606

Phone: 801-861-7366

Fax:      801-861-4025

EMail: scott_isaacson@novell.com

Harry Lewis

IBM Corporation

6300 Diagonal Hwy

Boulder, CO 80301

Phone: (303) 924-5337

Fax:      (303) 924-4662

Email: harryl@us.ibm.com

Bob Pentecost

Hewlett-Packard Corporation

11311 Chinden Boulevard

Boise, ID 83714

Phone: (208) 396-3312

Fax:      (208) 396-4122

Email: bpenteco@boi.hp.com

Send comments to the printmib WG using the Job Monitoring Project

(JMP) Mailing List:    jmp@pwg.org

For further information, access the PWG web page under "JMP":

http://www.pwg.org/

Other Participants:

Chuck Adams - Tektronix

Keith Carter - IBM Corporation

Angelo Caruso - Xerox

Jeff Copeland - QMS

Andy Davidson - Tektronix

Mabry Dozier - QMS

Lee Ferrel - Canon

David Kellerman - Northlake Software

Rick Landau - Digital

Jay Martin - Underscore

Ira McDonald - Xerox

Stuart Rowley - Kyocera

Bob Setterbo - Adobe

Gail Songer - EFI

Mike Timperman - Lexmark

William Wagner - DPI/Osicom

Chris Wellens - Interworking Labs

Rob Whittle - Novell

Don Wright - Lexmark

Lloyd Young - Lexmark