V0.4 20061009.1200

Optics Group

The Optics Group contains all settings that relate to the physical optics of the projector, including focus, zoom, and keystoning adjustments, if these adjustments can be electromechanically controlled by the management agent.  Focus and zoom corrections will typically be mechanical; keystone corrections may be mechanical or electronic.

The properties in this group must be available for writing even when the projector is not fully powered up, that is, when the light source is not turned on.  A projector may have one or more reduced-power states in which communications are possible.  At least one of those states must permit writing a new value into the settings of this group.  Additionally, the values of settings must persist across power cycles.

If the projector is running in a low-power state (auxiliary power, also called standby power or flea power), then changing the setting value will in general not cause a mechanical action.  Mechanical adjustment will be made when the projector is powered on based on the current value of the setting, which might have changed while the projector was powered down.