Appendix D - Overall Printer Status Table

The following table defines a suggested relationship between various printer states and the variables Printer hrDeviceState, hrPrinterStatus, hrPrinterDetectedErrorState, prtAlertGroup, prtAlertCode,  and prtChannelStatus.  This table is the recommended implementation of these variables. It is provided to guide implementors of this MIB and users of the MIB by providing a sample set of states and the variable values that are expected to be produced as result of that state.

This information supplements that provided in Section 2.2.13.2 "Overall Printer Status". This is not an exhaustive list rather it is a guideline. More detailed status, if needed, should be obtained from the alert table and the sub-unit status variables.

|                                               |                               | Group Status

|           General Printer Status              |         prtAlertTable         | (PrtSubUnitStatusTC)

|-----------------------------------------------+-------------------------------+---------------------

|             |                 |               |               |               | prtInputStatus

|             |                 |               |               |               | (per input tray)

|             |                 |               |               |               | prtOutputStatus

|             |                 |               |               |               | (per output bin)

|             |                 |               |               |               | prtMarkerStatus

|             |                 |               |               |               | (per marker)

|             |                 |               |               |               | prtMediaPathStatus

|             |                 | hrPrinter     |               |               | (per path)

Printer            | hrDevice    | hrPrinter       | Detected      |               |               | prtChannelStatus

State              | Status      | Status          | ErrorState    | prtAlertGroup | prtAlertCode  | (per channel)

-------------------+-------------+-----------------+---------------+---------------+---------------+---------------------

Normal or idle     |             |                 |               |               |               | all groups are:

(no binary alerts) | running (2) | idle (3)        | (0x0)         | (no binary    | (no binary    |   Idle (0)

|             |                 |               |  alerts)      |               |

Printing           |             |                 |               |               |               | Availability may be:

(no binary alerts) | running (2) | printing (4)    | (0x0)         | (no binary    | (no binary    |   Idle (0) or

|             |                 |               |  alerts)      |  alerts)      |   Active (4) or

|             |                 |               |               |               |   Busy (6)

|             |                 |               |               |               | depending on

|             |                 |               |               |               | subsystem state

|             |                 |               |               |               |

Off-line           |             |                 |               |               |               | prtChannelStatus(32)

(no binary alerts) | down (5)    | other (1)       | Offline (2)   | (no binary    | (no binary    | other groups are:

|             |                 |               | alerts)       |  alerts)      |   (indeterminate)

|             |                 |               |               |               |

Standby or Power   |             |                 |               |               |               | all groups are:

Saver Mode         | running (2) | idle (3)        | (0x0)         | (no binary    | (no binary    |   Standby (2)

(no binary alerts) |             |                 |               |               |  alerts)      |

|             |                 |               |               |               |

Initial Power Up   |             |                 |               |               |               | all groups are:

(no binary alerts  | down (5)    | warmup (5)      | Offline (2)   | (no binary    | (no binary    |   Availability=

(while warming up) |             |                 |               |  alerts)      |  alerts)      |   Unknown+Off-line+

|             |                 |               |               |               |   Transitioning

|             |                 |               |               |               |   (5+32+64)

Warming Up         | running (2) | warmup (5)      | (0x0)         | (no binary    | (no binary    | all groups are:

(no binary alerts) |             |                 |               |  alerts)      |  alerts)      |   Availability=

|             |                 |               |               |               |   Standby+

|             |                 |               |               |               |   Transitioning

|             |                 |               |               |               |   (2+64)

Critical Errors: the printer can not print

(the error may be current while printing)

Jam                | down (5)    | other (1)       | Jammed(8)     | appropriate   | jammed(8)     | PrtSubUnitStatusTC

|             |                 |               | group         |               | (3+16+

|             |                 |               |               |               |  On-Line+

|             |                 |               |               |               |  Transitioning)

|             |                 |               |               |               |

Cover/Door         | down (5)    | other (1)       | Door Open(16) | cover(6)      | coverOpened(3)| prtCoverStatus

Open (listed       |             |                 |               |               | coverOpened   |    coverOpen (3)

in cover table)    |             |                 |               |               | (501)#        |

|             |                 |               |               |               |

Input Tray         | down (5)    | other (1)       | No Paper(64)  | input(8)      | subunitMissing| prtInputStatus

Missing            |             |                 |               |               | (9)           | (3+16+

|             |                 |               |               | inputMediaTray|  On-Line+

|             |                 |               |               | Missing(801)# |  Transitioning)

|             |                 |               |               |               |

Input Tray         | down (5)    | other (1)       | No Paper(64)  | input(8)      | subunitEmpty  | prtInputStatus

Empty              |             |                 |               |               | (13)          | (3+16+

|             |                 |               |               | inputMediaSup-|  On-Line+

|             |                 |               |               | plyEmpty(808)#|  Transitioning)

|             |                 |               |               |               |

Output Tray        | down (5)    | other (1)       | Service       | output(9)     | subunitMissing| prtOutputStatus

Missing            |             |                 | Requested(1)  |               | (9)           | (3+16+

|             |                 | + Offline (2) |               | outputMedia   |  On-Line+

|             |                 |               |               | TrayMissing   | Transitioning)

|             |                 |               |               | (901)#        |

|             |                 |               |               |               |

Output Tray        | down (5)    | other (1)       | Service       | output(9)     | subunitFull   | prtOutputStatus

Full               |             |                 | Requested(1)  |               | (15)          | (3+16+

|             |                 | + Offline (2) |               | outputMedia   |  On-Line+

|             |                 |               |               | TrayFull(903)#|  Transitioning)

|             |                 |               |               |               |

Marker Supply      | down (5)    | other (1)       | No Toner(16)  | markerSupplies| subunitMissing| prtMarkerStatus

Missing            |             |                 |               | (11)          | (9)           | (3+16+

( toner missing,   |             |                 |               |               | markerToner   |  On-Line+

ink missing )    |             |                 |               |               | Cartridge     |  Transitioning)

|             |                 |               |               | Missing(1115)#|

|             |                 |               |               |               |

Marker Supply      | down (5)    | other (1)       | No Toner(16)  | markerSupplies| subunitEmpty  | prtMarkerStatus

Empty (i.e.,       |             |                 |               | (11)          | (13)          | (3+16+

toner empty,      |             |                 |               |               | markerToner   |  On-Line+

ink low)          |             |                 |               |               | Empty (1101)# |  Transitioning)

|             |                 |               |               | markerInk     |

|             |                 |               |               | Empty (1102)# |

Non-Critical Errors: the printer can still print while condition exists

(the warning may be current while printing)

Input Tray         | warning (3) | idle (3) or     | Low           | input(8)      | subunitAlmost | prtInputStatus

Low                |             | printing (4) or | Paper(128)    |               | Empty(12)     | (8+Availability+

|             | warmup (5)      |               |               |               |  On-Line+

|             |                 |               |               |               |  Transitioning)

|             |                 |               |               |               |

Output Tray        | warning (3) | idle (3) or     | Service       | output(9)     | subunitAlmost | prtOutputStatus

Almost Full        |             | printing (4) or | Requested(1)  |               | Full(14)      | (8+Availability+

|             | warmup (5)      |               |               | outputTray    |  On-Line+

|             |                 |               |               | AlmostFull    |  Transitioning)

|             |                 |               |               | (902)#        |

|             |                 |               |               |               |

Marker Supply      | warning (3) | idle (3) or     | Low           | markerSupplies| subunitAlmost | prtMarkerStatus

Almost Empty       |             | printing (4) or | Toner (32)    | (11)          | Empty(12)     | (8+Availability+

(i.e., toner low,  |             | warmup (5)      |               |               | markerToner   |  On-Line+

ink low)          |             |                 |               |               | AlmostEmpty   |  Transitioning)

|             |                 |               |               | (1104)#       |

|             |                 |               |               | markerInk     |

|             |                 |               |               | AlmostEmpty   |

|             |                 |               |               | (1105)#       |

|             |                 |               |               |               |

Input Tray         | warning (3) | idle (3) or     | Low           | input(8)      | subunitMissing| prtInputStatus

Missing (when n-1  |             | printing (4) or | Paper(128)    |               | (9)           | (8+Availability+

trays are missing  |             | warmup (5)      |               |               | inputMediaTray|  On-Line+

with linking)      |             |                 |               |               | Missing(801)# |  Transitioning)

|             |                 |               |               |               |

Input Tray         | warning (3) | idle (3) or     | Low           | input(8)      | subunitEmpty  | prtInputStatus

Empty (when n-1    |             | printing (4) or | Paper(128)    |               | (13)          | (8+Availability+

trays are empty    |             | warmup (5)      |               |               | inputMediaSup-|  On-Line+

with linking)      |             |                 |               |               | plyEmpty(808)#|  Transitioning)

|             |                 |               |               |               |

Output Tray        | warning (3) | idle (3) or     | Service       | output(9)     | subunitMissing| prtOutputStatus

Missing (when n-1  |             | printing (4) or | Requested(1)  |               | (9)           | (8+Availability+

trays are missing  |             | warmup (5)      |               |               | outputMedia   |  On-Line+

with linking)      |             |                 |               |               | TrayMissing   | Transitioning)

|             |                 |               |               | (901)#        |

|             |                 |               |               |               |

Output Tray        | warning (3) | idle (3) or     | Service       | output(9)     | subunitFull   | prtOutputStatus

Full (when n-1     |             | printing (4) or | Requested(1)  |               | (15)          | (8+Availability+

trays are full     |             | warmup (5)      |               |               | outputMedia   |  On-Line+

with linking)      |             |                 |               |               | TrayFull(903)#|  Transitioning)

The definition of PrtSubUnitStatusTC specifies that SubUnitStatus is an integer that is the

sum of 5 distinct values, Availability, Critical, Non-Critical, On-line and Transitioning.

These values are...

# The generic codes are recommended for use. These type specific codes should not be used

in newer implementations of the printer MIB.