-- The Input Switching Group

--

-- The input switching group allows the administrator to set the manual

-- input subunit timeout for the printer and to control the automatic

-- input subunit switching by the printer when an input subunit becomes

-- empty.

--

-- This group is optional. However, to claim conformance to this group,

-- it is required to implement every object in the group.

prtInputMediaLoadTimeout OBJECT-TYPE

SYNTAX		Integer32

MAX-ACCESS	read-write

MIN-ACCESS	read-only

STATUS		current

DESCRIPTION

"The duration (in seconds) after which the printer shall

either:

(a) switch to another input subunit, if the value of

prtInputNextIndex is non-zero

or

(b) abort any job waiting for input, if the

value of prtInputNextIndex is zero

.

The event which causes the printer to enter the waiting state

is product specific. A value of (-1) implies 'other' or

'infinite' which translates to 'wait forever'.

A value of (-2) implies 'unknown'."

::= { prtInputEntry 24 }

prtInputNextIndex OBJECT-TYPE

SYNTAX		Integer32

MAX-ACCESS	read-write

MIN-ACCESS	read-only

STATUS		current

DESCRIPTION

"The value of prtInputIndex corresponding to the input subunit

which will be used when this input subunit is emptied and the

timeout specified by prtInputMediaLoadTimeout expires. A value

of zero(0) indicates that auto input switching will not occur

when this input subunit is emptied."

::= { prtInputEntry 26 }