IANA Considerations for MFD Alert Codes

The following new Printer Alert Codes have been defined as a result of the MFD Faults Analysis performed during the development of the new Alert Table Groups.

1.  subunitLocked(39)

2.  subunitUnlocked(40)

3.  subunitIniitiaizationFailure(41)

4.  subunitFanMotorFailure(42)

This section contains the exact registration information for IANA to update the IANA-PRINTER-MIB PrtAlertCodeTC Registry [IANAPTRMIB], according to the procedures defined in RFC 3805 [RFC3805] section 5, to cover the new group definitions defined in this document.

PrtAlertCodeTC ::= TEXTUAL-CONVENTION

-- This TC was extracted from prtAlertCode in RFC 1759.

STATUS    current

DESCRIPTION

"The code that describes the type of alert for this entry in

the table.  Binary change event alerts describe states of the

subunit while unary change event alerts describe a single

event.  The same alert code can be used for a binary change

event or a unary change event, depending on implementation.

Also, the same alert code can be used to indicate a critical

or non-critical (warning) alert, depending on implementation.

The value of prtAlertSeverityLevel specifies binary vs. unary

and critical vs. non-critical for each event for the

implementation.

While there are some specific codes for many subunits, the

generic codes should be used for most subunit alerts.  The

network management station can then query the subunit

specified by prtAlertGroup to determine further subunit

status and other subunit information.

An agent shall not add two entries to the alert table for the

same event, one containing a generic event code and the other

containing a specific event code; the agent shall add only

one entry in the alert table for each event; either generic

(preferred) or specific, not both.

Implementation of the unary change event

alertRemovalOfBinaryChangeEntry(1801) is optional.  When

implemented, this alert code shall indicate to network

management stations that the trailing edge of a binary change

event has occurred and the corresponding alert entry has been

removed from the alert table.  As with all events, the

alertRemovalOfBinaryChangeEntry(1801) alert shall be placed

at the end of the alert table.  Such an alert table entry

shall specify the following information:

prtAlertSeverityLevel   warningUnaryChangeEvent(4)

prtAlertTrainingLevel   noInterventionRequired(7)

prtAlertGroup           alert(18)

prtAlertGroupIndex      the index of the row in the

alert table of the binary

change event that this event

has removed.

prtAlertLocation        unknown (-2)

prtAlertCode            alertRemovalOfBinaryChangeEntry(1801)

prtAlertDescription     <description or null string>

prtAlertTime            the value of sysUpTime at

the time of the removal of the

binary change event from the

alert table.

Optionally, the agent may generate a trap coincident with

removing the binary change event and placing the unary change

event alertRemovalOfBinaryChangeEntry(1801) in the alert

table.  For such a trap, the prtAlertIndex sent with the above

trap parameters shall be the index of the

alertRemovalOfBinaryChangeEvent row that was added to the

prtAlertTable; not the index of the row that was removed from

the prtAlertTable."

SYNTAX    INTEGER {

other(1),

-- an event that is not represented

-- by one of the alert codes

-- specified below.

unknown(2),

-- The following generic codes are common to

-- multiple groups.  The NMS may examine the

-- prtAlertGroup object to determine what group

-- to query for further information.

coverOpen(3),

coverClosed(4),

interlockOpen(5),

interlockClosed(6),

configurationChange(7),

jam(8),

subunitMissing(9),           -- Not in RFC 1759

-- The subunit tray, bin, etc.

-- has been removed.

subunitLifeAlmostOver(10),   -- Not in RFC 1759

subunitLifeOver(11),         -- Not in RFC 1759

subunitAlmostEmpty(12),      -- Not in RFC 1759

subunitEmpty(13),            -- Not in RFC 1759

subunitAlmostFull(14),       -- Not in RFC 1759

subunitFull(15),             -- Not in RFC 1759

subunitNearLimit(16),        -- Not in RFC 1759

subunitAtLimit(17),          -- Not in RFC 1759

subunitOpened(18),           -- Not in RFC 1759

subunitClosed(19),           -- Not in RFC 1759

subunitTurnedOn(20),         -- Not in RFC 1759

subunitTurnedOff(21),        -- Not in RFC 1759

subunitOffline(22),          -- Not in RFC 1759

subunitPowerSaver(23),       -- Not in RFC 1759

subunitWarmingUp(24),        -- Not in RFC 1759

subunitAdded(25),            -- Not in RFC 1759

subunitRemoved(26),          -- Not in RFC 1759

subunitResourceAdded(27),    -- Not in RFC 1759

subunitResourceRemoved(28),  -- Not in RFC 1759

subunitRecoverableFailure(29),

-- Not in RFC 1759

subunitUnrecoverableFailure(30),

-- Not in RFC 1759

subunitRecoverableStorageError(31),

-- Not in RFC 1759

subunitUnrecoverableStorageError(32),

-- Not in RFC 1759

subunitMotorFailure(33),     -- Not in RFC 1759

subunitMemoryExhausted(34),  -- Not in RFC 1759

subunitUnderTemperature(35), -- Not in RFC 1759

subunitOverTemperature(36),  -- Not in RFC 1759

subunitTimingFailure(37),    -- Not in RFC 1759

subunitThermistorFailure(38), -- Not in RFC 1759

subunitLocked(39),           -- MFD addition

subunitUnlocked(40),         -- MFD addition

subunitInitializationFailure(41), -- MFD Addition

subunitFanMotorFailure(42),  -- MFD addition

-- General Printer group

doorOpen(501),    -- DEPRECATED

-- Use coverOpened(3)

doorClosed(502),  -- DEPRECATED

-- Use coverClosed(4)

powerUp(503),

powerDown(504),

printerNMSReset(505),        -- Not in RFC 1759

-- The printer has been reset by some

-- network management station(NMS)

-- writing into 'prtGeneralReset'.

printerManualReset(506),     -- Not in RFC 1759

-- The printer has been reset manually.

printerReadyToPrint(507),    -- Not in RFC 1759

-- The printer is ready to print. (i.e.,

-- not warming up, not in power save

-- state, not adjusting print quality,

-- etc.).

-- Input Group

inputMediaTrayMissing(801),

inputMediaSizeChange(802),

inputMediaWeightChange(803),

inputMediaTypeChange(804),

inputMediaColorChange(805),

inputMediaFormPartsChange(806),

inputMediaSupplyLow(807),

inputMediaSupplyEmpty(808),

inputMediaChangeRequest(809), -- Not in RFC 1759

-- An interpreter has detected that a

-- different medium is need in this input

-- tray subunit.  The prtAlertDescription may

-- be used to convey a human readable

-- description of the medium required to

-- satisfy the request.

inputManualInputRequest(810), -- Not in RFC 1759

-- An interpreter has detected that manual

-- input is required in this subunit.  The

-- prtAlertDescription may be used to convey

-- a human readable description of the medium

-- required to satisfy the request.

inputTrayPositionFailure(811), -- Not in RFC 1759

-- The input tray failed to position correctly.

inputTrayElevationFailure(812),

-- Not in RFC 1759

inputCannotFeedSizeSelected(813),

-- Not in RFC 1759

-- Output Group

outputMediaTrayMissing(901),

outputMediaTrayAlmostFull(902),

outputMediaTrayFull(903),

outputMailboxSelectFailure(904),

-- Not in RFC 1759

-- Marker group

markerFuserUnderTemperature(1001),

markerFuserOverTemperature(1002),

markerFuserTimingFailure(1003),

-- Not in RFC 1759

markerFuserThermistorFailure(1004),

-- Not in RFC 1759

markerAdjustingPrintQuality(1005),

-- Not in RFC 1759

-- Marker Supplies group

markerTonerEmpty(1101),

markerInkEmpty(1102),

markerPrintRibbonEmpty(1103),

markerTonerAlmostEmpty(1104),

markerInkAlmostEmpty(1105),

markerPrintRibbonAlmostEmpty(1106),

markerWasteTonerReceptacleAlmostFull(1107),

markerWasteInkReceptacleAlmostFull(1108),

markerWasteTonerReceptacleFull(1109),

markerWasteInkReceptacleFull(1110),

markerOpcLifeAlmostOver(1111),

markerOpcLifeOver(1112),

markerDeveloperAlmostEmpty(1113),

markerDeveloperEmpty(1114),

markerTonerCartridgeMissing(1115),

-- Not in RFC 1759

-- Media Path Device Group

mediaPathMediaTrayMissing(1301),

mediaPathMediaTrayAlmostFull(1302),

mediaPathMediaTrayFull(1303),

mediaPathCannotDuplexMediaSelected(1304),

-- Not in RFC 1759

-- Interpreter Group

interpreterMemoryIncrease(1501),

interpreterMemoryDecrease(1502),

interpreterCartridgeAdded(1503),

interpreterCartridgeDeleted(1504),

interpreterResourceAdded(1505),

interpreterResourceDeleted(1506),

interpreterResourceUnavailable(1507),

interpreterComplexPageEncountered(1509),

-- Not in RFC 1759

-- The interpreter has encountered a page

-- that is too complex for the resources that

-- are available.

-- Alert Group

alertRemovalOfBinaryChangeEntry(1801)

-- Not in RFC 1759

-- A binary change event entry has been

-- removed from the alert table.  This unary

-- change alert table entry is added to the

-- end of the alert table.

}