| Printer MIB Working Group | Ron Bergman |
| --- | --- |
| [Request for Comments: XXXX] | Hitachi Printing Solutions |
| [Obsoletes: RFC 1759] | Harry Lewis |
| [Target Category: Standards Track] | IBM Corp. |
|  | Ira McDonald |
|  | High North Inc |
| Expires 10 August 2003 | 10 February 2003 |

Printer MIB v2

<draft-ietf-printmib-mib-info-14.txt>

Status of this Memo

This document is an Internet-Draft and is in full conformance with all provisions of Section 10 of RFC 2026 [RFC2026].

Internet-Drafts are working documents of the Internet Engineering Task Force (IETF), its areas, and its working groups. Note that other groups may also distribute working documents as Internet-Drafts.

Internet-Drafts are draft documents valid for a maximum of six months and may be updated, replaced, or made obsolete by other documents at any time. It is inappropriate to use Internet-Drafts as reference material or to cite them other than as "work in progress."

To learn the current status of any Internet-Draft, please check the "1id-abstracts-txt" listing contained in the Internet-Drafts Shadow Directories on ftp.is.co.za (Africa), nic.nordu.net (Europe), munnari.oz.au (Pacific Rim), ftp.ietf.org (US East Coast), or ftp.isi.edu (US West Coast).

For tips on printing this document or on modifying the original Microsoft Word source document, see [RFC3285].

Copyright Notice

Copyright (C) The Internet Society (2002).  All Rights Reserved.

Abstract

This document provides definitions of models and manageable objects for printing environments. The objects included in this MIB apply to physical, as well as logical entities within a printing device. This document obsoletes RFC 1759.

Table of Contents

Status of this Memo	1

Abstract	1

1.  Introduction	4

1.1  Network Printing Environment	4

1.2  Printer Device Overview	5

1.3  Categories of Printer Information	6

1.3.1  Descriptions	6

1.3.2  Status	6

1.3.3  Alerts	6

1.4  The Internet-Standard Management Framework	7

1.5 Requirement Levels	7

2.  Printer Model	7

2.1  Overview of the Printer Model	10

2.2  Printer Sub-Units	10

2.2.1  General Printer	10

2.2.1.1  International Considerations	11

2.2.2  Inputs	11

2.2.3  Media	12

2.2.4  Outputs	12

2.2.5  Finishers	12

2.2.6  Markers	13

2.2.7  Media Paths	13

2.2.8  System Controller	14

2.2.9  Interfaces	14

2.2.10  Print Job Delivery Channels	14

2.2.11  Interpreters	15

2.2.12  Console	15

2.2.13  Alerts	15

2.2.13.1  Status and Alerts	16

2.2.13.2  Overall Printer Status	16

2.2.13.2.1  Host Resources MIB Printer Status	18

2.2.13.2.2  Sub-unit Status	22

2.2.13.3  Alert Tables	23

2.2.13.4  Alert Table Management	23

2.3  Read-Write Objects	25

2.4  Enumerations	26

2.4.1  Registering Additional Enumerated Values	27

3.  Groups from other MIB Specifications	27

3.1  System Group	27

3.2  System Controller	27

3.3  Interface Group objects	28

3.3.1  Interface Types	28

4.  Differences from RFC 1759	28

5.  The IANA Printer MIB	31

6.  The Printer MIB	60

-- Textual conventions for this MIB module	63

-- The General Printer Group	71

-- The Responsible Party group	75

-- The Auxiliary Sheet Group	78

-- Administrative section  (The General V2 Group)	79

-- General alert table section  (Alert Table V2 Group)	79

-- The Cover Table	80

-- The Localization Table	82

-- The System Resources Tables	83

-- The Input Group	86

-- The Extended Input Group	92

-- The Input Media Group	93

-- The Input Switching Group	95

-- The Output Group	96

-- The Extended Output Group	100

-- The Output Dimensions Group	101

-- The Output Features Group	103

-- The Marker Group	105

-- The Marker Supplies Group	111

-- The Marker Colorant Group	114

-- The Media Path Group	117

-- The Print Job Delivery Channel Group	121

-- The Interpreter Group	129

-- The Console Group	133

-- The Alerts Group	138

-- Conformance Information	143

7.  IANA Considerations	157

8.  Intellectual Property	157

9.  Internationalization Considerations	158

10.  Security Considerations	158

11.  Copyright Section	160

12.  Normative References	161

13.  Informative References	162

Appendix A - Glossary of Terms	163

Appendix B - Media Size Names	168

Appendix C - Media Names	170

Appendix D - Roles of Users	175

Appendix E - Overall Printer Status Table	179

Appendix F - Participants	180

Authors' Addresses	181

1.  Introduction

1.1  Network Printing Environment

The management of producing a printed document, in any computer environment, is a complex subject. Basically, the task can be divided into two overlapping pieces, the management of printing and the management of the printer. Printing encompasses the entire process of producing a printed document from generation of the file to be printed, selection of a printer, choosing printing properties, routing, queuing, resource management, scheduling, and final printing including notifying the user.  Most of the printing process is outside the scope of the model presented here; only the management of the printer is covered.

Figure 1 - One Printer's View of the Network

system   printer    asset     user          user           user

manager  operator   manager

O         O         O         O             O              O

/|\       /|\       /|\       /|\           /|\            /|\

/ \       / \       / \       / \           / \            / \

|         |         |         |             |              |

+---------+ +-------+ +-------+ +-------+   +-----------+ +-----------+

|configur-| |printer| | asset | |printer|   |   user    | |   user    |

|ator     | |manager| |manager| |browser|   |application| |application|

+---------+ +-------+ +-------+ +-------+   +-----------+ +-----------+

^            ^         ^         ^             |             |

|R/W         |R/W      |R        |R      +-----------+ +-----------+

|            |         |         |       |  spooler  | |  spooler  |

|            |         |         |       +-----------+ +-----------+

|            |         |         |             |             |

|            |         |         |       +-----------+ +-----------+

|            |         |         |       |supervisor | |supervisor |

|            |         |         |       +-----------+ +-----------+

|            |         |         |        ^       ^     ^       ^

|            |         |         |        |R      |R/W  |R      |R/W

v            v         |         |        |       |     |       |

==================================================   |   =====     |

|                          print|        print|

|SNMP                       data|         data|

+-----+        +-------+                        PCL|          PCL|

| MIB |<------>| agent |                 PostScript|   PostScript|

+-----+        +-------+                       NPAP|         NPAP|

|unspecified                etc.|         etc.|

+=============+  +-----------------+   |             |

|             |--|channel/interface|<--+             |

|             |  +-----------------+                 |

|   PRINTER   |                                      |

|             |  +-----------------+                 |

|             |--|channel/interface|<----------------+

+=============+  +-----------------+

1.2  Printer Device Overview

A printer is the physical device that takes media from an input source, produces marks on that media according to some page description or page control language and puts the result in some output destination, possibly with finishing applied. Printers are complex devices that consume supplies, produce waste and may have mechanical problems. In the management of the physical device the description, status and alert information concerning the printer and its various subparts has to be made available to the management application so that it can be reported to the end user, key operators for the replenishment of supplies or the repair or maintenance of the device. The information needed in the management of the physical printer and the management of a printing job overlap highly and many of the tasks in each management area require the same or similar information.

1.3  Categories of Printer Information

Information about printers is classified into three basic categories: descriptions, status and alerts.

1.3.1  Descriptions

Descriptions convey information about the configuration and capabilities of the printer and its various sub-units. This information is largely static information and does not generally change during the operation of the system but may change as the printer is repaired, reconfigured or upgraded. The descriptions are one part of the visible state of the printer where state means the condition of being of the printer at any point in time.

1.3.2  Status

Status is the information regarding the current operating state of the printer and its various sub-units. As an example of the use of status, a management application must be able to determine if the various sub-units are ready to print or are in some state that prevents printing or may prevent printing in the future.

1.3.3  Alerts

An Alert is the representation of a reportable event in the printer. An event is a change in the state of the printer. Some of those state changes are of interest to a management application and are therefore reportable. Typically, these are the events that affect the printer's ability to print. Alerts usually occur asynchronously to the operation of the computer system(s) to which the printer is attached. For convenience below, "alert" will be used for both the event caused by a change in the printer's state and for the representation of that event.

Alerts can be classified into two basic categories, critical and non-critical.  A critical alert is one that is triggered by entry into a state in which the printer is stopped and printing can not continue until the condition that caused the critical alert is eliminated. "Out of paper", "toner empty" and "output bin full" are examples of critical alerts. Non-critical alerts are triggered by those events that enter a state in which printing is not stopped. Such a non-critical state may, at some future time, lead to a state in which printing may be stopped.  Examples of these kinds of non-critical alerts are "input media low", "toner low" and "output bin nearly full". Or, a non-critical alert may simply provide information, such as signaling a configuration changed in the printer.

Description, status and alert information about the printer can be thought of as a database describing the printer. The management application for a printer will want to view the printer data base differently depending on how and for what purposes the information in the database is needed.

1.4  The Internet-Standard Management Framework

For a detailed overview of the documents that describe the current

Internet-Standard Management Framework, please refer to section 7 of

RFC 3410 [RFC3410].

Managed objects are accessed via a virtual information store, termed

the Management Information Base or MIB.  MIB objects are generally

accessed through the Simple Network Management Protocol (SNMP).

Objects in the MIB are defined using the mechanisms defined in the

Structure of Management Information (SMI).  This memo specifies a MIB

module that is compliant to the SMIv2, which is described in STD 58,

RFC 2578 [RFC2578], STD 58, RFC 2579 [RFC2579] and STD 58, RFC 2580

[RFC2580].

1.5 Requirement Levels

The key words "MUST", "MUST NOT", "REQUIRED", "SHALL", "SHALL NOT",

"SHOULD", "SHOULD NOT", "RECOMMENDED", "MAY", and "OPTIONAL" in this

document are to be interpreted as described in [RFC2119].

Compliant implementations must follow this specification.

2.  Printer Model

In order to accomplish the management of the printer, an abstract model of the printer is needed to represent the sub-units from which the printer is composed. A printer can be described as consisting of 13 types of sub-units. It is important to note that the sub-units of a printer do not necessarily relate directly to any physically identifiable mechanism. Sub-units can also be a set of definable logical processes, such as interpreters for page description languages or command processors that set various operating modes of the printer.

Figure 2 shows a block diagram of the printer and its basic 13 sub-units.

Figure 2 - Printer Block Diagram

Physical Connections

|

+-----------+

|           |

+-------------+ |

|  Interface  |-+

|    MIB-II   |

+-------------+

|

+-----------+

|           |

+-------------+ |    +-----------+

| Channel     |-+    | Operator  |

|             |      |  Console  |

+-------------+      +-----------+

|

+-----------+        +---------+

|           |        |         |

+-----------+       +-------------+ |    +-----------+ |

|  General  |       | Interpreter |-+    |  Alerts   |-+

|  Printer  |       |             |      |           |

+-----------+       +-------------+      +-----------+

|

+-------------------------------+

|        System Controller      |

|        HOST-RESOURCES-MIB     |

+-------------------------------+

+------+                    +--------+                  +--------+

|      |                    |        |                  |        |

+-------+ |    +-------+    +---------+ |    +-------+   +--------+ |

| Input |-+  +--------+|    |  Marker |-+  +--------+|   | Output |-+

|       |===>|        |+<==>|         |<==>|        |+==>|        |

+-------+    +--+  +--+     +---------+    +--+  +--+    +--------+

\            |  ||                         |  ||         \

\           |  ||                         |  ||          \

\          |  ||                         |  ||           \

+--------+  |  |+-------------------------|  ||         +---------+

|        |  |  +--------------------------+  ||         |         |

+----------+ |  |            Media Path          |+      +----------+ |

|  Media   |-+  +--------------------------------+       | Finisher |-+

|(optional)|                                             |(optional)|

+----------+                                             +----------+

2.1  Overview of the Printer Model

The model has three basic parts: (1) the flow of a print file into an interpreter and onto the marker, (2) the flow of media through the marker and (3) the auxiliary sub-units that control and facilitate the two prior flows.  The flow of the print data comes through a physical connection on which some form of transport protocol stack is running.  The data provided by the transport protocol (interface) appears on a channel, which is the input to an interpreter. The interpreter converts the print data into a form suitable for marking on the media.

The media resides in Input sub-units from which the media is selected and then transported via a Media Path first to a Marking sub-unit and then onto an Output sub-unit with (optionally) some finishing operations being performed.  The auxiliary sub-units facilitate control of the printer, inquiry/control of the operator panel, reporting of alerts and the adaptation of the printer to various natural languages and characters sets. All the software sub-units run on the System Controller that represents the processor, memory and storage systems of the Printer.  Each of the sub-units is discussed in more detail below.

All of the sub-units other than the Alerts report only state information, either a description or a status. The Alerts sub-unit reports event information.

2.2  Printer Sub-Units

A printer is composed of 13 types of sub-units, called groups.  The following sections describe the different types of sub-units.

2.2.1  General Printer

The general printer sub-unit is responsible for the overall control and status of the printer. There is exactly one general printer sub-unit in a printer. The General Printer Group in the model represents the general printer sub-unit. In addition to the providing the status of the whole printer and allowing the printer to be reset, this Group provides information on the status of the packaging of the printer, in particular, the covers. The general printer sub-unit is usually implemented on the system controller.

2.2.1.1  International Considerations

The localization portion of the general printer sub-unit is responsible for identifying the natural language, country, and character set in which certain character strings are expressed in this MIB.  Character sets are identified in this MIB using the IANACharset textual convention imported from the IANA Character Set MIB [CHARMIB].

There may be one or more localizations supported per printer. The available localizations are specified in the Localization table. Localization SHOULD only be performed on string objects which are named 'xxxDescription' (sub-unit descriptions) or 'prtConsoleDisplayBufferText' (local console text).

The agent SHALL return all other character strings in coded character sets in which code positions 0-127 (decimal) are US-ASCII [ASCII].  The agent SHOULD return all other character strings in the UTF-8 [RFC2279] transform of ISO 10646 [ISO10646], to conform with the IETF Policy on Character Sets and Languages [RFC2277]. Control codes (code positions 0-31 and 127 decimal) SHALL NOT be used unless specifically required in the DESCRIPTION of an object.

The character set portion of the general printer Localization table is responsible for identifying the possible character sets for the operator console, and network management requests for display objects. There may be one or more character sets per printer. Default coded character sets for interpreter unit and output octets are described in the interpreter sub-unit by prtInterpreterDefaultCharSetIn and prtInterpreterDefaultCharSetOut. These input/output character sets may be overridden by commands in the interpreter language itself.

2.2.2  Inputs

Input sub-units are mechanisms that feed media to be marked on into the printer. A printer contains one or more input sub-units. The Input Group in the model represents these. The model does not distinguish fixed input bins from removable trays, except to report when a removable tray has been removed.

There are as many input sub-units as there are distinctly selectable input "addresses".  For example, if one tray has both a manual and auto feeding option, then this is two input sub-units if these two sources can be (must be) separately selected. However, the above would be considered one input sub-unit if putting a sheet in the manual feed slot overrides feeding from the contents of the tray. In the second case there is no way to separately select or address the manual feed slot.

2.2.3  Media

An input sub-unit can hold one or more instances of the media on which marking is to be done. Typically, there is a large set of possible media that can be associated with an input. The Media Group is an extension of the Input Group, which represents media in an input sub-unit. The Media Group only describes the current contents of each input and not the possible content of the input sub-unit.

2.2.4  Outputs

Output sub-units are mechanisms that receive media that has been marked on. The Output Group in the model represents the one or more output mechanisms contained by a printer. The model does not distinguish fixed output bins from removable output bins, except to report when a removable bin has been removed.

There are as many output sub-units as there are distinctly selectable output "addresses".  Output sub-units can be addressed in two different ways: (1) as a set of "mailboxes" which are addressed by a specific mailbox selector such as a bin number or a bin name, or (2) as a set of "slots" into which multiple copies are collated. Sometimes both modes of using the output sub-units can be used on the same printer.  All that is important from the viewpoint of the model is that the output units can be separately selected.

2.2.5  Finishers

A finisher is a sub-unit that performs some operations on the media other than marking.  The Finisher Group in the model represents the finisher sub-units.  Some examples of finishing processes are stapling, punching, binding, inserting, or folding.  Finishing processes may have supplies associated with the process.  Stapling, binding, and punching are examples of processes that have supplies. A printer may have more than one finishing sub-unit and each finishing sub-unit may be associated with one or more output sub-units. Finishers are described in the companion Finisher MIB [RFCnnnn].

Note to RFC Editor:  Insert RFC number 'nnnn' when assigned.

The model does not specify the exact interaction and sequencing between an output device and its associated finisher. It depends on the type of finishing process and the exact implementation of the printer system. This standard allows for the logical association of a finishing process with an output device but does not put any restrictions on the exact sequence or interaction with the associated output device. The output and finisher sub-units may or may not be separate identifiable physical mechanisms depending on the exact implementation of a printer.  In addition, a single output device may be associated with multiple finishing sub-units and a single finishing sub-unit may be associated with multiple output devices.

2.2.6  Markers

A marker is the mechanism that produces marks on the print media. The Marker Group in the model represents the marker sub-units and their associated supplies. A printer can contain one or more marking mechanisms.  Some examples of multiple marker sub-units are a printer with separate markers for normal and magnetic ink or an imagesetter that can output to both a proofing device and final film. Each marking device can have its own set of characteristics associated with it, such as marking technology and resolution.

In this model the marker sub-unit is viewed as very generalized and encompasses all aspects of a marking process. For example, in a xerographic process, the marking process as well as the fusing process would be included in the generalized concept of the marker. With the generalized concept of a marking process, the concept of multiple marking supplies associated with a single marking sub-unit results. For example, in the xerographic process, there is not only a supply of toner, but there can also be other supplies such as a fuser supply (e.g., fuser oil) that can be consumed and replaced separately. In addition there can be multiple supplies of toner for a single marker device, as in a color process.

2.2.7  Media Paths

The media paths encompass the mechanisms in the printer that move the media through the printer and connect all other media related sub-units: inputs, outputs, markers and finishers. A printer contains one or more media paths. The Media Path Group in the model represents these.  The Media Path group has some objects that apply to all paths plus a table of the separate media paths.

In general, the design of the media paths determines the maximum speed of the printer as well as the maximum media size that the printer can handle. Media paths are complex mechanisms and can contain many different identifiable sub-mechanisms such as media movement devices, media buffers, duplex units and interlocks. Not all of the various sub-mechanisms reside on every media path.  For example, one media path may provide printing only on one surface of the media (a simplex path) and another media path may have a sub-mechanism that turns the media over and feeds it a second time through the marker sub-unit (a duplex path).  The duplex path may even have a buffer sub-mechanism that allows multiple copies of the obverse side to be held before the reverse side of all the copies is marked.

2.2.8  System Controller

The System Controller is the sub-unit upon which the software components of the Printer run. The Host Resources MIB [RFC2790] represents the System Controller in the model. The Host Resources MIB allows for the specification of the processor(s), memory, disk storage, file system and other underlying sub-mechanisms of the printer. The controller can range from simple single processor systems to multiprocessor systems. In addition, controllers can have a full range of resources such as hard disks. The printer is modeled to have one system controller even though it may have more than one processor and multiple other resources associated with it.

2.2.9  Interfaces

An interface is the communications port and associated protocols that are responsible for the transport of data to the printer. A printer has one or more interface sub-units. The interfaces are represented by the Interfaces Group of MIB-II [RFC1213], [RFC2863]. Some examples of interfaces are serial ports (with little or no protocol) and Ethernet ports on which one might run Internet IP, Novell IPX, etc.

2.2.10  Print Job Delivery Channels

The print job delivery channel sub-units identify the independent sources of print data (here print data is the information that is used to construct printed pages and may have both data and control aspects).  A printer may have one or more channels. The channel sub-units are represented by the Print Job Delivery Channel Group in the Model. The electronic path typically identifies each channel and service protocol used to deliver print data to the printer. A channel sub-unit may be independently enabled (allowing print data to flow) or disabled (stopping the flow of print data). It has a current Control Language that can be used to specify which interpreter is to be used for the print data and to query and change environment variables used by the interpreters (and SNMP). There is also a default interpreter that is to be used if an interpreter is not explicitly specified using the Control Language. Print Job Delivery Channel sub-units can, and usually are, based on an underlying interface.

2.2.11  Interpreters

The interpreter sub-units are responsible for the conversion of a description of intended print instances into images that are to be marked on the media. A printer may have one or more interpreters. The Interpreter Group in the Model represents the interpreter sub-units. Each interpreter is generally implemented with software running on the System Controller sub-unit. The Interpreter Table has one entry per interpreter where the interpreters include both Page Description Language (PDL) Interpreters and Control Language Interpreters.

2.2.12  Console

Many printers have a console on the printer, the operator console that is used to display and modify the state of the printer.  The console can be as simple as a few indicators and switches or as complicated as full screen displays and keyboards. There can be at most one such console.  The Console Group in the model represents this console sub-unit.  Although most of the information displayed there is also available in the state of the printer as represented by the various Groups, it is useful to be able to query and modify the operator console remotely.  For example, a management application might like to display to its user the current message on the operator console of the remote printer or the management application user might like to modify the current message on the operators console of the remote printer.  As another example, one might have a remote application that puts up a pseudo console on a workstation screen. Since the rules by which the printer state is mapped onto the console and vice versa are not standardized, it is not possible to reproduce the console state or the action of console buttons and menus. Therefore, the Console Group provides access to the console. The operator console is usually implemented on the system controller with additional hardware for input and display.

2.2.13  Alerts

The alert sub-unit is responsible for detecting reportable events, making an entry in the alert table and, if and only if the event is a critical event, initiating a trap. The exception to this rule is when the "alertRemovalofBinaryChangeEntry" trap is generated. The alert sub-unit is represented by the Alerts Group and, in particular, the Alert Table. This table contains information on the severity, sub-unit, and detailed location within the sub-unit, alert code and description of each alert that is currently active within the printer. Each reportable event causes an entry to be made in the Alert Table.

2.2.13.1  Status and Alerts

Summary information about the state of the printer is reported at three separate levels: (1) The status of the printer as a whole is reported in the Host Resources MIB, (2) The status of various sub-units is reported in the principle table of the Group that represents the sub-unit, and (3) Alert codes are reported in the Alert Table.

2.2.13.2  Overall Printer Status

Of the many states a printer can be in, certain states are more "interesting" because of the distinct actions they are likely to provoke in the administrator.  These states may be applied to the printer as a whole, or to a particular sub-unit of the printer. These named states are:

Non Critical Alert Active - For the printer this means that one or more sub-units have a non-critical alert active.  For a sub-unit, this means that the sub-unit has a non-critical alert active.

Critical Alert Active - For the printer this means that one or more sub-units have a critical alert active.  For a sub-unit, this means that the sub-unit has a critical alert active.

Unavailable - The printer or sub-unit is unavailable for use (this is the same as "broken" or "down" in other terminology).  A trained service person is typically necessary to make it available.

Moving on-line or off-line - The printer is either off-line, in the process of moving off-line or moving back on-line. For example, on printers with motorized hoppers, reloading paper involves a transition to off-line to open the paper bin, filling the hopper and, finally, a transition back to on-line as the paper bin is repositioned for printing.

Standby - The printer or sub-unit is not immediately available but can accept new instructions.

Available - The printer or subunit is functioning normally.

Idle - The printer or subunit is immediately available.

Active - The printer or subunit is performing its primary function.

Busy - The printer or subunit is performing a function (not necessarily its primary function) and is not immediately available for its primary function.

The Host Resources MIB [RFC2790] provides three status objects that can be used to describe the status of a printer: (1) hrDeviceStatus in the entry in the hrDeviceTable; (2) hrPrinterStatus in the hrPrinterTable; and (3) hrPrinterDetectedErrorState in the hrPrinterTable.  These objects describe many of the states that a printer can be in.  The following table shows how the values of the three printer-related objects in the Host Resources MIB relate to the states named above:

Printer       hrDeviceStatus hrPrinterStatus hrPrinterDetected-

Status                                       ErrorState

Idle           running(2)     idle(3)        none set

Busy/          running(2)     printing(4)

Active

Non Critical   warning(3)     idle(3) or     could be: lowPaper,

Alert Active                  printing(4)    lowToner, or

serviceRequested

Critical       down(5)        other(1)       could be: jammed,

Alert Active                                 noPaper, noToner,

coverOpen, or

serviceRequested

Unavailable    down(5)        other(1)

Moving off-    warning(3)     idle(3) or     offline

line                          printing(4)

Off-line       down(5)        other(1)       offline

Moving         down(5)        warmup(5)

on-line

Standby        running(2)     other(1)

These named states are only a subset of the possible states - they are not an exhaustive list of the possible states.  Nevertheless, several things should be noted.  When using these states, it is not possible to detect when both critical and non-critical alerts are pending - if both are pending, the Critical Alert Active state will prevail.  In addition, a printer in the Standby state will be represented in the Host Resources MIB with a device status of running(2) and a printer status of other(1), a set of states that don't uniquely distinguish this important printer state.

Detailed status per sub-unit is reported in the sub-unit status fields.

2.2.13.2.1  Host Resources MIB Printer Status

For completeness, the definitions of the Printer Status objects of the Host Resources MIB [RFC2790] are given below:

hrDeviceStatus OBJECT-TYPE

SYNTAX  INTEGER {

unknown(1),

running(2),

warning(3),

testing(4),

down(5)

}

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The current operational state of the device

described by this row of the table.  A value

unknown(1) indicates that the current state of the

device is unknown.  running(2) indicates that the

device is up and running and that no unusual error

conditions are known.  The warning(3) state

indicates that agent has been informed of an

unusual error condition by the operational software

(e.g., a disk device driver) but that the device

is still 'operational'.  An example would be high

number of soft errors on a disk.  A value of

testing(4), indicates that the device is not

available for use because it is in the testing

state.  The state of down(5) is used only when

the agent has been informed that the device is

not available for any use."

::= { hrDeviceEntry 5 }

hrPrinterStatus OBJECT-TYPE

SYNTAX INTEGER {

other(1),

unknown(2),

idle(3),

printing(4),

warmup(5)

}

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The current status of this printer device.  When in the

idle(3), printing(4), or warmup(5) state, the corresponding

hrDeviceStatus should be running(2) or warning(3).  When in

the unknown(2) state, the corresponding hrDeviceStatus

should be unknown(1)."

::= { hrPrinterEntry 1 }

hrPrinterDetectedErrorState OBJECT-TYPE

SYNTAX OCTET STRING (0..128)

ACCESS read-only

STATUS mandatory

DESCRIPTION

"This object represents any error conditions detected by the

printer.  The error conditions are encoded as an OCTET STRING

with the following definitions:

Condition          Bit #

lowPaper             0

noPaper              1

lowToner             2

noToner              3

doorOpen             4

jammed               5

offline              6

serviceRequested     7

inputTrayMissing     8

outputTrayMissing    9

markerSupplyMissing 10

outputNearFull      11

outputFull          12

inputTrayEmpty      13

overduePreventMaint 14

Bit # 15 is not assigned.

If multiple conditions are currently detected and the

hrDeviceStatus would not otherwise be unknown(1) or

testing(4), the hrDeviceStatus shall correspond to the worst

state of those indicated, where down(5) is worse than

warning(3), which is worse than running(2).

Bits are numbered starting with the most significant bit of

the first byte being bit 0, the least significant bit of the

first byte being bit 7, the most significant bit of the

second byte being bit 8, and so on.  A one bit encodes that

the condition was detected, while a zero bit encodes that

the condition was not detected.

This object is useful for alerting an operator to specific

warning or error conditions that may occur, especially those

requiring human intervention."

::= { hrPrinterEntry 2 }

2.2.13.2.2  Sub-unit Status

Sub-unit status is reported in the entries of the principle table in the Group that represents the sub-unit. For sub-units that report a status, there is a status column in the table and the value of this column is always an integer formed in the following way.

The SubUnitStatus is an integer that is the sum of 5 distinct values, Availability, Non-Critical, Critical, On-line, and Transitioning. These values are:

Availability                           value

Available and Idle               0       000'b

Available and Standby            2       010'b

Available and Active             4       100'b

Available and Busy               6       110'b

Unavailable and OnRequest        1       001'b

Unavailable because Broken       3       011'b

Unknown                          5       101'b

Non-Critical

No Non-Critical Alerts           0

Non-Critical Alerts              8

Critical

No Critical Alerts               0

Critical Alerts                 16

On-Line

State is On-Line                 0

State is Off-Line               32

Transitioning

At intended state                0

Transitioning to intended state 64

For example, an input (tray) that jammed on the next to the last page may show a status of 27 (unavailable because broken (3) + a critical state (16), jammed, and a noncritical state (8), low paper).

2.2.13.3  Alert Tables

The Alert Group consists of a single table in which all active alerts are represented.  This section provides an overview of the table and a description of how it is managed.  The basic content of the alert table is the severity (critical or non-critical) of the alert, the Group and entry where a state change caused the alert, additional information about the alert (a more detailed location, an alert code, and a description), and an indication of the level of training needed to service the alert.

The Alert Table contains some information that is redundant, for example that an event has occurred, and some information that is only represented in the Alert Table, for example the additional information.  A single table was used because a single entry in a group could cause more than one alert, for example paper jams in more than one place in a media path. Associating the additional information with the entry in the affected group would only allow one report where associating the additional information with the alert makes multiple reports possible. Every time an alert occurs in the printer, the printer makes one or more entries into the Alert Table. The printer determines if an event is to be classified as critical or non-critical. If the severity of the Alert is "critical", the printer sends a trap or event notification to the host indicating that the table has changed. Whether or not a trap is sent, the management application is expected to poll the printer on a regular basis and to read and parse the table to determine what conditions have changed, in order to provide reliable information to the management application user.

2.2.13.4  Alert Table Management

The alert tables are sparsely populated tables. This means the tables will only contain entries of the alerts that are currently active and the number of rows, or entries in the table will be dynamic. More than one event can be added or removed from the event tables at a time depending on the implementation of the printer.

There are basically two kinds of events that produce alerts: binary change events and unary change events. Binary change events come in pairs: the leading edge event and the trailing edge event. The leading edge event enters a state from which there is only one exit; for example, going from running to stopped with a paper jam. The only exit from this state is fixing the paper jam and it is clear when that is accomplished.  The trailing edge event exits the state that was entered by the leading edge event. In the example above, fixing the paper jam is the trailing edge event.

It is relatively straightforward to manage binary change events in the Alert Table. Only the leading edge event makes an entry in the alert table.  This entry persists in the Alert Table until the trailing edge event occurs at which point this event is signaled by the removal of the leading edge event entry in the Alert Table.  That is, a trailing edge event does not create an entry; it removes the corresponding leading edge event. Removing the leading edge entry may cause the unary change event "alertRemovalofBinaryChangeEntry" to be added to the table.  With binary change events it is possible to compute the maximum number that can occur at the same time and construct an Alert Table that would hold that many events. There would be no possibility of table overflow and no information about outstanding events would be lost.

Unfortunately, there are some events that are not binary changes. This other category of event, the unary change event, is illustrated by the configuration change event. With this kind of event the state of the machine has changed, but to a state which is (often) just as valid as the state that was left and from which no return is necessary.  For example, an operator may change the paper that is in the primary input source from letter to legal. At some time in the future the paper may be changed back to letter, but it might be changed to executive instead.  This is where the problem occurs. It is not obvious how long to keep unary change event entries in the Alert Table. If they were never removed, the Alert Table would continue to grow indefinitely.

The agent needs to have an algorithm implemented for the management of the alert table, especially in the face of combinations of binary and unary alerts that would overflow the storage capacity of the table.  When the table is full and new alerts need to be added, an old alert to be deleted should be chosen using the following rules:

1. Find a non-critical unary alert and delete it.  If there are multiple non-critical unary alerts, it is suggested that the oldest one is chosen.  If there are no non-critical unary alerts, then,

2. Find a non-critical binary alert and delete it.  If there are multiple non-critical binary alerts, it is suggested that the oldest one is chosen.  If there are no non-critical binary alerts, then,

3. Find a critical (binary) alert and delete it.  If there are multiple critical alerts, it is suggested that the oldest one be chosen.  Agent implementers are encouraged to provide at least enough storage space for the maximum number of critical alerts that could occur simultaneously.  Note that all critical alerts are binary.

In the event that a critical binary alert has been deleted out of the alert table; when space allows and the alert condition still exists, the alert should be re-added to the alert table even if there was no subsequent transition into the associated state. It is recommended that this be done for non-critical binary alerts as well. Note that the new alert entry will not have the same index as the original entry that was moved out of the table.

Note that because the Alert Index is a monotonically increasing integer there will be gaps in the values in the table when an alert is deleted. The management application may want to re-acquire the Printer state and check for state changes that it did not observe in the Alert Table if such gaps are detected.

2.3  Read-Write Objects

Some objects in the printer MIB reflect the existence or amount of a given resource within the printer.  Some examples of such resources are the size and number of sheets in a paper tray or the existence of certain output options.  Some printers have automatic sensors for these resources.  Most printers lack sensors for every property of every resource. The management application is allowed to write into objects that hold descriptive or existence values for printers that cannot sense these values. The ability to change the value of a read-write object may depend on the implementation of the agent. Many  objects in the MIB are given read-write access, but a printer implementation might only permit a management application to change the value if the printer can not sense the value itself. Note that even though some objects explicitly state the behavior of conditional ability to change values, any read-write object may act this way.

Generally, an object is given read-write access in the Printer MIB specification if:

1. The object involves installation of a resource that some printers cannot themselves detect.  Therefore, external means are needed to inform the printer of the installation.  (Here external means include using the operator console, or remote management application) and

2. The printer will behave differently if the installation of the resource is reported than the printer would if the installation were not reported; that is, the object is not to be used as a place to put information not used by the printer, i.e., not a "sticky-note".  Another way of saying this is that the printer believes that information given it and acts as if the information were true.  For example, on a printer that cannot sense the size, if one paper size is loaded, but another size is set into the paper size object, then the printer will use the size that was set as its current paper size in its imaging and paper handling.

3. The printer may get hints that it may not know about the existence or properties of certain resources.  For example, a paper tray may be removed and re-inserted.  When this removal and insertion happens, the printer may either assume that a property, such as the size of paper in the tray, has not changed or the printer may change the value of the associated object to "unknown", as might be done for the amount of paper in the tray.  As long as the printer acts according to the value in  the object either strategy is acceptable.

4. It is an implementation-specific matter as to whether or not MIB object values are persistent across power cycles or cold starts.  It is particularly important that the values of the prtMarkerLifeCount object persist throughout the lifetime of the printer.  Therefore, if the value of any MIB object persists across power cycles, then the prtMarkerLifeCount object must also persist.

2.4  Enumerations

Enumerations (enums) are sets of symbolic values defined for use with

one or more objects.  Commonly used enumeration sets are assigned a

symbolic data type name (textual convention), rather than being

specified in the SYNTAX clause of each individual object definition.

Textual conventions defined in the Printer MIB or the companion IANA

Printer MIB are extensible by RFC publication or by Designated Expert

Review (see the 'IANA Considerations' section of this Printer MIB and

the DESCRIPTION clause in MODULE-IDENTITY of IANA Printer MIB).  All

of these textual conventions are:

a) used more than once in the Printer MIB itself; or

b) imported and used in the companion Finisher MIB; or

c) imported and used in any other, including vendor private, MIB

modules.

The Printer MIB has also defined the following special values for

use with objects of the syntax "Integer32" to define conditions that

are outside of the normal numeric range:  other(-1), unknown(-2), and

partial(-3).  The 'partial' value means that there is some supply

remaining (but the amount is indeterminate) or there is some capacity

remaining (but the amount is indeterminate).  The Integer32 range

field indicates in which objects these special values are valid.

2.4.1  Registering Additional Enumerated Values

The Printer MIB and the companion IANA Printer MIB each defines one

category of textual convention, according to the process employed

to control the addition of new enumerations:

Type 1 - All of the legal values are defined in the Printer MIB.

Additional enumerated values require the publication of a new

Printer MIB.

Type 2 - All of the legal values are registered in the IANA

Printer MIB.  Additional enumerated values require a Designated

Expert Review defined in "Guidelines for Writing an IANA

Considerations Section in RFCs" [RFC2434].  The Designated Expert

will be selected by the IETF Area Director(s) of the Applications

Area.

3.  Groups from other MIB Specifications

This section identifies the groups from other MIBs that shall be supported to supplement and complete a printer MIB implementation. The section also describes some of the less obvious characteristics of the Printer MIB structure that are related to the inclusion of these other MIB groups.

3.1  System Group

All objects in the system group of MIB-II [RFC1213] shall be implemented; however, as described in paragraph 2.4, implementers should carefully consider what constitutes the "system".

3.2  System Controller

The storage and device groups of the Host Resources MIB [RFC2790] shall be implemented to support the printer(s) system controller, and any supporting devices. If deemed appropriate by the implementer, other groups of the Host Resources MIB (System, Running Software, Running Software Performance, and Installed Software) may be implemented.

Because of the structure of the Host Resources MIB, the devices constituting the system controller are at the same level as the printer.

3.3  Interface Group objects

All objects in the Interfaces Group of MIB-II [RFC1213] shall be implemented for all print information interfaces to the printer, including non-network interfaces.

3.3.1  Interface Types

The interfaces group of RFC 1213 [RFC1213] contains only a partial list of interface types that can be specified in the "ifType" object. For a complete list of interface types, refer to the IANA registry at "ftp://ftp.isi.edu/mib/iana.mib/ianaiftype.mib"

4.  Differences from RFC 1759

This document supercedes and replaces RFC 1759.  However, a compliant implementation of RFC 1759 is also compliant with this document.  The following changes to RFC 1759 are included:

- Minor editorial corrections and changes.  Updated the cover page and added the "SNMP Management Framework" boilerplate to section 1.

- Updated figure 2 to use MIB names instead of RFC numbers.

- Updated Coded Character Set description and IANA registration process.

- Change hrPrinterDetectedErrorState "coverOpen" (bit 4) to "doorOpen" per RFC 2790.

- Added second octet of hrPrinterDetectedErrorState as partially described and assigned in the updated Host Resources MIB (RFC 2790).

- Remove fixed association of hrDeviceStatus (warning/down) from hrPrinterDetetctedErrorState per RFC 2790.

- Instead of showing bit 15 as "not assigned" in the quote from RFC 2790 in the hrPrinterDetectedErrorState object, removed that from the tabular form and added it as a sentence, because the RFC doesn't show bit 15 in the tabular form.

- Clarified the international considerations.

- Added prtChannelInformation to the Channel Group textual-conventions on a per channel basis to clarify the channel description and enhance interoperability.

- Deprecated some obsolete channel types.

- Extended the Alert Table and PrtMarkerSuppliesSupplyUnit textual conventions to include values from the Finisher MIB.

- Clarified alerts based on unary vs. binary change events.

- Added (optional) unary change event alertRemovalOfBinaryChangeEntry(1801).

- Establish a convention for contact information for prtGeneralCurrentOperator and prtGeneralServicePerson.

- Added prtAuxiliarySheetStartupPage     PresentOnOff

- Added prtAuxiliarySheetBannerPage      PresentOnOff

- Added prtGeneralPrinterName            OCTET STRING

- Added prtGeneralSerialNumber           OCTET STRING

- Added prtInputNextIndex                Integer32

- Added the Input Switching Group

- Added prtAlertCriticalEvents           Counter32

- Added prtAlertAllEvents                Counter32

- Updated PrtAlertCode enums including generic alert codes.

- Created five OBJECT-GROUPs (prtAuxilliarySheetGroup, prtInputSwitchingGroup, prtGeneralV2Group, prtAlertTableV2Group, prtChannelV2Group).  Added the nine new objects to them (prtAuxiliarySheetStartupPage, prtAuxiliarySheetBannerPage, prtGeneralPrinterName, prtGeneralSerialNumber, prtAlertCriticalEvents, prtAlertAllEvents, prtInputMediaLoadTimeout, prtInputNextIndex, prtChannelInformation).  Created one new NOTIFICATION-GROUP (prtAlertTrapGroup) to contain printerV2Alert.  Included the new OBJECT-GROUPs and the NOTIFICATION_GROUP in prtMIBCompliance, all in GROUP (not MANDATORY-GROUP) clauses.  The nine new objects are optional, i.e., this draft is backward compatible with RFC 1759.

- prtAlertTime is strongly recommended.

- Deprecated the use of alert codes doorOpen(501) and doorClosed(502), in favor of coverOpened(3) and coverClosed(4).

- Added the PrtConsoleDisableTC and PrtMarkerAddressabilityUnitTC textual conventions, and changed the PrtConsoleDisable and PrtMarkerAddressabilityUnit objects' syntax to use those TCs, and changed the PrtGeneralEntry and PrtMarkerColorantEntry SEQUENCEs to reflect the new syntax.

- Added textual conventions "PrtLocalizedDescriptionStringTC" and "PrtConsoleDescriptionStringTC" and updated several objects to use them.

- Changed most enumerations to textual conventions and therefore

changed the SYNTAX of many objects from RFC 1759 to specify the

appropriate textual conventions.  (28 TCs were added.)

- Changed the TC names "MediaUnit" to "PrtMediaUnitTC",

"CapacityUnit" to "PrtCapacityUnitTC", and "SubUnitStatus" to

"PrtSubUnitStatusTC"

- All objects with a MAX-ACCESS of read-write now have a MIN-ACCESS of read-only.

- Added 'IANA Considerations' and 'Internationalization Considerations' as top level sections, per IETF guidelines.

- Updated Security and Copyright sections.

- Updated references.

- Added Appendix E - Overall Printer Status Table.

- Updated participant and contact information.

- Removed CodedCharSet Textual Convention, replaced with an import of the IANACharset.

- Removed all comment statements that indicated objects or groups are mandatory or optional.  Avoids any potential conflicts with the conformance section.

- Added text to empty description clauses.  (prtStorageRefTable, prtDeviceRefTable, prtMarkerTable, prtMediaPathTable, prtChannelTable, prtInterpreterTable, prtConsoleLightTable, and prtAlertTable)

- Added "DEFVAL { unknown }" to prtInterpreterDefaultCharSetIn and prtInterpreterDefaultCharSetOut.

- Changed "...values are expected to remain stable..." to "...values SHOULD remain stable..." in the description clauses for the index object in all tables.

- Added ranges to all objects with a syntax of Integer32.

- Revised the description clause for prtAlertGroupIndex.

- Added additional text to the description clause for prtMediaPathEntry, prtChannelEntry, prtInterpreterEntry, and printerV2Alert.

- Added text to section 2.4 to explain the usage of textual conventions in this MIB and others.  Also added a note defining the common usage of the enumerations 'other(-1)' and 'unknown(-2)'

- Changed range of prtStorageRefSeqNumber, prtDeviceRefSeqNumber, and prtConsoleLightIndex from (0..65535) to (1..65535) since index values cannot be zero.  (Typo in RFC 1759)

- The PWG Standard for Standardized Media Names is now recommended for the objects prtInputMediaName, prtInputMediaColor, and prtInputMediaType.

- Added chSMTP(45) to prtChannelTypeTC.

5.  The IANA Printer MIB

IANA-PRINTER-MIB DEFINITIONS ::= BEGIN

IMPORTS

MODULE-IDENTITY,

mib-2

FROM SNMPv2-SMI

TEXTUAL-CONVENTION

FROM SNMPv2-TC;

ianaPrinterMIB MODULE-IDENTITY

LAST-UPDATED  "200302100000Z" -- February 10, 2003

ORGANIZATION  "IANA"

CONTACT-INFO  "Internet Assigned Numbers Authority

Postal: ICANN

4676 Admiralty Way, Suite 330

Marina del Rey, CA 90292

Tel:    +1 310 823 9358

E-Mail: iana@iana.org"

DESCRIPTION   "This MIB module defines a set of printing-related

textual conventions for use in Printer MIB (RFC xxxx),

Finisher MIB (RFC yyyy), and other MIBs which need to

specify printing mechanism details.

Any additions or changes to the contents of this MIB

module require either publication of an RFC, or

Designated Expert Review as defined in RFC 2434,

Guidelines for Writing an IANA Considerations Section

in RFCs.  The Designated Expert will be selected by

the IESG Area Director(s) of the Applications Area.

Copyright (C) The Internet Society (2003). This

version of this MIB module is part of RFC xxxx;

see the RFC itself for full legal notices."

-- RFC Editor assigns above RFC xxxx and RFC yyyy.

REVISION      "200302100000Z"  -- February 10, 2003

DESCRIPTION   "Original version, published in coordination

with Printer MIB (RFC xxxx)."

-- RFC Editor assigns above RFC xxxx.

::= { mib-2 nnn } -- nnn to be assigned by IANA

--

-- Generic textual conventions

--

PrtCoverStatusTC ::= TEXTUAL-CONVENTION

-- This TC was extracted from prtCoverStatus in RFC 1759.

-- "coverOpen" and "coverClosed" in this MIB version were "doorOpen"

-- and "doorClosed" in RFC 1759. A name change is not formally

-- allowed in SNMP, but was agreed to by the WG group since a door

-- has a more restrictive meaning than a cover and Cover group is

-- intended to support doors as a subset of covers.

STATUS    current

DESCRIPTION

"Values for encoding the state of a particular cover or

access panel on the printer case or enclosure."

SYNTAX    INTEGER {

other(1),

coverOpen(3),

coverClosed(4),

interlockOpen(5),

interlockClosed(6)

}

--

-- General Group textual conventions

--

PrtGeneralResetTC ::= TEXTUAL-CONVENTION

-- This TC was extracted from prtGeneralReset in RFC 1759.

STATUS    current

DESCRIPTION

"Values for reading and writing the prtGeneralReset object.

If a device does not have NVRAM, the device shall none the

less respond to a SET with the value resetToNVRAM(5) with a

sort of warm reset that resets the device to implementation-

defined state that is preferably under control of the system

administrator by some means outside the scope of the Printer

MIB specification."

SYNTAX    INTEGER {

notResetting(3),

powerCycleReset(4), -- Cold Start

resetToNVRAM(5), -- Warm Start

resetToFactoryDefaults(6) -- Reset contents of

-- NVRAM to factory

-- defaults

}

--

-- Channel Group textual conventions

--

PrtChannelTypeTC ::= TEXTUAL-CONVENTION

-- This TC was extracted from prtChannelType in RFC 1759.

STATUS    current

DESCRIPTION

"This enumeration indicates the type of channel that is

receiving jobs."

SYNTAX    INTEGER {

other(1),

chSerialPort(3),

chParallelPort(4),

chIEEE1284Port(5),

chSCSIPort(6),

chAppleTalkPAP(7),

-- AppleTalk Printer

-- Access Protocol (PAP)

--

-- prtChannelInformation entry:

--

-- Printer Name

--   Keyword:      Name

--   Syntax:       Name

--   Status:       Optional

--   Multiplicity: Single

--   Description:  The name of the printer

--     within the AppleTalk naming scope

chLPDServer(8),

-- prtChannelInformation entry:

--

-- Printer queue name

--   Keyword:      Queue

--   Syntax:       Name

--   Status:       Mandatory

--   Multiplicity: Single

--   Description: queue name as

--     defined in [RFC1179].

chNetwareRPrinter(9),

-- Novell, Inc.

-- For each entry of this type, the

-- prtChannelInformation must have a pair of

-- keywords. For Netware 3.x channels this must

-- be a (PServer, Printer) pair. For Netware

-- 4.x channels and for IntranetWare channels

-- this must be a (NDSTree, NDSPrinter) pair.

--

-- prtChannelInformation entries:

-- Print Server Name

--   Keyword:      PServer

--   Syntax:       Name

--   Status:       Mandatory

--   Multiplicity: Single

--   Description:  The Pserver's SAP name

--

-- Printer Number

--   Keyword:      Printer

--   Syntax:       Integer

--   Status:       Mandatory

--   Multiplicity: Single

--   Description:  The printer number

--

-- NDSTree

--   Keyword:      NDSTree

--   Syntax:       Name

--   Multiplicity: Single

--   Description:  The tree's SAP name

--

-- NDS Printer object

--   Keyword:      NDSPrinter

--   Syntax:       Text (Unicode)

--   Status:       Mandatory

--   Multiplicity: Single

--   Description:  The fully qualified

--                 name of the Printer

--

-- In the Netware 3.x environment, the

-- client checks the Bindery object

-- representing the named PServer. The

-- client then checks for queues which

-- are associated with the numbered

-- printer. In the 4.x and IntraNetware

-- environment, the client looks up the

-- queues which are associated with the

-- NDS Printer Object in the named Tree.

-- Depending on client access rights to

-- those queues, the client submits jobs

-- to the appropriate queue.

chNetwarePServer(10),

-- Novell,Inc.

-- For each entry of this type, the

-- prtChannelInformation must have a pair

-- of keywords. For Netware 3.x channels

-- this must be a (Server, PServer) pair.

-- For Netware 4.x and IntranetWare

-- channels, this must be a

-- (NDSTree, NDSPServer) pair.

--

-- prtChannelInformation entries:

--

-- Server Name

--   Keyword:      Server

--   Syntax:       Name

--   Status:       Mandatory

--   Multiplicity: Single

--   Description:  The SAP name of the

--   server for which the PServer is defined.

--

-- PServer

--   Keyword:      PServer

--   Syntax:       Name

--   Status:       Mandatory

--   Multiplicity: Single

--   Description:  The bindery name of

--                 the PServer

--

-- NDS Tree

--   Keyword:      NDSTree

--   Syntax:       Name

--   Status:       Mandatory

--   Multiplicity: Single

--   Description:  The NDS Tree name

--

-- PServer

--   Keyword:      NDSPServer

--   Syntax:       Text (Unicode)

--   Status:       Mandatory

--   Multiplicity: Single

--   Description:  The fully qualified

--     name of the PServer object in the tree.

--

-- In the 3.x environment, the client

-- checks the bindery object

-- representing the named PServer on the

-- named Server. In the 4.x and

-- IntranetWare environment,

-- the client checks the NDS object

-- representing the named PServer in the

-- named Tree. In either case, the

-- client then checks for all queues

-- associated with the Pserver object.

-- Depending on client access rights

-- to those queues, the client submits

-- jobs to the appropriate queue.

chPort9100(11),

-- DEPRECATED

-- (see chPortTCP - 37; chBidirPortTCP - 38)

chAppSocket(12),

-- A bi-directional, LPD-like, protocol using

-- 9101 for control and 9100 for data.

-- Adobe Systems, Inc.

chFTP(13),         -- [RFC959]

chTFTP(14),        -- [RFC1350]

chDLCLLCPort(15),

chIBM3270(16),     -- IBM Coax

chIBM5250(17),     -- IBM Twinax

chFax(18),

chIEEE1394(19),

chTransport1(20),

-- TCP port 35, for reserved TCP port list see

-- [RFC3232]. This RFC should also be

-- referenced for other channel

-- enumerations utilizing TCP port

-- numbers 0 through 1024.

chCPAP(21),      -- TCP port 170

-- Digital Equipment Corp.

chDCERemoteProcCall(22), -- OSF

-- DEPRECATED

chONCRemoteProcCall(23), -- SUN Microsystems

-- DEPRECATED

chOLE(24),         -- Microsoft

-- DEPRECATED

chNamedPipe(25),

chPCPrint(26),   -- Banyan

chServerMessageBlock(27),

-- File/Print sharing protocol used by

-- various network operating systems

-- from IBM 3Com, Microsoft and others

--

-- prtChannelInformation entry:

--

-- Service Name

--   Keyword:      Name

--   Syntax:       Name

--   Status:       Optional

--   Multiplicity: Single

--   Description:  The service name of

--                 the printer

chDPMF(28),   -- IBM Infoprint

chDLLAPI(29), -- Microsoft

-- DEPRECATED

chVxDAPI(30), -- Microsoft

-- DEPRECATED

chSystemObjectManager(31), -- IBM

chDECLAT(32),

-- Digital Equipment Corp.

--

-- prtChannelInformation entries:

--

-- Port Name

--   Keyword:      Port

--   Syntax:       Name

--   Status:       Conditionally

--                 Mandatory

--                 (see note below)

--   Multiplicity: Single

--   Description:  LAT port name

--

-- Service Name

--   Keyword:      Service

--   Syntax:       Name

--   Status:       Conditionally

--                 Mandatory

--   Multiplicity: Single

--   Description:  LAT service name

--

-- The LAT channel may be

-- identified by either a port or

-- service, so either a

-- Port or Service entry must be

-- specified, but not both.

chNPAP(33),

chUSB(34), -- Not in RFC 1759

-- Universal Serial Bus

chIRDA(35), -- Not in RFC 1759

-- Infrared Data Assoc. Prot.

chPrintXChange(36),  -- Not in RFC 1759

-- PrintXChange Protocol

chPortTCP(37),  -- Not in RFC 1759

-- A unidirectional "raw" TCP

-- channel that uses an administratively

-- assigned TCP port address.

--

-- prtChannelInformation entry:

--

-- Port Number

--   Keyword:      Port

--   Syntax:       decimal number

--   Status:       Mandatory

--   Multiplicity: Single

--   Description:  TCP port number

chBidirPortTCP(38),  -- Not in RFC 1759

-- A bi-directional version of chPortTCP

--

-- prtChannelInformation entries:

-- (See chPortTCP)

chUNPP(39),  -- Not in RFC 1759

-- Universal Network Printing

-- Protocol(UNPP). A bi-directional,

-- multiport network printing

-- application protocol available on

-- multiple transport protocols.

-- Underscore, Inc.

-- Contact: info@underscore.com

chAppleTalkADSP(40),  -- Not in RFC 1759

-- AppleTalk Data Stream Protocol.

-- ADSP is part of the AppleTalk

-- suite of protocols.

-- It is a symmetric, connection-

-- oriented protocol that makes

-- possible the establishment

-- and maintenance of full-duplex

-- streams of data bytes between

-- two sockets in an AppleTalk

-- internet.

-- See [APPLEMAC].

chPortSPX(41),  -- Not in RFC 1759

-- Sequenced Packet Exchange (SPX)

-- socket.

-- Novell, Inc. Similar to TCP, a

-- bi-directional data pipe using

-- Novell SPX as a transport.

--

-- prtChannelInformation entries:

--

-- Network Number

--   Keyword:      Net

--   Syntax:       HexString

--   Status:       Mandatory

--   Multiplicity: Single

--   Description:  The network number

--

-- Node Number

--   Keyword:      Node

--   Syntax:       HexString

--   Status:       Mandatory

--   Multiplicity: Single

--   Description:  The node number

--

-- Socket Number

--   Keyword:      Socket

--   Syntax:       HexString

--   Status:       Mandatory

--   Multiplicity: Single

--   Description:  The SPX socket number

--

-- There must be exactly one "Net" and

-- one "Node" and one "Socket" entry. A

-- HexString is a binary value

-- represented as a string of

-- ASCII characters using hexadecimal

-- notation.

chPortHTTP(42),  -- Not in RFC 1759

-- Hypertext Transfer Protocol. See [RFC1945]

-- and [RFC2616].

chNDPS(43),  -- Not in RFC 1759

-- Novell, Inc.

--

-- prtChannelInformation entry:

--

-- Printer Agent Name

--   Keyword:      PA

--   Syntax:       Name

--   Status:       Mandatory

--   Multiplicity: Single

--   Description:  The NDPS Printer

--                 Agent Name

chIPP(44),  -- Not in RFC 1759

-- Internet Printing Protocol (IPP),

-- (IPP/1.1 - see [RFC2910] and [RFC2911])

-- also applies to all future versions of IPP.

--

-- IPP Printer URI

--   Keyword:      URI

--   Syntax:       URI (Unicode UTF-8 per

--                 [RFC2396])

--   Status:       Mandatory

--   Multiplicity: Single

--   Default:      not applicable

--   Description:  URI of this IPP Printer

--     within Internet naming scope.  Unicode

--     UTF-8 [RFC2279] string with

--     hexadecimal escapes for any non-ASCII

--     characters (per [RFC2396]).

--   Conformance: An IPP Printer shall list all

--     IPP URI it supports (one per IPP Channel

--     entry). If a URI contains the 'http:'

--     scheme it must have an explicit port.

--   See: [RFC2279], [RFC2396], [RFC2910],

--     [RFC2911].

--

-- IPP Printer Client Authentication

--   Keyword:      Auth

--   Syntax:       Keyword

--   Status:       Optional

--   Multiplicity: Single

--   Default:      'none'

--   Description:  A client authentication

--     mechanism supported for this IPP Printer

--     URI:

--       'none'

--         no client authentication mechanism

--       'requesting-user-name'

--         authenticated user in 'requesting-

--         user-name'

--       'basic'

--         authenticated user via HTTP Basic

--         mechanism

--       'digest'

--         authenticated user via HTTP Digest

--         mechanism

--       'certificate'

--         authenticated user via certificate

--         mechanism

--   Conformance: An IPP Printer should list

--     all IPP client authentication mechanisms

--     it supports (one per IPP Channel entry).

--     See: [RFC2911] and [RFC2910].

--

-- IPP Printer Security

--   Keyword:      Security

--   Syntax:       Keyword

--   Status:       Optional

--   Multiplicity: Single

--   Default:      'none'

--   Description:  A security mechanism

--     supported for this IPP Printer URI:

--     'none'

--       no security mechanism

--     'ssl3'

--       SSL3 secure communications channel

--       protocol

--     'tls'

--       TLS secure communications channel

--       protocol

--   Conformance: An IPP Printer should list

--     all IPP security mechanisms it supports

--     (one per IPP Channel entry).

--   See: [RFC2246], [RFC2911].

--

-- IPP Printer Protocol Version

--   Keyword:      Version

--   Syntax:       Keyword

--   Status:       Optional

--   Multiplicity: Multiple

--   Default:      '1.1'

--   Description:  All of the IPP protocol

--     versions (major.minor) supported for

--     this IPP Printer URI:

--     '1.0'

--       IPP/1.0 conforming Printer

--     '1.1'

--       IPP/1.1 conforming Printer

--   Conformance:  An IPP Printer should list

--     all IPP versions it supports (all listed

--     in each IPP Channel entry). An IPP

--     Client should select the highest

--     numbered version the IPP Client supports

--     for use in all IPP Requests (for optimum

--     interworking).

--   See: [RFC2911].

chSMTP(45)

-- Print Job submission via Simple Mail

-- Transfer Protocol (SMTP) - see [RFC2821]

--

-- prtChannelInformation entry:

--

--   Keyword:      Mailto

--   Syntax:       Name

--   Status:       Mandatory

--   Multiplicity: Single

--   Default:      not applicable

--   Description:  The SMTP URL of the printer.

}

--

-- Interpreter Group textual conventions

--

PrtInterpreterLangFamilyTC ::= TEXTUAL-CONVENTION

-- This TC was extracted from prtInterpreterLangFamily in RFC 1759.

STATUS    current

DESCRIPTION

"This enumeration indicates the type of interpreter that is

receiving jobs."

SYNTAX    INTEGER {

other(1),

unknown(2),          -- Not in RFC 1759

langPCL(3),          -- PCL.  Starting with PCL version 5,

-- HP-GL/2 is included as part of the

-- PCL language.

-- PCL and HP-GL/2 are registered

-- trademarks of Hewlett-Packard

-- Company.

langHPGL(4),         -- Hewlett-Packard Graphics Language.

-- HP-GL is a registered trademark of

-- Hewlett-Packard Company.

langPJL(5),          -- Peripheral Job Language. Appears in

-- the data stream between data intended

-- for a page description language.

-- Hewlett-Packard Co.

langPS(6),           -- PostScript (tm) Language

-- Postscript - a trademark of Adobe

-- Systems Incorporated which may be

-- registered in certain jurisdictions

langIPDS(7),         -- Intelligent Printer Data Stream

-- Bi-directional print data stream for

-- documents consisting of data objects

-- (text, image, graphics, bar codes),

-- resources (fonts, overlays) and page,

-- form and finishing instructions.

-- Facilitates system level device

-- control, document tracking and error

-- recovery throughout the print

-- process.

-- IBM Corporation.

langPPDS(8),         -- IBM Personal Printer Data Stream.

-- Originally called IBM ASCII, the name

-- was changed to PPDS when the Laser

-- Printer was introduced in 1989.

-- Lexmark International, Inc.

langEscapeP(9),      -- Epson Corp.

langEpson(10),

langDDIF(11),        -- Digital Document Interchange Format

-- Digital Equipment Corp., Maynard MA

langInterpress(12),

-- Xerox Corp.

langISO6429(13),     -- ISO 6429.  Control functions for

-- Coded Character Sets (has ASCII

-- control characters, plus additional

-- controls for

-- character imaging devices.)

langLineData(14),    -- line-data:  Lines of data as

-- separate ASCII or EBCDIC records

-- and containing no control functions

-- (no CR, LF, HT, FF, etc.)

-- For use with traditional line

-- printers.  May use CR and/or LF to

-- delimit lines, instead of records.

-- See ISO 10175 Document Printing

-- Application (DPA) [ISO10175].

langMODCA(15),       -- Mixed Object Document Content

-- Architecture

-- Definitions that allow the

-- composition, interchange, and

-- presentation of final form

-- documents as a collection of data

-- objects (text, image, graphics, bar

-- codes), resources (fonts, overlays)

-- and page, form and finishing

-- instructions.

-- IBM Corporation.

langREGIS(16),       -- Remote Graphics Instruction Set,

-- Digital Equipment Corp., Maynard MA

langSCS(17),         -- SNA Character String

-- Bi-directional print data stream for

-- SNA LU-1 mode of communication.

-- IBM

langSPDL(18),        -- ISO 10180 Standard Page Description

-- Language

-- ISO Standard

langTEK4014(19),     -- Tektronix Corp.

langPDS(20),

langIGP(21),         -- Printronix Corp.

langCodeV(22),       -- Magnum Code-V, Image and printer

-- control language used to control

-- impact/dot-matrix printers.

-- QMS, Inc., Mobile AL

langDSCDSE(23),      -- DSC-DSE:  Data Stream Compatible and

-- Emulation Bi-directional print data

-- stream for non-SNA (DSC) and SNA LU-3

-- 3270 controller (DSE) communications

-- IBM

langWPS(24),         -- Windows Printing System, Resource

-- based command/data stream used by

-- Microsoft At Work Peripherals.

-- Developed by the Microsoft

-- Corporation.

langLN03(25),        -- Early DEC-PPL3, Digital Equipment

-- Corp.

langCCITT(26),

langQUIC(27),        -- QUIC (Quality Information Code), Page

-- Description Language for laser

-- printers. Included graphics, printer

-- control capability and emulation of

-- other well-known printer.

-- QMS, Inc.

langCPAP(28),        -- Common Printer Access Protocol

-- Digital Equipment Corp.

langDecPPL(29),      -- Digital ANSI-Compliant Printing

-- Protocol

-- (DEC-PPL)

-- Digital Equipment Corp.

langSimpleText(30),

-- simple-text:  character coded data,

-- including NUL, CR , LF, HT, and FF

-- control characters.  See ISO 10175

-- Document Printing Application (DPA)

-- [ISO10175].

langNPAP(31),        -- Network Printer Alliance Protocol

-- (NPAP). This protocol has been

-- superseded by the IEEE 1284.1 TIPSI

-- Std (ref. LangTIPSI(49)).

langDOC(32),         -- Document Option Commands, Appears in

-- the data stream between data

-- intended for a page description.

-- QMS, Inc.

langimPress(33),     -- imPRESS, Page description language

-- originally developed for the

-- ImageServer product line. A binary

-- language providing representations

-- of text, simple graphics, and some

-- large forms (simple

-- bit-map and CCITT group 3/4

-- encoded).The

-- language was intended to be sent over

-- an 8-bit channel and supported early

-- document preparation languages (e.g.,

-- TeX and TROFF).

-- QMS, Inc.

langPinwriter(34),

-- 24 wire dot matrix printer  for

-- USA, Europe, and Asia except

-- Japan.

-- More widely used in Germany, and

-- some Asian countries than in US.

-- NEC

langNPDL(35),        -- Page printer  for Japanese market.

-- NEC

langNEC201PL(36),    -- Serial printer language used in

-- the Japanese market.

-- NEC

langAutomatic(37),

-- Automatic PDL sensing.  Automatic

-- sensing of the interpreter

-- language family by the printer

-- examining the document content.

-- Which actual interpreter language

-- families are sensed depends on

-- the printer implementation.

langPages(38),       -- Page printer Advanced Graphic

-- Escape Set

-- IBM Japan

langLIPS(39),        -- LBP Image Processing System

langTIFF(40),        -- Tagged Image File Format (Aldus)

langDiagnostic(41),

-- A hex dump of the input to the

-- interpreter

langPSPrinter(42),

-- The PostScript Language used for

-- control (with any PDLs)

-- Adobe Systems Incorporated

langCaPSL(43),       -- Canon Print Systems Language

langEXCL(44),        -- Extended Command Language

-- Talaris Systems Inc.

langLCDS(45),        -- Line Conditioned Data Stream

-- Xerox Corporation

langXES(46),         -- Xerox Escape Sequences

-- Xerox Corporation

langPCLXL(47),       -- Not in RFC 1759

-- Printer Control Language. Extended

-- language features for printing, and

-- printer control.

-- Hewlett-Packard Co.

langART(48),         -- Not in RFC 1759

-- Advanced Rendering Tools (ART).

-- Page Description language

-- originally developed for the Laser

-- Press printers.

-- Technical reference manual: "ART IV

-- Reference Manual", No F33M.

-- Fuji Xerox Co., Ltd.

langTIPSI(49),       -- Not in RFC 1759

-- Transport Independent Printer

-- System Interface (ref. IEEE Std.

-- 1284.1)

langPrescribe(50),   -- Not in RFC 1759

-- Page description and printer

-- control language. It can be

-- described with ordinary ASCII

-- Technical reference manual:

-- "PRESCRIBE II Programming Manual"

langLinePrinter(51), -- Not in RFC 1759

-- A simple-text character stream which

-- supports the control codes LF, VT,

-- FF, and plus Centronics or

-- Dataproducts Vertical Format Unit

-- (VFU) language is commonly used on

-- many older model line and matrix

-- printers.

langIDP(52),         -- Not in RFC 1759

-- Imaging Device Protocol

-- Apple Computer.

langXJCL(53),        -- Not in RFC 1759

-- Xerox Job Control Language (JCL).

-- A Job Control language originally

-- developed for the LaserPress printers

-- and is capable of switching PDLs.

-- Technical reference manual:

-- "ART IV Reference Manual", No F33M.

-- Fuji Xerox Co., Ltd.

langPDF(54),         -- Not in RFC 1759

-- Adobe Portable Document Format

-- Adobe Systems, Inc.

langRPDL(55),        -- Not in RFC 1759

-- Ricoh Page Description Language for

-- printers.

-- Technical manual "RPDL command

-- reference" No.307029

-- RICOH, Co. LTD

langIntermecIPL(56), -- Not in RFC 1759

-- Intermec Printer Language for label

-- printers.

-- Technical Manual: "IPL Programmers

-- Reference Manual"

-- Intermec Corporation

langUBIFingerprint(57),  -- Not in RFC 1759

-- An intelligent basic-like programming

-- language for label printers.

-- Reference Manual: "UBI Fingerprint

-- 7.1", No. 1-960434-00

-- United Barcode Industries

langUBIDirectProtocol(58),  -- Not in RFC 1759

-- An intelligent control language for

-- label printers.

-- Programmers guide: " UBI Direct

-- Protocol", No. 1-960419-00

-- United Barcode Industries

langFujitsu(59),     -- Not in RFC 1759

-- Fujitsu Printer Language

-- Reference Manual:

-- "FM Printer Sequence" No. 80HP-0770

-- FUJITSU LIMITED

langCGM(60),         -- Not in RFC 1759

-- Computer Graphics Metafile

-- MIME type 'image/cgm'

langJPEG(61),        -- Not in RFC 1759

-- Joint Photographic Experts Group

-- MIME type 'image/jpeg'

langCALS1(62),       -- Not in RFC 1759

-- US DOD CALS1 (see MIL-STD-1840)

-- MIME type 'application/cals-1840'

langCALS2(63),       -- Not in RFC 1759

-- US DOD CALS2 (see MIL-STD-1840)

-- MIME type 'application/cals-1840'

langNIRS(64),        -- Not in RFC 1759

-- US DOD NIRS (see MIL-STD-1840)

-- MIME type 'application/cals-1840'

langC4(65)           -- Not in RFC 1759

-- US DOD C4 (see MIL-STD-1840)

-- MIME type 'application/cals-1840'

}

--

-- Input/Output Group textual conventions

--

PrtInputTypeTC ::= TEXTUAL-CONVENTION

-- This TC was extracted from prtInputType in RFC 1759.

STATUS    current

DESCRIPTION

"The type of technology (discriminated primarily according to

feeder mechanism type) employed by a specific component or

components."

SYNTAX    INTEGER {

other(1),

unknown(2),

sheetFeedAutoRemovableTray(3),

sheetFeedAutoNonRemovableTray(4),

sheetFeedManual(5),

continuousRoll(6),

continuousFanFold(7)

}

PrtOutputTypeTC ::= TEXTUAL-CONVENTION

-- This TC was extracted from prtOutputType in RFC 1759.

STATUS    current

DESCRIPTION

"The Type of technology supported by this output subunit."

SYNTAX    INTEGER {

other(1),

unknown(2),

removableBin(3),

unRemovableBin(4),

continuousRollDevice(5),

mailBox(6),

continuousFanFold(7)

}

--

-- Marker Group textual conventions

--

PrtMarkerMarkTechTC ::= TEXTUAL-CONVENTION

-- This TC was extracted from prtMarkerMarkTech in RFC 1759.

STATUS    current

DESCRIPTION

"The type of marking technology used for this marking

subunit."

SYNTAX    INTEGER {

other(1),

unknown(2),

electrophotographicLED(3),

electrophotographicLaser(4),

electrophotographicOther(5),

impactMovingHeadDotMatrix9pin(6),

impactMovingHeadDotMatrix24pin(7),

impactMovingHeadDotMatrixOther(8),

impactMovingHeadFullyFormed(9),

impactBand(10),

impactOther(11),

inkjetAqueous(12),

inkjetSolid(13),

inkjetOther(14),

pen(15),

thermalTransfer(16),

thermalSensitive(17),

thermalDiffusion(18),

thermalOther(19),

electroerosion(20),

electrostatic(21),

photographicMicrofiche(22),

photographicImagesetter(23),

photographicOther(24),

ionDeposition(25),

eBeam(26),

typesetter(27)

}

PrtMarkerSuppliesTypeTC ::= TEXTUAL-CONVENTION

-- This TC was extracted from prtMarkerSuppliesType in RFC 1759.

STATUS    current

DESCRIPTION

"The type of this supply."

SYNTAX    INTEGER {

other(1),

unknown(2),

-- Values for Printer MIB

toner(3),

wasteToner(4),

ink(5),

inkCartridge(6),

inkRibbon(7),

wasteInk(8),

opc(9),               -- photo conductor

developer(10),

fuserOil(11),

solidWax(12),

ribbonWax(13),

wasteWax(14),

fuser(15),            -- Not in RFC 1759

coronaWire(16),       -- Not in RFC 1759

fuserOilWick(17),     -- Not in RFC 1759

cleanerUnit(18),      -- Not in RFC 1759

fuserCleaningPad(19), -- Not in RFC 1759

transferUnit(20),     -- Not in RFC 1759

tonerCartridge(21),   -- Not in RFC 1759

fuserOiler(22),       -- Not in RFC 1759

-- End of values for Printer MIB

-- Values for Finisher MIB

water(23),            -- Not in RFC 1759

wasteWater(24),       -- Not in RFC 1759

glueWaterAdditive(25),-- Not in RFC 1759

wastePaper(26),       -- Not in RFC 1759

bindingSupply(27),    -- Not in RFC 1759

bandingSupply(28),    -- Not in RFC 1759

stitchingWire(29),    -- Not in RFC 1759

shrinkWrap(30),       -- Not in RFC 1759

paperWrap(31),        -- Not in RFC 1759

staples(32),          -- Not in RFC 1759

inserts(33),          -- Not in RFC 1759

covers(34)            -- Not in RFC 1759

-- End of values for Finisher MIB

}

--

-- Media Path textual conventions

--

PrtMediaPathTypeTC ::= TEXTUAL-CONVENTION

-- This TC was extracted from prtMediaPathType in RFC 1759.

STATUS    current

DESCRIPTION

"The type of the media path for this media path."

SYNTAX    INTEGER {

other(1),

unknown(2),

longEdgeBindingDuplex(3),

shortEdgeBindingDuplex(4),

simplex(5)

}

--

-- Console Group textual conventions

--

PrtConsoleColorTC ::= TEXTUAL-CONVENTION

-- This TC was extracted from prtConsoleColor in RFC 1759.

STATUS     current

DESCRIPTION

"The color of this light."

SYNTAX    INTEGER {

other(1),

unknown(2),

white(3),

red(4),

green(5),

blue(6),

cyan(7),

magenta(8),

yellow(9),

orange(10)        -- Not in RFC 1759

}

PrtConsoleDisableTC ::= TEXTUAL-CONVENTION

-- This TC was extracted from prtConsoleDisable in RFC 1759.

STATUS     current

DESCRIPTION

"This value indicates whether or not input is accepted from

the operator console.  A value of 'enabled' indicates that

input is accepted from the console, and a value of 'disabled'

indicates that input is not accepted from the console. "

SYNTAX    INTEGER {

enabled(3),

disabled(4)

}

--

-- Alert Group textual conventions

--

PrtAlertTrainingLevelTC ::= TEXTUAL-CONVENTION

-- This TC was extracted from prtAlertTrainingLevel in RFC 1759.

STATUS    current

DESCRIPTION

"The level of training required to handle this alert, if

human intervention is required. The noInterventionRequired

value should be used if the event does not require any human

intervention. The training level is an enumeration that is

determined and assigned by the printer manufacturer based on

the information or training required to handle this alert.

The printer will break alerts into these different training

levels. It is the responsibility of a management application

in the system to determine how a particular alert is handled

and how and to whom that alert is routed.  The following are

the four training levels of alerts:

Field Service - Alerts that typically require advanced

training and technical knowledge of the printer and its

subunits. An example of a technical person would be a

manufacturer's Field Service representative, or other

person formally trained by the manufacturer or similar

representative.

Trained - Alerts that require an intermediate or moderate

knowledge of the printer and its subunits. A typical

example of such an alert is replacing a toner cartridge.

Untrained -     Alerts that can be fixed without prior

training either because the action to correct the alert

is obvious or the printer can help the untrained person

fix the problem. A typical example of such an alert is

reloading paper trays or emptying output bins on a low

end printer.

Management -    Alerts that have to do with overall operation

of and configuration of the printer. Examples of such

management events are configuration change of subunits."

SYNTAX    INTEGER {

other(1),

unknown(2),

untrained(3),

trained(4),

fieldService(5),

management(6),

noInterventionRequired(7)  -- Not in RFC 1759

}

PrtAlertGroupTC ::= TEXTUAL-CONVENTION

-- Values in the range 1 to 29 must not be IANA-assigned without

-- re-publishing Printer MIB.

-- Values of 30 and greater are for use in MIBs that augment

-- the Printer MIB, such as the Finisher MIB.

-- This TC extracted from prtAlertGroup in RFC 1759.

STATUS    current

DESCRIPTION

"The type of subunit within the printer model that this alert

is related.  Input, output, and markers are examples of

printer model groups, i.e., examples of types of subunits.

Wherever possible, the enumerations match the sub-identifier

that identifies the relevant table in the Printer MIB.

NOTE: Alert type codes have been added for the Host Resources

MIB storage table and device table. These additional types

are for situations in which the printer's storage and device

objects must generate alerts (and possibly traps for critical

alerts)."

SYNTAX    INTEGER {

other(1),

-- (2) is reserved for conformance information

-- Values for Host Resources MIB

hostResourcesMIBStorageTable(3),

hostResourcesMIBDeviceTable(4),

-- Values for Printer MIB

generalPrinter(5),

cover(6),

localization(7),

input(8),

output(9),

marker(10),

markerSupplies(11),

markerColorant(12),

mediaPath(13),

channel(14),

interpreter(15),

consoleDisplayBuffer(16),

consoleLights(17),

alert(18),                   -- Not in RFC 1759

-- Values (5) to (29) reserved for Printer MIB

-- Values for Finisher MIB

finDevice(30),               -- Not in RFC 1759

finSupply(31),               -- Not in RFC 1759

finSupplyMediaInput(32),     -- Not in RFC 1759

finAttribute(33)             -- Not in RFC 1759

-- Values (30) to (39) reserved for Finisher MIB

}

PrtAlertCodeTC ::= TEXTUAL-CONVENTION

-- This TC was extracted from prtAlertCode in RFC 1759.

STATUS    current

DESCRIPTION

"The code that describes the type of alert for this entry in

the table. Binary change event alerts describe states of the

subunit while unary change event alerts describe a single

event. The same alert code can be used for a binary change

event or a unary change event, depending on implementation.

Also, the same alert code can be used to indicate a critical

or non-critical (warning) alert, depending on implementation.

The value of prtAlertSeverityLevel specifies binary vs. unary

and critical vs. non-critical for each event for the

implementation.

While there are some specific codes for many subunits, the

generic codes should be used for most subunit alerts. The

network management station can then query the subunit

specified by prtAlertGroup to determine further subunit

status and other subunit information.

An agent shall not add two entries to the alert table for the

same event, one containing a generic event code and the other

containing a specific event code; the agent shall add only

one entry in the alert table for each event; either generic

(preferred) or specific, not both.

Implementation of the unary change event

alertRemovalOfBinaryChangeEntry(1801) is optional. When

implemented, this alert code shall indicate to network

management stations that the trailing edge of a binary change

event has occurred and the corresponding alert entry has been

removed from the alert table. As with all events, the

alertRemovalOfBinaryChangeEntry(1801) alert shall be placed

at the end of the alert table. Such an alert table entry

shall specify the following information:

prtAlertSeverityLevel   warningUnaryChangeEvent(4)

prtAlertTrainingLevel   noInterventionRequired(7)

prtAlertGroup           alert(18)

prtAlertGroupIndex      the index of the row in the

alert table of the binary

change event that this event

has removed.

prtAlertLocation        unknown (-2)

prtAlertCode            alertRemovalOfBinaryChangeEntry(1801)

prtAlertDescription     <description or null string>

prtAlertTime            the value of sysUpTime at

the time of the removal of the

binary change event from the

alert table.

Optionally, the agent may generate a trap coincident with

removing the binary change event and placing the unary change

event alertRemovalOfBinaryChangeEntry(1801) in the alert

table. For such a trap, the prtAlertIndex sent with the above

trap parameters shall be the index of the

alertRemovalOfBinaryChangeEvent row that was added to the

prtAlertTable; not the index of the row that was removed from

the prtAlertTable."

SYNTAX    INTEGER {

other(1),

-- an event that is not represented

-- by one of the alert codes

-- specified below.

unknown(2),

-- The following generic codes are common to

-- multiple groups. The NMS may examine the

-- prtAlertGroup object to determine what group

-- to query for further information.

coverOpen(3),

coverClosed(4),

interlockOpen(5),

interlockClosed(6),

configurationChange(7),

jam(8),

subunitMissing(9),           -- Not in RFC 1759

-- The subunit tray, bin, etc.

-- has been removed.

subunitLifeAlmostOver(10),   -- Not in RFC 1759

subunitLifeOver(11),         -- Not in RFC 1759

subunitAlmostEmpty(12),      -- Not in RFC 1759

subunitEmpty(13),            -- Not in RFC 1759

subunitAlmostFull(14),       -- Not in RFC 1759

subunitFull(15),             -- Not in RFC 1759

subunitNearLimit(16),        -- Not in RFC 1759

subunitAtLimit(17),          -- Not in RFC 1759

subunitOpened(18),           -- Not in RFC 1759

subunitClosed(19),           -- Not in RFC 1759

subunitTurnedOn(20),         -- Not in RFC 1759

subunitTurnedOff(21),        -- Not in RFC 1759

subunitOffline(22),          -- Not in RFC 1759

subunitPowerSaver(23),       -- Not in RFC 1759

subunitWarmingUp(24),        -- Not in RFC 1759

subunitAdded(25),            -- Not in RFC 1759

subunitRemoved(26),          -- Not in RFC 1759

subunitResourceAdded(27),    -- Not in RFC 1759

subunitResourceRemoved(28),  -- Not in RFC 1759

subunitRecoverableFailure(29),

-- Not in RFC 1759

subunitUnrecoverableFailure(30),

-- Not in RFC 1759

subunitRecoverableStorageError(31),

-- Not in RFC 1759

subunitUnrecoverableStorageError(32),

-- Not in RFC 1759

subunitMotorFailure(33),     -- Not in RFC 1759

subunitMemoryExhausted(34),  -- Not in RFC 1759

subunitUnderTemperature(35), -- Not in RFC 1759

subunitOverTemperature(36),  -- Not in RFC 1759

subunitTimingFailure(37),    -- Not in RFC 1759

subunitThermistorFailure(38), -- Not in RFC 1759

-- General Printer group

doorOpen(501),    -- DEPRECATED

-- Use coverOpened(3)

doorClosed(502),  -- DEPRECATED

-- Use coverClosed(4)

powerUp(503),

powerDown(504),

printerNMSReset(505),        -- Not in RFC 1759

-- The printer has been reset by some

-- network management station(NMS)

-- writing into 'prtGeneralReset'.

printerManualReset(506),     -- Not in RFC 1759

-- The printer has been reset manually.

printerReadyToPrint(507),    -- Not in RFC 1759

-- The printer is ready to print. (i.e.,

-- not warming up, not in power save

-- state, not adjusting print quality,

-- etc.).

-- Input Group

inputMediaTrayMissing(801),

inputMediaSizeChange(802),

inputMediaWeightChange(803),

inputMediaTypeChange(804),

inputMediaColorChange(805),

inputMediaFormPartsChange(806),

inputMediaSupplyLow(807),

inputMediaSupplyEmpty(808),

inputMediaChangeRequest(809), -- Not in RFC 1759

-- An interpreter has detected that a

-- different medium is need in this input

-- tray subunit. The prtAlertDescription may

-- be used to convey a human readable

-- description of the medium required to

-- satisfy the request.

inputManualInputRequest(810), -- Not in RFC 1759

-- An interpreter has detected that manual

-- input is required in this subunit. The

-- prtAlertDescription may be used to convey

-- a human readable description of the medium

-- required to satisfy the request.

inputTrayPositionFailure(811), -- Not in RFC 1759

-- The input tray failed to position correctly.

inputTrayElevationFailure(812),

-- Not in RFC 1759

inputCannotFeedSizeSelected(813),

-- Not in RFC 1759

-- Output Group

outputMediaTrayMissing(901),

outputMediaTrayAlmostFull(902),

outputMediaTrayFull(903),

outputMailboxSelectFailure(904),

-- Not in RFC 1759

-- Marker group

markerFuserUnderTemperature(1001),

markerFuserOverTemperature(1002),

markerFuserTimingFailure(1003),

-- Not in RFC 1759

markerFuserThermistorFailure(1004),

-- Not in RFC 1759

markerAdjustingPrintQuality(1005),

-- Not in RFC 1759

-- Marker Supplies group

markerTonerEmpty(1101),

markerInkEmpty(1102),

markerPrintRibbonEmpty(1103),

markerTonerAlmostEmpty(1104),

markerInkAlmostEmpty(1105),

markerPrintRibbonAlmostEmpty(1106),

markerWasteTonerReceptacleAlmostFull(1107),

markerWasteInkReceptacleAlmostFull(1108),

markerWasteTonerReceptacleFull(1109),

markerWasteInkReceptacleFull(1110),

markerOpcLifeAlmostOver(1111),

markerOpcLifeOver(1112),

markerDeveloperAlmostEmpty(1113),

markerDeveloperEmpty(1114),

markerTonerCartridgeMissing(1115),

-- Not in RFC 1759

-- Media Path Device Group

mediaPathMediaTrayMissing(1301),

mediaPathMediaTrayAlmostFull(1302),

mediaPathMediaTrayFull(1303),

mediaPathCannotDuplexMediaSelected(1304),

-- Not in RFC 1759

-- Interpreter Group

interpreterMemoryIncrease(1501),

interpreterMemoryDecrease(1502),

interpreterCartridgeAdded(1503),

interpreterCartridgeDeleted(1504),

interpreterResourceAdded(1505),

interpreterResourceDeleted(1506),

interpreterResourceUnavailable(1507),

interpreterComplexPageEncountered(1509),

-- Not in RFC 1759

-- The interpreter has encountered a page

-- that is too complex for the resources that

-- are available.

-- Alert Group

alertRemovalOfBinaryChangeEntry(1801)

-- Not in RFC 1759

-- A binary change event entry has been

-- removed from the alert table. This unary

-- change alert table entry is added to the

-- end of the alert table.

}

END

6.  The Printer MIB

Printer-MIB DEFINITIONS ::= BEGIN

IMPORTS

MODULE-IDENTITY, OBJECT-TYPE, Counter32, Integer32, TimeTicks,

NOTIFICATION-TYPE, OBJECT-IDENTITY, mib-2 FROM SNMPv2-SMI

TEXTUAL-CONVENTION FROM SNMPv2-TC

MODULE-COMPLIANCE, OBJECT-GROUP, NOTIFICATION-GROUP FROM

SNMPv2-CONF

hrDeviceIndex, hrStorageIndex FROM HOST-RESOURCES-MIB

InterfaceIndexOrZero FROM IF-MIB

PrtCoverStatusTC, PrtGeneralResetTC, PrtChannelTypeTC,

PrtInterpreterLangFamilyTC, PrtInputTypeTC, PrtOutputTypeTC,

PrtMarkerMarkTechTC, PrtMarkerSuppliesTypeTC, PrtConsoleColorTC,

PrtConsoleDisableTC, PrtMediaPathTypeTC, PrtAlertGroupTC,

PrtAlertTrainingLevelTC, PrtAlertCodeTC FROM IANA-PRINTER-MIB

IANACharset FROM IANA-CHARSET-MIB;

printmib MODULE-IDENTITY

LAST-UPDATED "200302100000Z"

ORGANIZATION "PWG IEEE/ISTO Printer Working Group"

CONTACT-INFO

"Harry Lewis

IBM

Phone (303) 924-5337

Email: harryl@us.ibm.com

http://www.pwg.org/index.html

Send comments to the printmib WG using the Printer MIB

Project (PMP) Mailing List:  pmp@pwg.org

For further information, access the PWG web page under 'Printer

MIB':      http://www.pwg.org/

Implementers of this specification are encouraged to join the

pmp mailing list in order to participate in discussions on any

clarifications needed and registration proposals being reviewed

in order to achieve consensus."

DESCRIPTION

"The MIB module for management of printers.

Copyright (C) The Internet Society (2003). This version of

this MIB module is part of RFC xxxx; see the RFC itself for

full legal notices."

-- RFC Editor assigns above RFC xxxx.

REVISION     "200302100000Z"

DESCRIPTION

"Printer MIB v2.

Moved all TCs to be maintained by IANA into the ianaPrinterMIB

which is contained in this document.

Five new OBJECT-GROUPs: prtAuxilliarySheetGroup,

prtInputSwitchingGroup, prtGeneralV2Group,

prtAlertTableV2Group, prtChannelV2Group.

Nine new objects added to those groups:

prtAuxiliarySheetStartupPage, prtAuxiliarySheetBannerPage,

prtGeneralPrinterName, prtGeneralSerialNumber,

prtAlertCriticalEvents, prtAlertAllEvents,

prtInputMediaLoadTimeout, prtInputNextIndex,

prtChannelInformation.

SYNTAX range changed from (0..65535) to (1..65535) for the

index objects prtStorageRefSeqNumber and

prtDeviceRefSeqNumber.

SYNTAX range changed from (0..65535) to (0..2147483647) for the

objects prtStorageRefIndex and prtDeviceRefIndex to agree

with the Host Resources MIB.

Defined a range for the objects with a SYNTAX of Integer32:

prtOutputDefaultIndex, prtInputMediaDimFeedDirDeclared,

prtInputMediaDimXFeedDirDeclared, prtInputMaxCapacity,

prtInputCurrentLevel, prtInputMediaDimFeedDirChosen,

prtInputMediaDimXFeedDirChosen, prtInputMediaWeight,

prtInputMediaFormParts, prtOutputIndex,

prtOutputMaxCapacity, prtOutputRemainingCapacity,

prtOutputMaxDimFeedDir, prtOutputMaxDimXFeedDir,

prtOutputMinDimFeedDir, prtOutputMinDimXFeedDir,

prtMarkerAddressibilityFeedDir,

prtMarkerAddressibilityXFeedDir, prtMarkerNorthMargin,

prtMarkerSouthMargin, prtMarkerWestMargin,

prtMarkerEastMargin, prtMarkerSuppliesMaxCapacity,

prtMarkerSuppliesLevel, prtMarkerColorantTonality,

prtMediaPathMaxSpeed, prtMediaPathMaxMediaFeedDir,

prtMediaPathMaxMediaXFeedDir, prtMediaPathMinMediaFeedDir,

prtMediaPathMinMediaXFeedDir, prtChannelIndex,

prtChannelCurrentJobCntlLangIndex, prtInterpreterIndex,

prtChannelDefaultPageDescLangIndex, prtConsoleOnTime,

prtInterpreterFeedAddressibility, prtConsoleOffTime,

prtInterpreterXFeedAddressibility, prtAlertIndex,

prtAlertGroupIndex, prtAlertLocation.

Changed SYNTAX from Integer32 to InterfaceIndexOrZero for

prtChannelIfIndex.

One new NOTIFICATION-GROUP: prtAlertTrapGroup which contains

printerV2Alert.

In MODULE-COMPLIANCE prtMIBCompliance, new OBJECT-GROUPs and

the NOTIFICATION_GROUP, all in GROUP (not MANDATORY-GROUP)

clauses.  The nine new objects are optional, i.e., this

draft is backward compatible with RFC 1759."

REVISION     "199411250000Z"

DESCRIPTION

"The original version of this MIB, published as RFC1759."

::= { mib-2 43 }

-- Textual conventions for this MIB module

--

-- Generic unspecific textual conventions

--

PrtMediaUnitTC ::= TEXTUAL-CONVENTION

-- Replaces MediaUnit in RFC 1759.

-- Even though it is formally not allowed to change TC names, the WG

-- decided this name change was necessary to clearly indicate this

-- TC is from the Printer MIB.

STATUS    current

DESCRIPTION

"Units of measure for media dimensions."

SYNTAX    INTEGER {

tenThousandthsOfInches(3),  -- .0001

micrometers(4)

}

MediaUnit ::= TEXTUAL-CONVENTION

-- Replaced by prtMediaUnitTC.

STATUS    deprecated

DESCRIPTION

"Units of measure for media dimensions."

SYNTAX    INTEGER {

tenThousandthsOfInches(3),  -- .0001

micrometers(4)

}

PrtCapacityUnitTC ::= TEXTUAL-CONVENTION

-- Replaces CapacityUnit in RFC 1759.

-- Even though it is formally not allowed to change TC names, the WG

-- decided this name change was necessary due to the added values

-- and to clearly indicate this TC is from the Printer MIB.

STATUS    current

DESCRIPTION

"Units of measure for media capacity."

SYNTAX    INTEGER {

other(1),                   -- New, not in RFC 1759

unknown(2),                 -- New, not in RFC 1759

tenThousandthsOfInches(3),  -- .0001

micrometers(4),

sheets(8),

feet(16),

meters(17),

-- Values for Finisher MIB

items(18),                  -- New, not in RFC 1759

percent(19)                 -- New, not in RFC 1759

}

CapacityUnit ::= TEXTUAL-CONVENTION

-- Replaced by prtCapacityUnitTC.

STATUS    deprecated

DESCRIPTION

"Units of measure for media capacity."

SYNTAX    INTEGER {

tenThousandthsOfInches(3),  -- .0001

micrometers(4),

sheets(8),

feet(16),

meters(17)

}

PrtPrintOrientationTC ::= TEXTUAL-CONVENTION

STATUS    current

DESCRIPTION

"A generic representation for printing orientation on a

'page'."

SYNTAX    INTEGER {

other(1),

portrait(3),

landscape(4)

}

PrtSubUnitStatusTC ::= TEXTUAL-CONVENTION

-- Replaces SubUnitStatus in RFC

-- 1759.  Even though it is formally not allowed to change TC names,

-- the WG decided this name change was necessary to indicate this TC

-- is from the Printer MIB.

STATUS    current

DESCRIPTION

"Status of a printer sub-unit.

The SubUnitStatus is an integer that is the sum of 5 distinct

values, Availability, Non-Critical, Critical, On-line, and

Transitioning. These values are:

Availability                           Value

Available and Idle                  0       0000'b

Available and Standby               2       0010'b

Available and Active                4       0100'b

Available and Busy                  6       0110'b

Unavailable and OnRequest           1       0001'b

Unavailable because Broken          3       0011'b

Unknown                             5       0101'b

Non-Critical

No Non-Critical Alerts              0       0000'b

Non-Critical Alerts                 8       1000'b

Critical

No Critical Alerts                  0       0000'b

Critical Alerts                    16     1 0000'b

On-Line

State is On-Line                    0       0000'b

State is Off-Line                  32    10 0000'b

Transitioning

At intended state                   0       0000'b

Transitioning to intended state    64   100 0000'b"

SYNTAX    INTEGER (0..126)

SubUnitStatus ::= TEXTUAL-CONVENTION

-- Replaced by prtSubUnitStatusTC.

STATUS    deprecated

DESCRIPTION

"Status of a printer sub-unit.

The SubUnitStatus is an integer that is the sum of 5 distinct

values, Availability, Non-Critical, Critical, On-line, and

Transitioning. These values are:

Availability                           Value

Available and Idle                  0       0000'b

Available and Standby               2       0010'b

Available and Active                4       0100'b

Available and Busy                  6       0110'b

Unavailable and OnRequest           1       0001'b

Unavailable because Broken          3       0011'b

Unknown                             5       0101'b

Non-Critical

No Non-Critical Alerts              0       0000'b

Non-Critical Alerts                 8       1000'b

Critical

No Critical Alerts                  0       0000'b

Critical Alerts                    16     1 0000'b

On-Line

State is On-Line                    0       0000'b

State is Off-Line                  32    10 0000'b

Transitioning

At intended state                   0       0000'b

Transitioning to intended state    64   100 0000'b"

SYNTAX    INTEGER (0..126)

PresentOnOff ::= TEXTUAL-CONVENTION

STATUS    current

DESCRIPTION

"Presence and configuration of a device or feature."

SYNTAX    INTEGER {

other(1),

on(3),

off(4),

notPresent(5)

}

PrtLocalizedDescriptionStringTC ::= TEXTUAL-CONVENTION

-- This TC did not appear in RFC 1759.

STATUS    current

DESCRIPTION

"An object MUST use this textual convention when its

'charset' is controlled by the value of

prtGeneralCurrentLocalization."

SYNTAX    OCTET STRING (SIZE(0..255))

PrtConsoleDescriptionStringTC ::= TEXTUAL-CONVENTION

-- This TC did not appear in RFC 1759.

STATUS    current

DESCRIPTION

"An object MUST use this textual convention when its

'charset' is controlled by the value of

prtConsoleLocalization."

SYNTAX    OCTET STRING (SIZE(0..255))

CodedCharSet ::= TEXTUAL-CONVENTION

-- Replaced by IANACharset textual convention in IANA Charset MIB.

STATUS     deprecated

DESCRIPTION

"The original description clause from RFC 1759 [RFC1759] was

technically inaccurate and therefore has been deleted."

SYNTAX     INTEGER {

other(1)               -- used if the designated coded

-- character set is not currently in

-- the enumeration

}

--

-- Channel Group textual-conventions

--

PrtChannelStateTC ::= TEXTUAL-CONVENTION

-- This TC was extracted from prtChannelState in RFC 1759.

STATUS    current

DESCRIPTION

"The state of this print job delivery channel. The value

determines whether print data is allowed through this channel."

SYNTAX    INTEGER {

other(1),

printDataAccepted(3),

noDataAccepted(4)

}

--

-- Input/Output Group Textual Conventions

--

PrtOutputStackingOrderTC ::= TEXTUAL-CONVENTION

-- This TC was extracted from prtOutputStackingOrder in RFC 1759.

STATUS    current

DESCRIPTION

"The current state of the stacking order for the associated

output sub-unit. 'firstToLast' means that as pages are output,

the front of the next page is placed against the back of the

previous page. 'lastToFirst' means that as pages are output,

the back of the next page is placed against the front of the

previous page."

SYNTAX    INTEGER {

unknown(2),

firstToLast(3),

lastToFirst(4)

}

PrtOutputPageDeliveryOrientationTC ::= TEXTUAL-CONVENTION

-- This TC was extracted from prtOutputPageDeliveryOrientation

-- in RFC 1759.

STATUS    current

DESCRIPTION

"The reading surface that will be 'up' when pages are delivered

to the associated output sub-unit. Values are Face-Up and Face

Down (Note: interpretation of these values is, in general,

context-dependent based on locale; presentation of these values

to an end-user should be normalized to the expectations of the

user."

SYNTAX    INTEGER {

faceUp(3),

faceDown(4)

}

--

-- Marker Group Textual Conventions

--

PrtMarkerCounterUnitTC ::= TEXTUAL-CONVENTION

-- This TC was extracted from prtMarkerCounterUnit in RFC 1759.

STATUS    current

DESCRIPTION

"The unit that will be used by the printer when reporting

counter values for this marking sub-unit.  The

time units of measure are provided for a device like a

strip recorder that does not or cannot track the physical

dimensions of the media and does not use characters,

lines or sheets."

SYNTAX    INTEGER {

tenThousandthsOfInches(3),  -- .0001

micrometers(4),

characters(5),

lines(6),

impressions(7),

sheets(8),

dotRow(9),

hours(11),

feet(16),

meters(17)

}

PrtMarkerSuppliesSupplyUnitTC ::= TEXTUAL-CONVENTION

-- This TC was extracted from prtMarkerSuppliesSupplyUnit

-- in RFC 1759.

STATUS    current

DESCRIPTION

"Unit of this marker supply container/receptacle."

SYNTAX    INTEGER {

other(1),                   -- New, not in RFC 1759

unknown(2),                 -- New, not in RFC 1759

tenThousandthsOfInches(3),  -- .0001

micrometers(4),

impressions(7),             -- New, not in RFC 1759

sheets(8),                  -- New, not in RFC 1759

hours(11),                  -- New, not in RFC 1759

thousandthsOfOunces(12),

tenthsOfGrams(13),

hundrethsOfFluidOunces(14),

tenthsOfMilliliters(15),

feet(16),                   -- New, not in RFC 1759

meters(17),                 -- New, not in RFC 1759

-- Values for Finisher MIB

items(18),   -- e.g. #staples. New, not in RFC 1759

percent(19)                 -- New, not in RFC 1759

}

PrtMarkerSuppliesClassTC ::= TEXTUAL-CONVENTION

-- This TC was extracted from prtMarkerSuppliesClass in RFC 1759.

STATUS    current

DESCRIPTION

"Indicates whether this supply entity represents a supply

that is consumed or a receptacle that is filled."

SYNTAX    INTEGER {

other(1),

supplyThatIsConsumed(3),

receptacleThatIsFilled(4)

}

PrtMarkerColorantRoleTC ::= TEXTUAL-CONVENTION

-- This TC was extracted from prtMarkerColorantRole in RFC 1759.

STATUS    current

DESCRIPTION

"The role played by this colorant."

SYNTAX    INTEGER { --  Colorant Role

other(1),

process(3),

spot(4)

}

PrtMarkerAddressabilityUnitTC ::= TEXTUAL-CONVENTION

-- This TC was extracted from prtMarkerAddressabilityUnit

-- in RFC 1759.

STATUS    current

DESCRIPTION

"The unit of measure of distances, as applied to the marker's

resolution."

SYNTAX    INTEGER {

tenThousandthsOfInches(3), -- .0001

micrometers(4)

}

--

-- Media Path Textual Conventions

--

PrtMediaPathMaxSpeedPrintUnitTC ::= TEXTUAL-CONVENTION

-- This TC was extracted from prtMediaPathMaxSpeedPrintUnit

-- in RFC 1759.

STATUS    current

DESCRIPTION

"The unit of measure used in specifying the speed of all

media paths in the printer."

SYNTAX    INTEGER {

tenThousandthsOfInchesPerHour(3),-- .0001/hour

micrometersPerHour(4),

charactersPerHour(5),

linesPerHour(6),

impressionsPerHour(7),

sheetsPerHour(8),

dotRowPerHour(9),

feetPerHour(16),

metersPerHour(17)

}

--

-- Interpreter Group Textual Conventions

--

PrtInterpreterTwoWayTC ::= TEXTUAL-CONVENTION

-- This TC was extracted from prtInterpreterTwoWay in RFC 1759.

STATUS    current

DESCRIPTION

"Indicates whether or not this interpreter returns information

back to the host."

SYNTAX    INTEGER {

yes(3),

no(4)

}

--

-- Alert Group Textual Conventions

--

PrtAlertSeverityLevelTC ::= TEXTUAL-CONVENTION

-- This TC was extracted from prtAlertSeverityLevel in RFC 1759.

STATUS    current

DESCRIPTION

"The level of severity of this alert table entry.  The printer

determines the severity level assigned to each entry in the

table. A critical alert is binary by nature and definition. A

warning is defined to be a non-critical alert. The original and

most common warning is unary. The binary warning was added later

and given a more distinguished name."

SYNTAX    INTEGER {

other(1),

critical(3),

warning(4),

warningBinaryChangeEvent(5)   -- New, not in RFC 1759

}

-- The General Printer Group

--

-- The general printer sub-unit is responsible for the overall

-- control and status of the printer.  There is exactly one

-- general printer sub-unit in a printer.

prtGeneral OBJECT IDENTIFIER ::= { printmib 5 }

prtGeneralTable OBJECT-TYPE

SYNTAX     SEQUENCE OF PrtGeneralEntry

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"A table of general information per printer.

Objects in this table are defined in various

places in the MIB, nearby the groups to

which they apply.  They are all defined

here to minimize the number of tables that would

otherwise need to exist."

::= { prtGeneral 1 }

prtGeneralEntry OBJECT-TYPE

SYNTAX     PrtGeneralEntry

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"An entry exists in this table for each device entry in the

host resources MIB device table with a device type of

'printer'.

NOTE: The above description has been modified from RFC 1759

for clarification."

INDEX    { hrDeviceIndex }

::= { prtGeneralTable 1 }

PrtGeneralEntry ::= SEQUENCE {

-- Note that not all of the objects in this sequence are in

-- the general printer group. The group to which an

-- object belongs is tagged with a label "General", "Input"

-- "Output", etc. after each entry in the following sequence.

--

prtGeneralConfigChanges         Counter32, -- General

prtGeneralCurrentLocalization   Integer32, -- General

prtGeneralReset                 PrtGeneralResetTC,

-- General

prtGeneralCurrentOperator       OCTET STRING,

-- Responsible Party

prtGeneralServicePerson         OCTET STRING,

-- Responsible Party

prtInputDefaultIndex            Integer32, -- Input

prtOutputDefaultIndex           Integer32, -- Output

prtMarkerDefaultIndex           Integer32, -- Marker

prtMediaPathDefaultIndex        Integer32, -- Media Path

prtConsoleLocalization          Integer32, -- Console

prtConsoleNumberOfDisplayLines  Integer32, -- Console

prtConsoleNumberOfDisplayChars  Integer32, -- Console

prtConsoleDisable               PrtConsoleDisableTC,

-- Console,

prtAuxiliarySheetStartupPage    PresentOnOff,

-- AuxiliarySheet

prtAuxiliarySheetBannerPage     PresentOnOff,

-- AuxiliarySheet

prtGeneralPrinterName           OCTET STRING,

-- General V2

prtGeneralSerialNumber          OCTET STRING,

-- General V2

prtAlertCriticalEvents          Counter32, -- Alert V2

prtAlertAllEvents               Counter32  -- Alert V2

}

prtGeneralConfigChanges OBJECT-TYPE

SYNTAX     Counter32

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"Counts configuration changes within the printer. A

configuration change is defined to be an action that results in

a change to any MIB object other than those that reflect status

or level, or those that act as counters or gauges. In addition,

any action that results in a row being added or deleted from

any table in the Printer MIB is considered a configuration

change. Such changes will often affect the capability of the

printer to service certain types of print jobs. Management

applications may cache infrequently changed configuration

information about sub units within the printer. This object

should be incremented whenever the agent wishes to notify

management applications that any cached configuration

information for this device is to be considered 'stale'. At

this point, the management application should flush any

configuration information cached about this device and fetch

new configuration information.

The following are examples of actions that would cause the

prtGeneralConfigChanges object to be incremented:

- Adding an output bin

- Changing the media in a sensing input tray

- Changing the value of prtInputMediaType

Note that the prtGeneralConfigChanges counter would not be

incremented when an input tray is temporarily removed to load

additional paper or when the level of an input device changes.

NOTE: The above description has been modified from RFC 1759

for clarification."

::= { prtGeneralEntry 1 }

prtGeneralCurrentLocalization OBJECT-TYPE

SYNTAX     Integer32 (1..65535)

MAX-ACCESS read-write

STATUS     current

DESCRIPTION

"The value of the prtLocalizationIndex corresponding to the

current language, country, and character set to be used for

localized string values that are identified as being dependent

on the value of this object.  Note that this object does not

apply to localized strings in the prtConsole group or to any

object that is not explicitly identified as being localized

according to prtGeneralCurrentLocalization.  When an object's

'charset' is controlled by the value of

prtGeneralCurrentLocalization, it MUST specify

PrtLocalizedDescriptionStringTC as its syntax.

NOTE: The above description has been modified from RFC 1759

for clarification."

::= { prtGeneralEntry 2 }

prtGeneralReset OBJECT-TYPE

-- NOTE: In RFC 1759, the enumeration values were implicitly defined

-- by this object.

SYNTAX     PrtGeneralResetTC

MAX-ACCESS read-write

STATUS     current

DESCRIPTION

"Setting this value to 'powerCycleReset', 'resetToNVRAM', or

'resetToFactoryDefaults' will result in the resetting of the

printer.  When read, this object will always have the value

'notResetting(3)', and a SET of the value 'notResetting' shall

have no effect on the printer.  Some of the defined values are

optional.  However, every implementation must support at least

the values 'notResetting' and 'resetToNVRAM'."

::= { prtGeneralEntry 3 }

-- The Responsible Party group

prtGeneralCurrentOperator OBJECT-TYPE

SYNTAX     OCTET STRING (SIZE(0..127))

MAX-ACCESS read-write

STATUS     current

DESCRIPTION

"The name of the person who is responsible for operating

this printer. It is suggested that this string include

information that would enable other humans to reach the

operator, such as a phone number. As a convention to

facilitate automatic notification of the operator by the

agent or network management station, the phone number,

fax number or email address should be indicated by the

URL schemes 'tel:', 'fax:' and 'mailto:', respectively.

If either the phone, fax, or email information is not

available, then a line should not be included for this

information.

NOTE: For interoperability purposes, it is advisable to

use email addresses formatted according to [RFC2822]

requirements.

NOTE: The above description has been modified from RFC 1759

for clarification."

::= { prtGeneralEntry 4 }

prtGeneralServicePerson OBJECT-TYPE

SYNTAX     OCTET STRING (SIZE(0..127))

MAX-ACCESS read-write

STATUS     current

DESCRIPTION

"The name of the person responsible for servicing this

printer. It is suggested that this string include

information that would enable other humans to reach the

service person, such as a phone number. As a convention

to facilitate automatic notification of the operator by

the agent or network management station, the phone

number, fax number or email address should be indicated

by the URL schemes 'tel:', 'fax:' and 'mailto:',

respectively. If either the phone, fax, or email

information is not available, then a line should not

be included for this information.

NOTE: For interoperability purposes, it is advisable to use

email addresses formatted per [RFC2822] requirements.

NOTE: The above description has been modified from RFC 1759

for clarification."

::= { prtGeneralEntry 5 }

-- Default indexes section

--

-- The following four objects are used to specify the indexes of

-- certain subunits used as defaults during the printing process.

prtInputDefaultIndex OBJECT-TYPE

SYNTAX     Integer32 (1..65535)

MAX-ACCESS read-write

STATUS     current

DESCRIPTION

"The value of prtInputIndex corresponding to the default input

sub-unit: that is, this object selects the default source of

input media."

::= { prtGeneralEntry 6 }

prtOutputDefaultIndex OBJECT-TYPE

--  A range has been added to the SYNTAX clause that was not in

--  RFC 1759.  Although this violates SNMP compatibility rules,

--  it provides a more reasonable guide for SNMP managers.

SYNTAX     Integer32 (1..65535)

MAX-ACCESS read-write

STATUS     current

DESCRIPTION

"The value of prtOutputIndex corresponding to the default

output sub-unit; that is, this object selects the default

output destination."

::= { prtGeneralEntry 7 }

prtMarkerDefaultIndex OBJECT-TYPE

SYNTAX     Integer32 (1..65535)

MAX-ACCESS read-write

STATUS     current

DESCRIPTION

"The value of prtMarkerIndex corresponding to the

default marker sub-unit; that is, this object selects the

default marker."

::= { prtGeneralEntry 8 }

prtMediaPathDefaultIndex OBJECT-TYPE

SYNTAX     Integer32 (1..65535)

MAX-ACCESS read-write

STATUS     current

DESCRIPTION

"The value of prtMediaPathIndex corresponding to

the default media path; that is, the selection of the

default media path."

::= { prtGeneralEntry 9 }

-- Console general section

--

-- The following four objects describe overall parameters of the

-- printer console subsystem.

prtConsoleLocalization OBJECT-TYPE

SYNTAX     Integer32 (1..65535)

MAX-ACCESS read-write

STATUS     current

DESCRIPTION

"The value of the prtLocalizationIndex corresponding to

the language, country, and character set to be used for the

console.  This localization applies both to the actual display

on the console as well as the encoding of these console objects

in management operations.  When an object's 'charset' is

controlled by the value of prtConsoleLocalization, it MUST

specify PrtConsoleDescriptionStringTC as its syntax.

NOTE: The above description has been modified from RFC 1759

for clarification."

::= { prtGeneralEntry 10 }

prtConsoleNumberOfDisplayLines OBJECT-TYPE

SYNTAX     Integer32 (0..65535)

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The number of lines on the printer's physical

display.  This value is 0 if there are no lines on the

physical display or if there is no physical display"

::= { prtGeneralEntry 11 }

prtConsoleNumberOfDisplayChars OBJECT-TYPE

SYNTAX     Integer32 (0..65535)

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The number of characters per line displayed on the physical

display. This value is 0 if there are no lines on the physical

display or if there is no physical display"

::= { prtGeneralEntry 12 }

prtConsoleDisable OBJECT-TYPE

-- In RFC 1759, the enumeration values were implicitly defined

-- by this object.

SYNTAX     PrtConsoleDisableTC

MAX-ACCESS read-write

STATUS     current

DESCRIPTION

"This value indicates how input is (or is not) accepted from

the operator console.

NOTE: The above description has been modified from RFC 1759

for clarification."

::= { prtGeneralEntry 13 }

-- The Auxiliary Sheet Group

--

-- The auxiliary sheet group allows the administrator to control

-- the production of auxiliary sheets by the printer. This group

-- contains only the "prtAuxiliarySheetStartupPage" and

-- "prtAuxiliarySheetBannerPage" objects.

prtAuxiliarySheetStartupPage OBJECT-TYPE

SYNTAX     PresentOnOff

MAX-ACCESS read-write

STATUS     current

DESCRIPTION

"Used to enable or disable printing a startup page. If enabled,

a startup page will be printed shortly after power-up, when the

device is ready. Typical startup pages include test patterns

and/or printer configuration information."

::= { prtGeneralEntry 14 }

prtAuxiliarySheetBannerPage OBJECT-TYPE

SYNTAX     PresentOnOff

MAX-ACCESS read-write

STATUS     current

DESCRIPTION

"Used to enable or disable printing banner pages at the

beginning of jobs. This is a master switch which applies to all

jobs, regardless of interpreter."

::= { prtGeneralEntry 15 }

-- Administrative section  (The General V2 Group)

--

-- The following two objects are used to specify administrative

-- information assigned to the printer.

prtGeneralPrinterName OBJECT-TYPE

SYNTAX     OCTET STRING (SIZE (0..127))

MAX-ACCESS read-write

STATUS     current

DESCRIPTION

"An administrator-specified name for this printer. Depending

upon implementation of this printer, the value of this object

may or may not be same as the value for the MIB-II 'SysName'

object."

::= { prtGeneralEntry 16 }

prtGeneralSerialNumber OBJECT-TYPE

SYNTAX     OCTET STRING (SIZE (0..255))

MAX-ACCESS read-write

STATUS     current

DESCRIPTION

"A recorded serial number for this device that indexes some

type device catalog or inventory. This value is usually set by

the device manufacturer but the MIB supports the option of

writing for this object for site-specific administration of

device inventory or tracking."

::= { prtGeneralEntry 17 }

-- General alert table section  (Alert Table V2 Group)

--

-- The following two objects are used to specify counters

-- associated with the Alert Table.

prtAlertCriticalEvents OBJECT-TYPE

SYNTAX     Counter32

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"A running counter of the number of critical alert events that

have been recorded in the alert table. The value of this object

is RESET in the event of a power cycle operation (i.e., the

value is not persistent."

::= { prtGeneralEntry 18 }

prtAlertAllEvents OBJECT-TYPE

SYNTAX     Counter32

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"A running counter of the total number of alert event entries

(critical and non-critical) that have been recorded in the

alert table"

::= { prtGeneralEntry 19 }

-- The Cover Table

--

-- The cover portion of the General print sub-unit describes the

-- covers and interlocks of the printer.  The Cover Table has an

-- entry for each cover and interlock.

prtCover OBJECT IDENTIFIER ::= { printmib 6 }

prtCoverTable OBJECT-TYPE

SYNTAX     SEQUENCE OF PrtCoverEntry

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"A table of the covers and interlocks of the printer."

::= { prtCover 1 }

prtCoverEntry OBJECT-TYPE

SYNTAX     PrtCoverEntry

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"Information about a cover or interlock.

Entries may exist in the table for each device

index with a device type of 'printer'.

NOTE: The above description has been modified from RFC 1759

for clarification."

INDEX  { hrDeviceIndex, prtCoverIndex }

::= { prtCoverTable 1 }

PrtCoverEntry ::= SEQUENCE {

prtCoverIndex            Integer32,

prtCoverDescription      PrtLocalizedDescriptionStringTC,

prtCoverStatus           PrtCoverStatusTC

}

prtCoverIndex OBJECT-TYPE

SYNTAX     Integer32 (1..65535)

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"A unique value used by the printer to identify this Cover sub

unit.  Although these values may change due to a major

reconfiguration of the device (e.g. the addition of new cover

sub-units to the printer), values SHOULD remain stable across

successive printer power cycles.

NOTE: The above description has been modified from RFC 1759

for clarification."

::= { prtCoverEntry 1 }

prtCoverDescription OBJECT-TYPE

-- In RFC 1759, the SYNTAX was OCTET STRING.  This has been changed

-- to a TC to better support localization of the object.

SYNTAX     PrtLocalizedDescriptionStringTC

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The manufacturer provided cover sub-mechanism name in the

localization specified by prtGeneralCurrentLocalization."

::= { prtCoverEntry 2 }

prtCoverStatus  OBJECT-TYPE

-- NOTE: In RFC 1759, the enumeration values were implicitly defined

-- by this object.

SYNTAX     PrtCoverStatusTC

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The status of this cover sub-unit."

::= { prtCoverEntry 3 }

-- The Localization Table

--

-- The localization portion of the General printer sub-unit is

-- responsible for identifying the natural language, country, and

-- character set in which character strings are expressed. There

-- may be one or more localizations supported per printer. The

-- available localizations are represented by the Localization

-- table.

prtLocalization  OBJECT IDENTIFIER ::= { printmib 7 }

prtLocalizationTable OBJECT-TYPE

SYNTAX     SEQUENCE OF PrtLocalizationEntry

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"The available localizations in this printer."

::= { prtLocalization 1 }

prtLocalizationEntry OBJECT-TYPE

SYNTAX     PrtLocalizationEntry

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"A description of a localization.

Entries may exist in the table for each device

index with a device type of 'printer'.

NOTE: The above description has been modified from RFC 1759

for clarification."

INDEX  { hrDeviceIndex, prtLocalizationIndex }

::= { prtLocalizationTable 1 }

PrtLocalizationEntry ::= SEQUENCE {

prtLocalizationIndex          Integer32,

prtLocalizationLanguage       OCTET STRING,

prtLocalizationCountry        OCTET STRING,

prtLocalizationCharacterSet   IANACharset

}

prtLocalizationIndex OBJECT-TYPE

SYNTAX     Integer32 (1..65535)

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"A unique value used by the printer to identify this

localization entry.  Although these values may change due to a

major reconfiguration of the device (e.g., the addition of new

localization data to the printer), values SHOULD remain

stable across successive printer power cycles.

NOTE: The above description has been modified from RFC 1759

for clarification."

::= { prtLocalizationEntry 1 }

prtLocalizationLanguage OBJECT-TYPE

-- Note: The size is fixed, was incorrectly 0..2 in RFC 1759.

SYNTAX     OCTET STRING (SIZE(2))

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"A two character language code from ISO 639.  Examples en,

es, fr, de.  NOTE: These examples were shown as upper case in

RFC 1759 and are now shown as lower case to agree with ISO 639."

::= { prtLocalizationEntry 2 }

prtLocalizationCountry OBJECT-TYPE

-- Note: The size is fixed, was incorrectly 0..2 in RFC 1759.

SYNTAX     OCTET STRING (SIZE(2))

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"A two character country code from ISO 3166, a blank string

(two space characters) shall indicate that the country is not

defined.  Examples: US, GB, FR, DE, ..."

::= { prtLocalizationEntry 3 }

prtLocalizationCharacterSet OBJECT-TYPE

SYNTAX     IANACharset

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The coded character set used for this localization."

::= { prtLocalizationEntry 4 }

-- The System Resources Tables

--

-- The Printer MIB makes use of the Host Resources MIB to

-- define system resources by referencing the storage

-- and device groups of the print group.

prtStorageRefTable OBJECT-TYPE

SYNTAX     SEQUENCE OF PrtStorageRefEntry

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"This table defines which printer, amongst multiple printers

serviced by one agent, owns which storage units.

NOTE: The above description has been modified from RFC 1759

for clarification."

::= { prtGeneral 2 }

prtStorageRefEntry OBJECT-TYPE

SYNTAX     PrtStorageRefEntry

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"This table will have an entry for each entry in the Host

Resources MIB storage table that represents storage associated

with a printer managed by this agent.

NOTE: The above description has been modified from RFC 1759

for clarification."

INDEX      { hrStorageIndex, prtStorageRefSeqNumber }

::= { prtStorageRefTable 1 }

PrtStorageRefEntry ::= SEQUENCE {

prtStorageRefSeqNumber  Integer32,

prtStorageRefIndex      Integer32

}

prtStorageRefSeqNumber OBJECT-TYPE

-- NOTE: The range has been changed from RFC 1759, which allowed a

-- minumum value of zero.  This was incorrect, since zero is not a

-- valid index.

SYNTAX     Integer32 (1..65535)

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"This value will be unique amongst all entries with a common

value of hrStorageIndex. This object allows a storage entry to

point to the multiple printer devices with which it is

associated."

::= { prtStorageRefEntry 1 }

prtStorageRefIndex OBJECT-TYPE

-- NOTE: The range has been changed from RFC 1759 to be compatible

-- with the defined range of hrDeviceIndex.

SYNTAX     Integer32 (0..2147483647)

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The value of the hrDeviceIndex of the printer device that this

storageEntry is associated with."

::= { prtStorageRefEntry 2 }

prtDeviceRefTable OBJECT-TYPE

SYNTAX     SEQUENCE OF PrtDeviceRefEntry

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"This table defines which printer, amongst multiple printers

serviced by one agent, is associated with which devices.

NOTE: The above description has been modified from RFC 1759

for clarification."

::= { prtGeneral 3 }

prtDeviceRefEntry OBJECT-TYPE

SYNTAX     PrtDeviceRefEntry

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"This table will have an entry for each entry in the Host

Resources MIB device table that represents a device associated

with a printer managed by this agent.

NOTE: The above description has been modified from RFC 1759

for clarification."

INDEX      { hrDeviceIndex, prtDeviceRefSeqNumber }

::= { prtDeviceRefTable 1 }

PrtDeviceRefEntry ::= SEQUENCE {

prtDeviceRefSeqNumber   Integer32,

prtDeviceRefIndex       Integer32

}

prtDeviceRefSeqNumber OBJECT-TYPE

-- NOTE: The range has been changed from RFC 1759, which allowed a

-- minumum value of zero.  This was incorrect, since zero is not a

-- valid index.

SYNTAX     Integer32 (1..65535)

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"This value will be unique amongst all entries with a common

value of hrDeviceIndex. This object allows a device entry to

point to the multiple printer devices with which it is

associated."

::= { prtDeviceRefEntry 1 }

prtDeviceRefIndex OBJECT-TYPE

-- NOTE: The range has been changed from RFC 1759 to be compatible

-- with the defined range of hrDeviceIndex.

SYNTAX     Integer32 (0..2147483647)

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The value of the hrDeviceIndex of the printer device that this

deviceEntry is associated with."

::= { prtDeviceRefEntry 2 }

-- The Input Group

--

-- Input sub-units are managed as a tabular, indexed collection

-- of possible devices capable of providing media for input to

-- the printing process. Input sub-units typically have a

-- location, a type, an identifier, a set of constraints on

-- possible media sizes and potentially other media

-- characteristics, and may be capable of indicating current

-- status or capacity.

prtInput   OBJECT IDENTIFIER ::= { printmib 8 }

prtInputTable OBJECT-TYPE

SYNTAX     SEQUENCE OF PrtInputEntry

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"A table of the devices capable of providing media for input to

the printing process."

::= { prtInput 2 }

prtInputEntry OBJECT-TYPE

SYNTAX     PrtInputEntry

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"Attributes of a device capable of providing media for input to

the printing process. Entries may exist in the table for each

device index with a device type of 'printer'.

NOTE: The above description has been modified from RFC 1759

for clarification."

INDEX  { hrDeviceIndex, prtInputIndex }

::= { prtInputTable 1 }

PrtInputEntry ::= SEQUENCE {

prtInputIndex                     Integer32,

prtInputType                      PrtInputTypeTC,

prtInputDimUnit                   PrtMediaUnitTC,

prtInputMediaDimFeedDirDeclared   Integer32,

prtInputMediaDimXFeedDirDeclared  Integer32,

prtInputMediaDimFeedDirChosen     Integer32,

prtInputMediaDimXFeedDirChosen    Integer32,

prtInputCapacityUnit              PrtCapacityUnitTC,

prtInputMaxCapacity               Integer32,

prtInputCurrentLevel              Integer32,

prtInputStatus                    PrtSubUnitStatusTC,

prtInputMediaName                 OCTET STRING,

prtInputName                      OCTET STRING,

prtInputVendorName                OCTET STRING,

prtInputModel                     OCTET STRING,

prtInputVersion                   OCTET STRING,

prtInputSerialNumber              OCTET STRING,

prtInputDescription               PrtLocalizedDescriptionStringTC,

prtInputSecurity                  PresentOnOff,

prtInputMediaWeight               Integer32,

prtInputMediaType                 OCTET STRING,

prtInputMediaColor                OCTET STRING,

prtInputMediaFormParts            Integer32,

prtInputMediaLoadTimeout          Integer32,

prtInputNextIndex                 Integer32

}

prtInputIndex OBJECT-TYPE

SYNTAX     Integer32 (1..65535)

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"A unique value used by the printer to identify this input

sub-unit.  Although these values may change due to a major

reconfiguration of the device (e.g. the addition of new input

sub-units to the printer), values SHOULD remain stable across

successive printer power cycles.

NOTE: The above description has been modified from RFC 1759

for clarification."

::= { prtInputEntry 1 }

prtInputType OBJECT-TYPE

-- NOTE: In RFC 1759, the enumeration values were implicitly

-- defined by this object.

SYNTAX     PrtInputTypeTC

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The type of technology (discriminated primarily according to

feeder mechanism type) employed by the input sub-unit.  Note,

the Input Class provides for a descriptor field to further

qualify the other choice."

::= { prtInputEntry 2 }

prtInputDimUnit OBJECT-TYPE

SYNTAX     PrtMediaUnitTC

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The unit of measurement for use calculating and relaying

dimensional values for this input sub-unit."

::= { prtInputEntry 3 }

prtInputMediaDimFeedDirDeclared OBJECT-TYPE

-- NOTE: In RFC 1759, the range was not defined.

SYNTAX     Integer32 (-2..2147483647)

MAX-ACCESS read-write

STATUS     current

DESCRIPTION

"This object provides the value of the declared dimension, in

the feed direction, of the media that is (or, if empty, was or

will be) in this input sub-unit.  The feed direction is the

direction in which the media is fed on this sub-unit.  This

dimension is measured in input sub-unit dimensional units

(controlled by prtInputDimUnit, which uses PrtMediaUnitTC).  If

this input sub-unit can reliably sense this value, the value is

sensed by the printer and may not be changed by management

requests.  Otherwise, the value may be changed. The value (-1)

means other and specifically means that this sub-unit places no

restriction on this parameter.  The value (-2) indicates

unknown."

::= { prtInputEntry 4 }

prtInputMediaDimXFeedDirDeclared OBJECT-TYPE

-- NOTE: In RFC 1759, the range was not defined.

SYNTAX     Integer32 (-2..2147483647)

MAX-ACCESS read-write

STATUS     current

DESCRIPTION

"This object provides the value of the declared dimension, in

the cross feed direction, of the media that is (or, if empty,

was or will be) in this input sub-unit.  The cross  feed

direction is ninety degrees relative to the feed direction

associated with this sub-unit. This dimension is measured in

input sub-unit dimensional units (controlled by

prtInputDimUnit,which uses PrtMediaUnitTC).  If this input sub-

unit can reliably sense this value, the value is sensed by the

printer and may not be changed by management requests.

Otherwise, the value may be changed. The value (-1) means other

and specifically means that this sub-unit places no restriction

on this parameter. The value (-2) indicates unknown."

::= { prtInputEntry 5 }

prtInputMediaDimFeedDirChosen OBJECT-TYPE

-- NOTE: In RFC 1759, the range was not defined.

SYNTAX     Integer32 (-2..2147483647)

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The printer will act as if media of the chosen dimension (in

the feed direction) is present in this input source.  Note that

this value will be used even if the input tray is empty. Feed

dimension measurements are taken relative to the feed direction

associated with that sub-unit and are in input sub-unit

dimensional units (controlled by prtInputDimUnit, which uses

PrtMediaUnitTC). If the printer supports the declared

dimension,the granted dimension is the same as the declared

dimension. If not, the granted dimension is set to the closest

dimension that the printer supports when the declared dimension

is set. The value (-1) means other and specifically indicates

that this sub-unit places no restriction on this parameter. The

value (-2)indicates unknown."

::= { prtInputEntry 6 }

prtInputMediaDimXFeedDirChosen OBJECT-TYPE

-- NOTE: In RFC 1759, the range was not defined.

SYNTAX     Integer32 (-2..2147483647)

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The printer will act as if media of the chosen dimension (in

the cross feed direction) is present in this input source. Note

that this value will be used even if the input tray is empty.

The cross feed direction is ninety degrees relative to the feed

direction associated with this sub-unit. This dimension is

measured in input sub-unit dimensional units (controlled by

prtInputDimUnit, which uses PrtMediaUnitTC).  If the printer

supports the declare dimension, the granted dimension is the

same as the declared dimension. If not, the granted dimension

is set to the closest dimension that the printer supports when

the declared dimension is set. The value (-1) means other and

specifically indicates that this sub-unit places no restriction

on this parameter.  The value (-2) indicates unknown."

::= { prtInputEntry 7 }

prtInputCapacityUnit OBJECT-TYPE

-- NOTE: In RFC 1759, the enumeration values were implicitly

-- defined by this object.

SYNTAX     PrtCapacityUnitTC

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The unit of measurement for use in calculating and relaying

capacity values for this input sub-unit."

::= { prtInputEntry 8 }

prtInputMaxCapacity OBJECT-TYPE

-- NOTE: In RFC 1759, the range was not defined.

SYNTAX     Integer32 (-2..2147483647)

MAX-ACCESS read-write

STATUS     current

DESCRIPTION

"The maximum capacity of the input sub-unit in input sub-unit

capacity units (PrtCapacityUnitTC).  There is no convention

associated with the media itself so this value reflects claimed

capacity. If this input sub-unit can reliably sense this value,

the value is sensed by the printer and may not be changed by

management requests; otherwise, the value may be written (by a

Remote Control Panel or a Management Application). The value

(-1) means other and specifically indicates that the sub-unit

places no restrictions on this parameter.  The value (-2) means

unknown."

::= { prtInputEntry 9 }

prtInputCurrentLevel OBJECT-TYPE

-- NOTE: In RFC 1759, the range was not defined.

SYNTAX     Integer32 (-3..2147483647)    -- in capacity units

-- (PrtCapacityUnitTC).

MAX-ACCESS read-write

STATUS     current

DESCRIPTION

"The current capacity of the input sub-unit in input sub-unit

capacity units (PrtCapacityUnitTC). If this input sub-unit can

reliably sense this value, the value is sensed by the printer

and may not be changed by management requests; otherwise, the

value may be written (by a Remote Control Panel or a Management

Application).  The value (-1) means other and specifically

indicates that the sub-unit places no restrictions on this

parameter. The value (-2) means unknown. The value (-3) means

that the printer knows that at least one unit remains."

::= { prtInputEntry 10 }

prtInputStatus OBJECT-TYPE

SYNTAX     PrtSubUnitStatusTC

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The current status of this input sub-unit."

::= { prtInputEntry 11 }

prtInputMediaName OBJECT-TYPE

SYNTAX     OCTET STRING (SIZE(0..63))

MAX-ACCESS read-write

STATUS     current

DESCRIPTION

"A description of the media contained in this input sub-unit;

This description is to be used by a client to format and

Localize a string for display to a human operator.  This

description is not processed by the printer.  It is used to

provide information not expressible in terms of the other

media attributes (e.g. prtInputMediaDimFeedDirChosen,

prtInputMediaDimXFeedDirChosen, prtInputMediaWeight,

prtInputMediaType)."

-- The following reference was not included in RFC 1759.

REFERENCE

"The PWG Standardized Media Names specification [PWGMEDIA]

contains the recommended values for this object.  See also

RFC XXXX Appendix C,'Media Names', which lists the values

Of standardized media names defined in ISO/IEC 10175.

RFC Editor: Replace XXXX with the number assigned to this document."

::= { prtInputEntry 12 }

--             INPUT MEASUREMENT

--

--     _______ |               |

--      ^      |               |

--      |      |               |                 |

--      |      |_ _ _ _ _ _ _ _| _______________ |direction

--      |      |               |      ^          v

-- MaxCapacity |    Sheets     |      |

--      |      |     left      | CurrentLevel

--      |      |      in       |      |

--      v      |     tray      |      v

--     _______ +_______________+ _______

-- The Extended Input Group

prtInputName OBJECT-TYPE

SYNTAX     OCTET STRING (SIZE(0..63))

MAX-ACCESS read-write

STATUS     current

DESCRIPTION

"The name assigned to this input sub-unit."

::= { prtInputEntry 13 }

prtInputVendorName OBJECT-TYPE

SYNTAX     OCTET STRING (SIZE(0..63))

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The vendor name of this input sub-unit."

::= { prtInputEntry 14 }

prtInputModel OBJECT-TYPE

SYNTAX     OCTET STRING (SIZE(0..63))

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The model name of this input sub-unit."

::= { prtInputEntry 15 }

prtInputVersion OBJECT-TYPE

SYNTAX     OCTET STRING (SIZE(0..63))

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The version of this input sub-unit."

::= { prtInputEntry 16 }

prtInputSerialNumber OBJECT-TYPE

SYNTAX     OCTET STRING (SIZE(0..32))

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The serial number assigned to this input sub-unit."

::= { prtInputEntry 17 }

prtInputDescription OBJECT-TYPE

-- In RFC 1759, the SYNTAX was OCTET STRING.  This has been changed

-- to a TC to better support localization of the object.

SYNTAX     PrtLocalizedDescriptionStringTC

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"A free-form text description of this input sub-unit in the

localization specified by  prtGeneralCurrentLocalization."

::= { prtInputEntry 18 }

prtInputSecurity OBJECT-TYPE

SYNTAX     PresentOnOff

MAX-ACCESS read-write

STATUS     current

DESCRIPTION

"Indicates if this input sub-unit has some security associated

with it."

::= { prtInputEntry 19 }

-- The Input Media Group

--

-- The Input Media Group supports identification of media

-- installed or available for use on a printing device.

-- Medium resources are identified by name, and include a

-- collection of characteristic attributes that may further be

-- used for selection and management of them.

-- The Input Media group consists of a set of optional

-- "columns" in the Input Table. In this manner, a minimally

-- conforming implementation may choose to not support reporting

-- of media resources if it cannot do so.

prtInputMediaWeight  OBJECT-TYPE

-- NOTE: In RFC 1759, the range was not defined.

SYNTAX     Integer32 (-2..2147483647)

MAX-ACCESS read-write

STATUS     current

DESCRIPTION

"The weight of the medium associated with this input sub-unit

in grams / per meter squared. The value (-2) means unknown."

::= { prtInputEntry 20 }

prtInputMediaType OBJECT-TYPE

SYNTAX     OCTET STRING (SIZE(0..63))

MAX-ACCESS read-write

STATUS     current

DESCRIPTION

"The name of the type of medium associated with this input sub

unit. This name need not be processed by the printer; it might

simply be displayed to an operator.

NOTE: The above description has been modified from RFC 1759."

-- The following reference was not included in RFC 1759.

REFERENCE

"The PWG Standardized Media Names specification [PWGMEDIA],

section 3 Media Type Names, contains the recommended values for

this object. Implementers may add additional string values. The

naming conventions in ISO 9070 are recommended in order to

avoid potential name clashes."

::= { prtInputEntry 21 }

prtInputMediaColor OBJECT-TYPE

SYNTAX     OCTET STRING (SIZE(0..63))

MAX-ACCESS read-write

STATUS     current

DESCRIPTION

"The name of the color of the medium associated with

this input sub-unit using standardized string values.

NOTE: The above description has been modified from RFC 1759."

-- The following reference was not included in RFC 1759.

REFERENCE

"The PWG Standardized Media Names specification [PWGMEDIA],

section 4 Media Color Names, contains the recommended values

for this object. Implementers may add additional string values.

The naming conventions in ISO 9070 are recommended in order to

avoid potential name clashes."

::= { prtInputEntry 22 }

prtInputMediaFormParts OBJECT-TYPE

-- NOTE: In RFC 1759, the range was not defined.

SYNTAX     Integer32 (-2..2147483647)

MAX-ACCESS read-write

STATUS     current

DESCRIPTION

"The number of parts associated with the medium

associated with this input sub-unit if the medium is a

multi-part form.  The value (-1) means other and

specifically indicates that the device places no

restrictions on this parameter.  The value (-2) means

unknown."

::= { prtInputEntry 23 }

-- The Input Switching Group

--

-- The input switching group allows the administrator to set the

-- input subunit time-out for the printer and to control the

-- automatic input subunit switching by the printer when an input

-- subunit becomes empty.

prtInputMediaLoadTimeout OBJECT-TYPE

SYNTAX     Integer32 (-2..2147483647)

MAX-ACCESS read-write

STATUS     current

DESCRIPTION

"When the printer is not able to print due to a subunit being

empty or the requested media must be manually loaded, the

printer will wait for the duration (in seconds) specified by

this object. Upon expiration of the time-out, the printer will

take the action specified by prtInputNextIndex.

The event which causes the printer to enter the waiting state

is product specific. If the printer is not waiting for manually

fed media, it may switch from an empty subunit to a different

subunit without waiting for the time-out to expire.

A value of (-1) implies 'other' or 'infinite' which translates

to 'wait forever'. The action which causes printing to continue

is product specific. A value of (-2) implies 'unknown'."

::= { prtInputEntry 24 }

prtInputNextIndex OBJECT-TYPE

SYNTAX     Integer32 (-3..2147483647)

MAX-ACCESS read-write

STATUS     current

DESCRIPTION

"The value of prtInputIndex corresponding to the input subunit

which will be used when this input subunit is emptied and the

time-out specified by prtInputMediaLoadTimeout expires. A value

of zero(0) indicates that auto input switching will not occur

when this input subunit is emptied. If the time-out specified

by prtInputLoadMediaTimeout expires and this value is zero(0),

the job will be aborted. A value of (-1) means other. The value

(-2)means 'unknown' and specifically indicates that an

implementation specific method will determine the next input

subunit to use at the time this subunit is emptied and the time

out expires. The value(-3) means input switching is not

supported for this subunit."

::= { prtInputEntry 25 }

-- The Output Group

--

-- Output sub-units are managed as a tabular, indexed collection

-- of possible devices capable of receiving media delivered from

-- the printing process.  Output sub-units typically have a

-- location, a type, an identifier, a set of constraints on

-- possible media sizes and potentially other characteristics,

-- and may be capable of indicating current status or capacity.

prtOutput  OBJECT IDENTIFIER ::= { printmib 9 }

prtOutputTable OBJECT-TYPE

SYNTAX     SEQUENCE OF PrtOutputEntry

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"A table of the devices capable of receiving media delivered

from the printing process."

::= { prtOutput 2 }

prtOutputEntry OBJECT-TYPE

SYNTAX     PrtOutputEntry

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"Attributes of a device capable of receiving media delivered

from the printing process. Entries may exist in the table for

each device index with a device type of 'printer'.

NOTE: The above description has been modified from RFC 1759

for clarification."

INDEX  { hrDeviceIndex, prtOutputIndex }

::= { prtOutputTable 1 }

PrtOutputEntry ::= SEQUENCE {

prtOutputIndex                    Integer32,

prtOutputType                     PrtOutputTypeTC,

prtOutputCapacityUnit             PrtCapacityUnitTC,

prtOutputMaxCapacity              Integer32,

prtOutputRemainingCapacity        Integer32,

prtOutputStatus                   PrtSubUnitStatusTC,

prtOutputName                     OCTET STRING,

prtOutputVendorName               OCTET STRING,

prtOutputModel                    OCTET STRING,

prtOutputVersion                  OCTET STRING,

prtOutputSerialNumber             OCTET STRING,

prtOutputDescription              PrtLocalizedDescriptionStringTC,

prtOutputSecurity                 PresentOnOff,

prtOutputDimUnit                  PrtMediaUnitTC,

prtOutputMaxDimFeedDir            Integer32,

prtOutputMaxDimXFeedDir           Integer32,

prtOutputMinDimFeedDir            Integer32,

prtOutputMinDimXFeedDir           Integer32,

prtOutputStackingOrder        PrtOutputStackingOrderTC,

prtOutputPageDeliveryOrientation

PrtOutputPageDeliveryOrientationTC,

prtOutputBursting                 PresentOnOff,

prtOutputDecollating              PresentOnOff,

prtOutputPageCollated             PresentOnOff,

prtOutputOffsetStacking           PresentOnOff

}

prtOutputIndex OBJECT-TYPE

-- NOTE: In RFC 1759, the range was not defined.

SYNTAX     Integer32 (1..65535)

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"A unique value used by this printer to identify this output

sub-unit. Although these values may change due to a major

reconfiguration of the sub-unit (e.g.  the addition of new

output devices to the printer), values SHOULD remain stable

across successive printer power cycles.

NOTE: The above description has been modified from RFC 1759

for clarification."

::= { prtOutputEntry 1 }

prtOutputType OBJECT-TYPE

-- NOTE: In RFC 1759, the enumeration values were implicitly defined

-- by this object.

SYNTAX     PrtOutputTypeTC

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The type of technology supported by this output sub-unit."

::= { prtOutputEntry 2 }

prtOutputCapacityUnit OBJECT-TYPE

SYNTAX     PrtCapacityUnitTC

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The unit of measurement for use in calculating and relaying

capacity values for this output sub-unit."

::= { prtOutputEntry 3 }

prtOutputMaxCapacity OBJECT-TYPE

-- NOTE: In RFC 1759, the range was not defined.

SYNTAX     Integer32 (-2..2147483647)

MAX-ACCESS read-write

STATUS     current

DESCRIPTION

"The maximum capacity of this output sub-unit in output sub-

unit capacity units (PrtCapacityUnitTC). There is no convention

associated with the media itself so this value essentially

reflects claimed capacity. If this output sub-unit can reliably

sense this value, the value is sensed by the printer and may

not be changed by management requests; otherwise, the value may

be written (by a Remote Control Panel or a Management

Application). The value (-1) means other and specifically

indicates that the sub-unit places no restrictions on this

parameter. The value (-2) means unknown."

::= { prtOutputEntry 4 }

prtOutputRemainingCapacity OBJECT-TYPE

-- NOTE: In RFC 1759, the range was not defined.

SYNTAX     Integer32 (-3..2147483647)

MAX-ACCESS read-write

STATUS     current

DESCRIPTION

"The remaining capacity of the possible output sub-unit

capacity in output sub-unit capacity units

(PrtCapacityUnitTC)of this output sub-unit. If this output sub-

unit can reliably sense this value, the value is sensed by the

printer and may not be modified by management requests;

otherwise, the value may be written (by a Remote Control Panel

or a Management Application). The value (-1) means other and

specifically indicates that the sub-unit places no restrictions

on this parameter.  The value (-2) means unknown.  The value

(-3) means that the printer knows that there remains capacity

for at least one unit."

::= { prtOutputEntry 5 }

prtOutputStatus OBJECT-TYPE

SYNTAX     PrtSubUnitStatusTC

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The current status of this output sub-unit."

::= { prtOutputEntry 6 }

--            OUTPUT MEASUREMENT

--

--   _______  |                | ________

--      ^     |                |     ^

--      |     |                |     |

--      |     |                |RemainingCapacity

-- MaxCapacity|                |     |

--      |     |                |     v           ^

--      |     |_ _ _ _ _ _ _ _ | _______________ |direction

--      |     |    Sheets      |                 |

--      |     |      in        |

--      v     |    Output      |

--   _______  +________________+

-- The Extended Output Group

prtOutputName OBJECT-TYPE

SYNTAX     OCTET STRING (SIZE(0..63))

MAX-ACCESS read-write

STATUS     current

DESCRIPTION

"The name assigned to this output sub-unit."

::= { prtOutputEntry 7 }

prtOutputVendorName OBJECT-TYPE

SYNTAX     OCTET STRING (SIZE(0..63))

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The vendor name of this output sub-unit."

::= { prtOutputEntry 8 }

prtOutputModel OBJECT-TYPE

SYNTAX     OCTET STRING (SIZE(0..63))

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The model name assigned to this output sub-unit.

NOTE: The above description has been modified from RFC 1759

for clarification."

::= { prtOutputEntry 9 }

prtOutputVersion OBJECT-TYPE

SYNTAX     OCTET STRING (SIZE(0..63))

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The version of this output sub-unit."

::= { prtOutputEntry 10 }

prtOutputSerialNumber OBJECT-TYPE

SYNTAX     OCTET STRING (SIZE(0..63))

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The serial number assigned to this output sub-unit."

::= { prtOutputEntry 11 }

prtOutputDescription OBJECT-TYPE

-- In RFC 1759, the SYNTAX was OCTET STRING.  This has been changed

-- to a TC to better support localization of the object.

SYNTAX     PrtLocalizedDescriptionStringTC

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"A free-form text description of this output sub-unit in the

localization specified by prtGeneralCurrentLocalization."

::= { prtOutputEntry 12 }

prtOutputSecurity OBJECT-TYPE

SYNTAX     PresentOnOff

MAX-ACCESS read-write

STATUS     current

DESCRIPTION

"Indicates if this output sub-unit has some security associated

with it and if that security is enabled or not."

::= { prtOutputEntry 13 }

-- The Output Dimensions Group

prtOutputDimUnit OBJECT-TYPE

SYNTAX     PrtMediaUnitTC

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The unit of measurement for use in calculating and relaying

dimensional values for this output sub-unit."

::= { prtOutputEntry 14 }

prtOutputMaxDimFeedDir OBJECT-TYPE

-- NOTE: In RFC 1759, the range was not defined.

SYNTAX     Integer32 (-2..2147483647)

MAX-ACCESS read-write

STATUS     current

DESCRIPTION

"The maximum dimensions supported by this output sub-unit

for measurements taken parallel relative to the feed

direction associated with that sub-unit in output

sub-unit dimensional units (controlled by prtOutputDimUnit,

which uses PrtMediaUnitTC). If this output sub-unit can

reliably sense this value, the value is sensed by the printer

and may not be changed with management protocol operations.

The value (-1) means other and specifically indicates that the

sub-unit places no restrictions on this parameter.  The value

(-2) means unknown.

NOTE: The above description has been modified from RFC 1759

for clarification and to explain the purpose of (-1) and (-2)."

::= { prtOutputEntry 15 }

prtOutputMaxDimXFeedDir OBJECT-TYPE

-- NOTE: In RFC 1759, the range was not defined.

SYNTAX     Integer32 (-2..2147483647)

MAX-ACCESS read-write

STATUS     current

DESCRIPTION

"The maximum dimensions supported by this output sub-unit

for measurements taken ninety degrees relative to the

feed direction associated with that sub-unit in output

sub-unit dimensional units (controlled by prtOutputDimUnit,

which uses PrtMediaUnitTC). If this output sub-unit can

reliably sense this value, the value is sensed by the printer

and may not be changed with management protocol operations.

The value (-1) means other and specifically indicates that the

sub-unit places no restrictions on this parameter.  The value

(-2) means unknown.

NOTE: The above description has been modified from RFC 1759

for clarification and to explain the purpose of (-1) and (-2)."

::= { prtOutputEntry 16 }

prtOutputMinDimFeedDir OBJECT-TYPE

-- NOTE: In RFC 1759, the range was not defined.

SYNTAX     Integer32 (-2..2147483647)

MAX-ACCESS read-write

STATUS     current

DESCRIPTION

"The minimum dimensions supported by this output sub-unit

for measurements taken parallel relative to the feed

direction associated with that sub-unit in output

sub-unit dimensional units (controlled by prtOutputDimUnit,

which uses PrtMediaUnitTC). If this output sub-unit can

reliably sense this value, the value is sensed by the printer

and may not be changed with management protocol operations.

The value (-1) means other and specifically indicates that the

sub-unit places no restrictions on this parameter.  The value

(-2) means unknown.

NOTE: The above description has been modified from RFC 1759

for clarification and to explain the purpose of (-1) and (-2)."

::= { prtOutputEntry 17 }

prtOutputMinDimXFeedDir OBJECT-TYPE

-- NOTE: In RFC 1759, the range was not defined.

SYNTAX     Integer32 (-2..2147483647)

MAX-ACCESS read-write

STATUS     current

DESCRIPTION

"The minimum dimensions supported by this output sub-unit

for measurements taken ninety degrees relative to the

feed direction associated with that sub-unit in output

sub-unit dimensional units (controlled by prtOutputDimUnit,

which uses PrtMediaUnitTC). If this output sub-unit can

reliably sense this value, the value is sensed by the printer

and may not be changed with management protocol operations.

The value (-1) means other and specifically indicates that the

sub-unit places no restrictions on this parameter.  The value

(-2) means unknown.

NOTE: The above description has been modified from RFC 1759

for clarification and to explain the purpose of (-1) and (-2)."

::= { prtOutputEntry 18 }

-- The Output Features Group

prtOutputStackingOrder OBJECT-TYPE

--  NOTE: In RFC 1759, the enumeration values were implicitly

--  defined by this object.

SYNTAX     PrtOutputStackingOrderTC

MAX-ACCESS read-write

STATUS     current

DESCRIPTION

"The current state of the stacking order for the

associated output sub-unit. 'FirstToLast' means

that as pages are output the front of the next page is

placed against the back of the previous page.

'LasttoFirst' means that as pages are output the back

of the next page is placed against the front of the

previous page."

::= { prtOutputEntry 19 }

prtOutputPageDeliveryOrientation OBJECT-TYPE

--  NOTE: In RFC 1759, the enumeration values were implicitly

--  defined by this object.

SYNTAX     PrtOutputPageDeliveryOrientationTC

MAX-ACCESS read-write

STATUS     current

DESCRIPTION

"The reading surface that will be 'up' when pages are

delivered to the associated output sub-unit. Values are

faceUp and faceDown. (Note: interpretation of these

values is in general context-dependent based on locale;

presentation of these values to an end-user should be

normalized to the expectations of the user)."

::= { prtOutputEntry 20 }

prtOutputBursting OBJECT-TYPE

SYNTAX     PresentOnOff

MAX-ACCESS read-write

STATUS     current

DESCRIPTION

"This object indicates that the outputting sub-unit supports

bursting, and if so, whether the feature is enabled. Bursting

is the process by which continuous media is separated into

individual sheets, typically by bursting along pre-formed

perforations."

::= { prtOutputEntry 21 }

prtOutputDecollating OBJECT-TYPE

SYNTAX     PresentOnOff

MAX-ACCESS read-write

STATUS     current

DESCRIPTION

"This object indicates that the output supports decollating,

and if so, whether the feature is enabled. Decollating is the

process by which the individual parts within a multi-part form

are separated and sorted into separate stacks for each part."

::= { prtOutputEntry 22 }

prtOutputPageCollated OBJECT-TYPE

SYNTAX     PresentOnOff

MAX-ACCESS read-write

STATUS     current

DESCRIPTION

"This object indicates that the output sub-unit supports page

collation, and if so, whether the feature is enabled. See RFC

XXXX Appendix A, Glossary Of Terms, for definition of how this

RFC Editor: change XXXX to the number assigned to this document.

document defines collation.

NOTE: The above description has been modified from RFC 1759

for clarification."

::= { prtOutputEntry 23 }

prtOutputOffsetStacking OBJECT-TYPE

SYNTAX     PresentOnOff

MAX-ACCESS read-write

STATUS     current

DESCRIPTION

"This object indicates that the output supports offset

stacking,and if so, whether the feature is enabled. See RFC

XXXX Appendix A, Glossary Of Terms,  for how Offset Stacking is

RFC Editor: change XXXX to the number assigned to this document.

defined by this document.

NOTE: The above description has been modified from RFC 1759

for clarification."

::= { prtOutputEntry 24 }

-- The Marker Group

--

-- A marker is the mechanism that produces marks on the print

-- media. The marker sub-units and their associated supplies are

-- represented by the Marker Group in the model.  A printer can

-- contain one or more marking mechanisms.  Some examples of

-- multiple marker sub-units are: a printer

-- with separate markers for normal and magnetic ink or an

-- imagesetter that can output to both a proofing device and

-- final film. Each marking device can have its own set of

-- characteristics associated with it, such as marking technology

-- and resolution.

prtMarker OBJECT IDENTIFIER ::= { printmib 10 }

-- The printable area margins as listed below define an area of

-- the print media which is guaranteed to be printable for all

-- combinations of input, media paths, and interpreters for this

-- marker.

prtMarkerTable OBJECT-TYPE

SYNTAX     SEQUENCE OF PrtMarkerEntry

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"The marker table provides a description of each marker

sub-unit contained within the printer.

NOTE: The above description has been modified from RFC 1759

for clarification."

::= { prtMarker 2 }

prtMarkerEntry OBJECT-TYPE

SYNTAX     PrtMarkerEntry

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"Entries in this table define the characteristics and status

of each marker sub-unit in the printer.

NOTE: The above description has been modified from RFC 1759

for clarification."

INDEX  { hrDeviceIndex, prtMarkerIndex }

::= { prtMarkerTable 1 }

PrtMarkerEntry ::= SEQUENCE {

prtMarkerIndex                  Integer32,

prtMarkerMarkTech               PrtMarkerMarkTechTC,

prtMarkerCounterUnit            PrtMarkerCounterUnitTC,

prtMarkerLifeCount              Counter32,

prtMarkerPowerOnCount           Counter32,

prtMarkerProcessColorants       Integer32,

prtMarkerSpotColorants          Integer32,

prtMarkerAddressabilityUnit     PrtMarkerAddressabilityUnitTC,

prtMarkerAddressabilityFeedDir  Integer32,

prtMarkerAddressabilityXFeedDir Integer32,

prtMarkerNorthMargin            Integer32,

prtMarkerSouthMargin            Integer32,

prtMarkerWestMargin             Integer32,

prtMarkerEastMargin             Integer32,

prtMarkerStatus                 PrtSubUnitStatusTC

}

prtMarkerIndex OBJECT-TYPE

SYNTAX     Integer32 (1..65535)

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"A unique value used by the printer to identify this marking

SubUnit.  Although these values may change due to a major

reconfiguration of the device (e.g. the addition of new marking

sub-units to the printer), values SHOULD remain stable across

successive printer power cycles.

NOTE: The above description has been modified from RFC 1759

for clarification."

::= { prtMarkerEntry 1 }

prtMarkerMarkTech OBJECT-TYPE

--  NOTE: In RFC 1759, the enumeration values were implicitly

--  defined by this object.

SYNTAX     PrtMarkerMarkTechTC

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The type of marking technology used for this marking

sub-unit."

::= { prtMarkerEntry 2 }

prtMarkerCounterUnit OBJECT-TYPE

--  NOTE: In RFC 1759, the enumeration values were implicitly

--  defined by this object.

SYNTAX     PrtMarkerCounterUnitTC

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The unit that will be used by the printer when reporting

counter values for this marking sub-unit.  The time units of

measure are provided for a device like a strip recorder that

does not or cannot track the physical dimensions of the media

and does not use characters, lines or sheets."

::= { prtMarkerEntry 3 }

prtMarkerLifeCount OBJECT-TYPE

SYNTAX     Counter32

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The count of the number of units of measure counted during the

life of printer using units of measure as specified by

prtMarkerCounterUnit.

Note: This object should be implemented as a persistent object

with a reliable value throughout the lifetime of the printer."

::= { prtMarkerEntry 4 }

prtMarkerPowerOnCount OBJECT-TYPE

SYNTAX     Counter32

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The count of the number of units of measure counted since the

equipment was most recently powered on using units of measure

as specified by prtMarkerCounterUnit."

::= { prtMarkerEntry 5 }

prtMarkerProcessColorants OBJECT-TYPE

SYNTAX     Integer32 (0..65535)

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The number of process colors supported by this marker.  A

process color of 1 implies monochrome.  The value of this

object and prtMarkerSpotColorants cannot both be 0.  The value

of prtMarkerProcessColorants must be 0 or greater.

NOTE: The above description has been modified from RFC 1759

for clarification."

::= { prtMarkerEntry 6 }

prtMarkerSpotColorants OBJECT-TYPE

SYNTAX     Integer32 (0..65535)

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The number of spot colors supported by this marker. The value

of this object and prtMarkerProcessColorants cannot both be 0.

Must be 0 or greater.

NOTE: The above description has been modified from RFC 1759

for clarification."

::= { prtMarkerEntry 7 }

prtMarkerAddressabilityUnit OBJECT-TYPE

--  NOTE: In RFC 1759, the enumeration values were implicitly

--  defined by this object.

SYNTAX     PrtMarkerAddressabilityUnitTC

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The unit of measure of distances, as applied to the marker's

resolution.

NOTE: The above description has been modified from RFC 1759

for clarification."

::= { prtMarkerEntry 8 }

prtMarkerAddressabilityFeedDir OBJECT-TYPE

-- NOTE: In RFC 1759, the range was not defined.

SYNTAX     Integer32 (-2..2147483647)

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The maximum number of addressable marking positions in the

feed direction per 10000 units of measure specified by

prtMarkerAddressabilityUnit.  A value of (-1) implies 'other'

or 'infinite' while a value of (-2) implies 'unknown'.

NOTE: The above description has been modified from RFC 1759

for clarification."

::= { prtMarkerEntry 9 }

prtMarkerAddressabilityXFeedDir OBJECT-TYPE

-- NOTE: In RFC 1759, the range was not defined.

SYNTAX     Integer32 (-2..2147483647)

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The maximum number of addressable marking positions in the

cross feed direction in 10000 units of measure specified by

prtMarkerAddressabilityUnit.  A value of (-1) implies 'other'

or 'infinite' while a value of (-2) implies 'unknown'.

NOTE: The above description has been modified from RFC 1759

for clarification."

::= { prtMarkerEntry 10 }

prtMarkerNorthMargin OBJECT-TYPE

-- NOTE: In RFC 1759, the range was not defined.

SYNTAX     Integer32 (-2..2147483647)

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The margin, in units identified by prtMarkerAddressabilityUnit,

from the leading edge of the medium as the medium flows through

the marking engine with the side to be imaged facing the

observer. The leading edge is the North edge and the other

edges are defined by the normal compass layout of  directions

with the compass facing the observer.  Printing within the area

bounded by all four margins is guaranteed for all interpreters.

The value (-2) means unknown."

::= { prtMarkerEntry 11 }

prtMarkerSouthMargin OBJECT-TYPE

-- NOTE: In RFC 1759, the range was not defined.

SYNTAX     Integer32 (-2..2147483647)

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The margin from the South edge  (see prtMarkerNorthMargin) of

the medium in units identified by prtMarkerAddressabilityUnit.

Printing within the area bounded by all four margins  is

guaranteed for all interpreters. The value (-2) means unknown."

::= { prtMarkerEntry 12 }

prtMarkerWestMargin OBJECT-TYPE

-- NOTE: In RFC 1759, the range was not defined.

SYNTAX     Integer32 (-2..2147483647)

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The margin from the West edge (see prtMarkerNorthMargin) of

the medium in units identified by prtMarkerAddressabilityUnit.

Printing within the area bounded by all four margins is

guaranteed for all interpreters. The value (-2) means unknown."

::= { prtMarkerEntry 13 }

prtMarkerEastMargin OBJECT-TYPE

-- NOTE: In RFC 1759, the range was not defined.

SYNTAX     Integer32 (-2..2147483647)

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The margin from the East edge (see prtMarkerNorthMargin) of

the medium in units identified by prtMarkerAddressabilityUnit.

Printing within the area bounded by all four margins is

guaranteed for all interpreters. The value (-2) means unknown."

::= { prtMarkerEntry 14 }

prtMarkerStatus OBJECT-TYPE

SYNTAX     PrtSubUnitStatusTC

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The current status of this marker sub-unit."

::= { prtMarkerEntry 15 }

-- The Marker Supplies Group

prtMarkerSupplies OBJECT IDENTIFIER ::= { printmib 11 }

prtMarkerSuppliesTable OBJECT-TYPE

SYNTAX     SEQUENCE OF PrtMarkerSuppliesEntry

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"A table of the marker supplies available on this printer."

::= { prtMarkerSupplies 1 }

prtMarkerSuppliesEntry OBJECT-TYPE

SYNTAX     PrtMarkerSuppliesEntry

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"Attributes of a marker supply. Entries may exist in the table

for each device index with a device type of 'printer'.

NOTE: The above description has been modified from RFC 1759

for clarification."

INDEX  { hrDeviceIndex, prtMarkerSuppliesIndex }

::= { prtMarkerSuppliesTable 1 }

PrtMarkerSuppliesEntry ::= SEQUENCE {

prtMarkerSuppliesIndex          Integer32,

prtMarkerSuppliesMarkerIndex    Integer32,

prtMarkerSuppliesColorantIndex  Integer32,

prtMarkerSuppliesClass          PrtMarkerSuppliesClassTC,

prtMarkerSuppliesType           PrtMarkerSuppliesTypeTC,

prtMarkerSuppliesDescription    PrtLocalizedDescriptionStringTC,

prtMarkerSuppliesSupplyUnit     PrtMarkerSuppliesSupplyUnitTC,

prtMarkerSuppliesMaxCapacity    Integer32,

prtMarkerSuppliesLevel          Integer32

}

prtMarkerSuppliesIndex OBJECT-TYPE

SYNTAX     Integer32 (1..65535)

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"A unique value used by the printer to identify this marker

supply.  Although these values may change due to a major

reconfiguration of the device (e.g. the addition of new marker

supplies to the printer), values SHOULD remain stable across

successive printer power cycles.

NOTE: The above description has been modified from RFC 1759

for clarification."

::= { prtMarkerSuppliesEntry 1 }

prtMarkerSuppliesMarkerIndex OBJECT-TYPE

SYNTAX     Integer32 (0..65535)

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The value of prtMarkerIndex corresponding to the marking sub

unit with which this marker supply sub-unit is associated."

::= { prtMarkerSuppliesEntry 2 }

prtMarkerSuppliesColorantIndex OBJECT-TYPE

SYNTAX     Integer32 (0..65535)

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The value of prtMarkerColorantIndex corresponding to the

colorant with which this marker supply sub-unit is associated.

This value shall be 0 if there is no colorant table or if this

supply does not depend on a single specified colorant.

NOTE: The above description has been modified from RFC 1759

for clarification."

::= { prtMarkerSuppliesEntry 3 }

prtMarkerSuppliesClass OBJECT-TYPE

--  NOTE: In RFC 1759, the enumeration values were implicitly

--  defined by this object.

SYNTAX     PrtMarkerSuppliesClassTC

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"Indicates whether this supply entity represents a supply that

is consumed or a receptacle that is filled.

NOTE: The above description has been modified from RFC 1759

for clarification."

::= { prtMarkerSuppliesEntry 4 }

prtMarkerSuppliesType OBJECT-TYPE

--  NOTE: In RFC 1759, the enumeration values were implicitly

--  defined by this object.

SYNTAX     PrtMarkerSuppliesTypeTC

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The type of this supply."

::= { prtMarkerSuppliesEntry 5 }

prtMarkerSuppliesDescription OBJECT-TYPE

-- In RFC 1759, the SYNTAX was OCTET STRING.  This has been changed

-- to a TC to better support localization of the object.

SYNTAX     PrtLocalizedDescriptionStringTC

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The description of this supply container/receptacle in the

localization specified by prtGeneralCurrentLocalization."

::= { prtMarkerSuppliesEntry 6 }

prtMarkerSuppliesSupplyUnit OBJECT-TYPE

--  NOTE: In RFC 1759, the enumeration values were implicitly

--  defined by this object.

SYNTAX     PrtMarkerSuppliesSupplyUnitTC

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"Unit of measure of this marker supply container/receptacle.

NOTE: The above description has been modified from RFC 1759

for clarification."

::= { prtMarkerSuppliesEntry 7 }

prtMarkerSuppliesMaxCapacity OBJECT-TYPE

-- NOTE: In RFC 1759, the range was not defined.

SYNTAX     Integer32 (-2..2147483647)

MAX-ACCESS read-write

STATUS     current

DESCRIPTION

"The maximum capacity of this supply container/receptacle

expressed in prtMarkerSuppliesSupplyUnit. If this supply

container/receptacle can reliably sense this value, the value

is reported by the printer and is read-only; otherwise, the

value may be written (by a Remote Control Panel or a Management

Application). The value (-1) means other and specifically

indicates that the sub-unit places no restrictions on this

parameter. The value (-2) means unknown."

::= { prtMarkerSuppliesEntry 8 }

prtMarkerSuppliesLevel OBJECT-TYPE

-- NOTE: In RFC 1759, the range was not defined.

SYNTAX     Integer32 (-3..2147483647)

MAX-ACCESS read-write

STATUS     current

DESCRIPTION

"The current level if this supply is a container; the remaining

space if this supply is a receptacle. If this supply

container/receptacle can reliably sense this value, the value

is reported by the printer and is read-only; otherwise, the

value may be written (by a Remote Control Panel or a Management

Application). The value (-1) means other and specifically

indicates that the sub-unit places no restrictions on this

parameter. The value (-2) means unknown.  A value of (-3) means

that the printer knows that there is some supply/remaining

space, respectively."

::= { prtMarkerSuppliesEntry 9 }

-- The Marker Colorant Group

prtMarkerColorant OBJECT IDENTIFIER ::= { printmib 12 }

prtMarkerColorantTable OBJECT-TYPE

SYNTAX     SEQUENCE OF PrtMarkerColorantEntry

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"A table of all of the colorants available on the printer."

::= { prtMarkerColorant 1 }

prtMarkerColorantEntry OBJECT-TYPE

SYNTAX     PrtMarkerColorantEntry

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"Attributes of a colorant available on the printer. Entries may

exist in the table for each device index with a device type of

'printer'.

NOTE: The above description has been modified from RFC 1759

for clarification."

INDEX { hrDeviceIndex, prtMarkerColorantIndex }

::= { prtMarkerColorantTable 1 }

PrtMarkerColorantEntry ::= SEQUENCE {

prtMarkerColorantIndex          Integer32,

prtMarkerColorantMarkerIndex    Integer32,

prtMarkerColorantRole           PrtMarkerColorantRoleTC,

prtMarkerColorantValue          OCTET STRING,

prtMarkerColorantTonality       Integer32

}

prtMarkerColorantIndex OBJECT-TYPE

SYNTAX     Integer32 (1..65535)

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"A unique value used by the printer to identify this colorant.

Although these values may change due to a major reconfiguration

of the device (e.g. the addition of new colorants to the

printer) , values SHOULD remain stable across successive

printer power cycles.

NOTE: The above description has been modified from RFC 1759

for clarification."

::= { prtMarkerColorantEntry 1 }

prtMarkerColorantMarkerIndex OBJECT-TYPE

SYNTAX     Integer32 (0..65535)

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The value of prtMarkerIndex corresponding to the marker sub

unit with which this colorant entry is associated."

::= { prtMarkerColorantEntry 2 }

prtMarkerColorantRole OBJECT-TYPE

--  NOTE: In RFC 1759, the enumeration values were implicitly

--  defined by this object.

SYNTAX     PrtMarkerColorantRoleTC

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The role played by this colorant."

::= { prtMarkerColorantEntry 3 }

prtMarkerColorantValue OBJECT-TYPE

-- NOTE: The string length range has been increased from RFC 1759.

SYNTAX     OCTET STRING (SIZE(0..255))

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The name of the color of this colorant using standardized

string names from ISO 10175 (DPA) and ISO 10180 (SPDL) such as:

other

unknown

white

red

green

blue

cyan

magenta

yellow

black

Implementers may add additional string values. The naming

conventions in ISO 9070 are recommended in order to avoid

potential name clashes"

::= { prtMarkerColorantEntry 4 }

prtMarkerColorantTonality OBJECT-TYPE

-- NOTE: In RFC 1759, the range was not defined.

SYNTAX     Integer32 (2..2147483647)

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The distinct levels of tonality realizable by a marking sub

unit when using this colorant.  This value does not include the

number of levels of tonal difference that an interpreter can

obtain by techniques such as half toning. This value must be at

least 2."

::= { prtMarkerColorantEntry 5 }

-- The Media Path Group

--

-- The media paths encompass the mechanisms in the printer that

-- move the media through the printer and connect all other media

-- related sub-units: inputs, outputs, markers and finishers. A

-- printer contains one or more media paths. These are

-- represented by the Media Path Group in the model.

prtMediaPath OBJECT IDENTIFIER ::= { printmib 13 }

prtMediaPathTable OBJECT-TYPE

SYNTAX     SEQUENCE OF PrtMediaPathEntry

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"The media path table includes both physical and logical paths

within the printer.

NOTE: The above description has been modified from RFC 1759

for clarification."

::= { prtMediaPath 4 }

prtMediaPathEntry OBJECT-TYPE

SYNTAX     PrtMediaPathEntry

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"Entries may exist in the table for each device index with a

device type of 'printer'  Each entry defines the physical

characteristics of and the status of the media path.  The data

provided indicates the maximum throughput and the media

size limitations of these subunits.

NOTE: The above description has been modified from RFC 1759

for clarification."

INDEX  { hrDeviceIndex, prtMediaPathIndex }

::= { prtMediaPathTable 1 }

PrtMediaPathEntry ::= SEQUENCE {

prtMediaPathIndex               Integer32,

prtMediaPathMaxSpeedPrintUnit   PrtMediaPathMaxSpeedPrintUnitTC,

prtMediaPathMediaSizeUnit       PrtMediaUnitTC,

prtMediaPathMaxSpeed            Integer32,

prtMediaPathMaxMediaFeedDir     Integer32,

prtMediaPathMaxMediaXFeedDir    Integer32,

prtMediaPathMinMediaFeedDir     Integer32,

prtMediaPathMinMediaXFeedDir    Integer32,

prtMediaPathType                PrtMediaPathTypeTC,

prtMediaPathDescription         PrtLocalizedDescriptionStringTC,

prtMediaPathStatus              PrtSubUnitStatusTC

}

prtMediaPathIndex OBJECT-TYPE

SYNTAX     Integer32 (1..65535)

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"A unique value used by the printer to identify this media

path.Although these values may change due to a major

reconfiguration of the device (e.g. the addition of new media

paths to the printer), values SHOULD remain stable across

successive printer power cycles.

NOTE: The above description has been modified from RFC 1759

for clarification."

::= { prtMediaPathEntry 1 }

prtMediaPathMaxSpeedPrintUnit OBJECT-TYPE

--  NOTE: In RFC 1759, the enumeration values were implicitly

--  defined by this object.

SYNTAX PrtMediaPathMaxSpeedPrintUnitTC

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The unit of measure used in specifying the speed of all media

paths in the printer."

::= { prtMediaPathEntry 2 }

prtMediaPathMediaSizeUnit OBJECT-TYPE

SYNTAX     PrtMediaUnitTC

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The units of measure of media size for use in calculating and

relaying dimensional values for all media paths in the

printer."

::= { prtMediaPathEntry 3 }

prtMediaPathMaxSpeed OBJECT-TYPE

-- NOTE: In RFC 1759, the range was not defined.

SYNTAX     Integer32 (-2..2147483647)

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The maximum printing speed of this media path expressed in

prtMediaPathMaxSpeedUnit's.  A value of (-1) implies 'other'."

::= { prtMediaPathEntry 4 }

prtMediaPathMaxMediaFeedDir OBJECT-TYPE

-- NOTE: In RFC 1759, the range was not defined.

SYNTAX     Integer32 (-2..2147483647)

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The maximum physical media size in the feed direction of this

media path expressed in units of measure specified by

PrtMediaPathMediaSizeUnit.  A value of (-1) implies 'unlimited'

a value of (-2) implies 'unknown'.

NOTE: The above description has been modified from RFC 1759

for clarification."

::= { prtMediaPathEntry 5 }

prtMediaPathMaxMediaXFeedDir OBJECT-TYPE

-- NOTE: In RFC 1759, the range was not defined.

SYNTAX     Integer32 (-2..2147483647)

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The maximum physical media size across the feed direction of

this media path expressed in units of measure specified by

prtMediaPathMediaSizeUnit.  A value of (-2) implies 'unknown'.

NOTE: The above description has been modified from RFC 1759

for clarification."

::= { prtMediaPathEntry 6 }

prtMediaPathMinMediaFeedDir OBJECT-TYPE

-- NOTE: In RFC 1759, the range was not defined.

SYNTAX     Integer32 (-2..2147483647)

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The minimum physical media size in the feed direction of this

media path expressed in units of measure specified by

prtMediaPathMediaSizeUnit. A value of (-2) implies 'unknown'.

NOTE: The above description has been modified from RFC 1759

for clarification."

::= { prtMediaPathEntry 7 }

prtMediaPathMinMediaXFeedDir OBJECT-TYPE

-- NOTE: In RFC 1759, the range was not defined.

SYNTAX     Integer32 (-2..2147483647)

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The minimum physical media size across the feed direction of

this media path expressed in units of measure specified by

prtMediaPathMediaSizeUnit.  A value of (-2) implies 'unknown'.

NOTE: The above description has been modified from RFC 1759

for clarification."

::= { prtMediaPathEntry 8 }

prtMediaPathType OBJECT-TYPE

--  NOTE: In RFC 1759, the enumeration values were implicitly

--  defined by this object.

SYNTAX     PrtMediaPathTypeTC

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The type of the media path for this media path."

::= { prtMediaPathEntry 9 }

prtMediaPathDescription OBJECT-TYPE

-- In RFC 1759, the SYNTAX was OCTET STRING.  This has been changed

-- to a TC to better support localization of the object.

SYNTAX     PrtLocalizedDescriptionStringTC

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The manufacturer-provided description of this media path in

the localization specified by prtGeneralCurrentLocalization."

::= { prtMediaPathEntry 10 }

prtMediaPathStatus OBJECT-TYPE

SYNTAX     PrtSubUnitStatusTC

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The current status of this media path."

::= { prtMediaPathEntry 11 }

-- The Print Job Delivery Channel Group

--

-- Print Job Delivery Channels are independent sources of print

-- data. Here, print data is the term used for the information

-- that is used to construct printed pages and may have both data

-- and control aspects. The output of a channel is in a form

-- suitable for input to one of the interpreters as a

-- stream. A channel may be independently enabled (allowing

-- print data to flow) or disabled (stopping the flow of

-- print data). A printer may have one or more channels.

--

-- The Print Job Delivery Channel table describes the

-- capabilities of the printer and not what is currently being

-- performed by the printer

--

-- Basically, the print job delivery channel abstraction

-- describes the final processing step of getting the print data

-- to an interpreter. It might include some level of

-- decompression or decoding of print stream data.

-- channel. All of these aspects are hidden in the channel

-- abstraction.

--

-- There are many kinds of print job delivery channels; some of

-- which are based on networks and others which are not. For

-- example, a channel can be a serial (or parallel) connection;

-- it can be a service, such as the UNIX Line Printer Daemon

-- (LPD), offering services over a network connection; or

-- it could be a disk drive into which a floppy disk with

-- the print data is inserted. Each print job delivery channel is

-- identified by the electronic path and/or service protocol

-- used to deliver print data to a print data interpreter.

--

-- Channel example                   Implementation

--

-- serial port channel            bi-directional data channel

-- parallel port channel          often uni-directional channel

-- IEEE 1284 port channel         bi-directional channel

-- SCSI port channel              bi-directional

-- Apple PAP channel              may be based on LocalTalk,

--                                Ethernet or Tokentalk

-- LPD Server channel             TCP/IP based, port 515

-- Netware Remote Printer         SPX/IPX based channel

-- Netware Print Server           SPX/IPX based channel

--

-- It is easy to note that this is a mixed bag.  There are

-- some physical connections over which no (or very meager)

-- protocols are run (e.g. the serial or old parallel ports)

-- and there are services which often have elaborate

-- protocols that run over a number of protocol stacks. In

-- the end, what is important is the delivery of print data

-- through the channel.

--

-- The print job delivery channel sub-units are represented by

-- the Print Job Delivery Channel Group in the Model. It has a

-- current print job control language, which can be used to

-- specify which interpreter is to be used for the print data and

-- to query and change environment variables used by the

-- interpreters (and Management Applications). There is also a

-- default interpreter that is to be used if an interpreter is

-- not explicitly specified using the Control Language.

-- The first seven items in the Print Job Delivery Channel Table

-- define the "channel" itself.  A channel typically depends on

-- other protocols and interfaces to provide the data that flows

-- through the channel.

--

-- Control of a print job delivery channel is largely limited to

-- enabling or disabling the entire channel itself. It is likely

-- that more control of the process of accessing print data

-- will be needed over time. Thus, the ChannelType will

-- allow type-specific data to be associated with each

-- channel (using ChannelType specific groups in a fashion

-- analogous to the media specific MIBs that are associated

-- with the IANAIfType in the Interfaces Table). As a first

-- step in this direction, each channel will identify the

-- underlying Interface on which it is based. This is the

-- eighth object in each row of the table.

-- The Print Job Delivery Channel Table

prtChannel OBJECT IDENTIFIER ::= { printmib 14 }

prtChannelTable OBJECT-TYPE

SYNTAX     SEQUENCE OF PrtChannelEntry

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"The channel table represents the set of input data sources

which can provide print data to one or more of the

interpreters available on a printer.

NOTE: The above description has been modified from RFC 1759

for clarification."

::= { prtChannel 1 }

prtChannelEntry OBJECT-TYPE

SYNTAX     PrtChannelEntry

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"Entries may exist in the table for each device index with a

device type of 'printer'.  Each channel table entry is

characterized by a unique protocol stack and/or addressing.

The channel may also have printer dependent features that are

associated with a printing language.

NOTE: The above description has been modified from RFC 1759

for clarification."

INDEX  { hrDeviceIndex, prtChannelIndex }

::= { prtChannelTable 1 }

PrtChannelEntry ::= SEQUENCE {

prtChannelIndex                     Integer32,

prtChannelType                      PrtChannelTypeTC,

prtChannelProtocolVersion           OCTET STRING,

prtChannelCurrentJobCntlLangIndex   Integer32,

prtChannelDefaultPageDescLangIndex  Integer32,

prtChannelState                     PrtChannelStateTC,

prtChannelIfIndex                   InterfaceIndexOrZero,

prtChannelStatus                    PrtSubUnitStatusTC,

prtChannelInformation               OCTET STRING

}

prtChannelIndex OBJECT-TYPE

-- NOTE: In RFC 1759, the range was not defined.

SYNTAX     Integer32 (1..65535)

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"A unique value used by the printer to identify this data

channel.  Although these values may change due to a major

reconfiguration of the device (e.g. the addition of new data

channels to the printer), values SHOULD remain stable across

successive printer power cycles.

NOTE: The above description has been modified from RFC 1759

for clarification."

::= { prtChannelEntry 1 }

prtChannelType OBJECT-TYPE

--  NOTE: In RFC 1759, the enumeration values were implicitly

--  defined by this object.

SYNTAX     PrtChannelTypeTC

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The type of this print data channel.  This object provides the

linkage to ChannelType-specific groups that may (conceptually)

extend the prtChannelTable with additional details about that

channel."

::= { prtChannelEntry 2 }

prtChannelProtocolVersion OBJECT-TYPE

SYNTAX     OCTET STRING (SIZE(0..63))

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The version of the protocol used on this channel.  The format

used for version numbering depends on prtChannelType."

::= { prtChannelEntry 3 }

prtChannelCurrentJobCntlLangIndex OBJECT-TYPE

-- NOTE: In RFC 1759, the range was not defined.

SYNTAX     Integer32 (0..65535)

MAX-ACCESS read-write

STATUS     current

DESCRIPTION

"The value of prtInterpreterIndex corresponding to the Control

Language Interpreter for this channel. This interpreter defines

the syntax used for control functions, such as querying or

changing environment variables and identifying job boundaries

(e.g. PJL, PostScript, NPAP). A value of zero indicates that

there is no current Job Control Language Interpreter for this

channel.

NOTE: The above description has been modified from RFC 1759

for clarification."

::= { prtChannelEntry 4 }

prtChannelDefaultPageDescLangIndex OBJECT-TYPE

-- NOTE: In RFC 1759, the range was not defined.

SYNTAX     Integer32 (0..65535)

MAX-ACCESS read-write

STATUS     current

DESCRIPTION

"The value of prtInterpreterIndex corresponding to the Page

Description Language Interpreter for this channel. This

interpreter defines the default Page Description Language

interpreter to be used for the print data unless the Control

Language is used to select a specific interpreter (e.g., PCL,

PostScript Language, auto-sense). A value of zero indicates

that there is no default page description language interpreter

for this channel.

NOTE: The above description has been modified from RFC 1759

for clarification."

::= { prtChannelEntry 5 }

prtChannelState OBJECT-TYPE

--  NOTE: In RFC 1759, the enumeration values were implicitly

--  defined by this object.

SYNTAX     PrtChannelStateTC

MAX-ACCESS read-write

STATUS     current

DESCRIPTION

"The state of this print data channel. The value determines

whether control information and print data is allowed through

this channel or not."

::= { prtChannelEntry 6 }

prtChannelIfIndex OBJECT-TYPE

SYNTAX     InterfaceIndexOrZero   -- Was Integer32 in RFC 1759.

MAX-ACCESS read-write

STATUS     current

DESCRIPTION

"The value of ifIndex in the ifTable; see the Interfaces Group

MIB [RFC2863] which corresponds to this channel.

When more than one row of the ifTable is relevant, this is the

index of the row representing the topmost layer in the

interface hierarchy.  A value of zero indicates that no

interface is associated with this channel.

NOTE: The above description has been modified from RFC 1759

for clarification."

::= { prtChannelEntry 7 }

prtChannelStatus OBJECT-TYPE

SYNTAX     PrtSubUnitStatusTC

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The current status of the channel."

::= { prtChannelEntry 8 }

prtChannelInformation OBJECT-TYPE

SYNTAX     OCTET STRING (SIZE (0..255))

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"Auxiliary information to allow a printing application to use

the channel for data submission to the printer.  An application

capable of using a specific PrtChannelType should be able to

use the combined information from the prtChannelInformation and

other channel and interface group objects to 'bootstrap' its

use of the channel.  prtChannelInformation is not intended to

provide a general channel description, nor to provide

information that is available once the channel is in use.

The encoding and interpretation of the prtChannelInformation

object is specific to channel type.  The description of each

PrtChannelType enum value for which prtChannelInformation is

defined specifies the appropriate encoding and interpretation,

including interaction with other objects.  For channel types

that do not specify a prtChannelInformation value, its value

shall be null (0 length).

When a new PrtChannelType enumeration value is registered, its

accompanying description must specify the encoding and

interpretation of the prtChannelInformation value for the

channel type.  prtChannelInformation semantics for an existing

PrtChannelType may be added or amended in the same manner as

described in section 2.4.1 for type 2 enumeration values.

The prtChannelInformation specifies values for a collection of

channel attributes, represented as text according to the

following rules:

1. The prtChannelInformation is not affected by localization.

2. The prtChannelInformation is a list of entries representing

the attribute values.  Each entry consists of the following

items, in order:

a. A keyword, composed of alphabetic characters (A-Z, a-z)

represented by their NVT ASCII [RFC854] codes, that

identifies a channel attribute,

b. The NVT ASCII code for an Equals Sign (=) (code 61) to

delimit the keyword,

c. A data value encoded using rules specific to the

PrtChannelType to with the prtChannelInformation applies which

must in no case allow an octet with value 10 (the NVT ASCII

Line Feed code),

d. the NVT ASCII code for a Line Feed character (code 10) to

delimit the data value.

No other octets shall be present.

Keywords are case-sensitive.  Conventionally, keywords are

capitalized (including each word of a multi-word keyword) and

since they occupy space in the prtChannelInformation, they are

kept short.

3. If a channel attribute has multiple values, it is

represented by multiple entries with the same keyword, each

specifying one value. Otherwise, there shall be at most one

entry for each attribute.

4. By default, entries may appear in any order.  If there are

ordering constraints for particular entries, these must be

specified in their definitions.

5. The prtChannelInformation value by default consists of text

represented by NVT ASCII graphics character codes. However,

other representations may be specified:

a. In cases where the prtChannelInformation value contains

information not normally coded in textual form, whatever

symbolic representation is conventionally used for the

information should be used for encoding the

prtChannelInformation value. (For instance, a binary port value

might be represented as a decimal number using NVT ASCII

codes.) Such encoding must be specified in the definition of

the value.

b. The value may contain textual information in a character set

other than NVT ASCII graphics characters.  (For instance, an

identifier might consist of ISO 10646 text encoded using the

UTF-8 encoding scheme.) Such a character set and its encoding

must be specified in the definition of the value.

6. For each PrtChannelType for which prtChannelInformation

entries are defined, the descriptive text associated with the

PrtChannelType enumeration value shall specify the following

information for each entry:

Title:        Brief description phrase, e.g.: 'Port name',

'Service Name', etc.

Keyword:      The keyword value, e.g.: 'Port' or 'Service'

Syntax:       The encoding of the entry value if it cannot be

directly represented by NVT ASCII.

Status:       'Mandatory', 'Optional', or 'Conditionally

Mandatory'

Multiplicity: 'Single' or 'Multiple' to indicate whether the

entry may be present multiple times.

Description:  Description of the use of the entry, other

information required to complete the definition

(e.g.: ordering constraints, interactions between

entries).

Applications that interpret prtChannelInformation should ignore

unrecognized entries, so they are not affected if new entry

types are added."

::= { prtChannelEntry 9 }

-- The Interpreter Group

--

-- The interpreter sub-units are responsible for the conversion

-- of a description of intended print instances into images that

-- are to be marked on the media. A printer may have one or more

-- interpreters. The interpreter sub-units are represented by the

-- Interpreter Group in the Model. Each interpreter is generally

-- implemented with software running on the System Controller

-- sub-unit. The Interpreter Table has one entry per interpreter

-- where the interpreters include both Page Description Language

-- (PDL) Interpreters and Control Language Interpreters.

prtInterpreter OBJECT IDENTIFIER ::= { printmib 15 }

--       Interpreter Table

prtInterpreterTable OBJECT-TYPE

SYNTAX     SEQUENCE OF PrtInterpreterEntry

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"The interpreter table is a table representing the

interpreters in the printer.  An entry shall be placed in the

interpreter table for each interpreter on the printer.

NOTE: The above description has been modified from RFC 1759

for clarification."

::= { prtInterpreter 1 }

prtInterpreterEntry OBJECT-TYPE

SYNTAX     PrtInterpreterEntry

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"Entries may exist in the table for each device index with a

device type of 'printer'.  Each table entry provides a complete

description of the interpreter, including version information,

rendering resolutions, default character sets, output

orientation, and communication capabilities.

NOTE: The above description has been modified from RFC 1759

for clarification."

INDEX  { hrDeviceIndex, prtInterpreterIndex }

::= { prtInterpreterTable 1 }

PrtInterpreterEntry ::= SEQUENCE {

prtInterpreterIndex                 Integer32,

prtInterpreterLangFamily            PrtInterpreterLangFamilyTC,

prtInterpreterLangLevel             OCTET STRING,

prtInterpreterLangVersion           OCTET STRING,

prtInterpreterDescription           PrtLocalizedDescriptionStringTC,

prtInterpreterVersion               OCTET STRING,

prtInterpreterDefaultOrientation    PrtPrintOrientationTC,

prtInterpreterFeedAddressability    Integer32,

prtInterpreterXFeedAddressability   Integer32,

prtInterpreterDefaultCharSetIn      IANACharset,

prtInterpreterDefaultCharSetOut     IANACharset,

prtInterpreterTwoWay                PrtInterpreterTwoWayTC

}

prtInterpreterIndex OBJECT-TYPE

-- NOTE: In RFC 1759, the range was not defined.

SYNTAX     Integer32 (1..65535)

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"A unique value for each PDL or control language for which

there exists an interpreter or emulator in the printer. The

value is used to identify this interpreter. Although these

values may change due to a major reconfiguration of the device

(e.g. the addition of new interpreters to the printer), values

SHOULD remain stable across successive printer power cycles.

NOTE: The above description has been modified from RFC 1759

for clarification."

::= { prtInterpreterEntry 1 }

prtInterpreterLangFamily OBJECT-TYPE

--  NOTE: In RFC 1759, the enumeration values were implicitly

--  defined by this object.

SYNTAX     PrtInterpreterLangFamilyTC

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The family name of a Page Description Language (PDL) or

control language which this interpreter in the printer can

interpret or emulate.

NOTE: The above description has been modified from RFC 1759

for clarification."

::= { prtInterpreterEntry 2 }

prtInterpreterLangLevel OBJECT-TYPE

SYNTAX     OCTET STRING (SIZE(0..31))

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The level of the language which this interpreter is

interpreting or emulating.  This might contain a value like

'5e'for an interpreter which is emulating level 5e of the PCL

language. It might contain '2' for an interpreter which is

emulating level 2 of the PostScript language. Similarly it

might contain '2' for an interpreter which is emulating level 2

of the HPGL language."

::= { prtInterpreterEntry 3 }

prtInterpreterLangVersion OBJECT-TYPE

SYNTAX     OCTET STRING (SIZE(0..31))

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The date code or version of the language which this

interpreter is interpreting or emulating."

::= { prtInterpreterEntry 4 }

prtInterpreterDescription OBJECT-TYPE

-- In RFC 1759, the SYNTAX was OCTET STRING.  This has been changed

-- to a TC to better support localization of the object.

SYNTAX     PrtLocalizedDescriptionStringTC

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"A string to identify this interpreter in the localization

specified by prtGeneralCurrentLocalization as opposed to the

language which is being interpreted.  It is anticipated that

this string will allow manufacturers to unambiguously identify

their interpreters."

::= { prtInterpreterEntry 5 }

prtInterpreterVersion OBJECT-TYPE

SYNTAX     OCTET STRING (SIZE(0..31))

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The date code, version number, or other product specific

information tied to this interpreter.  This value is associated

with the interpreter, rather than with the version of the

language which is being interpreted or emulated."

::= { prtInterpreterEntry 6 }

prtInterpreterDefaultOrientation OBJECT-TYPE

--  NOTE: In RFC 1759, the enumeration values were implicitly

--  defined by this object.

SYNTAX     PrtPrintOrientationTC

MAX-ACCESS read-write

STATUS     current

DESCRIPTION

"The current orientation default for this interpreter. This

value may be overridden for a particular job (e.g., by a

command in the input data stream)."

::= { prtInterpreterEntry 7 }

prtInterpreterFeedAddressability OBJECT-TYPE

-- NOTE: In RFC 1759, the range was not defined.

SYNTAX     Integer32 (-2..2147483647)

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The maximum interpreter addressability in the feed

direction in 10000 prtMarkerAddressabilityUnits (as specified

by prtMarkerDefaultIndex) for this interpreter. The

value (-1) means other and specifically indicates that the

sub-unit places no restrictions on this parameter. The value

(-2) means unknown.

NOTE: The above description has been modified from RFC 1759

for clarification."

::= { prtInterpreterEntry 8 }

prtInterpreterXFeedAddressability OBJECT-TYPE

-- NOTE: In RFC 1759, the range was not defined.

SYNTAX     Integer32 (-2..2147483647)

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The maximum interpreter addressability in the cross feed

direction in 10000 prtMarkerAddressabilityUnits (as specified

by prtMarkerDefaultIndex) for this interpreter. The

value (-1) means other and specifically indicates that the

sub-unit places no restrictions on this parameter. The value

(-2) means unknown.

NOTE: The above description has been modified from RFC 1759

for clarification."

::= { prtInterpreterEntry 9 }

prtInterpreterDefaultCharSetIn OBJECT-TYPE

SYNTAX     IANACharset

MAX-ACCESS read-write

STATUS     current

DESCRIPTION

"The default coded character set for input octets encountered

outside a context in which the Page Description Language

established the interpretation of the octets. (Input octets are

presented to the interpreter through a path defined in the

channel group.)"

::= { prtInterpreterEntry 10 }

prtInterpreterDefaultCharSetOut OBJECT-TYPE

SYNTAX     IANACharset

MAX-ACCESS read-write

STATUS     current

DESCRIPTION

"The default character set for data coming from this

interpreter through the printer's output channel (i.e. the

'backchannel')."

::= { prtInterpreterEntry 11 }

prtInterpreterTwoWay OBJECT-TYPE

--  NOTE: In RFC 1759, the enumeration values were implicitly

--  defined by this object.

SYNTAX     PrtInterpreterTwoWayTC

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"Indicates whether or not this interpreter returns information

back to the host."

::= { prtInterpreterEntry 12 }

-- The Console Group

--

-- Many printers have a console on the printer, the operator

-- console, that is used to display and modify the state of the

-- printer. The console can be as simple as a few indicators and

-- switches or as complicated as full screen displays and

-- keyboards. There can be at most one such console.

-- The Display Buffer Table

prtConsoleDisplayBuffer OBJECT IDENTIFIER ::= { printmib 16 }

prtConsoleDisplayBufferTable OBJECT-TYPE

SYNTAX     SEQUENCE OF PrtConsoleDisplayBufferEntry

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"Physical display buffer for printer console display or

operator panel

NOTE: The above description has been modified from RFC 1759

for clarification."

::= { prtConsoleDisplayBuffer 5 }

prtConsoleDisplayBufferEntry OBJECT-TYPE

SYNTAX     PrtConsoleDisplayBufferEntry

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"This table contains one entry for each physical line on

the display.  Lines cannot be added or deleted. Entries may

exist in the table for each device index with a device type of

'printer'.

NOTE: The above description has been modified from RFC 1759

for clarification."

INDEX  { hrDeviceIndex, prtConsoleDisplayBufferIndex }

::= { prtConsoleDisplayBufferTable 1 }

PrtConsoleDisplayBufferEntry ::= SEQUENCE {

prtConsoleDisplayBufferIndex    Integer32,

prtConsoleDisplayBufferText     PrtConsoleDescriptionStringTC

}

prtConsoleDisplayBufferIndex OBJECT-TYPE

SYNTAX     Integer32 (1..65535)

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"A unique value for each console line in the printer. The value

is used to identify this console line. Although these values

may change due to a major reconfiguration of the device (e.g.

the addition of new console lines to the printer). Values

SHOULD remain stable across successive printer power cycles.

NOTE: The above description has been modified from RFC 1759

for clarification."

::= { prtConsoleDisplayBufferEntry 1 }

prtConsoleDisplayBufferText OBJECT-TYPE

-- In RFC 1759, the SYNTAX was OCTET STRING.  This has been changed

-- to a TC to better support localization of the object.

SYNTAX     PrtConsoleDescriptionStringTC

MAX-ACCESS read-write

STATUS     current

DESCRIPTION

"The content of a line in the logical display buffer of

the operator's console of the printer.  When a write

operation occurs, normally a critical message, to one of

the LineText strings, the agent should make that line

displayable if a physical display is present.  Writing a zero

length string clears the line.  It is an implementation-

specific matter as to whether the agent allows a line to be

overwritten before it has been cleared. Printer generated

strings shall be in the localization specified by

prtConsoleLocalization.Management Application generated strings

should be localized by the Management Application."

::= { prtConsoleDisplayBufferEntry 2 }

-- The Console Light Table

prtConsoleLights OBJECT IDENTIFIER ::= { printmib 17 }

prtConsoleLightTable OBJECT-TYPE

SYNTAX     SEQUENCE OF PrtConsoleLightEntry

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"The console light table provides a description and state

information for each light present on the printer console.

NOTE: The above description has been modified from RFC 1759

for clarification."

::= { prtConsoleLights 6 }

prtConsoleLightEntry OBJECT-TYPE

SYNTAX     PrtConsoleLightEntry

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"Entries may exist in the table for each device index with a

device type of 'printer'.

NOTE: The above description has been modified from RFC 1759

for clarification."

INDEX  { hrDeviceIndex, prtConsoleLightIndex }

::= { prtConsoleLightTable 1 }

PrtConsoleLightEntry ::= SEQUENCE {

prtConsoleLightIndex            Integer32,

prtConsoleOnTime                Integer32,

prtConsoleOffTime               Integer32,

prtConsoleColor                 PrtConsoleColorTC,

prtConsoleDescription           PrtConsoleDescriptionStringTC

}

prtConsoleLightIndex OBJECT-TYPE

SYNTAX     Integer32 (1..65535)  -- Lower limit invalid in RFC 1759

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"A unique value used by the printer to identify this light.

Although these values may change due to a major

reconfiguration of the device (e.g. the addition of new lights

to the printer). Values SHOULD remain stable across successive

printer power cycles.

NOTE: The above description has been modified from RFC 1759

for clarification."

::= { prtConsoleLightEntry 1 }

prtConsoleOnTime OBJECT-TYPE

-- NOTE: In RFC 1759, the range was not defined.

SYNTAX     Integer32 (0..2147483647)

MAX-ACCESS read-write

STATUS     current

DESCRIPTION

"This object, in conjunction with prtConsoleOffTime, defines

the current status of the light.  If both prtConsoleOnTime and

prtConsoleOffTime are non-zero, the lamp is blinking and the

values presented define the on time and off time, respectively,

in milliseconds. If prtConsoleOnTime is zero and

prtConsoleOffTime is non-zero, the lamp is off. If

prtConsoleOffTime is zero and prtConsoleOnTime is non-zero, the

lamp is on. If both values are zero the lamp is off.

NOTE: The above description has been modified from RFC 1759

for clarification."

::= { prtConsoleLightEntry 2 }

prtConsoleOffTime OBJECT-TYPE

-- NOTE: In RFC 1759, the range was not defined.

SYNTAX     Integer32 (0..2147483647)

MAX-ACCESS read-write

STATUS     current

DESCRIPTION

"This object, in conjunction with prtConsoleOnTime, defines the

current status of the light. If both prtConsoleOnTime and

prtConsoleOffTime are non-zero, the lamp is blinking and the

values presented define the on time and off time, respectively,

in milliseconds. If prtConsoleOnTime is zero and

prtConsoleOffTime is non-zero, the lamp is off. If

prtConsoleOffTime is zero and prtConsoleOnTime is non-zero, the

lamp is on. If both values are zero the lamp is off.

NOTE: The above description has been modified from RFC 1759

for clarification."

::= { prtConsoleLightEntry 3 }

prtConsoleColor OBJECT-TYPE

--  NOTE: In RFC 1759, the enumeration values were implicitly

--  defined by this object.

SYNTAX     PrtConsoleColorTC

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The color of this light."

::= { prtConsoleLightEntry 4 }

prtConsoleDescription OBJECT-TYPE

-- In RFC 1759, the SYNTAX was OCTET STRING.  This has been changed

-- to a TC to better support localization of the object.

SYNTAX     PrtConsoleDescriptionStringTC

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The vendor description or label of this light in the

localization specified by prtConsoleLocalization."

::= { prtConsoleLightEntry 5 }

-- The Alerts Group

--

-- The table contains information on the severity, component,

-- detail location within the component, alert code and

-- description of each critical alert that is currently active

-- within the printer.  See 2.2.13 for a more complete

-- description of the alerts table and its management.

--

-- Each parameter in the Trap PDU is a full OID which itself is

-- indexed by the host resources MIB "hrDeviceIndex" object. In

-- order for a management station to obtain the correct

-- "hrDeviceIndex" associated with a particular Trap PDU, the

-- "hrDeviceIndex" value can be extracted from the returned OID

-- value in the Trap PDU when the PDU is received by the

-- Management station.

prtAlert OBJECT IDENTIFIER ::= { printmib 18 }

prtAlertTable OBJECT-TYPE

SYNTAX     SEQUENCE OF PrtAlertEntry

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"The prtAlertTable lists all the critical and non-critical

alerts currently active in the printer.  A critical alert is

one that stops the printer from printing immediately and

printing can not continue until the critical alert condition

is eliminated.  Non-critical alerts are those items that do

not stop printing but may at some future time.

NOTE: The above description has been modified from RFC 1759

for clarification."

::= { prtAlert 1 }

prtAlertEntry OBJECT-TYPE

SYNTAX     PrtAlertEntry

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"Entries may exist in the table for each device

index with a device type of 'printer'.

NOTE: The above description has been modified from RFC 1759

for clarification."

INDEX  { hrDeviceIndex, prtAlertIndex }

::= { prtAlertTable 1 }

PrtAlertEntry ::= SEQUENCE {

prtAlertIndex               Integer32,

prtAlertSeverityLevel       PrtAlertSeverityLevelTC,

prtAlertTrainingLevel       PrtAlertTrainingLevelTC,

prtAlertGroup               PrtAlertGroupTC,

prtAlertGroupIndex          Integer32,

prtAlertLocation            Integer32,

prtAlertCode                PrtAlertCodeTC,

prtAlertDescription         PrtLocalizedDescriptionStringTC,

prtAlertTime                TimeTicks

}

prtAlertIndex OBJECT-TYPE

-- NOTE: In RFC 1759, the range was not defined.

SYNTAX     Integer32 (1..2147483647)

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"The index value used to determine which alerts have been added

or removed from the alert table. This is an incrementing

integer initialized to 1 when the printer is reset. (i.e. The

first event placed in the alert table after a reset of the

printer shall have an index value of 1.)  When the printer adds

an alert to the table, that alert is assigned the next higher

integer value from the last item entered into the table.  If

the index value reaches its maximum value, the next index value

used must be 1.

NOTE: The management application will read the alert table when

a trap or event notification occurs or at a periodic rate and

then parse the table to determine if any new entries were added

by comparing the last known index value with the current

highest index value. The management application will then

update its copy of the alert table.  When the printer discovers

that an alert is no longer active, the printer shall remove the

row for that alert from the table and shall reduce the number

of rows in the table.  The printer may add or delete any number

of rows from the table at any time.  The management station can

detect when binary change alerts have been deleted by

requesting an attribute of each alert, and noting alerts as

deleted when that retrieval is not possible.  The objects

'prtAlertCriticalEvents'and 'prtAlertAllEvents' in the

'prtGeneralTable' reduce the need for management applications

to scan the 'prtAlertTable'.

NOTE: The above description has been modified from RFC 1759

for clarification."

::= { prtAlertEntry 1 }

prtAlertSeverityLevel OBJECT-TYPE

--  NOTE: In RFC 1759, the enumeration values were implicitly

--  defined by this object.

SYNTAX     PrtAlertSeverityLevelTC

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The level of severity of this alert table entry.  The printer

determines the severity level assigned to each entry into the

table."

::= { prtAlertEntry 2 }

prtAlertTrainingLevel OBJECT-TYPE

--  NOTE: In RFC 1759, the enumeration values were implicitly

--  defined by this object.

SYNTAX     PrtAlertTrainingLevelTC

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"See textual convention PrtAlertTrainingLevelTC.

NOTE: The above description has been modified from RFC 1759

for clarification."

::= { prtAlertEntry 3 }

prtAlertGroup OBJECT-TYPE

--  NOTE: In RFC 1759, the enumeration values were implicitly

--  defined by this object.

SYNTAX     PrtAlertGroupTC

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The type of sub-unit within the printer model that this alert

is related. Input, output, and markers are examples of printer

model groups, i.e., examples of types of sub-units. Wherever

possible, these enumerations match the sub-identifier that

identifies the relevant table in the printmib."

::= { prtAlertEntry 4 }

prtAlertGroupIndex OBJECT-TYPE

-- NOTE: In RFC 1759, the range was not defined.

SYNTAX     Integer32 (-1..2147483647)

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The low-order index of the row within the table identified

by prtAlertGroup that represents the sub-unit of the printer

that caused this alert, or -1 if not applicable.  The

combination of the prtAlertGroup and the prtAlertGroupIndex

defines exactly which printer sub-unit caused the alert; for

example, Input #3, Output#2, and Marker #1. Every object in

this MIB is indexed with hrDeviceIndex and optionally, another

index variable.  If this other index variable is present in the

table that generated the alert, it will be used as the value

for this object.  Otherwise, this value shall be -1.

NOTE: The above description has been modified from RFC 1759

for clarification."

::= { prtAlertEntry 5 }

prtAlertLocation OBJECT-TYPE

-- NOTE: In RFC 1759, the range was not defined.

SYNTAX     Integer32 (-2..2147483647)

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The sub-unit location that is defined by the printer

manufacturer to further refine the location of this alert

within the designated sub-unit.  The location is used in

conjunction with the Group and GroupIndex values; for example,

there is an alert in Input #2 at location number 7. The value

(-2) indicates unknown.

NOTE: The above description has been modified from RFC 1759

for clarification."

::= { prtAlertEntry 6 }

prtAlertCode OBJECT-TYPE

--  NOTE: In RFC 1759, the enumeration values were implicitly

--  defined by this object.

SYNTAX     PrtAlertCodeTC

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"See associated textual convention PrtAlertCodeTC.

NOTE: The above description has been modified from RFC 1759

for clarification."

::= { prtAlertEntry 7 }

prtAlertDescription OBJECT-TYPE

-- In RFC 1759, the SYNTAX was OCTET STRING.  This has been changed

-- to a TC to better support localization of the object.

SYNTAX     PrtLocalizedDescriptionStringTC

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"A description of this alert entry in the localization

specified by prtGeneralCurrentLocalization.  The description is

provided by the printer to further elaborate on the enumerated

alert or provide information in the case where the code is

classified as 'other' or 'unknown'.  The printer is required to

return a description string but the string may be a null

string."

::= { prtAlertEntry 8 }

prtAlertTime OBJECT-TYPE

SYNTAX     TimeTicks

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The value of sysUpTime at the time that this alert was

generated."

::= { prtAlertEntry 9 }

printerV1Alert OBJECT-IDENTITY

STATUS  current

DESCRIPTION

"The value of the enterprise-specific OID in an SNMPv1 trap

sent signaling a critical event in the prtAlertTable."

::= { prtAlert 2 }

printerV2AlertPrefix OBJECT IDENTIFIER ::= { printerV1Alert 0 }

printerV2Alert NOTIFICATION-TYPE

OBJECTS { prtAlertIndex, prtAlertSeverityLevel, prtAlertGroup,

prtAlertGroupIndex, prtAlertLocation, prtAlertCode }

STATUS  current

DESCRIPTION

"This trap is sent whenever a critical event is added to the

prtAlertTable.

NOTE: The prtAlertIndex object was redundantly included in the

bindings of the 'printerV2Alert' notification in RFC 1759, even

though the value exists in the instance qualifier of all the

other bindings.  This object has been retained to provide

compatiblity with existing RFC 1759 implementaions."

::= { printerV2AlertPrefix 1 }

-- Note that the SNMPv2 to SNMPv1 translation rules dictate that

-- the preceding structure will result in SNMPv1 traps of the

-- following form:

--

-- printerAlert TRAP-TYPE

--     ENTERPRISE printerV1Alert

--     VARIABLES { prtAlertIndex, prtAlertSeverityLevel,

--                 prtAlertGroup, prtAlertGroupIndex,

--                 prtAlertLocation, prtAlertCode }

--     DESCRIPTION

--        "This trap is sent whenever a critical event is added

--         to the prtAlertTable."

--     ::= 1

-- Conformance Information

prtMIBConformance OBJECT IDENTIFIER ::= { printmib 2 }

-- compliance statements

prtMIBCompliance MODULE-COMPLIANCE

STATUS  current

DESCRIPTION

"The compliance statement for agents that implement the

printer MIB as defined by RFC 1759."

MODULE -- this module

MANDATORY-GROUPS { prtGeneralGroup, prtInputGroup,

prtOutputGroup,

prtMarkerGroup, prtMediaPathGroup,

prtChannelGroup, prtInterpreterGroup,

prtConsoleGroup, prtAlertTableGroup }

OBJECT    prtGeneralReset

SYNTAX    INTEGER {

notResetting(3),

resetToNVRAM(5)

}

DESCRIPTION

"It is conformant to implement just these two states in this

object.  Any additional states are optional."

OBJECT    prtConsoleOnTime

MIN-ACCESS  read-only

DESCRIPTION

"It is conformant to implement this object as read-only"

OBJECT    prtConsoleOffTime

MIN-ACCESS  read-only

DESCRIPTION

"It is conformant to implement this object as read-only"

::= { prtMIBConformance 1 }

prtMIB2Compliance MODULE-COMPLIANCE

STATUS  current

DESCRIPTION

"The compliance statement for agents that implement the

printer MIB V2."

-- The changes from RFC 1759 fall into 2 categories:

-- 1. New objects plus existing objects with a MIN-ACCESS of

--    read-only are included.  Existing objects have been added

--    to this category due to feedback from implementers and

--    interoperability testing.  This allows products to be

--    be designed with a higher degree of SNMP security.

-- 2. New object groups have been added to include all new

--    objects in this MIB.  All new object groups are optional.

-- Any MIB that is compliant with RFC 1759 will also be

-- compliant with this version of the MIB.

MODULE -- this module

MANDATORY-GROUPS { prtGeneralGroup, prtInputGroup,

prtOutputGroup,

prtMarkerGroup, prtMediaPathGroup,

prtChannelGroup, prtInterpreterGroup,

prtConsoleGroup, prtAlertTableGroup }

OBJECT    prtGeneralReset

SYNTAX    INTEGER {

notResetting(3),

resetToNVRAM(5)

}

DESCRIPTION

"It is conformant to implement just these two states in this

object.  Any additional states are optional."

OBJECT    prtGeneralCurrentLocalization

MIN-ACCESS  read-only

DESCRIPTION

"It is conformant to implement this object as read-only"

OBJECT    prtGeneralCurrentOperator

MIN-ACCESS  read-only

DESCRIPTION

"It is conformant to implement this object as read-only"

OBJECT    prtGeneralServicePerson

MIN-ACCESS  read-only

DESCRIPTION

"It is conformant to implement this object as read-only"

OBJECT    prtGeneralPrinterName

MIN-ACCESS  read-only

DESCRIPTION

"It is conformant to implement this object as read-only"

OBJECT    prtGeneralSerialNumber

MIN-ACCESS  read-only

DESCRIPTION

"It is conformant to implement this object as read-only"

OBJECT    prtInputDefaultIndex

MIN-ACCESS  read-only

DESCRIPTION

"It is conformant to implement this object as read-only"

OBJECT    prtInputMediaDimFeedDirDeclared

MIN-ACCESS  read-only

DESCRIPTION

"It is conformant to implement this object as read-only"

OBJECT    prtInputMaxCapacity

MIN-ACCESS  read-only

DESCRIPTION

"It is conformant to implement this object as read-only"

OBJECT    prtInputCurrentLevel

MIN-ACCESS  read-only

DESCRIPTION

"It is conformant to implement this object as read-only"

OBJECT    prtInputMediaName

MIN-ACCESS  read-only

DESCRIPTION

"It is conformant to implement this object as read-only"

OBJECT    prtInputName

MIN-ACCESS  read-only

DESCRIPTION

"It is conformant to implement this object as read-only"

OBJECT    prtInputSecurity

MIN-ACCESS  read-only

DESCRIPTION

"It is conformant to implement this object as read-only"

OBJECT    prtInputMediaWeight

MIN-ACCESS  read-only

DESCRIPTION

"It is conformant to implement this object as read-only"

OBJECT    prtInputMediaType

MIN-ACCESS  read-only

DESCRIPTION

"It is conformant to implement this object as read-only"

OBJECT    prtInputMediaColor

MIN-ACCESS  read-only

DESCRIPTION

"It is conformant to implement this object as read-only"

OBJECT    prtInputMediaFormParts

MIN-ACCESS  read-only

DESCRIPTION

"It is conformant to implement this object as read-only"

OBJECT    prtOutputDefaultIndex

MIN-ACCESS  read-only

DESCRIPTION

"It is conformant to implement this object as read-only"

OBJECT    prtOutputMaxCapacity

MIN-ACCESS  read-only

DESCRIPTION

"It is conformant to implement this object as read-only"

OBJECT    prtOutputRemainingCapacity

MIN-ACCESS  read-only

DESCRIPTION

"It is conformant to implement this object as read-only"

OBJECT    prtOutputName

MIN-ACCESS  read-only

DESCRIPTION

"It is conformant to implement this object as read-only"

OBJECT    prtOutputSecurity

MIN-ACCESS  read-only

DESCRIPTION

"It is conformant to implement this object as read-only"

OBJECT    prtOutputMaxDimFeedDir

MIN-ACCESS  read-only

DESCRIPTION

"It is conformant to implement this object as read-only"

OBJECT    prtOutputMaxDimXFeedDir

MIN-ACCESS  read-only

DESCRIPTION

"It is conformant to implement this object as read-only"

OBJECT    prtOutputMinDimFeedDir

MIN-ACCESS  read-only

DESCRIPTION

"It is conformant to implement this object as read-only"

OBJECT    prtOutputMinDimXFeedDir

MIN-ACCESS  read-only

DESCRIPTION

"It is conformant to implement this object as read-only"

OBJECT    prtOutputStackingOrder

MIN-ACCESS  read-only

DESCRIPTION

"It is conformant to implement this object as read-only"

OBJECT    prtOutputPageDeliveryOrientation

MIN-ACCESS  read-only

DESCRIPTION

"It is conformant to implement this object as read-only"

OBJECT    prtOutputBursting

MIN-ACCESS  read-only

DESCRIPTION

"It is conformant to implement this object as read-only"

OBJECT    prtOutputDecollating

MIN-ACCESS  read-only

DESCRIPTION

"It is conformant to implement this object as read-only"

OBJECT    prtOutputPageCollated

MIN-ACCESS  read-only

DESCRIPTION

"It is conformant to implement this object as read-only"

OBJECT    prtOutputOffsetStacking

MIN-ACCESS  read-only

DESCRIPTION

"It is conformant to implement this object as read-only"

OBJECT    prtMarkerDefaultIndex

MIN-ACCESS  read-only

DESCRIPTION

"It is conformant to implement this object as read-only"

OBJECT    prtMarkerSuppliesMaxCapacity

MIN-ACCESS  read-only

DESCRIPTION

"It is conformant to implement this object as read-only"

OBJECT    prtMarkerSuppliesLevel

MIN-ACCESS  read-only

DESCRIPTION

"It is conformant to implement this object as read-only"

OBJECT    prtMediaPathDefaultIndex

MIN-ACCESS  read-only

DESCRIPTION

"It is conformant to implement this object as read-only"

OBJECT    prtChannelCurrentJobCntlLangIndex

MIN-ACCESS  read-only

DESCRIPTION

"It is conformant to implement this object as read-only"

OBJECT    prtChannelDefaultPageDescLangIndex

MIN-ACCESS  read-only

DESCRIPTION

"It is conformant to implement this object as read-only"

OBJECT    prtChannelState

MIN-ACCESS  read-only

DESCRIPTION

"It is conformant to implement this object as read-only"

OBJECT    prtChannelIfIndex

MIN-ACCESS  read-only

DESCRIPTION

"It is conformant to implement this object as read-only"

OBJECT    prtInterpreterDefaultOrientation

MIN-ACCESS  read-only

DESCRIPTION

"It is conformant to implement this object as read-only"

OBJECT    prtInterpreterDefaultCharSetIn

MIN-ACCESS  read-only

DESCRIPTION

"It is conformant to implement this object as read-only"

OBJECT    prtInterpreterDefaultCharSetOut

MIN-ACCESS  read-only

DESCRIPTION

"It is conformant to implement this object as read-only"

OBJECT    prtConsoleLocalization

MIN-ACCESS  read-only

DESCRIPTION

"It is conformant to implement this object as read-only"

OBJECT    prtConsoleDisable

MIN-ACCESS  read-only

DESCRIPTION

"It is conformant to implement this object as read-only"

OBJECT    prtConsoleDisplayBufferText

MIN-ACCESS  read-only

DESCRIPTION

"It is conformant to implement this object as read-only"

OBJECT    prtConsoleOnTime

MIN-ACCESS  read-only

DESCRIPTION

"It is conformant to implement this object as read-only"

OBJECT    prtConsoleOffTime

MIN-ACCESS  read-only

DESCRIPTION

"It is conformant to implement this object as read-only"

GROUP    prtResponsiblePartyGroup

DESCRIPTION

"This group is unconditionally optional."

GROUP    prtExtendedInputGroup

DESCRIPTION

"This group is unconditionally optional."

GROUP    prtInputMediaGroup

DESCRIPTION

"This group is unconditionally optional."

GROUP    prtExtendedOutputGroup

DESCRIPTION

"This group is unconditionally optional."

GROUP    prtOutputDimensionsGroup

DESCRIPTION

"This group is unconditionally optional."

GROUP    prtOutputFeaturesGroup

DESCRIPTION

"This group is unconditionally optional."

GROUP    prtMarkerSuppliesGroup

DESCRIPTION

"This group is unconditionally optional."

GROUP    prtMarkerColorantGroup

DESCRIPTION

"This group is unconditionally optional."

GROUP    prtAlertTimeGroup

DESCRIPTION

"This group is unconditionally optional."

-- the prtResponsiblePartyGroup, prtExtendedInputGroup,

-- prtInputMediaGroup, prtExtendedOutputGroup,

-- prtOutputDimensionsGroup, prtOutputFeaturesGroup,

-- prtMarkerSuppliesGroup, prtMarkerColorantGroup, and the

-- prtAlertTimeGroup are completely optional. However, it is

-- strongly RECOMMENDED that the prtAlertTimeGroup be implemented.

-- New to version 2 of this printer MIB:

OBJECT    prtAuxiliarySheetStartupPage

MIN-ACCESS  read-only

DESCRIPTION

"It is conformant to implement this object as read-only"

OBJECT    prtAuxiliarySheetBannerPage

MIN-ACCESS  read-only

DESCRIPTION

"It is conformant to implement this object as read-only"

OBJECT    prtInputMediaLoadTimeout

MIN-ACCESS  read-only

DESCRIPTION

"It is conformant to implement this object as read-only"

OBJECT    prtInputNextIndex

MIN-ACCESS  read-only

DESCRIPTION

"It is conformant to implement this object as read-only"

GROUP    prtAuxiliarySheetGroup

DESCRIPTION

"This group is unconditionally optional."

GROUP    prtInputSwitchingGroup

DESCRIPTION

"This group is unconditionally optional."

GROUP    prtGeneralV2Group

DESCRIPTION

"This group is unconditionally optional."

GROUP    prtAlertTableV2Group

DESCRIPTION

"This group is unconditionally optional."

GROUP    prtChannelV2Group

DESCRIPTION

"This group is unconditionally optional."

GROUP    prtAlertTrapGroup

DESCRIPTION

"This group is unconditionally optional."

::= { prtMIBConformance 3 }

prtMIBGroups    OBJECT IDENTIFIER ::= { prtMIBConformance 2 }

-- These groups are from RFC 1759 and are applicable to Printer MIB V2

prtGeneralGroup OBJECT-GROUP

OBJECTS { prtGeneralConfigChanges,

prtGeneralCurrentLocalization,

prtGeneralReset, prtCoverDescription,

prtCoverStatus,

prtLocalizationLanguage, prtLocalizationCountry,

prtLocalizationCharacterSet, prtStorageRefIndex,

prtDeviceRefIndex }

STATUS  current

DESCRIPTION

"The general printer group."

::= { prtMIBGroups 1 }

prtResponsiblePartyGroup OBJECT-GROUP

OBJECTS { prtGeneralCurrentOperator, prtGeneralServicePerson }

STATUS  current

DESCRIPTION

"The responsible party group contains contact information for

humans responsible for the printer."

::= { prtMIBGroups 2 }

prtInputGroup OBJECT-GROUP

OBJECTS { prtInputDefaultIndex, prtInputType, prtInputDimUnit,

prtInputMediaDimFeedDirDeclared,

prtInputMediaDimXFeedDirDeclared,

prtInputMediaDimFeedDirChosen,

prtInputMediaDimXFeedDirChosen, prtInputCapacityUnit,

prtInputMaxCapacity, prtInputCurrentLevel, prtInputStatus,

prtInputMediaName }

STATUS  current

DESCRIPTION

"The input group."

::= { prtMIBGroups 3 }

prtExtendedInputGroup OBJECT-GROUP

OBJECTS { prtInputName, prtInputVendorName, prtInputModel,

prtInputVersion, prtInputSerialNumber,

prtInputDescription, prtInputSecurity }

STATUS  current

DESCRIPTION

"The extended input group."

::= { prtMIBGroups 4 }

prtInputMediaGroup OBJECT-GROUP

OBJECTS { prtInputMediaWeight, prtInputMediaType,

prtInputMediaColor, prtInputMediaFormParts }

STATUS  current

DESCRIPTION

"The input media group."

::= { prtMIBGroups 5 }

prtOutputGroup OBJECT-GROUP

OBJECTS { prtOutputDefaultIndex, prtOutputType,

prtOutputCapacityUnit, prtOutputMaxCapacity,

prtOutputRemainingCapacity,  prtOutputStatus }

STATUS  current

DESCRIPTION

"The output group."

::= { prtMIBGroups 6 }

prtExtendedOutputGroup OBJECT-GROUP

OBJECTS { prtOutputName, prtOutputVendorName, prtOutputModel,

prtOutputVersion, prtOutputSerialNumber,

prtOutputDescription, prtOutputSecurity }

STATUS  current

DESCRIPTION

"The extended output group."

::= { prtMIBGroups 7 }

prtOutputDimensionsGroup OBJECT-GROUP

OBJECTS { prtOutputDimUnit, prtOutputMaxDimFeedDir,

prtOutputMaxDimXFeedDir, prtOutputMinDimFeedDir,

prtOutputMinDimXFeedDir }

STATUS  current

DESCRIPTION

"The output dimensions group"

::= { prtMIBGroups 8 }

prtOutputFeaturesGroup OBJECT-GROUP

OBJECTS { prtOutputStackingOrder,

prtOutputPageDeliveryOrientation, prtOutputBursting,

prtOutputDecollating, prtOutputPageCollated,

prtOutputOffsetStacking }

STATUS  current

DESCRIPTION

"The output features group."

::= { prtMIBGroups 9 }

prtMarkerGroup OBJECT-GROUP

OBJECTS { prtMarkerDefaultIndex, prtMarkerMarkTech,

prtMarkerCounterUnit, prtMarkerLifeCount,

prtMarkerPowerOnCount, prtMarkerProcessColorants,

prtMarkerSpotColorants, prtMarkerAddressabilityUnit,

prtMarkerAddressabilityFeedDir,

prtMarkerAddressabilityXFeedDir, prtMarkerNorthMargin,

prtMarkerSouthMargin, prtMarkerWestMargin,

prtMarkerEastMargin, prtMarkerStatus }

STATUS  current

DESCRIPTION

"The marker group."

::= { prtMIBGroups 10 }

prtMarkerSuppliesGroup OBJECT-GROUP

OBJECTS { prtMarkerSuppliesMarkerIndex,

prtMarkerSuppliesColorantIndex, prtMarkerSuppliesClass,

prtMarkerSuppliesType, prtMarkerSuppliesDescription,

prtMarkerSuppliesSupplyUnit,

prtMarkerSuppliesMaxCapacity, prtMarkerSuppliesLevel }

STATUS  current

DESCRIPTION

"The marker supplies group."

::= { prtMIBGroups 11 }

prtMarkerColorantGroup OBJECT-GROUP

OBJECTS { prtMarkerColorantMarkerIndex, prtMarkerColorantRole,

prtMarkerColorantValue, prtMarkerColorantTonality }

STATUS  current

DESCRIPTION

"The marker colorant group."

::= { prtMIBGroups 12 }

prtMediaPathGroup OBJECT-GROUP

OBJECTS { prtMediaPathDefaultIndex, prtMediaPathMaxSpeedPrintUnit,

prtMediaPathMediaSizeUnit, prtMediaPathMaxSpeed,

prtMediaPathMaxMediaFeedDir,

prtMediaPathMaxMediaXFeedDir,

prtMediaPathMinMediaFeedDir,

prtMediaPathMinMediaXFeedDir, prtMediaPathType,

prtMediaPathDescription, prtMediaPathStatus}

STATUS  current

DESCRIPTION

"The media path group."

::= { prtMIBGroups 13 }

prtChannelGroup OBJECT-GROUP

OBJECTS { prtChannelType, prtChannelProtocolVersion,

prtChannelCurrentJobCntlLangIndex,

prtChannelDefaultPageDescLangIndex, prtChannelState,

prtChannelIfIndex, prtChannelStatus

}

STATUS  current

DESCRIPTION

"The channel group."

::= { prtMIBGroups 14 }

prtInterpreterGroup OBJECT-GROUP

OBJECTS { prtInterpreterLangFamily, prtInterpreterLangLevel,

prtInterpreterLangVersion, prtInterpreterDescription,

prtInterpreterVersion, prtInterpreterDefaultOrientation,

prtInterpreterFeedAddressability,

prtInterpreterXFeedAddressability,

prtInterpreterDefaultCharSetIn,

prtInterpreterDefaultCharSetOut, prtInterpreterTwoWay }

STATUS  current

DESCRIPTION

"The interpreter group."

::= { prtMIBGroups 15 }

prtConsoleGroup OBJECT-GROUP

OBJECTS { prtConsoleLocalization, prtConsoleNumberOfDisplayLines,

prtConsoleNumberOfDisplayChars, prtConsoleDisable,

prtConsoleDisplayBufferText, prtConsoleOnTime,

prtConsoleOffTime, prtConsoleColor,

prtConsoleDescription }

STATUS  current

DESCRIPTION

"The console group."

::= { prtMIBGroups 16 }

prtAlertTableGroup OBJECT-GROUP

OBJECTS { prtAlertSeverityLevel, prtAlertTrainingLevel,

prtAlertGroup, prtAlertGroupIndex, prtAlertLocation,

prtAlertCode, prtAlertDescription }

STATUS  current

DESCRIPTION

"The alert table group."

::= { prtMIBGroups 17 }

prtAlertTimeGroup OBJECT-GROUP

OBJECTS { prtAlertTime }

STATUS  current

DESCRIPTION

"The alert time group. Implementation of prtAlertTime is

strongly RECOMMENDED."

::= { prtMIBGroups 18 }

prtMIB2Groups    OBJECT IDENTIFIER ::= { prtMIBConformance 4 }

-- These groups are unique to Printer MIB V2

prtAuxiliarySheetGroup OBJECT-GROUP

OBJECTS { prtAuxiliarySheetStartupPage,

prtAuxiliarySheetBannerPage }

STATUS  current

DESCRIPTION

"The auxiliary sheet group."

::= { prtMIBGroups 19 }

prtInputSwitchingGroup OBJECT-GROUP

OBJECTS { prtInputMediaLoadTimeout, prtInputNextIndex }

STATUS  current

DESCRIPTION

"The input switching group."

::= { prtMIBGroups 20 }

prtGeneralV2Group OBJECT-GROUP

OBJECTS { prtGeneralPrinterName, prtGeneralSerialNumber }

STATUS  current

DESCRIPTION

"The general printer group with new v2 objects."

::= { prtMIBGroups 21 }

prtAlertTableV2Group OBJECT-GROUP

OBJECTS { prtAlertCriticalEvents, prtAlertAllEvents }

STATUS  current

DESCRIPTION

"The alert table group with new v2 objects."

::= { prtMIBGroups 22 }

prtChannelV2Group OBJECT-GROUP

OBJECTS { prtChannelInformation }

STATUS  current

DESCRIPTION

"The channel group with a new v2 object."

::= { prtMIBGroups 23 }

prtAlertTrapGroup NOTIFICATION-GROUP

NOTIFICATIONS { printerV2Alert }

STATUS  current

DESCRIPTION

"The alert trap group."

::= { prtMIBGroups 24 }

END

7.  IANA Considerations

The initial version the IANA Printer MIB defined in section 5 of this document is to be archived by IANA and subsequently maintained according to the Process specified in section 2.4.1 of this document.

8.  Intellectual Property

The IETF takes no position regarding the validity or scope of any

intellectual property or other rights that might be claimed to

pertain to the implementation or use of the technology described in

this document or the extent to which any license under such rights

might or might not be available; neither does it represent that it

has made any effort to identify any such rights.  Information on the

IETF's procedures with respect to rights in standards-track and

standards-related documentation can be found in RFC 2028.  Copies of

claims of rights made available for publication and any assurances of

licenses to be made available, or the result of an attempt made to

obtain a general license or permission for the use of such

proprietary rights by implementors or users of this specification can

be obtained from the IETF Secretariat.

The IETF invites any interested party to bring to its attention any

copyrights, patents or patent applications, or other proprietary

rights which may cover technology that may be required to practice

this standard.  Please address the information to the IETF Executive

Director.

9.  Internationalization Considerations

See section 2.2.1.1, 'International Considerations'.

10.  Security Considerations

There are a number of management objects defined in this MIB module

with a MAX-ACCESS clause of read-write and/or read-create.  Such

objects may be considered sensitive or vulnerable in some network

environments.  The support for SET operations in a non-secure

environment without proper protection can have a negative effect on

network operations.  These are the tables and objects and their

sensitivity/vulnerability:

prtGeneralTable:

prtGeneralCurrentLocalization  -  Possible data loss

prtGeneralReset  -  Possible data loss

prtGeneralCurrentOperator  -  Possible severe inconvenience

prtGeneralServicePerson  -  Possible severe inconvenience

prtInputDefaultIndex  -  Possible data loss

prtOutputDefaultIndex  -  Possible minor inconvenience

prtMarkerDefaultIndex  -  Possible minor inconvenience

prtMediaPathDefaultIndex  -  Possible minor inconvenience

prtConsoleLocalization  -  Possible severe inconvenience

prtConsoleDisable  -  Possible severe inconvenience

prtAuxiliarySheetStartupPage  -  Possible minor inconvenience

prtAuxiliarySheetBannerPage  -  Possible minor inconvenience

prtGeneralPrinterName  -  Possible severe inconvenience

prtGeneralSerialNumber  -  Possible severe inconvenience

prtInputTable:

prtInputMediaDimFeedDirDeclared  -  Possible data loss

prtInputMediaDimXFeedDirDeclared  -  Possible data loss

prtInputMaxCapacity  -  Possible minor inconvenience

prtInputCurrentLevel  -  Possible minor inconvenience

prtInputMediaName  -  Possible minor inconvenience

prtInputName  -  Possible minor inconvenience

prtInputSecurity  -  Possible minor inconvenience

prtInputMediaWeight  -  Possible minor inconvenience

prtInputMediaType  -  Possible minor inconvenience

prtInputMediaColor  -  Possible minor inconvenience

prtInputMediaFormParts  -  Possible minor inconvenience

prtInputMediaLoadTimeout  -  Possible minor inconvenience

prtInputNextIndex  -  Possible minor inconvenience

prtOutputTable

prtOutputMaxCapacity  -  Possible minor inconvenience

prtOutputRemainingCapacity  -  Possible minor inconvenience

prtOutputName  -  Possible minor inconvenience

prtOutputSecurity  -  Possible minor inconvenience

prtOutputMaxDimFeedDir  -  Possible minor inconvenience

prtOutputMaxDimXFeedDir  -  Possible minor inconvenience

prtOutputMinDimFeedDir  -  Possible minor inconvenience

prtOutputMinDimXFeedDir  -  Possible minor inconvenience

prtOutputStackingOrder  -  Possible minor inconvenience

prtOutputPageDeliveryOrientation  -  Possible minor inconvenience

prtOutputBursting  -  Possible minor inconvenience

prtOutputDecollating  -  Possible minor inconvenience

prtOutputPageCollated  -  Possible minor inconvenience

prtOutputOffsetStacking  -  Possible minor inconvenience

prtMarkerSuppliesTable

prtMarkerSuppliesMaxCapacity  -  Possible minor inconvenience

prtMarkerSuppliesLevel  -  Possible minor inconvenience

prtChannelTable

prtChannelCurrentJobCntlLangIndex  -  Possible data loss

prtChannelDefaultPageDescLangIndex  -  Possible data loss

prtChannelState  -  Possible minor inconvenience

prtChannelIfIndex  -  Possible minor inconvenience

prtInterpreterTable

prtInterpreterDefaultOrientation  -  Possible data loss

prtInterpreterDefaultCharSetIn  -  Possible data loss

prtInterpreterDefaultCharSetOut  -  Possible minor inconvenience

prtConsoleDisplayBufferTable

prtConsoleDisplayBufferText  -  Possible minor inconvenience

prtConsoleLightTable

prtConsoleOnTime  -  Possible minor inconvenience

prtConsoleOffTime  -  Possible minor inconvenience

SNMP versions prior to SNMPv3 did not include adequate security.

Even if the network itself is secure (for example by using IPSec),

even then, there is no control as to who on the secure network is

allowed to access and GET/SET (read/change/create/delete) the objects

in this MIB module.

It is RECOMMENDED that implementers consider the security features as

provided by the SNMPv3 framework (see [RFC3410], section 8),

including full support for the SNMPv3 cryptographic mechanisms (for

authentication and privacy).

Further, deployment of SNMP versions prior to SNMPv3 is NOT

RECOMMENDED.  Instead, it is RECOMMENDED to deploy SNMPv3 and to

enable cryptographic security.  It is then a customer/operator

responsibility to ensure that the SNMP entity giving access to an

instance of this MIB module is properly configured to give access to

the objects only to those principals (users) that have legitimate

rights to indeed GET or SET (change/create/delete) them.

Where the operational capability of the printing device are especially vulnerable or difficult to administer, certain objects within this MIB have been tagged as READ-ONLY, preventing modification. Further, for all READ-WRITE objects within the MIB, the working group has included specific conformance guidelines stating that vendors are free to implement these objects as READ-ONLY. This conformance allowance should cover cases where specific vendor vulnerabilities may differ from product to product. (See conformance section with regards to MIN-ACCESS clauses).

11.  Copyright Section

"Copyright (C) The Internet Society 1995, 1997, 2002. All Rights Reserved.

This document and translations of it may be copied and furnished to others, and derivative works that comment on or otherwise explain or assist in its implementation may be prepared, copied, published and distributed, in whole or in part, without restriction of any kind, provided that the above copyright notice and this paragraph are included on all such copies and derivative works.  However, this document itself may not be modified in any way, such as by removing the copyright notice or references to the Internet Society or other Internet organizations, except as needed for the purpose of developing Internet standards in which case the procedures for copyrights defined in the Internet Standards process must be followed, or as required to translate it into languages other than English.

The limited permissions granted above are perpetual and will not be revoked by the Internet Society or its successors or assigns.

This document and the information contained herein is provided on an "AS IS" basis and THE INTERNET SOCIETY AND THE INTERNET ENGINEERING TASK FORCE DISCLAIMS ALL WARRANTIES, EXPRESS OR IMPLIED, INCLUDING BUT NOT LIMITED TO ANY WARRANTY THAT THE USE OF THE INFORMATION HEREIN WILL NOT INFRINGE ANY RIGHTS OR ANY IMPLIED WARRANTIES OF MERCHANTABILITY OR FITNESS FOR A PARTICULAR PURPOSE.

12.  Normative References

Find IETF (Internet Engineering Task Force) RFCs at www.ietf.org.

Find IANA (Internet Assigned Numbers Authority) at www.iana.org.

Find ISO (International Organization for Standardization) at www.iso.ch.

Find PWG (IEEE/ISTO Printer Working Group) at www.pwg.org.

[ASCII] ANSI, "Coded Character Set - 7-bit American Standard Code

for Information Interchange", ANSI X3.4-1986.

[CHARSET] IANA Character Set Registry:

http://www.iana.org/assignments/character-sets

[CHARMIB] IANA Character Set MIB:

http://www.iana.org/assignments/ianacharset-mib

[ISO10175] ISO, "Document Printing Application (DPA)", ISO 10175,

1996.

[ISO10646] ISO, "Universal Multiple-Octet Coded Character Set

(UCS) - Part 1: Architecture and Basic Multilingual Plane", ISO

10646-1, September 2000.

ISO, "Universal Multiple-Octet Coded Character Set (UCS) -

Part 2: Supplemental Planes", ISO 10646-2, January 2001.

[PWGMEDIA] IEEE-ISTO "The Printer Working Group Standard for Media

Standardized Names", IEEE-ISTO 5101.1-2002.

[RFC1213] McCloghrie & Rose, "Management Information Base for

Network Management of TCP/IP-based internets: MIB-II", RFC 1213,

1991.

[RFC2026] Bradner, "The Internet Standards Process - Revision 3",

RFC 2026, 1996.

[RFC2119] Bradner, "Key words for use in RFCs to Indicate

Requirement Levels", BCP 14, RFC 2119, March 1997.

[RFC2277] Alvestrand, "IETF Policy on Character Sets and Languages",

BCP 18, RFC 2277, 1998.

[RFC2279] Yergeau, "UTF-8, a transformation format of ISO 10646",

RFC 2279, 1998.

[RFC2434] Narten & Alvestrand, "Guidelines for Writing an IANA

Considerations Section in RFCs", RFC 2434, October 1998.

[RFC2578] McCloghrie et al, "Structure of Management Information

Version 2 (SMIv2)", STD 58, RFC 2578, April 1999.

[RFC2579] McCloghrie et al, "Textual Conventions for SMIv2", STD 58,

RFC 2579, April 1999.

[RFC2580] McCloghrie et al, "Conformance Statements for SMIv2", STD

58, RFC 2580, April 1999.

[RFC2790] Waldbusser & Grillo, "Host Resources MIB", RFC 2790, 2000.

[RFC2863] McCloghrie & Kastenholz, "The Interfaces Group MIB", RFC

2863, June 2000.

Note to RFC Editor:  Insert RFC number 'nnnn' when assigned.

[RFCnnnn] Bergman & Lewis, "Printer Finishing MIB", RFC nnnn, 2002.

13.  Informative References

Find IETF (Internet Engineering Task Force) RFCs at www.ietf.org.

[APPLEMAC] Apple staff, "Inside MacIntosh:  Networking", 1994.

[RFC854] Postel & Reynolds, "Telnet Protocol Specification", RFC

854, 1983.

[RFC959] Postel & Reynolds, "File Transfer Protocol", RFC 959, 1985.

[RFC1179] McLaughlin, "Line printer daemon protocol", RFC 1179,

1990.

[RFC1350] Sollins, "The TFTP Protocol (Revision 2)", RFC 1350, 1991.

[RFC1945] Berners-Lee et al, "Hypertext Transfer Protocol -

HTTP/1.0", RFC 1945, 1996.

[RFC2246] Dierks & Allen, "The TLS Protocol Version 1.0", RFC 2246,

1999.

[RFC2396] Berners-Lee et al, "Uniform Resource Identifiers (URI):

Generic Syntax", RFC 2396, 1998.

[RFC2616] Fielding et al, "Hypertext Transfer Protocol - HTTP/1.1",

RFC 2616, 1999.

[RFC2821] Klensin, "Simple Mail Transfer Protocol", RFC 2821, 2001.

[RFC2822] Resnick, "Internet Message Format", RFC 2822, April 2001.

[RFC2910] Herriot et al, "Internet Printing Protocol/1.1:  Encoding

and Transport", RFC 2910, 2000.

[RFC2911] Hastings et al, "Internet Printing Protocol/1.1:  Model

and Semantics", RFC 2911, 2000.

[RFC2978] Freed & Postel, "IANA Charset Registration Procedures",

BCP 19, RFC 2978, 2000.

[RFC3232] Reynolds, "Assigned Numbers:  RFC 1700 is Replaced by an

On-line Database", RFC 3232, January 2002.

[RFC3285] Gahrns & Hain, "Using Microsoft Word to create Internet

Drafts and RFCs", RFC 3285, May 2002.

[RFC3410] Case, J., Mundy, R., Partain, D. and B. Stewart,

"Introduction and Applicability Statements for Internet-

Standard Management Framework", RFC 3410, December 2002.

Appendix A - Glossary of Terms

Addressability - On the marker, the number of distinct marking units (pels) per unit of addressability unit that can be set; for example, 300 dots per inch is expressed as 300 per 1000 Thousandths Of Inches and 4 dots per millimeter is 4 per 1000 Micrometers. Addressability is not resolution because marks that are one addressability position apart may not be independently resolvable by the eye due to factors such as gain in the area of marks so they overlap or nearly touch.

Alert - A reportable event for which there is an entry in the alert table.

Bin - An output sub-unit which may or may not be removable.

Binary Change Event - An event which comes in pairs; the leading edge event and the trailing edge event. The leading edge event enters a state from which there is only one exit. A binary change event may be critical or non-critical. See unary change event.

Bursting - The process by which continuous media is separated into individual sheets, typically by bursting along pre-formed perforations.

Channel - A term used to describe a single source of data which is presented to a printer.  The model that we use in describing a printer allows for an arbitrary number of channels.  Multiple channels can exist on the same physical port.  This is commonly done over Ethernet ports where EtherTalk, TCP/IP, and SPX/IPX protocols can be supplying different data streams simultaneously to a single printer on the same physical port.

Collation - In multiple copy output, placing the pages from separate copies into separate ordered sets, ready for binding.

Control Language - A data syntax or language for controlling the printer through the print data channel.

Critical Alert - An alert triggered by an event which leads to a state in which printing is no longer possible; the printer is stopped.

Decollating - The process by which the individual parts within a multi-part form are separated and sorted into separate stacks for each part.

Description - Information about the configuration and capabilities of the printer and its various sub-units.

DPA - ISO 10175 Document Printing Application standard.  A standard for a client server protocol for a print system, including (1) submitting print jobs to and (2) managing print jobs in a spooler.

Event - A state change in the printer.

Group - A collection of objects that represent a type of sub-unit of the printer.

Host Resources MIB - See [RFC2790].

IANA - Internet Assigned Numbers Authority.  See [RFC3232].

Idempotent - Idempotence is the property of an operation that results in the same state no matter how many times it is executed (at least once).  This is a property that is shared by true databases in which operations on data items only change the state of the data item and do not have other side effects.  Because the SNMP data model is that of operations on a database, SNMP MIB objects should be assumed to be idempotent.  If a MIB object is defined in a non-idempotent way, the this data model can break in subtle ways when faced with packet loss, multiple managers, and other common conditions.

In order to fulfill the common need for actions to result from SNMP Set operations, SNMP MIB objects can be modeled such that the change in state from one state to another has the side effect of causing an action.  It is important to note that with this model, an SNMP operation that sets a value equal to its current value will cause no action.  This retains the idempotence of a single command, while allowing actions to be initiated by SNMP SET requests.

Input - A tray or bin from which instances of the media are obtained and fed into the Media Path.

Interpreter - The embodiment of an algorithm that processes a data stream consisting of a Page Description Language (PDL) and/or a Control Language.

Localization - The specification of human language, country, and character set needed to present information to people in their native languages.

Management Application (a.k.a. Manager) - A program which queries and controls one or more managed nodes.

Management Station - A physical computer on which one or more management applications can run.

Media Path - The mechanisms that transport instances of the media from an input, through the marker, possibly through media buffers and duplex pathways, out to the output with optional finishing applied.  The inputs and outputs are not part of the Media Path.

Non-critical Alert - An alert triggered by a reportable event which does not lead to a state in which printing is no longer possible; such an alert may lead to a state from which printing may no longer be possible in the future, such as the low toner state or the alert may be pure informational, such as a configuration change at the printer.

Output - A bin or stacker which accepts instances of media that have been processed by a printer.

Page Description Language (PDL) - A data syntax or language for the electronic representation of a document as a sequence of page images.

Printer - A physical device that takes media from an input source, produces marks on that media according to some page description or page control language and puts the result in some output destination, possibly with finishing applied.

Printing - The entire process of producing a printed document from generation of the file to be printed, choosing printing properties, selection of a printer, routing, queuing, resource management, scheduling, and finally printing including notifying the user.

Reportable event - An event that is deemed of interest to a management station watching the printer.

Status - Information regarding the current operating state of the printer and its various sub-units. This is an abstraction of the exact physical condition of the printer.

Sub-mechanism - A distinguishable part of a sub-unit.

Sub-unit - A part of the printer which may be a physical part, such as one of the input sources or a logical part such as an interpreter.

Tray - An input sub-unit which is typically removable.

Unary Change Event - An event that indicates a change of state of the printer, but to a state which is (often) just as valid as the state that was left, and from which no return is necessary. See binary change event.

Visible state - The portion of the state of the printer that can be examined by a management application.

Warning - A non-critical alert. See non-critical alert.

Appendix B - Media Size Names

The PWG Standardized Media Names specification [PWGMEDIA], section 5 Self  Describing Names, contains the currently recommended media size names. This appendix lists the standardized media size names from ISO/IEC 10175 Document Printing Application (DPA), [ISO10175] as presented in RFC 1759. Management applications are encouraged to use the names from the PWG standard. However, many legacy systems exist that use the DPA names and they are presented here for the convenience of developers.

A printer implementing the Printer MIB has no knowledge of these names, however; all media sizes in the MIB are given in terms of media dimensions as the values of prtInputMediaDimFeedDirChosen and prtInputMediaDimXFeedDirChosen.

String name               Description

other

unknown

na-letter or letter     North American letter size: 8.5 by 11 inches

na-legal or legal       North American legal size:  8.5 by 14 inches

na-10x13-envelope       North American 10x13 envelope

size:  10 by 13 inches

na-9x12-envelope        North American 9x12 envelope

size:  9 by 12 inches

na-number-10-envelope   North American number 10 business envelope

size:  4.125 by 9.5 inches

na-7x9-envelope         North American 7x9 size:  7 by 9 inches

na-9x11-envelope        North American 9x11 size: 9 by 11 inches

na-10x14-envelope       North American 10x14 envelope

size: 10 by 14 inches

na-number-9-envelope    North American number 9 business envelope

size:  3.875 by 8.875 inches

na-6x9-envelope         North American 6x9 envelope

size:  6 by 9 inches

na-10x15-envelope       North American 10x15 envelope

size: 10 by 15 inches

a                       engineering A size 8.5 inches by 11 inches

b                       engineering B size 11 inches by 17 inches

c                       engineering C size 17 inches by 22 inches

d                       engineering D size 22 inches by 34 inches

e                       engineering E size 34 inches by 44 inches

iso-a0                  ISO A0  size:  841 mm by 1189 mm

iso-a1                  ISO A1  size:  594 mm by  841 mm

iso-a2                  ISO A2  size:  420 mm by  594 mm

iso-a3                  ISO A3  size:  297 mm by  420 mm

iso-a4                  ISO A4  size:  210 mm by  297 mm

iso-a5                  ISO A5  size:  148 mm by  210 mm

iso-a6                  ISO A6  size:  105 mm by  148 mm

iso-a7                  ISO A7  size:   74 mm by  105 mm

iso-a8                  ISO A8  size:   52 mm by   74 mm

iso-a9                  ISO A9  size:   37 mm by   52 mm

iso-a10                 ISO A10 size:   26 mm by   37 mm

iso-b0                  ISO B0  size: 1000 mm by 1414 mm

iso-b1                  ISO B1  size:  707 mm by 1000 mm

iso-b2                  ISO B2  size:  500 mm by  707 mm

iso-b3                  ISO B3  size:  353 mm by  500 mm

iso-b4                  ISO B4  size:  250 mm by  353 mm

iso-b5                  ISO B5  size:  176 mm by  250 mm

iso-b6                  ISO B6  size:  125 mm by  176 mm

iso-b7                  ISO B7  size:   88 mm by  125 mm

iso-b8                  ISO B8  size:   62 mm by   88 mm

iso-b9                  ISO B9  size:   44 mm by   62 mm

iso-b10                 ISO B10 size:   31 mm by   44 mm

iso-c0                  ISO C0 size:   917 mm by 1297 mm

iso-c1                  ISO C1 size:   648 mm by  917 mm

iso-c2                  ISO C2 size:   458 mm by  648 mm

iso-c3                  ISO C3 size:   324 mm by  458 mm

iso-c4                  ISO C4 size:   229 mm by  324 mm

iso-c5                  ISO C5 size:   162 mm by  229 mm

iso-c6                  ISO C6 size:   114 mm by  162 mm

iso-c7                  ISO C7 size:    81 mm by  114 mm

iso-c8                  ISO C8 size:    57 mm by   81 mm

iso-designated          ISO Designated Long

size:   110 mm by  220 mm

jis-b0                  JIS B0  size  1030 mm by 1456 mm

jis-b1                  JIS B1  size   728 mm by 1030 mm

jis-b2                  JIS B2  size   515 mm by  728 mm

jis-b3                  JIS B3  size   364 mm by  515 mm

jis-b4                  JIS B4  size   257 mm by  364 mm

jis-b5                  JIS B5  size   182 mm by  257 mm

jis-b6                  JIS B6  size   128 mm by  182 mm

jis-b7                  JIS B7  size    91 mm by  128 mm

jis-b8                  JIS B8  size    64 mm by   91 mm

jis-b9                  JIS B9  size    45 mm by   64 mm

jis-b10                 JIS B10 size    32 mm by   45 mm

Appendix C - Media Names

For the convenience of management application developers, this appendix lists the standardized media names from ISO/IEC 10175 Document Printing Application (DPA), [ISO10175]. Management applications that present a dialogue for choosing media may wish to use these names as an alternative to separately specifying, size, color, and/or type. New names may also be created using this format and the names defined in the PWG Standardized Media Names specification [PWGMEDIA].

Using standard media names will mean that a single management application dealing with printers from different vendors and under different system mangers will tend to use the same names for the same media. If selection of media by name is used, the attributes (size, type or color) implied by the name must be explicitly mapped to the appropriate object (prtInputMediaDimFeedDirDeclared, prtInputMediaDimXFeedDirDeclared, prtInputMediaType and prtInputMediaColor) in the MIB. The object prtInputMediaName is intended for display to an operator and is purely descriptive. The value in prtInputMediaName is not interpreted by the printer so using a standard name for this value will not change any of the other media attributes nor will it cause an alert if the media in the input sub-unit does not match the name.

Simple Name                 Descriptor Text

other

unknown

iso-a4-white        Specifies the ISO A4 white medium with

size: 210 mm by 297 mm as defined in ISO 216

iso-a4-coloured     Specifies the ISO A4 colored medium with

size: 210 mm by 297 mm as defined in ISO 216

iso-a4-transparent  Specifies the ISO A4 transparent medium with

size: 210 mm by 297 mm as defined in ISO 216

iso-a3-white        Specifies the ISO A3 white medium with

size: 297 mm by 420 mm as defined in ISO 216

iso-a3-coloured     Specifies the ISO A3 colored medium with

size: 297 mm by 420 mm as defined in ISO 216

iso-a5-white        Specifies the ISO A5 white medium with

size: 148 mm by 210 mm as defined in ISO 216

iso-a5-coloured     Specifies the ISO A5 colored medium with

size: 148 mm by 210 mm as defined in ISO 216

iso-b4-white        Specifies the ISO B4 white medium with

size: 250 mm by 353 mm as defined in ISO 216

iso-b4-coloured     Specifies the ISO B4 colored medium with

size: 250 mm by 353 mm as defined in ISO 216

iso-b5-white        Specifies the ISO B5 white medium with

size: 176 mm by 250 mm as defined in ISO 216

iso-b5-coloured     Specifies the ISO B5 colored medium with

size: 176 mm by 250 mm as defined in ISO 216

jis-b4-white        Specifies the JIS B4 white medium with

size: 257 mm by 364 mm as defined in JIS P0138

jis-b4-coloured     Specifies the JIS B4 colored medium with

size: 257 mm by 364 mm as defined in JIS P0138

jis-b5-white        Specifies the JIS B5 white medium with

size: 182 mm by 257 mm as defined in JIS P0138

jis-b5-coloured     Specifies the JIS B5 colored medium with

size: 182 mm by 257 mm as defined in JIS P0138

The following standard values are defined for North American media:

na-letter-white     Specifies the North American letter white

medium with size: 8.5 inches by 11 inches

na-letter-coloured  Specifies the North American letter colored

medium with size: 8.5 inches by 11 inches

na-letter-transparent

Specifies the North American letter

transparent medium with size: 8.5 inches

by 11 inches

na-legal-white      Specifies the North American legal white

medium with size: 8.5 inches by 14 inches

na-legal-coloured   Specifies the North American legal colored

medium with size: 8.5 inches by 14 inches

The following standard values are defined for envelopes:

iso-b5-envelope     Specifies the ISO B5 envelope medium

with size: 176 mm by 250 mm

as defined in ISO 216 and ISO 269

iso-b4-envelope     Specifies the ISO B4 envelope medium

with size: 250 mm by 353 mm

as defined in ISO 216

iso-c4-envelope     Specifies the ISO C4 envelope medium

with size: 229 mm by 324 mm

as defined in ISO 216 and ISO 269

iso-c5-envelope     Specifies the ISO C5 envelope medium

with size: 162 mm by 229 mm

as defined in ISO 269

iso-designated-long-envelope

Specifies the ISO Designated Long envelope

medium with size: 110 mm by 220 mm

as defined in ISO 269

na-10x13-envelope   Specifies the North American 10x13 envelope

medium with size: 10 inches by 13 inches

na-9x12-envelope    Specifies the North American 9x12 envelope

medium with size: 9 inches by 12 inches

na-number-10-envelope

Specifies the North American number 10

business envelope medium with size: 4.125

inches by 9.5 inches

na-7x9-envelope     Specifies the North American 7x9 inch envelope

na-9x11-envelope    Specifies the North American 9x11 inch envelope

na-10x14-envelope   Specifies the North American 10x14 inch envelope

na-number-9-envelope

Specifies the North American number 9

business envelope 3.875 by 8.875 inches

na-6x9-envelope     Specifies the North American 6x9 inch envelope

na-10x15-envelope   Specifies the North American 10x15 inch envelope

The following standard values are defined for the less commonly

used media (white-only):

iso-a0-white  Specifies the ISO A0 white medium

with size:  841 mm by 1189 mm

as defined in ISO 216

iso-a1-white  Specifies the ISO A1 white medium

with size:  594 mm by 841 mm

as defined in ISO 216

iso-a2-white  Specifies the ISO A2 white medium

with size:  420 mm by 594 mm

as defined in ISO 216

iso-a6-white  Specifies the ISO A6 white medium

with size:  105 mm by 148 mm

as defined in ISO 216

iso-a7-white  Specifies the ISO A7 white medium

with size:  74 mm by 105 mm

as defined in ISO 216

iso-a8-white  Specifies the ISO A8 white medium

with size:  52 mm by 74 mm

as defined in ISO 216

iso-a9-white  Specifies the ISO A9 white medium

with size:  39 mm by 52 mm

as defined in ISO 216

iso-a10-white  Specifies the ISO A10 white medium

with size:  26 mm by 37 mm

as defined in ISO 216

iso-b0-white  Specifies the ISO B0 white medium

with size: 1000 mm by 1414 mm

as defined in ISO 216

iso-b1-white  Specifies the ISO B1 white medium

with size:  707 mm by 1000 mm

as defined in ISO 216

iso-b2-white  Specifies the ISO B2 white medium

with size:  500 mm by 707 mm

as defined in ISO 216

iso-b3-white  Specifies the ISO B3 white medium

with size:  353 mm by 500 mm

as defined in ISO 216

iso-b6-white  Specifies the ISO B6 white medium

with size:  125 mm by 176 mm i

as defined in ISO 216

iso-b7-white  Specifies the ISO B7 white medium

with size:  88 mm by 125 mm

as defined in ISO 216

iso-b8-white  Specifies the ISO B8 white medium

with size:  62 mm by 88 mm

as defined in ISO 216

iso-b9-white  Specifies the ISO B9 white medium

with size:  44 mm by 62 mm

as defined in ISO 216

iso-b10-white Specifies the ISO B10 white medium

with size:  31 mm by 44 mm

as defined in ISO 216

jis-b0-white  Specifies the JIS B0 white medium with size:

1030 mm by 1456 mm

jis-b1-white  Specifies the JIS B1 white medium with size:

728 mm by 1030 mm

jis-b2-white  Specifies the JIS B2 white medium with size:

515 mm by 728 mm

jis-b3-white  Specifies the JIS B3 white medium with size:

364 mm by 515 mm

jis-b6-white  Specifies the JIS B6 white medium with size:

257 mm by 364 mm

jis-b7-white  Specifies the JIS B7 white medium with size:

182 mm by 257 mm

jis-b8-white  Specifies the JIS B8 white medium with size:

128 mm by 182 mm

jis-b9-white  Specifies the JIS B9 white medium with size:

91 mm by 128 mm

jis-b10-white Specifies the JIS B10 white medium with size:

64 mm by 91 mm

The following standard values are defined for engineering media:

a        Specifies the engineering A size medium with

size: 8.5 inches by 11 inches

b        Specifies the engineering B size medium with

size: 11 inches by 17 inches

c        Specifies the engineering C size medium with

size: 17 inches by 22 inches

d        Specifies the engineering D size medium with

size: 22 inches by 34 inches

e        Specifies the engineering E size medium with

size: 34 inches by 44 inches

Appendix D - Roles of Users

Background

The need for "Role Models" stemmed in large part from the need to understand the importance of any given proposed object for the MIB.  Many times the real world need for a proposed object would be debated within the group; the debate would typically result in the need to describe the potential usage of the object in terms of a "live" person performing some type of printing-related task.

Determining the value of a proposed object through identification of the associated human users was found to be so common that a more formalized model was required for consistent analysis.  The model describing categories of human-oriented tasks is called "Role Models" in this document.

In developing the Role Models it was necessary to identify the common, primary tasks that humans typically face when interacting with a printer and its related printing system(s).  It was expected that certain kinds of tasks would serve to identify the various Role Models.

In presenting the set of Role Models, the set of "Common Print System Tasks" are first presented, followed by the set of Role Model definitions.  Finally, a simple matrix is presented in which Role Models and Tasks are cross-compared.

Common Print System Tasks

Upon researching the many tasks encountered by humans in dealing with printers and printing systems, the following were found to be pervasive within any operating environment:

Printer job state - Determine the status of a job without a printer.

Printer capabilities - Determine the current capabilities of a printer, for example, the available media sizes, two-sided printing, a particular type of interpreter, etc.

Printer job submission - Submit a print job to a printer.

Printer job removal - Remove a job from a printer.

Notification of events - Receive notification of the existence of a defined printer event.  An event can be of many types, including warnings, errors, job stage completion (e.g., "job done"), etc.

Printer configuration - Query the current configuration of a printer.

Printer consumables - Determine the current state of any and all consumables within a printer.

Print job identification - Determine the identification of a job within a printer.

Internal printer status - Determine the current status of the printer.

Printer identification - Determine the identity of a printer.

Printer location - Determine the physical location of a printer.

Local system configuration - Determine various aspects of the current configuration of the local system involved with the operation of a printer.

These "tasks" cover a large spectrum of requirements surrounding the operation of a printer in a network environment.  This list serves as the basis for defining the various Role Models described below.

Proposed Role Models

Following is the list of "Role Models" used to evaluate the requirements for any given Printer MIB object.  Note that the keyword enclosed in parentheses represents an abbreviation for the particular Role Model in the matrix described later in this document.

User  (USER) - A person or application that submits print jobs to the printer; typically viewed as the "end user" within the overall printing environment.

Operator  (OP) - A person responsible for maintaining a printer on a day-to-day basis, including such tasks as filling empty media trays, emptying full output trays, replacing toner cartridges, clearing simple paper jams, etc.

Technician  (TECH) - A person responsible for repairing a malfunctioning printer, performing routine preventive maintenance, and other tasks that typically require advanced training on the printer internals.  An example of a "technician" would be a manufacturer's Field Service representative, or other person formally trained by the manufacturer or similar representative.

System Manager  (MGR) - A person responsible for configuration and troubleshooting of components involved in the overall printing environment, including printers, print queues and network connectivity issues.  This person is typically responsible for ensuring the overall operational integrity of the print system components, and is typically viewed as the central point of coordination among all other Role Models.

Help Desk  (HELP) - A person responsible for supporting Users in their printing needs, including training Users and troubleshooting Users' printing problems.

Asset Manager  (AM) - A person responsible for managing an organization's printing system assets (primarily printers).  Such a person needs to be able to identify and track the location of printing assets on an ongoing basis.

Capacity Planner  (CP) - A person responsible for tracking the usage of printing resources on an ongoing basis for the purpose of planning printer acquisitions and/or placement of printers based on usage trends.

Installer  (INST) - A person or application responsible for installing or configuring printing system components on a local system.

Accountant  (ACCT) - A person responsible for tracking the usage of printing resources on an ongoing basis for the purpose of charging Users for resources used.

Matrix of Common Print System Tasks and Role Models

To better understand the relationship between the set of defined "Common Print System Tasks" and the various "Role Models," the following matrix is provided.

It is important to recognize that many of the tasks will appear to be applicable to many of the Role Models.  However, when considering the actual context of a task, it is very important to realize that often the actual context of a task is such that the Role Model can change.

For example, it is obvious that a "System Manager" must be able to submit print jobs to a printer; however, when submitting a print job, a person identified as a "System Manager" is actually operating in the context of a "User" in this case; hence, the requirement to submit a print job is not listed as a requirement for a System Manager.

Conversely, while a "User" must be able to remove a job previously submitted to a printer, an "Operator" is often expected to be able to remove any print job from any printer; hence, print job removal is a (subtly different) requirement for both the "User" and "Operator" Role Models.

Role Models

-----------

Requirement Area     USER OP  TECH MGR HELP  AM  CP INST ACCT

Print job status         xx  xx   xx  xx   xx

Printer capabilities     xx           xx   xx

Print job submission     xx

Print job removal        xx  xx

Notification of events       xx   xx

Printer configuration                 xx                xx

Printer consumables          xx                     xx  xx

Print job identification     xx       xx   xx       xx       xx

Internal printer status      xx   xx  xx

Printer identification       xx   xx  xx   xx   xx  xx  xx

Printer location                                    xx

Local system configuration            xx                xx

Appendix E - Overall Printer Status Table

The Status Table establishes a convention for the top 25 printer errors. The table defines a suggested relationship between various printer states and the variables Printer hrDeviceStatus, hrPrinterStatus, hrPrinterDetectedErrorState, prtAlertGroup, prtAlertCode and various sub-unit status variables (prtInputStatus, prtOutputStatus, prtMarkerStatus, prtMediaPathStatus and prtChannelStatus). This table is the recommended implementation of these variables. It is provided to guide implementors of this MIB and users of the MIB by providing a sample set of states and the variable values that are expected to be produced as result of that state. This information supplements that provided in Section 2.2.13.2 "Overall Printer Status". This is not an exhaustive list rather it is a guideline.

The definition of PrtSubUnitStatusTC specifies that SubUnitStatus is an integer that is the sum of 5 distinct values/states: Availability, Critical, Non-Critical, On-line and Transitioning.

Thus when a non-critical alert or alerts are present the values for Availability, On-Line and Transitioning will be summed with the Non-Critical Alerts (8) value.

The table was generated in landscape format and is located at  ftp://ftp.pwg.org/pub/pwg/pmp/contributions/Top25Errors.pdf.

Appendix F - Participants

The Printer MIB Working Group would like to extend a special thank you to the following individuals that put forth a significant effort to review this document and provide numerous suggestions for improvement.

David Harrington – Enterasys Networks

Juergen Schoenwaelder – TU Braunschweig

Bert Wijnen – Lucent Technologies and IETF Op & Mngmt, Area Director

This version of the Printer MIB would not be possible without the previous work that resulted in RFC 1759.  The authors of the Printer MIB version 2 would like to acknowledge the following individuals for their efforts in developing the base for this document.  A special recognition is also extended to Steve Waldbusser, who provided significant technical guidance in the development of the architecture of the Printer MIB.

Joel Gyllenskog - Microworks

Tom Hastings - Xerox

Jay Martin - Underscore, Inc.

Ron Smith - Texas Instruments

Steve Waldbusser - Lucent Technologies

Don Wright - Lexmark

Steve Zilles - Adobe

The following people attended at least one meeting of the Printer MIB Working Group for version 2; many attended most meetings.

Ron Bergman - Hitachi Printing Solutions

Luis Cubero - Hewlett-Packard

Jay Cummings - Novell

Andy Davidson - Tektronix

Lee Farrell - Canon

Tom Hastings - Xerox

Scott Isaacson - Novell

Binnur Al-Kazily - Hewlett-Packard

Rick Landau - Digital Equipment Corporation

David Kellerman - Northlake Software

Harry Lewis - IBM

Pete Loya - Hewlett-Packard

Jay Martin - Underscore, Inc.

Bob Pentecost - Hewlett-Packard

Dave Roach - Unisys

Stuart Rowley - Kyocera

Bob Setterbo - Adobe

Mike Timperman - Lexmark

Randy Turner - 2Wire, Inc.

Bill Wagner - NETsilicon, Inc.

Chris Wellens - Interworking Labs

Craig Whittle - Sharp Labs

Don Wright - Lexmark

Lloyd Young - Lexmark

Atsushi Yuki - Kyocera

Steve Zilles - Adobe

Authors' Addresses

Ron Bergman (Chairman)

Hitachi Printing Solutions America

1757 Tapo Canyon Road

Simi Valley, CA  93063-3394

Phone: (805) 578-4421

Email: Ron.Bergman@hitachi-ps.us

Harry Lewis

IBM

6300 Diagonal Hwy.

Boulder, CO 80301

Phone (303) 924-5337

Email: harryl@us.ibm.com

Ira McDonald

High North Inc

P.O. Box 221

Grand Marais, MI  49839

Phone: (906) 494-2434 or (906) 494-2697

Email: imcdonald@sharplabs.com

Significant contributors

Ray Casterline

Lighthouse Solutions, LLC

Phone: (716) 218-9910

Email: RayCasterline@lhsolutions.com

Gary Gocek

Phone: (585) 223-3826

Email: gary@gocek.org

Thomas N. Hastings

Xerox Corporation

Phone:  (310) 333-6413

Email: hastings@cp10.es.xerox.com

Scott Isaacson

Novell

Phone: (801) 861-7366

Email: sisaacson@novell.com

Binnur Al-Kazily

Hewlett-Packard, Inc.

Phone: (208) 396-6372

Email: binnur_al-kazily@hp.com

David Kellerman

Northlake Software

Phone: (503) 228-3383

Email: kellerman@nls.com

Matt King

Lexmark International

Phone: (859) 232-6907

Email: emking@lexmark.com

Jay Martin

Underscore, Inc.

Phone: (603) 889-7000

Email: jkm@underscore.com

Mike McKay

Novell, Inc.

Bob Pentecost

Hewlett-Packard

Phone: (208) 396-3312

Email: bpenteco@boi.hp.com

Stuart Rowley

Kyocera

Phone: (510) 299-7206

Email: stuart.rowley@kyocera.com

Gail Songer

Peerless Systems Networking

Phone: (650) 569-4414

Email: gsonger@peerless.com

Randy Turner

2Wire, Inc.

Phone (408) 895-1216

Email: rturner@2wire.com

William Wagner

NETsilicon, Inc.

Phone: (781) 398-4588

Email: bwagner@digprod.com

Chris Wellens

Interworking Labs

Phone: (408) 685-3190

Email: chrisw@iwl.com

F.D. Wright

Lexmark International

Phone: (859) 232-4808

Email: don@lexmark.com

Lloyd Young

Lexmark International

Phone: (859) 232-5150

Email: lpyoung@lexmark.com

Stephen N. Zilles

Adobe Systems, Inc.

Phone: (415) 962-4766

Email: szilles@mv.us.adobe.com