Printer Management Interface

Management Object Summary

Version 0.5

By

Ronald L. Smith

Texas Instruments

email:	rlsmith@nb.ppd.ti.com
phone:	(817) 774-6151

F.D. Wright

Lexmark International

email:   don@lexmark.com

phone:    606-232-4808

Enumerations

This working group has defined several type of enumerations.  These enumerations differ in the method employed to control the addition of new enumerations.  Throughout this document, references to "enum(n)", where N can be 1, 2 or 3 can be found in the various tables.  The definitions of these types of enumerations are:

ENUM(1)	Additional enumerated values require a new RFC.

ENUM(2)	Additional enumerated values are registered after review by this working group

ENUM(3)	Additional enumerated values are registered without working group review.

Printer Model

This working group has developed a model of a printer which is useful in understanding the classification of the various object.  Graphically this model is as shown below:

µ §

Printer Component

To be supplied by DMTF to correspond to MIB II

DMTF MIF "Component ID: (From DMI Specification - Draft 4.2 - 12 January 1994)

1.	Manufacturer	String64	Name of the organization that produces this component.

2.	Product	String64	Name of this Printer

3.	Version	String64

4.	Serial Number	String64

5.	Installation	Date	Time and Date of the Last Installation

6.	Verify	Integer	Verification Level for this printer

"Asking for the 'Verify' attribute causes the component instrumentation code to perform checks to verify that the component is still in the system and working properly. It should return on of the following values:

Value	Meaning

0	An error occurred; Check status code

1	Component does not exist

2	Verify not supported

4	Component exists, functionally untested

5	Component exists, functionally unknown

6	Component exists, functionally no good

7	Component exists, functionally good

RFC 1573

This is a place holder for the RFC 1573 objects........

General Printer Group Objects

| prtGeneral Mandatory |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| ConfigChange | Read-Only | Counter32 |  | Counts configuration changes |
| DefaultOrientation | Read-Write | Enum (1) | MediaOrientation |  |
| CurrentLocalization | Read-Write | Integer32 |  | Index into the localization table specifying the current language, country and character set for description object/attribute string values |

Notes: Overall printer status can be determined using the following objects from the host MIB: (hrDevice Status, hrPrinterStatus, and hrPrinterDetectedErrorState.

| prtGeneral Mandatory prtSkinTable |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| ID | Read-Only | Integer32 |  | Index of Skin component |
| Description | Read-Only | String |  | Manufacturer provided skin component name |
| Status | Read-Only | Enum(2) | SkinStatus |  |

Notes:

| prtGeneral Mandatory prtLocalizationTable |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| Language | Read-Only | String(2) |  | A two character language code from ISO639 |
| Country | Read-Only | String(2) |  | A two character country code from ISO3166, blank string will indicate the country is not defined. |
| CharacterSet | Read-Only | Integer32 |  | An index into the character set table identifying the character set used for this localization |

Notes:

| prtGeneral Mandatory prtCharacterSetTable |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| CharacterSet | Read-Only | String |  | A characterset name or ID that identifies the character set |
| Description | Read-Only | String |  | Description of this character set |

Notes:

| prtGeneral - Extended Optional |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| Reset | Write-Only | Enum(3) | ResetType | Writing causes a reset |

Notes:

Input Group Objects - Mandatory

| prtInput Mandatory |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| DefaultID | Read-Write | Integer32 |  | ID of the Default Input Component |

Notes:

| prtInput Mandatory prtInputTable |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| ID | Read-Only | Integer32 |  | Value used by the printer to identify associated input device |
| Type | Read-Only | Enum (2) | InputType |  |
| DimUnits | Read-Write | Enum (1) | PhyMediaUnit |  |
| MediaDimAcrossFeedDir | Read-Write | Integer32 |  | -1 = Other, -2 = Unknown |
| MediaDimFeedDir | Read-Write | Integer32 |  | -1 = Other, -2 = Unknown |
| CapacityUnit | Read-Write | Enum (1) | CapacityUnit |  |
| MaxCapacity | Read-Write | Integer32 |  | -1 = Other, -2 = Unknown |
| CurrentLevel | Read-Write | Integer32 |  | -1 = Other, -2 = Unknown |
| Status | Read-Only | Enum | EntStatus | Current Status of Input Device |
| MediaName | Read-Write | String |  | "Hillary's Pink Presidential Letterhead" |

Notes:

µ §

Input Group Objects - Optional

| prtInput - TypeExtended Optional prtInputTableAugment |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| Name | Read-Write | String |  | Name assigned to this device |
| VendorName | Read-Only | String |  |  |
| Model | Read-Only | String |  |  |
| Version | Read-Only | String |  |  |
| SerialNumber | Read-Only | String |  |  |
| Description | Read-Only | String |  |  |
| Security | Read-Write | Enum (2) | YesNo-OnOff | Indication if the Device has some security associated with it. |

Notes:

| prtInput - DimExtended Optional prtInputTableAugment |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| PrintAreaOffsetAcrossFeed | Read-Only | Integer32 |  | -1 = Other, -2 = Unknown |
| PrintAreaExtentAcross Feed | Read-Only | Integer32 |  | -1 = Other, -2 = Unknown |
| PrintAreaOffset Feed | Read-Only | Integer32 |  | -1 = Other, -2 = Unknown |
| PrintAreaExtent Feed | Read-Only | Integer32 |  | -1 = Other, -2 = Unknown |

Note:

| prtInput - MediaExtended Optional prtInputTableAugment |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| MediaWeight | Read-Write | Integer32 | grams/meter2 | -1 = Other, -2 = Unknown |
| MediaType | Read-Write | String |  | ISO10175 (DPA) & ISO10180 (SPDL) Definitions |
| MediaColor | Read-Write | String |  | ISO10175 (DPA) & ISO10180 (SPDL) Definitions |
| MediaFormParts | Read-Write | Integer32 |  | -1 = Other, -2 = Unknown |

Notes:

Output Group Objects - Mandatory

| prtOutput Mandatory |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| DefaultID | Read-Write | Integer32 |  | ID of Default Output Device |

Notes:

| prtOutput Mandatory prtOutputTable |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| ID | Read-Only | Integer32 |  | Value used by the printer to identify associated output device. |
| Type | Read-Only | Enum (2) | OutputType |  |
| CapacityUnit | Read-Only | Enum (1) | CapacityUnit |  |
| MaxCapacity | Read-Write | Integer32 |  | -1 = Other, -2 = Unknown, based on CapacityUnit |
| RemainingCapacity | Read-Write | Integer32 |  | -1 = Other, -2 = Unknown, based on CapacityUnit |
| Status | Read-Only | Enum | EntStatus |  |

Notes:

µ §

Output Group Objects - Optional

| prtOutput - TypeExtended Optional prtOutputTableAugment |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| Name | Read-Write | String |  | Name assigned to this Device |
| VendorName | Read-Only | String |  |  |
| Model | Read-Only | String |  |  |
| Version | Read-Only | String |  |  |
| SerialNumber | Read-Only | String |  |  |
| Description | Read-Only | String |  |  |
| Security | Read-Write | Enum (2) | YesNo-OnOff | Note 1, Note 2 |

Notes:

| prtOutput - DimExtended Optional prtOutputTableAugment |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| DimUnit | Read-Only | Enum (1) | PhyMediaUnit |  |
| MaxDimAcrossFeedDir | Read-Write | Integer32 |  | -1 = Other, -2 = Unknown |
| MaxDimFeedDir | Read-Write | Integer32 |  | -1 = Other, -2 = Unknown |
| MinDimAcrossFeedDir | Read-Write | Integer32 |  | -1 = Other, -2 = Unknown |
| MinDimFeedDir | Read-Write | Integer32 |  | -1 = Other, -2 = Unknown |

Notes:

| prtOutput - Features Optional prtOutputTableAugment |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| StackingOrder | Read-Write | Enum (1) |  | "1 to n" or "n to 1" |
| PageDeliveryOrientation | Read-Write | Enum (1) |  | "face up" or "face down" |
| Bursting | Read-Write | Enum (1) | YesNo-OnOff | Indicates a burster is installed |
| Decollating | Read-Write | Enum (1) | YesNo-OnOff | Indicates a decollater is installed |
| PageCollated | Read-Write | Enum (1) | YesNo-OnOff | Indicates a page collater is installed |
| Job Collated | Read-Write | Enum (1) | YesNo-OnOff | Indicates a job collater is installed |
| OffsetStacking | Read-Write | Enum (1) | YesNo-OnOff | Indicates an offset stacking is installed |
| Slipsheet | Read-Write | Enum (1) | YesNo-OnOff | Indicates a slip sheet inserter is installed |

Notes:

| prtOutput - Associated Finisher Optional prtOutputAssocFinisher |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| ID | Read-Only | Integer32 |  | ID which points to an output to which it is connected |
| Stitching | Read-Only | Enum(1) | YesNo-OnOff |  |
| Folding | Read-Only | Enum(1) | YesNo-OnOff |  |
| Binding | Read-Only | Enum(1) | YesNo-OnOff |  |
| Punching | Read-Only | Enum(1) | YesNo-OnOff |  |
| Trimming | Read-Only | Enum(1) | YesNo-OnOff |  |
| Insertion | Read-Only | Enum(1) | YesNo-OnOff |  |

Notes:

Finisher Group Objects - Optional

| prtFinisher Optional |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| DefaultID | Read-Write | Integer32 |  | 0 = Finisher Not Used in Default Configuration |

Notes:

| prtFinisher Optional prtFinisherTable |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| ID | Read-Only | Integer32 |  | Value used by the printer to identify associated finisher device. |
| Type | Read-Only | Enum() | Finisher Type |  |
| NamedFinishing | Read-Only | Enum() |  | Note 1 |
| ConsumableUnit | Read-Only | Enum() | Finisher Consumable Unit |  |
| MaxConsumableCapacity | Read-Write | Integer32 |  | -1 = Other, -2 = Unknown |
| CurrentConsumableCapacity | Read-Write | Integer32 |  | -1 = Other, -2 = Unknown |
| Status | Read-Only | Enum | EntStatus | Current Status of Device |

Notes:

1) What are the enumerated values here?

Marker Group Objects - Mandatory

| prtMarker Mandatory |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| DefaultID | Read-Write | Integer32 |  | ID of Default Marker |

Notes:

| prtMarker Mandatory prtMarkerTable |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| ID | Read-Only | Integer32 |  | Value used by the printer to identify associated marker device. |
| MarkTech | Read-Only | Enum (2) | MarkingTech | Type of Marking Technology used for this making device |
| CounterUnits | Read-Only | Enum (1) | CounterUnits | Units that will be used to report counter values |
| LifeCount | Read-Only | Counter32 |  | Life of printer counter in CounterUnits |
| PowerOnCount | Read-Only | Counter32 |  | Number of units counted since power on in CounterUnits |
| ProcessColorants | Read-Only | Integer32 |  | Number of Process Colors (-1 = Other, -2 = Unknown) |
| SpotColorants | Read-Only | Integer32 |  | Number of Spot Colors (-1 = Other, -2 = Unknown) |
| AddressabilityUnit | Read-Only | Enum (1) | AddrUnits | Resolution Unit |
| AddressabilityFeedDir | Read-Only | Integer32 |  | Feed Resolution (-1 = Other, -2 = Unknown) |
| AddressabilityCrossFeedDir | Read-Only | Integer32 |  | CrossFeed Resolution (-1 = Other, -2 = Unknown) |
| Status | Read-Only | Enum(1) | EntStatus | Current Status of Marker Component |

Notes:

| prtMarkerSupplies Mandatory prtMarkerSuppliesTable |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| MarkerSupplyID | Read-Only | Integer32 |  | ID of each marker supply |
| MarkerID | Read-Only | Integer32 |  | Maker ID of this supply entity |
| ColorantID | Read-Only | Integer32 |  | Associate Colorant ID (0 if no colorant table) |
| Class | Read-Only | Enum (1) | MkrSupClass | Class (Supply or receptacle) |
| Type | Read-Only | Enum(3) | MkrSupType | Type of supply |
| Description | Read-Only | String |  | Description of the supply or receptacle |
| SupplyUnit | Read-Only | Enum(1) | MkrSupUnit | Units of the marker supply container/receptacle |
| MaxCapacity | Read-Only | Integer32 |  | Maximum capacity expressed in SupplyUnit |
| Level | Read-Only | Integer32 |  | Current level if supply, remaining space if receptacle |

Notes:

| prtMarkerColorant Optional prtMarkerColorantTable |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| ColorantID | Read-Only | Integer32 |  | ID of this colorant |
| MarkerID | Read-Only | Integer32 |  | ID of Marker that this colorant entry is associated with. |
| Role | Read-Only | Enum (1) | ColorantRole | Role played by this colorant |
| Value | Read-Only | String | ColorantValue | Name of the color. |
| Tonality | Read-Only | Integer32 |  | Number of levels of tonality. Must be at least 2. |

Notes:

Media Path Group Objects - Mandatory

| prtMediaPath Mandatory |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| DefaultID | Read-Write | Integer32 |  | ID number of the default Media Path |
| MaxSpeedPrintUnit | Read-Only | Enum (1) | SpeedPrintUnit | Numerator units of print speed |
| MaxSpeedTimeUnit | Read-Only | Enum (1) | SpeedTimeUnit | Denominator units of print speed |
| PhyMediaUnit | Read-Only | Enum (1) | PhyMediaUnit | Units of media size |

Notes:

| prtMediaPath Mandatory prtMediaPathTable |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| ID | Read-Only | Integer32 |  | Value used by printer to identify Media Path |
| MaxSpeed | Read-Only | Integer32 |  | -1 = Other, -2 = Unknown |
| MaxMediaFeedDir | Read-Only | Integer32 |  | -1 = Other, -2 = Unknown |
| MaxMediaAcrossFeedDir | Read-Only | Integer32 |  | -1 = Other, -2 = Unknown |
| MinMediaFeedDir | Read-Only | Integer32 |  | -1 = Other, -2 = Unknown |
| MinMediaAcrossFeedDir | Read-Only | Integer32 |  | -1 = Other, -2 = Unknown |
| MediaPathType | Read-Only | Enum(2) | MediaPathType |  |
| MediaPathDescription | Read-Only | String |  | Manufacturer provided description of this Media Path |
| Status | Read-Only | Enum | EntStatus | Status of this Media Path |

Notes:

Channel Group Objects

| prtChannel Mandatory prtChannelTable |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| ChannelIndex | Read-Only | Integer32 |  | Index of channels |
| CurrentJobCntlLang | Read-Write | Integer32 |  | Index into the interpreter table |
| DefaultPageDescLang | Read-Write | Integer32 |  | Index into the interpreter table |
| ChannelState | Read-Write | Enum(1) |  | State of the Channel |
| ChannelType | Read-Only | Enum(2) |  | Type of Channel |
| InterfaceIndex | Read-Write | Integer |  | The value of this variable is value of the IfIndex in the interface section of MIB-2/RFC1573 for this channel. |

Notes:

Console Group Objects

| prtDisplayConsole |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| Localization Index | Read-Write | Integer32 |  | Index into localization table |
| NumberOfDisplayLines | Read-Only | Integer32 |  | Number of lines displayable by the printer |
| NumberOfDisplayChars | Read-Only | Integer32 |  | Number of Characters per Lines displayable by the printer |

| prtOperatorConsole Mandatory prtConsoleDisplayTable |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| LineIndex | Read-Only | Integer32 |  | Index into display |
| LineText | Read-Write | String |  | Text on or to be written to the display |

Notes:

| prtOperatorConsole Mandatory prtConsoleLightsTable |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| LightID | Read-Only | Integer32 |  | Index of lights |
| OnTime | Read-Only | Integer32 |  | On time in milliseconds of blinking (0=off always) |
| OffTime | Read-Only | Integer32 |  | Off time in milliseconds of blinking (0=on always) |
| Color | Read-Only | Enum(2) | Light Color | Color of the light |
| Description | Read-Only | String |  | Vendor Description or Lable of the light |

Notes:	Is this "Console" or "Operator Console"

Interpreter Group Objects - Mandatory

| prtInterpreter Mandatory |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| DefaultID | Read-Write | Integer32 | IntpLangFam |  |

Notes:

| prtInterpreter Mandatory prtInterpreterTable |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| ID | Read-Only | Integer32 |  | A unique value for each PDL or control language |
| LangFamily | Read-Only | Enum(2) | IntpLangFam |  |
| LangLevel | Read-Only | String |  |  |
| LangVersion | Read-Only | String |  |  |
| Description | Read-Only | String |  |  |
| Version | Read-Only | String |  |  |
| FeedAddressability | Read-Only | Integer32 |  | -1 = Other, -2 = Unknown |
| CrossFeedAddressability | Read-Only | Integer32 |  | -1 = Other, -2 = Unknown |
| DefaultCharSetIn | Read-Write | Integer32 |  | An index into the CharacterSetTable that identifies the default character set for data into the interpreter |
| DefaultCharSetOut | Read-Write | Integer32 |  | An index into the CharacterSetTable that identifies the default character set for data coming from the interpreter through the printer's output channel |
| TwoWay | Read-Only | Enum(1) | YesNo-OnOff | This object indicates whether or not the interpreter returns information back across the data channel. |

Notes:

Alerts Group Object - Mandatory

| prtAlert Mandatory prtAlertTable |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| Index | Read-Only | Integer32 |  |  |
| SeverityLevel | Read-Only | Enum (1) |  |  |
| TrainingLevel | Read-Only | Enum (2) | Training Level |  |
| Group | Read-Only | Enum (1) |  | Group within the model causing the alert |
| Group Index | Read-Only | Integer32 |  | Arbitrary integer provided by mfgr |
| Location | Read-Only | Integer32 |  |  |
| Alert Code | Read-Only | Enum (2) |  | Group dependent alert code |
| Description | Read-Only | String |  |  |

Notes:

Group Status

| EntStatus 1 Other 2 Unknown 3 Ready/Available 4 Critical State 5 Non-critical state 6 Both Critical and Non-critical state 7 On-request 8 Unavailable 9 Busy/Temp Unavailable |  |
| --- | --- |

PMI Enumerations - Units

| Addressabililty Unit 1 Other 2 Unknown 3 .0001 Inches 4 .001 millimeters | Physical Media Unit 1 Other 2 Unknown 3 .0001 Inches 4 .001 Millimeters 8 Sheets | Counter Unit 1 Other 2 Unknown 3 .0001 inches 4 .001 millimeters 5 Characters 6 Lines 7 Impressions 8 Sheets 9 Dot-Row 11 Hours 16 Feet 17 Meters |
| --- | --- | --- |
| Speed Print Unit 1 Other 2 Unknown 3 .0001 inches 4 .001 millimeters 5 Characters 6 Lines 7 Impressions 8 Sheets 9 Dot-Row | Speed Time Unit 1 Other 2 Unknown 11 Hours | Finisher Consumable Unit 1 Other 2 Unknown |
| Marker Supply Units 1 Other 2 Unknown 3 .0001 inches 4 .001 millimeters 8 Sheets 12 .001 ounces 13 .1 grams 14 .01 fluid ounces 15 .1 millileters | Capacity Units 1 Other 2 Unknown 3 .0001 inces 4 .001 millimeters 8 Sheets 16 Feet 17 Meters |  |

PMI Enumerations - Device Types

| Input Type 1 Other 2 Unknown 3 Sheet-feed-auto-removable-tray 4 sheet-feed-auto-non removable tray 5 Sheet Feed Manual 6 continuous-roll 7 continuous-fan-fold | Output Type 1 Other 2 Unknown 3 Removable Bin 4 UnRemovable Bin 5 Continuous Roll Device 6 Mail Box | Media Path Type 1 Other 2 Unknown 3 Long Edge Binding Duplex 4 Short Edge Binding Duplex 5 Either edge Duplex 6 Simplex |
| --- | --- | --- |
| Finisher Type 1 Other 2 Unknown 3 Stitching 4 Binding 5 Trimming 6 Die-cutting 7 Punching 8 Perforating 9 Slitting 10 Inserting 11 Covers 12 Folding | Marker Supply Type 1 Other 2 Unknown 3 Toner 4 Waste Toner 5 Ink 6 Ink Cartridge 7 Ink Ribbon 8 Waste Ink 9 OPC (Organic Photo Contuctor) 10 Developer 11 Fuser Oil 12 Wax, Solid 13 Wax, ribbon 14 Waste Wax | Marker Supply Class 1 Other 2 Unknown 3 Supply that is consumed 4 Receptacle that is filled |
| Marking Technology 1 Other 2 Unknown 3 Electophotographic:LED 4 Electrophotographic:Laser 5 Electrophotographic:Other 6 Impact:Moving_Head:Dot_Matrix:9pin 7 Impact:Moving_Head:Dot_Matrix:24pin 8 Impact:Moving_Head:Dot_Matrix:other 9 Impact:Moving_Head:Fully_Formed 10 Impact:band 11 Impact:other 12 Inkjet:Aqueous 13 Inkjet:Solid 14 Inkjet:Other 15 Pen 16 Thermal:Transfer 17 Thermal:Sensitive 18 Thermal:Diffusion 19 Thermal:Other 20 Electroerosion 21 Electrostatic 22 Photographic:Microfiche 23 Photographic:Imagesetter 24 Photographic:Other 25 Ion-deposition 26 E-beam 27 Typesetter | Channel Type 1. Other 2. Unknown 3. Serial Port 4. Parallel Port 5. IEEE1284 Port 6. SCSI Port 7. Apple PAP 8. LPD Server 9. Novell RPrinter 10. Novell Pserver 11. port 9100 12. FTP ("PUT" to printer) 13. TFTP (to printer) 14. Adobe AppSocket (9101) 15. DLC/LLC Port 16. Coax (3270) 17. Twinax (5250) 18. Fax 19. IEEE 1394 20. CPAP (port 170) 21. Transport 1 (port 35) 22. IPC | Interpreter Language Family 1 Other 2 Unknown 3 PCL 4 HPGL 5 PJL 6 PostScript 7 IPDS 8 PPDS 9 EscapeP 10 Epson 11 DDIF 12 Interpress 13 ISO6429 14 LinePrinter 15 MODCA 16 ReGIS 17 SCS 18 SPDL 19 Tek4014 20 PDS 21 IGP 22 Code-V 23 DSC-DSE 24 WPS 25 LN03 26 CCITT 27 QUIC 28 CPAP 29 DEC-PPL 30 SimpleText 31 NPAP 32 Automatic 33 DOC (QMS) 34 imPress 35 Diagnostic` |
| SkinStatus 1 Other 2 Unknown 3 Door Open 4 Door Closed 5 Interlock Open 6 Interlock Closed |  |  |

PMI Enumerations - Misc

| Media Orientation 1 Other 2 Unknown 3 Portrait 4 Reverse Portrait 5 Landscape 6 Reverse Landscape | Media Color 1 Other 2 Unknown 3 White 4 Pink 5 Yellow 6 Buff 7 Goldenrod 8 Blue 9 Green 10 Transparent | Line Status 1 Other 2 Unknown 3 On Line 4 Pending On Line 5 Off Line 6 Pending Off Line |
| --- | --- | --- |
| Named Finshing 1 Other 2 Unknown 3 Staple 4 Staple Top Left 5 Staple Bottom Left 6 Stable Top Right 7 Staple Bottom Right 8 Saddle Stitch 9 Edge Stitch 10 Punch 11 Cover 12 Bind | ColorantValue - String Other Unknown White Red Green Blue Cyan Magenta Yellow Black Additional names using ISO9070 naming convention. | Light Color 1. Other 2. Unknown 3. White 4. Red 5. Green 6. Blue 7. Cyan 8. Magenta 9. Yellow |
| YesNo -OnOff 1. Other 2. Unknown 3. Yes - On 4. No - Off | ResetType 3. Power Cycle Reset 4. Reset NVRAM 5. Reset to Factory Defaults | ColorantRole 1. Other 2. Unknown 3. Process 4. Spot |
| Training Level 1 Other 2 Unknown 3 Untrained 4 Trained 5 Field Service 6 Management | Channel State 1. Other 2. Unknown 3. Only Print Data Accepted 4. Only Control Data Accepted 5. Print and Control Data Accepted 6. No Data Accepted | PageDeliveryOrientation 1. Other 2. Unknown 3. Face-Up 4. Face-Down |
| StackingOrder 1 Other 2 Unknown 3 1 to n 4 n to 1 |  |  |

PMI Enumerations - Alert Codes

| Severity Level Alert Code 1 Other 2 Unknown 3 Critical 4 Warning |  | Alert Group Code 1 Other 2 Unknown 3 Input 4 Output 5 Finisher 6 Media Path 7 General Printer 8 Marker 9 System Controller 10 Interface 11 Interpreter 12 Operator Console 13 Channel |
| --- | --- | --- |
| Input Alert Code 1 Other 2 Unknown 3 Cover Open 4 Cover Closed 5 Interlock Open 6 Interlock Closed 7 Configuration Change 8 Jam 9 Media Tray Missing 10 Media Size Change 11 Media Weight Change 12 Media Type Change 13 Media Color Change 14 Media Form Parts Change 15 Media Supply Low 16 Media Supply Empty | Output Alert Code 1 Other 2 Unknown 3 Cover Open 4 Cover Closed 5 Interlock Open 6 Interlock Closed 7 Configuration Change 8 Jam 9 Media Tray Missing 10 Media Tray Almost Full 11 Media Tray Full | Marker Alert Code 1 Other 2 Unknown 3 Cover Open 4 Cover Closed 5 Interlock Open 6 Interlock Closed 7 Configuration Change 8 Jam 9 Toner Empty 10 Ink Empty 11 Print Ribbon Empty 12 Toner Almost Empty 13 Ink Almost Empty 14 Print Ribbon Almost Empty 15 Waste Toner Receptical Almost Full 16 Waste Ink Receptical Almost Full 17 Waste Toner Receptical Full 18 Waste Ink Receptical Full 19 Fuser Under Temperature 20 Fuser Over Temperature 21 OPC LIfe Almost Over 22 OPC Life Over 23 Developer Almost Empty 24 Developer Empty |
| General Printer Alert Code 1 Other 2 Unknown 3 Cover Open 4 Cover Closed 5 Interlock Open 6 Interlock Closed 7 Door Open 8 Door Closed 9 Power Up 10 Power Down | Media Path Device Alert Code 1 Other 2 Unknown 3 Cover Open 4 Cover Closed 5 Interlock Open 6 Interlock Closed 7 Configuration Change 8 Jam 9 Media Tray Missing 10 Media Tray Almost Full 11 Media Tray Full | Finisher Alert Code 1 Other 2 Unknown 3 Cover Open 4 Cover Closed 5 Interlock Open 6 Interlock Closed 7 Configuration Change 8 Jam 9 Media Tray Missing 10 Supply Empty 11 Supply Low 12 Receptical Almost Full 13 Receptical Full |
| Interpreter Alert Code 1 Other 2 Unknown 3 Configuration Change 4 Memory Increase 5 Memory Decrease 6 Cartridge Added 7 Cartridge Deleted 8 Resource Added 9 Resource Deleted 10 Resource Unavailable | Interface Alert Code 1 Other 2 Unknown 3 Configuration Change | Channel Alert Code 1 Other 2 Unknown 3 Configuration Change |