Considerations for CIM Alert Indications

Printer PRTALERTS vs CIM Alert Indications

| Printer MIB element | Type | CIM Alert Indication |
| --- | --- | --- |
| prtAlertIndex | Integer32 |  |
| prtAlertSeverityLevel | PrtAlertSeverityLevelTC | PerceivedSeverity |
| prtAlertTrainingLevel | PrtAlertTrainingLevelTC |  |
| prtAlertGroup | PrtAlertGroupTC |  |
| prtAlertGroupIndex | Integer32 |  |
| prtAlertLocation | Integer32 |  |
| prtAlertCode | PrtAlertCodeTC |  |
| prtAlertDescription | PrtLocalizedDescriptionStringTC | Description |
| prtAlertTime | TimeTicks |  |
|  |  | AlertingManagedElement |
|  |  | AlertType |
|  |  | ProbableCause |
|  |  | MessageID |
|  |  | MessageArguments |
|  |  | Message |
|  |  | OwningEntity |

Review of PRTALERT element definitions

PrtAlertSeverityLevel

other(1),

critical(3),

warning(4),

warningBinaryChangeEvent(5)

PrtAlertTrainingLevel

"The level of training required to handle this alert, if human intervention is required.  The noInterventionRequired value should be used if the event does not require any human intervention.  The training level is an enumeration that is determined and assigned by the printer manufacturer based on the information or training required to handle this alert.”

other(1),

unknown(2),

untrained(3),	 Alerts that can be fixed without prior training either because the action to correct the alert is obvious or the printer can help the untrained person fix the problem.  A typical example of such an alert is reloading paper trays or emptying output bins on a low end printer.

trained(4),	 Alerts that require an intermediate or moderate knowledge of the printer and its subunits.  A typical example of such an alert is replacing a toner cartridge.

fieldService(5),	 Alerts that typically require advanced training and technical knowledge of the printer and its subunits.  An example of a technical person would be a manufacturer's Field Service representative, or other person formally trained by the manufacturer or similar representative.

management(6),	 Alerts that have to do with overall operation of and configuration of the printer.  Examples of such management events are configuration change of subunits."

noInterventionRequired(7)

Alert Group

| Code | Group |  |
| --- | --- | --- |
| 1 | Other |  |
| 3 | hostResourcesMIBStorageTable |  |
| 4 | hostResourcesMIBDeviceTable |  |
| 5 | generalPrinter |  |
| 6 | cover |  |
| 7 | localization |  |
| 8 | input |  |
| 9 | output |  |
| 10 | marker |  |
| 11 | markerSupplies |  |
| 12 | markerColorant |  |
| 13 | mediaPath |  |
| 14 | channel |  |
| 15 | interpreter |  |
| 16 | consoleDisplayBuffer |  |
| 17 | consoleLights |  |
| 18 | alert |  |
| 5 to 29 |  | reserved for Printer MIB |
| 30 | finDevice |  |
| 31 | finSupply |  |
| 32 | finSupplyMediaInput |  |
| 33 | finAttribute |  |
| 30 to 39 |  | reserved for Finisher MIB |

prtAlertGroupIndex

"The low-order index of the row within the table identified  by prtAlertGroup that represents the sub-unit of the printer  that caused this alert, or -1 if not applicable.  The  combination of the prtAlertGroup and the prtAlertGroupIndex  defines exactly which printer sub-unit caused the alert; for  example, Input #3, Output#2, and Marker #1.  Every object in  this MIB is indexed with hrDeviceIndex and optionally, another  index variable.  If this other index variable is present in the  table that generated the alert, it will be used as the value  for this object.  Otherwise, this value shall be -1.”

prtAlertLocation

"The sub-unit location that is defined by the printer manufacturer to further refine the location of this alert  within the designated sub-unit.  The location is used in  conjunction with the Group and GroupIndex values; for example,  there is an alert in Input #2 at location number 7.  The value  (-2) indicates unknown.

Alert Code

| Code | Alert |  |
| --- | --- | --- |
| 1 | Other | an event that is not represented by one of the other specified alert codes |
| 2 | unknown | The following generic codes are common to multiple groups. The NMS may examine the prtAlertGroup object to determine what group to query for further information. |
| 3 | coverOpen |  |
| 4 | coverClosed |  |
| 5 | interlockOpen |  |
| 6 | interlockClosed |  |
| 7 | configurationChange |  |
| 8 | jam |  |
| 9 | subunitMissing |  |
| 10 | subunitLifeAlmostOver |  |
| 11 | subunitLifeOver |  |
| 12 | subunitAlmostEmpty |  |
| 13 | subunitEmpty |  |
| 14 | subunitAlmostFull |  |
| 15 | subunitFull |  |
| 16 | subunitNearLimit |  |
| 17 | subunitAtLimit |  |
| 18 | subunitOpened |  |
| 19 | subunitClosed |  |
| 20 | subunitTurnedOn |  |
| 21 | subunitTurnedOff |  |
| 22 | subunitOffline |  |
| 23 | subunitPowerSaver |  |
| 24 | subunitWarmingUp |  |
| 25 | subunitAdded |  |
| 26 | subunitRemoved |  |
| 27 | subunitResourceAdded |  |
| 28 | subunitResourceRemoved |  |
| 29 | subunitRecoverableFailure |  |
| 30 | subunitUnrecoverableFailure |  |
| 31 | subunitRecoverableStorageError |  |
| 32 | subunitUnrecoverableStorageError |  |
| 33 | subunitMotorFailure |  |
| 34 | subunitMemoryExhausted |  |
| 35 | subunitUnderTemperature |  |
| 36 | subunitOverTemperature |  |
| 37 | subunitTimingFailure |  |
| 38 | subunitThermistorFailure |  |
|  | -- General Printer group | Alert Group 5 |
| 503 | powerUp |  |
| 504 | powerDown |  |
| 505 | printerNMSReset | The printer has been reset by some network management station(NMS) writing into 'prtGeneralReset' |
| 506 | printerManualReset | The printer has been reset manually. |
| 507 | printerReadyToPrint | The printer is ready to print. (i.e., not warming up, not in power save state, not adjusting print quality, etc.). |
|  | -- Input Group | Alert Group 8 |
| 801 | inputMediaTrayMissing |  |
| 802 | inputMediaSizeChange |  |
| 803 | inputMediaWeightChange |  |
| 804 | inputMediaTypeChange |  |
| 805 | inputMediaColorChange |  |
| 806 | inputMediaFormPartsChange |  |
| 807 | inputMediaSupplyLow |  |
| 808 | inputMediaSupplyEmpty |  |
| 809 | inputMediaChangeRequest | An interpreter has detected that a different medium is need in this input tray subunit. The prtAlertDescription may be used to convey a human readable description of the medium required to satisfy the request. |
| 810 | inputManualInputRequest | An interpreter has detected that manual input is required in this subunit. The prtAlertDescription may be used to convey a human readable description of the medium required to satisfy the request. |
| 811 | inputTrayPositionFailure | The input tray failed to position correctly. |
| 812 | inputTrayElevationFailure |  |
| 813 | inputCannotFeedSizeSelected |  |
|  | -- Output Group | Alert Group 9 |
| 901 | outputMediaTrayMissing |  |
| 902 | outputMediaTrayAlmostFull |  |
| 903 | outputMediaTrayFull |  |
| 904 | outputMailboxSelectFailure |  |
|  | -- Marker group | Alert Group 10 |
| 1001 | markerFuserUnderTemperature |  |
| 1002 | markerFuserOverTemperature |  |
| 1003 | markerFuserTimingFailure |  |
| 1004 | markerFuserThermistorFailure |  |
| 1005 | markerAdjustingPrintQuality |  |
|  | -- Marker Supplies group | Alert Group 11 |
| 1101 | markerTonerEmpty |  |
| 1102 | markerInkEmpty |  |
| 1103 | markerPrintRibbonEmpty |  |
| 1104 | markerTonerAlmostEmpty |  |
| 1105 | markerInkAlmostEmpty |  |
| 1106 | markerPrintRibbonAlmostEmpty |  |
| 1107 | markerWasteTonerReceptacleAlmostFull |  |
| 1108 | markerWasteInkReceptacleAlmostFull |  |
| 1109 | markerWasteTonerReceptacleFull |  |
| 1110 | markerWasteInkReceptacleFull |  |
| 1111 | markerOpcLifeAlmostOver |  |
| 1112 | markerOpcLifeOver |  |
| 1113 | markerDeveloperAlmostEmpty |  |
| 1114 | markerDeveloperEmpty |  |
| 1115 | markerTonerCartridgeMissing |  |
|  | -- Media Path Device Group | Alert Group 13 |
| 1301 | mediaPathMediaTrayMissing |  |
| 1302 | mediaPathMediaTrayAlmostFull |  |
| 1303 | mediaPathMediaTrayFull |  |
| 1304 | mediaPathCannotDuplexMediaSelected |  |
|  | -- Interpreter Group | Alert Group 15 |
| 1501 | interpreterMemoryIncrease |  |
| 1502 | interpreterMemoryDecrease |  |
| 1503 | interpreterCartridgeAdded |  |
| 1504 | interpreterCartridgeDeleted |  |
| 1505 | interpreterResourceAdded |  |
| 1506 | interpreterResourceDeleted |  |
| 1507 | interpreterResourceUnavailable |  |
| 1509 | interpreterComplexPageEncountered | The interpreter has encountered a page that is too complex for the resources that are available. |
|  | -- Alert Group | Alert Group 18 |
| 1801 | alertRemovalOfBinaryChangeEntry | A binary change event entry has been removed from the alert table. This unary change alert table entry is added to the end of the alert table. |